ATN AIR/GROUND APPLICATIONS

DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CM-016 |  |
| --- | --- | --- | --- |
| SARPs affected: | CM | SARPs Version/Date | 3.0 (21 June 1996) |
| Originator Name: | CSC |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | 19 August 96 |  |  |
| Location of Defect (including Section Number): 2.1.5.3.2.3.1.1 |  |  |  |
| Summary of the Defect: The check for the D-START User Data parameter to be a [CMLogonResponse] APDU will not work for the case where the CM-air-ASE version number is greater than the CM-ground-ASE version number. This is because the logon response in that case does not contain a [CMLogonResponse] APDU; it only contains the D-START DS User Version Number (the CM-ground-ASE version number) and the D-START Result (rejected(permanent)). This case must not be excluded. Additionally, the paragraph numbering should be 2.1.5.3.2.3.1 instead of 2.1.5.3.2.3.1.1. |  |  |  |
| Proposed solution or assumptions made (if any): Reword as follows: ###### 2.1.5.3.2.3.1 Upon receipt of a D-START confirmation, if the CM-air-ASE is in the LOGON state and if the APDU contained in the D-START User Data parameter is a [CMLogonResponse] APDU or if the D-START User Data parameter is not present but the D-START DS User Version Number parameter is present, the CM-air-ASE shall: |  |  |  |
| Editors Comment: |  |  |  |
| Date of Resolution: |  |  |  |