ATN AIR/GROUND APPLICATIONS

DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CM-017 |  |
| --- | --- | --- | --- |
| SARPs affected: | CM | SARPs Version/Date | 3.0 (21 June 1996) |
| Originator Name: | CSC |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | 19 August 96 |  |  |
| Location of Defect (including Section Number): 2.1.7.1.1.3, 2.1.7.1.2.2 |  |  |  |
| Summary of the Defect: The CM-Ground-User requirements state that upon receiving a CM-Logon service indication, it creates an actual TSAP for each aircraft application contained in the Logon Request based on the IDP and long TSAP. There is no corresponding requirement for the CM-Air-User upon receiving a CM-Logon confirmation or a CM-Update indication. These should be added for consistency. |  |  |  |
| Proposed solution or assumptions made (if any): Add the following: 2.1.7.1.1.3 Upon receipt of a CM-logon service confirmation, the CM-air-user shall create the actual TSAP for each ground application information contained in the Logon Response based on the IDP and long TSAP for each application as defined in 2.1.4. 2.1.7.1.2.2 Upon receipt of a CM-update service indication, the CM-air-user shall create the actual TSAP for each ground application information contained in the Update Information based on the IDP and long TSAP for each application as defined in 2.1.4. |  |  |  |
| Editors Comment: |  |  |  |
| Date of Resolution: |  |  |  |