ATN AIR/GROUND APPLICATIONS

DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CM-018 |  |
| --- | --- | --- | --- |
| SARPs affected: | CM | SARPs Version/Date | 3.0 (21 June 1996) |
| Originator Name: | CSC |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | 26 August 96 |  |  |
| Location of Defect (including Section Number): Chapter 4 |  |  |  |
| Summary of the Defect: The ASN.1 module is formally incomplete. The module defines 2 groups of messages for the CM application with a CHOICE identifier for each group. Without a CHOICE identifier at a higher level, the ASN.1 definition does not enable a PER encoder/decoder to distinguish the group to which a given message belongs. In addition, the VersionNumber ASN.1 needs to be modified so it cannot be zero (which is reserved for the dialogue service). |  |  |  |
| Proposed solution or assumptions made (if any): Added to chapter 4: 2.1.4.1 Encoding/decoding Rules 2.1.4.1.1 A CM-air-ASE shall be capable of encoding CMAircraftMessage APDUs and decoding CMGroundMessage APDUs. 2.1.4.1.2 A CM-ground-ASE shall be capable of encoding and decoding CMGroundMessage APDUs and decoding CMAircraftMessage ADPUs. VersionNumber ::= INTEGER (1..255) --VersionNumber 0 is reserved for the Dialogue Service |  |  |  |
| Editors Comment: |  |  |  |
| Date of Resolution: |  |  |  |