ATN AIR/GROUND APPLICATIONS

DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CM-023 |  |
| --- | --- | --- | --- |
| SARPs affected: | CM | SARPs Version/Date | 3.0 (21 June 1996) |
| Originator Name: | Thomson |  |  |
| Originator Reference: | P/C Report 22, 23 |  |  |
| Date Raised: | 26 July 96 |  |  |
| Location of Defect (including Section Number): 2.1.5.4.3.1, 2.1.5.4.4.1 |  |  |  |
| Summary of the Defect: The definitions of invalid PDU and not permitted PDU are not explicit. |  |  |  |
| Proposed solution or assumptions made (if any): Add/modify the following to clarify/correct: 2.1.5.3.1.2 Upon receipt of a PDU, if no actions are described for the arrival of that PDU when a CM-ASE is in a specific state, then that PDU is considered not permitted and exception handling procedures as described in 2.1.5.4.4 shall apply. #### 2.1.5.3.1.3 If a PDU is received that cannot be decoded, then that PDU is considered an invalid PDU and exception handling procedures as described in 2.1.5.4.3 shall apply. 2.1.5.3.1.4 If a PDU is not received where one is expected, then that PDU is considered an invalid PDU and exception handling procedures as described in 2.1.5.4.3 shall apply. 2.1.5.4.3.1 If the User Data parameter of a D-START indication or D-DATA indication does not contain a valid PDU as defined in 2.1.5.3.1.3 and 2.1.5.3.1.4, the CM-ASE shall: 2.1.5.4.3.2 If the User Data parameter of a D-START confirmation does not contain a valid PDU as defined in 2.1.5.3.1.3 and 2.1.5.3.1.4, the CM-ASE shall: 2.1.5.4.4.1 If the User Data parameter of a D-START indication or D-DATA indication is a valid PDU, but is not a permitted PDU as defined within 2.1.5.3.1.2, the CM-ASE shall: 2.1.5.4.4.2 If the User Data parameter of a D-START confirmation is a valid PDU, but is not a permitted PDU as defined within 2.1.5.3.1.2, the CM-ASE shall: |  |  |  |
| Editors Comment: |  |  |  |
| Date of Resolution: |  |  |  |