SARPs Version Number 				CM 3.1 / 07-10-1996

Defect Report Number 				CM034

Originator					SG2

Originator's Reference (where applicable)

Date						30 September 1996

Location in SARPs				2.1.5.3.3.2.1, 2.1.5.3.3.2.2

Status (OPEN, CLOSED or REJECTED)		OPEN

Date of Resolution

Problem

Both aircraft and ground messages can be received by a ground ASE (normal aircraft messages and forward request ground messages), and the ground ASE must be able to recognize both (see defect CM018).  However, there is no wording to describe how a ground ASE will know if the originator of a D-START Indication is an aircraft or a ground system.

Editor's Proposed Solution (optional):

Modify sections 2.1.5.3.3.2.1, 2.1.5.3.3.2.2

2.1.5.3.3.2.1  Upon receipt of a D-START indication, if the CM-ground-ASE is in the IDLE state and the APDU contained in the D-START User Data parameter is a [CMLogonRequest] APDU and the abstract value of the D-START Calling Peer ID parameter is a 24 bit Aircraft Address then:

2.1.5.3.3.2.2 Upon receipt of a D-START indication, if the receiving CM-ground-ASE is in the IDLE state and the APDU contained in the D-START User Data parameter is a [CMForwardRequest] APDU abstract value of the D-START Peer ID  8 caracter ICAacility Designator then: