SARPs Version Number 				ICAO CM 1.0 /  14 - 11-1996

Defect Report Number 				IV1.0_CM01

Originator					TES (EUROCONTROL)

Originator's Reference (where applicable)		tes-cm-062

Date						7 November 1996

Location in SARPs				2.1.5.3.3.6.1, 2.1.7.2.2.7

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				17 December 1996

Fixed in 					Proposed ICAO CM 1.1

Problem

When the CM-ground-user does not support the maintain dialougue capability (MA-FU), the maintain dialogue parameter of the CM-logon service response shall be ignored if provided by the CM-ground-user and the CM-ground-ASE shall act as if the maintain parameter was not provided by the CM-ground-user.

This is not the case in the current version of the SARPs.  In section 2.1.5.3.3.6.1, whatever the configuration of the CM-ground-ASE is, the maintain parameter is taken into account.

It is proposed to change section 2.1.5.3.3.6.1 as follows:

b) invoke D-START response with the following:

1)

2)  if the CM-ground-ASE does not support the dialogue maintain service of if the CM-logon Maintain parameter is no provided by the CM-ground-user,

i)

ii)

3) if the CM-ground-ASE does not support the dialogue maintain service of ir the CM-logon Maintain parameter is not provided by the CM-ground-user,

i)

ii)

In addition, section 2.1.7.2.2.7 should be modified to reflect the fact that the CM-ground-user can request a CM dialogue to be maintained if and only if the dialogue maintain service is supported by its CM service provider.

Editor's Proposed Solution (optional)

It is agreed that a user requirement needs to added to prohibit the user from asking for maintaining a dialogue when one is not allowed.  With this user requirement added, the change to the protocol in section 2.1.5.3.3.6.1 is not needed.

The proposed text change is:

2.1.7.2.2.7   When the CM-ground-user requires a CM dialogue to be maintained, the CM-ground-user shall set the CM-logon service response Maintain Dialogue parameter, if and only if the dialogue maintain service is supported by the CM-ground-ASE.