SARPs Version Number 				ICAO CM 1.0 /  14 - 11-1996

Defect Report Number 				IV1.0_CM02

Originator					TES (EUROCONTROL)

Originator's Reference (where applicable)		tes-cm-064

Date						5 December 1996

Location in SARPs				2.1.7

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				17 December 1996

Fixed in 					Proposed ICAO CM 1.1

Problem

When the CM-ground-user invokes the CM-update request primitive, if it uses the ShortTSAP form instead of the LongTSAP form for the address of the air initiated applications, the aircraft is unable to build the complete address (it does not know the RDP and ARS parts of the calling ground CM and has no means to retrieve them).

Chapter 7 shall mandate the use of the LongTSAP form in the addressing data of the CM-update service.

Editor's Proposed Solution (optional)

Add section in Chapter 2.1.7:

2.1.7.2.3.2	When invoking the CM-update service request, the CM-ground-user shall use the Long TSAP for each application address provided.