DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CPDLC-v3-11 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | EOLIA Project | CENA/DGAC |  |
| Originator Reference: | Stéphane Marche | cpc0003 |  |
| Date Raised: | 20 August 1996 |  |  |
| Location of Defect (including Section Number: Chapters 4 and 7 |  |  |  |
| Summary of the Defect: The ERROR message as defined in the SARPs allows for several values but there is no place for defining new error reasons, e.g. regionally defined. As there is a restriction defined in chapter 7 which prevents the users from using the ERROR message element with another one. It is therefore not possible to send with ERROR message element a FREE TEXT message element containing a reason. For instance, some combinations of message elements could be prohibited in some areas (e.g. in Europe the message elements CONTACT and MONITOR will be prohibited). It should be possible to send an error code (defined for Europe) identifying a wrong combination of messages. |  |  |  |
| Proposed solution or assumptions made (if any): Solution #1: The ASN.1 could be modified as follows: ErrorInformation::=CHOICE { definedValues CPDLCErrorCode regionalVaues INTEGER } CPDCErrorCode::=ENUMERATED { unrecognizedMsgReferenceNumber (0), endServiceWithPendingMsgs (1), logicalAcknowledgmentNotAccepted (2), MoreThanOneNextDataAuthorityElement (3), insufficientMessageStorageCapacity (4), ... } Solution #2: add a new value ‘undefined’ in the ASN.1 type ErrorInformation indicating that the error code is regionally defined and that a FREE TEXT message element is attached with the ERROR message element containing the reason. |  |  |  |
| Editors Comment: ### This defect was discussed at the August WG3/SG2 meeting in Silver Spring. It was rejected for several reasons. In ASN.1 there can be no regional adaptation, all elements must be understood by everyone even if they are not used. Also the ERROR massage cannot be combined with any other message element (including FREE TEXT) so solution 2 won’t work. . |  |  |  |
| Date of Resolution: | August 22 1996/Rejected/Closed |  |  |