DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | v3-13 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | S. Sanders | CPDLC/SS-10 |  |
| Originator Reference: |  |  |  |
| Date Raised: | 3 July 1996 |  |  |
| Location of Defect (including Section Number: Chapter 7 Section 7.6.3.1 |  |  |  |
| Summary of the Defect: Section 7.6.3.1 does not prohibit a message transfer attempt after an End request has been invoked. |  |  |  |
| Proposed solution or assumptions made (if any): Section 7.6.3.1 should prohibit a message transfer attempt after an End request has been invoked. Add the following requirement to section 7.6.3.1: 7.6.3.1.x.x If a CPDLC-ground user has invoked a CPDLC-end Service Request, the ground-user shall be prohibited from invoking a message transfer attempt. |  |  |  |
| Editors Comment: I think you are right I guess we need this new section, but before I make the change I will review it with the sub-group (change control board) meeting Aug 26-30 Proposed new section: 2.3.7.5.3.1.3. If a CPDLC-ground user has invoked a CPDLC-end service request primitive, the ground-user shall be prohibited from invoking any CPDLC service primitive with this aircraft, except the CPDLC-user-abort request primitive, until after It has received a CPDLC-end service confirmation primitive. The same will apply to the air user invoking a DSC request primitive so the following new section is also needed. 2.3.7.4.5.1.2. If a CPDLC-air user has invoked a DSC-end service request primitive, the air-user shall be prohibited from invoking a any CPDLC service primitive with this ground system (except the CPDLC-user-abort request primitive) until it receives a DSC-end service confirmation primitive. Steve I think this is basically right but there is period of time where you could have a DSC and a CPDLC dialogue simultaneously with the same aircraft/ground system pair and they are independent things so I may have to tighten this up more yet |  |  |  |
| Date of Resolution: | 25 August 1996, Accepted |  |  |