DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | v3-19 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | T. Kerr |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | 3 August |  |  |
| Location of Defect (including Section Number: 2.3.3 Figure 2.3.3-1 and corresponding notes. |  |  |  |
| Summary of the Defect: Since the CPDLC service is abstract, the word interface should not be used since this infers a specific implementation.. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: The figure has been updated to remove any reference to “interface” Notes 3-8 have been updated as follows: Note 3. — Figure 2.3. 3-1 shows the functional model of the CPDLC Application. The functional modules identified in this model are the following : - the CPDLC-user, - the CPDLC Application Entity (CPDLC-AE) service, - the CPDLC-AE, - the CPDLC Control Function (CPDLC-CF), - the CPDLC Application Service Element (CPDLC-ASE) service, - the CPDLC-ASE, and - the Dialogue Service (DS). Note 4. — The CPDLC-user represents the operational part of the CPDLC system. This user does not perform the communication functions but relies on a communication service provided to it via the CPDLC-AE through the CPDLC-AE service. The individual actions possible through the CPDLC-AE service called service primitives.. Note 5. — The CPDLC-AE consists of several elements including the CPDLC-ASE and the CPDLC-CF. Note 6. — The CPDLC-ASE is the element in the communication system which executes the CPDLC specific protocol. In other words, it takes care of the CPDLC specific service primitive sequencing , message creation, timer management, error and exception handling. Note 7. —This CPDLC-CF is responsible for mapping service primitives received from one element (such as the CPDLC-ASE and the CPDLC-user) to service primitives of other abstract elements.. Note 8. — The CPDLC-ASE has two abstract boundaries with the CPDLC-CF: the CPDLC-ASE service and the dialogue service. The CPDLC-CF maps CPDLC-AE service primitives to CPDLC-ASE service primitives transparently and vice versa. The CPDLC-CF maps dialogue service primitives to other abstract elements in the CPDLC-AE and the underlying communication service, and vice versa.]. |  |  |  |
| Date of Resolution: | 25 August 1996, Accepted |  |  |