DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | v3-28 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | ADSP/4 |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | 15 September 1996 |  |  |
| Location of Defect (including Section Number: Chapter 4, 5, 7 |  |  |  |
| Summary of the Defect: Change in operational requirements. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: #### 1. The following uplink messages response requirements have changed form a “R” to a “W/U” and chapter 4 and chapter 7 have been modified 127,128,129, 130, 180 #### 2. Downlink messages 185 and 186 have had [direction][distanceOffset] changed to [distanceOffset][direction]. #### 3. Downlink message 185 has had a time variable added. #### 4. Definition of Frequencyvhf resolution modified from 0.00833 to 0.025. #### 5 The priority for CPDLC and ADS has been changed from “normal priority flight safety messages” to “high priority flight safety messages” This affects sections 2.3.5.3.7.1, 2.3.5.3.9.1, 2.3.5.5.7.1, and 2.3.5.5.13.1 |  |  |  |
| Date of Resolution: | 28 September 1996, Accepted/Closed |  |  |