DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CPDLC-v3-6 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | R. Beppel | CSC |  |
| Originator Reference: | TSP1A-043 |  |  |
| Date Raised: | June 24 1996 |  |  |
| Location of Defect (including Section Number: CPDLC Version Number |  |  |  |
| Summary of the Defect: The newly added Forward Service provides requirements for ensuring version number compatibility between two ground users. However, no requirements are provided to ensure version number compatibility between ground and air users during dialog establishment. Clarification is needed on the distinction between the two case and the use of version numbers. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: The reason version negotiation is in the ground/ground is because there is no CM to do the version negotiation as in the air/ground case. In the air/ground case CM gives all available applications and version numbers so when CPDLC is initiated for air/ground, version numbers are already known and should be used appropriately. A problem could occur when they are not but when sub-group 2 was working on the SARPs we decided CM version negotiation was sufficient for air/ground. This is not the case for ground/ground. There is no concept of CM here so each ground/ground application has to perform version negotiation upon dialogue establishment. |  |  |  |
| Date of Resolution: | August 22 1996, Nochange, Closed |  |  |