DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-12 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | Sub-group 2 |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Dec 96 |  |  |
| Location of Defect (including Section Number: Chapter 4 |  |  |  |
| Summary of the Defect: FacilityDesignation should be mapped to FacilityDesignation not UnitName |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: ASN.1 definitions have been modified as follows: FacilityDesignationAltimeter ::= SEQUENCE { facilityDesignation UnitNameFacilityDesignation, altimeter Altimeter } FacilityDesignationATISCode ::= SEQUENCE { facilityDesignation UnitNameFacilityDesignation, aTISCode ATISCode } |  |  |  |
| Date of Resolution: | Dec 1996 |  |  |