DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-16 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | Dung Nguyen |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Jan 97 |  |  |
| Location of Defect (including Section Number: Chapter 4, chapter 7 |  |  |  |
| Summary of the Defect: Reserved message element should never be sent. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: 1. New ERROR in chapter 4: ErrorInformation ::= ENUMERATED { unrecognizedMsgReferenceNumber (0), endServiceWithPendingMsgs (1), logicalAcknowledgmentNotAccepted (21), moreThanOneNextDataAuthorityElement (3), insufficientResources (42), invalidMessageElementCombination (3), invalidMessageElement (4), ... } 2. New sections added in chapter 7: 2.3.7.3.10 Invalid Message Elements The CPDLC-ground-user shall be prohibited from sending any CPDLC message containing either the uplink message element 33 or 178. 2.3.7.3.11.6 Invalid Message Elements If the CPDLC-air-user receives a message containing either the uplink message element identifier 33 or 178 the CPDLC-air-user shall invoke the CPDLC-message request with the ERROR [errorinformation] message element with the [invalidMessageElement] value as the CPDLC Message parameter, and disregard the received message. |  |  |  |
| Date of Resolution: |  |  |  |