DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-18 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | Jane Hamelink |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Feb 97 |  |  |
| Location of Defect (including Section Number: Chapter 7 |  |  |  |
| Summary of the Defect: Since NDA was modified to a stand alone message element chapter 7 contains redundant requirements.. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: Mods as Noted: 2.3.7.4.3.1.2 If a CPDLC-message indication is received from the Current Data Authority containing at least only the uplink message element NEXT DATA AUTHORITY, and having no more than one NEXT DATA AUTHORITY message elements, indicating that the specifiedspecifying a facility designation isas the Next Data Authority the CPDLC-air-user shall do the following in the order listed: 2.3.7.4.3.1.3 If a CPDLC-message indication is received from the Current Data Authority containing more than one NEXT DATA AUTHORITY uplink message elements, the CPDLC-air-user shall disregard the Next Data Authority designations, and invoke CPDLC-message service request with the ERROR [errorinformation] message element with the [moreThanOneNextDataAuthorityElement] value as the CPDLC Message parameter value. 2.3.7.4.3.1.4 If a CPDLC-message indication is received containing at least the uplink message element NEXT DATA AUTHORITY, and it is not from the Current Data Authority the message shall be disregarded. |  |  |  |
| Date of Resolution: |  |  |  |