DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-19 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | Dung Nguyen |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Jan 97 |  |  |
| Location of Defect (including Section Number: Chapter 7 |  |  |  |
| Summary of the Defect: Requirements specify what to do with CDA, NDA if air user receives an abort, but what if air user issues an abort? |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: Following Requirements added: 2.3.7.4.6.1 Issuing a CPDLC-user-abort Request [commanded-termination] 2.3.7.4.6.1.1 The CPDLC-air-user shall have the capability to invoke CPDLC-user-abort request with the Reason parameter set to CPDLCUserAbortReason value [commanded-termination]. 2.3.7.4.6.2 Invoking a CPDLC-user-abort Request 2.3.7.4.6.2.1 If the CPDLC-air-user invokes CPDLC-user-abort request with the Current Data Authority, the CPDLC-air-user shall: - Delete any association of a ground system to a Current Data Authority, - If a ground system is designated as Next Data Authority and an association with a CPDLC-ASE exists, invoke CPDLC-user-abort request with the Reason parameter set to the value [current-data authority-abort], and - Delete any association of a ground system to a Next Data Authority. 2.3.7.4.6.2.2 If the CPDLC-air-user invokes CPDLC-user-abort request with the Next Data Authority, and the does not set the Reason parameter as [current-data-authority-abort] or [no-longer-next-data-authority], the CPDLC-air-user shall continue to maintain the association of the ground system to the Next Data Authority. |  |  |  |
| Date of Resolution: |  |  |  |