DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-2 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | sub-group 2 |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | 20 Dec 96 |  |  |
| Location of Defect (including Section Number: 2.3.3.9.2 |  |  |  |
| Summary of the Defect: 1. CPDLC SARPs are different than the other 3 applications SARPs that they require an Indication of the Reason parameter in the CPDLC-user-abort services. This needs an explanation an clarifying requirement. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: Because the CPDLC-user can abort a CPDLC or DSC dialogue for operational reasons beyond that which is detectable by the ASE, this capability is included in the ASN.1 as a APDU and is made available for the receiving user. If the initiating user does not supply a reason for the CPDLC user-abort its ASE does (chapter 5 protocol) An explanatory comment has been added to the CPDLC-user-abort service (2.3.3.9.2) as follows: Note 2.— If provided by the user, the parameter indicated to the peer user is that provided by the user, else it is what the ASE supplies. In addition, because a U to the left of an M in the CPDLC-user-abort Reason parameter is an unusual configuration the following requirement has been added to the CPDLC-user-abort service. When this parameter is provided by the user, the same value shall be indicated to the peer user. |  |  |  |
| Date of Resolution: | 5 Feb 1997 |  |  |