DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-20 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | Jane Hamelink |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Feb 97 |  |  |
| Location of Defect (including Section Number: Chapter 4, 7 |  |  |  |
| Summary of the Defect: Need to abort when invalid responses are received. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: User Abort PDU in chapter 4 has been modified as follows: CPDLCUserAbortReason ::= ENUMERATED { undefined (0), no-message-identification-numbers-available (1), duplicate-message-identification-numbers (2), no-longer-next-data-authority (3), current-data-authority-abort (4), commanded-termination (5), invalid-response (6), ... } Chapter 7 additions: 2.3.7.3.11.8 Invalid Message Response 2.3.7.3.11.8.1 If the CPDLC-ground-user sends a message that has a W/U response attribute, and a response to this message is received by the CPDLC-ground-user that does not contain an error or any of the following message elements: WILCO, UNABLE, STANDBY, LOGICAL ACKNOWLEDGMENT, or ERROR[errrorInformation] the CPDLC-ground-user shall invoke CPDLC-user-abort request with a CPDLC message containing the CPDLCUserAbortReason with the value [invalid-response]. 2.3.7.3.11.8.2 If the CPDLC-ground-user sends a message that has an A/N response attribute, and a response to this message is received by the CPDLC-ground-user that does not contain an error or any of the following message elements: AFFIRM, NEGATIVE, STANDBY, LOGICAL ACKNOWLEDGMENT, or ERROR[errrorInformation] the CPDLC-ground-user shall invoke CPDLC-user-abort request with a CPDLC message containing the CPDLCUserAbortReason with the value [invalid-response]. 2.3.7.3.11.8.3 If the CPDLC-ground-user sends a message that has a R response attribute, and a response to this message is received by the CPDLC-ground-user that does not contain an error or any of the following message elements: ROGER, UNABLE, STANDBY, LOGICAL ACKNOWLEDGMENT, or ERROR[errrorInformation] the CPDLC-ground-user shall invoke CPDLC-user-abort request with a CPDLC message containing the CPDLCUserAbortReason with the value [invalid-response]. 2.3.7.3.11.8.4 If the CPDLC-user sends a message that has a N response attribute and requires a logical acknowledgment, and a response to this message is received by the CPDLC-user that does not contain an error or any of the following message elements: LOGICAL ACKNOWLEDGMENT, or ERROR[errrorInformation] the CPDLC-user shall invoke CPDLC-user-abort request with a CPDLC message containing the CPDLCUserAbortReason with the value [invalid-response]. 2.3.7.3.11.8.5 If the CPDLC-user sends a message that has a N response attribute and does not require a logical acknowledgment, and a response to this message is received by the CPDLC-user that does not contain an error the CPDLC-user shall invoke CPDLC-user-abort request with a CPDLC message containing the CPDLCUserAbortReason with the value [invalid-response]. |  |  |  |
| Date of Resolution: | Feb 1997 |  |  |