DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-7 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | Jane Hamelink/ Dung Nguyen |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Feb 97 |  |  |
| Location of Defect (including Section Number: Chapter 4, Chapter 7 |  |  |  |
| Summary of the Defect: To facility transition from any systems prior to the CNS/ATM-1 it was agree no existing message element would be changed, UM 178 was inadvertently deleted and re-used. It will now be reserved and extraneous UM message element 199 has attributes modified to be as UM178. Sending of reserved message elements (UM 33 and UM 178) should result in an error. In addition Reserved messages must continue to havemessage attributes to allow proper processing. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: 1. The ASN.1 has been modified: -- [freetext]Reserved Urg(L)/Alr(L)/Resp(Y) uM33FreeTextNULL [33] FreeTextNULL, -- [freetext]Reserved Urg(L)/Alr(L)/Resp(Y) uM178FreeTextNULL [178] FreeTextNULL, -- [freetext] Urg(N)/Alr(ML)/Resp(W/UN) uM199FreeText [199] FreeText, 2. Chapter 7 Tables as indicated below: |  |  |  |
| Date of Resolution: | 5 Feb 1997 |  |  |

| 178 | Reserved | [free text] | N | L | N |
| --- | --- | --- | --- | --- | --- |
| 199 |  | [free text] | N | ML | W/UN |