SARPs Version Number 				FIS 3.0 /  28-06-1996

Defect Report Number 				drfis025

Originator					SG2 (Silver Spring Meeting)

Originator's Reference (where applicable)		none

Date						26 August 1996 (Silver Spring SG2 meeting)

Location in SARPs				2.4.5.4.1

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				26 August 1996 (Silver Spring SG2 meeting)

Fixed in 					FIS 3.1

Problem

- In the case of an unrecoverable system error, some actions described in 2.4.5.4.1 might be impossible to realize with a degraded system (e.g. creation of an Abort APDU). This may lead to the detection of a new unrecoverable error system situation, and therefore there is a risk of live lock.

- A solution could be to make these actions as part of a recommendation.

Editor's Proposed Solution (optional)

Modified Section 2.4.5.4.1.1

- Original

When any module has an unrecoverable system error, the module shall:

- request the AB module to abort with reason "unrecoverable internal error", and

- enter its IDLE state.

- New

Recommendation. — When any module has an unrecoverable system error, the module should:

- request the AB module to abort with reason "unrecoverable internal error", and

- enter its IDLE state.

Agreed Solution.