SARPs Version Number 				FIS 3.0 /  28-06-1996

Defect Report Number 				drfis034

Originator					ADSP/4 (September 96 Panel Meeting)

Originator's Reference (where applicable)		none

Date						20 September 1996

Location in SARPs				2.4.1.1.2, 2.4.6.2.2.1

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				27 September 1996

Fixed in 					FIS 3.1

Problem

Part V of the Data Link Manual (Data Link FIS) has been modified during the Panel meeting. The FIS SARPs need to be upgraded to reflect the following changes :

- the Data Link Manual  refers to the Demand Mode and the Contract Mode, the FIS SARPs refer to the Demand Contract and the Update Contract,

- the communication priority level is now defined for each FIS service (e.g. ATIS, METAR, etc...) and not for the FIS application.

Editor's Proposed Solution (optional)

Proposed Solution

Added Note to  Section 2.4.1.1.2

- New

Note.— The concept of FIS Demand Contract and the FIS Update Contract used in 2.4  is equivalent to the concept of FIS Demand Mode and FIS Contract Mode developed in the draft ICAO Manual of Air Traffic Services (ATS) Data Link Applications.

Modified  Section 2.4.6.2.2.1

- Old

The application service priority for FIS shall have the abstract value of "Aeronautical Information Service messages".

- New

The application service priority for ATIS shall have the abstract value of "Aeronautical Information Service messages".

- Redline

The application service priority for ATIS shall have the abstract value of "Aeronautical Information Service messages".

Agreed Solution.