Defect Number (to be supplied by SARPs editor):		UL-DR 107

SARPs affected:						ULCS

SARPs Version / Date					ICAO Version 1.0   ATNP/2 (Nov 96)

Originator:						ATNP/WG3/SG2

Originator’s Reference					Bracknell-DR2

Date Raised:						17 December 1996

Location of Defect (including Section Number):		4.2.3.2.1

Status (OPEN, CLOSED or REJECTED)	CLOSED

Date of resolution:		18 February 1997

Fixed in:		Proposed ICAO V2.0 (March 1997)

Problem

QOS parameter support in D-START primitives.

Table 4.2-4 implies that the QOS parameters received in a D-START indication primitive can be different to the values supplied by the peer in the D-START request primitive, and that the QOS parameters received in a D-START confirmation primitive can be different to the values supplied by the peer in the D-START response primitive.  This is not the case.

Section 4.2.3.2.1 Note 7 states: “The [QOS] parameter may be modified by the DS-provider according to what QOS can actually be supported, so that the value in the indication primitive may reflect a reduced QOS compared with the value in the request.”

This is incorrect.  In fact, the ATN Internet will not modify any of the QOS parameters (Transport priority, RER and Security Label) provided by the TS-User.

Proposed solution or assumptions made (if any):

Editor’s Comment:

Agreed Solution:

Modification to Section 4.2.3.1.1, Table 4.2-4

Original text:

Quality of Service	M 	M 	U 	M

Replacement text:

Quality of Service	M	M(=)	U	M(=)

Modification to Section 4.2.3.1.1, Note 7

Original text:

The parameter may be modified by the DS-provider according to what QOS can actually be supported, so that the value in the indication primitive may reflect a reduced QOS compared with the value in the request.

Replacement text:

For ATN, the parameter is not modified by the DS-provider, so the value in the indication primitive is equal to the value in the request.