Defect Number (to be supplied by SARPs editor):		UL-DR 108

SARPs affected:						ULCS

SARPs Version / Date					ICAO Version 1.0   ATNP/2 (Nov 96)

Originator:						CSC Robert Nowlan

Originator’s Reference					(TES PCR-80)

Date Raised:						2 December 1996

Location of Defect (including Section Number):		various

Status (OPEN, CLOSED or REJECTED)	CLOSED

Date of resolution:		18 February 1997

Fixed in:		N/A

Problem

Presentation Provider Abort Handling.

When using the null encoding presentation protocol option, then the ARP (Provider Abort) PPDU is not applicable, as denoted in the ULA SARPs, Table 4.5-8.  Hence, an abnormal release by the Presentation Layer would encapsulate no user data in an S-U-Abort request.  How do we determine then, when a S-U-Abort Ind is received if the remote presentation entity experienced an abnormal release?.  Or do we just map it to a P-U-Abort Ind and let the ACSE CF resolve the fact that there is no ACSE ABRT PDU contained within the P-U-Abort Indication.  The SARPs I don't think mentions handling this case where a P-U-Abort Indication does not contain any user data.

Proposed solution or assumptions made (if any):

In our implementation we in fact added functionality to the CF to handle this case.  A P-U-Abort Ind received with no user data would be mapped to an A-P-Abort Ind.  This will work but I think if this is the desired result then the SARPs should mention handling this occurrence.

Editor’s Comment:

The proposed solution is incorrect.  The ACSE standard says that a P-U-ABORT ind with no User-data implies the existence of an ABRT APDU (i.e. a null-encoded ABRT with no parameters), and this is mapped by the ACSE protocol machine to an A-ABORT indication with Abort Source = "ACSE service-user" (8650-1 para 7.3.3.2.a) - NOT an A-P-ABORT ind.

There is an omission in the ISO Presentation Protocol DAM - it doesn't say exactly how the Presentation connection is abnormally released when an ARP cannot be sent.  ISO 8823-1 says "...the PPM shall issue an ARP PPDU (if there is an underlying session-connection) and a P-P-ABORT indication."

It should say something like:

"If the incoming event is related to a received PPDU or SS-provider event, the PPM shall issue a P-P-ABORT indication.  If there is an underlying session-connection, the PPM shall issue an ARP PPDU if possible, or if the null encoding protocol option has been selected, then issue an S-U-ABORT request primitive with no SS-user-data parameter."

The PPM should then treat a received S-U-ABORT containing no User-data in the same way as it treats an S-U-ABORT with an embedded ARP, i.e. map it to a P-P-ABORT indication primitive.

Agreed Solution:

No change is required to the SARPs text.

Modification to Referenced Standard

A ballot comment has been sent to the ISO DAM editor.

For information, and wider dissemination, the effect of applying the ballot comment to the Presentation Protocol is shown below:

{Amend the first paragraph of 6.2.7.2 as follows}

6.2.7.2	ARU PPDU, ARP PPDU and S-P-ABORT

If the initiating PPM receives an S-P-ABORT indication service primitive or an ARP PPDU, or an S-U-ABORT indication service primitive with no SS-user-data parameter, it shall issue a P-P-ABORT indication service primitive and the presentation-connection shall not be established.

## 6.4 Abnormal release of connection

## {Amend text as indicated}

6.4.4.2 Protocol error

When a PPM receives an unrecognised or unexpected PPDU, or an unexpected session-service primitive, it shall issue a P-P-ABORT indication service primitive and, if possible, send an ARP PPDU. If the null encoding protocol option has been selected, then issue an S-U-ABORT request primitive with no SS-user-data parameter. The presentation-connection shall be released.

6.4.4.3 Invalid PPDU

When a PPM receives a PPDU, containing an invalid PPDU parameter value or an unrecognised or unexpected PPDU parameter, including a PPDU with an unexpected presentation context identifier, or one for which the received bitstring does not represent a valid presentation data value (including any embedded presentation data value) in the corresponding abstract syntax, it shall issue a P-P-ABORT indication service primitive and send an ARP PPDU, if possible. If the null encoding protocol option has been selected, then issue an S-U-ABORT request primitive with no SS-user-data parameter.  The presentation-connection shall be released.

6.4.4.6 ARP PPDU

When a PPM receives an ARP PPDU, or an S-U-ABORT indication service primitive with no SS-user-data parameter, it shall issue a P-P-ABORT indication service primitive and the presentation-connection shall be released.

{Replace A.4.1.2 as follows}

A.4.1.2 If the incoming event is related to a received PPDU or SS-provider event, the PPM shall issue a P-P-ABORT indication.  If there is an underlying session-connection, the PPM shall issue an ARP PPDU if possible, or if the null encoding protocol option has been selected, then issue an S-U-ABORT request primitive with no SS-user-data parameter.

{Add the following row to Table 16 after S-P-ABind)

| Abbreviated Name | Category | Name and Description |
| --- | --- | --- |
| S-U-ABind-nd | SS primitive | S-U-ABORT indication with no SS-user-data parameter |

{Add the following bottom row to Table 23}

|  | STA11 await CPA | STA12 await P-CONrsp | STAac0 await ACA | STAac1 await P-ALTERrsp | STAac2 await ACA or P-ALTERrsp | STAt0 connected - data transfer |
| --- | --- | --- | --- | --- | --- | --- |
| S-U-ABind-nd | P-PABind STAI0 | P-PABind STAI0 | P-PABind STAI0 | P-PABind STAI0 | P-PABind STAI0 | P-PABind STAI0 |