# ADS USER REQUIREMENTS

## General

### General Primitive Requirements

#### When an ADS-ground-user invokes ADS-demand-contract request, ADS-event-contract request or ADS-periodic-contract request, it shall  ([298])  then be prohibited from invoking ADS-start-forward request, ADS-forward-report request and ADS-end-forward request.

#### When an ADS-ground-user invokes ADS-start-forward request, it shall  ([299])  then be prohibited from invoking ADS-demand-contract request, ADS-event-contract request, ADS-periodic-contract request, ADS-cancel request, ADS-cancell-all-contracts request and ADS-modify-emergency-contract request.

### General Parameter Requirements

#### When an ADS-ground-user invokes ADS-demand-contract request, ADS-event-contract request or ADS-periodic-contract request, it shall  ([300])  set the aircraft identifier parameter to be the 24 bit ICAO code of the aircraft with which it requires a contract.

#### When an ADS-ground-user invokes ADS-demand-contract request, ADS-event-contract request or ADS-periodic-contract request and requires a particular class of communication service, it shall  ([301])  set the class of communication service parameter to be the class of communication service it requires.

#### When an ADS-ground-user invokes ADS-demand-contract request, ADS-event-contract request or ADS-periodic-contract request and has no preference for the class of communication service to be used, or there is an existing contract in place and the ADS-ground-user requires the existing class of communication service parameter, it shall  ([302])  not provide the class of communication service parameter.

Note 1.— It will be necessary for an implementation to provide information which is both accurate and timely in the ADS reports, however, quantification of the age and accuracy of the information is beyond the scope of this document.

### Timing Requirements

#### Whenever an ADS-air-user recieves an indication that requires a response, it shall  ([303])  invoke the response within 0.5 seconds.

#### Whenever a periodic contract or an emergency contract is in force, the ADS-air-user shall  ([304])  invoke ADS-report request or ADS-emergency report request (as described below) as close to the reporting rate as possible, and in any case within 0.5 seconds of the reporting rate as measured from the previous report.

### Error Handling

#### If the ADS-air-user has an unrecoverable system error, and if it is able to do so, the following shall  ([305])  happen:

##### The ADS-air-user shall  ([306])  cease the operation of all contracts with ground systems which are effected by the error.

##### For each ground system that the aircraft had contracts with at the time of the unrecoverable system error, and which are effected by the error, the ADS-air-user shall  ([307])  invoke ADS-user-abort request.

##### The ADS-air-user shall  ([308])  inform the end user.

#### If the ADS-ground-user has an unrecoverable system error, and if it is able to do so, the following shall  ([309])  happen:

##### For each aircraft that the ADS-ground-user has contracts with and which are effected by the error, the ADS-air-user shall  ([310])  invoke ADS-user-abort request.

##### The ADS-ground-user shall  ([311])  cease operation of all contracts with aircraft which are effected by the error.

##### The ADS-ground-user shall  ([312])  inform the end user.

#### If the ADS-air-user receives an ADS-user-abort indication or an ADS-service-provider-abort indication, the following shall  ([313])  happen:

##### The ADS-air-user shall  ([314])  cease operation of all contracts with that ground system.

##### The ADS-air-user shall  ([315])  inform the end user.

#### If the ADS-ground-user receives an ADS-user-abort indication or if it receives an ADS-service-provider-abort indication, the following shall  ([316])  happen:

##### The ADS-ground-user shall  ([317])  cease operation of all contracts with the aircraft.

##### The ADS-ground-user shall  ([318])  inform the end user.

### Miscellaneous Air User Requirements

#### With the permissable exception of ADS-user-abort and ADS-provider-abort, the ADS-air-user shall  ([319])  respond to indications and confirmations from the ADS-air-ASE in the order in which they are invoked.

#### The ADS-air-user shall  ([320])  be capable of supporting contracts from at least four different ground systems at the same time.

#### If the ADS-air-user receives an ADS-demand-contract request, or an ADS-event-contract-request or an ADS-periodic-contract-request which exceeds its capacity for supporting ground systems, then it shall ([321])  :

- reject the contract with the reply parameter set to negative acknowlegement;

- set the reason element of negative acknowlegement to maximum-capacity-exceeded, and

- include the set of ICAO facility designators of all the ground systems with which it has contracts in the maximum-capacity-exceeded element.

#### The ADS-air-user shall  ([322])  accept a maximum of  one periodic contract, one event contract and one demand contract from each ADS-ground-user at any given time.

### Miscellaneous Ground User Requirements

#### The ADS-ground-user shall  ([323])  check the contents of the ADS reports received for conformance to the contracts in place, and shal.l flag any non-conformance.

#### The ADS-ground-user shall  ([324])  be able to retrieve the naming information required to establish a contract with an aircraft.

#### An ADS-ground-user shall  ([325])  initiate a maximum of one demand contract, one event contract and one periodic contract with each ADS-air-user at any given time.

## Establishment and operation of a Demand Contract

Note 1.— This section details the actions taken by the ADS-ground-user and the ADS-air-user in the establishment and operation of a demand contract.

### If the ADS-ground-user requires to establish a demand contract with the ADS-air-user it shall  ([326])  invoke ADS-demand-contract request.

### If, when the ADS-air-user receives an ADS-demand-contract indication, and it is not able to accept the contract, then the ADS-air-user shall  ([327])  invoke an ADS-demand-contract response with the reply parameter set to negative-acknowledgement, and the reason parameter set to the code for the reason that it cannot accept the contract.

### If, when the ADS-air-user receives ADS-demand-contract indication, and it is able to accept the contract in full, the following events shall  ([328])  occur:

#### The ADS-air-user shall  ([329])  invoke an ADS-report  request within 0.5 seconds

#### The ADS-report request shall  ([330])  include a positive acknowledgement parameter.

### If, when the ADS-air-user receives an ADS-demand-contract indication, and it is able to accept the contract, but is not able to supply all the requested information, the following events shall  ([331])  occur:

#### The ADS-air-user shall  ([332])  invoke ADS-demand-contract response, within 0.5 seconds, with the reply parameter set to noncompliance-notification, the demand-ncn parameter element containing an indication of the reports that were requested but cannot be provided.

#### The ADS-air-user shall  ([333])  treat the ADS-demand-contract as if those items indicated in demand-ncn (in the noncompliance-notification parameter) were not present in the ADS-demand-contract indication, and invoke ADS-report request containing the information that it is able to send within 0.5 seconds, with the positive acknowledgement parameter absent.

### When the ADS-air-user invokes ADS-report request in response to an ADS-demand-contract, then the report details parameter shall  ([334])  be formed as follows:

#### Position, time-stamp and FOM shall  ([335])  be present.

#### If flight-id is present in the contract details parameter of the demand-contract indication, then flight-id shall  ([336])  be returned in the ADS-report.

#### If projected-profile is present in the contract details parameter of the demand-contract indication, then projected-profile shall  ([337])  be returned in the ADS-report.

#### If ground-vector is present in the contract details parameter of the demand-contract indication, then ground-vector shall  ([338])  be returned in the ADS-report.

#### If air-vector is present in the contract details parameter of the demand-contract indication, then air-vector shall  ([339])  be returned in the ADS-report.

#### If weather is present in the contract details parameter of the demand-contract indication, then weather shall  ([340])  be returned in the ADS-report.

#### If airframe-id is present in the contract details parameter of the demand-contract indication, then airframe-id shall  ([341])  be returned in the ADS-report.

#### If navigational-intent is present in the contract details parameter of the demand-contract indication, then short-term-intent shall  ([342])  be returned in the ADS-report, and shal.l cover the time period indicated in navigational-intent.

#### If extended-projected-profile is present in the contract details parameter of the demand-contract indication, then extended-projected-profile shall  ([343])  be returned in the ADS-report covering the time period indicated in time-interval or the number of way points indicated in number-of-way-points of extended-project-profile.

### When the ADS-air-user invokes ADS-report request in response to an ADS-demand-contract indication, the contract type parameter shall  ([344])  be set to demand-contract.

### When the ADS-air-user invokes ADS-report request in response to an ADS-demand-contract indication, the event type parameter shall  ([345])  be absent.

## Establishment and operation of an Event Contract

Note 1.— This section details the actions taken by the ADS-ground-user and the ADS-air-user in the establishment and operation of a event contract.

### If the ADS-ground-user requires to establish an event contract with the ADS-air-user, it shall  ([346])  invoke ADS-event-contract request.

### When invoking the ADS event contract request, the ADS ground user shall  ([347])  specify at least one event type.

### If, when the ADS-air-user receives an ADS-event-contract indication, it is not able to accept the contract, then the ADS-air-user shall  ([348])  invoke an ADS-event-contract response with the reply parameter set to negative-acknowledgement and reason set to the code for the reason that it cannot accept the contract, within 0.5 seconds.

Note 1.— In the event of the new contract not being accepted, any existing contract will remain in place

### If, when the ADS-air-user receives an ADS-event-contract indication, it is able to accept the contract in full the following actions shall  ([349])  occur:

#### If the terms of the contract require an ADS-report to be sent immediately, and the ADS-air-user is able to invoke an ADS-report request within 0.5 seconds then it shall  ([350])  do so, and this ADS-report request shal.l include a positive acknowledgement parameter.

#### If the terms of the contract do not require an ADS-report to be sent immediately, or the ADS-air-user is not able to invoke an ADS-report request within 0.5 seconds then it shall  ([351])  invoke ADS-event-contract response with reply set to positive acknowledgement.

### If, when the ADS-air-user receives ADS-event-contract indication, it is able to accept the contract, but is not able to detect some of the events in the contract, then the following actions shall  ([352])  happen:

#### The ADS-air-user shall  ([353])  invoke ADS-event-contract response with the reply parameter set to noncompliance-notification, and the event-ncn element set to the events that cannot be complied with, within 0.5 seconds.

#### The ADS-air-user shall ([354])  , for the duration of the contract, otherwise behave as if the event type elements listed in event-ncn are not in the contract.

### If the ADS-air-user accepts the event contract with a noncompliance-notification, or with a positive-acknowledgement (either in an ADS-event-contract response or an ADS-report request) then the following realisation of the event contract shall  ([355])  occur:

#### Any other event contract in force at the time shall  ([356])  be implicitly cancelled.

#### On receipt of the ADS-event-contract indication, if one or more of the event types listed under Event type in Table 7-1 were in the ADS-event-contract indication, the ADS-air-user shall  ([357])  invoke a single ADS-report request, which contains the Optional Parameter Elements, which correspond to the Event type in ADS-event-contract indication in Table 7-1.

Note 1.— This provides a baseline reference against which possible deviations are compared.

| Event type | Optional parameter elements |
| --- | --- |
| air-speed-change | air-vector |
| ground-speed-change | ground-vector |
| heading-change | air-vector |
| track-angle-change | ground-vector |
| altitude-change | none |

Table 7-1:  Events giving rise to an immediate event report

Note 1.— Any event occasioned report includes a basic position report.

#### If lateral-deviation-change is provided in the ADS-event-contract the contract details parameter, then the following shall  ([358])  apply:

##### If the lateral deviation of the aircraft relative to the active route of flight is more than the value of lateral-deviation-change then the ADS-air-user shall  ([359])  invoke ADS-report request.

##### The ADS-air-user shall  ([360])  continue to invoke ADS-report request at a rate of once every 60 seconds while the lateral deviation of the aircraft is more than the value of lateral-deviation-change.

##### The ADS-air-user shall  ([361])  cease invoking ADS-report request when the lateral deviation of the aircraft is less than or equal to than the value of lateral-deviation-change.

##### The ADS-report request the report details parameter shall  ([362])  contain the ground-vector element.

#### If vertical-rate-change is provided in the ADS-event-contract the contract details parameter and it is zero or positive, then the following shall  ([363])  apply:

##### If the aircraft’s rate of climb is more than the value of vertical-rate-change, then the ADS-air-user shall  ([364])  invoke ADS-report request.

##### The ADS-air-user shall  ([365])  continue to invoke ADS-report request at a rate of once every 60 seconds while the aircraft’s rate of climb is more than the value of vertical-rate-change.

##### The ADS-air-user shall  ([366])  cease invoking ADS-report request when the aircraft’s rate of climb is less than or equal to the value of vertical-rate-change.

##### The ADS-report request the report details parameter shall  ([367])  contain the ground-vector element.

#### If vertical-rate-change is provided in the ADS-event-contract the contract details parameter and it is negative, then the following shall  ([368])  apply:

##### If the aircraft’s rate of climb is less than the value of vertical-rate-change, then the ADS-air-user shall  ([369])  invoke ADS-report request.

##### The ADS-air-user shall  ([370])  continue to invoke ADS-report request at a rate of once every 60 seconds while the aircraft’s rate of descent is less than the value of vertical-rate-change.

##### The ADS-air-user shall  ([371])  cease invoking ADS-report request when the aircraft’s rate of climb is greater than or equal to the value of vertical-rate-change.

##### The ADS-report request report details parameter shall  ([372])  contain the ground-vector element.

#### If altitude-threshold is provided in the ADS-event-contract the contract details parameter, then the following shall  ([373])  apply:

##### If the altitude of the aircraft is more than the value of ceiling, or less than the value of floor, then the ADS-air-user shall  ([374])  invoke ADS-report request.

##### The ADS-air-user shall  ([375])  continue to invoke ADS-report request at a rate of once every 60 seconds while the altitude of the aircraft is more than the value of ceiling, or less than the value of floor.

##### The ADS-air-user shall  ([376])  cease invoking ADS-report request when the altitude of the aircraft is less than or equal to the value of ceiling, and greater than or equal to the value of floor

##### The ADS-report request report details parameter shall  ([377])  contain the ground-vector element.

#### If way-point-change is provided in the ADS-event-contract the contract details parameter, then the following shall  ([378])  apply:

##### If the aircraft’s next way-point changes, then the ADS-air-user shall  ([379])  invoke ADS-report request.

##### The ADS-report request report details parameter shall  ([380])  contain the projected-profile element.

#### If fom-change is provided in the ADS-event-contract the contract details parameter, then the following shall  ([381])  apply:

##### If the aircraft’s navigational accuracy, navigational system redundancy or traffic alert and collision avoidance system (TCAS) availability changes, then the ADS-air-user shall  ([382])  invoke ADS-report request.

#### If extended-projected-profile-change is provided in the ADS-event-contract the contract details parameter, and contains the time-interval element, then the following shall  ([383])  apply:

##### If one or more way-points on the active route of flight within the time-interval as measured from the current time changes, then the ADS-air-user shall  ([384])  invoke ADS-report request.

##### The ADS-report request report details parameter shall  ([385])  contain the extended-projected-profile element containing way-points covering the time-interval from the current time.

#### If extended-projected-profile-change is provided in the ADS-event-contract the contract details parameter, and contains the number-of-way-points element, then the following shall  ([386])  apply:

##### If one or more way-points on the active route of flight that are in the next number-of-way-points changes, then the ADS-air-user shall  ([387])  invoke ADS-report request.

##### The ADS-report request report details parameter shall  ([388])  contain the extended-projected-profile element containing way-points covering the number-of-way-points.

#### If air-speed-change is provided in the ADS-event-contract the contract details parameter, then the following shall  ([389])  apply:

##### The aircraft’s airspeed shall  ([390])  continually be compared with the airspeed transmitted in the most recent ADS-report request that contained an air-vector element.

##### If the aircraft’s current airspeed differs negatively or positively from this transmitted value by an amount that is greater than or equal to the value of air-speed-change, then the ADS-air-user shall  ([391])  invoke ADS-report request.

##### The ADS-report request report details parameter shall  ([392])  contain the air-vector element.

#### If ground-speed-change is provided in the ADS-event-contract the contract details parameter, then the following shall  ([393])  apply:

##### The aircraft’s ground speed shall  ([394])  continually be compared with the ground speed transmitted in the most recent ADS-report request or ADS-emergency-report request that contained a ground-vector element.

##### If the aircraft’s current  ground speed differs negatively or positively from this transmitted value, by an amount that is greater than or equal to the value of ground-speed-change, then the ADS-air-user shall  ([395])  invoke ADS-report request.

##### The ADS-report request report details parameter shall  ([396])  contain the ground-vector element.

#### If heading-change is provided in the ADS-event-contract the contract details parameter, then the following shall  ([397])  apply:

##### The aircraft’s heading shall  ([398])  continually be compared with the heading transmitted in the most recent ADS-report request that contained an air-vector element.

##### If the aircraft’s current heading differs negatively or positively from this value by an amount that is greater than or equal to the value of heading-change, then the ADS-air-user shall  ([399])  invoke ADS-report request.

##### The ADS-report request report details parameter shall  ([400])  contain the air-vector element.

#### If track-angle-change is provided in the ADS-event-contract the contract details parameter, then the following shall  ([401])  apply:

##### The aircraft’s track angle shall  ([402])  continually be compared with the track angle transmitted in the most recent ADS-report request or ADS-emergency-report request that contained a ground-vector element.

##### If the aircraft’s current track angle differs negatively or positively from this value by an amount that is greater than or equal to the value of track-angle-change, then the ADS-air-user shall  ([403])  invoke ADS-report request.

##### The ADS-report request report details parameter shall  ([404])  contain the ground-vector element.

#### If altitude-change is provided in the ADS-event-contract the contract details parameter, then the following shall  ([405])  apply:

##### The aircraft’s altitude shall  ([406])  continually be compared with the altitude transmitted in the most recent ADS-report request or ADS-emergency-report request.

##### If the aircraft’s altitude differs negatively or positively from this value by an amount that is greater than or equal to the value of altitude-change, then the ADS-air-user shall  ([407])  invoke ADS-report request.

##### The ADS-report request report details parameter shall  ([408])  contain the ground-vector element.

#### If more than one of the events described above occurs at the same time, the ADS-air-user shall  ([409])  invoke separate ADS-report requests as described above, for each event independently. (ie the same report cannot be used to report on more than one event even if the same information is being transmitted.)

### Apart from circumstances detailed in section 1.3.4.1, the positive acknowledgement parameter shall  ([410])  not be present in any ADS-report request made in response to an ADS-event-contract indication.

### When the ADS-air-user invokes ADS-report request in response to an ADS-event-contract indication, the contract type parameter shall  ([411])  be set to event-contract.

### When the ADS-air-user invokes ADS-report request in response to an ADS-event-contract indication, the event type parameter shall  ([412])  be set indicate the type of event in the contract that this report is in response to.

## Establishment and operation of a Periodic Contract

Note 1.— This section details the actions taken by the ADS-ground-user and the ADS-air-user in the establishment and operation of a periodic contract while not in emergency mode.

### If the ADS-ground-user requires to establish a periodic contract with the ADS-air-user it shall  ([413])  invoke ADS-periodic-contract request.

### If, when the ADS-air-user receives an ADS-periodic-contract indication, it is not able to accept the contract, then the ADS-air-user shall  ([414])  invoke an ADS-periodic-contract response with the reply parameter set to negative-acknowledgement and reason set to the code for the reason that it cannot accept the contract.

Note 1.— In the event of the new contract not being accepted, any existing contract will remain in place

### If, when the ADS-air-user receives an ADS-periodic-contract indication, it is able to accept the contract in full, then the following shall  ([415])  occur:

#### If the ADS-air-user is able to send the first ADS-report request of the contract within 0.5 seconds, then the ADS-air-user shall  ([416])  invoke the first ADS-report request of the contract within 0.5 seconds, and this ADS-report request shal.l include a positive acknowledgement parameter.

#### If the ADS-air-user is not able to send the first ADS-report request of the contract within 0.5 seconds, either due to the information being temporaly unavailable, or due to a suspension of the periodic contract because of an emergency, then the following shall  ([417])  occur:

##### The ADS-air-user shall  ([418])  invoke ADS-periodic-contract response with the reply parameter set to positive acknowledgement within 0.5 seconds.

##### If the aircraft is not in emergency mode, the ADS-air-user shall  ([419])  send the first ADS-report request of the contract within a further 29.5 seconds (ie 30 seconds from the receipt of the ADS-periodic-contract request).

### If, when the ADS-air-user receives an ADS-periodic-contract indication, it is able to supply some of the information required in the contract, but is not able generate all the report elements, or it is not able to meet the requested reporting rate, or both, then the following shall  ([420])  occur:

#### The ADS-air-user shall  ([421])  invoke ADS-periodic-contract response with the reply parameter set to noncompliance-notification, and with periodic-ncn set to indicate the reports that cannot be generated and/or that the reporting rate cannot be met, within 0.5 seconds.

#### If the ADS-air-user indicated that it cannot generate all the reports, then it shall ([422])  , for the duration of the contract, otherwise behave as if the report elements listed in periodic-ncn are not in the contract.

#### If the ADS-air-user indicated that it cannot meet the reporting rate, then it shall ([423])  , for the duration of the contract, otherwise behave as if the reporting rate in the contract is 60 seconds.

### If the ADS-air-user accepts the periodic contract with a noncompliance-notification, or with a positive-acknowledgement (either in an ADS-periodic-contract response or an ADS-report request) then the following realisation of the periodic contract shall  ([424])  occur:

#### Any other periodic contract in force with the ground system at the time shall  ([425])  be implicitly cancelled.

#### The ADS-air-user shall  ([426])  invoke ADS-report requests at the rate indicated by reporting-interval  from the contract details parameter, as modified by the noncompliance notification, if applicable, until such time as the contract is cancelled, or suspended due to an emergency.

Note 1.— If emergency mode is already in place, the periodic contract will be immediately suspended due to the provisions stated in the later section on emergency mode.

#### Apart from circumstances detailed in section 1.4.3.1, the positive acknowledgement parameter shall  ([427])  not be present in the ADS-report request.

#### The ADS-report report details elements shall  ([428])  have the following elements, as modified by the noncompliance notification, if applicable:

##### Position, time-stamp and fom shall  ([429])  be present.

##### If flight-id-modulus is present in the contract details parameter of the ADS-periodic-contract indication with a value of N, then the flight-id element shall  ([430])  be returned in the first ADS-report and every Nth following ADS-report.

##### If projected-profile-modulus present in the contract details parameter of the ADS-periodic-contract indication with a value of N, then the projected-profile element shall  ([431])  be returned in the first ADS-report and every Nth following ADS-report.

##### If ground-vector-modulus is present in the contract details parameter of the ADS-periodic-contract indication with a value of N, then the ground-vector element shall  ([432])  be returned in the first ADS-report and every Nth following ADS-report.

##### If air-vector-modulus is present in the contract details parameter of the ADS-periodic-contract indication with a value of N, then the air-vector element shall  ([433])  be returned in the first ADS-report and every Nth following ADS-report.

##### If weather-modulus is present in the contract details parameter of the ADS-periodic-contract indication with a value of N, then the weather element shall  ([434])  be returned in the first ADS-report and every Nth following ADS-report.

##### If airframe-id-modulus is present in the contract details parameter of the ADS-periodic-contract indication with a value of N, then the airframe-id element shall  ([435])  be returned in the first ADS-report and every Nth following ADS-report.

##### If navigational-intent is present in the contract details parameter of the ADS-periodic-contract indication with the intent-modulus having a value of N, then the short-term-intent element shall  ([436])  be returned in the first ADS-report and every Nth following ADS-report, covering way points and estimated times of arrival for the following intent-projection-time as measured from the time of arrival of the ADS-periodic-contract indication.

##### If extended-projected-profile-modulus is present in the contract details parameter of the ADS-periodic-contract indication with a value of N, then the extended-projected-profile element shall  ([437])  be returned in the first ADS-report and every Nth following ADS-report, and contain extended projected profile information for the time-interval or the number-of-way-points indicated in the extended-projected-profile element or the extended-projected-profile-modulus.

### When the ADS-air-user invokes ADS-report request in response to an ADS-periodic-contract indication, the contract type parameter shall  ([438])  be set to periodic-contract.

### When the ADS-air-user invokes ADS-report request in response to an ADS-periodic-contract indication, the event type parameter shall  ([439])  not be included in the ADS-report request.

## Cancellation of Contracts

Note 1.— This section details the actions taken by the ADS-ground-user and the ADS-air-user in the cancellation of contracts.

### If an ADS-ground-user requires to cancel an event contract or a periodic contract, then it shall  ([440])  either invoke ADS-cancel request with the contract type parameter set to event-contract or periodic-contract respectively.

### If the ADS-air-user receives an ADS-cancel-contract with contract type parameter set to event-contract or periodic-contract, the ADS-air-user shall  ([441])  cancel any event contract or periodic contract respectively with that ground system that is in force at the time.

### If an ADS-ground-user requires to cancel all contracts with the aircraft it should invoke ADS-cancel-all-contracts request.

### When the ADS-air-user receives an ADS-cancel-all-contracts indication, it shall  ([442])  cancel any contract in force with that ground system.

Note 1.— There is no other provision for cancellation of demand contracts.

## Establishment and Operation of Emergency Mode

Note 1.— This section details the actions taken by the ADS-ground-user and the ADS-air-user in the establishment and operation of emergency mode.

Note 2.— The emergency mode may be initiated either by human or aircraft system intervention.

### On entering emergency mode, the ADS-air-user shall  ([443])  establish an emergency contract with every ground system with which it has an event contract or a periodic contract (or both).

### If, while in emergency mode, an ADS-periodic-contract indication or ADS-event-contract indication occurs for a ground system with which the aircraft has not got an event contract or periodic contract, the following shall  ([444])  happen:

#### The ADS-air-user shall  ([445])  acknowledge the contract in the manner indicated in the earlier sections on event and periodic contracts, with either a response or an ADS-report.

#### If the contract is still in force after the acknowledgement (ie a negative acknowledgement was not sent), the ADS-air-user shall  ([446])  establish an emergency contract with the ground system.

### When the ADS-air-user establishes an emergency contract with an ADS-ground-user, the following shall  ([447])  happen:

#### If a periodic contract is in force, the ADS-air-user shall  ([448])  suspend the operation of the contract.

#### If no periodic contract is in force at the time of establishing the emergency contract, the ADS-air-user shall  ([449])  invoke ADS-emergency-report requests at intervals of 60 seconds.

#### If a periodic contract is in force at the time of establishing the emergency contract, with a reporting period of 1 second, the ADS-air-user shall  ([450])  invoke ADS-emergency-report requests at intervals of 1 second.

#### If a periodic contract is in force at the time of establishing the emergency contract, with a reporting period greater than two minutes, the ADS-air-user shall  ([451])  invoke ADS-emergency-report requests at intervals of 60 seconds.

#### If a periodic contract is in force at the time of establishing the emergency contract, with a reporting period less than or equal to two minutes and greater than 1 second, the ADS-air-user shall  ([452])  invoke ADS-emergency-report requests at intervals of half the reporting rate of the periodic contract rounded down to the nearest second.

#### If the ADS-air-user receives an ADS-periodic-contract indication while the emergency contract is in operation, then the following shall  ([453])  apply:

##### The ADS-air-user shall  ([454])  invoke ADS-periodic-contract response giving a reply that would be appropriate if the emergency contract were not in operation.

Note1. — This implies that a reply of an ADS-report containing a positive acknowledgement is not possible.

#### The ADS-air-user shall  ([455])  store details of the most recent of the following:

- ADS-periodic-contract indication, which had a positive acknowledgement as a response;

- ADS-periodic-contract indication, which had a noncompliance notification as a response;

- ADS-cancel-contract indication with value periodic-contract.

#### Apart from conditions specified in the section on modifying an emergency contract, the positive acknowledgement parameter shall  ([456])  not be present in any ADS-emergency-report request.

#### The emergency report details parameter in the ADS-emergency-report request shall  ([457])  be formed as follows:

##### location, timestamp and fom shall  ([458])  always be present.

##### ground-vector and flight-id shall  ([459])  be present the first time ADS-emergency-report request is invoked after the emergency contract has been established, and subsequently every fifth time (ie on the 1st, 6th, 11th, 16th etc. ADS-emergency-report request).

### When emergency mode is initiated, the ADS-air-user shall  ([460])  invoke the first ADS-emergency-report request within 0.5 seconds.

## Modifying an Emergency Contract

Note 1.— This section details the actions taken by the ADS-ground-user and the ADS-air-user when modifying an emergency contract.

### When the ADS-ground-user requires to modify the reporting rate of an emergency contract it shall  ([461])  invoke ADS-modify-emergency-contract request.

### If, when the ADS-air-user receives an ADS-modify-emergency-contract indication, and it is able comply with the request, the following shall  ([462])  happen:

#### The ADS-air-user shall  ([463])  change the rate of invoking ADS-emergency-report requests to the time indicated in the reporting interval parameter.

#### The next ADS-emergency-report request shall  ([464])  behave as follows:

##### It shall  ([465])  contain a positive acknowledgement parameter.

##### It shall  ([466])  be invoked within 0.5 second.

### If, when the ADS-air-user receives an ADS-modify-emergency-contract indication, it is not able comply with the request, then the following shall  ([467])  happen:

#### The ADS-air-user shall  ([468])  invoke ADS-modify-emergency-contract response.

#### The ADS-air-user shall  ([469])  continue to invoke ADS-emergency-report requests at the same rate that it had already been doing.

#### The ADS-emergency-report request generated after the ADS-modify-emergency-contract response shall  ([470])  be generated as soon as possible and in any case within 0.5 second.

### The ADS-air-user shall  ([471])  retain its position in the five ADS-emergency-report cycle (as described in the section on Establishment and Operation of Emergency mode) on acceptance or rejection of the ADS-modify-emergency-contract indication.

## Cancellation of Emergency Mode

Note 1.— This section details the actions taken by the ADS-ground-user and the ADS-air-user when the ADS-air-user requires to cancel emergency contracts.

Note 2.— The emergency mode may only be cancelled by human intervention in the aircraft.

Note 3.— If the ground user requires to cancel the emergency mode indication, this shall  ([472])  be handled locally.

### When the emergency mode is to be cancelled, the ADS-air-user shall  ([473])  cancel the emergency mode by cancelling the emergency contract with each ground system with which it has an emergency contract.

### When the ADS-air-user requires to cancel an emergency contract with an ADS-ground-user the following shall  ([474])  occur:

#### The ADS-air-user shall  ([475])  cancel operation of the emergency contract.

#### The ADS-air-user shall  ([476])  invoke ADS-cancel-emergency request.

### When the ADS-air-user cancels the emergency contract, the following shall  ([477])  apply:

#### If a periodic contract was in operation before the emergency contract was established and no ADS-periodic-contract requests or ADS-cancel-contract requests (with a value of periodic contract) were received during the emergency contract, then the periodic contract shall  ([478])  be reinstated.

#### If the latest event to be stored (as indicated in section 1.6.3.7) was an ADS-periodic-contract request, then it shall  ([479])  take effect immediately.

#### If the latest event to be stored (as indicated in section 1.6.3.7) was an ADS-cancel-contract request, then no periodic contract shall  ([480])  be reinstated.

#### If the ADS-air-user reinstates a periodic contract, it shall  ([481])  invoke the next ADS-report request within 0.5 second.

#### If the ADS-air-user reinstates a periodic contract, it shall  ([482])  restart in the same position in the cycle of reports as it was in when it left.

## Establishment and operation of a Forward Contract

<editor’s note: TBC>.