# GENERAL REQUIREMENTS

## Performance Requirements

Note 1.— Systems developed to support ADS functionality will be capable of meeting the Required Communications Performance (RCP) appropriate for the phase of operation as specified in [5]..

## Time Accuracy Requirements

### Absolute times which are sent as parameters over the data link shall  ([1])  be as accurate as the required resolution of the time parameter.

### Absolute times shall  ([2])  be UTC.

### Dates shall  ([3])  be expressed as UTC date (ie, the day increments at 2359:59z).

### Relatives times shall  ([4])  be accurate to ñ 0.1 second.

### Where timestamps are used they shall  ([5])  consist of year, month, day and hour, minute, second.

### Security Requirements

Note 1.— There are no internationally approved operational requirements relating to data link application security.

## Backwards Compatibility Requirements

Note 1.— This specification describes the version 1 of the CNS/ATM-1 Package ADS application. Best efforts have to be made to ensure that subsequent versions of this protocol will be backwards compatible.

### For CNS/ATM-1 the ADS-air-ASE and the ADS-ground-ASE version numbers shall  ([6])  both be set to one.

## Error Processing Requirements

### In the event of information input by the user being incompatible with that able to be processed by the system, the user shall  ([7])  be notified.

### In the event of a user invoking an ADS service primitive, when it is not in a state specified in chapter 5, the invocation shall  ([8])  be rejected and the user shal.l be notified.