|  | ATNP/WG3/SG2 (Air/Ground Application) Version 2.0 15 February 1996 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Sub Group 2 (Air/Ground Applications)

Part 1: Context Management Application SARPs

Prepared by:		Air/Ground Applications Subgroup

SUMMARY

This document is the draft of the Context Management application SARPs for the ATN CNS/ATM-1 Package.

CONFIGURATION SHEET

Title :			 Draft Context Management SARPs for the ATN CNS/ATM-1 Package

Version :		2.0

Date:			15 February 1996

Contact:		Airservices Australia, Eurocontrol, Federal Aviation Administration, Direction Generale 			de l’Aviation Civile

Status:			Draft

Change History :

| Version | Description | Affected Parts | Date |
| --- | --- | --- | --- |
| 1.0 | Banff Proposed Draft Context Management SARPs |  | 6 Oct 1995 |
| 1.1 | Input to ATN/WG3 at Brisbane |  | 5 Feb 1996 |
|  | Chapter 2 |  |  |
|  | - Time accuracy clarified | §2.2.1 |  |
|  | Chapter 3 |  |  |
|  | - CM-end service added due to dialogue changes | §3.3.1 |  |
|  | - Maintain Dialogue parameter added to CM-logon service | §3.5.8 |  |
|  | - CM-update service changes resulting from dialogue changes | §3.6 |  |
|  | - CM-contact service changes resulting from dialogue changes | §3.7 |  |
|  | - CM-end service added | §3.8 |  |
|  | Chapter 4 |  |  |
|  | - The cmUpdate element was added for clarification purposes | §5.3.2.4.1.1, §5.2.3.5.1.1, §5.3.3.3.1 |  |
|  | - Abort reasons added due to dialogue changes | §5.3.2.7, §5.3.2.8, §5.3.2.9, §5.3.3.10, §5.3.3.11, §5.3.3.12, all of §5.4 |  |
|  | - Application name and version number clarified |  |  |
|  | - Long TSAP clarifications, specifically adding ARS |  |  |
|  | Chapter 5 |  |  |
|  | - CM-update time sequence diagrams added | §5.1.4, figures 5-3, 5-4 |  |
|  | - Textual changes and new time sequence diagrams for dialogue changes | §5.1.5, figures 5-5, 5-6, 5-7, 5-8, 5-9, 5-10 §5.3.2.2.2, §5.3.2.3.1.1, §5.3.2.5, §5.3.2.5.1.2, §5.3.2.6, §5.3.3.3.2 §5.3.3.4.2, §5.3.3.6, §5.3.3.9 |  |
|  | - Addition of CM-end service text and time sequence diagrams | §5.1.6, figure 5-11, §5.3.3.8 |  |
|  | - Addition of CM-end timer | §5.2, table 5-1 |  |
|  | - Due to modified D-ABORT (new name, new variables) all of - section 5.4 was deleted and rewritten. | §5.4 |  |
|  | Chapter 6 |  |  |
|  | - Addition of D-DATA, D-END | §6.2.1.1 |  |
|  | - Addition of note for ATSC value references | §6.2.2.3.1 |  |
|  | Chapter 7 |  |  |
|  | - CM-logon request information clarified | §7.1.3.1 |  |
|  | - Changes resulting from dialogue updates added | §7.2.2.4, , §7.2.2.8, §7.2.4.2, §7.2.5, §7.2.6 |  |
|  | - Changes resulting from address/version number updates added | §7.2.2.3, §7.2.3.2 |  |
|  | - CM-end service reqts added | §7.2.7 |  |
|  | Appendix A |  |  |
|  | - State tables updated for dialogue changes | §A.2, tables A-1 and A-2 |  |
| 2.0 | Output from Brisbane |  | 15 Feb 1996 |
|  | Chapter 1 |  |  |
|  | - Addition of acronyms and definitions relating to long/short/actual TSAP addresses | §1.1.4.1, §1.1.4.2 |  |
|  | - Changed reference of traffic type values to “[5]” | §1.1.5.1, §3.5.7.1, §3.6.5.1, §3.7.6.1, §3.9.7.1, §6.2.2.3.1 |  |
|  | - Addition of Forward Function (FD4) | §1.2.2.1 |  |
|  | - Addition of Forward Functionality | §1.3.4 |  |
|  | Chapter 2 |  |  |
|  | - Timing requirements clarified | §2.2 |  |
|  | Chapter 3 |  |  |
|  | - Addition of CM-forward service | §3.3.1 |  |
|  | - Insertion of new section (prior to CM-abort services) that provides the description of the CM-forward service | §3.9 |  |
|  | Chapter 4 |  |  |
|  | - Automatic tagging/removing of tags in ASN.1 |  |  |
|  | - Alphabetising ASN.1 |  |  |
|  | - Renaming CMDownlinkMessage to CMAircraftMessage, CMUplinkMessage to CMGroundMessage in order to allow for ground-ground message transfer | §5.3.2.5.1.1, §5.3.2.6.1, §5.3.2.6.2, §5.4.1.1, §5.4.2.1, §5.4.3.1, §5.4.3.2, §5.4.4.1, §5.4.4.2, §5.4.5.2, §5.3.3.6.1, §5.3.3.7.1, §5.3.3.7.2, §5.3.3.8.1, §5.3.3.8.2, §5.3.3.10.1 |  |
|  | - Addition of appropriate ASN.1 for the CM-forward service | §5.4.5.1, §5.4.6.1 §3.9.6.1, §5.3.3.1.2 §5.3.3.10.1 |  |
|  | - Full definition of TSAP addresses, both long and short | §7.1.3.1, §7.1.3.2, §7.2.2.2, §7.2.2.5, §7.2.6.4 |  |
|  | - Definition of logon unsuccessful response | §7.2.2.7 |  |
|  | - Passing all version numbers/ addresses for each application | §7.1.3.1, §7.1.3.3, §7.2.2.4, §7.2.3.2 |  |
|  | - ICAO facility designator modified from 4 characters to 8 | §3.5.2.1, §3.6.3.1, §3.7.3.1, §3.9.2.1, §3.9.4 |  |
|  | Chapter 5 |  |  |
|  | - Addition of CM-forward service | §5.1.7 |  |
|  | - Addition of abort time sequence diagrams for ground-ground case | §5.1.10, §5.1.11, figures 5-18, 5-19, 5-20, 5-21, 5-22 |  |
|  | - Addition of CM-forward timer | §5.2, table 5-1 |  |
|  | - General re-ordering of section 5.3, so the dialogue service primitives are defined first followed by the CM service primitives (for both air and ground protocol descriptions) | §5.3 |  |
|  | - Addition to D-START cnf and D-START ind requirements for inclusion of CM-forward | §5.3.3.2.1.3, §5.3.3.1.2 |  |
|  | - Addition of CM-forward Request | §5.3.3.10 |  |
|  | - State tables moved from Appendix A to section 5.5 | §5.5 |  |
|  | - Addition of FORWARD state to table 5-2 along with additional forward events and forward timer | §5.5, table 5-2 |  |
|  | Chapter 6 |  |  |
|  | - NULL removal from QOS table | §6.2.2.3.1, table 6-1 |  |
|  | - Addition of RER clarification (Note 2) | §6.2.2.3.1 |  |
|  | Chapter 7 |  |  |
|  | - Long/Short TSAP clarifications for Air and Ground user requirements | §7.1.3.1, §7.1.3.2, §7.2.2.2, §7.2.2.5, §7.2.6.4 |  |
|  | - Addition of CM-forward service requirements | §7.2.6 |  |
|  | - Passing all version numbers/ addresses for each application (CM-logon request CM-logon response and CM-forward) | §7.1.3.1, §7.2.2.4, §7.2.6.2 |  |
|  | Appendix A |  |  |
|  | - State table moved to Chapter 5; appendix deleted | §A |  |

Table Of Contents

1. APPLICATION OVERVIEW

1.1 Introduction

1.2 Application Functionality

1.3 Functional Descriptions

2. GENERAL REQUIREMENTS

2.1 Performance Requirements

2.2 Time Accuracy Requirements

2.3 Security Requirements

2.4 Backwards Compatibility Requirements

2.5 Error Processing Requirements

3. THE ABSTRACT SERVICE

3.1 Introduction

3.2 The CM Functional Model

3.3 The CM-ASE Abstract Service

3.4 Conventions

3.5 CM-logon Service

3.6 CM-update Service

3.7 CM-contact Service

3.8 CM-end Service

3.9 CM-forward Service

3.10 CM-user-abort Service

3.11 CM-provider-abort Service

4. FORMAL DEFINITIONS OF MESSAGES

4.1 ASN.1

5. PROTOCOL DEFINITION

5.1 Sequence Rules

5.2 CM Application Timers

5.3 CM-ASE Protocol Description

5.4 Exception Handling

5.5 CM ASE StateTables

6. COMMUNICATION REQUIREMENTS

6.1 Encoding Rules

6.2 Dialogue Service Requirements

6.3 CM AE Control Functions Requirements

7. CM USER REQUIREMENTS

7.1 CM-Air-User Requirements

7.2 CM-Ground-User Requirements

# APPLICATION OVERVIEW

## Introduction

### Purpose

#### This document contains draft Standards and Recommended Practices (SARPs) for the Context Management (CM) application.  The CM process supports addressing requirements for Air Traffic Services Communication (ATSC) such as Air Traffic Control (ATC) Controller Pilot Data Link Communications (CPDLC), Automatic Dependent Surveillance (ADS), and Flight Information Services (FIS).  The Aeronautical Telecommunication Network (ATN) provides the media and protocols to conduct data link communications for the CM process.

#### Compliance with these standards is a means of assuring that the ATC system will perform its intended functions using data link, and that the CM application is implemented in a globally uniform and interoperable manner.

### Background

#### The CM application provides the capability to establish a logon between  ATC ground and aircraft systems via the ATN.  Once an appropriate connection is established, CM provides data link application information, the capability to log-on to another ground system, and the capability to update log-on information.

#### In the performance of this role, the CM application interacts with the following:

Aeronautical Telecommunication Network (ATN),

Other available data link applications (e.g., ADS, CPDLC, and FIS),

The ground ATC system, and

The aircraft avionics system.

### Structure of Document

#### Chapter 1:  APPLICATION OVERVIEW contains the document’s purpose and structure, a summary of the ADSP operational requirements (ORs) that relate to CM and maps these ORs to the functions of CM.

#### Chapter 2:  GENERAL REQUIREMENTS contains performance, time accuracy, security, backwards compatibility, and error processing requirements.

#### Chapter 3:  ABSTRACT SERVICE DEFINITION contains the description of the abstract service provided by the CM Application Service Element (CM-ASE).

#### Chapter 4:  FORMAL DEFINITION OF MESSAGES contains the formal definition of messages exchanged by CM-ASEs using Abstract Syntax Notation Number One (ASN.1).

#### Chapter 5: PROTOCOL DEFINITION describes the exchanges of messages allowed by the CM protocol, as well as time constraints and CM-ASE protocol descriptions.

#### Chapter 6:  COMMUNICATION REQUIREMENTS  contains the requirements that the CM application imposes on the underlying communication system.

#### Chapter 7 : CM USER REQUIREMENTS contains requirements imposed on the user of the CM-ASE service.

### Explanation of Terms

#### Acronyms

##### The following abbreviations are used in this specification:

| ADS | Automatic Dependent Surveillance |
| --- | --- |
| ADSP | Automatic Dependent Surveillance Panel |
| AE | Application Entity |
| APDU | Application Protocol Data Unit |
| ARS | Administrative Region Selector |
| ASE | Application Service Element |
| ASN.1 | Abstract Syntax Notation One |
| ATC | Air Traffic Control |
| ATM | Air Traffic Management |
| ATN | Aeronautical Telecommunication Network |
| ATS | Air Traffic Service |
| ATSC | Air Traffic Services Communication |
| CF | Control Function |
| CM | Context Management |
| CNS | Communication Navigation Surveillance |
| CNS/ATM | Communications Navigation Surveillance / Air Traffic Management |
| CPC | Controller Pilot Communications |
| CPDLC | Controller Pilot Data Link Communications |
| DS | Dialogue Service |
| FD | Functional Description |
| FDPS | Flight Data Processing System |
| FIS | Flight Information Services |
| IA5 | International Alphabet #5 |
| ICAO | International Civil Aviation Organization |
| ID | Identification |
| IDP | Initial Domain Part |
| IEC | International Electrotechnical Committee |
| IS | International Standard |
| ISO | International Organization for Standardization |
| ITU | International Telecommunications Union |
| LOC | Location Identifier |
| OR | Operational Requirement |
| PDU | Protocol Data Unit |
| PER | Packed Encoding Rules |
| QOS | Quality of Service |
| RCP | Required Communication Performance |
| RDP | Router Domain Part |
| RER | Residual Error Rate |
| SARPs | Standards and Recommended Practices |
| SEL | (Transport) Selector |
| SYS | System Identifier |
| TSAP | Transport Service Access Point |
| UTC | Coordinated Universal Time |

#### General Definitions

##### For the purpose of this document the following definitions apply:

- Active User:  a user which is currently involved in a CM dialogue.

- Actual TSAP:  the actual TSAP is composed of the IDP and the long TSAP.

- Application Entity:  a model of  those aspects of an application process that are significant from the viewpoint of accessing OSI capabilities.

- Application Process:  an element within an open system which performs information processing tasks for a particular application.

- AE Qualifier:  that part of the AE title that uniquely identifies the particular application entity.

- AE Title:  a unique name for an application entity.

- APDU:  basic unit of information exchanged between the airborne CM AE and the ground CM AE.

- ASE:  an abstract module of a system providing service to other parts of the system.

- Context Management:  an independent service that meets ATSC addressing requirements.  It provides the mechanism for aircraft and ATC ground systems to indicate availability to other ATN users and to convey the addresses to be employed.

- CM AE abstract service interface:  the abstract interface between the CM-users and the CM-service provider.

- CM-ASE abstract service interface:  the abstract interface through which the CM-ASE service are accessed.

Note: — In version 1 of the CM application, this interface coincides with the CM-AE abstract service interface.

- CM-air-ASE:  an abstract part of the aircraft system which performs the communication related functions of CM.

- CM-air-user:  the abstract part of the aircraft system which performs the non communication related functions of CM.

- CM-CF:  that abstract part of the application entity which performs the mapping between the CM-ASE service primitives and other elements within the CM application.

- CM-ground-ASE:  an abstract part of the ground system which performs the communication related functions of CM.

- CM-ground-user:  the abstract part of the ground system which performs the non communication related functions of CM.

- CM service primitive: a function of a CM-AE that is not broken down further into sub-functions, and is presented as part of the CM-AE abstract service interface (i.e., request, indication, response, or confirmation).

- CM service provider: the CM-service provider is composed of the ground and airborne CM AEs, all underlying data communication protocol entities and the physical media.  As a consequence, it encompasses everything between the CM-AE service interfaces of the end-users of the CM application.

- Dialogue Service:  the service which allows the CM-air-ASE to communicate CM-ground-ASE and vice-versa.

- Long TSAP:  the long TSAP is composed of the RDP and the short TSAP.

- Message:   basic unit of information exchanged between the airborne CM Application and the ground CM Application.

- Residual Error Rate:  is defined as the ratio of messages mis-delivered, non-delivered, or delivered with an error undetected by the system, over the total number of messages delivered to the system.

- Short TSAP:  the short TSAP is composed of the ARS, the LOC, the SYS and the SEL.

#### Conventions For Expressing Requirements

##### The following conventions apply for expressing requirements in this document:

- shal.l - used to state a mandatory requirement.

- should - used to state a recommended practice.

### References

#### The following references are used in this document:

- Automatic Dependent Surveillance (ADS) and Air Traffic Services (ATS) Data Link Applications Guidance Material, June 1995.

- ISO/IEC IS 8825-2, ITU-T Recommendation X.691, Information Technology - ASN.1 Encoding Rules - Packed Encoding Rules (PER).

- ISO/IEC 8824-1, ITU-T Recommendation X.682, Information Technology - Abstract Syntax Notation One (ASN.1).

- Draft SARPs for ATN Upper Layers for CNS/ATM-1 Package, (as proposed in 15 December 1995).

- Reference Document for Traffic Types.

## Application Functionality

### This section lists the operational requirements as defined by the ADSP that CM addresses.

- Establishment of Data Link Between Aircraft Avionics and the FDPS; and

- Registration of the Communications and Surveillance Capabilities of an Aircraft.

### Mapping of Operational Requirements to Functional Descriptions

#### This section presents how each of the two CM related ORs listed above can be mapped to the functions described in Section 1.3.  Table 1-1 shows which functional descriptions assist in achieving each OR and are labeled as follows (section reference in parentheses):

- FD1:  Logon (Section 1.3.1),

- FD2:  Update (Section 1.3.2),

- FD3:  Contact (Section 1.3.3),

- FD4:  Forward (Section 1.3.4), and

- FD5:  Registration (Section 1.3.5).

| Operational Requirement | FD1 | FD2 | FD3 | FD4 | FD5 |
| --- | --- | --- | --- | --- | --- |
| Establishment of Data Link Between Aircraft Avionics and the FDPS | Ö | Ö | Ö | Ö |  |
| Registration of the Communications and Surveillance Capabilities of an Aircraft |  |  |  |  | Ö |

Table 1-1:  OR to Functional Description Mapping

## Functional Descriptions

### FD1:  Logon

#### Functional Description

#### The Logon function can only be air initiated.  The aircraft system can use the logon function to provide an aircraft application name and version number for each air-only initiated application, and an aircraft application name, address, and version number for each application that the aircraft wishes to use that can be ground initiated, along with flight plan information as required by the ground system.  In response, the ground provides a ground application name for each ground-only initiated requested application and a ground application name, address and version number for each requested application that can be air initiated that the ground  can support

#### Up to a maximum of 256 applications can be supported.

#### Each time a logon is accomplished between a given aircraft and a ground system the latest exchanged information replaces any previous information for the indicated application.

#### Message Descriptions

##### The CM Logon Request message provides required flight plan information, the aircraft’s CM application name and address, and information for each aircraft application for which data link services are desired.  For each application that can be ground initiated the aircraft must provide the application name, version number and address.  For each application which is only air initiated the aircraft must provide the aircraft application name and version number.

##### The CM Logon Response message provides an indication of success or failure of the logon, as well as information for the logon-indicated air-initiated applications.  For each desired air-initiated application the ground provides the application name, version number, and address.

### FD2:  Update

#### Functional Description

##### This function provides a method for the ground system to provide updated ground system application information for its application information to the aircraft system.  This function presumes that the log-on function has been accomplished.

#### Message Descriptions

##### The CM Update message can provide updated ground information for up to 256 applications.  For each updated application the ground provides the application’s name, version number and address.

### FD3:  Contact

#### Functional Description

##### This function provides a method for the ground system to request the aircraft system to initiate the Logon function with a designated ground system.  It is expected that the Contact function will only be used when ground connectivity is not available between respective ground system applications.  This function presumes that the log-on function has been accomplished with the ground system initiating the contact function.  The ground initiates this function with a contact request specifying which ground system the aircraft should log-on with.  The aircraft initiates a log-on as specified above and indicates the success or lack thereof of the logon.

#### Message Descriptions

#### The CM Contact Request message provides the ground system CM application address that the initiating ground system is requesting the aircraft to logon with.

##### The CM Contact Response message provides the information indicating whether or not the requested contact was successful.

### FD4:  Forwarding

#### Functional Description

##### This function provides a method for a ground system to forward aircraft information received from the CM Logon function to another ground system.  This function is initiated by a ground system having completed a successful logon, who can then forward the aircraft CM Logon information to other ground systems.  It is a one way forwarding of information (no response is permitted from the receiving CM-ground-user).  The ground system receiving CM information can then initiate a CM Update function to provide the information to the aircraft for any air-initiated applications.

#### Message Descriptions

##### The CM Forward Request message contains the flight plan information, the aircraft’s CM application name, address, and application information as provided in the initial logon.

### FD5:  Registration

#### Functional Description

##### This function provides a method for the air and ground CM applications to make available the application name, address, and version number for each application exchanged in the logon, update or forward functions to other applications or communications systems in the aircraft or on the ground.

#### Message Descriptions

##### There are no aircraft system to/from ground system message exchanges for this function.

# GENERAL REQUIREMENTS

## Performance Requirements

- Note: — Systems developed to support Context Management functionality will be capable of meeting the Required Communication Performance (RCP) appropriate for the phase of operation as specified in [5].

## Time Accuracy Requirements

### Absolute times which are sent as parameters over the data link shall  ([1])  be as accurate as the required resolution of the time parameter.

### Absolute times shall  ([2])  be UTC.

### Dates shall  ([3])  be expressed as UTC date (i.e., the day increments at 2359:59z).

### Relative times shall  ([4])  be accurate to ± 0.1 second.

### Where timestamps are used they shall  ([5])  consist of year, month, day and hour, minute, second.

## Security Requirements

### There are no internationally approved operational requirements relating to data link application security.

## Backwards Compatibility Requirements

- Note: — This specification describes the version 1 of the CNS/ATM-1 Package CM application. .  Best efforts will be made to ensure that subsequent versions of this protocol will be backwards compatible.

### For CNS/ATM-1 the CM-air-ASE and CM-ground-ASE version numbers shall  ([6])  both be set to one.

## Error Processing Requirements

### In the event of information input by the user being incompatible with that able to be processed by the system, the user shall  ([7])  be notified.

### In the event of a user invoking a CM-ASE service when it is not in a state specified in chapter 5, the invocation shall  ([8])  be rejected and the user shal.l be notified.

# THE ABSTRACT SERVICE

## Introduction

### This section defines the abstract service interface for the CM service.  There is no requirement to implement the service in a CM product; however, it is necessary to implement the ground based and air based system in such a way that it shall  ([9])  be impossible to detect (from the peer system) whether or not an interface has been built.

Note: — The CM-ASE abstract service is described in this chapter from the viewpoint of the CM-ASE-air-user, the CM-ASE-ground-user and the CM service-provider.

## The CM Functional Model

Note 1: — Figure 3-1 shows the functional model of the CM Application. The functional modules identified in this model are the following :

- the CM-user,

- the CM Application Entity (CM-AE) service interface,

- the CM-AE,

- the CM Control Function (CM-CF),

- the CM Application Service Element (CM-ASE) service interface,

- the CM-ASE, and

- the Dialogue Service (DS) interface.

Figure 3-1:  Functional Model of the CM Application

- Note 2: — The CM-user represents the operational part of the CM system. This user does not perform the communication functions but relies on a communication service provided to it via the CM-AE through the CM-AE service interface. The individual actions at this interface are called CM-AE service primitives. Similarly, individual actions at other interfaces in the communication system are called service primitives at these interfaces.

- Note 3: — The CM-AE consists of several elements including the CM-ASE and the CM-CF.  The DS interface is made available by the CM-CF to the CM-ASE for communication with the peer CM-ASE.

- Note 4: — The CM-ASE is the element in the communication system which executes the CM specific protocol. In other words, it takes care of the CM specific service primitive sequencing actions, message creation, timer management, error and exception handling.

- Note 5: — The CM-ASE interfaces only with the CM-CF. This CM-CF is responsible for mapping service primitives received from one element (such as the CM-ASE and the CM-user) to other elements which interface with it. The part of the CM-CF which is relevant from the point of view of these SARPs, i.e. the part between the CM-user and the CM-ASE, will map CM-AE service primitives to CM-ASE service primitives transparently in the CNS/ATM-1 Package.

- Note 6: — The DS interface is the interface between the CM-ASE and part of CM-CF underneath, and provides the dialogue service [4].

## The CM-ASE Abstract Service

### An implementation of either the CM ground based service or the CM air based service shall  ([10])  exhibit external behavior consistent with having implemented a CM-ground-ASE, or CM-air-ASE respectively, with the following abstract service interface primitives, making them available to the CM-ground-user or CM-air-user respectively:

- CM-logon service as defined in section 3.5,

- CM-update service as defined in section 3.6,

- CM-contact service as defined in section 3.7,

- CM-end service as defined in section 3.8,

- CM-forward service as defined in section 3.9,

- CM-user-abort service as defined in section 3.10, and

- CM-provider-abort service as defined in section 3.11.

## Conventions

- Note 1: — For a given primitive, the presence of each parameter is described by one of the following values in the parameter tables in chapter 3.

| a) | blank | not present; |
| --- | --- | --- |
| b) | C | conditional upon some predicate explained in the text; |
| c) | C(=) | conditional upon the value of the parameter to the left being present, and equal to that value; |
| d) | M | mandatory; |
| e) | M(=) | mandatory, and equal to the value of the parameter to the left; |
| f) | U | user option. |

- Note 2: — The following abbreviations are used in this document:

- Req - request; data is input by CM-user initiating the service to its respective ASE

- Ind - indication; data is indicated by the receiving ASE to its respective CM-user

- Rsp - response; data is input by receiving CM user to its respective ASE

- Cnf - confirmation; data is confirmed by the initiating ASE to its respective CM-user.

- Note 3: — An unconfirmed service allows a message to be transmitted in one direction without a corresponding response.

- Note 4: — A confirmed service provides end-to-end confirmation that a message sent by one user was received by its peer user.

## CM-logon Service

Note: —The CM-logon service allows the CM-air-user to initiate data link service.  The CM-air-user provides information on each data link application for which it desires a data link service.  The CM-ground-user responds indicating whether or not the CM-logon was successful, and if successful, includes information on each data link application it can support.  It is a confirmed service.

### The CM-logon service primitives shall  ([11])  contain the parameters as presented tables 3-1 and 3-2.

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| ICAO Facility Designator | M |  |  |  |
| Aircraft Identifier | M | M(=) |  |  |
| CM ASE Version Number |  | C |  |  |
| Logon Request | M | M(=) |  |  |
| Logon Response |  |  | M | M(=) |
| Class of Communication Service | U |  |  |  |
| Maintain Dialogue |  |  | U | C(=) |

Table 3-1:  CM-logon Service Parameters

Air-ASE version Number £ Ground-ASE Version Number

| Parameter Name | Req | Cnf |
| --- | --- | --- |
| ICAO Facility Designator | M |  |
| Aircraft Identifier | M |  |
| CM ASE Version Number |  | M |
| Logon Request | M |  |
| Class of Communication Service | U |  |

Table 3-2:  CM-logon Service Parameters

Air-ASE version Number > Ground-ASE Version Number

### ICAO Facility Designator

- Note: — This parameter contains the addressed ground system’s ICAO facility designator.

#### The ICAO Facility Designator parameter value shall  ([12])  conform to the abstract syntax eight-character ICAO facility designator.

### Aircraft Identifier

- Note: — This parameter contains 24-bit aircraft identifier of the aircraft initiating the CM-logon service.

#### The Aircraft Identifier parameter value shall  ([13])  conform to the abstract syntax 24-bit aircraft-id

### CM ASE Version Number

- Note: — This parameter contains the version number of the CM-air-ASE.

#### Only if the CM-air-ASE version number is less than the CM-ground-ASE version number shall  ([14])  the CM-air-ASE version number be indicated to the CM-ground-user.

#### Only if the CM-air-ASE version number is greater than the CM-ground-ASE version number shall  ([15])  the CM-ground-ASE version number be confirmed to the CM-air-user.

- Note 1: — If the CM-air-ASE version number is the same as the CM-ground-ASE version number, an indication is not given to the CM-ground-user, nor a confirmation to the CM-air-user.

- Note 2: — For CNS/ATM-1 the CM-air-ASE and CM-ground-ASE version numbers are both set to 1.

### Logon Request

- Note: —The logon request parameter contains the following data:

- information for each data link application available on the aircraft, for which the aircraft requires data link service, and

- aircraft flight plan information (e.g. flight id, aircraft destination and departure airport and time) as required by the addressed ground system,

#### This Logon Request parameter value shall  ([16])  conform to the ASN.1 abstract syntax CMLogonRequest.

#### Only if the CM-air-ASE version number is less than or equal to the CM-ground-ASE version number (i.e. the CM-ground-ASE can decode and present the information to the CM-ground user) shall  ([17])  the Logon Request parameter be indicated to the CM-ground-user.

### Logon Response

- Note: — This parameter contains the following data:

- an indication whether or not the logon was successful, and, if successful,

- information for each requested data link application for which the ground is able to provide data link service.

#### The Logon Response parameter value shall  ([18])  conform to the ASN.1 abstract syntax CMLogonResponse.

#### If the Logon Request parameter was indicated to the CM-ground-user, the CM-ground-user shall  ([19])  provide the Logon Response parameter.

### Class of Communication Service

- Note: — This parameter contains the value of the required class of communication service. if specified by the CM-air-user

#### When this parameter is specified by the CM-air-user, the Class of Communication Service parameter value shall  ([20])  have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note 1: — Classes of Communication Service are detailed in [5].

- Note 2: — If not specified by the CM-air-user, this indicates that there is no routing preference.

### Maintain Dialogue

- Note 1: — This parameter is used to indicate whether or not the requested CM dialogue is to remain open after a Logon Response.

- Note 2: — Whenever a CM dialogue is kept open by the CM-ground-user, it must later be explicitly closed by the CM-ground-user.

- Note 3: — This parameter is only be provided by the CM-ground-user when the CM-ground-user wishes to keep the CM dialogue open.

#### If provided by the CM-ground-user this parameter shall  ([21])  have the abstract value “accepted”.

## CM-update Service

- Note: — The CM-update service can be used by the CM-ground-user to transmit updated ground information for its applications to update previously coordinated CM-logon information.  It is an unconfirmed service.

### The CM-update service primitives shall  ([22])  contain the parameters as presented table 3-3.

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Aircraft Identifier | C |  |
| ICAO Facility Designator | C | C(=) |
| Update Information | M | M(=) |
| Class of Communication Service | U |  |

Table 3-3:  CM-update Service Parameters

### Aircraft Identifier

- Note: — This parameter contains the addressed aircraft’s 24-bit aircraft identifier.

#### The Aircraft Identifier parameter value shall  ([23])  conform to the abstract syntax 24-bit aircraft-id

#### If a CM dialogue does not exist when a CM-ground user invokes the CM-update service request, the CM-ground-user shall  ([24])  provide the Aircraft Identifier parameter value.

- Note: — The CM-update service does not use this parameter when a CM dialogue exists.

### ICAO Facility Designator

- Note: — This parameter contains the ground system’s ICAO facility designator.

#### The ICAO Facility Designator parameter value shall  ([25])  conform to the abstract syntax eight-character ICAO facility designator.

#### If a CM dialogue does not exist when a CM-ground user invokes the CM-update service request, the CM-ground-user shall  ([26])  provide the ICAO Facility Designator parameter value.

- Note: — The CM-update service does not use this parameter when a CM dialogue exists.

### Update Information

- Note: —This parameter contains information on each updated data link application.

#### The Update Information parameter value shall  ([27])  conform to the ASN.1 abstract syntax CMUpdate.

### Class of Communication Service

- Note: — This parameter contains the value of the required class of communication service. if specified by the CM-ground-user

#### When this parameter is specified by the CM-ground-user, the Class of Communication Service parameter value shall  ([28])  have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note 1: — Classes of Communication Service are detailed in [5].

- Note 2: — If not specified by the CM-ground-user, this indicates that there is no routing preference.

## CM-contact Service

- Note: — The CM-contact service can be used by the CM-ASE-ground-user after successful completion of a CM logon, to request that an aircraft logon with another ground system.  It is a confirmed service.

### The CM-contact service primitives shall  ([29])  contain the parameters as presented table 3-4.

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| Aircraft Identifier | C |  |  |  |
| ICAO Facility Designator | C | C(=) |  |  |
| Contact Request | M | M(=) |  |  |
| Contact Response |  |  | M | M(=) |
| Class of Communication Service | U |  |  |  |

Table 3-4:  CM-contact Service Parameters

### Aircraft Identifier

- Note: — This parameter contains the addressed aircraft’s 24-bit aircraft identifier.

#### The Aircraft Identifier parameter value shall  ([30])  conform to the abstract syntax 24-bit aircraft-id

#### If a CM dialogue does not exist when a CM-ground user invokes the CM-contact service request, the CM-ground-user shall  ([31])  provide the Aircraft Identifier parameter value.

- Note: — The CM-contact service does not use this parameter when a CM dialogue exists.

### ICAO Facility Designator

- Note: — This parameter contains the ground system’s ICAO facility designator.

#### The ICAO Facility Designator parameter value shall  ([32])  conform to the abstract syntax eight-character ICAO facility designator.

#### If a CM dialogue does not exist when a CM-ground user invokes the CM-contact service request, the CM-ground-user shall  ([33])  provide the ICAO Facility Designator parameter value.

- Note: — The CM-contact service does not use this parameter when a CM dialogue exists.

### Contact Request

- Note: —This parameter contains the ICAO facility designator for the ground system that the CM-ground-user requests the aircraft to contact.

#### The Contact Request parameter value shall  ([34])  conform to the ASN.1 abstract syntax CMContactRequest.

### Contact Response

- Note: — This parameter indicates success, or lack thereof, of the requested contact.

#### The Contact Response parameter value shall  ([35])  conform to the ASN.1 abstract syntax CMContactResponse.

### Class of Communication Service

- Note: — This parameter contains the value of the required class of communication service. if specified by the CM-ground-user

#### When this parameter is specified by the CM-ground-user, the Class of Communication Service parameter value shall  ([36])  have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note 1: — Classes of Communication Service are detailed in [5].

- Note 2: — If not specified by the CM-ground-user, this indicates that there is no routing preference.

## CM-end Service

- Note: — This service provides the capability for the CM-ground-user to terminate a CM dialogue.  This service is only needed when the CM-ground-user accepts a request for a CM dialogue during the logon process. It is an unconfirmed service.

### Only the CM-ground-user shall  ([37])  be capable of initiating the CM-end service.

### The CM-end service primitives shall  ([38])  contain the parameters as presented table 3-5.

| Parameter Name | Req | Ind |
| --- | --- | --- |
| none |  |  |

Table 3-5:  CM-end Service Parameters

## CM-forward Service

- Note: — The CM-forward service can be used by a CM-ground-user to forward data received in a CM-logon request to another CM-ground system.  It is a confirmed service.

### The CM-forward service primitives shall  ([39])  contain the parameters as presented in tables 3-6 and 3-7.

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Called ICAO Facility Designator | M |  |
| Calling ICAO Facility Designator | M | M(=) |
| CM ASE Version Number |  | C |
| Forward Request | M | M(=) |
| Class of Communication Service | U |  |

Table 3-6:  CM-forward Service Parameters

Sending Ground-ASE version Number £ Receiving Ground-ASE Version Number

| Parameter Name | Req | Cnf |
| --- | --- | --- |
| Called ICAO Facility Designator | M |  |
| Calling ICAO Facility Designator | M |  |
| CM ASE Version Number |  | M |
| Forward Request | M |  |
| Class of Communication Service | U |  |

Table 3-7:  CM-forward Service Parameters

Sending Ground-ASE version Number > Receiving Ground-ASE Version Number

### Called ICAO Facility Designator

- Note: — This parameter contains the called ground system’s ICAO facility designator.

#### The Called ICAO Facility Designator parameter value shall  ([40])  conform to the abstract syntax eight-character ICAO facility designator.

### Calling ICAO Facility Designator

- Note: — This parameter contains the initiating ground system’s ICAO facility designator.

### The Calling ICAO Facility Designator parameter value shall  ([41])  conform to the abstract syntax eight-character ICAO facility designator.

### CM ASE Version Number

- Note: — This parameter contains the version number of the sending CM-ground-ASE.

#### Only if the sending CM-ground-ASE version number is less than the receiving CM-ground-ASE version number shall  ([42])  the sending CM-ground-ASE version number be indicated to the receiving CM-ground-user.

#### Only if the sending CM-ground-ASE version number is greater than the receiving CM-ground-ASE version number shall  ([43])  the receiving CM-ground-ASE version number be confirmed to the sending CM-ground-user.

- Note 1: — If the sending CM-ground-ASE version number is the same as the receiving CM-ground-ASE version number, an indication is not given to the receiving CM-ground-user, nor a confirmation to the sending CM-ground-user.

- Note 2: — For CNS/ATM-1 the sending CM-ground-ASE and receiving CM-ground-ASE version numbers are both set to 1.

### Forward Request

- Note: —This parameter contains information as provided in the CM Logon Request.

#### The Forward Request  parameter value shall  ([44])  conform to the ASN.1 abstract syntax CMForwardRequest.

### Class of Communication Service

- Note: — This parameter contains the value of the required class of communication service if specified by the initiating CM-ground-user

#### When this parameter is specified by the CM-ground-user, the Class of Communication Service parameter value shall  ([45])  have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note 1: — Classes of Communication Service are detailed in [5].

- Note 2: — If not specified by the CM-ground-user, this indicates that there is no routing preference.

## CM-user-abort Service

- Note: — This service provides the capability for either the CM-air-user or the CM-ground-user to abort communication with its peer. This can be used for operational or technical reasons. It can be invoked at any time by an active user.  It is an unconfirmed service.

### The CM-user-abort service primitives shall  ([46])  contain the parameters as presented table 3-8.

| Parameter Name | Req | Ind |
| --- | --- | --- |
| none |  |  |

Table 3-8:  CM-user-abort Service Parameters

## CM-provider-abort Service

- Note: — This service provides the capability for the CM-service provider to inform its users, if active, that it can no longer provide the CM service.

### The CM-provider-abort service primitives shall  ([47])  contain the parameters as presented table 3-9.

| Parameter Name | Ind |
| --- | --- |
| Reason | M |

Table 3-9:  CM-provider-abort Service Parameters

### Reason

- Note: — This parameter identifies the reason for the abort.

#### The Reason parameter value shall  ([48])  conform to the ASN.1 abstract syntax CMAbortReason

# FORMAL DEFINITIONS OF MESSAGES

## ASN.1

### The abstract syntax of the CM protocol data units shall  ([49])  comply with the description contained in the ASN.1 module CMMessageSetVersion1 (conforming to [3]), as defined below:

CMMessageSetVersion1 DEFINITIONS AUTOMATIC TAGS :: =

BEGIN

-- ----------------------------------------------------------------------------------

-- CM Message Structure

-- ----------------------------------------------------------------------------------

--  Aircraft-generated messages

CMAircraftMessage ::=  CHOICE

{

cmLogonRequest			CMLogonRequest,

cmContactResponse		CMContactResponse,

cmAbortReason			CMAbortReason

...

}

--  Ground-generated messages

CMGroundMessage ::=  CHOICE

{

cmLogonResponse		CMLogonResponse,

cmUpdate			CMUpdate,

cmContactRequest		CMContactRequest,

cmForwardRequest		CMForwardRequest,

cmAbortReason			CMAbortReason

...

}

-- ----------------------------------------------------------------------------------

-- CM Message Components

-- ----------------------------------------------------------------------------------

AircraftFlightIdentification ::= IA5String (SIZE(2..8))

Airport ::= IA5String (SIZE(4))

APAddress :: = CHOICE

{

longTsap	LongTsap,

shortTsap	ShortTsap

}

APName ::= IA5String (SIZE(3))

APNameVersion ::= SEQUENCE

{

apName		APName,

apVersion	VersionNumber

}

APNameVersionAddress ::= SEQUENCE

{

apName			APName,

apVersion		VersionNumber,

apAddress		APAddress

}

CMAbortReason ::= ENUMERATED

{

timer-expired				(0),

undefined-error				(1),

invalid-PDU				(2),

not-permitted-PDU			(3),

dialogue-acceptance-not-permitted		(4),

dialogue-end-not-accepted			(5),

communication-service-error		(6),

communication-service-failure		(7)

}

CMContactRequest ::= SEQUENCE

{

icaoFacilityDesignation		ICAOFacilityDesignation,

address				LongTsap

}

CMContactResponse ::= Response

CMForwardRequest :: = SEQUENCE

{

aircraftFlightIdentification	AircraftFlightIdentification,

cMNameLongTSAP		CMNameLongTSAP,

groundInitiatedApplications	SEQUENCE SIZE (1..256) OF 	APNameVersionAddress													OPTIONAL,

airOnlyInitiatedApplications	SEQUENCE SIZE (1..256) OF	APNameVersion														OPTIONAL,

airportDeparture			Airport						OPTIONAL,

airportDestination		Airport						OPTIONAL,

timeDepartureetd			Time						OPTIONAL

}

CMLogonRequest :: = SEQUENCE

{

aircraftFlightIdentification		AircraftFlightIdentification,

cMNameLongTSAP			CMNameLongTSAP,

groundInitiatedApplications		SEQUENCE SIZE (1..256) OF 	APNameVersionAddress												OPTIONAL,

airOnlyInitiatedApplications		SEQUENCE SIZE (1..256) OF	APNameVersion													OPTIONAL,

icaoFacilityDesignation			ICAOFacilityDesignation			OPTIONAL,

airportDeparture				Airport					OPTIONAL,

airportDestination			Airport					OPTIONAL,

timeDepartureetd				Time					OPTIONAL

}

CMLogonResponse :: = SEQUENCE

{

response				Response,

airInitiatedApplications		SEQUENCE SIZE (1..256) OF	APNameVersionAddress											OPTIONAL,

groundOnlyInitiatedApplications	SEQUENCE SIZE (1..256) OF	APNameVersion												OPTIONAL

}

CMNameLongTSAP :: = SEQUENCE

{

CMName	APName,

longTSAP	LongTsap

}

CMUpdate :: = CMLogonResponse

ICAOFacilityDesignation ::= IA5String (SIZE(8))

LongTsap ::= SEQUENCE

{

rDP			OCTET STRING (SIZE(5)),

shortTsap		ShortTsap

}

Response ::= ENUMERATED

{

success				(0),

logonNotSuccessful		(1),

contactNotSuccessful		(2)

}

ShortTsap ::= SEQUENCE

{

aRS			OCTET STRING (SIZE(3))		OPTIONAL,

locSysTsel		OCTET STRING  (SIZE(9..11))

}

Time ::= SEQUENCE

{

hours			Timehours,

minutes			Timeminutes

}

Timehours ::= INTEGER (0..23)

-- units = hour, range (0..23), resolution = 1 hour

Timeminutes ::= INTEGER (0..59)

-- units = minute, range (0..59), resolution = 1 minute

VersionNumber ::= INTEGER (0..255)

END

# PROTOCOL DEFINITION

## Sequence Rules

- Note: — The following figures define the valid sequences of primitives that are possible to be invoked during the operation of the CM application.  It shows the relationship in time between the service request and the resulting indication, and if applicable, the subsequent response and resulting confirmation.

### With the exception of abort primitives, only the sequence of primitives described below shall  ([50])  be permitted.

- Note: — Abort primitives may interrupt and terminate any of the normal message sequences outlined below.

### With the exception of abort primitives, the CM-air-ASE and CM-ground-ASE shall  ([51])  process primitives in the order in which they are received.

- Note: — This ensures that the ASE willguarantee message sequencing, with the exception of aborts.

### CM-logon-Service

#### CM-Air-ASE Version Less Than Or Equal To The CM-Ground-ASE Version

- Note: — The following sequence of messages, shown in Figure 5-1, occurs when the CM-logon service is initiated with a CM-air-ASE version number less than or equal to the CM-ground-ASE version number.

Figure 5-1:  Sequence Diagram for CM-logon Service

CM-Air-ASE Version £ CM-Ground-ASE Version

#### CM-Air-ASE Version Greater Than The CM-Ground-ASE Version

- Note: — The following sequence of messages, shown in Figure 5-2, occurs when the CM-logon service is initiated with a CM-air-ASE version number greater than the CM-ground-ASE version number.

Figure 5-2:  Sequence Diagram for CM-logon Service

CM-Air-ASE Version >CM-Ground-ASE Version

### CM-update Service

- Note 1: — The following sequence of messages, shown in Figure 5-3, occurs when the CM-update service is initiated when there is no CM dialogue in place.

Figure 5-3:  Sequence Diagram for CM-update Service

No Existing CM Dialogue

- Note 2: — The following sequence of messages, shown in Figure 5-4, occurs when the CM-update service is initiated when there is a CM dialogue in place.

Figure 5-4:  Sequence Diagram for CM-update Service

Existing CM Dialogue

### CM-contact Service

#### CM-contact With CM-logon as Illustrated in Figure 5-1 with No Existing CM Dialogue

- Note: — The following sequence of messages, shown in Figure 5-5, occurs when the CM-contact service is initiated with no existing CM dialogue in place and a CM-logon service occurs as indicated in Figure 5-1.

Figure 5-5:  Sequence Diagram for CM-contact Service

No Existing CM Dialogue

With CM-logon Service as in Figure 5-1

#### CM-contact With CM-logon as Illustrated in Figure 5-1 with Existing CM Dialogue

- Note: — The following sequence of messages, shown in Figure 5-6, occurs when the CM-contact service is initiated with existing CM dialogue and a CM-logon service occurs as indicated in Figure 5-1.

Figure 5-6:  Sequence Diagram for CM-contact Service

With Existing CM Dialogue

With CM-logon Service as in Figure 5-1

#### CM-contact With CM-logon as Illustrated in Figure 5-2 No Existing CM Dialogue

- Note 1: — The following sequence of messages, shown in Figure 5-7, occurs when the CM-contact service is initiated with no existing CM dialogue and a CM-logon service occurs as indicated in Figure 5-2.

Figure 5-7:  Sequence Diagram for CM-contact Service

No Existing CM Dialogue

With CM-logon Service as in Figure 5-2

- Note 2: — Note: this figure does not imply that only one CM-logon attempt is made for a given CM-Contact request.

#### CM-contact With CM-logon as Illustrated in Figure 5-2,  Existing CM Dialogue

- Note 1: — The following sequence of messages, shown in Figure 5-8, occurs when the CM-contact service is initiated with an existing CM dialogue and a CM-logon service occurs as indicated in Figure 5-2.

Figure 5-8:  Sequence Diagram for CM-contact Service

Existing CM Dialogue

With CM-logon Service as in Figure 5-2

- Note 2: —This figure does not imply that only one CM-logon attempt is made for a given CM-Contact request.

#### CM-contact Without CM-logon as Requested No Existing CM Dialogue

- Note: — The following sequence of messages, shown in Figure 5-9, occurs when the CM-contact service is initiated with no existing CM dialogue and the CM-logon service is not initiated as requested.

Figure 5-9:  Sequence Diagram for CM-contact Service

No Existing CM Dialogue

Without CM-logon Service as Requested

#### CM-contact Without CM-logon as Requested With Existing CM Dialogue

- Note: — The following sequence of messages, shown in Figure 5-10, occurs when the CM-contact service is initiated with  existing CM dialogue and the CM-logon service is not initiated as requested.

Figure 5-10:  Sequence Diagram for CM-contact Service

With Existing CM Dialogue

Without CM-logon Service as Requested

### CM-end Service

- Note: — The following sequence of messages, shown in Figure 5-11, occurs when the CM-end service is initiated by the CM-ground-user.

Figure 5-11:  Sequence Diagram for CM-end Service

### CM-forward Service (Ground-Ground)

#### Sending CM-Ground-ASE Version Less Than Or Equal To The Receiving CM-Ground-ASE Version

- Note: — The following sequence of messages, shown in Figure 5-12, occurs when the CM-forward service is initiated with a sending CM-ground-ASE version number less than or equal to the receiving CM-ground-ASE version number.

Figure 5-12:  Sequence Diagram for CM-forward Service

Sending CM-Ground-ASE Version £ Receiving CM-Ground-ASE Version

#### Sending CM-Ground-ASE Version Greater Than The Receiving CM-Ground-ASE Version

- Note: — The following sequence of messages, shown in Figure 5-13, occurs when the CM-forward service is initiated with a sending CM-ground-ASE version number greater than the receiving CM-ground-ASE version number.

Figure 5-13:  Sequence Diagram for CM-forward Service

Sending CM-Ground-ASE Version >Receiving CM-Ground-ASE Version

### CM-user-abort Service

#### CM-user-abort Service; CM-Air-User Initiated

- Note: — The following sequence of messages, shown in Figure 5-14, occurs when the CM-user-abort service is initiated by the CM-air-user.

Figure 5-14:  Sequence Diagram for CM-user-abort Service

CM-Air-User Initiated

#### CM-user-abort Service, CM-Ground-User Initiated

- Note: — The following sequence of messages, shown in Figure 5-15, occurs when the CM-user-abort service is initiated by the CM-ground-user.

Figure 5-15:  Sequence Diagram for CM-user-abort Service

CM-Ground-User Initiated

### CM-provider-abort-Service

#### Dialogue Service Abort

- Note: — The following sequence of messages, shown in Figure 5-16, occurs when the dialogue service provider (below the level of the ASE) aborts.

Figure 5-16:  Sequence Diagram for CM-provider-abort Service:

Dialogue Service Abort

#### CM-Air-ASE Abort

- Note: — The following sequence of messages, shown in Figure 5-17, occurs when the CM-air-ASE aborts.

Figure 5-17:  Sequence Diagram for CM-provider-abort Service:

CM-Air-ASE Abort

#### CM-Ground-ASE Abort

- Note: — The following sequence of messages, shown in Figure 5-18, occurs when the CM-ground-ASE aborts.

Figure 5-18:  Sequence Diagram for CM-provider-abort Service:

CM-Ground-ASE Abort

### CM-user-abort Service (Ground-Ground)

#### CM-user-abort Service; Receiving CM-Ground-User Initiated

- Note: — The following sequence of messages, shown in Figure 5-18, occurs when the CM-user-abort service is initiated by the receiving CM-ground-user.

Figure 5-18:  Sequence Diagram for CM-user-abort Service

Receiving CM-Ground-User Initiated

#### CM-user-abort Service, Sending CM-Ground-User Initiated

- Note: — The following sequence of messages, shown in Figure 5-19, occurs when the CM-user-abort service is initiated by the Sending CM-ground-user.

Figure 5-19:  Sequence Diagram for CM-user-abort Service

Sending CM-Ground-User Initiated

### CM-provider-abort-Service (Ground-Ground)

#### Dialogue Service Abort

- Note: — The following sequence of messages, shown in Figure 5-20,  occurs when the dialogue service provider (below the level of the ASE) aborts.

Figure 5-20:  Sequence Diagram for CM-provider-abort Service:

Dialogue Service Abort

#### Receiving CM-Ground-ASE Abort

- Note: — The following sequence of messages, shown in Figure 5-21, occurs when the receiving CM-ground-ASE aborts.

Figure 5-21:  Sequence Diagram for CM-provider-abort Service:

Receiving CM-Ground-ASE Abort

#### CM-Ground-ASE Abort (Ground

- Note: — The following sequence of messages, shown in Figure 5-22, occurs when the Sending CM-ground-ASE aborts.

Figure 5-22:  Sequence Diagram for CM-provider-abort Service:

Sending CM-Ground-ASE Abort

## CM Application Timers

- Note: —Table 5-1 lists the time constraints related to the CM application.  Each time constraint requires a time to be set in the CM protocol machine.

### A CM-ASE shall  ([52])  measure the time between the initial event and the corresponding final event.

### If the maximum time is exceeded before the final event has occurred, a CM-ASE shall  ([53])  take appropriate action as defined in section 5.4.

#### The action shall  ([54])  be taken when the maximum time as indicated in Table 5-1 has expired.

| CM Service | Timer | Timer Value | Timer Start Event | Timer Stop Event |
| --- | --- | --- | --- | --- |
| CM-logon | tlogon | 4 min | D-START request | D-START confirmation |
| CM-update | tupdate | 4 min | D-START request | D-START confirmation |
| CM-contact | tcontact | 8 min | D-START request | D-START confirmation |
| CM-forward | tforward | 4 min | D-START request | D-START confirmation |
| CM-end | tend | 4 min | D-END request | D-END confirmation |

Table 5-1:  CM Application Timers

## CM-ASE Protocol Description

### Introduction

- Note: — This section presents requirements for CM-ASEs in specific states. Section 5.5 contains state tables for the CM-ASEs.

#### If no requirements are described for a CM service primitive when a CM-ASE is in a specific state, then the invocation of that service primitive shall  ([55])  be prohibited while the CM-ASE is in that state.

#### If no requirements are described for the arrival of a PDU when a CM-ASE is in a specific state, then exception handling procedures as described in section 5.4 shall  ([56])  apply.

### CM-Air-ASE Protocol Description

#### D-START Indication

##### Upon receipt of a D-START indication, if the CM-air-ASE is in the IDLE state then:

###### If the APDU contained in the D-START User Data parameter is a [CMUpdate] APDU the CM-air-ASE shall ([57])  :

- invoke CM-update indication with the following parameters:

  - the D-START Calling Peer ID parameter value as the CM-update ICAO Facility Designator parameter value, and

  - the APDU contained in the D-START User Data parameter as the CM-update Update Information parameter value, and

- invoke D-START response with “Rejected (permanent)” provided as D-START Result parameter value, and

- enter the IDLE state.

###### If the APDU contained in the D-START User Data parameter is a [CMContactRequest] APDU the CM-air-ASE shall ([58])  :

- invoke CM-contact indication with the following:

  - The D-START Calling Peer ID parameter value as the CM-contact ICAO Facility Designator parameter value, and

  - The APDU contained in the D-START User Data parameter as the CM-contact Contact Request parameter value, and

- enter the CONTACT state.

#### D-START Confirmation

###### Upon receipt of a D-START confirmation, if the CM-air-ASE is in the LOGON state and if the APDU contained in the D-START User Data parameter is a [CMLogonResponse] APDU the CM-air-ASE shall ([59])  :

- stop timer tlogon, and

- if the abstract value of the D-START Result parameter is “rejected (permanent)” and the abstract value of the D-START Reject Source parameter is “DS user” then:

  - if the version number of the CM-air-ASE is greater than the D-START DS User Version Number parameter value the CM-air-ASE shall  ([60])  invoke CM-logon confirmation with the DS User Version Number parameter value as the CM-logon CM-ASE Version Number parameter value, or

  - else the CM-air-ASE shall  ([61])  invoke CM-logon confirmation with the APDU contained in the D-START User Data parameter as the CM-logon Logon Response parameter, and

  - enter the IDLE state.

- if the abstract value of the D-START Result parameter is “accepted” then:

  - if the version number of the CM-air-ASE is greater than the D-START DS User Version Number parameter value, the CM-air-ASE shall  ([62])  invoke CM-logon confirmation with the D-START DS User Version Number parameter value as the CM-logon CM ASE Version Number parameter value, or

  - else the CM-air-ASE shall  ([63])  invoke CM-logon confirmation with the APDU contained in the D-START User Data parameter as the CM-logon Logon Response parameter value and

  - enter the DIALOGUE state.

#### D-DATA Indication

##### Upon receipt of a D-DATA indication, if the CM-air-ASE is in the DIALOGUE state then:

###### If the APDU contained in the D-DATA User Data parameter is a [CMUpdate] APDU the CM-air-ASE shall ([64])  :

- invoke CM-update indication with the APDU contained in the D-START User Data parameter as the CM-update service Update Information parameter value, and

- remain in the DIALOGUE state.

###### If the APDU contained in the D-DATA User Data parameter is a [CMContactRequest] APDU the CM-air-ASE shall ([65])  :

- invoke CM-contact indication with the APDU contained in the D-START User Data parameter as the CM-contact service Contact Request parameter value, and

- enter the CONTACT DIALOGUE state.

#### D-END Indication

###### Upon receipt of a D-END indication, if the CM-air-ASE is in the DIALOGUE state then the CM-air-ASE shall ([66])  :

- invoke CM-end indication,

- invoke D-END response with the D-END Result parameter set to the abstract value “accepted”, and

- enter the IDLE state.

#### CM-logon Request

##### Upon receipt of a CM-logon request:

###### If the CM-air-ASE is in the IDLE state the CM-air-ASE shall ([67])  :

- create a CMAircraftMessage APDU with a cmLogonRequest APDU-element based on the Logon Request parameter value,

- invoke D-START request with the following:

  - the CM-logon ICAO Facility Designator parameter value as the D-START Called Peer ID parameter value,

  - the CM-logon Aircraft Identifier parameter value as the D-START Calling Peer ID parameter value,

  - the CM-air-ASE version number as the D-START DS User Version Number parameter value,

  - the D-START Quality of Service parameters set as follows:

    - if provided, the CM-logon service Class of Communication parameter value as the D-START QOS Routing Class parameter value,

    - the abstract value of “flight regularity communications”, as the D-START QOS Priority parameter value, and

    - the abstract value of “low” as the D-START QOS Residual Error Rate, and

  - the CMAircraftMessage APDU as the D-START User Data parameter value,

- start timer tlogon, and

- enter the LOGON state.

#### CM-contact Response

##### Upon receipt of a CM-contact response, if the CM-air-ASE is in the CONTACT state the CM-air-ASE shall ([68])  :

- create a CMAircraftMessage APDU with cmContactResponse APDU-element based on the CM-contact Contact Response parameter value,

- invoke D-START response with the following:

  - the abstract value “rejected (permanent)” as D-START Result parameter value, and

  - the CMAircraftMessage APDU as the D-START User Data parameter value, and

- enter the IDLE state.

##### Upon receipt of a CM-contact response, if the CM-air-ASE is in the CONTACT DIALOGUE state the CM-air-ASE shall ([69])  :

- create a CMAircraftMessage APDU with a cmContactResponse APDU-element based on the CM-contact Contact Response parameter value,

- invoke D-DATA request with the CMAircraftMessage APDU as the D-DATA User Data parameter value, and

- enter the DIALOGUE state.

#### CM-user-abort Request

##### Upon receipt of a CM-user-abort request, if the CM-air-ASE is not in the IDLE state the CM-air-ASE shall ([70])  :

- stop timer tlogon, if set,

- invoke D-ABORT request with the D-ABORT Originator parameter set to the abstract value “user”, and

- enter the IDLE state.

#### D-ABORT Indication

##### Upon receipt of a D-ABORT indication, if the CM-air-ASE is not in the IDLE state the CM-air-ASE shall ([71])  :

- stop timer tlogon, if set

- if the CM-air-user is an active user, then:

  - if the D-ABORT Originator parameter contains the abstract value “user” invoke CM-user-abort indication,

  - else invoke CM-provider-abort indication with the APDU contained in the D-ABORT User Data parameter as the CM-provider-abort service Reason parameter value, and

- enter IDLE state.

#### D-P-ABORT Indication

##### Upon receipt of a D-P-ABORT indication, if the CM-air-ASE is not in the IDLE state the CM-air-ASE shall ([72])  :

- stop timer tlogon, if set,

- if the CM-air-user is an active user, invoke CM-provider-abort indication with the CM-provider-abort Reason parameter set to the abstract value “communication-service-failure”, and

- enter the IDLE state.

### CM-Ground-ASE Protocol Description

#### D-START Indication

##### Upon receipt of a D-START indication, if the CM-ground-ASE is in the IDLE state and the APDU contained in the D-START User Data parameter is a [CMLogonRequest] APDU then:

###### If the DS User Version Number is greater than the CM-ground-ASE version number the CM-ground-ASE shall ([73])  :

- invoke D-START response with the following:

  - the CM-ground-ASE version number as the D-START DS User Version Number parameter value,

  - the abstract value of “rejected (permanent)” as the D-START Result parameter value, and

- enter the IDLE state.

###### If the DS User Version Number is less than the CM-ground-ASE version number the CM-ground-ASE shall ([74])  :

- invoke CM-logon indication with the following:

  - the D-START Calling Peer ID parameter value as the CM-logon service Aircraft Identifier parameter value,

  - the D-START DS User Version Number parameter value as the CM-logon service CM ASE Version Number parameter value,

  - the APDU in the D-START User Data parameter as the CM-logon service Logon Request parameter value, and

- enter the LOGON state.

###### Else (the DS User Version Number is equal to CM-ground-ASE version number) the CM-ground-ASE shall ([75])  :

- invoke CM-logon indication with:

  - the D-START Calling Peer ID parameter value as the CM-logon service Aircraft Identifier parameter value,

  - the APDU in the D-START User Data parameter as the CM-logon service Logon Request parameter value, and

- enter the LOGON state

##### Upon receipt of a D-START indication, if the receiving CM-ground-ASE is in the IDLE state and the APDU contained in the D-START User Data parameter is a [CMForwardRequest] APDU then the receiving CM-ground-ASE shall ([76])  :

###### If the DS User Version Number is greater than the receiving CM-ground-ASE version number the receiving CM-ground-ASE shall ([77])  :

- invoke D-START response with the following:

  - the receiving CM-ground-ASE version number as the D-START DS User Version Number parameter value,

  - the abstract value of “rejected (permanent)” as the D-START Result parameter value, and

- enter the IDLE state.

###### If the DS User Version Number is less than the receiving CM-ground-ASE version number the receiving CM-ground-ASE shall ([78])  :

- invoke CM-forward indication with the following:

  - the D-START Calling Peer ID parameter value as the CM-forward service Calling ICAO Facility Designator parameter value,

  - the D-START DS User Version Number parameter value as the CM-forward service CM ASE Version Number parameter value,

  - the APDU in the D-START User Data parameter as the CM-forward service Forward Request parameter value, and

- invoke D-START response with the abstract value of “rejected (permanent)” as the D-START Result parameter value, and

- enter the IDLE state.

###### Else (the DS User Version Number is equal to receiving CM-ground-ASE version number) the receiving CM-ground-ASE shall ([79])  :

- invoke CM-forward indication with:

  - the D-START Calling Peer ID parameter value as the CM-forward service Calling ICAO Facility Designator parameter value,

  - the APDU in the D-START User Data parameter as the CM-forward service Forward Request parameter value, and

- invoke D-START response with the abstract value of “rejected (permanent)” as the D-START Result parameter value, and

- enter the IDLE state.

#### D-START Confirmation

##### Upon receipt of a D-START confirmation:

###### If the CM-ground-ASE is in the UPDATE state and the D-START User Data parameter is not provided, the CM-ground-ASE shall ([80])  :

- stop timer tupdate,

- if the abstract value of the D-START Result parameter is “rejected (permanent)” and the abstract value of the D-START Reject Source parameter is “DS user” the CM-ground-ASE shall  ([81])  enter the IDLE state.

###### If the CM-ground-ASE is in the CONTACT state and the APDU contained in the D-START User Data parameter is a [CMContactResponse] APDU, the CM-ground-ASE shall ([82])  :

- stop timer tcontact,

- if the abstract value of the D-START Result parameter is “rejected (permanent)” and the abstract value of the D-START Reject Source parameter is “DS user” then:

  - invoke CM-contact confirmation with the APDU in the D-START User Data parameter as the as the CM-contact Contact Response parameter, and

  - enter the IDLE state.

###### If the CM-ground-ASE is in the FORWARD state, and the D-START User Data parameter is not provided, the CM-ground-ASE shall ([83])  :

- stop timer tforward,

- if the abstract value of the D-START Result parameter is “rejected (permanent)” and the abstract value of the D-START Reject Source parameter is “DS user” then:

  - if the version number of the sending CM-ground-ASE is greater than the D-START DS User Version Number parameter value, the sending CM-ground-ASE shall  ([84])  invoke CM- forward confirmation with the DS User Version Number parameter value as the CM-forward CM-ASE Version Number parameter value, and

  - enter the IDLE state.

#### D-DATA Indication

#### Upon receipt of a D-DATA indication if the CM-ground-ASE is in the CONTACT DIALOGUE state and the APDU contained in the D-DATA User Data parameter is a [CMContactResponse] APDU, the CM-ground-ASE shall ([85])  :

- invoke CM-contact confirmation with the APDU contained in the D-DATA User Data parameter as the CM-contact Contact Response parameter value, and

- enter the DIALOGUE state.

#### D-END Confirmation

##### Upon receipt of a D-END confirmation, if the CM-ground-ASE is in the END state and the abstract values of the D-END Result is “accepted”, the CM-ground-ASE shall ([86])  :

- stop timer tend, and

- enter the IDLE state.

#### CM-logon Response

##### Upon receipt of a CM-logon response, if the CM-ground-ASE is in the LOGON state the CM-ground-ASE shall ([87])  :

- create a CMGroundMessage APDU with a cmLogonResponse APDU-element based on the CM-logon Logon Response parameter value, and

- invoke D-START response with the following:

  - the CMGroundMessage APDU as the D-START User Data parameter value, and

  - if the CM-logon Maintain Dialogue parameter is provided by the CM-ground-user

    - set the abstract value “accepted” as the D-START Result parameter value, and

    - enter the DIALOGUE state.

  - else (the CM-logon Maintain Dialogue parameter is not provided by the CM-ground-user)

    - set the abstract value “rejected (permanent)” as the D-START Result parameter value, and

    - enter the IDLE state.

#### CM-update Request

##### Upon receipt of a CM-update request, if the CM-ground-ASE is in the IDLE state, the CM-ground-ASE shall ([88])  :

- create a CMGroundMessage APDU with a cmUpdate APDU-element based on the CM-update Update Information parameter value,

- invoke D-START request with the following:

  - the CM-update Aircraft Identifier parameter value as the D-START Called Peer ID parameter value,

  - the CM-update ICAO Facility Designator parameter value as the D-START Calling Peer ID parameter value,

  - set the D-START Quality of Service parameter as follows:

    - Class of Communication from the CM-update request if provided by the CM-user as the D-START QOS Routing Class parameter value,

    - the abstract value of “flight regularity communications”, as the D-START QOS Priority parameter value, and

    - the abstract value of “low” as the D-START QOS Residual Error Rate,

  - the CMGroundMessage APDU as the D-START User Data parameter value,

- set timer tupdate, and

- enter the UPDATE state.

##### Upon receipt of a CM-update request, if the CM-ground-ASE is in the DIALOGUE state, the CM-ground-ASE shall ([89])  :

- create a CMGroundMessage APDU with a cmUpdate APDU-element based on the CM-update Update Information parameter value,

- invoke D-DATA request with the CMGroundMessage APDU as the D-START User Data parameter value, and

- remain in the DIALOGUE state.

#### CM-contact Request

##### Upon receipt of a CM-contact request, if the CM-ground-ASE is in the IDLE state the CM-ground-ASE shall ([90])  :

- create a CMGroundMessage APDU with a cmContactRequest APDU-element based on the CM-contact Contact Request parameter value,

- invoke D-START request with the following:

  - the CM-contact Aircraft Identifier parameter value as the D-START Called Peer ID parameter value,

  - the CM-contact ICAO Facility Designator parameter value as the D-START Calling Peer ID parameter value,

  - set the D-START Quality of Service parameters as follows:

    - Class of Communication from the CM-contact request if provided by the CM-user as the D-START QOS Routing Class parameter value,

    - The abstract value of “flight regularity communications”, as the D-START QOS Priority parameter value, and

    - The abstract value of “low” as the D-START QOS Residual Error Rate parameter value,

  - the CMGroundMessage APDU as the D-START User Data parameter value,

- start timer tcontact, and

- enter the CONTACT state.

##### Upon receipt of a CM-contact request, if the CM-ground-ASE is in the DIALOGUE state the CM-ground-ASE shall ([91])  :

- create a CMGroundMessage APDU with a cmContactRequest APDU-element based on the CM-contact Contact Request parameter value,

- invoke D-DATA request with the CMGroundMessage APDU as the D-START User Data parameter value, and

- enter in the CONTACT DIALOGUE state.

#### CM-end Request

##### Upon receipt of a CM-end request if the CM-ground-ASE is in the DIALOGUE state the CM-ground-ASE shall ([92])  :

- invoke D-END request, and

- enter the END state.

#### CM-forward Request

##### Upon receipt of a CM-forward request, if the CM-ground-ASE is in the IDLE state, the CM-ground-ASE shall ([93])  :

- create a CMGroundMessage APDU with a cmForwardRequest APDU-element based on the CM-forward service Forward Request parameter value,

- invoke D-START request with the following:

  - the CM-forward Called ICAO Facility Designator parameter value as the D-START Called Peer ID parameter value,

  - the CM-forward Calling ICAO Facility Designator parameter value as the D-START Calling Peer ID parameter value,

  - the sending CM-ground-ASE version number as the D-START DS User Version Number parameter value,

  - set the D-START Quality of Service parameter as follows:

    - Class of Communication from the CM-forward request if provided by the CM-user as the D-START QOS Routing Class parameter value,

    - the abstract value of “flight regularity communications” as the D-START QOS Priority parameter value, and

    - the abstract value of “low” as the D-START QOS Residual Error Rate,

  - the CMForwardMessage APDU as the D-START User Data parameter value,

- set timer tforward, and

- enter the FORWARD state.

#### CM-user-abort Request

##### Upon receipt of a CM-user-abort request, if the CM-ground-ASE is not in the IDLE state the CM-ground-ASE shall ([94])  :

- stop any timer, if set,

- invoke D-ABORT request with the D-ABORT Originator parameter set to the abstract value “user”, and

- enter the IDLE state.

#### D-ABORT Indication

###### Upon receipt of a D-ABORT indication, if the CM-ground-ASE is not in the IDLE state the CM-ground-ASE shall ([95])  :

- stop any timer, if set,

- if the CM-ground-user is an active user, then:

  - if the D-ABORT Originator parameter contains the abstract value “user” invoke CM-user-abort indication,

  - else invoke CM-provider-abort indication with the APDU contained in the D-ABORT User Data parameter as the CM-provider-abort service Reason parameter value, and

- enter IDLE state.

#### D-P-ABORT Indication

##### Upon receipt of a D-P-ABORT indication, if the CM-ground-ASE is in not in the IDLE state the CM-ground-ASE shall ([96])  :

- stop any timer, if set,

- if the CM-ground-user is an active user, invoke CM-provider-abort indication with the CM-provider-abort Reason parameter set to the abstract value “communication-service-failure”, and

- enter the IDLE state.

## Exception Handling

### Timer Expiration

#### If any timer expires, the CM-ASE shall ([97])  :

- interrupt any current activity,

- if the CM-ASE is a CM-air-ASE, create a CMAircraftMessage APDU with a cmAbortReason [timer-expired] APDU message element,

- if the CM-ASE is a CM-ground-ASE, create a CMGroundMessage APDU with a cmAbortReason [timer-expired] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value,

  - the APDU as the D-ABORT User Data parameter value, and

- if the CM-user is an active user, invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

### Unrecoverable System Error

#### If a CM-ASE has an unrecoverable system error, the CM-ASE shall ([98])  :

- if the CM-ASE is a CM-air-ASE, create a CMAircraftMessage APDU with a cmAbortReason [undefined-error] APDU message element,

- if the CM-ASE is a CM-ground-ASE, create a CMGroundMessage APDU with a cmAbortReason [undefined-error] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value,

  - the APDU as the D-ABORT User Data parameter value, and

- if the CM-user is an active user, invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

### Invalid PDU

#### If the User Data parameter of a D-START or D-DATA indication does not contain a valid PDU, the CM-ASE shall ([99])  :

- if the CM-ASE is a CM-air-ASE, create a CMAircraftMessage APDU with a cmAbortReason [invalid-PDU] APDU message element,

- if the CM-ASE is a CM-ground-ASE, create a CMGroundMessage APDU with a cmAbortReason [invalid-PDU] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value,

  - the APDU as the D-ABORT User Data parameter value, and

- if the CM-user is an active user, invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

#### If the User Data parameter of a D-START confirmation does not contain a valid PDU, the CM-ASE shall ([100])  :

- if the CM-ASE is a CM-air-ASE, create a CMAircraftMessage APDU with a cmAbortReason [invalid-PDU] APDU message element,

- if the CM-ASE is a CM-ground-ASE, create a CMGroundMessage APDU with a cmAbortReason [invalid-PDU] APDU message element,

- if the D-START Result parameter is set to the abstract value “accepted”, invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value,

  - the APDU as the D-ABORT User Data parameter value, and

- if the CM-user is an active user, invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

### Not permitted PDU

#### If the User Data parameter of a D-START or D-DATA indication is a valid PDU, but there are no instructions in 5.3 (i.e., not a permitted PDU) the CM-ASE shall ([101])  :

- if the CM-ASE is a CM-air-ASE, create a CMAircraftMessage APDU with a cmAbortReason [not-permitted-PDU] APDU message element,

- if the CM-ASE is a CM-ground-ASE, create a CMGroundMessage APDU with a cmAbortReason [not-permitted-PDU] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value,

  - the APDU as the D-ABORT User Data parameter value, and

- if the CM-user is an active user, invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

#### If the User Data parameter of a D-START confirmation is a valid PDU, but there are no instructions in 5.3 (i.e., not a permitted PDU) the CM-ASE shall ([102])  :

- if the CM-ASE is a CM-air-ASE, create a CMAircraftMessage APDU with a cmAbortReason [not-permitted-PDU] APDU message element,

- if the CM-ASE is a CM-ground-ASE, create a CMGroundMessage APDU with a cmAbortReason [not-permitted-PDU] APDU message element,

- if the D-START Result parameter is set to the abstract value “accepted”, invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value,

  - the APDU as the D-ABORT User Data parameter value, and

- if the CM-user is an active user, invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

### D-START Confirmation Result or Reject Source Parameter Values Not as Expected

#### If the CM-ground-ASE receives a D-START confirmation with the D-START Result parameter having the abstract value of “accepted”, the CM-ground-ASE shall ([103])  :

- create a CMGroundMessage APDU with a cmAbortReason [dialogue-acceptance-not-permitted] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value,

  - the APDU as the D-ABORT User Data parameter value, and

- if the CM-ground-user is an active user, invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

#### If the CM-ASE receives a D-START confirmation with the D-START Result parameter having the abstract value of “rejected (transient)” or if the D-START Reject Source parameter has the abstract value of “DS provider”, and if the CM-user is an active user, the CM-ASE shall ([104])  :

- if the CM-ASE is a CM-air-ASE, create a CMAircraftMessage APDU with a cmAbortReason [communication-service-error] APDU message element,

- if the CM-ASE is a CM-ground-ASE, create a CMGroundMessage APDU with a cmAbortReason [communication-service-error] APDU message element, and

- invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

### D-END Confirmation Not as Expected

#### If the CM-ground-ASE receives a D-END confirmation with the D-END Result parameter that does not have the abstract value of “accepted”, the CM-ground-ASE shall ([105])  :

- create a CMGroundMessage APDU with a cmAbortReason [dialogue-end-not-accepted] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value,

  - the APDU as the D-ABORT User Data parameter value, and

- if the CM-ground-user is an active user, invoke CM-provider-abort indication with the APDU as the CM-provider-abort Reason parameter value.

## CM ASE StateTables

### Introduction

- Note: — This section defines the state tables for the CM-air-ASE and the CM-ground-ASE.  If the state tables shown in this section conflict with the textual statements made elsewhere in this SARPs, the textual statements take precedence.

### CM-Ground-ASE State Table

- Note: — Table 5-2 reflects the possible states of the CM-Ground-ASE.

| STATE Þ EVENT ß | IDLE | LOGON | UPDATE | CONTACT | DIALOGUE | CONTACT DIALOGUE | END | FORWARD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DIALOGUE Service Events |  |  |  |  |  |  |  |  |
| D-START Indication Version Number is greater than the CM-ground-ASE version number | - D-START response - IDLE | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | - D-START response - IDLE |
| D-START Indication Version Number is less than or equal to than the CM-ground-ASE version number, User Data = CMLogonRequest | - CM-logon indication - LOGON | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Indication Version Number is less than or equal to than the CM-ground-ASE version number, User Data = CMForwardRequest | - CM-forward indication - D-START response - IDLE | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Confirmation Result “rejected (permanent)” and Reject Source “DS user” | cannot occur | cannot occur | - Stop timer tupdate - IDLE | - Stop timer tcontact - CM-contact confirmation - IDLE | cannot occur | cannot occur | cannot occur | - Stop timer tforward - If sending ASE version > receiving ASE then CM-forward conf - IDLE |
| - D-DATA Indication | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | - CM contact confirmation - DIALOGUE | cannot occur | cannot occur |
| D-END Confirmation Result “accepted” | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | - stop timer tend - IDLE | cannot occur |
| CM-User Events |  |  |  |  |  |  |  |  |
| CM-update Request | - D-START request - UPDATE | not permitted | not permitted | not permitted | - D-DATA request - DIALOGUE | not permitted | not permitted | not permitted |
| CM-contact Request | - D-START request - CONTACT | not permitted | not permitted | not permitted | - D-DATA request - CONTACT DIALOGUE | not permitted | not permitted | not permitted |
| CM-forward Request | - D-START request - FORWARD | not permitted | not permitted | not permitted | not permitted | not permitted | not permitted | not permitted |
| CM-logon Response Maintain Dialogue not supplied by CM-ground user | not permitted | - D-START response - IDLE | not permitted | not permitted | not permitted | not permitted | not permitted | not permitted |
| CM-logon Response Maintain Dialogue “accepted” | not permitted | - D-START response - DIALOGUE | not permitted | not permitted | not permitted | not permitted | not permitted | not permitted |
| CM-end Request | not permitted | not permitted | not permitted | not permitted | - D-END request - Start timer tend - END | not permitted | not permitted | not permitted |
| ABORT Events |  |  |  |  |  |  |  |  |
| CM-user-abort Request | not permitted | - D-ABORT request - IDLE | - stop timer tupdate - D-ABORT request - IDLE | - stop timer tcontact - D-ABORT request - IDLE | - D-ABORT request - IDLE | - D-ABORT request - IDLE | - stop timer tend - D-ABORT request - IDLE | - stop timer tforward - D-ABORT request - IDLE |
| - D-ABORT Indication Originator is “provider” | cannot occur | - CM-provider-abort indication - IDLE | - stop timer tupdate - CM-provider-abort indication - IDLE | - stop timer tcontact - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE | - stop timer tend - CM-provider-ABORT indication - IDLE | - stop timer tforward - CM-provider-ABORT indication - IDLE |
| - D-ABORT Indication Originator is “user” | cannot occur | - CM-user-abort indication - IDLE | - stop timer tupdate - CM-user-abort indication - IDLE | - stop timer tcontact - CM-user-abort indication - IDLE | - CM-user-abort indication - IDLE | - CM-user-abort indication - IDLE | - stop timer tend - CM-user-ABORT indication - IDLE | - stop timer tforward - CM-user-ABORT indication - IDLE |
| - D-P-ABORT indication | cannot occur | - CM-provider-abort indication - IDLE | - stop timer tupdate - CM-provider-abort indication - IDLE | - stop timer tcontact - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE | - stop timer tend - CM-provider-abort indication - IDLE | - stop timer tforward - CM-provider-abort indication - IDLE |
| - Tupdate Expires | cannot occur | cannot occur | - D-ABORT request - CM-provider-abort indication - IDLE | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| Tcontact Expires | cannot occur | cannot occur | cannot occur | - D-ABORT request - CM-provider-abort indication - IDLE | cannot occur | cannot occur | cannot occur | cannot occur |
| Tend Expires | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | - D-ABORT request - CM-provider-abort indication - IDLE | cannot occur |
| Tforward Expires | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur | - D-ABORT request - CM-provider-abort indication - IDLE |

Table 5-2:  CM-Ground-ASE

### CM-Air-ASE State Table

- Note: — Table 5-3 reflects the possible states of the CM-Air-ASE.

| STATE Þ EVENT ß | IDLE | LOGON | CONTACT | DIALOGUE | CONTACT DIALOGUE |
| --- | --- | --- | --- | --- | --- |
| DIALOGUE Service Events |  |  |  |  |  |
| D-START Indication User Data CMUpdate | - CM-update indication - D-START response - IDLE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Indication User Data CMContactRequest | - CM-contact indication - CONTACT | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Confirmation Result “rejected (permanent)” and Reject Source “DS user” | cannot occur | - CM-logon confirmation - IDLE | cannot occur | cannot occur | cannot occur |
| D-START Confirmation Result “accepted” | cannot occur | - CM-logon confirmation - DIALOGUE | cannot occur | cannot occur | cannot occur |
| D-DATA Indication User Data CMUpdate | cannot occur | cannot occur | cannot occur | - CM-update indication - DIALOGUE | cannot occur |
| D-DATA Indication User Data CMContactRequest | cannot occur | cannot occur | cannot occur | - CM-contact indication - CONTACT DIALOGUE | cannot occur |
| D-END Indication | cannot occur | cannot occur | cannot occur | - CM-end indication - D-END response - IDLE | cannot occur |
| CM-User Events |  |  |  |  |  |
| CM-contact Response | not permitted | not permitted | - D-START response - IDLE | not permitted | - D-DATA request - DIALOGUE |
| - CM-logon Request | - D-START request - Start timer tlogon - LOGON | not permitted | not permitted | not permitted | not permitted |
| ABORT Events |  |  |  |  |  |
| CM-user-abort Request | not permitted | - stop timer tlogon - D-ABORT request - IDLE | - D-ABORT request - IDLE | - D-ABORT request - IDLE | - D-ABORT request - IDLE |
| - D- ABORT Indication Originator is “provider” | cannot occur | - stop timer tlogon - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE |
| - D-ABORT Indication Originator is “user” | cannot occur | - stop timer tlogon - CM-user-abort indication - IDLE | - CM-user-abort indication - IDLE | - CM-user-abort indication - IDLE | - CM-user-abort indication - IDLE |
| - D-P-ABORT indication | cannot occur | - stop timer tlogon - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE | - CM-provider-abort indication - IDLE |
| - Tlogon Expires | cannot occur | - D-ABORT request - CM-provider-abort indication - IDLE | cannot occur | cannot occur | cannot occur |

Table 5-3:  CM-Air-ASE

# COMMUNICATION REQUIREMENTS

## Encoding Rules

### The CM application shall  ([106])  use PER encoding as defined in reference [1], using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in part 4, or a functionally equivalent means which provides the same result.

## Dialogue Service Requirements

### Primitive Requirements

#### Where dialogue service primitives, that is D-START, D-DATA, D-END, D-ABORT, and D-P-ABORT are described as being invoked in chapter 5, the CM-ground-ASE and the CM-air-ASE shall  ([107])  exhibit external behavior consistent with the dialogue service, as described in reference [4], having been implemented and its primitives invoked.

### ATN Quality-of-Service Requirements

#### The application service priority for CM shall  ([108])  be abstract value of “flight regularity communications”.

#### The RER Quality of Service Parameter of the D-START shall  ([109])  be set to the abstract value of “low”.

- Note 1: — The application service residual error rate (RER) for CM is required to be 10-6 or better.

- Note 2: — The RER takes into account non-delivery for messages over a given dialogue.

#### Routing Policy

##### The CM-ASE shall  ([110])  map the CM-start service Class of Communication abstract values to the Routing Class abstract value part of the D-START QOS parameter as presented in Table 6-1.

| Class of Communication Abstract Value | Routing Class Abstract Value |
| --- | --- |
| A | Traffic only follows Class A ATSC route(s) |
| B | Traffic only follows Class B ATSC route(s) |
| C | Traffic only follows Class C ATSC route(s) |
| D | Traffic only follows Class D ATSC route(s) |
| E | Traffic only follows Class E ATSC route(s) |
| F | Traffic only follows Class F ATSC route(s) |
| G | Traffic only follows Class G ATSC route(s) |
| H | Traffic only follows Class H ATSC route(s) |

Table 6-1:  Mapping Between Class of Communication and Routing Class Abstract Values

- Note: — ATSC values are defined in [5].

## CM AE Control Functions Requirements

The title of the CM application used as the AE-Qualifier for the CM application shall  ([111])  be the IA5 character string “CMA”.

# CM USER REQUIREMENTS

## CM-Air-User Requirements

### The ASE Invocation/Aircraft Identifier Association

#### For each CM-air-ASE invocation, the CM-air-user shall  ([112])  establish an association between a CM-air-ASE invocation and the ICAO facility designator.

#### Upon the initiation of a CM-logon request, or upon receipt of a CM-update indication or a  CM-contact indication the ASE invocation association shall  ([113])  be based on the ICAO facility designator in the ICAO Facility Designator primitive of the respective CM service.

#### The association shall  ([114])  be maintained for the duration of the ASE invocation.

### CM-logon Service Requirements

### Only the CM-air-user shall  ([115])  be permitted to initiate the CM-logon service.

#### When invoking the CM-logon request the CM-air-user shall  ([116])  provide the following as part of the CMLogonRequest:

- its CM name, long TSAP, and version number,

- information on each application for which it requires a data link service as follows:

  - for air-only initiated services: application name and version number,

  - for applications that can be ground initiated: application name, version number, and address; and

- flight information data as required by the addressed ground system.

#### When invoking the CM-logon request, if any RDP for a given application address is different than the CM RDP, the CM-air-user shall  ([117])  use the long TSAP for each application address provided.

#### When invoking the CM-logon request the CM-air-user shall  ([118])  provide all of the versions and addresses, as required for ground-initiated applications, for each application the aircraft is capable of supporting.

#### Upon receipt of a CM-logon confirmation, the CM-air-user shall  ([119])  make the information contained in the CMLogonResponse available to the other CM-air applications (i.e., ADS, CPDLC, and FIS), as well as to the dialogue service.

#### Upon the receipt of a CMLogonResponse from a CM-logon confirmation from an ICAO ground facility designator for which CM information has previously been received, the CM-air-user shall  ([120])  only replace the previous information for which new logon information has been received.

### CM-update Service Requirements

#### Upon the receipt of Update Information from a CM-update indication from an ICAO ground facility designator for which CM information has previously been received, the CM-air-user shall  ([121])  only replace the previous information for which updated information has been received.

#### The CM-air-user shall  ([122])  make the updated information contained in the Update Information available to the other CM-air applications (i.e., ADS, CPDLC, and FIS), as well as to the dialogue service.

### CM-contact Service Requirements

#### Upon receipt of a CM-contact indication the CM-air-user shall  ([123])  attempt to initiate a CM-logon request with the indicated ICAO facility designator.

#### If a CM-logon service request is initiated, the CM-air-user shall  ([124])  comply with the CM-logon requirements as stated above.

#### In addition to the above CM-logon requirements, upon receipt of a CM-logon response from the indicated ICAO facility designator, or if no CM-logon service request can be initiated, the CM-air-user shall  ([125])  invoke the CM-contact response indicating the success or lack thereof of the CM-logon request.

## CM-Ground-User Requirements

### The ASE Invocation/Aircraft Identifier Association

#### For each CM-ground-ASE invocation, the CM-ground-user shall  ([126])  establish an association between a CM-ground-ASE invocation and the aircraft 24 bit address.

#### Upon the initiation a CM-update request or CM-contact request, or upon receipt of a CM-logon indication the ASE invocation association shall  ([127])  be based on the 24-bit aircraft identification in the Aircraft Identifier primitive of the respective CM service.

#### The association shall  ([128])  be maintained for the duration of the ASE invocation.

### CM-logon Service Requirements

#### Upon receipt of a CM-logon indication, the CM-ground-user shall  ([129])  make the aircraft application information contained in the Logon Request available to the other CM-ground applications (i.e., ADS, CPDLC, and FIS), as well as to the dialogue service.

#### Upon receipt of a CM-logon indication, the CM-ground-user shall  ([130])  create the actual TSAP for each aircraft application information contained in the Logon Request based on the IDP and long TSAP for each application as defined in Chapter 4 of this sub-document.

- Note: — The actual TSAP is composed of the IDP and the long TSAP as in Chapter 4.  The long TSAP = RDP + short TSAP.  The short TSAP = ARS + LOC + SYS + SEL.

#### Upon the receipt of a Logon Request from a CM-logon indication from a aircraft identifier for which CM information has previously been received and still being maintained, the CM-ground-user shall  ([131])  update the aircraft information accordingly.

#### Upon receipt of a CM-logon indication, the CM-ground-user shall  ([132])  invoke a CM-logon response with a CMLogonResponse containing:

- an indication of whether or not the CM logon was successful,

- application names, addresses, and version numbers for the requested applications that can be air-initiated that the ground system can support, and

- applications names and version numbers for the requested ground-only initiated applications that the ground system can support.

#### When invoking the CM-logon response, if any RDP for a given application address is different than the CM RDP, the CM-ground-user shall  ([133])  use the long TSAP for each application address provided.

#### When invoking the CM-logon response the CM-ground-user shall  ([134])  provide all of the versions for each application the ground is capable of supporting.

#### When a CM-ground-user cannot support any of the air or ground applications requested the CM-ground-user shall  ([135])  indicate that the logon was unsuccessful.

#### If the CM-ground-user wishes for a CM dialogue to be maintained the CM-ground-user shall  ([136])  set the CM-logon response Maintain Dialogue parameter.

### CM-update Service Requirements

#### Only the CM-ground-user shall  ([137])  be permitted to initiate the CM-update-service.

#### When invoking the CM-update service request, the CM-ground-user shall  ([138])  provide a CMUpdate containing application names, addresses, and version numbers  for each of the data link applications it wishes to update.

### CM-contact Service Requirements

#### Only the CM-ground-user shall  ([139])  be permitted to initiate the CM-contact-service.

#### If a CM-dialogue exists when the CM-ground-user invokes the CM-contact request, the CM-ground user shall  ([140])  be prohibited from requesting any other CM service primitive, except the CM-user-abort service request primitive, until it receives the CM-contact confirmation.

#### When invoking the CM-contact request, the CM-ground-user shall  ([141])  provide a CMContactRequest containing the ICAO facility designator of the ground facility that the ground requests the aircraft to contact.

### CM-end Service Requirements

#### Only the CM-ground-user shall  ([142])  be permitted to initiate the CM-end-service.

#### If the CM-ground-user establishes a CM dialogue with the CM-logon Maintain Dialogue parameter set, the CM-ground user shall  ([143])  be responsible for closing the CM dialogue with the CM-end service.

### CM-forward Service Requirements

#### Only the CM-ground-user shall  ([144])  be permitted to initiate the CM-forward-service.

#### When requesting the CM-forward service the CM-ground-user shall  ([145])  provide all of the information from either a CM-logon request message or a CM-forward request message, whichever is the more recent.

#### Upon receipt of a CM-forward indication, the CM-ground-user shall  ([146])  make the aircraft application information contained in the Forward Request available to the other CM-ground applications (i.e., ADS, CPDLC, and FIS), as well as to the dialogue service.

#### Upon receipt of a CM- forward indication, the CM-ground-user shall  ([147])  create the actual TSAP for each aircraft application information contained in the Forward Request based on the IDP and long TSAP for each application as defined in Chapter 4 of this sub-document.

- Note: — The actual TSAP is composed of the IDP and the long TSAP as in Chapter 4.  The long TSAP = RDP + short TSAP.  The short TSAP = ARS + LOC + SYS + SEL.

#### Upon the receipt of a Forward Request from a CM-forward indication concerning an aircraft identifier for which CM information has previously been received and is still being maintained, the CM-ground-user shall  ([148])  update the aircraft information accordingly.

#### Upon the receipt of a Forward Request from a CM-forward indication containing air-only initiated applications that are supported by the receiving CM-ground-user, the receiving CM-ground-user shall  ([149])  invoke a CM-update request containing the requested application addresses.

| ServiceÞ Functionß | CM-logon | CM-update | CM-contact | CM-end | CM-forward |
| --- | --- | --- | --- | --- | --- |
| Logon | Ö |  |  | Ö |  |
| Update |  | Ö |  | Ö |  |
| Contact |  |  | Ö | Ö |  |
| Forward |  |  |  |  | Ö |
| Registration | Ö |  |  |  | Ö |