|  | ATNP/WG3/SG2 (Air/Ground Application) Version 2.0 15th February 1996 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Sub Group 2 (Air/Ground Applications)

Draft CPDLC SARPs for the ATN CNS/ATM-1 Package

Part 3: CPDLC Application SARPs

Prepared by:		Air/Ground Applications Subgroup

SUMMARY

This document is the draft of the Controller/Pilot Data Link Communications  application SARPs for the ATN CNS/ATM-1 Package.

CONFIGURATION SHEET

Title : Draft CPDLC SARPs for the ATN CNS/ATM-1 Package

Version :		2.0 Draft

Date:			15th February 1996

Contact:		Airservices Australia, Eurocontrol, Federal Aviation Administration, Direction Generale de l’Aviation Civile

Status:			Draft

Change History :

| Version | Description | Affected Parts | Date |
| --- | --- | --- | --- |
| 1.0 | Banff Proposed Draft CPDLC SARPs |  | 6 Oct. 1995 |
| 1.1 | Input to ATN/WG3 at Brisbane |  | 5 Feb. 1996 |
|  | Chapter 1 |  |  |
|  | - addition of DSC functionality | §1.1.4, §1.2.2, §1.3.3 |  |
|  | - clarifications of AE/ASE definitions | §1.1.4 |  |
|  | Chapter 2 |  |  |
|  | - Time accuracy clarified | §2.2.1 |  |
|  | Chapter 3 |  |  |
|  | - DSC changes |  |  |
|  | - DSC-start service | §3.3.1, new §3.8 |  |
|  | - DSC-end service | §3.3.1, new §3.9 |  |
|  | Chapter 4 |  |  |
|  | - DSC addition - high level ASN.1 change to permit DSC mode (DownlinkPDUs definition), Mode, and StartdownMessage | §4.1 |  |
|  | Chapter 5 |  |  |
|  | - DSC addition |  |  |
|  | - additional time sequence diagrams | §5.1.4, §5.17 |  |
|  | - addition of timer for DSC | §5.2.2.1 |  |
|  | - addition of DSC mode | §5.3.1.4 |  |
|  | - modifications to D-START confirmation | §5.3.3.1, §5.3.3.2 |  |
|  | - modifications to D-START indication | §5.3.2.1, §5.3.2.3 |  |
|  | - modifications to D-END indication | §5.3.5.2 |  |
|  | - modifications to D-END confirmation | §5.3.6.2 |  |
|  | - modifications to CPDLC-start (mode) | §5.3.7.1.a |  |
|  | - addition of DSC-start request | §5.3.9 |  |
|  | - addition of DSC-start response | §5.3.10 |  |
|  | - addition of DSC-end request | §5.3.13 |  |
|  | - addition of DSC-end response | §5.3.15 |  |
|  | - Change from D-U-ABORT to D-ABORT plus parameters | § |  |
|  | - D-ABORT modified sequence diagrams | §5.1.9, §5.1.10 |  |
|  | - modified user-abort | §5.3.17 |  |
|  | - modified D-ABORT | §5.3.18 |  |
|  | - Due to modified D-ABORT (new name, new variables) all of section 5.4 was deleted and rewritten. | §5.4 |  |
|  | Chapter 6 |  |  |
|  | - Addition of note for ATSC value references | §6.2.2.3.1 |  |
|  | Chapter 7 |  |  |
|  | - DSC modifications | §7.1.3.1 |  |
|  | - freeing of message identification | §7.2.3.3 |  |
|  | - CPDLC/DSC distinction | §7.3.3 |  |
|  | - CPDLC-start service modifications | §7.4.2.1 |  |
|  | - addition of DSC-start service section | §7.4.5, §7.5.5 |  |
|  | - modification of CPDLC-message indication |  |  |
|  | - addition of DSC-end service | §7.4.10, §7.5.8 |  |
|  | - Logical Acknowledgment both ways | §7.3.5, |  |
|  | - Initiation/closure manual/data link | §7.3.7 notes |  |
|  | - Clarification of Response requirements | §7.3.7 |  |
|  | - receipt of start when a connection is assumed (one side has crashed and other side is not yet aware) | §7.4.3.1, §7.4.3.2, §7.5.3.1 |  |
|  | Appendix A |  |  |
|  | - State tables updated for DSC - State tables updated for D-ABORT Change | §A |  |
| 2.0 | Output from Brisbane |  | 15 Feb. 1996 |
|  | Chapter 1 |  |  |
|  | - addition of ground forward capability | §1.2.2.1, §1.3.4 |  |
|  | - modified header for ground/ground | §1.4.2, §1.4.3 |  |
|  | - Changed reference of traffic type values to “[5]” | §1.1.5.1 |  |
|  | - modification to message response tables from ADSP San Francisco | §1.4.6.5 |  |
|  | - removal of appendix references | §1.3.3, §1.4.6.6 |  |
|  | Chapter 2 |  |  |
|  | - Timing requirements clarified | §2.2 |  |
|  | Chapter 3 |  |  |
|  | - Change of ICAO Facility Designator from 4 to 8 characters | §3.5.2.2, §3.5.3.1, §3.6.2.1 |  |
|  | - Insertion of reference [5] for Class of Communications | §3.5.7.1, §3.6.7.1 |  |
|  | - Addition of CPDLC-forward service | §3.3.1, new §3.10 |  |
|  | Chapter 4 |  |  |
|  | - Automatic tagging/removing of tags in ASN.1 |  |  |
|  | - Alphabetizing ASN.1 |  |  |
|  | - Addition of forward service |  |  |
|  | - Renaming DownlinkPDUs to AircraftPDUs, UplinkPUs to GroundPDUs | §4.1 |  |
|  | - Addition of ATCForwardMessage, ForwardHeader, and ForwardMessage ASN.1 types | §4.1 |  |
|  | - ICAO facility designator modified from 4 character to 8 | §4.1 |  |
|  | - Change of Pre-departure clearance to departure clearance | §4.1 |  |
|  | - Additional message variables (clearance type, speed type) | §4.1 |  |
|  | Chapter 5 |  |  |
|  | - Addition of CPDLC-forward service |  |  |
|  | - New sequence diagrams for forward | §5.1.8, §5.1.11, §5.1.12 |  |
|  | - New timers for forward | §5.2.2.1 |  |
|  | - Modifications to Protocol Descriptions |  |  |
|  | - New CPDLC forward request | §5.3.16 |  |
|  | - Modifications to D-START Indication | §5.3.2.2 |  |
|  | - Modifications to D-START Confirmation | §5.3.3.3 |  |
|  | - Modifications to state tables | §5.5 |  |
|  | - General re-ordering of section 5.3, so the dialogue service primitives are defined first followed by the CPDLC service primitives (for both air and ground protocol descriptions) | §5.3 |  |
|  | - State tables moved from Appendix A to section 5.5 | §5.5 |  |
|  | Chapter 6 |  |  |
|  | - NULL removal from QOS table | §6.2.2.3.1, table 6-1 |  |
|  | - Addition of RER clarification (Note 2) | §6.2.2.3.1 |  |
|  | Chapter 7 |  |  |
|  | - Modifications for forward function | § |  |
|  | - differentiation of ground/ground from air/ground messages | §7.3.4 |  |
|  | - differentiation of air/ground ground/ground message headers | §7.2.1, §7.2.2 |  |
|  | - differing air/ground ground/ground response requirements | §7.3.7.19 |  |
|  | - addition of sections for forward service |  |  |
|  | - CPDLC-start service modification | §7.5.2.1 |  |
|  | - new forward service section | §7.5.10 |  |
|  | - Duplicate message id causes abort not error | §7.3.8.1.1 |  |
|  | - Clarification of YES response requirements | §7.3.7.13 |  |
|  | - Moving of Appendix B to Chapter 7 | §7.6 |  |
|  | Appendix A |  |  |
|  | - State table moved to Chapter 5; appendix deleted | §A |  |
|  | Appendix B |  |  |
|  | - Message Descriptions Moved to section 7.6, appendix B deleted | §B |  |

Table Of Contents

1. APPLICATION OVERVIEW

1.1 Introduction

1.2 Application Functionality

1.3 Functional Descriptions

1.4 A CPDLC Message

2. GENERAL REQUIREMENTS

2.1 Performance Requirements

2.2 Time Accuracy Requirements

2.3 Security Requirements

2.4 Backwards Compatibility Requirements

2.5 Error Processing Requirements

3. THE ABSTRACT SERVICE

3.1 Introduction

3.2 The CPDLC Functional Model

3.3 The CPDLC-ASE Abstract Service

3.4 Conventions

3.5 CPDLC-start Service

3.6 DSC-start Service

3.7 CPDLC-message Service

3.8 CPDLC-end Service

3.9 DSC-end Service

3.10 CPDLC-forward Service

3.11 CPDLC-user-abort Service

3.12 CPDLC-provider-abort Service

4. FORMAL DEFINITIONS OF MESSAGES

4.1 ASN.1

5. PROTOCOL DEFINITION

5.1 Sequence Rules

5.2 CPDLC Service Provider Timers

5.3 CPDLC ASE Protocol Description

5.4 Exception Handling

5.5 CPDLC ASE State Tables

6. COMMUNICATION REQUIREMENTS

6.1 Encoding Rules

6.2 Dialogue Service Requirements

6.3 CPDLC AE Control Functions Requirements

7. CPDLC USER REQUIREMENTS

7.1 Introduction

7.2 CPDLC Message Generation Requirements

7.3 CPDLC Message Receipt Requirements

7.4 CPDLC-air-user Requirements

7.5 CPDLC-Ground-User Requirements

7.6 Message Intent

# APPLICATION OVERVIEW

## Introduction

### Purpose

#### This document contains draft Standards and Recommended Practices (SARPs) for the Air Traffic Control (ATC) Controller Pilot Data Link Communication (CPDLC) application.  CPDLC addressing requirements are supported by the Context Management (CM) application.  The Aeronautical Telecommunication Network (ATN) provides the media and protocols to conduct CPDLC.

#### Compliance with these standards is a means of assuring that the ATC system will perform its intended functions using data link, and that the CPDLC application is implemented in a globally uniform and interoperable manner.

### Background

#### The CPDLC application provides the capability to establish, manage, and terminate CPDLC dialogues between ATC ground and aircraft system peers via the ATN.  Once a dialogue is established, CPDLC provides for controller/pilot message exchange.

#### The CPDLC application also provides the capability to establish, manage, and terminate CPDLC dialogues between two ATC ground system peers via the ATN for the purpose of ground/ground forwarding of a CPDLC message.

#### In the performance of this role, the CPDLC application interacts with the following:

- Aeronautical Telecommunication Network (ATN),

- Context Management (CM) Application,

- The ground ATC system, and

- The aircraft avionics.

#### The CPDLC application provides air-ground and ground-ground data communications for ATC service.  The CNS/ATM-1 service defined in this document includes a set of clearance/information/request message elements which correspond to existing phraseology employed by current Air Traffic Control procedures.  The controller is provided with the capability to issue altitude assignments, crossing constraints, lateral deviations, route changes and clearances, speed assignments, radio frequency assignments, and various requests for information, as well as forward CPDLC messages to another controller.  The pilot is provided with the capability to respond to messages, to request clearances and information, to report information, and to declare/rescind an emergency.  A "free text" capability is also provided to exchange information not conforming to defined formats.

#### Controllers and pilots will use data link services to augment the existing voice communication.  It is expected to be used for routine or frequent types of transactions.  Although initial implementation is intended to conform to existing procedures, it is anticipated that future evolution of the system and procedures will result in the greater automation of functions for both aircraft and ground systems.

### Structure of Document

#### Chapter 1:  APPLICATION OVERVIEW contains the document’s purpose and structure, a summary of the ADSP operational requirements (ORs) that relate to CPDLC and maps these ORs to the functions of CPDLC.

#### Chapter 2:  GENERAL REQUIREMENTS contains performance, time accuracy, security, backwards compatibility, and error processing requirements.

#### Chapter 3:  ABSTRACT SERVICE DEFINITION contains the description of the abstract service provided by the CPDLC Application Service Element (CPDLC-ASE).

#### Chapter 4:  FORMAL DEFINITION OF MESSAGES contains the formal definition of messages exchanged by CPDLC ASEs using Abstract Syntax Notation Number One (ASN.1).

#### Chapter 5: PROTOCOL DEFINITION describes the exchanges of messages allowed by the CPDLC protocol, as well as time constraints and CPDLC-ASE protocol descriptions and state tables.

#### Chapter 6:  COMMUNICATION REQUIREMENTS  contains the requirements that the CPDLC application imposes on the underlying communication system.

#### Chapter 7 : CPDLC USER REQUIREMENTS contains requirements imposed on the user of the CPDLC ASE service and message description tables.

### Explanation of Terms

#### Acronyms

##### The following abbreviations are used in this specification:

| ADS | Automatic Dependent Surveillance |
| --- | --- |
| ADSP | Automatic Dependent Surveillance Panel |
| AE | Application Entity |
| APDU | Application Protocol Data Unit |
| ASE | Application Service Element |
| ASN.1 | Abstract Syntax Notation One |
| ATC | Air Traffic Control |
| ATM | Air Traffic Management |
| ATN | Aeronautical Telecommunication Network |
| ATS | Air Traffic Service |
| ATSC | Air Traffic Services Communication |
| CF | Control Function |
| CM | Context Management |
| CNS | Communication Navigation Surveillance |
| CNS/ATM | Communications Navigation Surveillance / Air Traffic Management |
| CPC | Controller Pilot Communications |
| CPDLC | Controller Pilot Data Link Communications |
| DS | Dialogue Service |
| DSC | Down Stream Clearance |
| FD | Functional Description |
| FDPS | Flight Data Processing System |
| IA5 | International Alphabet #5 |
| ICAO | International Civil Aviation Organization |
| ID | Identification |
| IEC | International Electrotechnical Committee |
| IS | International Standard |
| ISO | International Organization for Standardization |
| ITU | International Telecommunications Union |
| OR | Operational Requirement |
| PDU | Protocol Data Unit |
| PER | Packed Encoding Rules |
| QOS | Quality of Service |
| RCP | Required Communication Performance |
| RER | Residual Error Rate |
| SARPs | Standards and Recommended Practices |
| UTC | Coordinated Universal Time |

#### General Definitions

##### For the purpose of this document, the following definitions apply:

- Active user:  the user is currently involved in a CPDLC dialogue.

- Application Entity:  a model of those aspects of an application process that are significant from the viewpoint of accessing OSI capabilities.

- Application Process:  an element within an open system which performs information processing tasks for a particular application.

- AE Qualifier:  that part of the AE title that uniquely identifies the particular application entity.

- AE Title:  a unique name for an application entity.

- APDU:  basic unit of information exchanged between two CPDLC application peers.

- ASE:  an abstract module of a system providing service to other parts of the system.

- Context Management:  an independent service that meets ATSC addressing requirements.  It provides the mechanism for aircraft and ATC ground systems to indicate availability to other ATN users and to convey the addresses to be employed.  The aircraft CM application interfaces with aircraft equipment to provide ATC ground system the addresses needed to establish communication with the aircraft.

- CPDLC AE abstract service interface:  the abstract interface between the CPDLC-users and the CPDLC-service provider.

- CPDLC ASE abstract service interface:  the abstract interface through which the CM-ASE service are accessed.

Note: — In version 1 of the CPDLC application, this interface coincide with the CPDLC-AE abstract service interface.

- CPDLC-air-ASE:  an abstract part of the aircraft system which performs the communication related functions of CPDLC.

- CPDLC-air-user:  the abstract part of the aircraft system which performs the non-communication related functions of CPDLC.

- CPDLC-CF:  that abstract part of the application entity which performs the mapping between the CPDLC-ASE service primitives and other elements within the CPDLC application.

- CPDLC-ground-ASE:  an abstract part of the ground system which performs the communication related functions of CPDLC.

- CPDLC-ground-user:  the abstract part of the ground system which performs the non-communication related functions of CPDLC.

- CPDLC service primitive:  a function of a CPDLC-AE that is not broken down further into sub-functions, and is presented as part of the CPDLC-AE abstract service interface (ie, request, indication, response, or confirmation).

- CPDLC service provider:  the CPDLC-service provider is composed of the ground and airborne CPDLC AEs, all underlying data communication protocol entities and the physical media.  As a consequence, it encompasses everything between the CPDLC-AE service interfaces of the end-users of the CPDLC application.

- Current Data Authority:  the ground system which is technically permitted to conduct a CPDLC dialogue with an aircraft.

- Dialogue Service:  the service which allows the CPDLC-air-ASE to communicate with the CPDLC-ground-ASE and vice-versa, or permits a CPDLC-ground-ASE to communicate with another CPDLC-ground-ASE.

- Downstream Data Authority:  the ground system which is technically permitted to conduct a DSC dialogue with an aircraft.

- Message:  information exchanged between the CPDLC air-user and the CPDLC-ground-user.

- Message Element:  a component of a message used to define the context of the information exchanged.

- Message Element Identifier:  the ASN.1 tag of the ATCUplinkMsgElementID or the ATCDownlinkMsgElementId.

- Message Header (air/ground):  control information used to maintain synchronization between the aircraft and the ground ATC system.

- Message Header (ground/ground):  control information used to maintain synchronization between the two ground ATC systems.

- Message Identification Number:  a unique number assigned to each air/ground message.  This number is used to differentiate messages and is conveyed in an air/ground message header.

- Message Reference Number:  used to uniquely associate a response with a previously received message.  The Message Identification Number of a previously received message becomes the Message reference number of the response message.  The Message Reference number is conveyed in the message header.

- Next Data Authority:  the ground system so designated by the Current Data Authority.

- Residual Error Rate:  the ratio of messages mis-delivered, non-delivered, or delivered with an error undetected by the system, over the total number of messages delivered to the system.

#### Conventions For Expressing Requirements

##### The following conventions apply for expressing requirements in this document:

- shal.l - used to state a mandatory requirement.

- should - used to state a recommended practice.

### References

#### The following references are used in this document:

- Automatic Dependent Surveillance (ADS) and Air Traffic Services (ATS) Data Link Applications Guidance Material, November 1994.

- ISO/IEC IS 8825-2, ITU-T Recommendation X.691, Information Technology - ASN.1 Encoding Rules - Packed Encoding Rules (PER).

- ISO/IEC 8824-1, ITU-T Recommendation X.682, Information Technology - Abstract Syntax Notation One (ASN.1).

- Draft SARPs for ATN Upper Layers for CNS/ATM-1 Package, (as proposed in 15 December 1995).

- QOS Levels reference.

## Application Functionality

### This section lists the operational requirements as defined by the ADSP that CPDLC addresses.  References in parentheses such a “(ADSP OR 4)” refer to the operational requirement number as identified by the ADS Panel.

- Comparison of Four-Dimensional (4-D) Profile Stored in the Aircraft System with Flight Data Stored in the FDPS.

- Approval for a Flight to Enter ADS-ATC Airspace

- Confirmation That the Aircraft’s Projected Profile Coincides With That Stored in the FDPS.

- Automatic Transfer of Control and Communications Between ADS-ATC Airspaces Using Digital Data Interchange.

- Airspace Automatic Transfer of Control and Communications From ADS-ATC to Non-ADS-ATC Airspace Using Digital Data Interchange.

- Provision Of Controller-Pilot Data Link Communications (CPDLC).

### Mapping of Operational Requirements to Functional Descriptions

#### This section presents how each CPDLC related OR listed above can be mapped to the functions described in Section 1.3.  Table 1-1 shows which functional descriptions assist in achieving each OR and are labeled as follows (section reference in parentheses):

- FD1:  Controller-Pilot Message Exchange (Section 1.3.1),

- FD2:  Transfer of Data Authority (Section 1.3.2),

- FD3:  Down Stream Clearance Function (Section 1.3.3), and

- FD4:  Ground Forward Function (Section 1.3.4).

| Operational Requirement | FD1 | FD2 | FD3 | FD4 |
| --- | --- | --- | --- | --- |
| Comparison of Four-Dimensional Profile Stored in the Aircraft System with Flight Data Stored in the FDPS | Ö |  |  |  |
| Approval for a Flight to Enter ADS-ATC Airspace | Ö | Ö | Ö | Ö |
| Confirmation That the Aircraft’s Projected Profile Coincides With That Stored in the FDPS | Ö |  |  |  |
| Automatic Transfer of Control and Communications Between ADS-ATC Airspaces Using Digital Data Interchange | Ö | Ö |  | Ö |
| Airspace Automatic Transfer of Control and Communications From ADS-ATC to Non-ADS-ATC Airspace Using Digital Data Interchange | Ö | Ö |  |  |
| Provision Of Controller-Pilot Data Link Communications | Ö |  |  |  |

Table 1-1:  OR to Functional Description Mapping

## Functional Descriptions

### FD1:  Controller-Pilot Message Exchange

#### Functional Description

##### The controller-pilot message exchange function defines a method for a controller and pilot to exchange messages via data link.  This function provides messages for the following :

- general information exchange;

- clearance

  - delivery,

  - request, and

  - response;

- altitude/identity surveillance;

- monitoring of current/planned position;

- advisories

  - request and

  - delivery;

- system management functions; and

- emergency situations.

### FD2:  Transfer of Data Authority

#### Functional Description

##### The Transfer of Data Authority Functions provides the capability for the current data authority to designate another ground system as the next data authority.  A CPDLC dialogue can be opened with or by the next data authority at a time before becoming the current data authority.  This capability is intended to prevent a loss of communication that would occur if the next data authority were prevented from actually setting up a dialogue with an aircraft until it became the current data authority.  The designation of a next data authority is accomplished using a CPDLC message.

### FD3:  Down Stream Clearance (DSC) Function

#### Functional Description

##### The Down Stream Clearance Function provides the capability for an aircraft to contact an air traffic service unit which is not the current data authority for the purpose of receiving a down stream clearance.  This information is exchanged using CPDLC message(s).

### FD4:  Ground Forward Function

#### Functional Description

##### The Ground Forward Function provides the capability for a ground system to forward information received in a CPDLC message to another ground system.  This information is exchanged using CPDLC message(s).

## A CPDLC Message

### A CPDLC message is composed of a message header, and from one to five message elements.

### For air/ground messages, the message header is composed of a message identification number, a message reference number, if required, a time stamp, and a logical acknowledgment requirement (optional).

### For ground/ground messages, the message header is composed of a time stamp and the aircraft identification to which the message refers.

### A message element consists of a message element identifier, data as indicated by the specified message element, and associated message element attributes.

### Message Identification Numbers

#### A message identification number pertains to a single peer to peer dialogue.

#### Message identification numbers used by a CPDLC ground system for uplink messages to an aircraft have no relationship to the message identification numbers used by the same ground system another aircraft.

#### Similarly, message identification numbers used by a CPDLC aircraft for downlink messages to a CPDLC ground system have no relationship to the message identification numbers used by the same aircraft with another ground system.

#### There is no relationship between message identification numbers assigned and managed by a the CPDLC ground system and those message identification numbers assigned and managed by the aircraft.

### Message Attributes

#### Message attributes dictate certain message handling requirements for the CPDLC-user receiving a message.  The CPDLC messages have Urgency, Alert, and Response attributes.

#### Attribute Association

##### Each message element has associated Urgency, Alert, and Response attributes as specified below.

##### When a message contains a single message element, the message attributes are the message element attributes.

##### When a message contains multiple message elements, the highest precedence message element attribute type associated with any element in the message become the message attribute type for the entire message.  Attribute type precedence is indicated below.  Message element attribute table entries are listed in order of precedence (ie, a precedence value of 1 is highest followed by 2, etc).  For example, this means that a message containing multiple message elements, where at least one element has a W/U attribute, has a W/U attribute for the whole message.

#### Urgency

##### The Urgency (URG) attribute delineates the queuing requirements for received messages that are displayed to the end-user.  Urgency types are presented in Table 1-2.  These Urgency attribute types are used for both air/ground and ground/ground messages.

| Type | Description | Precedence |
| --- | --- | --- |
| D | Distress | 1 |
| U | Urgent | 2 |
| N | Normal | 3 |
| L | Low | 4 |

Table 1-2:  Urgency Attribute (Air/Ground and Ground/Ground)

#### Alert

##### The alert (ALRT) attribute delineates the type of end-user alerting required by the CPDLC-user upon message receipt.  Alert types are presented in Table 1-3.  These Urgency attribute types are used for both air/ground and ground/ground messages.

| Type | Description | Precedence |
| --- | --- | --- |
| H | High | 1 |
| M | Medium | 2 |
| L | Low | 3 |
| N | No alerting required | 4 |

Table 1-3:  Alert Attribute (Air/Ground and Ground/Ground)

#### Response

##### The response (RESP) attribute mandates CPDLC-user response requirements for a given message element.  Response types are presented in Table 1-4 for uplink messages and Table 1-5 for downlink messages.  Response message attribute do not apply to ground/ground messages.

| Type | Response Required | Valid Responses Description | Precedence |
| --- | --- | --- | --- |
| W/U | Yes | Response required: WILCO, UNABLE, STANDBY permitted, LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 1 |
| A/N | Yes | Response required: AFFIRM, NEGATIVE, STANDBY permitted, LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 2 |
| R | Yes | Response required: ROGER, UNABLE, STANDBY permitted LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 3 |
| Y | Yes | Any CPDLC downlink message, LOGICAL ACKNOWLEDGMENT (only if required), | 4 |
| N | No, unless logical acknowledgment required | LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary, only when logical acknowledgment is required) | 5 |

Table 1-4:  Response Attribute (Up-Link)

| Type | Response Required | Valid Responses Description | Precedence |
| --- | --- | --- | --- |
| Y | Yes | Any CPDLC uplink message LOGICAL ACKNOWLEDGMENT (only if required), | 1 |
| N | No, unless logical acknowledgment required | LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary, only when logical acknowledgment is required) | 2 |

Table 1-5:  Response Attribute (Down-Link)

#### See Chapter 7 for detailed CPDLC message intent/use descriptions.

# GENERAL REQUIREMENTS

## Performance Requirements

- Note: — Systems developed to support CPDLC functionality will be capable of meeting the Required Communication Performance (RCP) appropriate for the phase of operation as specified in [5].

## Time Accuracy Requirements

### Absolute times which are sent as parameters over the data link shall  ([1])  be as accurate as the required resolution of the time parameter.

### Absolute times shall  ([2])  be UTC.

### Dates shall  ([3])  be expressed as UTC date (ie, the day increments at 2359:59z).

### Relative times shall  ([4])  be accurate to ± 0.1 second.

### Where time stamps are used they shall  ([5])  consist of year, month, day and hour, minute, second.

## Security Requirements

- Note: — There are no internationally approved operational requirements relating to data link application security.

## Backwards Compatibility Requirements

- Note: — This specification describes the version 1 of the CNS/ATM-1 Package CPDLC application.  Best efforts will be made to ensure that subsequent versions of this protocol are backwards compatible.

### For CNS/ATM-1 the CPDLC-air-ASE and CPDLC-ground-ASE version numbers shall  ([6])  both be set to one.

## Error Processing Requirements

### In the event of information input by the user being incompatible with that able to be processed by the system, the user shall  ([7])  be notified.

### In the event of a user invoking a CPDLC ASE service when when the CPDLC-ASE is not in a state specified in chapter 5, the invocation  be rejected and the user shal.l be notified.

# THE ABSTRACT SERVICE

## Introduction

### This section defines the abstract service interface for the CPDLC service.  There is no requirement to implement the service in a CPDLC product; however, it is necessary to implement the ground based and air based system in such a way that it shall  ([8])  be impossible to detect (from the peer system) whether or not an interface has been built.

- Note: — The CPDLC-ASE abstract service is described in this chapter from the viewpoint of the CPDLC-ASE-air-user, the CPDLC-ASE-ground-user and the CPDLC service-provider.

## The CPDLC Functional Model

- Figure 3-1 shows the functional model of the CPDLC Application. The functional modules identified in this model are the following :

- the CPDLC-user,

- the CPDLC Application Entity (CPDLC-AE) service interface,

- the CPDLC-AE,

- the CPDLC Control Function (CPDLC-CF),

- the CPDLC Application Service Element (CPDLC-ASE) service interface,

- the CPDLC-ASE, and

- the Dialogue Service (DS) interface.

Figure 3-1:  Functional Model of the CPDLC Application

- Note 2: — The CPDLC-user represents the operational part of the CPDLC system.  This user does not perform the communication functions but relies on a communication service provided to it via the CPDLC-AE through the CPDLC-AE service interface.  The individual actions at this interface are called CPDLC-AE service primitives.  Similarly, individual actions at other interfaces in the communication system are called service primitives at these interfaces.

- Note 3: — The CPDLC-AE consists of several elements including the CPDLC-ASE and the CPDLC-CF.  The DS interface is made available by the CPDLC-CF to the CPDLC-ASE for communication with the peer CPDLC-ASE.

- Note 4: — The CPDLC-ASE is the element in the communication system which executes the CPDLC specific protocol.  In other words, it takes care of the CPDLC specific service primitive sequencing actions, message creation, timer management, error and exception handling.

- Note 5: — The CPDLC-ASE interfaces only with the CPDLC-CF.  This CPDLC-CF is responsible for mapping service primitives received from one element (such as the CPDLC-ASE and the CPDLC-user) to other elements which interface with it.  The part of the CPDLC-CF which is relevant from the point of view of these SARPs, ie the part between the CPDLC-user and the CPDLC-ASE, will map CPDLC-AE service primitives to CPDLC-ASE service primitives transparently in the CNS/ATM-1 Package.

- Note 6: — The DS interface is the interface between the CPDLC-ASE and part of CPDLC-CF underneath, and provides a generic dialogue service.

## The CPDLC-ASE Abstract Service

### An implementation of the CPDLC ground and air service shall  ([9])  exhibit external behavior consistent with having implemented a CPDLC-ground-ASE, or CPDLC-air ASE respectively, with the following abstract service interface primitives, making them available to the CPDLC-ground-user or CPDLC-air-user respectively:

- CPDLC-start service as defined in section 3.5,

- DSC-start service as defined in section 3.6,

- CPDLC-message service as defined in section 3.7,

- CPDLC-end service as defined in section 3.8,

- DSC-end service as defined in section 3.9,

- CPDLC-forward service as defined in section 3.10,

- CPDLC-user-abort service as defined in section 3.11, and

- CPDLC-provider-abort service as defined in section 3.12.

## Conventions

- Note: 1 — For a given primitive, the presence of each parameter is described by one of the following values in the parameter tables in chapter 3.

| a) | blank | not present; |
| --- | --- | --- |
| b) | C | conditional upon some predicate explained in the text; |
| c) | C(=) | conditional upon the value of the parameter to the left being present, and equal to that value; |
| d) | M | mandatory; |
| e) | M(=) | mandatory, and equal to the value of the parameter to the left; |
| f) | U | user option. |

- Note 2: — The following abbreviations are used in this document:

- Req - request; data is input by CPDLC-user initiating the service to its respective ASE,

- Ind - indication; data is indicated by the receiving ASE to its respective CPDLC-user,

- Rsp - response; data is input by receiving CPDLC user to its respective ASE, and

- Cnf - confirmation; data is confirmed by the initiating ASE to its respective CPDLC-user.

- Note 3: — An unconfirmed service allows a  message to be transmitted, in one direction without a corresponding response.

- Note 4: — A confirmed service provides end-to-end confirmation that a message sent by one user was received by its peer user.

## CPDLC-start Service

- Note 1: — The CPDLC-start service is used by the CPDLC-air user or CPDLC-ground user to establish a CPDLC dialogue. It is a confirmed service.

- Note 2: — Once a CPDLC dialogue is established it remains open until explicitly closed.  (See CPDLC-end and CPDLC-abort services.)

### The CPDLC-start service primitives shall  ([10])  contain the parameters as presented table 3-1.

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| Called Peer Identifier | M |  |  |  |
| Calling Peer Identifier | M | M(=) |  |  |
| CPDLC Message | U | C(=) |  |  |
| Reject Reason |  |  | C | C(=) |
| Result |  |  | M | M(=) |
| Class of Communication Service | U |  |  |  |

Table 3-1:  CPDLC-start Service Parameters

### Called Peer Identifier

- Note 1: — If the service is ground initiated, this parameter contains the addressed aircraft’s 24-bit aircraft identifier.

- Note 2: — If the service is air initiated, this parameter contains the addressed ground system’s ICAO facility designator.

#### If the service is ground initiated, the Called Peer Identifier parameter value shall  ([11])  conform to the abstract syntax 24-bit aircraft-id.

#### If the service is ground initiated, the Called Peer Identifier parameter value shall  ([12])  conform to the abstract syntax eight-character ICAO facility designator.

### Calling Peer Identifier

- Note 1: — If the service is ground initiated, this parameter contains the sending ground system’s ICAO facility designator.

- Note 2: — If the service is air initiated, this parameter contains the sending aircraft’s 24-bit aircraft identifier.

#### If the service is ground initiated, the Calling Peer Identifier parameter value shall  ([13])  conform to the abstract syntax eight-character ICAO facility designator.

#### If the service is air initiated, the Calling Peer Identifier parameter value shall  ([14])  conform to the abstract syntax 24-bit aircraft-id.

### CPDLC Message

- Note: — The CPDLC-user can use this parameter to send a CPDLC message to its peer user.

#### The CPDLC Message parameter value shall  ([15])  conform to the ASN.1 abstract syntax ATCUplinkMessage, if supplied by the CPDLC-ground-user.

#### The CPDLC Message parameter value shall  ([16])  conform to the ASN.1 abstract syntax ATCDownlinkMessage, if supplied by the CPDLC-air-user.

### Reject Reason

- Note: — This parameter is used to provide a reason for rejecting a CPDLC dialogue.

#### If the CPDLC-user rejects the request to open a CPDLC dialogue, the CPDLC user shall  ([17])  provide a reason (a CPDLC message) for the rejection.

#### The Reject Reason parameter value shall  ([18])  conform to the ASN.1 abstract syntax ATCUplinkMessage if supplied by the CPDLC-ground-user.

#### The Reject Reason parameter value shall  ([19])  conform to the ASN.1 abstract syntax ATCDownlinkMessage if supplied by the CPDLC-air-user.

### Result

- Note: — This parameter is used to indicate whether or not a requested CPDLC dialogue is accepted.

#### This parameter shall  ([20])  have one of two abstract values: “accepted” or “rejected”.

### Class of Communication Service

- Note: — This parameter contains the value of the required class of communication service.  If not specified by the CPDLC-air-user, this indicates that there is no routing preference.

#### When this parameter is specified by the CPDLC-air-user, the Class of Communication Service parameter value shall  ([21])  have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note: — Class of Communication parameter values are detailed in [5].

## DSC-start Service

- Note 1: — The DSC-start service is  used to establish a DSC dialogue for the purpose of providing down stream clearances.  It is a confirmed service.

- Note 2: — Once a DSC dialogue is established it remains open until explicitly closed.  (See DSC-end and CPDLC-abort services.)

### The DSC-start service primitives shall  ([22])  contain the parameters as presented table 3-2.

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| ICAO Facility Designator | M |  |  |  |
| Aircraft Identifier | M | M(=) |  |  |
| CPDLC Message | U | C(=) |  |  |
| Reject Reason |  |  | C | C(=) |
| Result |  |  | M | M(=) |
| Class of Communication Service | U |  |  |  |

Table 3-2:  DSC-start Service Parameters

### ICAO Facility Designator

- Note: — This parameter contains the addressed ground system’s  ICAO facility designator.

#### The ICAO Facility Designator parameter value shall  ([23])  conform to the abstract syntax eight-character ICAO facility designator.

### Aircraft Identifier

#### The Aircraft Identifier parameter value shall  ([24])  conform to the abstract syntax 24-bit aircraft-id.

- Note: — This parameter contains the aircraft’s 24 bit ICAO address.

### CPDLC Message

- Note: — The CPDLC-air-user can use this parameter to send a CPDLC message to a CPDLC-ground-user.

#### The CPDLC Message parameter value shall  ([25])  conform to the ASN.1 abstract syntax ATCDownlinkMessage.

### Reject Reason

- Note: — The parameter is used to provide a reason for rejecting a DSC dialogue.

#### If the CPDLC-ground-user rejects the request to open a DSC dialogue, the CPDLC-ground-user shall  ([26])  provide a reason (a CPDLC message) for the rejection.

#### The Reject Reason parameter shall  ([27])  conform to the ASN.1 abstract syntax ATCUplinkMessage.

### Result

- Note: — This parameter is used to indicate whether or not a requested DSC dialogue is accepted.

#### The Result parameter value shall  ([28])  have one of two abstract values: “accepted” or “rejected”.

### Class of Communication Service

- Note: — This parameter contains the value of the required class of communication service.  If not specified by the CPDLC-user, this indicates that there is no routing preference.

#### When this parameter is specified by the CPDLC-user, it shall  ([29])  have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note: — Class of Communication parameter values are detailed in [5].

## CPDLC-message Service

- Note: — The CPDLC-message service can be used for pilot/controller message exchange, once a dialogue is established.  It is an unconfirmed service.

### The CPDLC-message service primitives shall  ([30])  contain the parameters as presented table 3-3.

| Parameter Name | Req | Ind |
| --- | --- | --- |
| CPDLC Message | M | M(=) |

Table 3-3:  CPDLC-message Service Parameters

### CPDLC Message

- Note: — This parameter contains a CPDLC message.

#### The CPDLC Message parameter value shall  ([31])  conform to the ASN.1 abstract syntax ATCUplinkMessage, if provided by the CPDLC-ground-user.

#### The CPDLC Message parameter value shall  ([32])  conform to the ASN.1 abstract syntax ATCDownlinkMessage, if provided by the CPDLC-air-user.

## CPDLC-end Service

- Note: — The CPDLC-end service is used by the CPDLC-ground-user to end a CPDLC dialogue with a CPDLC-air-user.  It is a confirmed service.

### The CPDLC-end service primitives shall  ([33])  contain the parameters as presented table 3-4.

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| CPDLC Message | U | C(=) | U | C(=) |
| Result |  |  | M | M(=) |

Table 3-4:  CPDLC-end Service Parameters

### CPDLC Message

- Note:  — This parameter contains a CPDLC message.

#### The CPDLC Message parameter value shall  ([34])  conform to the ASN.1 abstract syntax ATCUplinkMessage, if provided by the CPDLC-ground-user.

#### The CPDLC Message parameter value shall  ([35])  conform to the ASN.1 abstract syntax ATCDownlinkMessage, if provided by the CPDLC-air-user.

### Result

- Note: — This parameter is used to indicate whether or not a request to terminate a CPDLC dialogue is accepted.

#### The Result parameter shall  ([36])  have one of two abstract values: “accepted” or “rejected”.

## DSC-end Service

- Note: — The DSC-end service is used by the DSC-air-user to end a DSC dialogue with a CPDLC-ground-user.  It is a confirmed service.

### The DSC-end service primitives shall  ([37])  contain the parameters as presented table 3-5.

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| CPDLC Message | U | C(=) | U | C(=) |
| Result |  |  | M | M(=) |

Table 3-5:  DSC-end Service Parameters

### CPDLC Message

- Note: — This parameter contains a CPDLC message.

#### The CPDLC Message parameter value shall  ([38])  conform to the ASN.1 abstract syntax ATCUplinkMessage, if provided by the CPDLC-ground-user.

#### The CPDLC Message parameter value shall  ([39])  conform to the ASN.1 abstract syntax ATCDownlinkMessage if provided by the CPDLC-air-user.

### Result

- Note: — This parameter is used to indicate whether or not a request to terminate a DSC dialogue is accepted.

#### The Result parameter shall  ([40])  have one of two abstract values: “accepted” or “rejected”.

## CPDLC-forward Service

- Note: — The CPDLC-forward service is used by a CPDLC-ground-user to send a CPDLC message to another CPDLC-ground-user.  Its primary use is for the forwarding of aircraft requests.

### The CPDLC-forward service primitives shall  ([41])  contain the parameters as presented table 3-6.

| Parameter Name | Req | Ind | Cnf |
| --- | --- | --- | --- |
| Called ICAO Facility Designator | M |  |  |
| Calling ICAO Facility Designator | M | M(=) |  |
| CPDLC Message | M | M(=) |  |
| Class of Communication Service | U |  |  |

Table 3-6:  CPDLC-forward Service Parameters

### Called ICAO Facility Designator

Note: — This parameter contains the addressed ground system’s ICAO facility designator.

#### The Called ICAO Facility Designator parameter value shall  ([42])  conform to the abstract syntax eight-character ICAO facility designator.

### Calling ICAO Facility Designator

Note: — This parameter contains the sending ground system’s ICAO facility designator.

#### The Calling ICAO Facility Designator parameter value shall  ([43])  conform to the abstract syntax eight-character ICAO facility designator.

### CPDLC Message

- Note: — The sending CPDLC-ground-user uses this parameter to forward a CPDLC message to another CPDLC-ground-user.

#### The CPDLC Message parameter value shall  ([44])  conform to the ASN.1 abstract syntax ATCForwardMessage, when supplied by the CPDLC-ground-user.

### Class of Communication Service

- Note: — This parameter contains the value of the required class of communication service.  If not specified by the CPDLC-user, this indicates that there is no routing preference.

#### When this parameter is specified by the CPDLC-ground-user, the Class of Communication Service parameter value shall  ([45])  have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note: — Class of Communication parameter values are detailed in [5].

## CPDLC-user-abort Service

- Note:  — This service provides the capability for either the CPDLC-air-user or the CPDLC-ground-user to abort communication with its peer.  It can be invoked at any time the user is aware that the CPDLC service is in operation.  The CPDLC-user-abort service can be used for operational or technical reasons.  It is an unconfirmed service.

### The CPDLC-user-abort service primitives shall  ([46])  contain the parameters as presented table 3-7.

.

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Reason | U | C(=) |

Table 3-7:  CPDLC-user-abort Service Parameters

### Reason

- Note: — This parameter is used to indicate a reason for aborting the CPDLC or DSC dialogue.

#### The Reason parameter value conforms to the ASN.1 abstract syntax CPDLCAbortReason.

## CPDLC-provider-abort Service

- Note: — This service provides the capability for the CPDLC-service provider to inform its users, if active, that it can no longer provide the CPDLC service.

### The CPDLC-provider-abort service primitives shall  ([47])  contain the parameters as presented table 3-8.

| Parameter Name | Ind |
| --- | --- |
| Reason | M |

Table 3-8:  CPDLC-provider-abort Service Parameters

### Reason

- Note: — This parameter identifies the reason for the abort.

#### The Reason parameter shall  ([48])  conform to the ASN.1 abstract syntax CPDLCAbortReason.

# FORMAL DEFINITIONS OF MESSAGES

## ASN.1

### The abstract syntax of the CPDLC protocol data units shall  ([49])  comply with the description contained in the ASN.1 module CPDLCMessageSetVersion1 (conforming to [3]), as defined below:

CPDLCMessageSetVersion1 DEFINITIONS AUTOMATIC TAGS:: =

BEGIN

-- ----------------------------------------------------------------------------------

-- Ground Generated Messages - Top level

-- ----------------------------------------------------------------------------------

GroundPDUs :: = CHOICE

{

abort		CPDLCAbortReason,

startup		UplinkMessage,

send		ATCUplinkMessage,

forward		ATCForwardMessage

...

}

UplinkMessage :: = CHOICE

{

noMessage		NULL,

aTCUplinkMessage	ATCUplinkMessage

}

ATCUplinkMessage :: = SEQUENCE

{

header			ATCMessageHeader,

ElementIds		SEQUENCE SIZE (1..5) OFATCUplinkMsgElementId

}

ATCForwardMessage :: = SEQUENCE

{

forwardHeader	ForwardHeader,

forwardMessage	ForwardMessage

}

ForwardHeader :: = SEQUENCE

{

dateTime		DateTimeGroup,

aircraftId		AircraftFlightIdentification

}

ForwardMessage :: = CHOICE

{

upElementIds		SEQUENCE SIZE (1..5) OF ATCUplinkMsgElementId,

downElementIds		SEQUENCE SIZE (1..5) OF ATCDownlinkMsgElementId

}

-- ----------------------------------------------------------------------------------

-- Aircraft Generated Messages - Top level

-- ----------------------------------------------------------------------------------

AircraftPDUs:: = CHOICE

{

abort		CPDLCAbortReason,

startdown	StartDownlinkMessage,

send		ATCDownlinkMessage

...

}

StartDownMessage :: = SEQUENCE

{

mode	Mode DEFAULT {cpdlc},

startDownlinkMessage 	DownlinkMessage

}

Mode :: = ENUMERATED

{

cpdlc		(0),

dsc		(1)

}

DownlinkMessage :: =   CHOICE

{

noMessage			NULL,

aTCDownlinkMessage		ATCDownlinkMessage

}

ATCDownlinkMessage :: =   SEQUENCE

{

header			ATCMessageHeader,

ElementIds		SEQUENCE SIZE (1..5) OF ATCDownlinkMsgElementId

}

-- ----------------------------------------------------------------------------------

-- Uplink and Downlink messages - Common Elements

-- ----------------------------------------------------------------------------------

ATCMessageHeader :: = SEQUENCE

{

messageIdNumber		MsgIdentificationNumber,

messageRefNumber		MsgReferenceNumber		OPTIONAL,

dateTime			DateTimeGroup,

logicalAck			LogicalAck			DEFAULT notRequired

}

MsgIdentificationNumber :: = INTEGER (0..63)

MsgReferenceNumber :: = INTEGER (0..63)

CPDLCAbortReason :: = ENUMERATED

{

CPDLC-user-abort				(0),

no-message-identification-numbers-available 	(1),

duplicate-message-identifiction-numbers		(2),

no-longer-next-data-authority			(3),

current-data-authority-abort			(4),

timer-expired					(5),

undefined-error					(6),

invalid-PDU					(7),

not-permitted-PDU				(8),

communication-service-error			(9),

communication-service-failure			(10)

...

}

LogicalAck :: = ENUMERATED

{

required				(0),

notRequired			(1)

}

-- ----------------------------------------------------------------------------------

-- Uplink message element

-- ----------------------------------------------------------------------------------

ATCUplinkMsgElementId :: = CHOICE

{

--	UNABLE					Urg(N)/Alr(L)/Resp(N )

uM0NULL					NULL,

--	STANDBY					Urg(N)/Alr(L)/Resp(N )

uM1NULL					NULL,

--	REQUEST DEFERRED				Urg(N)/Alr(L)/Resp(N )

uM2NULL					NULL,

-- 	ROGER						Urg(N)/Alr(L)/Resp(N )

uM3NULL					NULL,

--	AFFIRM 					Urg(N)/Alr(L)/Resp(N )

uM4NULL					NULL,

--	NEGATIVE					Urg(N)/Alr(L)/Resp(N )

uM5NULL					NULL,

--	EXPECT [altitude]				Urg(L)/Alr(L)/Resp( R )

uM6Altitude					Altitude,

--	EXPECT CLIMB AT [time]			Urg(L)/Alr(L)/Resp( R )

uM7Time					Time,

--	EXPECT CLIMB AT [position]			Urg(L)/Alr(L)/Resp( R )

uM8Position					Position,

--	EXPECT DESCENT AT [time]			Urg(L)/Alr(L)/Resp( R )

uM9Time					Time,

--	EXPECT DESCENT AT [position]		Urg(L)/Alr(L)/Resp( R )

uM10Position					Position,

--	EXPECT CRUISE CLIMB AT [time]		Urg(L)/Alr(L)/Resp( R )

uM11Time					Time,

--	EXPECT CRUISE CLIMB AT [position]		Urg(L)/Alr(L)/Resp( R )

uM12Position					Position,

--	AT [time] EXPECT CLIMB TO [altitude]		Urg(L)/Alr(L)/Resp( R )

uM13TimeAltitude				TimeAltitude,

--	AT [position] EXPECT CLIMB TO [altitude]	Urg(L)/Alr(L)/Resp( R )

uM14PositionAltitude				PositionAltitude,

--	AT [time] EXPECT DESCENT TO [altitude]	Urg(L)/Alr(L)/Resp( R )

uM15TimeAltitude				TimeAltitude,

--	AT [position] EXPECT DESCENT TO [altitude]	Urg(L)/Alr(L)/Resp( R )

uM16PositionAltitude				PositionAltitude,

--	AT [time] EXPECT CRUISE CLIMB TO [altitude]	Urg(L)/Alr(L)/Resp( R )

uM17TimeAltitude				TimeAltitude,

--	AT [position] EXPECT CRUISE CLIMB TO [altitude]													Urg(L)/Alr(L)/Resp( R )

uM18PositionAltitude				PositionAltitude,

--	MAINTAIN [altitude]				Urg(N)/Alr(M)/Resp(W/U)

uM19Altitude					Altitude,

--	CLIMB TO AND MAINTAIN [altitude]		Urg(N)/Alr(M)/Resp(W/U)

uM20Altitude					Altitude,

--	AT [time] CLIMB TO AND MAINTAIN [altitude]	Urg(N)/Alr(M)/Resp(W/U)

uM21TimeAltitude				TimeAltitude,

--	AT [position] CLIMB TO AND MAINTAIN [altitude]													Urg(N)/Alr(M)/Resp(W/U)

uM22PositionAltitude				PositionAltitude,

--	DESCEND TO AND MAINTAIN [altitude]	Urg(N)/Alr(M)/Resp(W/U)

uM23Altitude 					Altitude,

--	AT [time] DESCEND TO AND MAINTAIN [altitude] 													Urg(N)/Alr(M)/Resp(W/U)

uM24TimeAltitude 				TimeAltitude,

--	AT [position] DESCEND TO AND MAINTAIN [altitude] 													Urg(N)/Alr(M)/Resp(W/U)

uM25PositionAltitude 				PositionAltitude,

--	CLIMB TO REACH [altitude] BY [time]		Urg(N)/Alr(M)/Resp(W/U)

uM26AltitudeTime 				AltitudeTime,

--	CLIMB TO REACH [altitude] BY [position]	Urg(N)/Alr(M)/Resp(W/U)

uM27AltitudePosition 				AltitudePositon,

--	DESCEND TO REACH [altitude] BY [time]	Urg(N)/Alr(M)/Resp(W/U)

uM28AltitudeTime 				AltitudeTime,

--	DESCEND TO REACH [altitude] BY [position]	Urg(N)/Alr(M)/Resp(W/U)

uM29AltitudePosition 				AltitudePositon,

--	MAINTAIN BLOCK [altitude] TO [altitude]	Urg(N)/Alr(M)/Resp(W/U)

uM30AltitudeAltitude 				AltitudeAltitude,

--	CLIMB TO AND MAINTAIN BLOCK [altitude] TO [altitude]												Urg(N)/Alr(M)/Resp(W/U)

uM31AltitudeAltitude 				AltitudeAltitude,

--	DESCEND TO AND MAINTAIN BLOCK [altitude] TO [altitude]												Urg(N)/Alr(M)/Resp(W/U)

uM32AltitudeAltitude 				AltitudeAltitude,

--	CLEARED OUT OF CONTROLLED AIRSPACE	Urg(N)/Alr(M)/Resp(W/U)

uM33NULL					NULL,

--	CRUISE CLIMB TO [altitude]			Urg(N)/Alr(M)/Resp(W/U)

uM34Altitude 					Altitude,

--	CRUISE CLIMB ABOVE [altitude]		Urg(N)/Alr(M)/Resp(W/U)

uM35Altitude 					Altitude,

--	EXPEDITE CLIMB TO [altitude]			Urg(U)/Alr(M)/Resp(W/U)

uM36Altitude 					Altitude,

--	EXPEDITE DESCENT TO [altitude]		Urg(U)/Alr(M)/Resp(W/U)

uM37Altitude 					Altitude,

--	IMMEDIATELY CLIMB TO [altitude]		Urg(D)/Alr(H)/Resp(W/U)

uM38Altitude 					Altitude,

--	IMMEDIATELY DESCEND TO [altitude]		Urg(D)/Alr(H)/Resp(W/U)

uM39Altitude 					Altitude,

--	IMMEDIATELY STOP CLIMB AT [altitude]	Urg(D)/Alr(H)/Resp(W/U)

uM40Altitude 					Altitude,

--	IMMEDIATELY STOP DESCENT AT [altitude]	Urg(D)/Alr(H)/Resp(W/U)

uM41Altitude 					Altitude,

--	EXPECT TO CROSS [position] AT [altitude] 	Urg(L)/Alr(L)/Resp( R )

uM42PositionAltitude				PositionAltitude,

--	EXPECT TO CROSS [position] AT OR ABOVE [altitude]													Urg(L)/Alr(L)/Resp( R )

uM43PositionAltitude				PositionAltitude,

--	EXPECT TO CROSS [position] AT OR BELOW [altitude]													Urg(L)/Alr(L)/Resp( R )

uM44PositionAltitude 				PositionAltitude,

--	EXPECT TO CROSS [position] AT AND MAINTAIN [altitude]												Urg(L)/Alr(L)/Resp( R )

uM45PositionAltitude 				PositionAltitude,

--	CROSS [position] AT [altitude]			Urg(N)/Alr(M)/Resp(W/U)

uM46PositionAltitude 				PositionAltitude,

--	CROSS [position] AT OR ABOVE [altitude]	Urg(N)/Alr(M)/Resp(W/U)

uM47PositionAltitude 				PositionAltitude,

--	CROSS [position] AT OR BELOW [altitude]	Urg(N)/Alr(M)/Resp(W/U)

uM48PositionAltitude 				PositionAltitude,

--	CROSS [position] AT AND MAINTAIN [altitude]	Urg(N)/Alr(M)/Resp(W/U)

uM49PositionAltitude 				PositionAltitude,

--	CROSS [position] BETWEEN [altitude] AND [altitude]													Urg(N)/Alr(M)/Resp(W/U)

uM50PositionAltitudeAltitude			PositionAltitudeAltitude,

--	CROSS [position] AT [time]			Urg(N)/Alr(M)/Resp(W/U)

uM51PositionTime 				PositionTime,

--	CROSS [position] AT OR BEFORE [time]		Urg(N)/Alr(M)/Resp(W/U)

uM52PositionTime 				PositionTime,

--	CROSS [position] AT OR AFTER [time]		Urg(N)/Alr(M)/Resp(W/U)

uM53PositionTime 				PositionTime,

--	CROSS [position] BETWEEN [time] AND [time]	Urg(N)/Alr(M)/Resp(W/U)

uM54PositionTimeTime				PositionTimeTime,

--	CROSS [position] AT [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM55PositionSpeed 				PositionSpeed,

--	CROSS [position] AT OR LESS THAN [speed]	Urg(N)/Alr(M)/Resp(W/U)

uM56PositionSpeed 				PositionSpeed,

--	CROSS [position] AT OR GREATER THAN [speed]													Urg(N)/Alr(M)/Resp(W/U)

uM57PositionSpeed 				PositionSpeed,

--	CROSS [position] AT [time] AT [altitude]		Urg(N)/Alr(M)/Resp(W/U)

uM58PositionTimeAltitude			PositionTimeAltitude,

--	CROSS [position] AT OR BEFORE [time] AT [altitude]													Urg(N)/Alr(M)/Resp(W/U)

uM59PositionTimeAltitude 			PositionTimeAltitude,

--	CROSS [position] AT OR AFTER [time] AT [altitude]													Urg(N)/Alr(M)/Resp(W/U)

uM60PositionTimeAltitude			PositionTimeAltitude,

--	CROSS [position] AT AND MAINTAIN [altitude] AT [speed]												Urg(N)/Alr(M)/Resp(W/U)

uM61PositionAltitudeSpeed 			PositionAltitudeSpeed,

--	AT [time] CROSS [position] AT AND MAINTAIN [altitude]													Urg(N)/Alr(M)/Resp(W/U)

uM62TimePositionAltitude			TimePositionAltitude,

--	AT [time] CROSS [position] AT AND MAINTAIN [altitude] AT [speed]

--							Urg(N)/Alr(M)/Resp(W/U)

uM63TimePositionAltitudeSpeed			TimePositionAltitudeSpeed,

--	OFFSET [distanceOffset] [direction] OF ROUTE	Urg(N)/Alr(M)/Resp(W/U)

uM64DistanceOffsetDirection			DistanceOffsetDirection,

--	AT [position] OFFSET [distanceOffset] [direction] OF ROUTE

--							Urg(N)/Alr(M)/Resp(W/U)

uM65PositionDistanceOffsetDirection		PositionDistanceOffsetDirection,

--	AT [time] OFFSET [distanceOffset] [direction] OF ROUTE													Urg(N)/Alr(M)/Resp(W/U)

uM66TimeDistanceOffsetDirection			TimeDistanceOffsetDirection,

--	PROCEED BACK ON ROUTE			Urg(N)/Alr(M)/Resp(W/U)

uM67NULL 					NULL,

--	REJOIN ROUTE BY [position]			Urg(N)/Alr(M)/Resp(W/U)

uM68Position 					Position,

--	REJOIN ROUTE BY [time]			Urg(N)/Alr(M)/Resp(W/U)

uM69Time 					Time,

--	EXPECT BACK ON ROUTE BY [position]		Urg(L)/Alr(L)/Resp( R )

uM70Position 					Position,

--	EXPECT BACK ON ROUTE BY [time]		Urg(L)/Alr(L)/Resp( R )

uM71Time 					Time,

--	RESUME OWN NAVIGATION			Urg(N)/Alr(M)/Resp(W/U)

uM72NULL 					NULL,

--	[DepartureClearance]				Urg(N)/Alr(M)/Resp(W/U)

uM73DepartureClearance 			DepartureClearance,

--	PROCEED DIRECT TO [position]			Urg(N)/Alr(M)/Resp(W/U)

uM74Position 					Position,

--	WHEN ABLE PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM75Position 					Position,

--	AT [time] PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM76TimePosition 				TimePosition,

--	AT [position] PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM77PositionPosition 				PositionPosition,

--	AT [altitude] PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM78AltitudePosition 				AltitudePosition,

--	CLEARED TO [position] VIA [routeClearance]	Urg(N)/Alr(M)/Resp(W/U)

uM79PositionRouteClearance			PositionRouteClearance,

--	CLEARED [routeClearance]			Urg(N)/Alr(M)/Resp(W/U)

uM80RouteClearance 				RouteClearance,

--	CLEARED [procedureName]			Urg(N)/Alr(M)/Resp(W/U)

uM81ProcedureName 				ProcedureName,

--	CLEARED TO DEVIATE UP TO [distanceOffset] [direction] OF ROUTE											Urg(N)/Alr(M)/Resp(W/U)

uM82DistanceOffsetDirection			DistanceOffsetDirection,

--	AT [position] CLEARED [routeClearance]		Urg(N)/Alr(M)/Resp(W/U)

uM83PositionRouteClearance			PositionRouteClearance,

--	AT [position] CLEARED [procedureName]		Urg(N)/Alr(M)/Resp(W/U)

uM84PositionProcedureName 			PositionProcedureName,

--	EXPECT [routeClearance]			Urg(L)/Alr(L)/Resp( R )

uM85RouteClearance 				RouteClearance,

--	AT [position] EXPECT [routeClearance] 		Urg(L)/Alr(L)/Resp( R )

uM86PositionRouteClearance			PositionRouteClearance,

--	EXPECT DIRECT TO [position]			Urg(L)/Alr(L)/Resp( R )

uM87Position 					Position,

--	AT [position] EXPECT DIRECT TO [position]	Urg(L)/Alr(L)/Resp( R )

uM88PositionPosition 				PositionPosition,

--	AT [time] EXPECT DIRECT TO [position]		Urg(L)/Alr(L)/Resp( R )

uM89TimePosition 				TimePosition,

--	AT [altitude] EXPECT DIRECT TO [position]	Urg(L)/Alr(L)/Resp( R )

uM90AltitudePosition 				AltitudePosition,

--	HOLD AT [position] MAINTAIN [altitude] INBOUND TRACK [degrees][direction]

--	TURNS [legtype]					Urg(N)/Alr(M)/Resp(W/U)

uM91HoldClearance 				HoldClearance,

--	HOLD AT [position] AS PUBLISHED MAINTAIN [altitude]													Urg(N)/Alr(M)/Resp(W/U)

uM92PositionAltitude 				PositionAltitude,

--	EXPECT FURTHER CLEARANCE AT [time]	Urg(L)/Alr(L)/Resp( R )

uM93Time 					Time,

--	TURN [direction] HEADING [degrees]		Urg(N)/Alr(M)/Resp(W/U)

uM94DirectionDegrees 				DirectionDegrees,

--	TURN [direction] GROUND TRACK [degrees]	Urg(N)/Alr(M)/Resp(W/U)

uM95DirectionDegrees				DirectionDegrees,

--	FLY PRESENT HEADING			Urg(N)/Alr(M)/Resp(W/U)

uM96NULL 					NULL,

--	AT [position] FLY HEADING [degrees]		Urg(N)/Alr(M)/Resp(W/U)

uM97PositionDegrees 				PositionDegrees,

--	IMMEDIATELY TURN [direction] HEADING [degrees]													Urg(D)/Alr(H)/Resp(W/U)

uM98DirectionDegrees 				DirectionDegrees,

--	EXPECT [procedureName]			Urg(L)/Alr(L)/Resp(R)

uM99ProcedureName 				ProcedureName,

--	AT [time] EXPECT [speed]			Urg(L)/Alr(L)/Resp(R)

uM100TimeSpeed 				TimeSpeed,

--	AT [position] EXPECT [speed]			Urg(L)/Alr(L)/Resp(R)

uM101PositionSpeed 				PositionSpeed,

--	AT [altitude] EXPECT [speed]			Urg(L)/Alr(L)/Resp(R)

uM102AltitudeSpeed 				AltitudeSpeed,

--	AT [time] EXPECT [speed] TO [speed] 		Urg(L)/Alr(L)/Resp(R)

uM103TimeSpeedSpeed 				TimeSpeedSpeed,

--	AT [position] EXPECT [speed] TO [speed]		Urg(L)/Alr(L)/Resp(R)

uM104PositionSpeedSpeed 			PositionSpeedSpeed,

--	AT [altitude] EXPECT [speed] TO [speed]		Urg(L)/Alr(L)/Resp(R)

uM105AltitudeSpeedSpeed 			AltitudeSpeedSpeed,

--	MAINTAIN [speed]				Urg(N)/Alr(M)/Resp(W/U)

uM106Speed 					Speed,

--	MAINTAIN PRESENT SPEED			Urg(N)/Alr(M)/Resp(W/U)

uM107NULL 					NULL,

--	MAINTAIN [speed] OR GREATER		Urg(N)/Alr(M)/Resp(W/U)

uM108Speed 					Speed,

--	MAINTAIN [speed] OR LESS			Urg(N)/Alr(M)/Resp(W/U)

uM109Speed 					Speed,

--	MAINTAIN [speed] TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM110SpeedSpeed 				SpeedSpeed,

--	INCREASE SPEED TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM111Speed 					Speed,

--	INCREASE SPEED TO [speed] OR GREATER  	Urg(N)/Alr(M)/Resp(W/U)

uM112Speed 					Speed,

--	REDUCE SPEED TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM113Speed 					Speed,

--	REDUCE SPEED TO [speed] OR LESS		Urg(N)/Alr(M)/Resp(W/U)

uM114Speed 					Speed,

--	DO NOT EXCEED [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM115Speed 					Speed,

--	RESUME NORMAL SPEED			Urg(N)/Alr(M)/Resp(W/U)

uM116NULL 					NULL,

--	CONTACT [icaounitname] [frequency]		Urg(N)/Alr(M)/Resp(W/U)

uM117ICAOUnitNameFrequency 			ICAOUnitNameFrequency,

--	AT [position] CONTACT [icaounitname] [frequency]													Urg(N)/Alr(M)/Resp(W/U)

uM118PositionICAOUnitNameFrequency		PositionICAOUnitNameFrequency,

--	AT [time] CONTACT [icaounitname] [frequency]	Urg(N)/Alr(M)/Resp(W/U)

uM119TimeICAOUnitNameFrequency		TimeICAOUnitNameFrequency,

--	MONITOR [icaounitname] [frequency]		Urg(N)/Alr(M)/Resp(W/U)

uM120ICAOUnitNameFrequency			ICAOUnitNameFrequency,

--	AT [position] MONITOR [icaounitname] [frequency]													Urg(N)/Alr(M)/Resp(W/U)

uM121PositionICAOUnitNameFrequency		PositionICAOUnitNameFrequency,

--	AT [time] MONITOR [icaounitname] [frequency]	Urg(N)/Alr(M)/Resp(W/U)

uM122TimeICAOUnitNameFrequency 		TimeICAOUnitNameFrequency,

--	SQUAWK [beaconcode]				Urg(N)/Alr(M)/Resp(W/U)

uM123BeaconCode 				BeaconCode,

--	STOP SQUAWK					Urg(N)/Alr(M)/Resp(W/U)

uM124NULL 					NULL,

--	SQUAWK MODE CHARLIE			Urg(N)/Alr(M)/Resp(W/U)

uM125NULL					NULL,

--	STOP SQUAWK MODE CHARLIE		Urg(N)/Alr(M)/Resp(W/U)

uM126NULL					NULL,

--	REPORT BACK ON ROUTE			Urg(N)/Alr(M)/Resp( R )

uM127NULL 					NULL,

--	REPORT LEAVING [altitude]			Urg(N)/Alr(M)/Resp( R )

uM128Altitude 					Altitude,

--	REPORT WHEN LEVEL AT [altitude]		Urg(N)/Alr(M)/Resp(R)

uM129Altitude					Altitude,

--	REPORT PASSING [position]			Urg(N)/Alr(M)/Resp( R )

uM130Position 					Position,

--	REPORT REMAINING FUEL AND SOULS ON BOARD													Urg(N)/Alr(M)/Resp(Y )

uM131NULL 					NULL,

--	REPORT POSITION				Urg(N)/Alr(M)/Resp(Y)

uM132NULL					NULL,

--	REPORT ALTITUDE				Urg(N)/Alr(M)/Resp(Y)

uM133NULL					NULL,

--	REPORT SPEED [speedqualifier] [speedtype]	Urg(N)/Alr(M)/Resp(Y)

uM134 SpeedQualifierSpeedType			SpeedQualifierSpeedType,

--	CONFIRM ASSIGNED LEVEL			Urg(N)/Alr(M)/Resp(Y )

uM135NULL 					NULL,

--	CONFIRM ASSIGNED SPEED			Urg(N)/Alr(M)/Resp(Y )

uM136NULL 					NULL,

--	CONFIRM ASSIGNED ROUTE			Urg(N)/Alr(M)/Resp(Y )

uM137NULL 					NULL,

--	CONFIRM TIME OVER REPORTED WAYPOINT 	Urg(N)/Alr(M)/Resp(Y)

uM138NULL 					NULL,

--	CONFIRM REPORTED WAYPOINT		Urg(N)/Alr(M)/Resp(Y)

uM139NULL 					NULL,

--	CONFIRM NEXT WAYPOINT			Urg(N)/Alr(M)/Resp(Y)

uM140NULL 					NULL,

--	CONFIRM NEXT WAYPOINT ETA		Urg(N)/Alr(M)/Resp(Y)

uM141NULL 					NULL,

--	CONFIRM ENSUING WAYPOINT		Urg(N)/Alr(M)/Resp(Y)

uM142NULL 					NULL,

--	CONFIRM REQUEST				Urg(N)/Alr(M)/Resp(Y)

uM143NULL 					NULL,

--	CONFIRM SQUAWK				Urg(N)/Alr(M)/Resp(Y)

uM144NULL 					NULL,

--	REPORT HEADING				Urg(N)/Alr(M)/Resp(Y)

uM145NULL 					NULL,

--	REPORT GROUND TRACK			Urg(N)/Alr(M)/Resp(Y)

uM146NULL 					NULL,

--	REQUEST POSITION REPORT			Urg(N)/Alr(M)/Resp(Y )

uM147NULL 					NULL,

--	WHEN CAN YOU ACCEPT [altitude]		Urg(N)/Alr(M)/Resp(Y)

uM148Altitude 					Altitude,

--	CAN YOU ACCEPT [altitude] AT [position]	Urg(N)/Alr(M)/Resp(A/N)

uM149AltitudePosition 				AltitudePosition,

--	CAN YOU ACCEPT [altitude] AT [time]		Urg(N)/Alr(M)/Resp(A/N)

uM150AltitudeTime 				AltitudeTime,

--	WHEN CAN YOU ACCEPT [speed]		Urg(N)/Alr(M)/Resp(Y)

uM151Speed 					Speed,

--	WHEN CAN YOU ACCEPT [distanceOffset] [direction] OFFSET												Urg(N)/Alr(M)/Resp(Y)

uM152DistanceOffsetDirection			DistanceOffsetDirection,

--	ALTIMETER [altimeter]				Urg(N)/Alr(M)/Resp(R)

uM153Altimeter 					Altimeter,

--	RADAR SERVICE TERMINATED		Urg(N)/Alr(M)/Resp(R)

uM154NULL					NULL,

--	RADAR CONTACT [position]			Urg(N)/Alr(M)/Resp(R)

uM155Position 					Position,

--	RADAR CONTACT LOST			Urg(N)/Alr(M)/Resp(R)

uM156NULL 					NULL,

--	CHECK STUCK MICROPHONE [frequency]	Urg(U)/Alr(M)/Resp(N)

uM157Frequency 				Frequency,

--	ATIS [atiscode]					Urg(N)/Alr(M)/Resp(R)

uM158AtisCode 					AtisCode,

--	ERROR [errorInformation]			Urg(U)/Alr(M)/Resp(N)

uM159ErrorInformation				ErrorInformation,

--	NEXT DATA AUTHORITY [icaofacilitydesignator] Urg(L)/Alr(N)/Resp(N)

uM160ICAOFacilityDesignator 			ICAOFacilityDesignator,

--	END SERVICE					Urg(L)/Alr(N)/Resp(N)

uM161NULL 					NULL,

--	SERVICE UNAVAILABLE			Urg(L)/Alr(L)/Resp(N )

uM162NULL 					NULL,

--	[icaofacilitydesignator]				Urg(L)/Alr(N)/Resp(N)

uM163ICAOFacilityDesignator			ICAOFacilityDesignator,

--	WHEN READY					Urg(L)/Alr(N)/Resp(N)

uM164NULL 					NULL,

--	THEN						Urg(L)/Alr(N)/Resp(N)

uM165NULL 					NULL,

--	DUE TO [traffictype]TRAFFIC			Urg(L)/Alr(N)/Resp(N)

uM166TrafficType 				TrafficType,

--	DUE TO AIRSPACE RESTRICTION		Urg(L)/Alr(N)/Resp(N)

uM167NULL 					NULL,

--	DISREGARD					Urg(N)/Alr(M)/Resp(R)

uM168NULL 					NULL,

--	[freetext]					Urg(N)/Alr(L)/Resp(R)

uM169FreeText					FreeText,

-- 	[freetext]					Urg(D)/Alr(H)/Resp(R)

uM170FreeText 					FreeText,

--	CLIMB AT [verticalRate] MINIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM171VerticalRate				VerticalRate,

--	CLIMB AT [verticalRate] MAXIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM172VerticalRate				VerticalRate,

--	DESCEND AT [verticalRate] MINIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM173VerticalRate				VerticalRate,

--	DESCEND AT [verticalRate] MAXIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM174VerticalRate				VerticalRate,

--	REPORT REACHING [altitude]			Urg(N)/Alr(M)/Resp(R)

uM175Altitude					Altitude,

--	MAINTAIN OWN SEPARATION AND VMC	Urg(N)/Alr(M)/Resp(W/U)

uM176NULL					NULL,

--	AT PILOTS DISCRETION			Urg(L)/Alr(L)/Resp(N)

uM177NULL					NULL,

--	[freetext]					Urg(N)/Alr(L)/Resp(N)

uM178FreeText					FreeText,

--	SQUAWK IDENT				Urg(N)/Alr(M)/Resp(W/U)

uM179NULL					NULL,

--	REPORT REACHING BLOCK [altitude] TO [altitude]													Urg(N)/Alr(M)/Resp(R)

uM180AltitudeAltitude				AltitudeAltitude,

--	REPORT DISTANCE [tofrom] [position]		Urg(N)/Alr(M)/Resp(Y)

uM181ToFromPosition				ToFromPosition,

--	CONFIRM ATIS CODE				Urg(N)/Alr(M)/Resp(Y)

uM182NULL					NULL,

--	[freetext]					Urg(N)/Alr(M)/Resp(N)

uM183FreeText					FreeText,

--	AT [time] REPORT DISTANCE [tofrom] [position]	Urg(N)/Alr(M)/Resp(Y)

uM184TimeToFromPosition			TimeToFromPosition,

--	AFTER PASSING [position] CLIMB TO AND MAINTAIN [altitude]												Urg(N)/Alr(M)/Resp(W/U)

uM185PositionAltitude				PositionAltitude,

--	AFTER PASSING [position] DESCEND TO AND MAINTAIN [altitude]											Urg(N)/Alr(M)/Resp(W/U)

uM186PositionAltitude				PositionAltitude,

--	[freetext]					Urg(L)/Alr(N)/Resp(N)

uM187FreeText					FreeText,

--	AFTER PASSING [position] MAINTAIN [speed]	Urg(N)/Alr(M)/Resp(W/U)

uM188PositionSpeed				PositionSpeed,

--	ADJUST SPEED TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM189Speed					NULL,

--	FLY HEADING [degrees]				Urg(N)/Alr(M)/Resp(W/U)

uM190Degrees					Degrees,

--	ALL ATS TERMINATED			Urg(N)/Alr(M)/Resp(R)

uM191NULL					NULL,

--	REACH [altitude] BY [time]			Urg(N)/Alr(M)/Resp(W/U)

uM192AltitudeTime				AltitudeTime,

--	IDENTIFICATION LOST			Urg(N)/Alr(M)/Resp(R)

uM193NULL					NULL,

--	[freetext]					Urg(N)/Alr(L)/Resp(Y)

uM194FreeText					FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(R)

uM195FreeText					FreeText,

--	[freetext]					Urg(N)/Alr(VA/Resp(W/U)

uM196FreeText					FreeText,

--	[freetext]					Urg(U)/Alr(M)/Resp(W/U)

uM197FreeText					FreeText,

--	[freetext]					Urg(D)/Alr(H)/Resp(W/U)

uM198FreeText					FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(W/U)

uM199FreeText					FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(R)

uM200FreeText					FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(W/U)

uM201FreeText					FreeText,

--	[freetext]					Urg(D)/Alr(H)/Resp(W/U)

uM202FreeText					FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(R)

uM203FreeText					FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(Y)

uM204FreeText					FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(A/N)

uM205FreeText					FreeText,

--	[freetext]					Urg(L)/Alr(N)/Resp(Y)

uM206FreeText					FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(Y)

uM207FreeText					FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(N)

uM208FreeText					FreeText,

--	REACH [altitude] BY [position]			Urg(N)/Alr(M)/Resp(W/U)

uM209AltitudePosition				AltitudePosition,

--	IDENTIFIED [position]				Urg(N)/Alr(M)/Resp(R)

uM210Position					Position,

--	REQUEST FORWARDED			Urg(N)/Alr(L)/Resp(N)

uM211NULL					NULL,

--	[icaofacilitydesignator] ATIS [atiscode] CURRENT	Urg(N)/Alr(M)/Resp(R)

uM212ICAOFacilityDesignatorATISCode		ICAOFacilityDesignatorATISCode,

--	[icaofacilitydesignator] ALTIMETER [altimeter]	Urg(N)/Alr(M)/Resp(R)

uM213ICAOFacilityDesignatorAltimeter		ICAOFacilityDesignatorAltimeter,

--	RUNWAY [runway] VISUAL RANGE [rvr]	Urg(N)/Alr(M)/Resp(R)

uM214RunwayRVR				RunwayRVR,

--	TURN [degrees][direction]			Urg(N)/Alr(M)/Resp(W/U)

uM215DegreesDirection				DegreesDirection,

--	REQUEST FLIGHT PLAN			Urg(N)/Alr(M)/Resp(Y)

uM216NULL					NULL,

--	REPORT ARRIVAL				Urg(N)/Alr(M)/Resp(Y)

uM217NULL					NULL,

--	REQUEST ALREADY RECEIVED		Urg(L)/Alr(N)/Resp(N)

uM218NULL					NULL,

--	STOP CLIMB AT [altitude]			Urg(U)/Alr(M)/Resp(W/U)

uM219Altitude					Altitude,

--	STOP DESCENT AT [altitude]			Urg(U)/Alr(M)/Resp(W/U)

uM220Altitude					Altitude,

--	STOPTURN HEADING [degrees]			Urg(U)/Alr(M)/Resp(W/U)

uM221Degrees					Degrees,

--	NO SPEED RESTRICTION			Urg(L)/Alr(L)/Resp(R)

uM222NULL					NULL,

,

--	REDUCE TO MINIMUM APPROACH SPEED	Urg(N)/Alr(L)/Resp(W/U)

uM223NULL					NULL,

--	NO DELAY EXPECTED				Urg(N)/Alr(L)/Resp(R)

uM224NULL					NULL,

--	DELAY NOT DETERMINED			Urg(N)/Alr(L)/Resp(R)

uM225NULL					NULL,

--	EXPECTED APPROACH TIME [time]		Urg(N)/Alr(L)/Resp(R)

uM226Time					Time,

--	LOGICAL ACKNOWLEDGMENT		Urg(N)/Alr(M)/Resp(N)

uM227NULL					NULL,

--	REPORT ETA [position]				Urg(L)/Alr(L)/Resp(Y)

uM228 Position					Position,

--	REPORT ALTERNATE AERODROME		Urg(L)/Alr(L)/Resp(Y)

uM229NULL					NULL,

--	IMMEDIATELY					Urg(D)/Alr(H)/Resp(N)

uM230NULL					NULL,

--	STATE PREFERRED ALTITUDE			Urg(L)/Alr(L)/Resp(Y)

uM231NULL					NULL,

--	STATE-TOP-OF-DESCENT			Urg(L)/Alr(L)/Resp(Y)

uM232NULL					NULL,

--	USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED													Urg(N)/Alr(M)/Resp(N)

uM233NULL					NULL,

--	FLIGHT PLAN NOT HELD			Urg(L)/Alr(L)/Resp(N)

uM234NULL					NULL

...

}

Robs error’s from ADSP

-- ----------------------------------------------------------------------------------

-- Downlink message element

-- ----------------------------------------------------------------------------------

ATCDownlinkMsgElementId :: = CHOICE

{

--	WILCO						Urg(N)/Alr(M)/Resp(N)

dM0NULL 					NULL,

--	UNABLE					Urg(N)/Alr(M)/Resp(N)

dM1NULL 					NULL,

--	STANDBY					Urg(N)/Alr(M)/Resp(N)

dM2NULL 					NULL,

--	ROGER						Urg(N)/Alr(M)/Resp(N)

dM3NULL 					NULL,

--	AFFIRM					Urg(N)/Alr(M)/Resp(N)

dM4NULL 					NULL,

--	NEGATIVE					Urg(N)/Alr(M)/Resp(N)

dM5NULL 					NULL,

--	REQUEST [altitude]				Urg(N)/Alr(L)/Resp(Y)

dM6Altitude 					Altitude,

--	REQUEST BLOCK [altitude] TO [altitude]		Urg(N)/Alr(L)/Resp(Y)

dM7AltitudeAltitude 				AltitudeAltitude,

--	REQUEST CRUISE CLIMB TO [altitude]		Urg(N)/Alr(L)/Resp(Y)

dM8Altitude 					Altitude,

--	REQUEST CLIMB TO [altitude]			Urg(N)/Alr(L)/Resp(Y)

dM9Altitude					Altitude,

--	REQUEST DESCENT TO [altitude]		Urg(N)/Alr(L)/Resp(Y)

dM10Altitude 					Altitude,

--	AT [position] REQUEST CLIMB TO [altitude] 	Urg(N)/Alr(L)/Resp(Y)

dM11PositionAltitude 				PositionAltitude,

--	AT [position] REQUEST DESCENT TO [altitude]	Urg(N)/Alr(L)/Resp(Y)

dM12PositionAltitude 				PositionAltitude,

--	AT [time] REQUEST CLIMB TO [altitude]		Urg(N)/Alr(L)/Resp(Y)

dM13TimeAltitude 				TimeAltitude,

--	AT [time] REQUEST DESCENT TO [altitude]	Urg(N)/Alr(L)/Resp(Y)

dM14TimeAltitude				TimeAltitude,

--	REQUEST OFFSET [distanceOffset] [direction] OF ROUTE

Urg(N)/Alr(L)/Resp(Y)

dM15DistanceOffsetDirection			DistanceOffsetDirection,

--	AT [position] REQUEST OFFSET [distanceOffset] [direction] OF ROUTE											Urg(N)/Alr(L)/Resp(Y)

dM16PositionDistanceOffsetDirection		PositionDistanceOffsetDirection,

--	AT [time] REQUEST OFFSET [distanceOffset] [direction] OF ROUTE											Urg(N)/Alr(L)/Resp(Y)

dM17TimeDistanceOffsetDirection			TimeDistanceOffsetDirection,

--	REQUEST [speed]				Urg(N)/Alr(L)/Resp(Y)

dM18Speed 					Speed,

--	REQUEST [speed] TO [speed]			Urg(N)/Alr(L)/Resp(Y)

dM19SpeedSpeed 				SpeedSpeed,

--	REQUEST VOICE CONTACT			Urg(N)/Alr(L)/Resp(Y)

dM20NULL 					NULL,

--	REQUEST VOICE CONTACT [frequency]		Urg(N)/Alr(L)/Resp(Y)

dM21Frequency 					Frequency,

--	REQUEST DIRECT TO [position]			Urg(N)/Alr(L)/Resp(Y)

dM22Position 					Position,

--	REQUEST [procedureName]			Urg(N)/Alr(L)/Resp(Y)

dM23ProcedureName 				ProcedureName,

--	REQUEST [routeClearance]			Urg(N)/Alr(L)/Resp(Y)

dM24RouteClearance 				RouteClearance,

--	REQUEST [clearanceType] CLEARANCE		Urg(N)/Alr(L)/Resp(Y)

dM25ClearanceType 				ClearanceType,

--	REQUEST WEATHER DEVIATION TO  [position] VIA [routeClearance]											Urg(N)/Alr(L)/Resp(Y)

dM26PositionRouteClearance 			PositionRouteClearance,

--	REQUEST WEATHER DEVIATION UP  TO [distanceOffset] [direction] OF ROUTE										Urg(N)/Alr(L)/Resp(Y)

dM27DistanceOffsetDirection			DistanceOffsetDirection,

--	LEAVING [altitude]				Urg(N)/Alr(L)/Resp(Y)

dM28Altitude 					Altitude,

--	CLIMBING TO [altitude]				Urg(N)/Alr(M)/Resp(N)

dM29Altitude 					Altitude,

--	DESCENDING TO [altitude]			Urg(N)/Alr(M)/Resp(N)

dM30Altitude 					Altitude,

--	PASSING [position]				Urg(N)/Alr(M)/Resp(N)

dM31Position 					Position,

--	PRESENT ALTITUDE [altitude]			Urg(N)/Alr(M)/Resp(N)

dM32Altitude 					Altitude,

--	PRESENT POSITION [position]			Urg(N)/Alr(M)/Resp(N)

dM33Position 					Position,

--	PRESENT SPEED [speed]			Urg(N)/Alr(M)/Resp(N)

dM34Speed 					Speed,

--	PRESENT HEADING [degrees]			Urg(N)/Alr(M)/Resp(N)

dM35Degrees 					Degrees,

--	PRESENT GROUND TRACK [degrees]		Urg(N)/Alr(M)/Resp(N)

dM36Degrees 					Degrees,

--	LEVEL [altitude]					Urg(N)/Alr(M)/Resp(N)

dM37Altitude 					Altitude,

--	ASSIGNED ALTITUDE [altitude]			Urg(N)/Alr(M)/Resp(N)

dM38Altitude 					Altitude,

--	ASSIGNED SPEED [speed]			Urg(N)/Alr(M)/Resp(N)

dM39Speed 					Speed,

--	ASSIGNED ROUTE [routeClearance]		Urg(N)/Alr(M)/Resp(N)

dM40RouteClearance 				RouteClearance,

--	BACK ON ROUTE				Urg(N)/Alr(M)/Resp(N)

dM41NULL 					NULL,

--	NEXT WAYPOINT [position]			Urg(N)/Alr(M)/Resp(N)

dM42Position 					Position,

--	NEXT WAYPOINT ETA [time]			Urg(N)/Alr(M)/Resp(N)

dM43Time 					Time,

--	ENSUING WAYPOINT [position]			Urg(N)/Alr(M)/Resp(N)

dM44Position 					Position,

--	REPORTED WAYPOINT [position]		Urg(N)/Alr(M)/Resp(N)

dM45Position 					Position,

--	REPORTED WAYPOINT [time]			Urg(N)/Alr(M)/Resp(N)

dM46Time 					Time,

--	SQUAWKING [beaconcode]			Urg(N)/Alr(M)/Resp(N)

dM47BeaconCode 				BeaconCode,

--	POSITION REPORT [positionreport]		Urg(N)/Alr(M)/Resp(N)

dM48PositionReport 				PositionReport,

--	WHEN CAN WE EXPECT [speed]		Urg(L)/Alr(L)/Resp(Y)

dM49Speed 					Speed,

--	WHEN CAN WE EXPECT [speed] TO [speed]	Urg(L)/Alr(L)/Resp(Y)

dM50SpeedSpeed 				SpeedSpeed,

--	WHEN CAN WE EXPECT BACK ON ROUTE	Urg(L)/Alr(L)/Resp(Y)

dM51NULL 					NULL,

--	WHEN CAN WE EXPECT LOWER ALTITUDE	Urg(L)/Alr(L)/Resp(Y)

dM52NULL 					NULL,

--	WHEN CAN WE EXPECT HIGHER ALTITUDE	Urg(L)/Alr(L)/Resp(Y)

dM53NULL 					NULL,

--	WHEN CAN WE EXPECT CRUISE CLIMB TO [altitude]													Urg(L)/Alr(L)/Resp(Y)

dM54Altitude 					Altitude,

--	PAN PAN PAN					Urg(U)/Alr(H)/Resp(N)

dM55NULL 					NULL,

--	MAYDAY MAYDAY MAYDAY			Urg(D)/Alr(H)/Resp(N)

dM56NULL 					NULL,

--	[remainingFuel] OF FUEL REMAINING AND [soulsonboard] SOULS ON BOARD										Urg(U)/Alr(H)/Resp(N)

dM57RemainingFuelSoulsOnBoard 		RemainingFuelSoulsOnBoard,

--	CANCEL EMERGENCY				Urg(U)/Alr(M)/Resp(N)

dM58NULL 					NULL,

--	DIVERTING TO [position] VIA [routeClearance]	Urg(U)/Alr(H)/Resp(N)

dM59PositionRouteClearance 			PositionRouteClearance,

--	OFFSETTING [distanceOffset] [direction] OF ROUTE													Urg(U)/Alr(H)/Resp(N)

dM60DistanceOffsetDirection 			DistanceOffsetDirection,

--	DESCENDING TO [altitude]			Urg(U)/Alr(H)/Resp(N)

dM61Altitude 					Altitude,

--	ERROR [errorInformation]			Urg(U)/Alr(L)/Resp(N)

dM62ErrorInformation 				ErrorInformation,

--	NOT CURRENT DATA AUTHORITY		Urg(L)/Alr(L)/Resp(N)

dM63NULL 					NULL,

--	[icaofacilitydesignator]				Urg(L)/Alr(L)/Resp(N)

dM64ICAOFacilityDesignator 			ICAOFacilityDesignator,

--	DUE TO WEATHER				Urg(L)/Alr(L)/Resp(N)

dM65NULL 					NULL,

--	DUE TO AIRCRAFT PERFORMANCE		Urg(L)/Alr(L)/Resp(N)

dM66NULL 					NULL,

--	[freetext]					Urg(N)/Alr(L)/Resp(N)

dM67FreeText 					FreeText,

--	[freetext]					Urg(D)/Alr(H)/Resp(Y)

dM68FreeText 					FreeText,

--	REQUEST VMC DESCENT			Urg(N)/Alr(L)/Resp(Y)

dM69NULL					NULL,

--	REQUEST HEADING [degrees]			Urg(N)/Alr(L)/Resp(Y)

dM70Degrees  					Degrees,

--	REQUEST GROUND TRACK [degrees]		Urg(N)/Alr(L)/Resp(Y)

dM71Degrees					Degrees,

--	REACHING [altitude]				Urg(N)/Alr(M)/Resp(N)

dM72Altitude					Altitude,

--	[versionnumber]					Urg(L)/Alr(L)/Resp(N)

dM73Versionnumber				VersionNumber,

--	REQUEST TO MAINTAIN OWN SEPARATION AND VMC												Urg(L)/Alr(L)/Resp(Y)

dM74NULL					NULL,

--	AT PILOTS DISCRETION			Urg(L)/Alr(L)/Resp(N)

dM75NULL					NULL,

--	REACHING BLOCK [altitude] TO [altitude]	Urg(N)/Alr(N)/Resp(N)

dM76AltitudeAltitude				AltitudeAltitude,

--	ASSIGNED BLOCK [altitude] TO [altitude]	Urg(N)/Alr(N)/Resp(N)

dM77AltitudeAltitude				AltitudeAltitude,

--	AT [time] [distance] [tofrom] [position]		Urg(N)/Alr(N)/Resp(N)

dM78TimeDistanceToFromPosition		TimeDistanceToFromPosition,

--	ATIS [atiscode]					Urg(N)/Alr(M)/Resp(N)

dM79AtisCode					AtisCode,

--	DEVIATING [distanceOffset] [direction] OF ROUTE													Urg(N)/Alr(M)/Resp(N)

dM80DistanceOffsetDirection			DistanceOffsetDirection,

--	WE CAN ACCEPT [altitude] AT [time]		Urg(N)/Alr(L)/Resp(N)

dM81AltitudeTime				AltitudeTime,

--	WE CANNNOT ACCEPT [altitude]		Urg(N)/Alr(L)/Resp(N)

dM82Altitude					Altitude,

--	WE CAN ACCEPT [speed] AT [time]		Urg(N)/Alr(L)/Resp(N)

dM83SpeedTime					SpeedTime,

--	WE CANNNOT ACCEPT [speed]			Urg(N)/Alr(L)/Resp(N)

dM84Speed					Speed,

--	WE CAN ACCEPT [direction] [distanceOffset]	Urg(N)/Alr(L)/Resp(N)

dM85DirectionDistanceOffset			DistanceOffsetDirection,

--	WE CANNNOT ACCEPT [direction] [distanceOffset]													Urg(N)/Alr(L)/Resp(N)

dM86DirectionDistanceOffset			DistanceOffsetDirection,

--	WHEN CAN WE EXPECT CLIMB TO [altitude]	Urg(L)/Alr(L)/Resp(Y)

dM87Altitude					Altitude,

--	WHEN CAN WE EXPECT DESCENT TO [altitude]													Urg(L)/Alr(L)/Resp(Y)

dM88Altitude					Altitude,

--	MONITORING[icaounitname] [frequency]		Urg(L)/Alr(M)/Resp(N)

dM89IcaounitnameFrequency			IcaoUnitNameFrequency,

--	[freetext]					Urg(N)/Alr(M)/Resp(N)

dM90FreeText 					FreeText,

--	[freetext]					Urg(N)/Alr(L)/Resp(Y)

dM91FreeText 					FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(Y)

dM92FreeText 					FreeText,

--	[freetext]					Urg(U)/Alr(H)/Resp(N)

dM93FreeText 					FreeText,

--	[freetext]					Urg(D)/Alr(H)/Resp(N)

dM94FreeText 					FreeText,

--	[freetext]					Urg(U)/Alr(M)/Resp(N)

dM95FreeText 					FreeText,

--	[freetext]					Urg(U)/Alr(L)/Resp(N)

dM96FreeText 					FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(N)

dM97FreeText 					FreeText,

--	[freetext]					Urg(N)/Alr(N)/Resp(N)

dM98FreeText 					FreeText,

--	CURRENT DATA AUTHORITY			Urg(L)/Alr(L)/Resp(N)

dM99NULL 					NULL,

--	LOGICAL ACKNOWLEDGMENT		Urg(N)/Alr(M)/Resp(N)

dM100NULL 					NULL,

REQUEST END OF SERVICE			Urg(L)/Alr(L)/Resp(Y)

dM101NULL 					NULL,

--	LANDING REPORT				Urg(N)/Alr(N)/Resp(N)

dM102NULL 					NULL,

--	CANCELLING IFR				Urg(N)/Alr(N)/Resp(Y)

dM103NULL 					NULL,

--	ETA[position][time]				Urg(L)/Alr(L)/Resp(N)

dM104PositionTime 				Position Time,

--	ALTERNATE AERODROME[airport]		Urg(L)/Alr(L)/Resp(N)

dM105Airport 					Airport,

--	PREFERRED LEVEL[altitude]			Urg(L)/Alr(L)/Resp(N)

dM106Altitude 					Altitude,

--	NOT AUTHORIZED NEXT DATA AUTHORITY	Urg(L)/Alr(L)/Resp(N)

dM107NULL					NULL,

--	DE-ICING COMPLETE				Urg(L)/Alr(L)/Resp(N)

dM108NULL 					NULL,

--	TOP OF DESCENT [time]			Urg(L)/Alr(L)/Resp(N),

dM109Time					Time,

--	TOP OF DESCENT [position]			Urg(L)/Alr(L)/Resp(N),

dM110Position					Position,

--	TOP OF DESCENT [time] [position]		Urg(L)/Alr(L)/Resp(N),

dM111TimePosition				TimePosition

...

}

AircraftEquipmentCode :: = SEQUENCE

{

approachEquipmentAvailable	BOOLEAN,

approachEquipmentStatus		SEQUENCE SIZE (1..16) OF COMNAVEquipmentStatus  													OPTIONAL,

sSREquipmentAvailable		SSREquipmentAvailable

}

AircraftFlightIdentification :: = IA5String (SIZE (2..8))

AircraftType :: = IA5String (SIZE (2..5))

Airport :: = IA5String (SIZE (4))

AirwayIdentifier :: = IA5String (SIZE (1..5))

AirwayIntercept :: = IA5String (SIZE (1..5))

Altimeter :: =  CHOICE

{

altimeterEnglish		AltimeterEnglish,

altimeterMetric		AltimeterMetric

}

AltimeterEnglish :: = INTEGER (2200..3200)

-- unit = Inchs Mercury, range (22.00 .. 32.00), resolution = 0.01

AltimeterMetric :: = INTEGER  (7500..12500)

-- unit = HectoPascal, Range (750.0..1250.0), resolution = 0.1

Altitude :: =  CHOICE

{

altitudeQnh		AltitudeQnh,

altitudeQnhmeters	AltitudeQnhmeters,

altitudeQfe 		AltitudeQfe,

altitudeQfemeters 	AltitudeQfemeters,

altitudeGnssfeet	 	AltitudeGnssfeet,

altitudeGnssmeters 	AltitudeGnssmeters,

altitudeFlightLevel 	AltitudeFlightLevel,

altitudeFlightLevelMetric	AltitudeFlightLevelMetric

}

AltitudeAltitude :: =  SEQUENCE SIZE (2) OF Altitude

AltitudeDepartureName :: = SEQUENCE

{

altitude			Altitude,

procedureName		ProcedureName

}

AltitudeFlightLevel :: = INTEGER (30..600)

--unit = Level (100 Feet), Range (030..600), resolution = 1

AltitudeFlightLevelMetric :: = INTEGER (100..2000)

--unit = Level (10 Meters), Range (100..2000), resolution = 1

AltitudeGnssfeet :: =  INTEGER (0..150000)

--unit = Foot, Range (0..150000), resolution = 1

AltitudeGnssmeters :: = INTEGER (0..50000)

--unit = Meter, Range (0..50000), resolution = 1

AltitudePosition :: = SEQUENCE

{

altitude			Altitude,

position			Position

}

AltitudeQfe :: =  INTEGER (0..2100)

--unit = Feet, Range (0..21000), resolution = 10

AltitudeQfemeters :: = INTEGER (0..7000)

--unit = Meter, Range (0..7000), resolution = 1

AltitudeQnh :: =  INTEGER (0..2500)

--unit = Feet, Range (0..25000), resolution = 10

AltitudeQnhmeters :: = INTEGER (0..16000)

--unit = Meter, Range (0..16000), resolution = 1

AltitudeSpeed :: = SEQUENCE

{

altitude			Altitude,

speed			Speed

}

AltitudeSpeedSpeed :: = SEQUENCE

{

altitude			Altitude,

speeds			SpeedSpeed

}

AltitudeTime :: = SEQUENCE

{

altitude			Altitude,

time			Time

}

ATISCode :: = IA5String (SIZE (1))

ATWAlongTrackWaypoint :: = SEQUENCE

{

position 			Position,

aTWDistance 		ATWDistance,

speed 			Speed			OPTIONAL,

aTWAltitudes		ATWAltitudeSequence	OPTIONAL

}

ATWAltitude :: = SEQUENCE

{

atw		ATWAltitudeTolerance,

altitude		Altitude

}

ATWAltitudeSequence :: = SEQUENCE SIZE (1..2) OF ATWAltitude

ATWAltitudeTolerance :: = ENUMERATED

{

at		(0),

atorabove	(1),

atorbelow	(2)

}

ATWDistance :: = SEQUENCE

{

ATWDistanceTolerance,

Distance

}

ATWDistanceTolerance :: = ENUMERATED

{

plus		(0),

minus		(1)

}

BeaconCode :: =  SEQUENCE SIZE (4) OF BeaconCodeOctalDigit

BeaconCodeOctalDigit :: = INTEGER (0..7)

ClearanceType :: = SEQUENCE

{

clearanceTypes		ClearanceTypes	OPTIONAL

}

ClearanceTypes :: = ENUMERATED

{

approach	(0),

departure	(1),

further		(2),

start-up		(3),

pushback	(4),

taxi		(5),

take-off		(6),

landing		(7),

oceanic		(8),

en-route		(9),

downstream	(10)

...

}

COMNAVEquipmentStatus :: = ENUMERATED

{

aloranA					(0),

cloranC					(1),

ddme					(2),

edecca					(3),

fadf					(4),

ggnss					(5),

hhfRTF					(6),

iinertialNavigation			(7),

lils					(8),

momega					(9),

ovor 					(10),

pdoppler				(11),

rrnavRouteEquipment			(12),

ttacan					(13),

uuhfRTF				(14),

vvhfRTF				(15)

...

}

ControlledTime :: = SEQUENCE

{

time		Time,

timeTolerance	TimeTolerance

}

Date :: = SEQUENCE

{

year		Year,

month		Month,

day		Day

}

DateTimeGroup :: = SEQUENCE

{

date		Date,

timehhmmss	Timehhmmss

}

Day :: = INTEGER (1..31)

--unit = Day, Range (1..31), resolution = 1

DegreeIncrement :: = INTEGER (1..20)

--unit = Degree, Range (1..20), resolution = 1

Degrees :: =  CHOICE

{

degreesMagnetic		DegreesMagnetic,

degreesTrue 		DegreesTrue

}

DegreesMagnetic :: = INTEGER (1..360)

--unit = degree, Range (1..360), resolution = 1

DegreesTrue :: = INTEGER (1..360)

--unit = degree, Range (1..360), resolution = 1

DepartureClearance :: = SEQUENCE

{

aircraftFlightIdentification	AircraftFlightIdentification,

clearanceLimit			Position,

flightInformation			FlightInformation	OPTIONAL,

furtherInstructions		FurtherInstructions	OPTIONAL

}

DepartureMinimumInterval :: = Integer (1..150)

--unit = Minute, Range (0.1..15.0), resolution = 0.1

Direction :: = ENUMERATED

{

left			(0),

right			(1),

eitherSide		(2),

north			(3),

south			(4),

east			(5),

west			(6),

northEast  		(7),

northWest		(8),

southEast 		(9),

southWest		(10)

}

DirectionDegrees :: =  SEQUENCE

{

direction		Direction,

degrees			Degrees

}

Distance :: = CHOICE

{

distanceNm 	DistanceNm,

distanceKm 	DistanceKm

}

DistanceKm :: = INTEGER (1..1024)

--	unit = Kilometer, Range (1..1024), resolution = 1

DistanceNm :: = INTEGER (0..9999)

--	unit = Nautical Mile, Range (0..999.9), resolution = 0.1

DistanceOffset :: = CHOICE

{

distanceOffsetNm	DistanceOffsetNm,

distanceOffsetKm 	DistanceOffsetKm

}

DistanceOffsetDirection :: =  SEQUENCE

{

distanceOffset		DistanceOffset,

direction		Direction

}

DistanceOffsetKm :: = INTEGER (1..256)

--	unit = Kilometer, Range (1..256), resolution = 1

DistanceOffsetNm :: = INTEGER (1..128)

--	unit = Nautical Mile, Range (1..128), resolution = 1

DistanceToNextPoint :: = CHOICE

{

distancetonextpointenglish	DistanceToNextPointEnglish,

distancetonextpointmetric		DistanceToNextPointMetric

}

DistanceToNextPointEnglish :: = INTEGER (1..1024)

--unit = Nautical Mile, Range (1..1024), resolution = 1

DistanceToNextPointMetric :: = INTEGER (1..2048)

--unit = Kilometer, Range (1..2048), resolution = 1

ErrorInformation :: =  ENUMERATED

{

unrecognizedMsgReferenceNumber	(0),

endServiceWithPendingMsgs 		(1),

logicalAcknowledgmentNotAccepted	(2)

...

}

FixName :: = IA5String (SIZE (1..5))

FlightInformation :: = CHOICE

{

routeOfFlight		RouteInformation,

levelsOfFlight		LevelsOfFlight,

routeAndLevels		RouteAndLevels

}

FreeText :: =  IA5String (SIZE (1..256))

Frequency :: =  CHOICE

{

frequencyhf		Frequencyhf,

frequencyvhf		Frequencyvhf,

frequencyuhf		Frequencyuhf,

frequencysatchannel	Frequencysatchannel

}

Frequencyhf :: = INTEGER (2850..28000)

--	unit = Kilohertz, Range (2850..28000), resolution = 1

Frequencysatchannel :: = NumericString (SIZE (12))

--	Frequencysatchannel corresponds to a 12 digit telephone number

Frequencyuhf  :: = INTEGER  (9000..15999)

--	unit = Megahertz, Range (225.000..399.975), resolution = 0.025

Frequencyvhf  :: = INTEGER (4680..5520)

--	unit = Megahertz, Range (117.000..138.000), resolution = 0.025

FurtherInstructions :: = SEQUENCE

{

beaconCode		BeaconCode			OPTIONAL,

frequencyDeparture	ICAOUnitNameFrequency	OPTIONAL,

clearanceExpiryTime	Time				OPTIONAL,

airportDeparture		Airport				OPTIONAL,

airportDestination	Airport				OPTIONAL,

timeDeparture		TimeDeparture			OPTIONAL,

runwayDeparture		Runway				OPTIONAL,

revisionNumber		RevisionNumber			OPTIONAL,

aTISCode		ATISCode			OPTIONAL

}

Holdatwaypoint :: = SEQUENCE

{

position				Position,

holdatwaypointspeedlow		Speed		OPTIONAL,

aTWaltitude			ATWAltitude	OPTIONAL,

holdatwaypointspeedhigh		Speed		OPTIONAL,

direction			Direction	OPTIONAL,

degrees				Degrees		OPTIONAL,

eFCtime				Time		OPTIONAL,

legtype				LegType		OPTIONAL

}

HoldClearance :: =  SEQUENCE

{

position 			Position,

altitude			Altitude,

degrees			Degrees,

direction		Direction,

legType			LegType		OPTIONAL

}

ICAOFacilityDesignator :: =  IA5String (SIZE (8))

ICAOFacilityFunction :: = ENUMERATED

{

center			(0),

approach 		(1),

tower			(2),

final			(3),

groundControl		(4),

clearanceDelivery	(5),

departure		(6),

control			(7)

}

ICAOFacilityDesignatorAltimeter :: = SEQUENCE

{

iCAOFacilityDesignator		ICAOUnitName,

altimeter			Altimeter

}

ICAOFacilityDesignatorATISCode :: = SEQUENCE

{

iCAOFacilityDesignator		ICAOUnitName,

aTISCode			ATISCode

}

ICAOFacilityIdentification :: =  CHOICE

{

iCAOFacilityDesignator		ICAOFacilityDesignator,

iCAOFacilityName		ICAOFacilityName

}

ICAOFacilityName :: = IA5String (SIZE (3..18))

ICAOUnitName :: =   SEQUENCE

{

iCAOFacilityId		ICAOFacilityIdentification,

iCAOFacilityFunction	ICAOFacilityFunction

}

ICAOUnitNameFrequency :: =  SEQUENCE

{

iCAOUnitName			ICAOUnitName,

frequency			Frequency

}

InterceptCourseFrom :: = SEQUENCE

{

fromSelection		InterceptCourseFromSelection,

degrees			Degrees

}

InterceptCourseFromSelection :: = CHOICE

{

publishedIdentifier 		PublishedIdentifier,

latitudeLongitude 		LatitudeLongitude,

placeBearingPlaceBearing 	PlaceBearingPlaceBearing,

placeBearingDistance 		PlaceBearingDistance

}

Icing :: = ENUMERATED

{

trace		(0),

light		(1),

moderate	(2),

severe		(3)

}

Latitude :: = SEQUENCE

{

latitudeDegrees		LatitudeDegrees,

minutesLatLon		MinutesLatLon		OPTIONAL,

latitudeDirection		LatitudeDirection

}

LatitudeDegrees  :: = INTEGER (0..90)

-- unit = Degree, Range (0..90), resolution = 1

LatitudeDirection :: = ENUMERATED

{

north		(0),

south		(1)

}

LatitudeLongitude :: = SEQUENCE

{

latitude		Latitude,

longitude	Longitude

}

LatitudeReportingPoints :: = SEQUENCE

{

latitudeDirection		LatitudeDirection,

latitudeDegrees		LatitudeDegrees

}

LatLonReportingPoints :: = CHOICE

{

latitudeReportingPoints		LatitudeReportingPoints,

longitudeReportingPoints		LongitudeReportingPoints

}

LegDistance :: = CHOICE

{

legDistanceEnglish		LegDistanceEnglish,

legDistanceMetric		LegDistanceMetric

}

LegDistanceEnglish :: = INTEGER (1..999)

-- unit = Nautical Mile, Range (.1..99.9), resolution = 0.1

LegDistanceMetric :: = INTEGER (1..128)

-- unit = Kilometer, Range (1..128), resolution = 1

LegTime :: = INTEGER (1..99)

--unit = Minute, Range (0.1..9.9), resolution = 0.1

LegType :: = CHOICE

{

legDistance	LegDistance,

legTime		LegTime

}

LevelsOfFlight :: = CHOICE

{

altitude			Altitude,

procedureName		ProcedureName,

altitudeProcedureName	AltitudeProcedureName

}

Longitude :: = SEQUENCE

{

longitudeDegrees		LongitudeDegrees,

minutesLatLon		MinutesLatLon		OPTIONAL,

longitudeDirection	LongitudeDirection

}

LongitudeDegrees :: = INTEGER (0..180)

--unit = Degree, Range (0..180), resolution = 1

LongitudeDirection :: = ENUMERATED

{

east		(0),

west		(1)

}

LongitudeReportingPoints :: = SEQUENCE

{

longitudeDirection	LongitudeDirection,

longitudeDegrees		LongitudeDegrees

}

MinutesLatLon :: = INTEGER (0..599)

--unit = Minute, Range (0.. 59.9), resolution = 0.1

Month :: = INTEGER (1..12)

--unit = 1 Month, Range (1..12), resolution = 1

Navaid :: =  IA5String (SIZE (1..4) )

PlaceBearing :: = SEQUENCE

{

fixName			FixName,

latitudeLongitude	LatitudeLongitude	OPTIONAL,

degrees			Degrees

}

PlaceBearingDistance :: = SEQUENCE

{

fixName			FixName,

latitudeLongitude	LatitudeLongitude	OPTIONAL,

degrees			Degrees,

distance			Distance

}

PlaceBearingPlaceBearing :: = SEQUENCE SIZE (2) OF PlaceBearing

PointAltitude :: = SEQUENCE

{

altitudeFlightAltitude	AltitudeFlightAltitude,

aTWAltitudeTolerance	ATWAltitudeTolerance	OPTIONAL

}

PointAltitudeBlock :: = SEQUENCE SIZE (2) OF AltitudeFlightAltitude

PointDetail :: = SEQUENCE

{

latitudeLongitude	LatitudeLongitude,

fixName			FixName		OPTIONAL,

truetrackangle		DegreesTrue		OPTIONAL,

distancetonextpoint	DistanceToNextPoint	OPTIONAL,

pointAltitude		PointAltitude		OPTIONAL,

pointAltitudeBlock	PointAltitudeBlock	OPTIONAL

}

Position :: =  CHOICE

{

fixName			FixName,

navaid			Navaid,

airport			Airport,

latitudeLongitude	LatitudeLongitude,

placeBearingDistance	PlaceBearingDistance

}

PositionAltitude :: = SEQUENCE

{

position			Position,

altitude			Altitude

}

PositionAltitudeAltitude :: = SEQUENCE

{

position			Position,

altitudes      		AltitudeAltitude

}

PositionAltitudeSpeed :: = SEQUENCE

{

positionaltitude		PositionAltitude,

speed			Speed

}

PositionDegrees :: =  SEQUENCE

{

position			Position,

degrees			Degrees

}

PositionDistanceOffsetDirection :: =  SEQUENCE

{

position			Position,

distanceOffsetDirection	DistanceOffsetDirection

}

PositionICAOUnitNameFrequency :: = SEQUENCE

{

positon			Position,

icaounitname		ICAOUnitName,

frequency		Frequency

}

PositionPosition :: =  SEQUENCE SIZE (2) OF Position

PositionProcedureName :: = SEQUENCE

{

position			Position,

procedureName		ProcedureName

}

PositionReport :: = SEQUENCE

{

positioncurrent		 	Position,

timeatpositioncurrent 		Time,

altitude				Altitude,

fixnext				Position		OPTIONAL,

timeetaatfixnext			Time		OPTIONAL,

fixnextplusone			Position		OPTIONAL,

timeetaatdestination		Time		OPTIONAL,

remainingFuel			RemainingFuel	OPTIONAL,

temperature			Temperature	OPTIONAL,

winds	 			Winds		OPTIONAL,

turbulence			Turbulence	OPTIONAL,

icing				Icing		OPTIONAL,

speed				Speed		OPTIONAL,

speedground			SpeedGround	OPTIONAL,

verticalChange			VerticalChange	OPTIONAL,

trackAngle			Degrees		OPTIONAL,

heading				Degrees		OPTIONAL,

distance				Distance		OPTIONAL,

supplementaryInformation	FreeText	OPTIONAL,

reportedWaypointPosition		Position		OPTIONAL,

reportedWaypointTime		Time		OPTIONAL,

reportedWaypointAltitude		Altitude		OPTIONAL

}

PositionRouteClearance :: = SEQUENCE

{

position			Position,

routeClearance		RouteClearance

}

PositionSpeed :: = SEQUENCE

{

position			Position,

speed			Speed

}

PositionSpeedSpeed :: = SEQUENCE

{

position			Position,

speeds			SpeedSpeed

}

PositionTime :: = SEQUENCE

{

position			Position,

time			Time

}

PositionTimeAltitude :: = SEQUENCE

{

positionTime		PositionTime,

altitude			Altitude

}

PositionTimeTime :: = SEQUENCE

{

position			Position,

times 	 		TimeTime

}

Procedure :: = IA5String (SIZE (1..6))

ProcedureName :: = SEQUENCE

{

type			ProcedureType,

procedure		Procedure,

transition		ProcedureTransition	OPTIONAL

}

ProcedureTransition :: = IA5String (SIZE (1..5))

ProcedureType :: = ENUMERATED

{

arrival			(0),

approach		(1),

departure		(2)

}

PublishedIdentifier :: = SEQUENCE

{

fixName			FixName,

latitudeLongitude	LatitudeLongitude	OPTIONAL

}

RemainingFuel :: = Time

RemainingFuelSoulsOnBoard :: =  SEQUENCE

{

remainingFuel	RemainingFuel,

soulsOnBoard	SoulsOnBoard

}

ReportingPoints :: = SEQUENCE

{

latLonReportingPoints	LatLonReportingPoints,

degreeIncrement		DegreeIncrement			OPTIONAL

}

RevisionNumber :: = INTEGER (1..16)

RouteAndLevels :: = SEQUENCE

{

routeOfFlight		RouteInformation,

levelsOfFlight		LevelsOfFlight

}

RouteClearance :: = SEQUENCE

{

airportDeparture			Airport				OPTIONAL,

airportDestination		Airport				OPTIONAL,

runwayDeparture			Runway				OPTIONAL,

procedureDeparture		ProcedureName			OPTIONAL,

runwayArrival			Runway				OPTIONAL,

procedureApproach		ProcedureName			OPTIONAL,

procedureArrival			ProcedureName			OPTIONAL,

airwayIntercept			AirwayIntercept 			OPTIONAL,

routeInformations		SEQUENCE SIZE (1..128)

OF RouteInformation		OPTIONAL,

routeInformationAdditional	RouteInformationAdditional	OPTIONAL

}

RouteInformation :: = CHOICE

{

publishedIdentifier 		PublishedIdentifier,

latitudeLongitude 		LatitudeLongitude,

placeBearingPlaceBearing 	PlaceBearingPlaceBearing,

placeBearingDistance 		PlaceBearingDistance,

airwayIdentifier 			AirwayIdentifier,

trackDetail 			TrackDetail

}

RouteInformationAdditional :: = SEQUENCE

{

aTWAlongTrackWaypoints	SEQUENCE SIZE (1..8) OF ATWAlongTrackWaypoint  OPTIONAL,

reportingpoints			ReportingPoints					       OPTIONAL,

interceptCourseFroms		SEQUENCE SIZE (1..4) OF InterceptCourseFrom	       OPTIONAL,

holdAtWaypoints			SEQUENCE SIZE (1..8) OF Holdatwaypoint	       OPTIONAL,

waypointSpeedAltitudes		SEQUENCE SIZE (1..32) OF WaypointSpeedAltitude     OPTIONAL,

rTARequiredTimeArrivals	SEQUENCE SIZE (1..32) OF RTARequiredTimeArrival													       OPTIONAL

}

RTARequiredTimeArrival :: = SEQUENCE

{

position			Position,

rTATime		RTATime,

rTATolerance		RTATolerance		OPTIONAL

}

RTATime :: = SEQUENCE

{

time			Time,

timeTolerance		TimeTolerance

}

RTATolerance :: = INTEGER (1..150)

--unit= Minute, Range (0.1..15.0), resolution = 0.1

RVR :: = CHOICE

{

rVRFeet 	RVRFeet,

rVRMeters 	RVRMeters

}

RVRFeet :: = INTEGER (0..6100)

-- unit = Feet, Range (0..6100), resolution = 1

RVRMeters :: = INTEGER (0..5000)

-- unit = Meters (0..5000), resolution = 1

Runway :: = SEQUENCE

{

direction		RunwayDirection,

configuration		RunwayConfiguration

}

RunwayDirection :: = INTEGER (1..36)

RunwayConfiguration :: = ENUMERATED

{

left		(0),

right		(1),

center		(2),

none		(3)

}

RunwayRVR :: = SEQUENCE

{

runway			Runway,

rVR			RVR

}

Speed :: =  CHOICE

{

speedIndicated 		SpeedIndicated,

speedIndicatedMetric 	SpeedIndicatedMetric,

speedTrue 		SpeedTrue,

speedTrueMetric 		SpeedTrueMetric,

speedGround 		SpeedGround,

speedGroundMetric 	SpeedGroundMetric,

speedMach 		SpeedMach,

speedMachLarge 	SpeedMachLarge

}

SpeedIndicated :: = INTEGER (7..38)

-- unit = Knots, Range (70..380), resolution = 10

SpeedIndicatedMetric :: = INTEGER (10..137)

-- unit = Kilometers/Hour, Range (100..1370), resolution = 10

SpeedGround :: = INTEGER (7..70)

-- unit = Knots, Range (70..700), resolution = 10

SpeedGroundMetric :: = INTEGER (10..265)

-- unit = Kilometers/Hour, Range (100..2650), resolution = 10

SpeedMach :: = INTEGER (61..92)

-- unit = Mach Range (.61 to .92), resolution = 0.01

SpeedMachLarge :: = INTEGER (93..604)

-- unit = Mach Range (.93 to 6.04), resolution = 0.01

SpeedQualifier :: = SEQUENCE

{

speedQualifiers	SpeedQualifiers	OPTIONAL

}

SpeedQualifiers :: = ENUMERATED

{

approach	(0),

minimum	(1),

cruise		(2)

...

}

SpeedSpeed :: =  SEQUENCE SIZE (2) OF Speed

SpeedTime :: = SEQUENCE

{

speed			Speed,

time			Time

}

SpeedTrue :: = INTEGER (7..70)

-- unit = Knots, Range (70..700), resolution = 10

SpeedTrueMetric :: = INTEGER (10..137)

-- unit = Kilometers/Hour, Range (100..1370), resolution = 10

SpeedType :: = SEQUENCE

{

speedTypes	SpeedTypes	OPTIONAL

}

SpeedTypes :: = ENUMERATED

{

indicated	(0),

true		(1),

ground		(2),

mach		(3),

machLarge	(4)

}

SoulsOnBoard :: = INTEGER (1..1024)

SSREquipmentAvailable :: = ENUMERATED

{

nnil				(0),

atransponderModeA		(1),

ctransponderModeAandC		(2),

xtransponderModeS		(3),

ptransponderModeSPA		(4),

itransponderModeSID		(5),

stransponderModeSPAID		(6)

...

}

--Note: PA Pressure Altitude; ID Aircraft Identification

Temperature :: = CHOICE

{

temperatureC 	TemperatureC,

temperatureF 	TemperatureF

}

TemperatureC :: = INTEGER (-80..47)

-- unit = Degree Centigrade, Range (-80..47), resolution = 1

TemperatureF :: = INTEGER (-105..150)

-- unit = Degree Fahrenheit, Range (-105..150), resolution = 1

Time :: = SEQUENCE

{

hours			TimeHours,

minutes			TimeMinutes

}

TimeAltitude :: = SEQUENCE

{

time		Time,

altitude		Altitude

}

TimeDeparture :: = SEQUENCE

{

timeDepartureAllocated		Time				OPTIONAL,

timeDepartureControlled		ControlledTime			OPTIONAL,

timeDepartureClearanceExpected	Time				OPTIONAL,

departureMinimumInterval	DepartureMinimumInterval	OPTIONAL

}

TimeDistanceOffsetDirection :: = SEQUENCE

{

time			Time,

distanceOffsetDirection	DistanceOffsetDirection

}

TimeDistanceToFromPosition :: =  SEQUENCE

{

time			Time,

distance			Distance,

tofrom			ToFrom,

position			Position

}

Timehhmmss :: = SEQUENCE

{

hoursminutes		Time,

seconds			TimeSeconds

}

TimeHours :: = INTEGER (0..23)

-- unit = Hour, Range (0..23), resolution = 1

TimeICAOUnitNameFrequency :: =  SEQUENCE

{

time			Time,

iCAOUnitName		ICAOUnitName,

frequency		Frequency

}

TimeMinutes :: = INTEGER (0..59)

-- unit = Minute, Range (0..59), resolution = 1

TimePosition :: =  SEQUENCE

{

time			Time,

position			Position

}

TimePositionAltitude :: =    SEQUENCE

{

timeposition		TimePosition,

altitude			Altitude

}

TimePositionAltitudeSpeed :: = SEQUENCE

{

timeposition		TimePosition,

altitudespeed		AltitudeSpeed

}

TimeSeconds :: = INTEGER (0..59)

-- unit = Second, Range (0..59), resolution = 1

TimeSpeed :: =  SEQUENCE

{

time			Time,

speed			Speed

}

TimeSpeedSpeed :: =  SEQUENCE

{

time			Time,

speedspeed	 	SpeedSpeed

}

TimeTime :: = SEQUENCE SIZE (2) OF Time

TimeToFromPosition :: = SEQUENCE

{

time			Time,

tofrom			ToFrom,

position			Position

}

TimeTolerance :: = ENUMERATED

{

at		(0),

atorafter		(1),

atorbefore	(2)

}

ToFrom :: = ENUMERATED

{

to		(0),

from		(1)

}

ToFromPosition :: =  SEQUENCE

{

toFrom		ToFrom,

position		Position

}

TrackDetail :: = SEQUENCE

{

trackName		TrackName,

latitudeLongitudes	SEQUENCE SIZE (1..128) OF LatitudeLongitude

}

TrackDetailMsg :: = SEQUENCE

{

trackname		TrackName,

datetimetrackgenerated	DateTimeGroup,

trackDetailMsgType	TrackDetailMsgType,

datetimetrackstart	DateTimeGroup,

datetimetrackstop	DateTimeGroup,

airportdeparture		Airport,

pointdetails		SEQUENCE SIZE (1..32) OF PointDetail,

airportdestination	Airport,

remarks			FreeText

}

TrackDetailMsgType :: = ENUMERATED

{

provisional	(0),

final		(1)

}

TrackName :: = IA5String (SIZE (3..6))

TrafficType :: = SEQUENCE

{

trafficTypes	TrafficTypes	OPTIONAL

}

TrafficTypes :: = ENUMERATED

{

oppositeDirections	(0),

sameDirectional		(1),

converging		(2),

crossing			(3)

}

Turbulence :: = ENUMERATED

{

light		(0),

moderate	(1),

severe		(2)

}

VersionNumber :: =  INTEGER (0..15)

VerticalChange :: = SEQUENCE

{

direction		VerticalDirection,

rate			VerticalRate

}

VerticalDirection :: = ENUMERATED

{

up	(0),

down	(1)

}

VerticalRate :: =  CHOICE

{

verticalRateEnglish 		VerticalRateEnglish,

verticalRateMetric 		VerticalRateMetric

}

VerticalRateEnglish :: = INTEGER (0..60)

-- unit = Feet/Minute, Range (0..6000), resolution = 100

VerticalRateMetric :: = INTEGER (0..200)

-- unit = Meters/Minute, Range (0..2000), resolution = 10

WaypointSpeedAltitude :: = SEQUENCE

{

position 			Position,

speed			Speed			OPTIONAL,

aTWAltitudes		ATWAltitudeSequence	OPTIONAL

}

WindDirection :: = INTEGER (1..360)

-- unit = Degree, Range (1..360), resolution = 1

Winds :: = SEQUENCE

{

direction		WindDirection,

speed			WindSpeed

}

WindSpeed :: = CHOICE

{

windSpeedEnglish	WindSpeedEnglish,

windSpeedMetric		WindSpeedMetric

}

WindSpeedEnglish :: = INTEGER (0..255)

-- unit = Knot, Range (0..255), resolution = 1

WindSpeedMetric :: = INTEGER (0..511)

-- unit = Kilometer/Hour, Range (0..511), resolution = 1

Year :: = INTEGER (0..99)

-- unit = Year, Range (0..99), resolution = 1

-- Altitude                  Block Altitudes?

END

# PROTOCOL DEFINITION

## Sequence Rules

- Note: — The following figures define the valid sequences of primitives that are possible to be invoked during the operation of the CPDLC application.  It shows the relationship in time between the service request and the resulting indication, and if applicable, the subsequent response and resulting confirmation.

### Only the sequence of primitives, except abort primitives, described below shall  ([50])  be permitted.

- Note: — Abort primitives may interrupt and terminate any of the normal message sequences outlined below.

### With the permissible exception of abort primitives, the CPDLC-air-ASE and CPDLC-ground-ASE shall  ([51])  process primitives in the order in which they are received.

- Note: — This ensures that the CPDLC-ASE will, with the exception of aborts, guarantee message sequencing.

### CPDLC-start Service

#### Air Initiated

- Note: — The following sequence of messages, shown in Figure 5-1, occurs when the CPDLC-Start service is air initiated.

Figure 5-1:  Sequence Diagram for CPDLC-start Service

Air Initiated

#### Ground Initiated

- Note: — The following sequence of messages, shown in Figure 5-2, occurs when the CPDLC-start service is ground initiated.

Figure 5-2:  Sequence Diagram for CPDLC-start Service

Ground Initiated

### DSC-start Service

- Note: —The following sequence of messages, shown in Figure 5-3, occurs when the DSC-start service is initiated.

Figure 5-3:  Sequence Diagram for DSC-start Service

### CPDLC-message Service

#### Air Initiated

- Note:— The following sequence of messages, shown in Figure 5-4, occurs when the CPDLC-message Service is air initiated.

Figure 5-4:  Sequence Diagram for CPDLC-message Service

Air Initiated

#### Ground Initiated

- Note: — The following sequence of messages, shown in Figure 5-5, occurs when the CPDLC-message Service is ground initiated.

Figure 5-5: Sequence Diagram for CPDLC-message Service

Ground Initiated

### CPDLC-end Service

- Note:. — The following sequence of messages, shown in Figure 5-6, occurs when the CPDLC-end service is initiated.

Figure 5-6:  Sequence Diagram for CPDLC-end Service

### DSC-end Service

- Note: — The following sequence of messages, shown in Figure 5-7, occurs when the DSC-end service is initiated.

Figure 5-7:  Sequence Diagram for DSC-end Service

### CPDLC-forward Service

- Note: — The following sequence of messages, shown in Figure 5-8, occurs when the CPDLC forward service is initiated.

Figure 5-8:  Sequence Diagram for CPDLC-forward Service

### CPDLC-user-abort-Service

#### CPDLC-Air-User Initiated

- Note: — The following sequence of messages, shown in Figure 5-8, occurs when the CPDLC-user-abort service is initiated by the CPDLC-air-user.

Figure 5-8:  Sequence Diagram for CPDLC-user-abort Service

CPDLC-Air-User Initiated

#### CPDLC-Ground-User Initiated

- Note: — The following sequence of messages, shown in Figure 5-9, occurs when the CPDLC-user-abort service is initiated by the CPDLC-ground-user.

Figure 5-9:  Sequence Diagram for CPDLC-user-abort Service

CPDLC-Ground-User Initiated

### CPDLC-provider-abort-Service

#### Dialogue Service Abort

- Note: — The following sequence of messages, shown in Figure 5-10, occurs when the dialogue service provider (below the level of the ASE) aborts.

Figure 5-10:  Sequence Diagram for CPDLC-provider-abort Service

Dialogue Service Abort

#### CPDLC-Air-ASE Abort

- Note: — The following sequence of messages, shown in Figure 5-11, occurs when the CPDLC-air-ASE aborts.

Figure 5-11:  Sequence Diagram for CPDLC-provider-abort Service

CPDLC-Air-ASE Abort

#### CPDLC-Ground-ASE Abort

- Note: — The following sequence of messages, shown in Figure 5-12, occurs when the CPDLC-ground-ASE aborts.

Figure 5-12:  Sequence Diagram for CPDLC-provider-abort Service

CPDLC-Ground-ASE Abort

### CPDLC-user-abort-Service (Ground-Ground)

#### CPDLC-Air-User Initiated

- Note: — The following sequence of messages, shown in Figure 5-13, occurs when the CPDLC-user-abort service is initiated by the receiving CPDLC-ground-user.

Figure 5-13:  Sequence Diagram for CPDLC-user-abort Service

Receiving CPDLC-Ground-User Initiated

#### CPDLC-Ground-User Initiated

- Note: — The following sequence of messages, shown in Figure 5-14, occurs when the CPDLC-user-abort service is initiated by the CPDLC-ground-user.

Figure 5-14:  Sequence Diagram for CPDLC-user-abort Service

Sending CPDLC-Ground-User Initiated

### CPDLC-provider-abort-Service (Ground-Ground)

#### Dialogue Service Abort

- Note: — The following sequence of messages, shown in Figure 5-15, occurs when the dialogue service provider (below the level of the ASE) aborts.

Figure 5-15:  Sequence Diagram for CPDLC-provider-abort Service

Dialogue Service Abort

#### Receiving CPDLC-Ground-ASE Abort (Ground-Ground)

- Note: — The following sequence of messages, shown in Figure 5-16, occurs when the receiving CPDLC-ground-ASE aborts.

Figure 5-16:  Sequence Diagram for CPDLC-provider-abort Service

Receiving CPDLC-Ground-ASE Abort

#### Sending CPDLC-Ground-ASE Abort (Ground-Ground)

- Note: — The following sequence of messages, shown in Figure 5-17, occurs when the sending CPDLC-ground-ASE aborts.

Figure 5-17:  Sequence Diagram for CPDLC-provider-abort Service

Sending CPDLC-Ground-ASE Abort

## CPDLC Service Provider Timers

- Note: — Table 5-1 lists the time constraints related to the CPDLC application.  Each time constraint requires a time to be set in the CPDLC protocol machine.

### The CPDLC-ASEs shall  ([52])  measure the time between the initial event and the corresponding final event.

### If the maximum time is exceeded before the final event has occurred, the CPDLC ASEs shall  ([53])  take appropriate action as defined in section 5.3.

#### The action shall  ([54])  be taken when the maximum time has expired.

| CPDLC Service | Timer | Timer Value | Timer Start Event | Timer Stop Event |
| --- | --- | --- | --- | --- |
| CPDLC-start | tstart | 6 minutes | D-START request | D-START confirmation |
| CPDLC-end | tend | 6 minutes | D-END request | D-END confirmation |
| DSC-start | tstart | 6 minutes | D-START request | D-START confirmation |
| DSC-end | tend | 6 minutes | D-END request | D-END confirmation |
| CPDLC-forward | tstart | 6 minutes | D-START request | D-START confirmation |

Table 5-1:  CPDLC Application Timers

## CPDLC ASE Protocol Description

### Introduction

- Note: — This section presents requirements for CPDLC-ASEs in particular states. Section 5.5 contains state tables for the CPDLC ASEs.

#### Where requirements are stated for a CPDLC-ASE in this section, they shall  ([55])  apply to both the CPDLC-air-ASE and the CPDLC-ground-ASE, otherwise the requirements apply to either the CPDLC-air-ASE or the CPDLC-ground-ASE as specified.

#### If no actions are described for a CPDLC service primitive when a CPDLC-ASE is in particular state, then the invocation of that service primitive shall  ([56])  be prohibited while the CPDLC-ASE is in that state.

#### If no actions are described for the arrival of a PDU when a CPDLC-ASE is in a particular state, then exception handling procedures as described in section 5.4 shall  ([57])  apply.

- Note: — The CPDLC-ASE contains a Boolean called DSC.  DSC has the abstract value “true” when the dialogue is a DSC dialogue, and has the abstract value “false” otherwise.

#### On the initiation of a CPDLC-ASE, DSC shall  ([58])  be set to the abstract value “false”.

### D-START Indication

#### Upon receipt of a D-START indication, if the CPDLC-ASE is in the IDLE state, and the D-START User Data parameter contains a GroundPDUs APDU and the APDU element is not an ATCForwardMessage or contains an AircraftPDUs APDU with the APDU-element mode “cpdlc”, the CPDLC-ASE shall ([59])  :

- Invoke CPDLC-start indication containing the following:

  - the D-START Calling Peer ID parameter value as the CPDLC-start service Calling Peer Identifier parameter value,

  - if the AircraftPDUs or GroundPDUs APDU-element contained in the D-START User Data parameter is a CPDLCMessage, set the AircraftPDUs or GroundPDUs APDU-element as the CPDLC-start service CPDLC Message parameter value, and

- Enter the START-IND state.

#### Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the D-START User Data parameter contains a GroundPDUs APDU and the APDU element is an ATCForwardMessage the CPDLC-ground-ASE shall ([60])  :

- Invoke CPDLC-forward indication containing the following:

  - the D-START Calling Peer ID parameter value as the CPDLC-forward service Calling Ground Facility Designator parameter value,

  - if the GroundPDUs APDU-element contained in the D-START User Data parameter is a CPDLCMessage, set the GroundPDUs APDU-element as the CPDLC-forward service CPDLC Message parameter value, and

- Invoke D-START response with the abstract value “rejected (permanent)” as the D-START Result parameter value, and

- Enter the IDLE state.

#### Upon receipt of a D-START indication, if the CPDLC-ASE is in the IDLE state, and the D-START User Data parameter contains an AircraftPDUs APDU with the APDU-element mode “dsc”, the CPDLC-ASE shall ([61])  :

- Invoke DSC-start indication containing the following:

  - the D-START Calling Peer ID parameter value as the DSC-start service Aircraft Identifier parameter value,

  - if the APDU AircraftPDUs APDU contained in the D-START is a CPDLCMessage, set the AircraftPDUs APDU as the DSC-start service CPDLC Message parameter value,

- Set DSC to “true”, and

- Enter the START-IND state.

### D-START Confirmation

#### Upon receipt of a D-START confirmation, if the CPDLC-ASE is in the START-REQ state and if the D-START Result and Reject Source parameters do not have the abstract values “rejected (transient)” and “DS provider” respectively , and DSC has the abstract value of “false”, the CPDLC-ASE shall ([62])  :

- Stop timer tstart,

- Invoke CPDLC-start confirmation containing the following:

  - the AircraftPDUs or GroundPDUs APDU-element contained in the D-START User Data parameter as the CPDLC-start service Reject Reason parameter value, if provided, and

  - the D-START Result parameter value as the CPDLC-start service Result parameter as follows

    - “accepted” as “accepted”, or

    - “rejected (permanent)” as “rejected”, and

- If the D-START Result parameter abstract value is not “accepted” enter the IDLE state, or

- Else enter the DIALOGUE state.

#### Upon receipt of a D-START confirmation, if the CPDLC-air-ASE is in the START-REQ state and if the D-START Result and Reject Source parameters do not have the abstract values “rejected (transient)” and “DS provider” respectively , and DSC has the abstract value of “true”, the CPDLC-air-ASE shall ([63])  :

- Stop timer tstart,

- Invoke DSC-start confirmation containing the following:

  - the GroundPDUs APDU-element contained in the D-START User Data parameter as the DSC-start service Reject Reason parameter value, if provided, and

  - the D-START Result parameter value as the DSC-start Result parameter value as follows

    - “accepted” as “accepted”, or

    - “rejected (permanent)” as “rejected”, and

- If the D-START Result parameter abstract value is not “accepted” enter the IDLE state, or

- Else enter the DIALOGUE state.

#### Upon receipt of a D-START confirmation, if the CPDLC-ground-ASE is in the FORWARD state and if the D-START Result parameter has the abstract value “rejected (permanent)” and the Reject Source parameter has the abstract value “DS user” the CPDLC-ground-ASE shall ([64])  :

- Stop timer tstart,

- Invoke CPDLC-start confirmation, and

- Enter the IDLE  state.

### D-DATA Indication

#### Upon receipt of a D-DATA Indication, if the CPDLC-ASE is in the DIALOGUE state, the CPDLC-ASE shall ([65])  :

- Invoke CPDLC-message indication with the APDU contained in the D-DATA User Data parameter as the CPDLC-message service CPDLC Message parameter value, and

- Remain in the DIALOGUE state.

#### Upon receipt of a D-DATA Indication, if the CPDLC-ground-ASE is in the END state, the CPDLC-ground-ASE shall ([66])  :

- Invoke CPDLC-message indication with the APDU contained in the D-DATA User Data parameter as the CPDLC-message service CPDLC Message parameter value, and

- Remain in the END state.

### D-END Indication

#### Upon receipt of a D-END indication, if the CPDLC-air-ASE is in the DIALOGUE state the CPDLC-air-ASE shall ([67])  :

- Invoke CPDLC-end service indication with the APDU contained in the D-END User Data parameter as the CPDLC-message service CPDLC Message parameter value, if provided, as the CPDLC-end CPDLC Message parameter value, and

- Enter the END state

#### Upon receipt of a D-END indication, if the CPDLC-ground-ASE is in the DIALOGUE state the CPDLC-ground-ASE shall ([68])  :

- Invoke DSC-end service indication with the APDU contained in the D-END User Data parameter as the DSC-end service CPDLC Message parameter, if provided, and

- Enter the END state

### D-END Confirmation

#### Upon receipt of a D-END confirmation, if the CPDLC-ground-ASE is in the END state and the abstract values of the D-END Result and Reject Source are not “rejected (transient)” or “DS provider” respectively, the CPDLC-ground-ASE shall ([69])  :

- Stop timer tend,

- Invoke CPDLC-end service confirmation with:

  - The APDU contained in the D-END User Data parameter as the CPDLC-end CPDLC Message parameter value, if provided, and

  - The D-END Result parameter value as the CPDLC-end service Result parameter value as follows:

    - “accepted as “accepted”, or

    - “rejected (permanent)” as “rejected”, and

- If the D-END Result is “rejected (permanent)” enter the DIALOGUE state, or

- Else enter the IDLE state.

#### Upon receipt of a D-END confirmation, if the CPDLC-air-ASE is in the END state and the abstract values of the D-END Result and Reject Source are not “rejected (transient)” or “DS provider” respectively, the CPDLC-air-ASE shall ([70])  :

- Stop timer tend,

- Invoke DSC-end confirmation with:

  - The APDU contained in the D-END User Data parameter as the DSC-end service CPDLC Message parameter value, if provided, and

  - The D-END Result parameter value as the DSC-end Result parameter value as follows:

    - “accepted as “accepted”, or

    - “rejected (permanent)” as “rejected”, and

- If the D-END Result is “rejected (permanent)” enter the DIALOGUE state, or

- Else enter the IDLE state.

### CPDLC-start Request

#### Upon receipt of a CPDLC-start service request, if the CPDLC-ASE is in the IDLE state, the CPDLC-ASE shall ([71])  :

- If the CPDLC-ASE is a CPDLC-air-ASE, create an AircraftPDUs APDU with a startdown APDU element containing:

  - the abstract value “cpdlc” as the mode,

  - if provided, the CPDLC Message parameter as the DownlinkMessage, or

  - else, NULL as the DownlinkMessage,

- If the CPDLC-ASE is a CPDLC-ground-ASE, create a GroundPDUs APDU with a startup APDU element containing:

  - if provided, the CPDLC Message parameter as the DownlinkMessage, or

  - else, NULL as the DownlinkMessage,

- Invoke D-START request with the following:

  - the CPDLC-start service Called Peer Identifier parameter value as the D-START Called Peer ID parameter value,

  - the CPDLC-start service Calling Peer Identifier parameter value as the D-START Calling Peer ID parameter value,

  - the D-START Quality of Service parameters set as follows:

    - if provided, the CPDLC-start service Class of Communication parameter value as the D-START QOS Routing Class, or

    - The abstract value of “normal priority flight safety messages”, as the D-START QOS Priority parameter value, and

    - The abstract value of “low” as the D-START QOS Residual Error Rate parameter value, and

  - if the CPDLC-ASE is a CPDLC-air-ASE, the AircraftPDUs APDU as the D-START User Data parameter value;

  - if the CPDLC-ASE is a CPDLC-ground-ASE, the GroundPDUs APDU as the D-START User Data parameter value;

- Start timer tstart, and

- Enter the START-REQ state.

### CPDLC-start Response

#### Upon receipt of a CPDLC-start response, if the CPDLC-ASE is in the START-IND state, the CPDLC-ASE shall ([72])  :

- If the CPDLC-ASE is a CPDLC-air-ASE, and the CPDLC-start service Reject Reason parameter is provided, create an AircraftPDUs APDU with a send APDU element based on the Reject Reason parameter,

- If the CPDLC-ASE is a CPDLC-ground-ASE, and the CPDLC-start service Reject Reason parameter is provided, create a GroundPDUs APDU with a send APDU element based on the Reject Reason parameter,

- Invoke D-START response with the following:

  - The send APDU element as D-START User Data parameter; if the Reject Reason parameter was provided by the CPDLC-user, and

  - the CPDLC-start service response Result parameter value as the D-START Result parameter value, and

- If the abstract value of the Result parameter is not “accepted” enter the IDLE state, or

- Else enter the DIALOGUE state.

### DSC-start Request

#### Upon receipt of a DSC-start service request, if the CPDLC-air-ASE is in the IDLE state, the CPDLC-ASE shall ([73])  :

- Create an AircraftPDUs APDU with a startdown APDU element containing:

  - the abstract value “dsc” as the mode,

  - if provided, the CPDLC Message parameter as the DownlinkMessage, or

  - else, NULL as the DownlinkMessage,

- Invoke D-START request with the following parameters:

  - the DSC-start service ICAO Facility Designator parameter value as the D-START Called Peer ID parameter value,

  - the DSC-start service Aircraft Identifier parameter value as the D-START Calling Peer ID parameter value,

  - Set the D-START Quality of Service parameters as follows:

    - Class of Communication from the DSC-start request if provided by the CPDLC-air-user,

    - The abstract value of “normal priority flight safety messages”, as the D-START QOS Priority parameter value, and

    - The abstract value of “low” as the D-START QOS Residual Error Rate parameter value, and

  - the AircraftPDUs APDU as the D-START User Data parameter value;

- Set DSC to “true”,

- Start timer tstart, and

- Enter the START-REQ state.

### DSC-start Response

#### Upon receipt of a DSC-start response, if the CPDLC-ground-ASE is in the START-IND state, the CPDLC-ground-ASE shall ([74])  :

- If the CPDLC-start service Reject Reason parameter is provided, create a GroundPDUs APDU with a send APDU element based on the Reject Reason parameter,

- Invoke D-START response with the following parameters:

  - The send APDU element as D-START User Data parameter; if the Reject Reason parameter was provided by the CPDLC-ground-user, and

  - The DSC-start service response Result parameter value as the D-START Result parameter value, and

- If the abstract value of the Result parameter is not “accepted” enter the IDLE state, or

- Else enter the DIALOGUE state.

### CPDLC-message Request

#### Upon receipt of a CPDLC-message request, if the CPDLC-ASE is in the DIALOGUE state, the CPDLC-ASE shall ([75])  :

- If the CPDLC-ASE is a CPDLC-air-ASE, create an AircraftPDUs APDU with a send APDU-element based on  the CPDLC-message service CPDLC Message parameter, or

- If the CPDLC-ASE is a CPDLC-ground-ASE, create a GroundPDUs APDU with a send APDU-element based on the CPDLC-message service CPDLC Message parameter,

- Invoke D-DATA request with the send APDU-element as the D-DATA User Data parameter value, and

- Remain in the DIALOGUE state.

### CPDLC-end Service Request

#### Upon receipt of a CPDLC-end service request, if the CPDLC-ground-ASE is in the DIALOGUE state, the CPDLC-ground-ASE shall ([76])  :

- Create a GroundPDUs APDU with a send APDU-element based on the CPDLC-end service CPDLC Message parameter, if provided,

- Invoke D-DATA request with the send APDU-element as the D-DATA User Data parameter value, if provided,

- Start timer tend, and

- Enter the END state.

### DSC-end Service Request

#### Upon receipt of a DSC-end request, if the CPDLC-air-ASE is in the DIALOGUE state, the CPDLC-air-ASE shall ([77])  :

- Create an AircraftPDUs APDU with a send APDU-element based on the DSC-end service CPDLC Message parameter, if provided,

- Invoke D-END request with the send APDU-element as the D-END User Data parameter value, if provided,

- Start timer tend, and

- Enter the END state.

### CPDLC-end Response

#### Upon receipt of a CPDLC-end service response, if the CPDLC-air-ASE is in the END state, the CPDLC-air-ASE shall ([78])  :

- Create an AircraftPDUs APDU with a send APDU-element based on the CPDLC-end service CPDLC Message parameter, if provided,

- Invoke D-END response with the following parameters:

  - The send APDU-element as the D-END User Data parameter value; if provided by the CPDLC-air-user, and

  - The CPDLC-end service Result parameter value as the D-END Result parameter value as follows:

    - “accepted as “accepted”, and

    - “rejected” as “rejected (permanent)”, and

- If Result is “rejected” enter the DIALOGUE state, or

- Else enter the IDLE state.

### DSC-end Response

#### Upon receipt of a DSC-end service response, if the CPDLC-ground-ASE is in the END state, the CPDLC-ground-ASE shall ([79])  :

- Create a GroundPDUs APDU with a send APDU-element based on the DSC-end service CPDLC Message parameter , if provided,

- Invoke D-END response with the following parameters:

  - The send APDU-element as the D-END User Data parameter; if provided by the CPDLC-ground-user, and

  - The DSC-end service Result parameter value as the D-END Result parameter value as follows:

    - “accepted as “accepted”, and

    - “rejected” as “rejected (permanent)”, and

- If Result is “rejected” enter the DIALOGUE state, or

- Else enter the IDLE state.

### CPDLC-forward Request

#### Upon receipt of a CPDLC-forward service request, if the CPDLC-ground-ASE is in the IDLE state, the CPDLC-ground-ASE shall ([80])  :

- Create a GroundPDUs APDU with a forward APDU element containing the CPDLC Message parameter as the Forward Message,

- Invoke D-START request with the following:

  - the CPDLC-forward service Called ICAO Facility Designator parameter value as the D-START Called Peer ID parameter value,

  - the CPDLC-start service Calling ICAO Facility Designator parameter value as the D-START Calling Peer ID parameter value,

  - the D-START Quality of Service parameters set as follows:

    - if provided, the CPDLC-start service Class of Communication parameter value as the D-START QOS Routing Class, or

    - The abstract value of “normal priority flight safety messages”, as the D-START QOS Priority parameter value, and

    - The abstract value of “low” as the D-START QOS Residual Error Rate parameter value, and

  - the GroundPDUs APDU as the D-START User Data parameter value;

- Start timer tstart, and

- Enter the FORWARD state.

### CPDLC-user-abort Request

#### Upon receipt of a CPDLC-user-abort request, if the CPDLC-ASE is not in the IDLE state, the CPDLC-ASE shall ([81])  :

- Stop any timer set,

- Invoke D-ABORT request with the Originator parameter set to the abstract value “user” and

- Return to the IDLE state.

### D-ABORT Indication

#### Upon receipt of a D-ABORT indication, if the CPDLC-ASE is not in the IDLE state, the CPDLC-ASE shall ([82])  :

- Stop any timer, if set,

- If the CPDLC-user is an active user, then

  - If the D-ABORT Originator parameter contains the abstract value “user” invoke CPDLC-user-abort indication,

  - Else, invoke CPDLC-provider-abort indication with the D-ABORT User Data parameter as the CPDLC-provider-abort Reason, if provided, and

- Enter the IDLE state.

### D-P-ABORT Indication

#### Upon receipt of a D-P-ABORT indication, if the CPDLC-ASE is not in the IDLE state, the CPDLC-ASE shall ([83])  :

- Stop any timer, if set,

- If the CPDLC-user is an active user, invoke CPDLC-provider-abort indication with the CPDLC-provider-abort Reason parameter set to the abstract value “communication-service-failure”, and

- Enter the IDLE state.

## Exception Handling

### A Timer Expires

#### If a timer expires the CPDLC-ASE shall ([84])  :

- Interrupt any current activity,

- If the CPDLC-ASE is a CPDLC-air-ASE, create a AircraftPDUs APDU-with a CPDLCAbortReason [timer-expired] APDU message element,

- If the CPDLC-ASE is a CPDLC-ground-ASE, create a GroundPDUs APDU-with a CPDLCAbortReason [timer-expired] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-user is an active user, invoke CPDLC-provider-abort indication with the APDU as the CPDLC-provider abort service Reason parameter value.

### Unrecoverable System Error

#### If a CPDLC-ASE has an unrecoverable system error, the CPDLC-ASE shall ([85])  :

- If the CPDLC-ASE is a CPDLC-air-ASE, create a AircraftPDUs APDU-with a CPDLCAbortReason [undefined-error] APDU message element,

- If the CPDLC-ASE is a CPDLC-ground-ASE, create a GroundPDUs APDU-with a CPDLCAbortReason [undefined-error] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-user is an active user, invoke CPDLC-provider-abort indication with the APDU as the CPDLC-provider abort service Reason parameter value.

### Invalid PDU

#### If the User Data parameter of a D-START indication, a D-DATA indication, a D-END indication, or a D-END confirmation with Result set to the abstract value “rejected (permanent)” is not a valid PDU, the CPDLC-ASE shall ([86])  :

- If the CPDLC-ASE is a CPDLC-air-ASE, create a AircraftPDUs APDU-with a CPDLCAbortReason [invalid-PDU] APDU message element,

- If the CPDLC-ASE is a CPDLC-ground-ASE, create a GroundPDUs APDU-with a CPDLCAbortReason [invalid-PDU] APDU message element,

- invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-user is an active user, invoke CPDLC-provider-abort indication with the APDU as the CPDLC-provider abort service Reason parameter value.

#### If the User Data parameter of a D-START confirmation with Result set to the abstract value “rejected (permanent)” or a D-END confirmation with Result set to the abstract value “accepted” is not a valid PDU and the CPDLC-user is an active user, then the CPDLC-ASE shall  ([87])  invoke CPDLC-provider-abort indication with the CPDLC-provider-abort Reason parameter set to the abstract value “invalid-PDU”.

### Not permitted PDU

#### If the User Data parameter of a D-START or D-DATA indication is a valid PDU, but there are no instructions in 5.3 (ie, not a permitted PDU) the CPDLC-ASE shall ([88])  :

- If the CPDLC-ASE is a CPDLC-air-ASE, create a AircraftPDUs APDU-with a CPDLCAbortReason [not-permitted-PDU] APDU message element,

- If the CPDLC-ASE is a CPDLC-ground-ASE, create a GroundPDUs APDU-with a CPDLCAbortReason [not-permitted-PDU] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-user is an active user, invoke CPDLC-provider-abort indication with the APDU as the CPDLC-provider abort service Reason parameter value.

#### If the User Data parameter of a D-START confirmation is a valid PDU, but there are no instructions in 5.3 (ie, not a permitted PDU) the CPDLC-ASE shall ([89])  :

- If the CPDLC-ASE is a CPDLC-air-ASE, create a AircraftPDUs APDU-with a CPDLCAbortReason [not-permitted-PDU] APDU message element,

- If the CPDLC-ASE is a CPDLC-ground-ASE, create a GroundPDUs APDU-with a CPDLCAbortReason [not-permitted-PDU] APDU message element,

- If the D-START Result parameter is set to the abstract value “accepted”, invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-user is an active user, invoke CPDLC-provider-abort indication with the APDU as the CDLC-provider-abort Reason parameter value.

### D-Start Confirmation Result or Reject Source not as Expected

#### If a D-START confirmation Result parameter has the abstract value of “rejected (transient)” or if the Reject Source parameter has the abstract value of “DS provider”, and if the CPDLC-user is an active user, the CPDLC-ASE shall  ([90])  invoke CPDLC-provider-abort indication with the CPDLC-provider-abort Reason parameter set to the abstract value “communication-service-error”.

## CPDLC ASE State Tables

- Note 1: — This section defines the state tables for the CPDLC-air-ASE and the CPDLC-ground-ASE.  If the state tables shown in this section conflict with the textual statements made elsewhere in this SARPs, the textual statements take precedence.

- Note 2: — Table A-1 reflects the possible states of the CPDLC-ASE.

| STATE Þ EVENT ß | IDLE | START-REQ | START-IND | DIALOGUE | END | FORWARD |
| --- | --- | --- | --- | --- | --- | --- |
| Dialogue Service Events |  |  |  |  |  |  |
| D-START Indication | - If mode is “dsc” then, DSC-start indication, set DSC=“true” else, CPDLC-start indication - START-IND | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Confirmation Result “rejected (permanent)” and Reject Source “DS user” | cannot occur | - Stop timer tstart - If DSC=“true” then DSC-start confirmation, else CPDLC-start confirmation - IDLE | cannot occur | cannot occur | cannot occur | - Stop timer tstart - CPDLC-forward confirmation - IDLE |
| D-START Confirmation Result “accepted” | cannot occur | - Stop timer tstart - If DSC=“true” then DSC-start confirmation, else CPDLC-start confirmation - DIALOGUE | cannot occur | cannot occur | cannot occur | - Stop timer tstart - if active user, CPDLC-provider-abort indication - D-ABORT request - IDLE |
| D-DATA Indication | cannot occur | cannot occur | cannot occur | - CPDLC-message indication - DIALOGUE | - CPDLC-message indication - END | cannot occur |
| D-END Indication: CPDLC-ground-ASE only | cannot occur | cannot occur | cannot occur | - DSC-end indication - END | cannot occur | cannot occur |
| D-END Indication: CPDLC-air-ASE only | cannot occur | cannot occur | cannot occur | - CPDLC-end indication - END | cannot occur | cannot occur |
| D-END Confirmation: CPDLC-ground-ASE only Result “rejected (permanent)” and Reject Source “DS user” | cannot occur | cannot occur | cannot occur | cannot occur | - Stop timer tend - CPDLC-end confirmation - DIALOGUE | cannot occur |
| D-END Confirmation: CPDLC-air-ASE only Result “rejected (permanent)” and Reject Source “DS user” | cannot occur | cannot occur | cannot occur | cannot occur | - Stop timer tend - DSC-end confirmation - DIALOGUE | cannot occur |
| D-END Confirmation: CPDLC-ground-ASE only Result “accepted” | cannot occur | cannot occur | cannot occur | cannot occur | - Stop timer tend - CPDLC-end confirmation - IDLE | cannot occur |
| D-END Confirmation: CPDLC-air-ASE only Result “accepted” | cannot occur | cannot occur | cannot occur | cannot occur | - Stop timer tend - DSC-end confirmation - IDLE | cannot occur |
| CPDLC-User Events |  |  |  |  |  |  |
| CPDLC-start Request | - D-START request - Start timer tstart - START-REQ | not permitted | not permitted | not permitted | not permitted | not permitted |
| CPDLC-start Response Result “rejected” | not permitted | not permitted | - D-START response - IDLE | not permitted | not permitted | not permitted |
| CPDLC-start Response Result “accepted” | not permitted | not permitted | - D-START response - DIALOGUE | not permitted | not permitted | not permitted |
| DSC-start Request | - D-START request - set DSC=“true” - Start timer tstart - START-REQ | not permitted | not permitted | not permitted | not permitted | not permitted |
| DSC-start Response Result “rejected” | not permitted | not permitted | - D-START response - IDLE | not permitted | not permitted | not permitted |
| DSC-start Response Result “accepted” | not permitted | not permitted | - D-START response - DIALOGUE | not permitted | not permitted | not permitted |
| CPDLC-message Request | not permitted | not permitted | not permitted | - D-DATA request - DIALOGUE | not permitted | not permitted |
| CPDLC-end Request: CPDLC-ground-user only | not permitted | not permitted | not permitted | - D-END request - Start timer tend - END | not permitted | not permitted |
| CPDLC-end Service Response CPDLC-air-user only Result “rejected” | cannot occur | cannot occur | cannot occur | not permitted | - D-END response - DIALOGUE | not permitted |
| CPDLC-end Service Response CPDLC-air-user only Result “accepted” | cannot occur | cannot occur | cannot occur | not permitted | - D-END response - IDLE | not permitted |
| DSC-end Request: CPDLC-air-user only | not permitted | not permitted | not permitted | - D-END request - Start timer tend - END | not permitted | not permitted |
| DSC-end Service Response CPDLC-ground-user only Result “rejected” | cannot occur | cannot occur | cannot occur | not permitted | - D-END response - DIALOGUE | not permitted |
| DSC-end Service Response CPDLC-ground-user only Result “accepted” | cannot occur | cannot occur | cannot occur | not permitted | - D-END response - IDLE | not permitted |
| CPDLC-forward Request | - D-START request - Start timer tstart - FORWARD | not permitted | not permitted | not permitted | not permitted | not permitted |
| ABORT Events |  |  |  |  |  |  |
| CPDLC-user-abort Request | not permitted | - Stop timer tstart, if set - D-ABORT request - IDLE | - Stop timer tstart, if set - D-ABORT request - IDLE | - D-ABORT request - IDLE | - Stop timer tend; if set - D-ABORT request - IDLE | - Stop timer tstart; if set - D-ABORT request - IDLE |
| D-ABORT Indication Originator “provider” | cannot occur | - Stop timer Tstart, if set - If active user: CPDLC-provider-abort indication - IDLE | - Stop timer Tstart, if set - If active user: CPDLC-provider-abort indication - IDLE | - If active user: CPDLC-provider-abort indication - IDLE | - Stop timer tend; if set - If active user: CPDLC-provider-abort indication - IDLE | - Stop timer Tstart, if set - If active user: CPDLC-provider-abort indication - IDLE |
| D-ABORT Indication Originator “user” | cannot occur | - Stop timer Tstart, if set - If active user: CPDLC-user-abort indication - IDLE | - Stop timer Tstart, if set - If active user: CPDLC-user-abort indication - IDLE | - If active user: CPDLC-user-abort indication - IDLE | - Stop timer tend; if set - If active user: CPDLC-user-abort indication - IDLE | - Stop timer Tstart, if set - If active user: CPDLC-user-abort indication - IDLE |
| D-P-ABORT indication | cannot occur | - Stop timer tstart, if set - If active user: CPDLC-provider-abort indication - IDLE | - Stop timer tstart, if set - If active user: CPDLC-provider-abort indication - IDLE | - If active user: CPDLC-provider-abort indication - IDLE | - Stop timer tend; if set - If active user: CPDLC-provider-abort indication - IDLE | - Stop timer tstart, if set - If active user: CPDLC-provider-abort indication - IDLE |
| Tstart Expires | cannot occur | - D-ABORT request - CPDLC-provider-abort indication - IDLE | cannot occur | cannot occur | cannot occur | - D-ABORT request - CPDLC-provider-abort indication - IDLE |
| Tend Expires | cannot occur | cannot occur | cannot occur | cannot occur | - D-ABORT request - CPDLC-provider-abort indication - IDLE | cannot occur |

Table 5-2: CPDLC-ASE State Table

# COMMUNICATION REQUIREMENTS

## Encoding Rules

### The CPDLC application shall  ([91])  use PER as defined in reference [1], using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in part 4, or a functionally equivalent means which provides the same result.

## Dialogue Service Requirements

### Primitive Requirements

#### Where dialogue service primitives, that is D-START, D-DATA, D-END, D-ABORT, and D-P-ABORT are described as being invoked in chapter 5, the CPDLC-ground-ASE and the CPDLC-air-ASE shall  ([92])  exhibit external behavior consistent with the dialogue service, as described in reference [4], having been implemented and its primitives invoked.

### Quality-of-Service Requirements

#### The application service priority for CPDLC shall  ([93])  have the abstract value of “normal priority flight safety messages”.

#### The RER Quality of Service Parameter of the D-START shall  ([94])  be set to the abstract value of “low”.

- Note 1. — The application service RER for CPDLC is required to be 10-7 or better

- Note 2: — The RER takes into account non-delivery for messages over a given dialogue.

#### Routing Policy

##### The CPDLC-ASE shall  ([95])  map the CPDLC-start service or DSC-start service Class of Communication parameter abstract value to the routing class abstract value part of the D-START QOS parameter as presented in Table 6-1.

| Class of Communication Abstract Value | Routing Class Abstract Value |
| --- | --- |
| A | Traffic only follows Class A ATSC route(s) |
| B | Traffic only follows Class B ATSC route(s) |
| C | Traffic only follows Class C ATSC route(s) |
| D | Traffic only follows Class D ATSC route(s) |
| E | Traffic only follows Class E ATSC route(s) |
| F | Traffic only follows Class F ATSC route(s) |
| G | Traffic only follows Class G ATSC route(s) |
| H | Traffic only follows Class H ATSC route(s) |

Table 6-1:  Mapping Between Class of Communication and Routing Class Abstract Values

- Note: — ATSC values are defined in [5].

## CPDLC AE Control Functions Requirements

### The title of the CPDLC application used as the AE-Qualifier for the CPDLC application shall  ([96])  be the IA5 character string “CPC”.

# CPDLC USER REQUIREMENTS

## Introduction

- Note 1: — Requirements imposed on CPDLC-user concerning CPDLC messages and interfacing with the CPDLC-ASEs are presented in this chapter.

- Note 2: — Where reference is made to the “CPDLC-user”, this implies both the CPDLC-air-user and the CPDLC-ground user.

- Note 3: — A CPDLC message is:

- what the CPDLC-user provides the CPDLC-service as the CPDLC Message or Reject Reason parameter, when invoking a CPDLC service request or response primitive, or

- what the CPDLC-user receives in the same parameters from the CPDLC service indication or confirmation primitives.

- Note 4: — In this chapter the terms CPDLC message, message, uplink message and downlink message are used interchangeably, and equate to a CPDLC message.  When the terms “send” and “transmit” are used this means that the CPDLC-user has invoked a CPDLC service request or response primitive.  When the term “receive” is used this means that a CPDLC indication or confirmation primitive parameter containing a CPDLC message has been provided by the CPDLC service.

## CPDLC Message Generation Requirements

- Note 1. — A response message is a message which is a reply to a received message.  It contains a message reference number identical to the message identification number of the message to which it refers.  Only response messages contain a message reference number.

- Note 2. — Message response attributes dictate a) if a response is require or prohibited; b) if a response is required, dictate the permitted response messages.

- Note 3: — A closure response message is a reply to a message or series of messages which terminates a sequence of message exchanges.  However due to the multiple element capability of a CPDLC message, a closure message may contain message element(s)in addition to the required closure message element that initiate a new sequence of messages.

### For each CPDLC message the CPDLC-user sends air/ground it shall  ([97])  provide the following information:

- a message identification number,

- a message reference number only if the message is a response message,

- date and time,

- a logical acknowledgment indication, if required,

- from one to five message element identifiers, and

- data as required for each message element identification included.

### For each CPDLC message the CPDLC-user sends ground/ground it shall  ([98])  provide the following information:

- the aircraft identification to which the ground/ground message refers,

- date and time,

- from one to five message element identifiers, and

- data as required for each message element identification included.

### Message Identification Number

#### The message identification number provided by the CPDLC-user shall  ([99])  be different from any other message identification number currently in use.

#### A message identification number shall  ([100])  be deemed currently in use until:

- if the message does not allow a response, the message is sent, or

- if the message requires a response, the closure response is received.

#### If a CPDLC or DSC dialogue is terminated, all message identification numbers pertaining to that dialogue, shall  ([101])  be considered available.

#### If the CPDLC-user sends a message containing the ERROR message element instead of the expected response message, the ERROR message shall  ([102])  contain the received message identification number as the message reference number.

### Error Conditions

#### No Available Message Identification Numbers

##### If the CPDLC-user attempts to send a CPDLC message and all message identification numbers are currently in use, the CPDLC-user shall  ([103])  invoke the CPDLC-user-abort request with CPDLC a message containing the ERROR [errorinformation] message element with the value [no-message-identification-numbers-available].value as the Reason parameter.

## CPDLC Message Receipt Requirements

### Urgency Requirements

#### When a CPDLC-user queues received messages, messages with the highest Urgency type shall  ([104])  be placed at the beginning of the queue.

#### When a CPDLC-user queues received messages, messages with the same Urgency type shall  ([105])  be queued in order of receipt.

### Alerting Requirements

#### Upon receipt of a CPDLC message, the CPDLC-user shall  ([106])  provide one of three distinct alerts as determined by the received message alert attribute.

### CPDLC/DSC Distinction

#### Upon receipt of a CPDLC message the CPDLC-user shall  ([107])  provide a distinction between CPDLC messages received from the Current Data Authority and those received from a Downstream Data Authority.

### Air/Ground - Ground/ground Distinction

#### Upon receipt of a CPDLC message the CPDLC-user shall  ([108])  provide a distinction between CPDLC messages received from an aircraft and those received from another ground system.

### Logical Acknowledgment Prohibited

#### Upon receipt of the CPDLC message USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED the CPDLC-air-user shall  ([109])  be prohibited from requiring a logical acknowledgment for any message sent for the duration of the CPDLC or DSC dialogue.

#### If the CPDLC-ground user receives a CPDLC message requiring a logical acknowledgment where the use of logical acknowledgment has been prohibited as above, the CPDLC-ground-user shall  ([110])  invoke the CPDLC-message service with a message containing the ERROR [errorinformation] message element with the [logicalAcknowledgmentNotAccepted]value as the CPDLC message parameter and discard the content of the received message.

### Message Reference Numbers

#### If a received message requires a response, the CPDLC-user shall  ([111])  provide a message reference number for each response message sent.

#### The message reference number shall  ([112])  be identical to the message identification number of the received message to which it refers.

### Message Response Requirements

- Note 1. — A message sequence initiated by data link should be closed by data link.

- Note 2. — If a message sequence exchange initiated by data link is subsequently closed by voice, local procedures must be in place to ensure deletion of outstanding data link messages requiring closure.

#### A CPDLC-user shall  ([113])  only be permitted to respond to a received message in its entirety.

#### Only one closure response shall  ([114])  be permitted for a given message.

#### If a message is received that requires a response, the CPDLC-user shall  ([115])  either:

- send any permitted response messages and then send a closure response message, or

- send a closure response message.

#### For a given message, once the CPDLC-user has sent the closure response message, no other response messages shall  ([116])  be sent referring to the given message.

#### When a message is received by the CPDLC-user requiring a logical acknowledgment response, the CPDLC-user shall  ([117])  respond with either a CPDLC message containing only a LOGICAL ACKNOWLEDGEMENT message element, or a message containing an ERROR message element.

#### A logical acknowledgment response message, if required, shall  ([118])  be sent prior to sending any other related response message(s), except a response message containing an ERROR message element.

#### When the CPDLC-air-user receives a message with a W/U RESP attribute, the only permitted responses shall  ([119])  be messages that contain a LOGICAL ACKNOWLEDGMENT (if required), STANDBY, WILCO, UNABLE, or ERROR message element.

#### When the CPDLC-air-user receives a message with a W/U RESP attribute, the closure response message shall  ([120])  contain at least a WILCO, UNABLE, or ERROR message element.

#### When the CPDLC-air-user receives a message with a A/N RESP attribute, the only permitted responses shall  ([121])  be messages that contain a LOGICAL ACKNOWLEDGMENT (if required), STANDBY, AFFIRM, NEGATIVE, or ERROR message element.

#### When the CPDLC-air-user receives a message with a A/N RESP attribute, the closure response message shall  ([122])  contain at least a AFFIRM, NEGATIVE, or ERROR message element.

#### When the CPDLC-air-user receives a message with a R RESP attribute, the only permitted responses shall  ([123])  be messages that contain a LOGICAL ACKNOWLEDGMENT (if required), STANDBY, ROGER, or ERROR message element.

#### When the CPDLC-air-user receives a message with a R RESP attribute, the closure response message shall  ([124])  contain at least a ROGER or ERROR message element.

#### When the CPDLC-air-user receives a message with a Y RESP attribute, a LOGICAL ACKNOWLEDGMENT only when requested, and all other CPDLC messages shall  ([125])  be permitted as a response message.

#### When the CPDLC-air-user receives a message with a Y RESP attribute, the first response message sent that does not contain a STANDBY or LOGICAL ACKNOWLEDGEMENT shall  ([126])  constitute the closure response message.

#### When the CPDLC-ground-user receives an air/ground message with a Y RESP attribute, a LOGICAL ACKNOWLEDGMENT, only when requested and all other CPDLC messages shall  ([127])  be permitted as a response message.

#### When the CPDLC-ground-user receives an air/ground message with a Y RESP attribute, the first response message sent that does not contain a STANDBY, REQUEST DEFERRED, or LOGICAL ACKNOWLEDGEMENT message element shall  ([128])  constitute the closure response message.

#### When the CPDLC-air-user receives a message with a N RESP attribute, the only permitted response shall  ([129])  be a closure response message that contains a LOGICAL ACKNOWLEDGMENT or ERROR message element, if a logical acknowledgment is required.

#### When the CPDLC-ground-user receives an air/ground message with a N RESP attribute, the only permitted response shall  ([130])  be a closure response message that contains a LOGICAL ACKNOWLEDGMENT or ERROR message element, if a logical acknowledgment is required.

#### When the CPDLC-ground-user receives a ground/ground message the ground-user shall  ([131])  be prohibited from generating a ground/ground response message.

Note: — Ground/ground forwarding of messages is a one-way exchange on a one message per dialogue basis.  There are no message identification or message reference numbers contained in the header of a ground/ground message.

### Error Conditions

#### Duplicate Message Identification Numbers

##### If the CPDLC-user attempts to send a CPDLC message and all message identification numbers are currently in use, the CPDLC-user shall  ([132])  invoke the CPDLC-user-abort request with CPDLC a message containing the ERROR [errorinformation] message element with the value [duplicate-message-identification-number].value as the Reason parameter.

#### Invalid Reference Number

##### If the CPDLC-user receives a message containing a message reference number which is not identical to any message identification number currently in use, the CPDLC-user shall ([133])  :

- invoke CPDLC-message request with the ERROR [errorinformation] message element with the [unrecognizedMsgReferenceNumber] value as the CPDLC Message parameter, and

- disregard the received message .

## CPDLC-air-user Requirements

### The CPDLC-start Service

### Invoking the CPDLC-start request

##### If there is no CPDLC service, the only CPDLC service primitives the CPDLC-air-user shall  ([134])  be permitted to invoke are the CPDLC-start request or the DSC-start request.

#### The CPDLC-air-user shall  ([135])  only be permitted to invoke the CPDLC-start request with:

- any ground system, if there is no existing CPDLC service for the CPDLC-air-user, or

- the Next Data Authority, if the CPDLC-air-user has received a message from the Current Data Authority designating a Next Data Authority.

#### If a CPDLC-air-user has invoked a CPDLC-start request, the CPDLC-air-user shall  ([136])  be prohibited from invoking any CPDLC-service primitive pertaining to the ground system addressed in the CPDLC-start service, except the CPDLC-user-abort request, until after it has received a CPDLC-start confirmation.

### Receipt of a CPDLC-start Indication and Invoking CPDLC-start Response

#### Upon receipt of a CPDLC-start indication the CPDLC-air user shall  ([137])  invoke the CPDLC-start response, with the response parameters set as follows:

- The Result parameter to the abstract value of “accepted” if:

  - There is no existing CPDLC service, or

  - CPDLC service exists and the request is from either the Current Data Authority or Next Data Authority,

- Else set the Result parameter is set to the abstract value “rejected” and the Reject Reason to a CPDLC message with the message element NOT AUTHORIZED NEXT DATA AUTHORITY.

#### If a CPDLC-start indication is received from either the Current Data Authority or the Next Data Authority, and this results in a second CPDLC dialogue being established with a given ground system, the CPDLC-air-user shall  ([138])  invoke the CPDLC-user-abort request primitive for the first connection with that ground system.

#### If the CPDLC-air-user sets the CPDLC-response Result parameter to the abstract value “rejected” any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter shall  ([139])  be disregarded.

#### If the CPDLC-air-user sets the CPDLC-response Result parameter to the abstract value “accepted” and the request is from the Current Data Authority any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter shall  ([140])  be processed.

#### If the CPDLC-air-user sets the CPDLC-response Result parameter to the abstract value “accepted” and the request is from the Next Data Authority any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter shall  ([141])  be discarded.

#### If the CPDLC-air-user sets the CPDLC-response Result parameter to the abstract value “accepted” the CPDLC-air-user shall ([142])  :

- Establish an association between a CPDLC-ASE invocation and a ground system ICAO facility designator contained in CPDLC-start indication Calling Peer Identifier parameter,

- If there is no Current Data Authority, associate this CPDLC-ASE invocation with the Current Data Authority, or

- If the ICAO facility designator contained CPDLC-start indication Calling Peer Identifier parameter is the Next Data Authority associate the CPDLC-ASE invocation with the Next Data Authority.

#### If CPDLC-start indication has been received, the CPDLC-air-user shall  ([143])  be prohibited from invoking any CPDLC-service primitive with this ground system, except the CPDLC-user-abort request, until after it has invoked the CPDLC-start response.

### Receipt of a CPDLC-start confirmation

#### If a CPDLC-start confirmation has been received with a Result parameter containing the abstract value “accepted” the CPDLC-air-user shall ([144])  :

- Establish an association between a CPDLC-ASE invocation and a ground system ICAO facility designator contained in CPDLC-start request Called Peer Identifier parameter,

- If there is no Current Data Authority, associate the CPDLC-ASE invocation with the Current Data Authority, or

- If the ICAO facility designator contained CPDLC-start indication Called Peer Identifier parameter is the Next Data Authority associate the CPDLC-ASE invocation with the Next Data Authority.

### The DSC-start Service

#### Invoking the DSC-start request

##### Only a CPDLC-air-user shall  ([145])  be permitted to invoke the DSC-start service request primitive.

##### A CPDLC-air-user shall  ([146])  only be permitted to invoke the DSC-start-service request primitive if the CPDLC-air-user has no existing DSC dialogue.

##### If a CPDLC-air-user has invoked a DSC-start request, the CPDLC-air-user shall  ([147])  be prohibited from invoking any CPDLC-service primitive with this ground system, except the CPDLC-user-abort request, until after it has received a DSC-start confirmation.

#### Receipt of a DSC-start confirmation

##### If a DSC-start confirmation has been received with a Result parameter containing the abstract value “accepted” the CPDLC-air-user shall ([148])  :

- Establish an association between a CPDLC-ASE invocation and the ground system ICAO facility designator contained in DSC-start request ICAO Facility Designator parameter,

- Associate the CPDLC-ASE invocation with a Downstream Data Authority.

### Receipt of a CPDLC-message Indication

#### Upon receipt of a CPDLC-message indication, if the indication is from the Current Data Authority or a Downstream Data Authority the CPDLC-air-user shall  ([149])  process the CPDLC message contained in the CPDLC Message parameter.

#### If a CPDLC-message indication is received from the Current Data Authority containing at least the uplink message element NEXT DATA AUTHORITY indicating that the specified ICAO facility designator is the Next Data Authority the CPDLC-air-user shall  ([150])  do the following in the order listed:

- Check that there is no other Next Data Authority already established;

- if there is, invoke CPDLC-user-abort request with the established Next Data Authority with the Reason parameter set to CPDLCAbortReason value [no-longer-next-data-authority]; and

- Then designate the ground system indicated in the CPDLC message from the CPDLC-message indication as the Next Data Authority.

#### Upon receipt of a CPDLC-message indication, if the indication is not from the Current Data Authority or a Downstream Data Authority, the CPDLC-air-user shall ([151])  :

- Invoke CPDLC-message request with the CPDLC Message parameter containing a message with the message element NOT CURRENT DATA AUTHORITY, and

- Disregard the received message contained in the CPDLC-message indication CPDLC Message parameter.

### The CPDLC-end Service

#### The CPDLC-end Request

##### The CPDLC-air-user shall  ([152])  be prohibited from invoking the CPDLC-end request.

#### Receipt of a CPDLC-end Indication and Invoking a CPDLC-end Response

#### If a CPDLC-end indication is received but it is not from the Current Data Authority the CPDLC-air user shall ([153])  :

- Invoke CPDLC-end response with

  - the CPDLC Message parameter containing a message with the message element NOT CURRENT DATA AUTHORITY, and

  - the Result parameter set to the abstract value “rejected”, and

- Disregard any message provided in the CPDLC-end indication CPDLC Message parameter.

#### If a CPDLC-end indication is received from the Current Data Authority and the CPDLC-air-user has sent a message that requires a response for which it has not received a closure response then the CPDLC-air user shall ([154])  :

- Invoke CPDLC-end response with:

  - the CPDLC Message parameter containing a CPDLC ATCDownlinkMessage with the ERROR [errorinformation] message element with the [endServiceWithPendingMsgs] value, and

  - the Result parameter set to the abstract value “rejected”, and

- Disregard any message provided in the CPDLC-end indication CPDLC Message parameter.

#### If a CPDLC-end indication is received from the Current Data Authority and the CPDLC-air-user has received a message for which a response is required, and it has not yet sent the closure response to that message then the CPDLC-air user shall ([155])  :

- Invoke CPDLC-end response with:

  - the CPDLC Message parameter containing a CPDLC ATCDownlinkMessage with the ERROR [errorinformation] message element with the [endServiceWithPendingMsgs] value, and

  - the Result parameter set to the abstract value “rejected”, and

- Disregard any message provided in the CPDLC-end indication CPDLC Message parameter.

#### If a CPDLC-end indication is received that is from the Current Data Authority and the CPDLC Message parameter contains a message requiring a response, the CPDLC-air user shall ([156])  :

- If responding with any permitted CPDLC message response that is not the closure response, invoke CPDLC-message request with the response message as the CPDLC Message parameter, then

- If desired, invoke CPDLC-message request with the closure response message as the CPDLC Message parameter, then

- Invoke CPDLC-end response with:

  - the CPDLC Message parameter as the CPDLC closure response if not already sent, and

  - the Result parameter set to the abstract value “rejected” or “accepted”.

#### If a CPDLC-end indication is received that is from the Current Data Authority without a CPDLC Message parameter, the CPDLC-air user shall  ([157])  invoke CPDLC-end response with:

- the CPDLC Message parameter with a CPDLC message if desired, and

- the Result parameter set to the abstract value “rejected” or “accepted”.

#### Upon invoking CPDLC-end response with Result parameter set to “accepted”, the CPDLC-air-user shall ([158])  :

- delete any association with a ground system and Current Data Authority, and

- if a ground system is designated as Next Data Authority and an association with a CPDLC-ASE exists, replace the Next Data Authority association with a Current Data Authority association, or

- if a ground system is designated as Next Data Authority and no association with a CPDLC-ASE exists, delete Next Data Authority association with any ground system.

#### If the CPDLC-air-ASE associated with the Current Data Authority ceases to exist for any reason other than in response to a CPDLC-end request as specified above, any existing Next Data Authority designation and/or association shall  ([159])  cease to exist.

### The DSC-end Service

#### The DSC-end Request

#### Only the CPDLC-air-user shall  ([160])  be permitted to invoke the DSC-end request.

### The CPDLC-user-abort Service

#### Receipt of a CPDLC-abort Indication

#### If the CPDLC-air-user receives a CPDLC-user-abort indication from the Current Data Authority or a CPDLC-provider-abort indication that causes the ASE invocation associated with the Current Data Authority to cease to exist, the CPDLC-air-user shall ([161])  :

- Delete any association of a ground system to a Current Data Authority,

- If a ground system is designated as Next Data Authority and an association with a CPDLC-ASE exists, invoke CPDLC-user-abort request with the Reason parameter set to the abstract value “ current data authority abort”, and

- Delete any association of a ground system to a Next Data Authority.

#### If the CPDLC-air-user receives a CPDLC-user-abort indication from the Next Data Authority or receive a CPDLC-provider-abort indication that causes the ASE invocation associated with the Next Data Authority to cease to exist, the CPDLC-air-user shall  ([162])  continue to maintain the association of the ground system to the Next Data Authority.

## CPDLC-Ground-User Requirements

### The CPDLC-start Service

### Invoking the CPDLC-start request

#### If there is no CPDLC service, the only CPDLC service primitives the CPDLC-ground-user shall  ([163])  be permitted to invoke are the CPDLC-start-request or the CPDLC-forward request.

#### If a CPDLC-ground-user has invoked a CPDLC-start request, the CPDLC-ground-user shall  ([164])  be prohibited from invoking any CPDLC-service primitive, except the CPDLC-user-abort request with that aircraft, until after it has received a CPDLC-start confirmation.

### Receipt of a CPDLC-start Indication and Invoking CPDLC-start Response

#### If a CPDLC-start indication is received from an aircraft with which the ground system currently has a CPDLC dialogue, the CPDLC-ground-user shall ([165])  :

- invoke the CPDLC-start response with the Result parameter set to the abstract value “accepted”, and

- invoke the CPDLC-user-abort request for the first CPDLC dialogue with that aircraft.

#### The CPDLC-ground-user shall  ([166])  be prohibited from invoking the CPDLC-start response unless and until it has received a CPDLC-start indication.

#### If the CPDLC-ground-user sets the CPDLC-start response Result parameter to the abstract value “rejected” then the Reject Reason shall  ([167])  be an uplink message containing only the SERVICE UNAVAILABLE message element.

#### If the CPDLC-ground-user sets the CPDLC-start response Result parameter to the abstract value “rejected” the CPDLC-ground-user shall  ([168])  disregard any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter.

#### If the CPDLC-ground-user sets the CPDLC-response Result parameter to the abstract value “accepted” any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter shall  ([169])  be processed.

#### If the CPDLC-ground-user sets the CPDLC-response Result parameter to the abstract value “accepted” the CPDLC-ground-user shall  ([170])  establish an association between a CPDLC-ASE invocation and a 24 bit aircraft address contained in CPDLC-start indication Calling Peer Identifier parameter.

#### If CPDLC-start indication has been received, the CPDLC-ground-user shall  ([171])  be prohibited from invoking any CPDLC-service primitive, except the CPDLC-user-abort request with that aircraft, until after it has invoked the CPDLC-start response.

### Receipt of a CPDLC-start confirmation

#### If a CPDLC-start confirmation has been received with a Result parameter containing the abstract value “accepted” the CPDLC-ground-user shall  ([172])  establish an association between a CPDLC-ASE invocation and a 24 bit aircraft address contained in CPDLC-start request Called Peer Identifier parameter.

### The DSC-start Service

#### Receipt of a DSC-start Indication and Invoking DSC-start Response

##### The CPDLC-ground-user shall  ([173])  be prohibited from invoking the DSC-start response unless and until it has received a DSC-start indication.

##### If a DSC-start indication is received from an aircraft with which the ground system currently has a DSC dialogue, the CPDLC-ground-user shall ([174])  :

- invoke the DSC-start response with the Result parameter set to the abstract value “accepted”, and

- invoke the CPDLC-user-abort request for the first DSC dialogue with that aircraft.

##### If the CPDLC-ground-user sets the DSC-start response Result parameter to the abstract value “rejected” then the Reject Reason shall  ([175])  be an uplink message containing only the SERVICE UNAVAILABLE message element.

##### If the CPDLC-ground-user sets the DSC-start response Result parameter to the abstract value “rejected” the CPDLC-ground-user shall  ([176])  disregard any CPDLC message contained in the DSC-start indication CPDLC Message parameter.

##### If the CPDLC-ground-user sets the DSC-start response Result parameter to the abstract value “accepted” any CPDLC message contained in the DSC-start indication CPDLC Message parameter shall  ([177])  be processed.

##### If the CPDLC-ground-user sets the DSC-start response Result parameter to the abstract value “accepted” the CPDLC-ground-user shall  ([178])  establish an association between a CPDLC-ASE invocation and a 24 bit aircraft address contained in the DSC-start indication Aircraft Identifier parameter.

##### If DSC-start indication has been received, the CPDLC-ground-user shall  ([179])  be prohibited from invoking any CPDLC-service primitive, except the CPDLC-user-abort request, until after it has invoked the DSC-start response.

### The CPDLC-end Service

#### The CPDLC-end Request

##### Only the CPDLC-ground-user shall  ([180])  be permitted to invoke the CPDLC-end request.

##### The CPDLC-ground-user shall  ([181])  be prohibited from invoking the CPDLC-end request if:

- it has sent a message that requires a response for which it has not received a closure response, or

- it has received a message for which a response is required, and it has not yet sent the closure response.

### The DSC-end Service

### Receipt of a DSC-end Indication and Invoking DSC-end Response

#### The CPDLC-ground-user shall  ([182])  be prohibited from invoking the DSC-end response unless and until it has received a DSC-end indication.

#### If DSC-end indication has been received, the CPDLC-ground-user shall  ([183])  be prohibited from invoking any CPDLC-service primitive, except the CPDLC-user-abort request with that aircraft, until after it has invoked the DSC-end response.

### The CPDLC-forward Service

### Invoking the CPDLC-start request

#### Only the CPDLC-ground-user shall  ([184])  be permitted to invoke the CPDLC-forward request.

## Message Intent

### Purpose

- Note: —  This section contains the message set for CPDLC.  Message attributes, message presentation guidance, and data structure presentation guidance are presented.  The actual information exchanged between an aircraft and ground peer or a ground and ground peer CPDLC applications is defined in section 4 of the SARPs; however, section 4 does not mandate any particular method for presenting this information.  The presentation of information to the controller and aircraft crew is a local implementation.  The message presentation recommendations contained within this section are one possible means of presenting the information.  These recommendations are generally consistent with current ICAO practices for displaying ATC information.

### Message elements shall  ([185])  comply with the intent and use as presented in the following tables in section 7.6.

### Up-Link Messages

- Note: — Uplink messages for CPDLC are presented in this section.

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 0 | Indicates that ATS cannot comply with the request. | UNABLE | N | M | N |
| 1 | Indicates that ATS has received the request and will respond shortly. | STANDBY | N | L | N |
| 2 | Indicates that ATS has received the request but it has been deferred until later. | REQUEST DEFERRED | N | L | N |
| 3 | Indicates that ATS has received and understood the request. | ROGER | N | L | N |
| 4 | Yes. | AFFIRM | N | L | N |
| 5 | No. | NEGATIVE | N | L | N |
| 211 | Indicates that the ATS has received the request and has passed it to the Next Control Authority. | REQUEST FORWARDED | N | L | N |
| 218 | Indicates to the pilot that the request has already been received on the ground. | REQUEST ALREADY RECEIVED | L | N | N |

Table 7-1:  Responses/Acknowledgments (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 6 | Notification that a altitude change instruction should be expected. | EXPECT [altitude] | L | L | R |
| 7 | Notification that an instruction should be expected for the aircraft to commence climb at the specified time. | EXPECT CLIMB AT [time] | L | L | R |
| 8 | Notification that an instruction should be expected for the aircraft to commence climb at the specified position. | EXPECT CLIMB AT [position] | L | L | R |
| 9 | Notification that an instruction should be expected for the aircraft to commence descent at the specified time. | EXPECT DESCENT AT [time] | L | L | R |
| 10 | Notification that an instruction should be expected for the aircraft to commence descent at the specified position. | EXPECT DESCENT AT [position] | L | L | R |
| 11 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time. | EXPECT CRUISE CLIMB AT [time] | L | L | R |
| 12 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position. | EXPECT CRUISE CLIMB AT [position] | L | L | R |
| 13 | Notification that an instruction should be expected for the aircraft to commence climb at the specified time to the specified altitude. | AT [time] EXPECT CLIMB TO [altitude] | L | L | R |
| 14 | Notification that an instruction should be expected for the aircraft to commence climb at the specified position to the specified altitude. | AT [position] EXPECT CLIMB TO [altitude] | L | L | R |
| 15 | Notification that an instruction should be expected for the aircraft to commence descent at the specified time to the specified altitude. | AT [time] EXPECT DESCENT TO [altitude] | L | L | R |
| 16 | Notification that an instruction should be expected for the aircraft to commence descent at the specified position to the specified altitude. | AT [position] EXPECT DESCENT TO [altitude] | L | L | R |
| 17 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time to the specified altitude. | AT [time] EXPECT CRUISE CLIMB TO [altitude] | L | L | R |
| 18 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position to the specified altitude. | AT [position] EXPECT CRUISE CLIMB TO [altitude] | L | L | R |
| 19 | Instruction to maintain the specified altitude. | MAINTAIN [altitude] | N | M | W/U |
| 20 | Combined instruction that a climb to a specified altitude is to commence and the altitude is to be maintained when reached. | CLIMB TO AND MAINTAIN [altitude] | N | M | W/U |
| 21 | Combined instruction that at the specified time, a climb to the specified altitude is to commence and once reached the specified altitude is to be maintained. | AT [time] CLIMB TO AND MAINTAIN [altitude] | N | M | W/U |
| 22 | Combined instruction that at the specified position, a climb to the specified altitude is to commence and once reached the specified altitude is to be maintained. | AT [position] CLIMB TO AND MAINTAIN [altitude] | N | M | W/U |
| 185 | Combined instruction that after passing the specified position, a climb to the specified altitude is to commence and once reached the specified altitude is to be maintained. | AFTER PASSING [position] CLIMB TO AND MAINTAIN [altitude] | N | M | W/U |
| 23 | Combined instruction that a descent to a specified altitude is to commence and the altitude is to be maintained when reached. | DESCEND TO AND MAINTAIN [altitude] | N | M | W/U |
| 24 | Combined instruction that at a specified time a decent to a specified altitude is to commence and once reached the specified altitude is to be maintained. | AT [time] DESCEND TO AND MAINTAIN [altitude] | N | M | W/U |
| 25 | Combined instruction that at the specified position a descent to the specified altitude is to commence and when the specified altitude is reached it is to be maintained. | AT [position] DESCEND TO AND MAINTAIN [altitude] | N | M | W/U |
| 186 | Combined instruction that after passing the specified position, a descent to the specified altitude is to commence and once reached the specified altitude is to be maintained. | AFTER PASSING [position] DESCEND TO AND MAINTAIN [altitude] | N | M | W/U |
| 26 | Combined instruction that a climb is to commence at a rate such that the specified altitude is reached at or before the specified time | CLIMB TO REACH [altitude] BY [time] | N | M | W/U |
| 27 | Combined instruction that a climb is to commence at a rate such that the specified altitude is reached at or before the specified position | CLIMB TO REACH [altitude] BY [position] | N | M | W/U |
| 28 | Combined instruction that a descent is to commence at a rate such that the specified altitude is reached at or before the specified time | DESCEND TO REACH [altitude] BY [time] | N | M | W/U |
| 29 | Combined instruction that a descent is to commence at a rate such that the specified altitude is reached at or before the specified position | DESCEND TO REACH [altitude] BY [position] | N | M | W/U |
| 197 | Combined instruction that a change of altitude is to commence at a rate such that the specified altitude is reached at or before the specified time. | REACH [altitude] BY [time] | N | M | W/U |
| 209 | Combined instruction that a change of altitude is to commence at a rate such that the specified altitude is reached at or before the specified position. | REACH [altitude] BY [position] | N | M | W/U |
| 30 | A altitude within the defined block altitude specified is to be maintained. | MAINTAIN BLOCK [altitude] TO [altitude] | N | M | W/U |
| 31 | Combined instruction that a climb to a altitude within the block altitude defined is to commence. | CLIMB TO AND MAINTAIN BLOCK [altitude] TO [altitude] | N | M | W/U |
| 32 | Combined instruction that a descent to a altitude within the block altitude defined is to commence. | DESCEND TO AND MAINTAIN BLOCK [altitude] TO [altitude] | N | M | W/U |
| 34 | A cruise climb is to commence and continue until the specified altitude is reached. | CRUISE CLIMB TO [altitude] | N | M | W/U |
| 35 | A cruise climb can commence once above the specified altitude. | CRUISE CLIMB ABOVE [altitude] | N | M | W/U |
| 219 | Instruction to stop the climb below the previously assigned altitude. | STOP CLIMB AT [altitude] | U | M | W/U |
| 220 | Instruction to stop the descent above the previously assigned altitude. | STOP DESCENT AT [altitude] | U | M | W/U |
| 36 | The climb to the specified altitude should be made at the aircraft’s best rate. | EXPEDITE CLIMB TO [altitude] | U | M | W/U |
| 37 | The descent to the specified altitude should be made at the aircraft’s best rate. | EXPEDITE DESCENT TO [altitude] | U | M | W/U |
| 38 | Urgent instruction to immediately climb to the specified altitude. | IMMEDIATELY CLIMB TO [altitude] | D | H | W/U |
| 39 | Urgent instruction to immediately descend to the specified altitude. | IMMEDIATELY DESCEND TO [altitude] | D | H | W/U |
| 40 | Urgent instruction to immediately stop a climb once the specified altitude is reached. | IMMEDIATELY STOP CLIMB AT [altitude] | D | H | W/U |
| 41 | Urgent instruction to immediately stop a descent once the specified altitude is reached. | IMMEDIATELY STOP DESCENT AT [altitude] | D | H | W/U |
| 171 | Instruction to climb at not less than the specified rate. | CLIMB AT [vertical rate] MINIMUM | N | M | W/U |
| 172 | Instruction to climb at not above the specified rate. | CLIMB AT [vertical rate] MAXIMUM | N | M | W/U |
| 173 | Instruction to descend at not less than the specified rate. | DESCEND AT [vertical rate] MINIMUM | N | M | W/U |
| 174 | Instruction to descend at not above the specified rate. | DESCEND AT [vertical rate] MAXIMUM | N | M | W/U |

Table 7-2:  Vertical Clearances (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 42 | Notification that a altitude change instruction should be expected which will require the specified position to be crossed at the specified altitude. | EXPECT TO CROSS [position] AT [altitude] | L | L | R |
| 43 | Notification that a altitude change instruction should be expected which will require the specified position to be crossed at or above the specified altitude. | EXPECT TO CROSS [position] AT OR ABOVE [altitude] | L | L | R |
| 44 | Notification that a altitude change instruction should be expected which will require the specified position to be crossed at or below the specified altitude. | EXPECT TO CROSS [position] AT OR BELOW [altitude] | L | L | R |
| 45 | Notification that a altitude change instruction should be expected which will require the specified position to be crossed at the specified altitude which is to be maintained subsequently. | EXPECT TO CROSS [position] AT AND MAINTAIN [altitude] | L | L | R |
| 46 | The specified position should be crossed at the specified altitude. This may require the aircraft to modify its climb or descent profile. | CROSS [position] AT [altitude] | N | M | W/U |
| 47 | The specified position is to be crossed at or above the specified altitude. | CROSS [position] AT OR ABOVE [altitude] | N | M | W/U |
| 48 | The specified position is to be crossed at or below the specified altitude. | CROSS [position] AT OR BELOW [altitude] | N | M | W/U |
| 49 | Combined instruction that the specified position is to be crossed at the specified altitude and that altitude is to be maintained when reached. | CROSS [position] AT AND MAINTAIN [altitude] | N | M | W/U |
| 50 | The specified position is to be crossed at a altitude between the specified altitudes. | CROSS [position] BETWEEN [altitude] AND [altitude] | N | M | W/U |
| 51 | The specified position is to be crossed at the specified time. | CROSS [position] AT [time] | N | M | W/U |
| 52 | The specified position is to be crossed at or before the specified time. | CROSS [position] AT OR BEFORE [time] | N | M | W/U |
| 53 | The specified position is to be crossed at or after the specified time. | CROSS [position] AT OR AFTER [time] | N | M | W/U |
| 54 | The specified position is to be crossed at a time between the specified times. | CROSS [position] BETWEEN [time] AND [time] | N | M | W/U |
| 55 | The specified position is to be crossed at the specified speed and the specified speed it to be maintained until further advised. | CROSS [position] AT [speed] | N | M | W/U |
| 56 | The specified position is to be crossed at a speed equal to or less than the specified speed and the specified speed or less it to be maintained until further advised. | CROSS [position] AT OR LESS THAN [speed] | N | M | W/U |
| 57 | The specified position is to be crossed at a speed equal to or greater than the specified speed and the specified speed or greater it to be maintained until further advised. | CROSS [position] AT OR GREATER THAN [speed] | N | M | W/U |
| 58 | The specified position is to be crossed at the specified time and the specified altitude. | CROSS [position] AT [time] AT [altitude] | N | M | W/U |
| 59 | The specified position is to be crossed at or before the specified time and at the specified altitude. | CROSS [position] AT OR BEFORE [time] AT [altitude] | N | M | W/U |
| 60 | The specified position is to be crossed at or after the specified time and at the specified altitude. | CROSS [position] AT OR AFTER [time] AT [altitude] | N | M | W/U |
| 61 | Combined instruction that the specified position is to be crossed at the specified altitude and speed and the altitude and speed are to be maintained. | CROSS [position] AT AND MAINTAIN [altitude] AT [speed] | N | M | W/U |
| 62 | Combined instruction that at the specified time the specified position is to be crossed at the specified altitude and the altitude is to be maintained. | AT [time] CROSS [position] AT AND MAINTAIN [altitude] | N | M | W/U |
| 63 | Combined instruction that at the specified time the specified position is to be crossed at the specified altitude and speed and the altitude and speed are to be maintained. | AT [time] CROSS [position] AT AND MAINTAIN [altitude] AT [speed] | N | M | W/U |

Table 7-3:  Crossing Constraints (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 64 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction. | OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 65 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified position. | AT [position] OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 66 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified time. | AT [time] OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 67 | The cleared flight route is to be rejoined. | PROCEED BACK ON ROUTE | N | M | W/U |
| 68 | The cleared flight route is to be rejoined at or before the specified position. | REJOIN ROUTE BY [position] | N | M | W/U |
| 69 | The cleared flight route is to be rejoined at or before the specified time. | REJOIN ROUTE BY [time] | N | M | W/U |
| 70 | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified position. | EXPECT BACK ON ROUTE BY [position] | L | L | R |
| 71 | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified time. | EXPECT BACK ON ROUTE BY [time] | L | L | R |
| 72 | Instruction to resume own navigation following a period of tracking or heading clearances. May be used in conjunction with an instruction on how or where to rejoin the cleared route. | RESUME OWN NAVIGATION | N | M | W/U |

Table 7-4:  Lateral Offsets (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 73 | Notification to the aircraft of the instructions to be followed from departure until the specified clearance limit. | [departure clearance] | N | M | W/U |
| 74 | Instruction to proceed directly from its present position to the specified position. | PROCEED DIRECT TO [position] | N | M | W/U |
| 75 | Instruction to proceed, when able, directly to the specified position. | WHEN ABLE PROCEED DIRECT TO [position] | N | M | W/U |
| 76 | Instruction to proceed, at the specified time, directly to the specified position. | AT [time] PROCEED DIRECT TO [position] | N | M | W/U |
| 77 | Instruction to proceed, at the specified position, directly to the next specified position. | AT [position] PROCEED DIRECT TO [position] | N | M | W/U |
| 78 | Instruction to proceed, upon reaching the specified altitude, directly to the specified position. | AT [altitude] PROCEED DIRECT TO [position] | N | M | W/U |
| 79 | Instruction to proceed to the specified position via the specified route. | CLEARED TO [position] VIA [route clearance] | N | M | W/U |
| 80 | Instruction to proceed via the specified route. | CLEARED [route clearance] | N | M | W/U |
| 81 | Instruction to proceed in accordance with the specified procedure. | CLEARED [procedure name] | N | M | W/U |
| 33 | Instruction to proceed to the intended destination which has no instrument approach procedure authorized or available, and is in uncontrolled airspace. | CLEARED OUT OF CONTROLLED AIRSPACE | N | M | W/U |
| 82 | Approval to deviate up to the specified distance from the cleared route in the specified direction. | CLEARED TO DEVIATE UP TO [distance offset] [direction] OF ROUTE | N | M | W/U |
| 83 | Instruction to proceed from the specified position via the specified route. | AT [position] CLEARED [route clearance] | N | M | W/U |
| 84 | Instruction to proceed from the specified position via the specified procedure. | AT [position] CLEARED [procedure name] | N | M | W/U |
| 85 | Notification that a clearance to fly on the specified route may be issued. | EXPECT [route clearance] | L | L | R |
| 86 | Notification that a clearance to fly on the specified route from the specified position may be issued. | AT [position] EXPECT [route clearance] | L | L | R |
| 87 | Notification that a clearance to fly directly to the specified position may be issued. | EXPECT DIRECT TO [position] | L | L | R |
| 88 | Notification that a clearance to fly directly from the first specified position to the next specified position may be issued. | AT [position] EXPECT DIRECT TO [position] | L | L | R |
| 89 | Notification that a clearance to fly directly to the specified position commencing at the specified time may be issued. | AT [time] EXPECT DIRECT TO [position] | L | L | R |
| 90 | Notification that a clearance to fly directly to the specified position commencing when the specified altitude is reached may be issued. | AT [altitude] EXPECT DIRECT TO [position] | L | L | R |
| 91 | Instruction to enter a holding pattern with the specified characteristics at the specified position and altitude. | HOLD AT [position] MAINTAIN [altitude] INBOUND TRACK [degrees] [direction] TURNS [leg type] | N | M | W/U |
| 92 | Instruction to enter a holding pattern with the published characteristics at the specified position and altitude. | HOLD AT [position] AS PUBLISHED MAINTAIN [altitude] | N | M | W/U |
| 93 | Notification that an onwards clearance may be issued at the specified time. | EXPECT FURTHER CLEARANCE AT [time] | L | L | R |
| 94 | Instruction to turn left or right as specified onto the specified heading. | TURN [direction] HEADING [degrees] | N | M | W/U |
| 95 | Instruction to turn left or right as specified onto the specified track. | TURN [direction] GROUND TRACK [degrees] | N | M | W/U |
| 215 | Instruction to turn a specified number of degrees left or right. | TURN [degrees][direction] | N | M | W/U |
| 190 | Instruction to fly on the specified heading. | FLY HEADING [degrees] | N | M | W/U |
| 96 | Instruction to continue to fly on the current heading. | CONTINUE PRESENT HEADING | N | M | W/U |
| 97 | Instruction to fly on the specified heading from the specified position. | AT [position] FLY HEADING [degrees] | N | M | W/U |
| 221 | Instruction to stop turn at the specified heading prior to reaching the previously assigned heading. | STOP TURN HEADING [degrees] | U | M | W/U |
| 98 | Instruction to turn immediately left or right as specified onto the specified heading. | IMMEDIATELY TURN [direction] HEADING [degrees] | D | H | W/U |
| 99 | Notification that a clearance may be issued for the aircraft to fly the specified procedure. | EXPECT [procedure name] | L | L | R |

Table 7-5:  Route Modifications (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 100 | Notification that a speed instruction may be issued to be effective at the specified time. | AT [time] EXPECT [speed] | L | L | R |
| 101 | Notification that a speed instruction may be issued to be effective at the specified position. | AT [position] EXPECT [speed] | L | L | R |
| 102 | Notification that a speed instruction may be issued to be effective at the specified altitude. | AT [altitude] EXPECT [speed] | L | L | R |
| 103 | Notification that a speed range instruction may be issued to be effective at the specified time. | AT [time] EXPECT [speed] TO [speed] | L | L | R |
| 104 | Notification that a speed range instruction may be issued to be effective at the specified position. | AT [position] EXPECT [speed] TO [speed] | L | L | R |
| 105 | Notification that a speed range instruction may be issued to be effective at the specified altitude. | AT [altitude] EXPECT [speed] TO [speed] | L | L | R |
| 106 | The specified speed is to be maintained. | MAINTAIN [speed] | N | M | W/U |
| 188 | After passing the specified position the specified speed is to be maintained. | AFTER PASSING [position] MAINTAIN [speed] | N | M | W/U |
| 107 | The present speed is to be maintained. | MAINTAIN PRESENT SPEED | N | M | W/U |
| 108 | The specified speed or a greater speed is to be maintained. | MAINTAIN [speed] OR GREATER | N | M | W/U |
| 109 | The specified speed or a lesser speed is to be maintained. | MAINTAIN [speed] OR LESS | N | M | W/U |
| 110 | A speed with the specified range is to be maintained. | MAINTAIN [speed] TO [speed] | N | M | W/U |
| 111 | The present speed is to be increased to the specified speed and maintained until further advised. | INCREASE SPEED TO [speed] | N | M | W/U |
| 112 | The present speed is to be increased to the specified speed or greater, and maintained at or above the specified speed until further advised. | INCREASE SPEED TO [speed] OR GREATER | N | M | W/U |
| 113 | The present speed is to be reduced to the specified speed and maintained until further advised. | REDUCE SPEED TO [speed] | N | M | W/U |
| 114 | The present speed is to be reduced to the specified speed or less and maintained at or below the specified speed until further advised. | REDUCE SPEED TO [speed] OR LESS | N | M | W/U |
| 115 | The specified speed is not to be exceeded. | DO NOT EXCEED [speed] | N | M | W/U |
| 116 | Notification that the aircraft need no longer comply with the previously issued speed restriction. | RESUME NORMAL SPEED | N | M | W/U |
| 189 | The present speed is to be changed to the specified speed. | ADJUST SPEED TO [speed] | N | M | W/U |
| 222 | Notification that the aircraft may keep its preferred speed without restriction. | NO SPEED RESTRICTION | L | L | R |
| 223 | Instruction to reduce present speed to the minimum safe approach speed | REDUCE TO MINIMUM APPROACH SPEED | N | L | W/U |

Table 7-6:  Speed Changes (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 117 | The ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | CONTACT [icaounitname] [frequency] | N | M | W/U |
| 118 | At the specified position the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | AT [position] CONTACT [icaounitname] [frequency] | N | M | W/U |
| 119 | At the specified time the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | AT [time] CONTACT [icaounitname] [frequency] | N | M | W/U |
| 120 | The ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | MONITOR [icaounitname] [frequency] | N | M | W/U |
| 121 | At the specified position the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | AT [position] MONITOR [icaounitname] [frequency] | N | M | W/U |
| 122 | At the specified time the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | AT [time] MONITOR [icaounitname] [frequency] | N | M | W/U |
| 123 | The specified beacon code (SSR code) is to be selected. | SQUAWK [beacon code] | N | M | W/U |
| 124 | The SSR transponder responses are to be disabled. | STOP SQUAWK | N | M | W/U |
| 125 | The SSR transponder responses should include altitude information. | SQUAWK MODE CHARLIE | N | M | W/U |
| 126 | The SSR transponder responses should no longer include altitude information. | STOP SQUAWK MODE CHARLIE | N | M | W/U |
| 179 | The ‘ident’ function on the SSR transponder is to be actuated. | SQUAWK IDENT | N | M | W/U |

Table 7-7:  Contact/Monitor/Surveillance Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 127 | Instruction to report when the aircraft is back on the cleared route. | REPORT BACK ON ROUTE | N | L | R |
| 128 | Instruction to report when the aircraft has left the specified altitude. | REPORT LEAVING [altitude] | N | L | R |
| 129 | Instruction to report when the aircraft is in altitude flight at the specified altitude. | REPORT WHEN LEVEL AT [altitude] | N | L | R |
| 175 | Instruction to report when the aircraft has reached the specified altitude. | REPORT REACHING [altitude] | N | L | R |
| 180 | Instruction to report when the aircraft is within the specified altitude range. | REPORT REACHING BLOCK [altitude] TO [altitude] | N | L | R |
| 130 | Instruction to report when the aircraft has passed the specified position. | REPORT PASSING [position] | N | L | R |
| 181 | Instruction to report the present distance to or from the specified position. | REPORT DISTANCE [to/from] [position] | N | M | Y |
| 184 | Instruction to report at the specified time the distance to or from the specified position. | AT TIME [time] REPORT DISTANCE [to/from] [position] | N | L | Y |
| 228 | Instruction to report the estimated time of arrival at the specified position | REPORT ETA [position] | L | L | Y |
| 131 | Instruction to report the amount of fuel remaining and the number of persons on board. | REPORT REMAINING FUEL AND SOULS ON BOARD | U | M | Y |
| 132 | Instruction to report the present position. | REPORT POSITION | N | M | Y |
| 133 | Instruction to report the present altitude. | REPORT PRESENT ALTITUDE | N | M | Y |
| 134 | Instruction to report the present speed. | REPORT [speed qualifier] [speed type] SPEED | N | M | Y |
| 135 | Instruction to confirm and acknowledge the currently assigned altitude. | CONFIRM ASSIGNED ALTITUDE | N | L | Y |
| 136 | Instruction to confirm and acknowledge the currently assigned speed. | CONFIRM ASSIGNED SPEED | N | L | Y |
| 137 | Instruction to confirm and acknowledge the currently assigned route. | CONFIRM ASSIGNED ROUTE | N | L | Y |
| 138 | Instruction to confirm the previously reported time over the last reported waypoint. | CONFIRM TIME OVER REPORTED WAYPOINT | N | L | Y |
| 139 | Instruction to confirm the identity of the previously reported waypoint. | CONFIRM REPORTED WAYPOINT | N | L | Y |
| 140 | Instruction to confirm the identity of the next waypoint. | CONFIRM NEXT WAYPOINT | N | L | Y |
| 141 | Instruction to confirm the previously reported estimated time at the next waypoint. | CONFIRM NEXT WAYPOINT ETA | N | L | Y |
| 142 | Instruction to confirm the identity of the next but one waypoint. | CONFIRM ENSUING WAYPOINT | N | L | Y |
| 143 | The request was not understood. It should be clarified and resubmitted. | CONFIRM REQUEST | N | L | Y |
| 144 | Instruction to report the selected beacon code. | CONFIRM SQUAWK | N | L | Y |
| 145 | Instruction to report the present heading. | REPORT HEADING | N | M | Y |
| 146 | Instruction to report the present ground track. | REPORT GROUND TRACK | N | M | Y |
| 182 | Instruction to report the identification code of the last ATIS received. | CONFIRM ATIS CODE | N | L | Y |
| 147 | Instruction to make a position report. | REQUEST POSITION REPORT | N | M | Y |
| 216 | Instruction to file a flight plan. | REQUEST FLIGHT PLAN | N | M | Y |
| 217 | Instruction to report that the aircraft has landed. | REPORT ARRIVAL | N | M | Y |
| 229 | Instruction to report the preferred alternate aerodrome for landing. | REPORT ALTERNATE AERODROME | L | L | Y |
| 231 | Instruction to indicate the pilot’s preferred altitude. | STATE PREFERRED LEVEL | L | L | Y |
| 232 | Instruction to indicate the pilot’s preferred time and/or position to commence descent to the aerodrome of intended arrival. | STATE TOP OF DESCENT | L | L | Y |

Table 7-8:  Report/Confirmation Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 148 | Request for the earliest time at which the specified altitude can be accepted. | WHEN CAN YOU ACCEPT [altitude] | N | L | Y |
| 149 | Instruction to report whether or not the specified altitude can be accepted at the specified position. | CAN YOU ACCEPT [altitude] AT [position] | N | L | A/N |
| 150 | Instruction to report whether or not the specified altitude can be accepted at the specified time. | CAN YOU ACCEPT [altitude] AT [time] | N | L | A/N |
| 151 | Instruction to report the earliest time when the specified altitude can be accepted. | WHEN CAN YOU ACCEPT [speed] | N | L | Y |
| 152 | Instruction to report the earliest time when the specified offset track can be accepted. | WHEN CAN YOU ACCEPT [distance offset] [direction] OFFSET | N | L | Y |

Table 7-9:  Negotiation Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 153 | ATS advisory that the altimeter setting should be the specified setting. | ALTIMETER [altimeter] | N | L | R |
| 213 | ATS advisory that the specified altimeter setting relates to the specified facility | [icao facility designator] ALTIMETER [altimeter] | N | L | R |
| 154 | ATS advisory that the radar service is terminated. | RADAR SERVICE TERMINATED | N | L | R |
| 191 | ATS advisory that the aircraft is entering airspace in which no air traffic services are provided and all existing air traffic services are terminated. | ALL ATS TERMINATED | N | M | R |
| 155 | ATS advisory that radar contact has been established at the specified position. | RADAR CONTACT [position] | N | M | R |
| 156 | ATS advisory that radar contact has been lost. | RADAR CONTACT LOST | N | M | R |
| 210 | ATS advisory that the aircraft has been identified on radar at the specified position. | IDENTIFIED [position] | N | M | R |
| 193 | Radar identification has been lost. | IDENTIFICATION LOST | N | M | R |
| 157 | A continuous transmission is detected on the specified frequency. Check the microphone button. | CHECK STUCK MICROPHONE [frequency] | U | M | N |
| 158 | ATS advisory that the ATIS information identified by the specified code is the current ATIS information. | ATIS [atis code] | N | L | R |
| 212 | ATS advisory that the specified ATIS information at the specified airport is current. | [icao facility designator] ATIS [atis code] CURRENT | N | L | R |
| 214 | ATS advisory that indicates the RVR value for the specified runway. | RUNWAY [runway] VISUAL RANGE [rvr] | N | M | R |
| 224 | ATS advisory that no delay is expected. | NO DELAY EXPECTED | N | L | R |
| 225 | ATS advisory that the expected delay has not been determined. | DELAY NOT DETERMINED | N | L | R |
| 226 | ATS advisory that the aircraft may expect to be cleared to commence its approach procedure at the specified time. | EXPECTED APPROACH TIME [time] | N | L | R |

Table 7-10:  Air Traffic Advisories (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 159 | A system generated message that the ground system has detected an error. | ERROR [error information] | U | M | N |
| 160 | Notification to the avionics that the next data authority is the specified ATSU. | NEXT DATA AUTHORITY [icao facility designator] | L | N | N |
| 161 | Notification to the avionics that the data link connection with the current data authority is being terminated. | END SERVICE | L | N | N |
| 162 | Notification that the ground system does not support this message. | SERVICE UNAVAILABLE | L | L | N |
| 234 | Notification that the ground system does not have a flight plan for that aircraft | FLIGHT PLAN NOT HELD | L | L | N |
| 163 | Notification to the pilot of an ATSU identifier. | [icao facility designator] | L | N | N |
| 227 | Confirmation to the aircraft system that the ground system has received the message to which the logical acknowledgment refers and found it acceptable for display to the responsible person. | LOGICAL ACKNOWLEDGMENT | N | M | N |
| 233 | Notification to the pilot that messages sent requiring a logical acknowledgment will not be accepted with this ground system | USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED | N | M | N |

Table 7-11:  System Management Messages (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 164 | The associated instruction may be complied with at any future time. | WHEN READY | L | N | N |
| 230 | The associated instruction is to be complied with immediately. | IMMEDIATELY | D | H | N |
| 165 | Used to link two messages, indicating the proper order of execution of clearances/ instructions. | THEN | L | N | N |
| 166 | The associated instruction is issued due to traffic considerations. | DUE TO [traffic type] TRAFFIC | L | N | N |
| 167 | The associated instruction is issued due to airspace restrictions. | DUE TO AIRSPACE RESTRICTION | L | N | N |
| 168 | The previous communication should be ignored. | DISREGARD | U | M | R |
| 176 | Notification that the operator is responsible for maintaining separation from other traffic and is also responsible for maintaining Visual Meteorological Conditions. | MAINTAIN OWN SEPARATION AND VMC | N | M | W/U |
| 177 | Used in conjunction with a clearance/instruction to indicate that the operator may execute when prepared to do so. | AT PILOTS DISCRETION | L | L | N |
| 169 |  | [free text] | N | L | R |
| 170 |  | [free text] | D | H | R |
| 194 |  | [free text] | N | L | Y |
| 178 |  | [free text] | N | L | N |
| 195 |  | [free text] | L | L | R |
| 196 |  | [free text] | N | M | W/U |
| 197 |  | [free text] | U | M | W/U |
| 198 |  | [free text] | D | H | W/U |
| 199 |  | [free text] | N | M | W/U |
| 200 |  | [free text] | L | L | R |
| 201 |  | [free text] | N | M | W/U |
| 202 |  | [free text] | D | H | W/U |
| 203 |  | [free text] | N | M | R |
| 204 |  | [free text] | N | M | Y |
| 183 |  | [free text] | N | M | N |
| 205 |  | [free text] | N | M | A/N |
| 206 |  | [free text] | L | N | Y |
| 187 |  | [free text] | L | N | N |
| 207 |  | [free text] | L | L | Y |
| 208 |  | [free text] | L | L | N |

Table 7-12:  Additional Messages (uplink)

### Downlink Messages

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 0 | The instruction will be complied with. | WILCO | N | M | N |
| 1 | The instruction cannot be complied with. | UNABLE | N | M | N |
| 2 | Wait for a reply. | STANDBY | N | M | N |
| 3 | Message received and understood. | ROGER | N | M | N |
| 4 | Yes. | AFFIRM | N | M | N |
| 5 | No. | NEGATIVE | N | M | N |

Table 7-13:  Responses (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 6 | Request to fly at the specified altitude. | REQUEST [altitude] | N | L | Y |
| 7 | Request to fly at a altitude within the specified altitude interval. | REQUEST BLOCK [altitude] TO [altitude] | N | L | Y |
| 8 | Request to cruise climb to the specified altitude. | REQUEST CRUISE CLIMB TO [altitude] | N | L | Y |
| 9 | Request to climb to the specified altitude. | REQUEST CLIMB TO [altitude] | N | L | Y |
| 10 | Request to descend to the specified altitude. | REQUEST DESCENT TO [altitude] | N | L | Y |
| 11 | Request that at the specified position a climb to the specified altitude be approved | AT [position] REQUEST CLIMB TO [altitude] | N | L | Y |
| 12 | Request that at the specified position a descent to the specified altitude be approved. | AT [position] REQUEST DESCENT TO [altitude] | N | L | Y |
| 13 | Request that at the specified time a climb to the specified altitude be approved. | AT [time] REQUEST CLIMB TO [altitude] | N | L | Y |
| 14 | Request that at the specified time a descent to the specified altitude be approved. | AT [time] REQUEST DESCENT TO [altitude] | N | L | Y |
| 69 | Request that a descent be approved on a see-and-avoid basis. | REQUEST VMC DESCENT | N | L | Y |

Table 7-14:  Vertical Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 15 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved. | REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |
| 16 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified position. | AT [position] REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |
| 17 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified time. | AT [time] REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |

Table 7-15:  Lateral Off-Set Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 18 | Request to fly at the specified speed. | REQUEST [speed] | N | L | Y |
| 19 | Request to fly within the specified speed range. | REQUEST [speed] TO [speed] | N | L | Y |

Table 7-16:  Speed Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 20 | Request for voice contact. | REQUEST VOICE CONTACT | N | L | Y |
| 21 | Request for voice contact on the specified frequency. | REQUEST VOICE CONTACT [frequency] | N | L | Y |

Table 7-17:  Voice Contact Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 22 | Request to track from the present position direct to the specified position. | REQUEST DIRECT TO [position] | N | L | Y |
| 23 | Request for the specified procedure clearance. | REQUEST [procedure name] | N | L | Y |
| 24 | Request for a route clearance. | REQUEST CLEARANCE [route clearance] | N | L | Y |
| 25 | Request for a clearance. | REQUEST [clearance type] CLEARANCE | N | L | Y |
| 26 | Request for a weather deviation to the specified position via the specified route. | REQUEST WEATHER DEVIATION TO [position] VIA [route clearance] | N | M | Y |
| 27 | Request for a weather deviation up to the specified distance off track in the specified direction. | REQUEST WEATHER DEVIATION UP TO [distance offset] [direction] OF ROUTE | N | M | Y |
| 70 | Request a clearance to adopt the specified heading. | REQUEST HEADING [degrees] | N | L | Y |
| 71 | Request a clearance to adopt the specified ground track. | REQUEST GROUND TRACK [degrees] | N | L | Y |

Table 7-18:  Route Modification Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 28 | Notification of leaving the specified altitude. | LEAVING [altitude] | N | L | N |
| 29 | Notification of climbing to the specified altitude. | CLIMBING TO [altitude] | N | L | N |
| 30 | Notification of descending to the specified altitude. | DESCENDING TO [altitude] | N | L | N |
| 31 | Notification of passing the specified position. | PASSING [position] | N | L | N |
| 78 | At the specified time, the aircraft’s position was as specified. | AT [time] [distance] [to/from] [position] | N | L | N |
| 32 | Notification of the present altitude. | PRESENT ALTITUDE[altitude] | N | L | N |
| 33 | Notification of the present position. | PRESENT POSITION [position] | N | L | N |
| 34 | Notification of the present speed. | PRESENT SPEED [speed] | N | L | N |
| 35 | Notification of the present heading in degrees. | PRESENT HEADING [degrees] | N | L | N |
| 36 | Notification of the present ground track in degrees. | PRESENT GROUND TRACK [degrees] | N | L | N |
| 37 | Notification that the aircraft is maintaining the specified altitude. | ALTITUDE [altitude] | N | L | N |
| 72 | Notification that the aircraft has reached the specified altitude. | REACHING [altitude] | N | L | N |
| 76 | Notification that the aircraft has reached a altitude within the specified altitude range. | REACHING BLOCK [altitude] TO [altitude] | N | L | N |
| 38 | Read-back of the assigned altitude. | ASSIGNED ALTITUDE[altitude] | N | M | N |
| 77 | Read-back of the assigned altitude range. | ASSIGNED BLOCK [altitude] TO [altitude] | N | M | N |
| 39 | Read-back of the assigned speed. | ASSIGNED SPEED [speed] | N | M | N |
| 40 | Read-back of the assigned route. | ASSIGNED ROUTE [route clearance] | N | M | N |
| 41 | The aircraft has regained the cleared route. | BACK ON ROUTE | N | M | N |
| 42 | The next waypoint is the specified position. | NEXT WAYPOINT [position] | N | L | N |
| 43 | the ETA at the next waypoint is as specified. | NEXT WAYPOINT ETA [time] | N | L | N |
| 44 | The next but one waypoint is the specified position. | ENSUING WAYPOINT [position] | N | L | N |
| 45 | Clarification of previously reported waypoint passage. | REPORTED WAYPOINT [position] | N | L | N |
| 46 | Clarification of time over previously reported waypoint. | REPORTED WAYPOINT [time] | N | L | N |
| 47 | The specified beacon code has been selected and the SSR transponder is operational. | SQUAWKING [beacon code] | N | L | N |
| 48 | Position report. | POSITION REPORT [position report] | N | M | N |
| 79 | The code of the latest ATIS received is as specified. | ATIS [atis code] | N | L | N |
| 89 | The specified ICAO unit is being monitored on the specified frequency. | MONITORING [icaounitname] [frequency] | U | M | N |
| 102 | Used to report that an aircraft has landed. | LANDING REPORT | N | N | N |
| 104 | Notification of estimated time of arrival at the specified position. | ETA [position][time] | L | L | N |
| 105 | Notification of the alternative aerodrome for landing. | ALTERNATE AERODROME [airport] | L | L | N |
| 106 | Notification of the preferred level. | PREFERRED LEVEL [altitude] | L | L | N |
| 109 | Notification of the preferred time to commence descent for approach | TOP OF DESCENT [time] | L | L | N |
| 110 | Notification of the preferred position to commence descent for approach | TOP OF DESCENT [position] | L | L | N |
| 111 | Notification of the preferred time and position to commence descent for approach | TOP OF DESCENT [time] position] | L | L | N |

Table 7-19:  Reports (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 49 | Request for the earliest time at which a clearance to the specified speed can be expected. | WHEN CAN WE EXPECT [speed] | L | L | Y |
| 50 | Request for the earliest time at which a clearance to a speed within the specified range can be expected. | WHEN CAN WE EXPECT [speed] TO [speed] | L | L | Y |
| 51 | Request for the earliest time at which a clearance to regain the planned route can be expected. | WHEN CAN WE EXPECT BACK ON ROUTE | L | L | Y |
| 52 | Request for the earliest time at which a clearance to descend can be expected. | WHEN CAN WE EXPECT LOWER ALTITUDE | L | L | Y |
| 53 | Request for the earliest time at which a clearance to climb can be expected. | WHEN CAN WE EXPECT HIGHER ALTITUDE | L | L | Y |
| 54 | Request for the earliest time at which a clearance to cruise climb to the specified altitude can be expected. | WHEN CAN WE EXPECT CRUISE CLIMB TO [altitude] | L | L | Y |
| 87 | Request for the earliest time at which a clearance to climb to the specified altitude can be expected. | WHEN CAN WE EXPECT CLIMB TO [altitude] | L | L | Y |
| 88 | Request for the earliest time at which a clearance to descend to the specified altitude can be expected. | WHEN CAN WE EXPECT DESCENT TO [altitude] | L | L | Y |

Table 7-20:  Negotiation Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 55 | Urgency prefix. | PAN PAN PAN | U | H | N |
| 56 | Distress prefix. | MAYDAY MAYDAY MAYDAY | D | H | N |
| 57 | Notification of fuel remaining and number of persons on board. | [remaining fuel] OF FUEL REMAINING AND [souls on board] SOULS ON BOARD | U | H | N |
| 58 | Notification that the pilot wishes to cancel the emergency condition. | CANCEL EMERGENCY | U | M | N |
| 59 | Notification that the aircraft is diverting to the specified position via the specified route. | DIVERTING TO [position] VIA [route clearance] | U | H | N |
| 60 | Notification that the aircraft is diverting the specified distance in the specified direction off the cleared route. | OFFSETTING [distance offset] [direction] OF ROUTE | U | H | N |
| 61 | Notification that the aircraft is descending to the specified altitude. | DESCENDING TO [altitude] | U | H | N |
| 80 | Notification that the aircraft is deviating from the cleared route by the specified distance in the specified direction. | DEVIATING [distance offset] [direction] OFF ROUTE | U | H | N |

Table 7-21:  Emergency Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 62 | A system generated message that the avionics has detected an error. | ERROR [error information] | U | L | N |
| 63 | A system generated denial to any CPDLC message sent to a ground facility that is not the Current Data Authority. . | NOT CURRENT DATA AUTHORITY | L | L | N |
| 99 | A system generated message to inform a ground facility that it is now the Current Data Authority | CURRENT DATA AUTHORITY | L | L | N |
| 107 | A system generated message sent to a ground system that tries to connect to an aircraft when a current data authority and next data authority are already in place | NOT AUTHORIZED NEXT DATA AUTHORITY | L | L | N |
| 64 | Notification to the ground system that the specified ATSU is the current data authority. | [icao facility designator] | L | L | N |
| 73 | A system generated message indicating the software version number. | [version number] | L | L | N |
| 100 | Notification to the ground system that the aircraft system has received the message to which the logical acknowledgment refers. | LOGICAL ACKNOWLEDGMENT | N | M | N |

Table 7-22:  System Management Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 65 | Used to explain reasons for aircraft operator’s message. | DUE TO WEATHER | L | L | N |
| 66 | Used to explain reasons for aircraft operator’s message. | DUE TO AIRCRAFT PERFORMANCE | L | L | N |
| 74 | States a desire by the aircraft operator to provide his/her own separation under see and avoid conditions. | REQUEST TO MAINTAIN OWN SEPARATION AND VMC | L | L | Y |
| 75 | Used in conjunction with another message to indicate that the operator wishes to execute request when the air crew is prepared to do so. | AT PILOTS DISCRETION | L | L | N |
| 101 | Allows the aircraft operator to indicate a desire for termination of CPDLC service with the current data authority. | REQUEST END OF SERVICE | L | L | Y |
| 103 | Allows the aircraft operator to indicate that he has canceled IFR flight plan. | CANCELLING IFR | N | L | Y |
| 108 | Notification that de-icing action has been completed | DE-ICING COMPLETE | L | L | N |
| 67 |  | [free text] | N | L | N |
| 68 |  | [free text] | D | H | Y |
| 90 |  | [free text] | N | M | N |
| 91 |  | [free text] | N | L | Y |
| 92 |  | [free text] | L | L | Y |
| 93 |  | [free text] | U | H | N |
| 94 |  | [free text] | D | H | N |
| 95 |  | [free text] | U | M | N |
| 96 |  | [free text] | U | L | N |
| 97 |  | [free text] | L | L | N |
| 98 |  | [free text] | N | N | N |

Table 7-23:  Additional Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 81 | We can accept the specified altitude at the specified time. | WE CAN ACCEPT [altitude] AT [time] | L | L | N |
| 82 | We cannot accept the specified altitude. | WE CANNOT ACCEPT [altitude] | L | L | N |
| 83 | We can accept the specified speed at the specified time. | WE CAN ACCEPT [speed] AT [time] | L | L | N |
| 84 | We cannot accept the specified speed. | WE CANNOT ACCEPT [speed] | L | L | N |
| 85 | We can accept a parallel track offset the specified distance in the specified direction at the specified time. | WE CAN ACCEPT [direction] [distance offset] at [time] | L | L | N |
| 86 | We cannot accept a parallel track offset the specified distance in the specified direction. | WE CANNOT ACCEPT [direction] [distance offset] | L | L | N |

Table 7-24:  Negotiation Responses (downlink)