# APPLICATION OVERVIEW

## Introduction

### Purpose

#### This document contains draft Standards and Recommended Practices (SARPs) for the Flight Information Services (FIS) application. The FIS application allows a pilot to request and receive FIS services from ground FIS systems. The Aeronautical Telecommunication Network (ATN) provides the media and protocols to conduct data link communications for the FIS application.

#### Compliance with these standards is a means of assuring that the ATC system will perform its intended functions using data link, and that the FIS application is implemented in a globally uniform and interoperable manner.

### Background

#### The FIS application is designed to enable FIS services to be provided to a pilot via the exchange of messages between aircraft avionics and FIS ground systems.

#### In the performance of this role, the FIS application interacts with the following:

- Aeronautical Telecommunication Network (ATN),

- Context Management (CM) Application,

- The ground FIS system, and

- The aircraft avionics.

#### Two types of FIS contract may be established on request of the pilot:

- the FIS Demand Contract  where the FIS ground system provides the information immediately and once only, and

- the FIS Update Contract where the FIS ground system provides the information and any subsequent update of this information.

#### Multiple "FIS services" may be supported by the FIS application, as for instance:

- Automatic Terminal Information Services (ATIS),

- Precipitation Map Service,

- Terminal Weather Service (TWS),

- Windshear Advisory Service,

- Pilot Report (PIREP) Service,

- Notice to Airmen (NOTAM) Service, and

- Runway Visual Range (RVR) Service.

#### Each of these services should be accessed and used independently of the others and are initiated by the aircraft avionics (and/or pilot). It will not be required that aircraft avionics include the capability for all of the FIS services.

#### The CNS/ATM-1 FIS application supports only the FIS service related to the Automatic Terminal Information Service (ATIS). Additional services and negotiation mechanisms could be incorporated in future packages.

### Structure of Document

#### Chapter 1:  APPLICATION OVERVIEW contains the document's purpose and structure, a summary of the ADSP operational requirements (data link service) that relate to FIS and maps these operational requirements to the functions of FIS.

#### Chapter 2: GENERAL REQUIREMENTS contains requirements of a general nature.

#### Chapter 3: ABSTRACT SERVICE contains the description of the abstract service provided by the application service element (ASE) defined for FIS.

#### Chapter 4: FORMAL DEFINITION OF MESSAGES contains the formal definition of the messages exchanged by FIS-ASEs using the Abstract Syntax Notation Number One (ASN.1).

#### Chapter 5: PROTOCOL DEFINITION describes the exchanges of messages allowed by the FIS protocol, as well as time constraints and the exception handling procedures associated with these exchanges. State tables are also included in this chapter.

#### Chapter 6: COMMUNICATION REQUIREMENTS contains the requirements that the FIS ASE imposes on the underlying communication system.

#### Chapter 7:  FIS USER REQUIREMENTS outlines the requirements that a user of an FIS ASE must meet.

### Explanation of Terms

#### Acronyms

##### The following abbreviations are used in this document:

| ADS | Automatic Dependent Surveillance |
| --- | --- |
| ADSP | ADS Panel |
| AE | Application Entity |
| APDU | Application Protocol Data Unit |
| ATSU | Air Traffic Services Unit |
| ASE | Application Service Element |
| ASI | Abstract Service Interface |
| ASN.1 | Abstract Syntax Notation One |
| ATC | Air Traffic Control |
| ATIS | Automatic Terminal Information Services |
| ATM | Air Traffic Management |
| ATN | Aeronautical Telecommunication Network |
| ATS | Air Traffic Services |
| ATNP | ATN Panel |
| CF | Control Function |
| CM | Context Management |
| CNS | Communications, Navigation and Surveillance |
| DC | Demand Contract |
| DS | Dialogue Service |
| FD | Functional Description |
| FIS | Flight Information Services |
| HI | High Interface |
| IA5 | International Alphabet Number 5 |
| ICAO | International Civil Aviation Organization |
| IEC | Internation Electrotechnical Committee |
| IS | International Standard |
| ISO | International Organization for Standardization |
| ITU | International Telecommunications Union |
| LI | Low Interface |
| NOTAM | Notice to Airmen |
| OSI | Open Systems Interconnection |
| PIREP | Pilot Report |
| PER | Packed Encoding Rules |
| QOS | Quality Of Service |
| RCP | Required Communication Performance |
| RER | Residual Error Rate |
| SARPs | Standards and Recommended Practices |
| TWS | Terminal Weather Service |
| UC | Update Contract |
| UTC | Coordinated Universal Time |

#### General Definitions

##### For the purpose of this document, the following definitions apply:

- Active User: the FIS-AE-user is currently involved in a FIS Contract.

- Application Entity: a module of those aspects of an Application Process that are significant for the view point of accessing OSI capabilities.

- Application Process: an element within an open system which performs information processing task for a particular application.

- AE-Qualifier:  that part of the AE-Title that uniquely identifies the particular Application Entity.

- AE-Title: a unique name for an Application Entity.

- APDU: basic unit of information exchange between the airborne FIS AE and the ground FIS AE.

- ASE: an abstract module of a system providing a service to other parts of the system.

- Context Management: an independent service that meets addressing requirements. It provides the mechanism for aircraft and FIS ground system peers to indicate availability to other ATN users and to convey the addresses to be employed.

- Demand Contract: a contract to provide a single piece of FIS information.

- Dialogue Service: the service which allows the FIS-air-ASE to communicate with a FIS-ground-ASE and vice-versa.

- FIS-AE Abstract Service Interface: the abstract interface between the FIS-users and the FIS-service-provider.

- FIS-ASE Abstract Service Interface: the abstract interface through which the FIS-ASE services are accessed.

- Note 1.— In version 1 of the FIS Application, this interface coincides with the FIS-AE abstract service interface

- FIS Abstract Service Interface: the abstract interface between the FIS-air-AE and the FIS-air-user or between the FIS-ground-AE and the FIS-ground-user. In CNS/ATM-1 Package, this interface identifies both the FIS AE abstract service interface and the FIS-ASE abstract service interface.

- FIS-air-ASE : the abstract part of the aircraft system which performs the communications related functions of FIS.

- FIS-air-user: the abstract part of the aircraft system which performs the non communication related functions of FIS.

- FIS-ground-ASE : the abstract part of the ground system which performs the communications related functions of FIS.

- FIS-ground-user: the abstract part of the ground system which performs the non communication related functions of FIS.

- FIS-CF:  the abstract part of the AE which performs the mapping between the FIS-ASE service primitives and others elements within the FIS Application.

- FIS-ASE: the FIS-air-ASE and the FIS-ground-ASE.

- FIS-user: the FIS-air-user or the FIS-ground-user.

- FIS service-primitive: a function of an FIS AE that is not broken down further into subfunctions, and is presented as part of the abstract service interface (ie request, indication, response or confirmation).

- FIS service-provider: the FIS service-provider is composed of the ground and airborne FIS AEs, all underlying data communication protocol entities and the physical media. As a consequence, it encompasses everything between the FIS-AE service interfaces of the end-users of the FIS application.

- Residual Error Rate : the ratio of messages mis-delivered, non-delivered, or delivered with an error undetected by the system, over the total number of messages delivered to the system.

- Update Contract: a contract to provide a piece of FIS information and any update of this information.

#### Conventions For Expressing Requirements

##### The following conventions apply for expressing requirements in this document:

- shal.l	 	used to state a mandatory requirement.

- should	used to state a recommended practice.

### References

#### The following references are used within this document:

- [1]	ADS Panel WG/B Detailed description of the ATIS Service. Version 0.5 		(10/07/1995).

- [2] 	ISO/IEC IS 8825-2, ITU-T Recommendation X.691,  Information 			Technology 	- ASN.1 Encoding Rules - Packed Encoding Rules (PER).

- [3]	ISO/IEC 8824-1, ITU-T Recommendation X.682, Information 			Technology - 	Abstract Syntax Notation One (ASN.1).

- [4]	Draft SARPs for ATN Upper Layers for the CNS/ATM-1 Package (as 		proposed in December 1995).

- [5]	 Reference Document for Traffic Types

## Application Functionality

### The operational and technical functional requirements for FIS are derived directly from the operational requirements stated by the ADS Panel in [1].

### Mapping of Data Link Service to Functional Descriptions

#### Table 1-1 shows which functional descriptions as described in chapter 1.3. assist in achieving the ATIS Data Link Service and are labelled as follows (section reference in parentheses):

- FD1: Establishment and Operation of a FIS Demand Contract (Section 1.3.1),

- FD2: Establishment and Operation of a FIS Update Contract  (Section 1.3.2).

- FD3: Cancellation of Contracts (Section 1.3.3)

| Service | FD 1 | FD2 | FD 3 |
| --- | --- | --- | --- |
| ATIS data link service | Ö | Ö | Ö |

Table 1-1 Data Link Service to Functional Description Mapping

## Functional Descriptions

### Establishment and Operation of a FIS Demand Contract (FD1)

#### Functional Description

##### This function allows the airborne system to establish a FIS Demand Contract with a ground FIS system. Realisation of the contract involves the sending of a single FIS report from the ground FIS system to the aircraft, optionally after the sending of a positive acknowledgement.

##### Any number of FIS demand contracts may be established in parallel with a ground FIS server.

##### The actions performed by the FIS systems supporting the Demand Contract function are the following:

- the airborne FIS system formats and sends to the ground FIS system a FIS demand contract message. This identifies the type of FIS information requested and the parameters of the request,

- the ground FIS system then determines whether or not it is able to comply with the request:

  - if the ground FIS system detects that the requested FIS information can be retrieved but is not yet available, the ground FIS system formats and sends to the airborne FIS system a FIS positive acknowledgement message first to indicate its acceptance of the contract, and the FIS report message later,

  - if the ground FIS system can comply promptly with the FIS demand contract request it sends the FIS report message as soon as possible, or

  - if there are errors in the demand contract request message, or if the ground FIS system cannot comply with the request, it sends a FIS Contract Reject message to the airborne system indicating the reason for its inability to accept the contract.

##### When an unrecoverable error situation is detected by the airborne or the ground FIS system or when either of the users request the abrupt termination of the FIS Demand Contract, the FIS system formats and sends to the peer system a FIS abort  message indicating the source and the reason of the abort. All pending FIS contracts are aborted.

#### Message Descriptions

##### The FIS Demand Contract message contains the following information:

- the reference of the FIS Contract,

- the type of FIS information requested (in CNS/ATM-1 Package, this field always has the abstract value ‘ATIS’),

- where ATIS information is requested, the parameters of the ATIS request, ie:

  - the airport identifier,

  - optionally, the type of the requested ATIS (arrival or departure).

##### The FIS Report message contains the following information:

- the reference of the FIS Contract,

- the type of the FIS information returned (in CNS/ATM-1 Package, this field always has the value ‘ATIS’),

- where ATIS information is returned, the parameters of the ATIS, ie:

  - the airport identifier,

  - the version number of the returned ATIS,

  - the type of the returned ATIS (departure, arrival, both or combined),

  - optionally, the time of observation of the returned ATIS,

  - the ATIS information elements, ie:

    - the mandatory ATIS elements: identification of the runway(s) in use, runway surface conditions,  other operational information, surface winds, visual visibility, cloud, air temperature, dew point temperature, altimeter setting, sigMet, specific ATIS instructions, and

    - the optional ATIS elements: approach type, braking action, holding delay, transition level, runway visibility range,  present weather, trend type landing forecast.

##### The FIS Positive Acknowledgement message contains the following information:

- the reference of the FIS Contract, and

- the type of the FIS information covered by this acknowledgement (in CNS/ATM-1 Package, this field always has the value ‘ATIS’),

##### The FIS Contract Reject message contains the following information:

- the reference of the FIS Contract,

- the type of the FIS request rejected (in CNS/ATM-1 Package, this field always has the value ‘ATIS’),

- where an ATIS request is rejected, the reason of the rejection.

##### The FIS Abort message contains the following information:

- the type of the FIS request aborted (in CNS/ATM-1 Package, this field always has the value ‘ATIS’),

- where an ATIS request is aborted:

  - the source of the abort of the FIS contract (ie the FIS service-provider or the FIS service-user), and

  - if the source is the FIS service-provider, the reason of the abort.

### Establishment and Operation of an FIS Update Contract (FD2)

#### Functional Description

##### This function allows the FIS airborne system to establish an Update Contract with a FIS ground system. Realisation of the contract involves the sending of FIS reports from the FIS ground system to the aircraft each time the requested FIS information is modified.

##### Any number of FIS update contracts may be established in parallel with a ground FIS server.

##### The actions performed by the FIS systems supporting the FIS Update Contract function are the following:

- the airborne FIS system formats and sends to the ground FIS system a FIS update contract message. This identifies the type of FIS information requested and the parameters of the request,

- the ground FIS system then determines whether or not it is able to comply with the request:

  - If the ground FIS system can comply with the FIS update contract request, it sends the first FIS report message as soon as possible,

  - if the ground FIS system detects that the requested FIS information can be retrieved but is not yet available, it formats and sends to the airborne FIS system a positive acknowledgement message first to indicate its acceptance of the contract, and sends the FIS report messages later,

  - if there are errors in the FIS update contract message, or if the ground FIS system cannot comply with the request, it sends a FIS Contract Reject message to the airborne system indicating the reason for its inability to accept the contract.,

  - where the ground FIS system does not support the update contract function but the requested FIS information is available, it sends a FIS negative acknowledgement message containing the requested FIS information and the reason for the rejection,

  - whenever the requested FIS information is modified, a new FIS report message is sent to the aircraft.

##### When an error situation is detected by the airborne or the ground FIS system or when either of the users request the abrupt termination of the FIS Update Contract, the FIS system formats and sends to the peer system a FIS abort message indicating the source and the reason of the abort. All pending FIS contracts are aborted.

#### Message Descriptions

##### The Update Contract message has the same contents as the Demand Contract message as described in section 1.3.1.2.

##### The FIS Report messages have the same contents as in operation of a demand contract as described in section 1.3.1.2.

##### The Contract Reject message contains the following information:

- the reference of the FIS contract,

- the type of the FIS request rejected (in CNS/ATM-1 Package, this field always has the value ‘ATIS’), and

- where an ATIS request is rejected:

  - the reason of the rejection of the FIS request,, and

  - the current value of the requested ATIS, if the reason of the rejection is « FIS Update Contract function not supported by the ground FIS system ».

##### The FIS Positive Acknowledgement has the same contents as in operation of a demand contract as described in section 1.3.1.2.

##### The FIS Abort message has the same contents as in operation of a demand contract as described in section 1.3.1.2.

### Cancellation of Contracts (FD3)

#### Functional Description

##### This function allows both air and ground system explicitly to cancel a particular FIS update contract that is in operation.

- an FIS update contract cancel message is sent by the system initiating the termination. Any FIS report message previously sent is delivered to the aircraft before the contract is effectively ended. Other pending FIS contracts are not disrupted by the termination of a particular FIS contract,

- the cancellation of the update contract is confirmed to the FIS-user by the system receiving the FIS update contract cancel message. A cancel update contract accept message is sent back.

##### The airborne system may also cancel several contracts (demand and update contracts) in a single FIS Cancel Contracts message. The ground system cancels these contracts and ackowledges the cancellation by sending the FIS Cancel Contracts Accept. The cancellation is made on the basis of the type(s) of contract (only ATIS for Package-1) supplied by the airborne system.

#### Message Description

##### The FIS Update Contract Cancel message contains the following information:

- the reference of the FIS contract,

- the type of the FIS-update-contract cancelled (in the CNS/ATM-1 Package, this field always has the value ‘ATIS’),

- where an ATIS contract is cancelled, there is no additional information.

##### The FIS Update Contract Cancel Accept message contains the following information:

- the reference of the FIS contract,

- the type of the FIS-update-contract cancelled (in the CNS/ATM-1 Package, this field always has the value ‘ATIS’),

- where an ATIS contract is cancelled, there is no additional information.

##### The FIS Contract Contracts message contains the following information:

- the types of the FIS contracts to be cancelled (in the CNS/ATM-1 Package, this field always has the value ‘ATIS’),

##### The FIS Contract Contracts Accept message contains the following information:

- the types of the FIS contracts cancelled (in the CNS/ATM-1 Package, this field always has the value ‘ATIS’),

# GENERAL REQUIREMENTS

## Performance Requirements

Note 1. — System developed to support CNS/ATM-1 Package FIS functionality will be capable of meeting the Required Communication Performance (RCP) appropriate for the phase of operation as specified in [5].

## Time Accuracy Requirements

### Absolute times which are sent as parametersz over the data link shall  ([1])  be as accurate as the required resolution of the time parameter.

### Absolute times shall  ([2])  be UTC.

### Dates shall  ([3])  be expressed as UTC date (ie, the day increments at 2359:59z).

### Relatives times shall  ([4])  be accurate to ñ 0.1 second.

### Where timestamps are used they shall  ([5])  consist of year, month, day and hour, minute, second.

## Security Requirements

Note 1.— There are no internationally approved operational requirements relating to data link application security.

## Backwards Compatibility Requirements

Note 1.— This specification describes the version 1 of the CNS/ATM-1 Package FIS application.  Best effort will be made to ensure that subsequent versions of this protocol will be backwards compatible.

### For CNS/ATM-1 the FIS-air-ASE and FIS-ground-ASE version numbers shall  ([6])  both be set to one.

## Error Processing Requirements

### In the event of information input by the user being incompatible with that able to be processed by the system, the user shall  ([7])  be notified.

### In the event of a user invoking a FIS service primitive when the FIS ASE is not in a state specified in chapter 5 the invocation shall  ([8])  be rejected and the user shal.l be notified.

# THE ABSTRACT SERVICE

## Introduction

Note 1.— This section defines the abstract service interface for the FIS service. The FIS-ASE abstract service is described in this chapter from the viewpoint of the FIS-air-user, the FIS-ground-user and the FIS-ASE-service-provider.

Note 2.— This section defines the static behaviour (ie the format) of the FIS abstract service. Its dynamic behaviour (ie how it is used) is described in section 7.

## The FIS Functional Model

Note 1. — Figure 3-1 shows the functional model of the FIS Application. The functional modules identified in this model are the following:

- the FIS-user,

- the FIS Application Entity (FIS-AE) service interface,

- the FIS-AE,

- the FIS Control Function (FIS-CF),

- the FIS Application Service Element (FIS-ASE) service interface,

- the FIS-ASE, and

- the Dialogue Service (DS) interface.

Figure 3-1: Functional Model of the FIS Application

Note 2.—The FIS-user represents the operational part of the FIS system. This user does not perform the communication functions but relies on a communication service provided to it via the FIS-AE through the FIS-AE service interface. The individual actions at this interface are called FIS-AE service primitives. Similarly, individual actions at other interfaces in the communication system are called service primitives at these interfaces.

Note 3.— The FIS-AE consists of several elements, including the FIS-ASE and the FIS-CF. The DS interface is made available by the FIS-CF to the FIS-ASE for communication with the peer FIS-ASE.

Note 4.— The FIS-ASE is the element in the communication system which executes the FIS specific protocol. In other words, it takes care of the FIS specific service primitive sequencing actions, message creation, timer management, error and exception handling.

Note 5.— The FIS-ASE interfaces only with the FIS-CF. This FIS-CF is responsible for mapping service primitives received from one element (such as the FIS-ASE and the FIS-user) to other elements which interface with it. The part of the FIS-CF which is relevant from the point of view of these SARPs, ie the part between the FIS-user and the FIS-ASE, will map FIS-AE service primitives to FIS-ASE service primitives transparently in the CNS/ATM-1 Package.

Note 6.— The DS interface is the interface between the FIS-ASE and part of FIS-CF underneath, and provides the dialogue service ({4}).

## The FIS-ASE Abstract Service

### Implementations of either the FIS ground based service or the FIS air based service shall  ([9])  exhibit external behaviour consistent with having implemented a FIS-ground-ASE, or FIS-air-ASE respectively.

Note 1.— There is no requirement to implement the service in an FIS product; however, it is necessary to implement the ground based and air based system in such a way that it will be impossible to detect (from the peer system) whether or not an interface has been built.

### The FIS-ASE abstract service shall  ([10])  consist of the following:

- FIS-demand-contract service as defined in section 3.5

- FIS-update-contract service as defined in section 3.6,

- FIS-report service as defined in section 3.7,

- FIS-cancel-contracts service as defined in section 3.8,

- FIS-cancel-update-contract service as defined in section 3.9,

- FIS-user-abort service as defined in section 3.10, and

- FIS-provider-abort service as defined in section 3.11.

## Conventions

Note 1.— For a given primitive, the presence of each parameter is described by one of the following values in the parameter tables of this chapter:

- blank		not present,

- C		conditional upon some predicate explained in the text,

- C(=)		conditional upon the value of the parameter to the immediate left 			being present, and equal to that value,

- M		mandatory,

- M(=)		mandatory, and equal to the value of the parameter to 				the immediate left, and

- U		user option.

Note 2.—The following abbreviations are used in this document:

- Req  		request; data is input by FIS-user initiating the service to its 				respective ASE,

- Ind  		indication; data is indicated by the receiving ASE to its 				respective FIS-user,

- Rsp 		response; data is input by receiving FIS-user to its 					respective ASE, and

- Cnf  		confirmation; data is confirmed by the initiating ASE to its 				respective FIS-user.

Note 3.— An unconfirmed service allows just one message to be transmitted, in one direction.

Note 4.— A confirmed service provides end-to-end confirmation that a message sent by one user was received by its peer user.

## FIS-demand-contract Service

Note 1.— The FIS-demand-contract service allows the FIS-air-user to request a FIS-demand-contract with a FIS ground server. It is a confirmed service, initiated by the FIS-air-user.

### The FIS-demand-contract service primitives shall  ([11])  contain the parameters as presented in Table 3-1.

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| ICAO Facility Designator | M |  |  |  |
| FIS Contract Number | M | M | M | M |
| Class of Communication Service | U |  |  |  |
| FIS Contract Details | M | M(=) |  |  |
| Result |  |  | M | M(=) |
| Reject Reason |  |  | C | C(=) |
| FIS Information |  |  | C | C(=) |

Table 3-1: FIS-demand-contract service parameters

### ICAO Facility Designator

Note 1.— This parameter contains the FIS-ground-ASE ICAO facility designator.

#### The ICAO Facility Designator parameter value shall  ([12])  conform to an abstract value corresponding to a eight-character ICAO Facility Designator.

### FIS Contract Number

Note 1.— This parameter contains the user-defined reference of the requested FIS demand contract.

#### The FIS Contract Number parameter value shall  ([13])  be mapped onto the ASN.1 abstract syntax ContractNumber.

Note 2.— The mapping function between the user-supplied FIS Contract Number parameter and the ContractNumber ASN.1 type is a local issue.

#### The values of the FIS Contract Number parameters in the request and the confirm primitives (the indication and response primitives respectively) shall  ([14])  be the same.

### Class of Communication Service

Note 1.— This parameter contains the value of the required class of communication service, if specified by the FIS-air-user.

#### Where specified by the FIS-air-user, the Class of Communication Service parameter value shall  ([15])  have one of the following abstract values: ”A”, ”B”, ”C”, ”D”, ”E”, ”F”, ”G” or ”H”.

Note 1.— Classes of Communication Service are detailed in [5].

Note 2.— If FIS contracts are currently in place, the Class of Communication Service parameter is not used by the FIS service provider.

Note 3.— Where not specified by the FIS-air-user, when there are no FIS Contracts already in force, this indicates that there is no routing preference.

### FIS Contract Details

Note 1.— This parameter contains the details of the FIS Demand Contract as requested by the FIS-air-user.

#### The FIS Contract Details parameter value shall  ([16])  conform to the ASN.1 abstract syntax FISRequestData.

Note 2.— This parameter identifies also the type of the requested FIS information. For CNS/ATM-1 Package, the requested information is always of type "ATIS".

### Result

Note 1.— The value of this parameter indicates whether the FIS-demand-contract request has been accepted or rejected by the FIS-ground-user.

#### The Result parameter value shall  ([17])  conform to one of the following abstract values:

- ”accepted”,

- ”positive acknowledgment”, and

- ”rejected”.

### Reject Reason

Note 1.— The value of this parameter contains the reason of the rejection.

#### The Reject Reason parameter value shall  ([18])  conform to one of the following abstract values:

- ”can not comply”,

- ”FIS service unavailable”,

- ”error detected in the FIS request”, and

- ”undefined”.

#### This parameter shall  ([19])  be present only when the Result parameter contains the abstract value "rejected ".

### FIS Information

Note 1.— This parameter contains the FIS information, as requested by the FIS-air-user.

#### The FIS Information parameter value shall  ([20])  conform to the ASN.1 abstract syntax FISReport Data.

Note 2.— This parameter identifies also the type of the returned FIS information. For CNS/ATM-1 Package, the requested information is always of type "ATIS".

#### This parameter shall  ([21])  be present only when the Result parameter contains the abstract value "accepted".

## FIS-update-contract Service

Note 1.— The FIS-update-contract service allows the FIS-air-user to request a FIS update contract with a FIS ground server. It is confirmed service, initiated by the FIS-air-user.

### The FIS-update-contract service primitives shall  ([22])  contain the parameters as presented in Table 3-2.

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| ICAO Facility Designator | M |  |  |  |
| FIS Contract Number | M | M | M | M |
| Class of Communication Service | U |  |  |  |
| FIS Contract Details | M | M(=) |  |  |
| Result |  |  | M | M(=) |
| Reject Reason |  |  | C | C(=) |
| FIS Information |  |  | C | C(=) |

Table 3-2: FIS-update-contract service parameters

### ICAO Facility Designator

Note 1.— This parameter contains the FIS-ground-ASE ICAO facility designator.

#### The ICAO Facility Designator parameter value shall  ([23])  conform to an abstract value corresponding to a eight-character ICAO Facility Designator.

### FIS Contract Number

Note 1.— This parameter contains the user-defined reference of the requested FIS demand contract.

#### The FIS Contract Number parameter value shall  ([24])  be mapped onto the ASN.1 abstract syntax ContractNumber.

Note 2.— The mapping function between the user-supplied FIS Contract Number parameter and the ContractNumber ASN.1 type is a local issue.

#### The values of the FIS Contract Number parameters in the request and the confirm primitives (the indication and response primitives respectively) shall  ([25])  be the same.

### Class of Communication Service

Note 1.— This parameter contains the value of the required class of communication service, if specified by the FIS-air-user.

#### Where specified by the FIS-air-user, the Class of Communication Service parameter value shall  ([26])  have one of the following abstract values: ”A”, ”B”, ”C”, ”D”, ”E”, ”F”, ”G” or ”H”.

Note 1.— Classes of Communication Service are detailed in [5].

Note 2.— If FIS contracts are currently in place, the Class of Communication Service parameter is not used by the FIS service provider.

Note 3.— Where not specified by the FIS-air-user, when there are no FIS Contracts already in force, this indicates that there is no routing preference.

### FIS Contract Details

Note 1.— This parameter contains the details of the FIS Demand Contract as requested by the FIS-air-user.

#### The FIS Contract Details parameter value shall  ([27])  conform to the ASN.1 abstract syntax FISRequestData.

Note 2.— This parameter identifies also the type of the requested FIS information. For CNS/ATM-1 Package, the requested information is always of type "ATIS".

### Result

Note 1.— The value of this parameter indicates whether the FIS-demand-contract request has been accepted or rejected by the FIS-ground-user.

#### The Result parameter value shall  ([28])  conform to one of the following abstract values:

- ”accepted”,

- ”positive acknowledgment”, and

- ”rejected”.

### Reject Reason

Note 1.— This parameter value contains the reason of the rejection.

#### The Reject Reason parameter value shall  ([29])  conform to one of the following abstract values:

- ”can not comply”,

- ”FIS service unavailable”,

- ”error detected in the FIS request”,

- ”update contract function not supported by the FIS-ground-user”, and

- ”undefined”.

#### This parameter shall  ([30])  be present only if the Result parameter contains the abstract value "rejected".

### FIS Information

Note 1.— This parameter contains the FIS information requested by the FIS-air-user.

#### The FIS Information parameter value shall  ([31])  conform to the ASN.1 abstract syntax FISReportData.

Note 2.— This parameter identifies also the type of the returned FIS information. For CNS/ATM-1 Package, the requested information is always of type "ATIS".

#### This parameter shall  ([32])  be present when:

- the Result parameter contains the abstract value "accepted", or

- the Result parameter contains the abstract value "rejected" and the RejectReason parameter contains the abstract value "update contract function not supported by the FIS-ground-user".

## FIS-report Service

Note 1.— The FIS-report service allows the FIS-ground-user to send to the FIS-air-user the requested FIS information and any update of this FIS information. It is a unconfirmed service initiated by the FIS-ground-user.

### The FIS-report service primitives shall  ([33])  contain the parameters as presented in Table 3-3.

| Parameter Name | Req | Ind |
| --- | --- | --- |
| FIS Contract Number | M | M(=) |
| FIS Information | M | M(=) |

Table 3-3: FIS-report service parameters

### FIS Contract Number

Note 1.— This parameter contains the user-defined reference of the requested FIS demand contract.

#### The FIS Contract Number parameter value shall  ([34])  be mapped onto the ASN.1 abstract syntax ContractNumber.

Note 2.— The mapping function between the user-supplied FIS Contract Number parameter and the ContractNumber ASN.1 type is a local issue.

### FIS Information

Note 1.— This parameter contains the FIS information requested by the FIS-air-user.

#### The parameter value shall  ([35])  conform to the ASN.1 abstract syntax FISReportData.

Note 2.— This parameter identifies also the type of the returned FIS information. For CNS/ATM-1 Package, the requested information is always of type "ATIS".

## FIS-cancel-contracts Service

Note 1.— The FIS-cancel-contracts service allows the FIS-air-user to cancel all FIS demand and update contracts of the same type with a particular FIS ground system. It is a confirmed service initiated by the FIS-air-user.

### The FIS-cancel-contracts service primitives shall  ([36])  contain the parameters as presented in Table 3-4.

| Parameter Name | Req | Ind | Cnf |
| --- | --- | --- | --- |
| FIS Service Type | M | M(=) | M(=) |

Table 3-4: FIS-cancel-contracts service parameters

### FIS Service Type

Note 1.— This parameter shall  ([37])  identify the types of the FIS contracts to be canceled.

#### The parameter value shall  ([38])  conform to the ASN.1 abstract syntax FISCancelContracts.

Note 2.— For CNS/ATM-1 Package, this parameter will always identify the service type "ATIS".

## FIS-cancel-update-contract Service

Note 1.— The FIS-cancel-update-contract service allows the FIS-air-user or the FIS-ground-user to cancel orderly an active FIS update contract. The FIS reports in transit in the communication system are delivered before the FIS update contract is canceled. This service does not affect the other pending FIS contracts. This is a confirmed service, initiated by the FIS-air-user or the FIS-ground-user.

### The FIS-update-contract service primitives shall  ([39])  contain the parameters as presented in Table 3-5.

| Parameter Name | Req | Ind | Cnf |
| --- | --- | --- | --- |
| FIS Contract Number | M | M(=) | M(=) |

Table 3-5: FIS-cancel-update-contract service parameters

### FIS Contract Number

Note 1.— This parameter contains the user-defined reference of the requested FIS demand contract.

#### The FIS Contract Number parameter value shall  ([40])  be mapped onto the ASN.1 abstract syntax ContractNumber.

Note 2.— The mapping function between the user-supplied FIS Contract Number parameter and the ContractNumber ASN.1 type is a local issue.

## FIS-user-abort Service

Note 1.— The FIS-user-abort service allows the FIS-air-user or the FIS-ground-user to abort all active FIS contracts (both FIS demand contracts and FIS update contracts). As a consequence, all active FIS contracts processed by the ASE invocation are canceled and all used contract numbers are freed. Messages in transit may be lost during this operation. This is an unconfirmed service, initiated by the FIS-air-user or the FIS-ground-user.

### The FIS-user-abort service primitives shall  ([41])  contain the parameters as presented in Table 3-6.

| Parameter Name | Req | Ind |
| --- | --- | --- |
| none |  |  |

Table 3-6: FIS-user-abort service parameters

## FIS-provider-abort Service

Note 1.— The FIS-provider-abort service is used by the FIS-ASE service-provider to inform its users if active that it can not continue to provide any more the FIS service. As a consequence, all active FIS contracts (both FIS demand contract and FIS update contract) are canceled and all used contract numbers are freed. This service is a FIS-ASE service-provider initiated service.

### The FIS-provider-abort service primitives shall  ([42])  contain the parameters as presented in Table 3-7.

| Parameter Name | Ind |
| --- | --- |
| Reason | M |

Table 3-7: FIS-provider-abort service parameters

### Reason

Note 1.— This parameter identifies the reason for the abort.

#### The Reason parameter value shall  ([43])  conform to the ASN.1 abstract syntax  FISProtocolErrorDiag.

# FORMAL DEFINITIONS OF MESSAGES

ASN.1

### Structure and content of messages exchanged between the FIS-ground-ASO and the FIS-air-ASO shall  ([44])  be conformed to the abstract syntax described in this section using ISO Abstract Syntax Notation One (ASN.1).

FISMessageSetVersion1 DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- ------------------------------------------------------------------------------

-- FIS messages (Top level)

-- ------------------------------------------------------------------------------

FISDownlinkAPDU

::= SEQUENCE

{

time	DateTimeGroup,

fisDownlinkAPDU	DownlinkAPDU

}

FISUplinkAPDU

::= SEQUENCE

{

time	DateTimeGroup,

fisUplinkAPDU	UplinkAPDU

}

DownlinkAPDU

::= CHOICE

{

fISRequest			FISRequest,

fISCancelUpdateContract			FISCancelUpdateContract,

fISCancelUpdateAccept			FISCancelUpdateAccept,

fisCancelContracts			FISCancelContracts,

fISAbort			FISAbort

...

}

UplinkAPDU

::= CHOICE

{

fISAccept		FISAccept,

fISReject		FISReject,

fISReport		FISReport,

fISCancelUpdateContract		FISCancelUpdateContract,

fISCancelUpdateAccept		FISCancelUpdateAccept,

fISCancelContractsAccept		FISCancelContractsAccept,

fISAbort		FISAbort

...

}

-- ------------------------------------------------------------------------------

-- FIS messages (2nd level)

-- ------------------------------------------------------------------------------

FISAbort

::= CHOICE

{

-- Automatic Terminal Information Service (ATIS) Request

atis		FISProtocolErrorDiag

...

}

FISAccept

::= SEQUENCE

{

contractNumber		ContractNumber,

fISAcceptData		FISAcceptData

}

FISAcceptData

::= CHOICE

{

-- Automatic Terminal Information Service (ATIS)

atis		ATISAccept

...

}

FISCancelContracts

::= SEQUENCE OF FISServiceType

FISCancelContractsAccept

::= SEQUENCE OF FISServiceType

FISCancelAcceptData

::= CHOICE

{

-- Automatic Terminal Information Service (ATIS) Request

atis		NULL

...

}

FISCancelUpdateAccept

::= SEQUENCE

{

fISUpdateContractNumber		ContractNumber,

fISCancelAcceptData		FISCancelAcceptData

}

FISCancelUpdateContract

::= SEQUENCE

{

fISUpdateContractNumber		ContractNumber,

fISCancelUpdateData		FISCancelUpdateData

}

FISCancelUpdateData

::= CHOICE

{

-- Automatic Terminal Information Service (ATIS) Request

atis		NULL

...

}

FISReject

::= SEQUENCE

{

contractNumber		ContractNumber,

fISRejectData		FISRejectData

}

FISRejectData

::= CHOICE

{

-- Automatic Terminal Information Service (ATIS)

atis		ATISReject

...

}

FISReport

::= SEQUENCE

{

contractNumber		ContractNumber,

fISReportData		FISReportData

}

FISReportData

::= CHOICE

{

-- Automatic Terminal Information Service (ATIS)

atis		ATISReport

...

}

FISRequest

::= SEQUENCE

{

contractNumber	ContractNumber,

contractType	ContractType DEFAULT demandContract,

fISRequestdata	FISRequestData

}

FISRequestData

::= CHOICE

{

-- Automatic Terminal Information Service (ATIS) Request

aTISRequest		ATISRequest

...

}

FISProtocolErrorDiag

::= ENUMERATED

{

timerExpiration		(0),

protocolError		(1),

sequenceError   		(2),

decodingError		(3),

unrecoverableInternalError		(4),

invalidContractNumber		(5),

undefined		(6)

...

}

FISServiceType

::= ENUMERATED

{

-- Automatic Terminal Information Service (ATIS) Request

atis		(0)

...

}

-- ------------------------------------------------------------------------------

-- ATIS messages (1rst level)

-- ------------------------------------------------------------------------------

ATISAccept

::= CHOICE

{

accept		ATISReport,

positiveAcknowledgement		NULL

}

ATISInformation

::= CHOICE

{

arrivalATIS		ArrivalATIS,

departureATIS		DepartureATIS,

combinedATIS		CombinedATIS,

arrivalAndDepartureATIS		ArrivalAndDepartureATIS

}

ATISReject

::= CHOICE

{

updateFISContractNotSupported	ATISReport,

otherReasons	ATISRejectReason

}

ATISRejectReason

::=	ENUMERATED

{

canNotComply	(0),

fISServiceUnavailable	(1),

errorInRequest	(2),

undefined	(3)

...

}

ATISReport

::= SEQUENCE

{

airportId			Airport,

aTISInformation			ATISInformation

}

ATISRequest

::= SEQUENCE

{

airportID	Airport,

arrivalDepartureIndicator	ArrivalDepartureIndicator

DEFAULT arrival

}

-- ------------------------------------------------------------------------

-- ATIS fields

-- ------------------------------------------------------------------------

Airport

::= PrintableString (SIZE(4))

AltimeterSetting

::= SEQUENCE

{

qNH               PressureMeasure,

qFE               SEQUENCE (SIZE(1..36)) OF RunwayQFE OPTIONAL

}

AltitudeFlightLevel

::= INTEGER (30..600)

-- units = flight level (100 feets), range (30..600), resolution = 1 level

AltitudeFlightLevelMetric

::= INTEGER (100..2000)

-- units = flight level (10 meters), range (100..2000), resolution = 1 level

Approach

::= ENUMERATED

{

ils						(0),

localizer						(1),

ndb						(2),

vor						(3),

vordme						(4),

nonprecisiongps					(5),

precisiongps					(6),

dmearc						(7),

precisionapproachradar				(8),

asr						(9),

visual						(10),

rnav						(11),

chartedvisualcvap				(12),

lda						(13),

fms						(14),

loran						(15),

mls						(16),

ilsdme						(17)

}

ApproachType

::= CHOICE

{

approachType			Approach,

approachTypeOther			FreeText

}

ArrivalAndDepartureATIS

::= SEQUENCE

{

arrivalATIS		ArrivalATIS,

departureATIS		DepartureATIS

}

ArrivalDepartureIndicator

::= ENUMERATED

{

arrival 		(0),

departure 		(1),

arrivalAndDeparture 		(2)

}

ArrivalATIS

::= SEQUENCE

{

aTISCode		ATISCode,

aTISTimeofObservation		Time OPTIONAL,

arrivalRunwaysinUse		SEQUENCE (SIZE (1..36)) OF 				ArrivalRunway,

departureRunwaysinUse		SEQUENCE (SIZE (1..36)) OF 			           DepartureRunway OPTIONAL,

commonATISInfo		CommonATISInformation,

arrivalATISInfo		SpecificATISArrivalInfo

}

ArrivalRunway

::= Runway

ATISCode

::= PrintableString (SIZE(1))

BrakingAction

::= SEQUENCE

{

brakingActionFirstHalf		BrakingActionDescription,

brakingActionSecondHalf		BrakingActionDescription OPTIONAL

}

BrakingActionDescription

::= CHOICE

{

brakingActionQuality		BrakingActionQuality,

brakingActionQualityOther	FreeText

}

BrakingActionQuality

::= ENUMERATED

{

poor 		(0),

fair		(1),

good		(2),

nil		(3)

}

CloudAmount

::= ENUMERATED

{

scattered		(0),

broken		(1),

overcast		(2)

}

CloudHeight

::= CHOICE

{

cloudHeightMeters		INTEGER (0..2000),

-- Units = meters, range (0..20000), resolution = 10

cloudHeightFeet		INTEGER (0..2000)

-- Units = feet, range (0..60000), resolution = 30

}

CloudInformation

::= SEQUENCE

{

cloudAmount		CloudAmount,

cloudHeight		CloudHeight,

cloudType		CloudType

}

CloudType

::= ENUMERATED

{

cumulonimbus		(0),

toweringCumulus		(1)

}

ContractNumber

::= INTEGER (1..256)

ContractType

::= ENUMERATED

{

demandContract		(0),

updateContract		(1)

}

CombinedATIS

::= SEQUENCE

{

aTISCode		ATISCode,

aTISTimeofObservation		Time	OPTIONAL,

runwaysinUse		SEQUENCE (SIZE (1..36)) OF

RunwayType,

commonATISInfo		CommonATISInformation,

arrivalATISInfo		SpecificATISArrivalInfo

}

CommonATISInformation

::= SEQUENCE

{

surfaceWinds      		SurfaceWinds,

visibility		VisualVisibility,

cloudSkyCoverGroup		CloudSkyCoverGroup,

airTemperature		Temperature,

dewPointTemperature		Temperature,

altimeterSetting		AltimeterSetting,

presentWeather		FreeText,

significantMetInformation    		SignificantMetInfo,

holdingDelay		Time 	       	OPTIONAL,

specificATISInstructs           		FreeText	        	OPTIONAL,

otherOperationInfo              		FreeText 	        	OPTIONAL,

runvayVisibilityRange         		RVR 	        	OPTIONAL

}

Date

::= SEQUENCE

{

year					Year,

month					Month,

day					Day

}

DateTimeGroup

::= SEQUENCE

{

date					Date,

time					Time

}

Day

::= INTEGER (1..31)

-- unit = day, range (1..31), resolution = 1

DepartureATIS

::= SEQUENCE

{

aTISCode		ATISCode,

aTISTimeofObservation		Time OPTIONAL,

departureRunwaysinUse		SEQUENCE (SIZE (1..36)) OF

DepartureRunway,

arrivalRunwaysinUse		SEQUENCE (SIZE (1..36)) OF 			           ArrivalRunway  OPTIONAL,

commonATISInfo		CommonATISInformation

}

DepartureRunway

::= Runway

FreeText

::= IA5String (SIZE (1..2047))

Level

::= CHOICE

{

altitudeFlightLevel		AltitudeFlightLevel,

altitudeFlightLevelMetric		AltitudeFlightLevelMetric

}

MeasuredWindVariations

::= SEQUENCE

{

direction1	WindDirection OPTIONAL,

direction2	WindDirection OPTIONAL,

speedMin	WindSpeed OPTIONAL,

speedMax	WindSpeed OPTIONAL

}

Month

::= INTEGER (1..12)

-- unit = month, range (1..12), resolution = 1

PressureMeasure

::= CHOICE

{

hPa			INTEGER (7500..12500),

-- units = hPa, range (750.0..1250.0), resolution = 0.1

inches			INTEGER (2200..3200)

-- units = inches of Mercury, range (22.00..32.00), resolution= 0.01

}

Runway

::= SEQUENCE

{

runwayNumber		RunwayNumber,

runwayLetter		RunwayLetter,

runwaySurfaceConditions		FreeText	         	OPTIONAL,

brakingAction		BrakingAction    	OPTIONAL,

runwayArrestingSystem		FreeText		OPTIONAL

}

RunwayLetter

::= ENUMERATED

{

noparrallelrunway 		(0),

runwayleft 		(1),

runwaycenter		(2),

runwayright 		(3),

invalid 		(4)

}

RunwayNumber

::= INTEGER (1..36)

RunwayQFE

::= SEQUENCE

{

runway		Runway,

qFE		PressureMeasure

}

RunwayType

::= CHOICE

{

arrivalRunway			ArrivalRunway,

departureRunway			DepartureRunway,

combinedRunway			Runway

}

RunwayVisibility

::= SEQUENCE

{

runway		Runway,

touchdownRVR		Visibility,

midRVR		Visibility,

rolloutRVR		Visibility

}

RVR

::= SEQUENCE (SIZE (1..36)) OF RunwayVisibility

SignificantMetInfo

::= CHOICE

{

nosig					NULL,

significantMetPhenomena		SignificantMetPhenomena

}

SignificantMetPhenomena

::= SEQUENCE

{

approachAreaMet		FreeText	OPTIONAL,

takeoffAreaMet		FreeText	OPTIONAL,

climboutAreaMet		FreeText	OPTIONAL

}

SpecificATISArrivalInfo

::= SEQUENCE

{

approachType		ApproachType    OPTIONAL,

transitionLevel		Level 		      OPTIONAL,

trendTypeLandingForecast		FreeText             OPTIONAL

}

SignificantMetInfo

::= CHOICE

{

nosig					NULL,

significantMetPhenomena		SignificantMetPhenomena

}

SignificantMetPhenomena

::= SEQUENCE

{

approachAreaMet		FreeText	OPTIONAL,

takeoffAreaMet		FreeText	OPTIONAL,

climboutAreaMet		FreeText	OPTIONAL

}

SurfaceWinds

::= CHOICE

{

calmIndicator		NULL,

-- if wind speed <= 2 km/h or 1 kt

surfaceWinds		Winds

}

Time

::= SEQUENCE

{

timeHours       		TimeHours,

timeMinutes     		TimeMinutes,

timeSeconds		TimeSeconds OPTIONAL

}

TimeHours

::= INTEGER (0..23)

-- units = hours, range (0..23), resolution = 1

TimeMinutes

::= INTEGER (0..59)

-- units = minutes, range (0..59), resolution = 1

TimeSeconds

::= INTEGER (0..59)

-- units = seconds, range (0..59), resolution = 1

Temperature

::= CHOICE

{

temperatureC		INTEGER (-80..60),

-- units = degree Celcius, range (-80..60), resolution = 1

temperatureF		INTEGER (-105..150)

-- units = degree Fahrenheit, range (-105..150), resolution = 1

}

VerticalVisibility

::= CHOICE

{

visibilityDistance		Visibility,

noValueAvailable		NULL

}

Visibility

::= CHOICE

{

visibilityMeters		INTEGER (0..1500),

-- units = meters, range (0..1500), resolution = 1

visibilityFeet		INTEGER (0..6000)

-- units = feet, range (0..6000), resolution = 1

}

VisibilityDirection

::= ENUMERATED

{

north				(0),

northwest			(1),

west				(2),

southwest			(3),

south				(4),

southeast			(5),

east				(6),

northeast			(7)

}

VisibilityNautMiles

::= SEQUENCE

{

nautMiles			INTEGER (0..127),

-- units = nautical mile, range (0..127), resolution = 1

fractionsOfNautMiles	INTEGER (0..240)

-- units = 16ths of Nautical Mile, range (0..15)

}

VisibilityValue

::= CHOICE

{

meters	INTEGER (0..49),

-- units = meters, range (0..4900), resolution = 100

kms	INTEGER (5..10),

-- units = kms, range (5..10), resolution = 1

nautMiles	VisibilityNautMiles

}

VisualVisibility

::= SEQUENCE

{

value					VisibilityValue,

direction				VisibilityDirection OPTIONAL

}

Winds

::= SEQUENCE

{

windDirection		WindDirection,

variations		WindVariations  OPTIONAL,

windSpeed		WindSpeed,

gustSpeed		WindSpeed        OPTIONAL

}

WindDirection

::= INTEGER (1..360)

-- units = degree, range (1..360), resolution = 1

WindSpeed

::= CHOICE

{

windSpeedMeters		INTEGER (0..500),

-- units = kilometerperhour, range (0..500), resolution = 1

windSpeedKnots		INTEGER (0..200),

-- units = knots, range (0..200), resolution = 1

windSpeedMiles		INTEGER (0..200)

-- units = mileperhour, range (0..200), resolution = 1

}

WindVariations

::= CHOICE

{

measuredWindVariations		MeasuredWindVariations,

vrbIndicator		WindSpeed

}

Year

::= INTEGER (0..99)

-- unit = year, range (0..99), resolution = 1

END

# PROTOCOL DEFINITION

## Sequence Rules

Note 1.— The following figures define the valid sequences of primitives that is possible to invoke during the operation of the FIS application. They show the relationship in time between the service request and the resulting indication, and if  applicable, the subsequent response and the resulting confirmation.

### Only the sequence of primitives, except abort primitives, described below shall  ([45])  be permitted.

Note 1.— All abort primitives may interrupt and terminate any of the normal message sequences outline below.

### With the permissible exception of abort primitives, the FIS-air-ASE and the FIS-ground-ASE shall  ([46])  process primitives in the order in which they are received.

Note 1.— This ensures that the FIS-ASE  will, with the exception of aborts, guarantee message sequencing.

### FIS-demand-contract

Note 1.— The following sequence of messages, shown in  Figure 5-1, occurs when the FIS-demand-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the Demand Contrat is either accepted or rejected.

T

I

M

E

Figure 5-1: use of FIS-demand-contract with no dialogue existing with contract accept or contract reject

Note 2.— The following sequence of messages, shown in Figure 5-2, occurs when the FIS-demand-contract service is initiated by the FIS-air-user, at least one FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the FIS demand contract is either accepted or rejected.

Figure 5-2: use of FIS-demand-contract with dialogue existing with contract accept, or contract reject

Note 3.— The following sequence of messages, shown in Figure 5-3, occurs when the FIS-demand-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and a FIS report is sent by the FIS-ground-user after a positive acknowledgment.

Figure 5-3:: Use of FIS-demand-contract service  with no existing dialogue and with a positive ackowledgment

Note 4.— The following sequence of messages, shown in Figure 5-4, occurs when the FIS-demand-contract service is initiated by the FIS-air-user, at least one FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and a FIS report is sent by the FIS-ground-user after a positive acknowledgment.

Figure 5-4: Use of FIS-demand-contract with existing dialogue and positive acknowledgment

### FIS-update-contract

Note 1.— The following sequence of messages, shown in Figure 5-5, occurs when the FIS-update-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, the Update Contract is accepted. The cancellation is air-initiated.

(*) : timer set only for positive acknowledgment

Figure 5-5: Use of FIS-update-contract with no existing dialogue and with contract accept and with air-initiated cancellation.

Note 2.— The following sequence of messages, shown in  Figure 5-6, occurs when the FIS-update-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE,  the Update Contract is accepted. The cancellation is ground-initiated.

(*) : timer set only for positive acknowledgment

Figure 5-6 : Use of FIS-update-contract with no existing dialogue and with contract accept and with ground-initiated cancellation.

Note 3.— The following sequence of messages, shown in  Figure 5-7, occurs when the FIS-update-contract service is initiated by the FIS-air-user, at least one FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the Update Contract is accepted. The cancellation is ground initiated.

(*): timer set only for positive acknowledgment

Figure 5-7:  use of FIS-update-contract with existing dialogue and with contract accept and with ground-initiated cancellation.

Note 4.— The following sequence of messages, shown in  Figure 5-8, occurs when the FIS-update-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the Update Contrat is rejected.

Figure 5-8: Use of the FIS-update-contract with no existing dialogue and with contract reject.

Note 5.— The following sequence of messages, shown in Figure 5-9, occurs when the FIS-update-contract service is initiated by the FIS-air-user, at least one FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the update contract is rejected.

Figure 5-9: Use of FIS-update-contract with existing dialogue and with contract reject

### FIS-cancel-update-contract

Note 1.— The following sequence of messages, shown in Figure 5-10, occurs when the FIS-cancel-update-contract service is initiated at the same time by both FIS-air-user and FIS-ground-user. If the cancelled contract is the last FIS Contract in operation, the dialogue is released.

Figure 5-10: Use of FIS-cancel-update by both FIS-users without other FIS Contract in place

Note 2.— The following sequence of messages, shown in  Figure 5-11, occurs when the FIS-update-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE and the ground-initiated cancellation is requested during contract establishement phase.

Figure 5-11 : Use of FIS-update-contract with no existing dialogue and with ground-initiated cancellation during contract establishment phase.

### FIS-cancel-contracts

Note 1.— The following sequence of messages, shown in Figure 5-12, occurs when the FIS-cancel-contracts service is initiated by the FIS-air-user, and all FIS contracts in place are impacted by the cancellation (this is always the case for CNS/ATM-1 Package).

Figure 5-12: Use of FIS-cancel-contracts with all contracts in place cancelled

### FIS-user Abort

Note 1.— The following sequence of messages, shown in Figure 5-13, occurs when the FIS-user-abort service is initiated by the FIS-air-user.

Figure 5-13: User of FIS-user-abort (air-initiated)

Note 2.— The following sequence of messages, shown in Figure 5-14, occurs when the FIS-user-abort service is initiated by the FIS-ground-user.

Figure 5-14: Use of FIS-user-abort   (ground-initiated )

### FIS-provider-abort Service

Note 1.— The following sequence of messages, shown in Figure 5-15, occurs when the dialogue service provider aborts.

Figure 5-15: Use of FIS-provider-abort  service (dialogue service provider aborts)

### FIS-ASE abort

Note 1.— The following sequence of messages, shown in Figure 5-16, occurs when the FIS-air-ASE aborts.

Figure 5-16: Use of  FIS-provider-abort  service (FIS-air-ASE aborts)

Note 2.— The following sequence of messages, shown in Figure 5-17, occurs when the FIS-ground-ASE aborts.

Figure 5-17: Use of FIS-provider-abort  service (FIS-ground-ASE aborts)

## FIS Service Provider Timers

Note 1.— Table 5-1 lists the time constraints related to the FIS application.  Each time constraint requires a timer to be set in the FIS protocol machine.

### The FIS-ASEs shall  ([47])  measure the time between the initial event and the corresponding final event.

### If the maximum time is exceeded before the final event has occurred, the following shall  ([48])  occur:

#### For all timers but the inactivity timer, the FIS ASEs shall  ([49])  take appropriate action as defined in 5.4.1.1.

#### For the inactivity timer, the FIS-ASE shall  ([50])  stop the dialogue by using the D-END service.

#### The action shall  ([51])  be taken immediately after the maximum time has expired.

Note.— The t-inactivity timer is an airborne timer value set on configuration basis.

| FIS Service | Timer | Timer Value | Timer Start Event | Timer Stop Event |
| --- | --- | --- | --- | --- |
| FIS- demand- | t-DC-1 | 6 minutes | FIS-demand-contract request | FIS-demand-contract confirmation |
| contract | t-DC-2 | 9 minutes + 1 sec + 10 sec | FIS-demand-contract request | FIS-report indication (in case a FIS-demand-contract ind with result = positive ack has been received) |
| FIS- update- | t-UC-1 | 6 minutes | FIS-update-contract request | FIS-update-contract confirmation |
| contract | t-UC-2 | 9 minutes + 1 sec + 10 sec | FIS-update-contract request | FIS-report indication (in case a FIS-update- update-contract ind with result = positive ack has been received) |
| FIS- cancel- update | t-UC- 3 | 6 minutes | FIS-cancel-update-contract request | FIS-cancel-update-contract confirmation |
| FIS-cancel- contracts | t-CL-1 | 6 minutes | FIS-cancel-contracts request | FIS-cancel-contracts confirmation |
| General | t-LI-1 | 6 minutes | D-END request | D-END confirmation |

Table 5-1:  FIS Application Timers

## FIS-ASE Protocol Description

### Description

#### Functional Model

Note 1.— The FIS-ASE is functionally made of 6 modules, as shown in Figure 5-17:

- the High Interface module (HI module). This module interfaces with the ASE-user through the abstract service interface as defined in chapter 3.

- the FIS Demand Contract module (DC module) : one DC module manages a single FIS Demand Contract.

- the FIS Update Contract module (UC module) : one UC module manages a single FIS Update Contract,

- the FIS CanceL contracts module  (CL module) : the CL module processes the termination of all contracts of the same type (ie ATIS) still in operation,

- the FIS Abort module (AB module): the AB module handles aborts in case of unrecoverable error, and

- the Low Interface module (LI module). This module interfaces the Dialogue Service Provider on behalf of the DC, UC and AB modules. It performs the multiplexing of FIS Contracts on a single dialogue.

Note 2.— This functional architecture allows simplification of the description of the protocol handled by the FIS-ASE. It does not constrain the implementation architecture.

Figure 5-18 : Functional Architecture of the FIS-ASE

Note 1.— The following subsections describe the actions of the individual modules in both the air and ground systems. Appendix A contains state tables for the individual modules.

### In the following sections, if no actions are described for an FIS-service primitive in a particular state, then the invocation of that service primitive shall  ([52])  be prohibited in that state.

### In the following sections, if no actions are described for the arrival of an APDU when in a particular state, then exception handling procedures as described in section 5.2 shall  ([53])  apply.

### High Interface module

#### Upon receipt of a FIS-ASE service primitive, the HI module shall ([54])  :

- if the primitive is a FIS-demand-contract request or a FIS-update-contract request primitive, reject the primitive if the Contract Number parameter corresponds to an existing FIS Contract,

- if the primitive is not a FIS-demand-contract request or a FIS-update-contract request primitive, reject the primitive if the Contract Number parameter does not correspond to an existing FIS Contract,

- pass the primitive to the relevant DC, UC, CL or AB module, as shown in Table 5-2.

| Primitive Name | Module |
| --- | --- |
| FIS-demand-contract request | air DC |
| FIS-update-contract request | air UC |
| FIS-cancel-update-contract request | air UC or ground UC |
| FIS-user-abort request | AB |
| FIS-demand-contract response | ground DC |
| FIS-update-contract response | ground UC |
| FIS-cancel-contracts request | air CL |
| FIS-report request | ground UC or ground DC |

Table 5-2: Request and response primitive to module mapping

#### Upon receipt of a request to invoke a service primitive by one of the modules in the FIS-ASE as shown in figure 5-3, and if the FIS-user is active, the HI module shall  ([55])  do so.

| Primitive Name | module |
| --- | --- |
| FIS-demand-contract cnf | air DC |
| FIS-update-contract cnf | air UC |
| FIS-cancel-update-contract ind | air UC or ground UC |
| FIS-cancel-update-contract cnf | air UC or ground UC |
| FIS-report ind | air DC or air UC |
| FIS-user-abort ind | AB |
| FIS-provider-abort ind | AB |
| FIS-demand-contract ind | ground DC |
| FIS-update-contract ind | ground UC |
| FIS-cancel-all-contracts ind | ground CL |
| FIS-cancel-all-contracts cnf | air CL |

Table 5-3  Indication and confirmation primitives to module mapping

### Air FIS DC module

#### The air FIS DC module shall  ([56])  be in one of the following states:

- DC-A-IDLE,

- DC-A-PENDING, or

- DC-A-ACTIVE.

#### On initiation, the air FIS DC module shall  ([57])  be in the DC-A-IDLE state.

#### Upon receipt of a FIS-demand-contract request, the following shall  ([58])  apply:

##### If in the DC-A-IDLE  state, the  DC module shall ([59])  :

- start timers t-DC-1 and t-DC-2,

- create a FISDownlinkAPDU [FISRequest] APDU with a demand contract APDU-element based on the FIS ContractNumber and FIS Contract Details parameters,

- request the LI module to send the APDU to the FIS-ground-ASE identified by the received ICAO Facility Designator parameter, and with the Class Of Communication Service parameter if specified by the user or the abstract value ”null” if not specified by the user,  and

- enter the DC-A-PENDING state.

#### Upon receipt of a [FISAccept] APDU with an accept APDU-element, the following shall  ([60])  apply:

##### If in the DC-A-PENDING state, the DC module shall ([61])  :

- stop timers t-DC-1 and t-DC-2 timer ,

- if there is no other FIS contract in place, start t-inactivity timer,

- request the HI module to send a FIS-demand-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "accepted",

  - the FIS Information parameter containing the information which has been received as the FISReportData APDU-element in the APDU, and

- enter the DC-A-IDLE state.

#### Upon receipt of a [FISAccept] APDU containing a positive acknowledgment  messsage element, the following shall  ([62])  apply:

##### If in the DC-A-PENDING state, the DC module shall ([63])  :

- stop timer t-DC-1,

- request the HI module to send a FIS-demand-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "positive acknowledgement"

- enter the DC-A-ACTIVE state.

#### Upon receipt of a [FISReject] APDU, the following shall  ([64])  apply:

##### If in the DC-A-PENDING state, the module shall ([65])  :

- stop timers t-DC-1  and t-DC-2 ,

- if there is no other FIS contract in place, start t-inactivity timer,

- request the HI module to send a FIS-demand-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "rejected", and

  - the RejectReason parameter containing the information which has been received  as the ATISRejectReason APDU-element in the APDU.

- enter the DC-A-IDLE state.

#### Upon receipt of a [FISReport] APDU, the following shall  ([66])  apply:

##### If in the DC-A-ACTIVE state, the module shall ([67])  :

- stop timer t-DC-2 ,

- if there is no other FIS contract in place, start t-inactivity timer,

- request the HI module to send a FIS-report indication to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Information parameter, containing the information which has been received as the FISReportData APDU-element in the APDU, and

- enter the DC-A-IDLE state.

#### Upon receipt on a request from the AB module to stop operation, the following shall  ([68])  apply:

##### If not in the DC-A-IDLE state, the DC module shall ([69])  :

- stop any timers,

- enter the DC-A-IDLE state.

##### If in the DC-A-IDLE, the DC module shall  ([70])  ignore the request.

#### Upon expiration of the t-DC-1 timer, the following shall  ([71])  apply:

##### If in the DC-A-PENDING state, the DC module shall ([72])  :

- stop timer t-DC-2,

- request the AB module to abort with reason timer-expiration, and

- enter the DC-A-IDLE state.

#### Upon expiration of the t-DC-2 timer, the following shall  ([73])  apply:

##### In in the DC-A-ACTIVE state, the DC module shall ([74])  :

- request the AB module to abort with reason timer-expiration, and

- enter the DC-A-IDLE state.

### Ground FIS DC module

#### The ground FIS DC module shall  ([75])  be in one of the following states:

- DC-G-IDLE,

- DC-G-PENDING, or

- DC-G-ACTIVE.

#### On initiation, the ground FIS DC module shall  ([76])  be in the DC-G-IDLE state.

#### Upon receipt of a [FISRequest] APDU with a demand contract messsage element, the following shall  ([77])  apply:

##### If in the DC-G-IDLE state, the DC module shall ([78])  :

- request the HI module to send a FIS-demand-contract indication to the FIS-ground-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Contract Details parameter, containing the information which has been received as the [FISRequestData] APDU-element in the APDU, and

- enter the DC-G-PENDING state.

#### Upon receipt of a FIS-demand-contract response with the Result parameter containing the abstract value "accepted", the following shall  ([79])  apply:

##### If in the DC-G-PENDING state,  the DC module shall ([80])  :

- create a FISUplinkAPDU [FISAccept] APDU with an accept APDU-element based on the FIS Contract Number and FIS Information parameters, and

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the DC-G-IDLE state.

#### Upon receipt of a FIS-demand-contract response with the Result parameter containing the abstract value "positive acknowledgement", the following shall  ([81])  apply:

- create a FISUplinkAPDU [FISAccept] APDU with a positive acknowledgement APDU-element based on the FIS Contract Number parameter,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the DC-G-ACTIVE state.

#### Upon receipt of a FIS-demand-contract  response with the Result parameter containing the abstract value "rejected", the following shall  ([82])  apply:

- create a FISUplinkAPDU [FISReject] APDU with an other reason APDU-element based on FIS Contract Number  and the RejectReason parameters, and

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the DC-G-IDLE state.

#### Upon receipt of a FIS-report request, the following shall  ([83])  apply:

##### If in the DC-G-ACTIVE state, the DC module shall ([84])  :

- create a FISUplinkAPDU [FISReport] APDU based on the FISContract Number and the FIS Information parameters, and

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the DC-G-IDLE state.

#### Upon receipt on a request from the AB module to stop operation, the following shall  ([85])  apply:

##### If not in the DC-G-IDLE state, the DC module shall  ([86])  enter the DC-G-IDLE state.

##### If in the DC-G-IDLE state, the DC module shall  ([87])  ignore the request.

### Air FIS UC module

#### The air FIS UC module shall  ([88])  be in one of the following states:

- UC-A-IDLE,

- UC-A-PENDING,

- UC-A-ACTIVE, or

- UC-A-CANCEL

#### On initiation, the air FIS UC module shall  ([89])  be in the UC-A-IDLE state.

#### Upon receipt of a FIS-update-contract request, the following shall  ([90])  apply:

##### If in the UC-A-IDLE state, the UC module shall ([91])  :

- start timers t-UC-1 and t-UC-2,

- create a FISDownlinkAPDU [FISRequest] APDU with an update contract message lement based on the FIS Contract Number and FIS Contract Details parameters,

- request the LI module to send the APDU to the FIS-ground-ASE identified by the received ICAO Facility Designator parameter, with the Class Of Communication Service  parameter if specified by the user or the abstract value ”null” if not specified by the user, and

- enter the UC-A-PENDING state.

#### Upon receipt of a [FISAccept] APDU containing an accept  APDU-element, the following shall  ([92])  apply:

##### If in the UC-A-PENDING state, the UC module shall ([93])  :

- stop timers t-UC-1 and t-UC-2,

- request the HI module to send a FIS-update-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "accepted",

  - the FIS Information parameter, containing the information which has been received as the FISReportData APDU-element in the APDU, and

- enter the UC-A-ACTIVE state.

#### Upon receipt of a [FISAccept] APDU containing a positive acknowledgement APDU-element, the following shall  ([94])  apply:

##### If in the UC-A-PENDING state, the UC module shall ([95])  :

- stop timer t-UC-1,

- request the HI module to send a FIS-update-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "positive acknowledgement",

- enter the UC-A-ACTIVE state.

#### Upon receipt of a [FISReject] APDU, the following shall  ([96])  apply;

##### If in the UC-A-PENDING state, the UC module shall ([97])  :

- stop timers t-UC-1 and t-UC-2 ,

- if there is no other FIS contract in place, start t-inactivity timer,

- request the HI module to send a FIS-update-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "rejected", and

  - the RejectReason parameter containing the information which has been received  as the ATISRejectReason APDU-element in the APDU.

- enter the UC-A-IDLE state.

#### Upon receipt of a [FISReport] APDU, the following shall  ([98])  apply:

##### If in the UC-A-ACTIVE state, the UC module shall ([99])  :

- if timer t-UC-2 is set, stop timer t-UC-2 ,

- request the HI module to send a FIS-report indication to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Information parameter, containing the information which has been received as the FISReportData APDU-element in the APDU,

- remain in the UC-A-ACTIVE state.

##### If in the UC-A-CANCEL state, the UC module shall ([100])  :

- request the HI module to send a FIS-report indication to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Information parameter, containing the information which has been received as the FISReportData APDU-element in the APDU.

- remain in the UC-A-CANCEL state.

#### Upon receipt of a FIS-cancel-update-contract request, the following shall  ([101])  apply:

##### If in the UC-A-ACTIVE state, the UC module shall ([102])  :

- if timer t-UC-2 set, stop timer t-UC-2 ,

- create a FISDownlinkAPDU [FISCancelUpdateContract]  APDU based on the received FIS Contract Number parameter, and

- request the LI module to send the APDU to the FIS-ground-ASE,

- start the t-UC-3 timer, and

- enter the UC-A-CANCEL state.

##### If in the UC-A-PENDING state and a dialogue is fully established, the UC module shall ([103])  :

- if timer t-UC-2 set, stop timer t-UC-2 ,

- create a FISDownlinkAPDU [FISCancelUpdateContract]  APDU based on the received FIS Contract Number parameter, and

- request the LI module to send the APDU to the FIS-ground-ASE,

- start the t-UC-3 timer, and

- enter the UC-A-CANCEL state.

#### Upon receipt of a [FISCancelUpdateContract] APDU, the following shall  ([104])  apply:

##### If in the UC-A-ACTIVE state, the UC module shall ([105])  :

- if timer t-UC-2 is set, stop timer t-UC-2 ,

- request the HI module to send a FIS-cancel-update-contract indication to the FIS-air-user with the FIS ContractNumber  parameter as received in the APDU,

- create a FISDownlinkAPDU [FISCancelUpdateAccept] APDU based on the FISContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-ground-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-A-IDLE state.

##### If in the UC-A-PENDING state, the UC module shall ([106])  :

- stop timers t-UC-1 and t-UC-2 ,

- request the HI module to send a FIS-cancel-update-contract indication to the FIS-air-user with the FIS ContractNumber  parameter as received in the APDU,

- create a FISDownlinkAPDU [FISCancelUpdateAccept] APDU based on the FISContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-ground-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-A-IDLE state.

##### If in the UC-A-CANCEL state (ie collision of cancel-contract-update requests), the module shall ([107])  :

- create a FISDownlinkAPDU [FISCancelUpdateAccept] APDU based on the FISContractNumber as received in the APDU, and

- request the LI module to send the APDU to the FIS-ground-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- remain in the UC-A-CANCEL state.

#### Upon receipt of a [FISCancelUpdateAccept] APDU, the following shall  ([108])  apply:

##### If in the UC-A-CANCEL state, the module shall ([109])  :

- stop the t-UC-3 timer,

- request the HI module to send a FIS-cancel-update-contract confirmation to the FIS-air-user with the FIS Contract Number parameter as received in the APDU,and

- enter the UC-A-IDLE state.

#### Upon receipt on a request from the AB module to stop operation, the following shall  ([110])  apply:

##### If not in UC-A-IDLE state, the UC module shall ([111])  :

- stop any timers, and

- enter the UC-A-IDLE state.

##### If in UC-A-IDLE state, the UC module shall  ([112])  ignore the request.

#### Upon expiration of the t-UC-1 timer, the following shall  ([113])  occur:

##### If in UC-A-PENDING state, the UC module shall ([114])  :

- stop timer t-UC-2,

- request the AB module to abort with reason timer-expiration, and

- enter the UC-A-IDLE state.

#### Upon expiration of the t-UC-2 timer, the following shall  ([115])  occur:

##### If in UC-A-ACTIVE state, the UC module shall ([116])  :

- request the AB module to abort with reason timer-expiration, and

- enter the UC-A-IDLE state.

#### Upon expiration of the t-UC-3 timer, the following shall  ([117])  occur:

##### If in UC-A-CANCEL state, the UC module shall ([118])  :

- request the AB module to abort with reason timer-expiration, and

- enter the UC-A-IDLE state.

### Ground FIS UC module

#### The ground FIS UC module shall  ([119])  be in one of the following states:

- UC-G-IDLE,

- UC-G-PENDING,

- UC-G-ACTIVE, or

- UC-G-CANCEL.

#### On initiation, the ground FIS UC module shall  ([120])  be in the UC-G-IDLE state.

#### Upon receipt of a [FISRequest] APDU containing an update contract APDU-element, the following shall  ([121])  apply:

##### If in the UC-G-IDLE state, the UC module shall ([122])  :

- request the HI module to send a FIS-update-contract indication to the FIS-ground-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Contract Details parameter, containing the information which has been received as the FISRequestData APDU-element in the APDU, and

- enter the UC-G-PENDING state.

#### Upon receipt of a FIS-update-contract response with a Result parameter containing the abstract value "accepted", the following shall  ([123])  apply:

##### If in the UC-G-PENDING state,  the UC module shall ([124])  :

- create a FISUplinkAPDU [FISAccept] APDU with an accept  APDU-element based on the FIS contract Number and FIS Information parameters,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the UC-G-ACTIVE state.

#### Upon receipt of a FIS-update-contract response with a Result parameter containing the abstract value "positive acknowledgement", the following shall  ([125])  apply:

##### If in the UC-G-PENDING state,  the UC module shall ([126])  :

- create a FISUplinkAPDU [FISAccept] APDU with a positive acknowledgement  APDU-element based on the FIS Contract Number parameter,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the UC-G-ACTIVE state.

#### Upon receipt of a FIS-update-contract response with a Result parameter containing the abstract value "rejected", the following shall  ([127])  apply:

##### If in the UC-G-PENDING state,  the UC module shall ([128])  :

- create a FISUplinkAPDU [FISReject] APDU based on:

  - the FIScontractNumber and FISInformation parameters if the RejectReason parameter contains the abstract value "update contract function not supported", or

  - the FIScontractNumber and RejectReason parameters if the RejectReason parameter does not contain the abstract value "update contract function not supported",

- request the LI module to send the APDU to the FIS-air-ASE,

- enter the UC-G-IDLE state.

#### Upon receipt of a FIS-report request, the following shall  ([129])  apply:

##### If in the UC-G-ACTIVE state, the UC module shall ([130])  :

- create a FISUplinkAPDU [FISReport] APDU based on the FIS Contract Number  and FIS Information parameters,

- request the LI module to send the APDU to the FIS-air-ASE,

- remain in the UC-G-ACTIVE state.

#### Upon receipt of a FIS-cancel-update-contract request, the following shall  ([131])  apply:

##### If in the UC-G-ACTIVE state, the UC module shall ([132])  :

- create a FISUplinkAPDU [FISCancelUpdateContract] APDU based on the FISContractNumber parameter,

- start timer t-UC-3,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the UC-G-CANCEL state.

##### If in the UC-G-PENDING state, the UC module shall ([133])  :

- create a FISUplinkAPDU [FISCancelUpdateContract] APDU based on the FISContractNumber parameter,

- start timer t-UC-3,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the UC-G-CANCEL state.

#### Upon receipt of a [FISCancelUpdateContract] APDU, the following shall  ([134])  apply:

##### If in the UC-G-ACTIVE state, the UC module shall ([135])  :

- request the HI module to send a FIS-cancel-update-contract indication to the FIS-ground-user with the FIS ContractNumber as received in the APDU,

- create a FISUplinkAPDU [FISCancelContractAccept] APDU based on the FIS ContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-air-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-G-IDLE state.

##### If in the UC-G-PENDING state, the UC module shall ([136])  :

- request the HI module to send a FIS-cancel-update-contract indication to the FIS-ground-user with the FIS ContractNumber as received in the APDU,

- create a FISUplinkAPDU [FISCancelContractAccept] APDU based on the FIS ContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-air-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-G-IDLE state.

##### If in the UC-G-CANCEL state (ie collision of cancel-contract-update requests), the UC module shall ([137])  :

- create a FISUplinkAPDU [FISCancelContractAccept] APDU based on the FIS ContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-air-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-G-IDLE state.

#### Upon receipt of a [FISCancelContractAccept] APDU, the following shall  ([138])  apply:

##### If in the UC-G-CANCEL state, the UC module shall ([139])  :

- stop timer t-UC-3,

- request the HI module to send send a FIS-cancel-update-contract confirmation to the FIS-ground-user with the FIScontractNumber parameter as received in the APDU,

- start the t-inactivity timer, and

- enter the UC-G-IDLE state.

#### Upon receipt on a request from the AB module to stop operation, the following shall  ([140])  occur:

##### In not in UC-G-IDLE state, the UC module shall  ([141])  enter the UC-G-IDLE state.

##### If in UC-G-IDLE state, the UC module shall  ([142])  ignore the request.

#### Upon expiration of the t-UC-3 timer, the following shall  ([143])  occur:

##### If in UC-G-CANCEL state, the UC module shall ([144])  :

- request the AB module to abort with reason timer-expiration, and

- enter the UC-G-IDLE state.

### Ground and Air AB module

Note 1.— All statements in this section apply to both the FIS ground AB module and the FIS air AB module.

#### Upon receipt of a request to abort from the HI, LI, DC or UC modules, the AB module shall ([145])  :

- request DC and UC modules to stop operation,

- request the HI module to send a FIS-provider-abort indication primitive to the FIS-user with the reason parameter set to value supplied by the module requesting the abort,

- create a FISDownlinkAPDU or FISUplinkAPDU [FISAbort] APDU, with the FISProtocolErrorDiag element set to the value supplied by the module requesting the abort,

- request the LI module to send a D-ABORT request with the originator parameter set to the abstract value « provider ».

#### Upon receipt of a HI request to send a FIS-user-abort request, the AB module shall ([146])  :

- request DC and UC modules to stop operation, and

- request the LI module to send a D-ABORT request with the originator parameter set to the abstract value « user ».

#### Upon receipt of a LI request to deliver a D-ABORT indication, the AB module shall ([147])  :

- request DC and UC modules to stop operation,

- if the originator parameter contains the abstract value « user », request the HI module to send a FIS-user-abort indication primitive to the FIS-user.

- if the originator parameter contains the abstract value « provider », request the HI module to send a FIS-provider-abort indication primitive to the FIS-user with the FISProtocolErrorDiag element of the received APDU as AbortReason parameter.

#### Upon receipt of a LI request to deliver a D-P-ABORT indication, the AB module shall ([148])  :

- request DC and UC modules to stop operation,

- request the HI module to send a FIS-provider-abort indication to the FIS-user with the abstract value "communication system failure" as AbortReason parameter.

### Air Cancel (CL) module

#### Upon receipt of a HI request to send a FIS-cancel-contracts request, if there are DC and UC modules handling contracts of the types specified in the FIS Service Type parameter, the CL module shall ([149])  :

- request these DC and UC modules to stop operation for the types of contracts concerned by the cancellation as specified in the FIS Service Type parameter,

- create a FISDownlinkAPDU  [FISCancelAllContract] APDU based on the FIS Service Type parameter,

- request the LI module to send the APDU to the peer FIS-ASE, and

- start the t-CL-1 timer.

#### Upon receipt of [FISCancelContractsAccept] APDU, the CL module shall ([150])  :

- stop the t-CL-1 timer,

- request the HI module to send a FIS-cancell-contracts confirmation primitive to the FIS-air-user with the types of contracts concerned by the cancellation specified in the [FISCancelAllContractAccept] APDU as FISServiceType parameter,

- start the t-inactivity timer if there is no other FIS contract in place.

#### Upon expiration of the timer t-CL-1, the CL module shall  ([151])  request the AB module to abort with reason timer-expiration.

### Ground Cancel (CL) module

#### Upon receipt of [FISCancelAllContract] APDU, the CL module shall ([152])  :

- request DC and UC modules to stop operation for the types of contracts concerned by the cancellation as specified in the [FISCancelAllContract] APDU,

- request the HI module to send a FIS-cancell-contracts indication primitive to the FIS-user with the types of contracts concerned by the cancellation specified in the [FISCancelAllContract] APDU as FISServiceType parameter,

- create a FISDownlinkAPDU  [FISCancelAllContractAccept] APDU based on the FIS Service Type parameter,

- request the LI module to send the APDU to the peer FIS-ASE.

### Ground and Air Low Interface module

Note 1.— All statements in this section apply to both the FIS ground LI module and the FIS air LI module.

Note 2.— Table 5-4 specifies the type of the APDUs the LI module can send to the peer FIS-ASE.

| APDU | Type |
| --- | --- |
| FISRequest | initial |
| FISAccept | normal |
| FISReject | normal |
| FISReport | normal |
| FISCancelUpdateContract | normal |
| FISCancelUpdateAccept | normal |
| FISCancelContracts | normal |
| FISCancelContractsAccept | normal |

Table 5-4 : Types of the FIS-ASE APDUs

Note3.— Table 5-5 specifies the recipient of APDU received by the LI module.

| APDU Name | module |
| --- | --- |
| FISRequest | ground DC, ground UC |
| FISCancelUpdateContract | air UC, ground UC |
| FISCancelUpdateAccept | air UC, ground UC |
| FISAccept | air DC, air UC |
| FISReject | air DC, air UC |
| FISReport | air DC, air UC |
| FISCancellContracts | ground CL |
| FISCancellContractsAccept | air CL |

Table 5-5 :PDU to module mapping

#### The LI module shall  ([153])  be in one of the following states:

- LI-IDLE,

- LI-START-I (initiator of the D-START req),

- LI-START-R (receptor of the D-START ind),

- LI-DIALOGUE,

- LI-END-I (initiator of the D-END req), or

- LI-END-R (receptor of the D-END ind).

#### On initiation, the LI module shall  ([154])  be in the LI-IDLE state.

#### When requested by a DC or UC module to send an initial APDU to the peer ASE, the following shall  ([155])  apply:

##### if in the LI-IDLE state, the LI module shall ([156])  :

- invoke the D-START request primitive with the following parameters:

  - the ICAO Facility Designator provided by the DC or UC module as the CalledPeer Id parameter,

  - as components of the Quality Of Service parameter:

    - the Routing Class based on the Class of Communication Service as provided by the DC or UC module,

    - the Application Service Priority as defined in section 6.3.1,

    - the RER as defined in section 6.3.2.

  - the APDU as User Data parameter,

- enter the LI-START-I state.

##### if in the LI-DIALOGUE state, the LI module shall ([157])  :

- invoke the D-DATA request primitive with the APDU as User Data parameter, and

- remain in the LI-DIALOGUE state.

#### When requested by a DC or UC module to send a normal APDU to the peer ASE, the following shall  ([158])  apply:

##### if in the LI-DIALOGUE state, the LI module shall ([159])  :

- invoke the D-DATA request primitive with the APDU as User Data parameter,  and

- remain in the LI-DIALOGUE state.

##### if in the LI-START-R state, the LI module shall ([160])  :

- invoke the D-START response primitive with the following parameters:

  - the abstract value "accepted" as the Result parameter, and

  - the APDU as the User Data parameter,

- enter the LI-DIALOGUE state.

#### When requested by the AB module send a D-ABORT request, the following shall  ([161])  apply:

##### In not in the LI-IDLE state, the LI module shall ([162])  :

- invoke the D-ABORT request primitive with the APDU as UserData parameter and the value supplied by the AB module as originator parameter.

- enter the LI-IDLE state.

##### In in the LI-IDLE state, the LI module shall  ([163])  ignore the request.

#### Upon receipt of a D-START indication with the UserData parameter containing a valid APDU, the following shall  ([164])  apply :

##### If in the LI-IDLE state, the LI module shall ([165])  :

- pass the APDU to the relevant DC or UC module, as defined in Table 5-5, and

- enter the LI-START-R state.

#### Upon receipt of a positive D-START confirmation with a UserData  parameter containing a valid APDU, the following shall  ([166])  apply:

##### If in the LI-START-I state, the LI module shall ([167])  :

- pass the APDU to the relevant DC or UC module, as defined in Table 5-5,

- enter the LI-DIALOGUE state.

#### Upon receipt of a negative D-START confirmation (ie Result  contains the abstract value "rejected(transient)") with the RejectSource containing the abstract value "DS provider", the following shall  ([168])  apply:

##### If in the LI-START-I state, the LI module shall ([169])  :

- request the HI module to send a FIS-provider-abort indication to the FIS-user with the abstract value "cannot establish contact with the peer" as AbortReason parameter.

- enter the LI-DIALOGUE state.

#### Upon receipt of a negative D-START confirmation (ie Result  contains the abstract value "rejected(transient) or "rejected(permanent)") with the RejectSource containing the abstract value "DS user", the following shall  ([170])  apply:

##### If in the LI-START-I state, the LI module shall ([171])  :

- request the HI module to send a FIS-provider-abort indication to the FIS-user with the abstract value "contact refused by the the peer" as AbortReason parameter.

- enter the LI-DIALOGUE state.

#### Upon receipt of a D-DATA indication with a UserData parameter containing a valid APDU, the following shall  ([172])  apply:

##### In the LI-DIALOGUE, LI-END-I or LI-END-R state, the LI module shall ([173])

- identify the module from the Contract Number parameter of the APDU,

- pass the APDU to the relevant module, as defined Table 5-5, and

- remain in the same state.

#### Upon receipt of a D-END indication, the following shall  ([174])  apply:

##### If in state LI-DIALOGUE, the LI module shall ([175])  :

- invoke the D-END response primitive with the abstract value "accepted" as Result  parameter, and

- enter the LI-IDLE state.

#### Upon receipt of a D-END confirmation with a Result parameter containing the abstract value "accepted", the following shall  ([176])  apply:

##### If in the LI-END-I state, the LI module shall  ([177])  enter the LI-IDLE state.

#### Upon receipt of a D-ABORT indication with the UserData  parameter containing a valid APDU, the LI module shall ([178])  :

- forward the primitive to the AB Module, and

- enter the LI-IDLE state.

#### Upon receipt of a D-P-ABORT indication, the LI module shall ([179])  :

- forward the primitive to the AB module, and

- enter the LI-IDLE state.

#### Upon receipt of an indication that the t-inactivity timer has expired, the following shall  ([180])  apply:

##### In in LI-DIALOGUE state, the LI module shall ([181])  :

- start the timer t-LI-1,

- invoke the D-END request primitive, and

- enters the  LI-END-I state

## Exception Handling

### Timer Expiration

#### When any of the timers in any of the modules reaches the maximum time stated in section 5.2, the module shall  ([182])  request the AB module to abort with reason "timer expiration".

### Unrecoverable internal error

#### When any module has an unrecoverable system error, the module shall  ([183])  request the AB module to abort with reason "unrecoverable internal error".

### Protocol Error

#### When the LI module detects that a dialogue service indication or confirmation does not contain an APDU, the module shall  ([184])  request the AB module to abort with the reason "protocol error".

### Sequence Error

#### When a PDU is passed to a module for which there are no instruction in section 5.3 (ie the PDU has arrived out of sequence), the module shall  ([185])  request the AB module to abort with the reason "sequence error".

### Decoding Error

#### When the LI module detects a error when decoding an APDU, the LI module shall  ([186])  request the AB module to abort with the reason "decoding error".

### Invalid FIS Contract Number

#### When the LI module detects that the FIS Contract Number in the decoded APDU is invalid (ie the ContractNumber in a non  initial APDU is not already in use or the ContractNumber in an initial APDU is already in use), the UC or DC module shall  ([187])  request the AB module to abort with the reason "invalid contract number".

## State Tables

Note 1.— This section defines the state tables for modules of  both FIS-air-ASE and FIS-ground-ASE.

Note 2.— If the state tables shown in this section conflict with the textual statements made elsewhere in chapter 5, the textual statements take precedence.

### Conventions for state tables

Note 1.— The following notation conventions apply for the following state tables :

- Module States are identified as follows : <function name>-<location>-<state>.

- Events between a source module and a recipient module are identified as shown in the following figure:

Figure 5-1: Functional Model for ASE state tables

### High Interface (HI) Module

Note 1.— As the HI Module provides the ASE-users with a straightforward pass-through service to the DC, UC, CL and AB Modules, state tables related to the HI Modules are not described.

Note 2.— FIS-xxx request and response primitives are mapped onto DC-xxx, UC-xxx, CL-xxx or AB-xxx request and response primitives; DC-xxx, UC-xxx, CL-xxx or AB-xxx indication and confirmation primitives are mapped onto FIS-xxx indication and confirmation primitives, with the following exception:

- When AB-user-abort indication or AB-provider-abort indication are received by the HI Module from the AB Module, the corresponding FIS-user-abort indication or FIS-provider-abort indication is only generated to the FIS-user when the FIS-user is active.

### Air Demand Contract (DC) Module

Note. — Table 5-6 reflects the possible states of the air DC Module.

| State Þ | DC-A-IDLE | DC-A-PENDING | DC-A-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Primitive Requests and Responses from HI module |  |  |  |
| DC-demand-contract req | LI-Send-Initial [FISRequest-DemandContract] APDU Start t-DC-1, t-DC-2 èDC-A-PENDING | not permitted | not permitted |
| FIS APDUs from LI module |  |  |  |
| [FISReport] APDU | cannot occur | AB-abort ("sequence error") èDC-A-IDLE | stop t-DC-2 if last FIScontract start t-INACTIVITY DC-report ind èDC-A-IDLE |
| [FISAccept-accept] APDU | cannot occur | stop t-DC-1, t-DC-2 if last FISContract start t-INACTIVITY DC-demand-contract cnf èDC-A-IDLE | AB-abort ("sequence error") èDC-A-IDLE |
| [FISAccept-positive acknowledgment ] APDU | cannot occur | stop t-DC-1 DC-demand-contract cnf èDC-A-ACTIVE | AB-abort ("sequence error") èDC-A-IDLE |
| [FISReject] APDU | cannot occur | stop t-DC-1, t-DC-2 if last FIScontract start t-INACTIVITY DC-demand-contract cnf èDC-A-IDLE | AB-abort ("sequence error") èDC-A-IDLE |
| Requests from AB module |  |  |  |
| DC-stop-operation | do nothing ! | stop t-DC-1, t-DC-2 èDC-A-IDLE | stop t-DC-2 èDC-A-IDLE |

| State Þ | DC-A-IDLE | DC-A-PENDING | DC-A-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Timer expiration |  |  |  |
| t-DC-2 | cannot occur | cannot occur | AB-abort ("timer expiration") èDC-A-IDLE |
| t-DC-1 | cannot occur | Stop t-DC-2 AB-abort ("timer expiration") èDC-A-IDLE | cannot occur |

Table 5-6:  FIS-demand-contract air based state table

### Ground Demand Contract (DC) Module

Note. — Table 5-7 reflects the possible states of the ground DC Module.

| State Þ | DC-G-IDLE | DC-G-PENDING | DC-G-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Primitive Requests and Responses from HI Module |  |  |  |
| DC-demand-contract rsp (accepted) | not permitted | LI-Send-Normal [FISAccept-accept] APDU èDC-G-IDLE | not permitted |
| DC-demand-contract rsp (rejected) | not permitted | LI-Send-Normal [[FISReject] APDU èDC-G-IDLE | not permitted |
| DC-demand-contract rsp (positive acknowledgment) | not permitted | LI-Send-Normal [FISAccept-positive-ack] APDU èDC-G-ACTIVE | not permitted |
| DC-report req | not permitted | not permitted | LI-Send-Normal [FISReport] APDU èDC-G-IDLE |
| APDU from LI Module |  |  |  |
| [FISRequest-DemandContract] APDU | DC-demand-contract ind èDC-G-PENDING | AB-abort ("sequence error") èDC-G-IDLE | AB-abort ("sequence error") èDC-G-IDLE |
| APDU from AB Module |  |  |  |
| DC-stop-operation | do nothing ! | èDC-G-IDLE | èDC-G-IDLE |

Table 5-7 :  FIS-demand-contract Ground based state table

### Air Update Contract (UC) Module

Note. — Table 5-8 reflects the possible states of the air UC Module.

| State Þ | UC-A-IDLE | UC-A-PENDING | UC-A-ACTIVE | UC-A-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| Primitive Requests and Responses from HI |  |  |  |  |
| UC-update-contract req | LI-Send-Initial [FIS-Request-UpdateContract] APDU Start t-UC-1, t-UC-2 èUC-A-PENDING | not permitted | not permitted | not permitted |
| UC-cancel-update-contract req | not permitted | if dialogue not established not permitted else stop t-UC-1, t-UC-2 LI-Send-Normal [FISCancelUpdate Contract] APDU start t-UC-3 èUC-A-CANCEL | if set, stop t-UC-2 LI-Send-Normal [FISCancelUpdateContract] APDU start t-UC-3 èUC-A-CANCEL | not permitted |
| FIS uplink PDUs from LI |  |  |  |  |
| [FISAccept-accept] APDU | cannot occur | stop t-UC-1, t-UC-2 UC-update-contract cnf èUC-A-ACTIVE | AB-abort ("sequence error") èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE |
| [FISAccept-positive acknowledgment] APDU | cannot occur | stop t-UC-1 UC-update-contract cnf èUC-A-ACTIVE | AB-abort ("sequence error") èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE |

| State Þ | UC-A-IDLE | UC-A-PENDING | UC-A-ACTIVE | UC-A-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| [FISReject] APDU | cannot occur | stop t-UC-1, t-UC-2 if last FIScontract start t-INACTIVITY UC-update-contract cnf èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE |
| [FISReport] APDU | cannot occur | AB-abort ("sequence error") èUC-A-IDLE | If set, stop t-UC-2 UC-report ind èUC-A-ACTIVE | UC-report ind èUC-A-CANCEL |
| [FISCancelUpdateContract] APDU | cannot occur | UC-cancel-update-contract ind LI-Send-Normal [FISCancelUpdateAccept] APDU if last FIS Contract start t-inactivity èUC-A-IDLE | If set, stop t-UC-2 UC-cancel-update-contract ind LI-Send-Normal [FISCancelUpdateAccept] APDU if last FIS Contract start t-inactivity èUC-A-IDLE | /* collision */ LI-Send-Normal [FISCancelUpdateAccept] APDU if last FIS Contract start t-inactivity èUC-A-CANCEL |
| [FISCancelUpdateAccept] APDU | cannot occur | AB-abort ("sequence error") èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE | stop t-UC-3 UC-cancel-update-contract cnf èUC-A-IDLE |
| Request from AB module |  |  |  |  |
| UC-stop-operation | do nothing ! | stop t-UC-1, t-UC-2 èUC-A-IDLE | stop t-UC-2 èUC-A-IDLE | èUC-A-IDLE |
| Timer Expiration |  |  |  |  |
| t-UC-1 | cannot occur | Stop t-UC-2 AB-abort ("timer expiration") èUC-A-IDLE | cannot occur | cannot occur |
| t-UC-2 | cannot occur | cannot occur | AB-abort ("timer expiration") èUC-A-IDLE | cannot occur |
| t-UC-3 | cannot occur | cannot occur | cannot occur | AB-abort ("timer expiration") èUC-A-IDLE |

Table 5-8:  FIS-update-contract air based state table

### Ground Update Contract (UC) Module

Note. — Table 5-9 reflects the possible states of the ground UC Module.

| State Þ | UC-G-IDLE | UC-G-PENDING | UC-G-ACTIVE | UC-G-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| Primitive Requests and Responses from HI |  |  |  |  |
| UC-update-contract rsp (accepted) | not permitted | LI-Send-Normal [FISAccept-accept] APDU èUC-G-ACTIVE | not permitted | not permitted |
| UC-update-contract rsp (rejected) | not permitted | LI-Send-Normal [FISReject] APDU èUC-G-IDLE | not permitted | not permitted |
| UC-update-contract rsp (processing) | not permitted | LI-Send-Normal [FISAccept-processing] APDU èUC-G-ACTIVE | not permitted | not permitted |
| UC-report req | not permitted | not permitted | LI-Send-Normal [FISReport] APDU èUC-G-ACTIVE | not permitted |
| UC-cancel-update-contract req | not permitted | LI-Send-Normal [FISCancelUpdateContract] APDU start t-UC-3 èUC-G-CANCEL | LI-Send-Normal [FISCancelUpdateContract] APDU start t-UC-3 èUC-G-CANCEL | not permitted |

| State Þ | UC-G-IDLE | UC-G-PENDING | UC-G-ACTIVE | UC-G-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| FIS downlink PDUs from LI |  |  |  |  |
| [FISRequest-UpdateContract] APDU | UC-update-contract ind èUC-G-PENDING | AB-abort ("protocol error") è UC-G-IDLE | AB-abort ("protocol error") è UC-G-IDLE | AB-abort ("protocol error") è UC-G-IDLE |
| [FISCancelUpdateContract] APDU | cannot occur | AB-abort ("protocol error") è UC-A-IDLE | UC-cancel-update-contract ind LI-Send-Normal [FISCancelContractAccept] APDU è UC-A-IDLE | /* collision */ LI-Send-Normal [FISCancelContractAccept] APDU è UC-A-IDLE |
| [FISCancelUpdateAccept] APDU | cannot occur | AB-abort ("protocol error") è UC-G-IDLE | AB-abort ("protocol error") è UC-G-IDLE | stop t-UC-3 FIS-cancel-update-contract cnf èUC-G-IDLE |
| Request from AB module |  |  |  |  |
| UC-stop-operation | do nothing ! | è UC-G-IDLE | è UC-G-IDLE | è UC-G-IDLE |
| Timer Expiration |  |  |  |  |
| t-UC-3 | cannot occur | cannot occur | cannot occur | AB-abort ("timer expiration") è UC-G-IDLE |

Table 5-9 :  FIS-update-contract ground based state table

### Air and Ground Low Interface (LI) Module

Note. — Table 5-10 reflects the possible states of the air and ground LI Modules.

| State Þ | LI-IDLE | LI-START-I | LI-DIALOGUE | LI-START-R | LI-END-I | LI-END-R |
| --- | --- | --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |  |  |
| Primitive Requests and Responses from DC, UC or AB Module |  |  |  |  |  |  |
| LI-Send-Initial (APDU) | D-START req è LI-START- I | not permitted | D-DATA req | not permitted /* only air-initiated */ | not permitted | cannot occur |
| LI-Send-Normal (APDU) | not permitted | not permitted | D-DATA req | D-START rsp è LI-DIALOGUE | not permitted | not permitted |
| Primitive Requests from AB Module |  |  |  |  |  |  |
| AB_D-ABORT req | /* do nothing */ | D-ABORT req è LI-IDLE | D-ABORT req è LI-IDLE | D-ABORT req è LI-IDLE | D-ABORT req è LI-IDLE | D-ABORT req è LI-IDLE |
| Timer Expiration |  |  |  |  |  |  |
| t-LI-1 | cannot occur | cannot occur | cannot occur | cannot occur | AB-abort ("timer expiration") èLI-IDLE | cannot occur |
| t-INACTIVITY | cannot occur | cannot occur | start t-LI-1 D-END req è LI-END-I | cannot occur | cannot occur | cannot occur |

| State Þ | LI-IDLE | LI-START-I | LI-DIALOGUE | LI-START-R | LI-END-I | LI-END-R |
| --- | --- | --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |  |  |
| Dialogue Service primitives from DSP |  |  |  |  |  |  |
| D-START ind | Pass APDU è LI-START- R | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START cnf positive (result=accepted) | cannot occur | Pass APDU è LI-DIALOGUE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START cnf negative (result=rejectedtransient & source=DS provider) | cannot occur | AB-dialogue-abort ("cannot establish contact with the peer") è LI-IDLE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START cnf negative (result=rejectedtransient & source=DS user) | cannot occur | AB-dialogue-abort ("contact refused by the peer") è LI-IDLE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-DATA ind | cannot occur | cannot occur | Pass APDU | cannot occur | Pass APDU | Pass APDU |
| D-END ind | cannot occur | cannot occur | if APDU Pass APDU to UCmod è LI-END-R else D-END rsp positive è LI-DLE | cannot occur | cannot occur | cannot occur |
| D-END cnf positive | cannot occur | cannot occur | cannot occur | cannot occur | è LI-IDLE | cannot occur |
| D-ABORT ind | cannot occur | AB_D-ABORT ind è LI-IDLE | AB_D-ABORT ind è LI-IDLE | AB_D-ABORT ind è LI-IDLE | AB_D-ABORT ind è LI-IDLE | AB_D-ABORT ind è LI-IDLE |
| D-P-ABORT ind | cannot occur | AB_D-P-ABORT ind è LI-DLE | AB_D-P-ABORT ind è LI-DLE | AB_D-P-ABORT ind è LI-DLE | AB_D-P-ABORT ind è LI-DLE | AB_D-P-ABORT ind è LI-DLE |

Table 5-10 : LI Module state table

### Air and Ground Abort (AB) Modules

Note. — Table 5-11 reflects the possible states of the air and ground AB Modules.

| AB module state Þ | AB-IDLE |
| --- | --- |
| Event ß |  |
| Primitive Requests from DC and UC Module |  |
| AB-abort (reason) | UC-stop-operation (all FIS services) DC-stop-operation AB-provider-abort ind (reason) AB-D-ABORT req (« provider »,[ FISAbort] APDU) |
| Primitive Indications and APDUs from LI Module |  |
| AB_D-P-ABORT ind (reason) | UC-stop-operation (all FIS services) DC-stop-operation (all FIS services) AB-provider-abort ind (reason) |
| AB_D-ABORT ind (« provider »,APDU) | UC-stop-operation (all FIS services) DC-stop-operation (all FIS services) AB-provider-abort ind (APDU.AbortReason) |
| AB_D-ABORT ind (« user ») | UC-stop-operation (all FIS services) DC-stop-operation (all FIS services) AB-user-abort ind () |
| Primitive Requests from HI Module |  |
| AB-user-abort req | UC-stop-operation (all FIS services) DC-stop-operation (all FIS services) AB-D-ABORT req (« user ») |

Table 5-11 : AB Module state table

### Air Cancel (CL) Module

Note. — Table 5-12 reflects the possible states of the air CL Module.

| CL module state Þ | CL-IDLE |
| --- | --- |
| Event ß |  |
| Primitive Request from the HI module |  |
| CL-cancel-contracts req (types of FIS service) | if UC or DC exist with these types of FIS service UC-stop-operation (type of FIS service) DC-stop-operation (type of FIS service) LI-Send-Normal [FISCancelContracts] APDU start t-CL-1 |
| APDUs from LI Module |  |
| [FISCancelContractsAccept] APDU | stop t-CL-1 CL-cancel-contracts cnf if last FIS Contract, start t-inactivity |
| Timer Expiration |  |
| t-CL-1 | AB-abort (« timer expiration ») |

Table 5-12 : Air CL Module state table

### Ground Cancel (CL) Module

Note. — Table 5-13 reflects the possible states of the ground CL Module.

| CL module state Þ | CL-IDLE |
| --- | --- |
| Event ß |  |
| APDUs from LI Module |  |
| [FISCancelContracts] APDU | UC-stop-operation (type of FIS service) DC-stop-operation (type of FIS service) CL-cancel-contracts ind LI-Send-Normal [FISCancelContractsAccept] APDU |

Table 5-13 : Ground CL Module state table

# COMMUNICATION REQUIREMENTS

## Encoding Rules

### The FIS application shall  ([188])  use PER as defined in reference [2], using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in chapter 4, or a functionally equivalent means which provides the same result.

## Dialogue Service Requirements

### Primitives Requirements

#### Where dialogue service primitives, that is D-START, D-END, D-ABORT, D-P-ABORT, and D-DATA are described as being invoked in chapter 5, the FIS-ground-ASE and the FIS-air-ASE shall  ([189])  exhibit external behaviour consistent with the dialogue service, as described in reference [4], having been implemented and its primitives invoked.

### Quality Of Service Requirements

#### The application service priority for FIS shall  ([190])  have the abstract value of "Aeronautical Information Service messages".

#### The RER Quality Of Service parameter of the D-START request shall  ([191])  be set to the abstract value of  "low".

Note 1.— The application service RER  for FIS is required to be 10-6  or better.

Note 2.— The RER takes into account non-delivery for messages over a given dialogue.

#### The FIS-ASE shall  ([192])  map the FIS-service Class of Communication Service abstract value to the Routing Class abstract value part of the D-START Quality Of Service parameter as presented in Table 6-1.

| Class of Communication Abstract Value | Routing Class Abstract Value |
| --- | --- |
| A | Traffic only follows Class A ATSC route(s) |
| B | Traffic only follows Class B ATSC route(s) |
| C | Traffic only follows Class C ATSC route(s) |
| D | Traffic only follows Class D ATSC route(s) |
| E | Traffic only follows Class E ATSC route(s) |
| F | Traffic only follows Class F ATSC route(s) |
| G | Traffic only follows Class G ATSC route(s) |
| H | Traffic only follows Class H ATSC route(s) |

Table 6-1:  Mapping between Class of Communication and Routing Class Abstract Values

Note — ATSC values are defined in [5].

## FIS AE Control Function Requirements

### The AE-Qualifier for the CNS/ATM-1 FIS ATIS Application shall  ([193])  be the IA5 character string "ATI".

# FIS USER REQUIREMENTS

## Introduction

Note 1.—  Requirements imposed on FIS-user interfacing with the FIS-ASEs are presented in this chapter.

Note 2.— Where reference is made to the ”FIS-user” this implies both the FIS-air-user and the FIS-ground-user.

## General

### General Parameter Requirements

#### When a FIS-air-user invokes one of the FIS-ASE primitives described in chapter 3, it shall  ([194])  provide a user-defined reference for the FIS contract.

#### When a FIS-ground-user invokes one of the FIS-ASE primitives described in chapter 3, it shall  ([195])  provide a user-defined reference for the FIS contract.

When a FIS-air-user invokes FIS-demand-contract request or FIS-update-contract request and has no preference for the class of communication service to be used, or there is an existing FIS contract in place and the FIS-air-user requires the existing class of communication service parameter, it shall  ([196])  not provide class of communication service parameter.

### Timing Requirements

#### Upon receipt of a FIS-ASE service indication that requires a response, the FIS-ground-user shall  ([197])  invoke the FIS-ASE service response within 1 seconds.

### Error Handling

#### If the FIS-air-user has an unrecoverable system error, and if it is able to do so, it shall ([198])  :

- for each FIS-ground-user that the aircraft has contracts with and which is effected by the error, invoke the FIS-user-abort request, and

- inform its end user.

#### If the FIS-ground-user has an unrecoverable system error, and if it is able to do so, it shall ([199])  :

- for each aircraft that the FIS-ground-user has contracts with and which is effected by the error, invoke the FIS-user-abort request, and

- inform its end user.

#### If the FIS-ground-user receives a FIS-provider-abort indication or FIS-user-abort indication, it shall ([200])  :

- cease operation of all contracts with that air system, and

- inform its end user.

#### If the FIS-air-user receives a FIS-provider-abort indication or a FIS-user-abort indication, it shall ([201])  :

- cease operation of all contracts with that FIS-ground-user, and

- the FIS-air-user shall  ([202])  inform the end user.

### Miscellaneous Air FIS System Requirements

#### The FIS-air-user shall  ([203])  be able to retrieve the naming information required to establish a contract with a ground FIS system.

#### The FIS-air- user shall  ([204])  respond to indications and confirmations from the FIS-air-ASE in the order in which they are invoked.

### Miscellaneous Ground FIS System Requirements

#### The FIS-ground-user shall  ([205])  be capable of detecting that the content of the FIS request is not valid or that the requested FIS information is not available.

#### The FIS-ground-user shall  ([206])  be capable of detecting that it can not continue to provide updates of the requested FIS information.

#### The FIS-ground-user shall  ([207])  be able to support multiple concurrent FIS Contracts  with a given aircraft.

#### The FIS-ground user shall  ([208])  respond to indications and confirmations from the FIS-ground-ASE in the order in which they are invoked.

### ATIS service requirements

#### If the arrival ATIS or departure ATIS is requested, and if the FIS-ground-user only provides combined ATIS, the FIS-ground-user shall  ([209])  send combined ATIS with a 'combined' indication.

#### If both arrival and departure ATIS are requested, the FIS-ground-user shall ([210])  :

- send a combined ATIS with a 'combined' indication, if the combined ATIS is available, or

- send an arrival ATIS and a departure ATIS if both information are available but not the combined ATIS, or

- send the available ATIS with an indication that the other is not available, if only one is available.

#### The ATIS fields shall  ([211])  be presented to the pilot in the following order:

- Airport identification,

- Departure or arrival indicator,

- ATIS Code

- Time of Observation, if present,

- Approach Type, if present,

- Runways In Use,

- RunwayArresting System, if present,

- RunwaySurfaceConditions, if present,

- RunwayBraking Action, if present,

- Holding Delay, if present,

- Transition Level, if present,

- Other Operational Information, if present,

- Surface Wind,

- Visibility,

- RVR, if present,

- Present Weather,

- Cloud Sky and Cover,

- Air Temperature,

- Dew Point Temperature,

- Altimeter Setting,

- Significant Met Information,

- Trend Type of Landing Forecast, if present, and

- Specific ATIS Instruction, if present.

## Establishment and operation of a FIS Demand Contract

### If the FIS-air-user requires to establish a demand contract with the FIS-ground-user it shall  ([212])  invoke FIS-demand-contract request primitive.

### Only the FIS-air-user shall  ([213])  be capable of initiating the FIS-demand-contract service.

### The FIS-air-user shall  ([214])  use a value for the ContractNumber parameter of the FIS-demand-contract request primitive which is not already affected to another FIS contract.

### The FIS-ground-user shall  ([215])  use the value received in the ContractNumber parameter of the FIS-demand-contract indication primitive for the ContractNumber parameter of any ground-initiated request or response primitive (ie FIS-demand-contract response and FIS-report request).

### The same type of FIS Information (ie ”ATIS” for CNS/ATM-1 package) shall  ([216])  be identified by the FIS-air-user and FIS-ground-user when invoking the FIS-demand-contract request and response primitives.

### Upon receipt of a FIS-demand-contract indication primitive, if the FIS-ground-user is able to accept the contract in full, the following shall  ([217])  apply:

#### If the FIS-ground-user is not able to send the FIS report within 1 seconds, it shall  ([218])  invoke the FIS-demand-contract response primitive with the result parameter set to abstract value ”positive acknowledgment” within 1 seconds and invoke the FIS-report request primitive within 10 seconds.

#### If the FIS-ground-user is able to send the FIS report within 1 seconds, it shall  ([219])  invoke the FIS-demand-contract response primitive with the result parameter set to the abstract value ”accepted” and the FIS report in the FIS Information parameter.

### Upon receipt of a FIS-demand-contract indication primitive, if the FIS-ground-user cannot comply with the demand contract request, it shall  ([220])  reject the contract by invoking an FIS-demand-contract response primitive with the following parameters:

- the abstract value "rejected"  as result  parameter, and

- one of the following  abstract values "cannot comply", "FIS service not available", "error detected in the FIS request" or "undefined" , as reject reason  parameter.

### The FIS-air-user shall  ([221])  be prevent from invoking a FIS-demand-contract request primitive before the very first request (either a FIS-demand-contract or a FIS-contract request) has been confirmed.

### The FIS-air-user shall  ([222])  be allowed to reuse the value of a FIS Contract Number used in a FIS-demand-contract request primitive for a new FIS Contract:

- upon receipt of a FIS-demand-contract confirmation primitive with a Result parameter containing the abstract value ”rejected” or ”accepted”,

- upon receipt of a FIS-report indication, a FIS-user-abort indication or a FIS-provider-abort indication primitive,

- after invocation of a FIS-user-abort request primitive.

## Establishment and operation of a FIS Update Contract

### If the FIS-air-user requires to establish an update contract with the FIS-ground-user it shall  ([223])  invoke the FIS-update-contract request primitive.

### Only the FIS-air-user shall  ([224])  be capable of initiating the FIS-update-contact service.

### The FIS-air-user shall  ([225])  use a value for the ContractNumber parameter of the FIS-update-contract request primitive which is not already affected to another FIS contract.

### The FIS-ground-user shall  ([226])  use the value received in the ContractNumber parameter of the FIS-update-contract indication primitive for the ContractNumber parameter of any ground-initiated request or response primitive (ie FIS-update-contract response and FIS-report request).

### The same type of FIS Information (ie ”ATIS” for CNS/ATM-1 package) shall  ([227])  be identified by the FIS-air-user and FIS-ground-user when invoking the FIS-demand-contract request and response primitives.

### Upon receipt of a FIS-update-contract indication primitive, if the FIS-ground-user is able to accept the contract in full the following events shall  ([228])  occur:

#### If the FIS-ground-user is not able to send the first FIS-report within 1 seconds,  then it shall  ([229])  invoke the FIS-update-contract response primitive with the result parameter set to the abstract value ”positive acknowledgment”  within 1 seconds and invoke the FIS-report request primitive within 10 seconds.

#### If the FIS-ground-user is able to send the FIS-report within 1 seconds, then it shall  ([230])  invoke the FIS-update-contract response primitive with the result parameter set to the abstract value ”accepted” and the FIS report in the FIS Information parameter.

#### If the FIS-ground-user does not support the update-contract function but the FIS report is available, it shall  ([231])  invoke the FIS-update-contract response primitive with the result parameter set to the abstract value ”rejected”, the reject reason parameter set to the abstract value ”update contract function not supported by the FIS-ground-user” and the FIS report in the FIS Information parameter.

### Upon receipt of a FIS-update-contract indication primitive, if the FIS-ground-user cannot comply with the update contract request, it shall  ([232])  reject the contract by invoking an FIS-update-contract response with the following parameters:

- the abstract value "rejected"  as result  parameter, and

- one of the following  abstract values "cannot comply", "FIS service not available", "error detected in the FIS request" or "undefined" , as reject reason  parameter.

### The FIS-air-user shall  ([233])  be prevent from invoking a FIS-update-contract request primitive before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### The FIS-ground-user shall  ([234])  invoke the FIS-report request primitive each time the FIS information requested in the FIS-update-contract indication primitive is updated until such time as the contract is canceled or aborted.

### The FIS-air-user shall  ([235])  be allowed to reuse the value of a FIS Contract Number used in a FIS-update-contract request primitive for a new FIS contract:

- upon receipt of a FIS-update-contract confirm primitive with a Result parameter containing the abstract value ”rejected”,

- upon receipt of a FIS-user-abort indication or a FIS-provider-abort indication primitive,

- upon receipt of a FIS-cancel-update confirm or a FIS-cancel-contracts confirm primitive,

- upon receipt of a FIS-cancel-update indication primitive, or

- after invocation of a FIS-user-abort request.

## Air and ground cancellation of a single update contract

Note 1. — There is no provision for the cancellation of a single demand contract.

### If a FIS-user requires to cancel an active update contract, it shall  ([236])  invoke the FIS-cancel-update-contract request primitive with contractnumber parameter set to contract number identifying an active update contract.

### Both FIS-air-user and FIS-ground-user shall  ([237])  be capable of initiating the FIS-cancel-update-contract service.

### The FIS-air-user shall  ([238])  be prevent from invoking a FIS-cancel-update-contract request primitive before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### Upon receipt of a FIS-cancel-update-contract indication, the FIS-user shall  ([239])  cancel the update contract identified by the ContractNumber parameter.

## Air cancellation of FIS contracts of the same type

### If a FIS-air-user requires to cancel all the FIS contracts of a given type or of several types (only ”ATIS” in CNS/ATM-1 Package), it shall  ([240])  invoke the FIS-cancel-contracts request primitive with the FISServiceType parameter identifying the contract type(s).

### Only the FIS-air-user shall  ([241])  be capable of initiating the FIS-cancel-contracts service.

### The FIS-air-user shall  ([242])  be prevent from invoking a FIS-cancel-contracts request primitive before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### The FIS-air-user shall  ([243])  invoke the FIS-cancel-contracts request primitive for each ground system  with which it has FIS contracts.

### When the FIS-ground-user receives a FIS-cancel-contracts indication, it shall  ([244])  cancel any demand contract and update contract for the type(s) identified in the indication (eg ATIS) in force with that aircraft.

## FIS-user-abort

### If an active FIS-user requires to stop brutally any activity of the current FIS-AE with the peer FIS-AE with a possible loss of FIS messages currently in transit in the communication system, it shall  ([245])  invoke the FIS-user-abort request primitive.

Note 1.— This FIS-user is no more considered by the FIS AE as an active user.

### Both FIS-air-user and FIS-ground-user shall  ([246])  be capable of initiating the FIS-user-abort service.