# COMMUNICATION REQUIREMENTS

## Encoding Rules

### The FIS application shall  ([188])  use PER as defined in reference [2], using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in chapter 4, or a functionally equivalent means which provides the same result.

## Dialogue Service Requirements

### Primitives Requirements

#### Where dialogue service primitives, that is D-START, D-END, D-ABORT, D-P-ABORT, and D-DATA are described as being invoked in chapter 5, the FIS-ground-ASE and the FIS-air-ASE shall  ([189])  exhibit external behaviour consistent with the dialogue service, as described in reference [4], having been implemented and its primitives invoked.

### Quality Of Service Requirements

#### The application service priority for FIS shall  ([190])  have the abstract value of "Aeronautical Information Service messages".

#### The RER Quality Of Service parameter of the D-START request shall  ([191])  be set to the abstract value of  "low".

Note 1.— The application service RER  for FIS is required to be 10-6  or better.

Note 2.— The RER takes into account non-delivery for messages over a given dialogue.

#### The FIS-ASE shall  ([192])  map the FIS-service Class of Communication Service abstract value to the Routing Class abstract value part of the D-START Quality Of Service parameter as presented in Table 6-1.

| Class of Communication Abstract Value | Routing Class Abstract Value |
| --- | --- |
| A | Traffic only follows Class A ATSC route(s) |
| B | Traffic only follows Class B ATSC route(s) |
| C | Traffic only follows Class C ATSC route(s) |
| D | Traffic only follows Class D ATSC route(s) |
| E | Traffic only follows Class E ATSC route(s) |
| F | Traffic only follows Class F ATSC route(s) |
| G | Traffic only follows Class G ATSC route(s) |
| H | Traffic only follows Class H ATSC route(s) |

Table 6-1:  Mapping between Class of Communication and Routing Class Abstract Values

Note — ATSC values are defined in [5].

## FIS AE Control Function Requirements

### The AE-Qualifier for the CNS/ATM-1 FIS ATIS Application shall  ([193])  be the IA5 character string "ATI".