# ACSE SPECIFICATION

Note:  The ACSE requirements in this chapter are described in many cases by means of completed protocol implementation conformance statement (PICS) proforma tables.  In such tables, the “Ref.” column contains a reference to the relevant section in the ACSE PICS proforma [30].

## Classification of requirements

Note: In each table, the "ISO Status" column reflects the level of support required for conformance to the base standard and the "ATN Support" column reflects the level of support required by these SARPs.  The specification of levels of support uses the classification defined in subclause 4.2 of ISO/IEC ISP 11188-1.

## Protocol details

### The specification of the ACSE protocol supported shall  ([163])  be as defined in Table 6.1.

Table 6.1:  Identification of ACSE Protocol Specification

| Identification of Protocol Specification | ATN Support | Comments |
| --- | --- | --- |
| ITU-T Rec. X.227 (1995) ¦ ISO/IEC 8650-1:1995 | M | M (see note) |

Note:  this is the second edition of the ACSE protocol specification.

## Protocol versions

### The version of the ACSE protocol supported shall  ([164])  be as specified in Table 6.2.

Table 6.2:  Identification of ACSE Protocol version

| Ref. |  | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.4.2/1 | Version 1 | o.1 | M | A-V1 |
| A.A.4.2/2 | Version 2 | o.1 |  |  |

o.1:  The ISO PICS requires support of the implementation of only one version of the protocol to be described.

## Supported roles

### Association establishment

#### The supported roles for Association Establishment shall  ([165])  be as specified in Table 6.3.

Table 6.3:  ACSE Roles for Association Establishment

| Ref. | Capability | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.6.1/1 | Association initiator | o.2 | C1 | A-CON_initiator |
| A.A.6.1/2 | Association responder | o.2 | C1 | A-CON_responder |

o.2:  The ISO standard requires a conforming implementation to support at least one of the roles.

C1:  either one or both of these roles shall  ([166])  be supported.

### Normal Release procedure

#### The supported roles for the Normal Release procedure shall  ([167])  be as specified in Table 6.4.

Table 6.4:  ACSE Roles for Normal Release

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.6.2/1 | Initiator | o | C1 | A-REL_initiator |
| A.A.6.2/2 | Responder | o | C1 | A-REL_responder |

C1:  either one or both of these roles shall  ([168])  be supported.

### Abnormal Release procedure

#### The supported roles for the Abnormal Release procedure shall  ([169])  be as specified in Table 6.5.

Table 6.5:  ACSE Roles for Abnormal Release

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.6.3/1 | Initiator | m | M |  |
| A.A.6.3/2 | Responder | m | M |  |

## Protocol mechanisms

### The ACSE protocol mechanisms specified as “M” in Table 6.6 shall  ([170])  be supported.

Table 6.6:  ACSE Protocol Mechanisms Supported

| Ref. | Protocol Mechanism | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.7/1 | Normal mode | o.4 | M |  |
| A.A.7/2 | X.410-1984 mode | o.4 | X |  |
| A.A.7/2 | Rules for extensibility | m | M |  |
| A.A.7/4 | Supports operation of Session version 2 | o | M | S-O-SESS-V2 |

o.4:  The ISO standard requires either Normal mode or X.410-1984 mode or both to be supported.

### Extensibility and Encoding

#### For the purposes of this document, the abstract syntax module defined in clause 9 of the ACSE protocol specification shall  ([171])  be augmented with the ASN.1 extensibility notation, as specified in [26].

#### The system shall  ([172])  support that encoding which results from applying the ASN.1 packed encoding rules (basic, unaligned variant), as specified in [6], to the abstract syntax module specified above.

#### Packed encoding (basic, unaligned) shall  ([173])  be used for encoding all ACSE PCI for interchange.

## ACSE Functional units

### The ACSE functional units selected shall  ([174])  be as specified in Table 6.7.

Table 6.7:  Selection of ACSE Functional Units

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.8/1 | Normal mode | m | M |  |
| A.A.8/2 | Authentication | o | C1 | A-FU(AU) |

C1:  If the Dialogue Service user requires the use of the Security Requirements parameter of the D-START primitives, then M, else X.

## Supported APDUs

### The ACSE Protocol data units specified as “M” in Table 6.8 shall  ([175])  be supported.

Table 6.8  Supported ACSE Protocol Data Units

| Ref. | APDU |  | Sender |  |  |  | Receiver |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ISO Status |  | ATN Support |  | ISO Status |  | ATN Support |  |  |  |
| A.A.9/1 |  | AARQ |  | c1 |  | M |  | c2 |  | M |  |  |
| A.A.9/2 |  | AARE |  | c2 |  | M |  | c1 |  | M |  |  |
| A.A.9/3 |  | RLRQ |  | c3 |  | M |  | c4 |  | M |  |  |
| A.A.9/4 |  | RLRE |  | c4 |  | M |  | c3 |  | M |  |  |
| A.A.9/5 |  | ABRT |  | c5 |  | M |  | c5 |  | M |  |  |

c1:  if [A-CON_initiator] then m else N/A

c2:  if [A-CON_responder] then m else N/A

c3:  if [A-REL_requestor] then m else N/A

c4:  if [A-REL_acceptor] then m else N/A

c5:  if [S-O-SESS-V2] then m else N/A

### Supported APDU parameters

#### A-associate-request (AARQ)

##### The parameters in the AARQ APDU shall  ([176])  be supported as specified in Table 6.9.

Table 6.9:  Supported AARQ Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.1/1 | Protocol Version | c6 | X | c2 | M |
| A.A.10.1/2 | Application Context Name | c1 | M | c2 | M |
| A.A.10.1/3 | Calling AP title | c6 | O | c2 | M |
| A.A.10.1/4 | Calling AE qualifier | c6 | O | c2 | M |
| A.A.10.1/5 | Calling AP invocation-identifier | c6 | O | c2 | M |
| A.A.10.1/6 | Calling AE invocation-identifier | c6 | O | c2 | M |
| A.A.10.1/7 | Called AP title | c6 | X | c2 | M |
| A.A.10.1/8 | Called AE qualifier | c6 | X | c2 | M |
| A.A.10.1/9 | Called AP invocation-identifier | c6 | X | c2 | X |
| A.A.10.1/10 | Called AE invocation-identifier | c6 | X | c2 | X |
| A.A.10.1/11 | ACSE-requirements | c8 | C10 | c9 | C11 |
| A.A.10.1/12 | Authentication-mechanism name | c8 | C10 | c9 | C11 |
| A.A.10.1/13 | Authentication-value | c8 | C10 | c9 | C11 |
| A.A.10.1/14 | Implementation information | c6 | X | c7 | X |
| A.A.10.1/15 | User information | c6 | M | c7 | M |

C10:  This parameter shall  ([177])  be supported only if the connection initiator role (A-CON_initiator) and the Authentication functional unit (A-FU(AU)) are supported.

C11:  This parameter shall  ([178])  be supported only if the connection responder role (A-CON_responder) and the Authentication functional unit (A-FU(AU)) are supported.

c1:  if [A-CON_initiator] then m else N/A

c2:  if [A-CON_responder] then m else N/A

c6:  if [A-CON_initiator] then o else N/A

c7:  if [A-CON_responder] then o else N/A

c8:  if [A-CON_initiator and A-FU(AU)] then m else N/A

c9:  if [A-CON_responder and A-FU(AU)] then m else N/A

#### A-associate-response (AARE)

##### The parameters in the AARE APDU shall  ([179])  be supported as specified in Table 6.10.

Table 6.10:  Supported AARE Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.2/1 | Protocol Version | c7 | X | c1 | M |
| A.A.10.2/2 | Application Context Name | c2 | M | c1 | M |
| A.A.10.2/3 | Responding AP title | c7 | X | c1 | M |
| A.A.10.2/4 | Responding AE qualifier | c7 | X | c1 | M |
| A.A.10.2/5 | Responding AP invocation-identifier | c7 | X | c1 | M |
| A.A.10.2/6 | Responding AE invocation-identifier | c7 | X | c1 | M |
| A.A.10.2/7 | Result | c2 | M | c1 | M |
| A.A.10.2/8 | Result source - diagnostic | c10 | M | c11 | M |
| A.A.10.2/9 | ACSE-requirements | c9 | C12 | c8 | C13 |
| A.A.10.2/10 | Authentication-mechanism name | c9 | C12 | c8 | C13 |
| A.A.10.2/11 | Authentication-value | c9 | C12 | c8 | C13 |
| A.A.10.2/12 | Implementation information | c7 | X | c6 | X |
| A.A.10.2/13 | User information | c7 | M | c6 | M |

C13:  This parameter shall  ([180])  be supported only if the connection initiator role (A-CON_initiator) and the Authentication functional unit (A-FU(AU)) are supported.

C12:  This parameter shall  ([181])  be supported only if the connection responder role (A-CON_responder) and the Authentication functional unit (A-FU(AU)) are supported.

c1:  if [A-CON_initiator] then m else N/A

c2:  if [A-CON_responder] then m else N/A

c6:  if [A-CON_initiator] then o else N/A

c7:  if [A-CON_responder] then o else N/A

c8:  if [A-CON_initiator and A-FU(AU)] then m else N/A

c9:  if [A-CON_responder and A-FU(AU)] then m else N/A

c10:  if [A-CON_responder] then (if [A-FU(AU)] then m (with a value range of 0 to 14) else m (with a value range of 0 to 10) else N/A

c11:  if [A-CON_initiator] then (if [A-FU(AU)] then m (with a value range of 0 to 14) else m (with a value range of 0 to 10) else N/A

#### A-release-request (RLRQ)

##### The parameters in the RLRQ APDU shall  ([182])  be supported as specified in Table 6.11.

Table 6.11:  Supported RLRQ Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.3/1 | Reason | c12 | M | c4 | M |
| A.A.10.3/2 | User information | c12 | M | c4 | M |

c4:  if [A-REL_acceptor] then m else N/A

c12:  if [A-REL_requestor] then o else N/A

#### A-release-response (RLRE)

##### The parameters in the RLRE APDU shall  ([183])  be supported as specified in Table 6.12.

Table 6.12  Supported RLRE Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.4/1 | Reason | c13 | M | c3 | M |
| A.A.10.4/2 | User information | c13 | M | c3 | M |

c3:  if [A-REL_requestor] then m else N/A

c13:  if [A-REL_acceptor] then o else N/A

#### A-abort (ABRT)

##### The parameters in the ABRT APDU shall  ([184])  be supported as specified in Table 6.13.

Table 6.13:  Supported ABRT Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.5/1 | Abort source | m | M | m | M |
| A.A.10.5/2 | Diagnostic | c14 | M | c14 | M |
| A.A.10.5/3 | User information | o | M | m | M |

c14:  if [A-FU(AU)] then m else N/A

### Supported parameter forms

#### AE title name form

##### The Application Entity Title parameter shall  ([185])  be supported in the forms specified in Table 6.14.

Table 6.14:  AE Title Name Form

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Syntax form | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.11.1/1 | Form 1 (Directory name) | o.5 | X | m | O |
| A.A.11.1/2 | Form 2 (Object identifier and integer) | o.5 | M | m | M |

o.5:  The ISO standard requires a conforming implementation to support at least one of the forms.

#### Authentication value form

##### The Authentication value parameter shall  ([186])  be supported in the forms specified in Table 6.15.

Table 6.15: Authentication Value Form

Prerequisite:  A-FU(AU)

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Authentication value form | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.11.2/1 | GraphicString | o.6 | C1 | c14 | M |
| A.A.11.2/2 | BIT STRING | o.6 | C1 | c14 | M |
| A.A.11.3/3 | EXTERNAL | o.6 | C1 | c14 | M |
| A.A.11.4/4 | Other | o.6 | C1 | c14 | M |

C1:  If the authentication functional unit is supported, at least one of these forms shall  ([187])  be implemented.

o.6:  The ISO standard requires a conforming implementation to support at least one of the forms.

c14:  if [A-FU(AU)] then m else N/A