# SESSION LAYER REQUIREMENTS

Note:  The session layer requirements in this chapter are described in many cases by means of completed protocol implementation conformance statement (PICS) proforma tables.  In such tables, the “Ref.” column contains a reference to the relevant section in the session layer PICS proforma [28].

## Protocol versions implemented

### The Session protocol versions specified as “M” in Table 4.1 shall  ([125])  be supported.

Table 4.1 : Session Protocol Versions Supported

| Ref. | Version | ISO Status | ATN Support |
| --- | --- | --- | --- |
| S.A.3/1 | Version 1 | o.1 | - |
| S.A.3/2 | Version 2 | o.1 | M |

o.1:  the ISO PICS requires that the implementation of one, and only one, version of the protocol is described.

## Session Functional units

### The Session functional units selected shall  ([126])  be as specified in Table 4.2.

Table 4.2 : Selection of Session functional units

| Ref. | Functional unit | ISO Status | ATN Support |
| --- | --- | --- | --- |
| S.A.6.1/1 | Kernel | m | M |
| S.A.6.1/2 | Negotiated release | o | X |
| S.A.6.1/3 | Half Duplex | o.2 | X |
| S.A.6.1/4 | Duplex | o.2 | M |
| S.A.6.1/5 | Expedited Data | o | X |
| S.A.6.1/6 | Typed Data | o | X |
| S.A.6.1/7 | Capability Data Exchange | c1 | X |
| S.A.6.1/8 | Minor Synchronize | o | X |
| S.A.6.1/9 | Symmetric Synchronize | o | X |
| S.A.6.1/10 | Data Separation | c2 | X |
| S.A.6.1/11 | Major Synchronize | o | X |
| S.A.6.1/12 | Resynchronise | o | X |
| S.A.6.1/13 | Exceptions | c3 | X |
| S.A.6.1/14 | Activity Management | o | X |
| Note 1 | No-orderly release (NOR) | o | M |

Note 1:  Functional unit added by efficiency enhancement X.225 Addendum 1

o.2:	The ISO standard requires at least one of the functional units Duplex and Half Duplex to be implemented.

c1:	if [S-FU(ACT)] then o else N/A

c2:	if [S-FU(SY) or S-FU(SS)] then o else N/A

c3:	if [S-FU(HD)] then o else N/A

## Protocol mechanisms

### The Session protocol mechanisms specified as “M” in Table 4.3 shall  ([127])  be supported.

Table 4.3 : Session Protocol Mechanisms Supported

| Ref. | Mechanism | ISO Status | ATN Support | Associated mnemonic |
| --- | --- | --- | --- | --- |
| S.A.6.2/1 | Use of transport expedited data (Extended control Quality of Service) | c4 | X | S-EXP_T |
| S.A.6.2/2 | Reuse of transport connection | o | O | S-REUSE_T |
| S.A.6.2/3 | Basic concatenation | m | M |  |
| S.A.6.2/4 | Extended concatenation (sending) | o | X |  |
| S.A.6.2/5 | Extended concatenation (receiving) | o | X | S-XCONC_RCV |
| S.A.6.2/6 | Segmenting (sending) | o | X | S-SEG_SDR |
| S.A.6.2/7 | Segmenting (receiving) | o | X | S-SEG_RCV |
| S.A.6.2/8 | Max. size of SS-user-data (S-CONNECT) > 512 | o | O | S-MAXSIZE_512 |
| S.A.6.2/9 | Max. size of SS-user-data (S-CONNECT) > 10240 | o | O | S-MAXSIZE_10240 |
| S.A.6.2/10 | Max. size of SS-user-data (S-ABORT) >9 | o | X | S-MAXSIZE_9 |
| Note 1 | Null-encoding protocol option | - | M |  |
| Note 1 | Short-connect protocol option | - | M |  |

Note 1:  Protocol options added by efficiency enhancement X.225 Addendum 1.

c4:	if [S-FU(EX)] then m else o

### The session protocol shall  ([128])  implement the efficiency enhancements in ITU-T Recommendation X.225 Addendum 1 as specified in this Chapter, together with all approved amendments and defect report resolutions.

### If the null encoding protocol option is offered by the initiating SPM, the responding SPM shall  ([129])  select only the kernel, full-duplex and no-orderly release functional units for use on this connection.

### SPDUs associated with the Short-connect protocol option (i.e. SCN, SAC and SRF) shall  ([130])  be transferred as User-data on the Transport layer T-CONNECT primitives, where possible.

Note: this is only possible if the complete SPDUs, including any User-data, meet any size restrictions of the T-CONNECT User-data.

## Supported Roles

### Session Connection

#### The roles for Session Connection specified as “M” in Table 4.4 shall  ([131])  be supported.

Table 4.4 : Session Connection Roles Supported

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| S.A.7.1.1.1/1 | Connection initiator | o.3 | M | S-CON_initiator |
| S.A.7.1.1.1/2 | Connection responder | o.3 | M | S-CON_responder |

o.3:  the ISO standard requires a conforming implementation to support at least one of these roles as required by the implementation.

### Orderly release

#### The roles for Session Orderly Release specified as “M” in Table 4.5 shall  ([132])  be supported.

Table 4.5 : Session Orderly Release Roles Supported

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| S.A.7.1.1.2/1 | Requestor | o.4 | - (Note 1) | S-REL_requestor |
| S.A.7.1.1.2/2 | Acceptor | o.4 | - (Note 1) | S-REL_acceptor |

o.4:  the ISO standard requires a conforming implementation to support at least one of these roles.

Note 1:  Not applicable, as the No Orderly Release functional unit is selected.

### Normal Data Transfer

#### The roles for Session Normal Data Transfer specified as “M” in Table 4.6 shall  ([133])  be supported.

Table 4.6 : Session Normal Data Transfer Roles Supported

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| S.A.7.1.1.3/1 | Requestor | o.5 | M | S-DATA_requestor |
| S.A.7.1.1.3/2 | Acceptor | o.5 | M | S-DATA_acceptor |

o.5:  the ISO standard requires a conforming implementation to support at least one of these roles.

## Support for the SPDUs associated with the Kernel functional unit

### The SPDUs specified as “M” in Table 4.7 shall  ([134])  be supported.

Table 4.7 : Supported Session Protocol Data Units

|  |  | Sender |  | Receiver |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Ref. | SPDU | ISO Status | ATN Support | ISO Status | ATN Support | Mnemonics |
| S.A.7.1.2/1 | Connect (CN) | c5 | Note 4 | c6 | - (Note 4) |  |
| S.A.7.1.2/2 | Overflow Accept (OA) | c7 | Note 4 | c8 | - (Note 4) | S-OA_SDR / S-OA_RCV |
| S.A.7.1.2/3 | Connect Data Overflow (CDO) | c9 | Note 4 | c10 | - (Note 4) | S-CDO_SDR / S-CDO_RCV |
| S.A.7.1.2/4 | Accept (AC) | c6 | Note 4 | c5 | - (Note 4) |  |
| S.A.7.1.2/5 | Refuse (RF) | c6 | Note 4 | c5 | - (Note 4) |  |
| S.A.7.1.2/6 | Finish (FN) | c11 | Note 2 | c12 | - (Note 2) |  |
| S.A.7.1.2/7 | Disconnect (DN) | c12 | Note 2 | c11 | - (Note 2) |  |
| S.A.7.1.2/8 | Abort | m | Note 3 | m | - (Note 3) |  |
| S.A.7.1.2/9 | Abort Accept (AA) | o | Note 3 | m | - (Note 3) |  |
| S.A.7.1.2/10 | Data Transfer (DT) | c13 | Note 3 | c14 | - (Note 3) |  |
| S.A.7.1.2/11 | Prepare (PR) | c15 | X | c15 | X | S-PR_SDR / S-PR_RCV |
| Note 1 | Short Connect (SCN) | - | M | - | M |  |
| Note 1 | Short Accept (SAC) | - | M | - | M |  |
| Note 1 | Short Refuse (SRF) | - | M | - | M |  |
| Note 1 | Null | - | M | - | M |  |

Note 1:  PDUs defined in efficiency enhancement X.225 Addendum 1

Note 2:  Not applicable, as the no-orderly-release functional unit is selected.

Note 3:  Not applicable, as the null-encoding protocol option is selected.

Note 4:  Not applicable, as the short-connect protocol option is selected.

c5: if [S-CON_initiator] then m else N/A

c6: if [S-CON_responder] then m else N/A

c7: if [S-V1 or (NOT S-CON_responder) ] then N/A  else if [S-MAXSIZE_10240] then m else o

c8: if [NOT S-V1 and S-CON_responder and S-MAXSIZE_10240] then m else N/A

c9: if [S-V1 or (NOT S-CON_initiator) ] then N/A  else if [S-MAXSIZE_10240] then m else o

c10: if [NOT S-V1 and S-CON_initiator and S-MAXSIZE_10240] then m else N/A

c11: if [S-REL_requestor] then m else N/A

c12: if [S-REL_acceptor] then m else N/A

c13: if [S-DATA_requestor] then m else N/A

c14: if [S-DATA_acceptor] then m else N/A

c15: if [NOT S-V1 and S-MAXSIZE_9 and S-EXP_T] then m else N/A

## Support for the SPDUs associated with Token Exchange

### The Session protocol data units associated with Token exchange shall  ([135])  be supported as specified in Table 4.8.

Table 4.8 : SPDUs associated with Token Exchange

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | SPDU | ISO Status | ATN Support | ISO Status | ATN Support |
| S.A.7.1.3/1 | Give Tokens (GT) | m | - (Note 2) | m | - (Note 2) |
| S.A.7.1.3/2 | Please Tokens (PT) | m | - (Note 2) | m | - (Note 2) |

Note 1:  The ISO PICS states that these two SPDUs are used for Token Exchange, but they are also used as category 0 SPDUs in basic concatenation.  Therefore, their implementation is mandatory even if no token is supported (reference ISO/IEC 8327-1 clauses 7.16 and 7.17).  However, if the null-encoding protocol option is selected, their encoding will be null, i.e. not present.

Note 2:  Not applicable, as the null-encoding protocol option is selected.

## Support for the SPDUs associated with the Duplex functional unit

### No additional SPDUs (this clause is present for completeness).

## Support for the SPDUs associated with the No Orderly Release functional unit

### No additional SPDUs (this clause is present for completeness).

## Mapping to the ATN Internet Transport Service

### The use of the connection-oriented transport service provided by the ATN Internet, and specified in Sub-volume 5 of the CNS/ATM-1 SARPs, shall  ([136])  be as specified in Chapter 6 of ISO/IEC 8327-1, except as stated in this section.

### The called and calling TSAP address shall  ([137])  be provided to the TS-Provider on a per Transport Connection basis, using the called and calling PSAP addresses as provided to ACSE in the A-ASSOCIATE request, with null presentation and session selectors.

### The TS-user shall  ([138])  indicate in all T-CONNECT requests that the transport expedited flow is not required.

### The required residual error rate shall  ([139])  be provided to the TS-Provider on a per Transport Connection basis, using the residual error rate quality of service parameters.  If the required residual error rate is set to the value zero, then the TS-provider shall  ([140])  use best endeavours to obtain the lowest available residual error rate, including the use of the transport checksum in all TPDUs.  If the required residual error rate is set to the value 1, then the TS-provider shall  ([141])  select non-use of the transport checksum.

Note:  Subvolume 5 requires, in section 5.1.2, that the TS-user specifies the required residual error rate to determine whether or not the transport checksum is required.  In the ATN, the Quality of Service provided to applications Is maintained using capacity planning techniques that are outside of the scope of this specification.  Network administrators are responsible for designing and implementing a network that will meet the QOS requirements of the CNS/ATM applications that use it.

### The Application Service Priority shall  ([142])  be provided to the TS-Provider on a per Transport Connection basis, via the TC priority quality of service parameter, using the values for Transport Layer Priority specified in Table 2-3 of subvolume 5 of the CNS/ATM-1 Package SARPs.

Note:  Although transport priority and network priority are semantically independent of each other, Subvolume 5 requires, in section 5.1.2, that the TS-user specifies the Application Service Priority, which in turn is mapped into the resulting CLNP PDUs according to Table 2-3, which defines the fixed relationship between transport priority and the network priority.

### The ATN Security Label shall  ([143])  be provided to the TS-Provider on a per Transport Connection basis.  The required ATN Security Label shall  ([144])  be conveyed by local means, using the encoding specified in Subvolume 5. section 6.2.2.1.  The QOS parameter “Routing Class” shall  ([145])  be conveyed as the Security Tag field of the security tag set for Traffic Type and Associated Routing Policies within the ATN Security Label.

Note 1:  Subvolume 5 states, in section 2.7.3.1, “The mechanism by which the [transport] connection initiator determines the appropriate ATN Security Label is a local matter.  For example, it may be identified by an extension to the transport service interface, be implicit in the choice of a given TSAP, or be identified using a Systems Management function.”

Note 2:  Subvolume 5 requires, in section 5.1.2, that the TS-User provides the complete ATN Security Label, although only security tag value is of relevance.  The encoding of the ATN Security Label is summarised below.  It consists of all fields under the heading “Security Label” (i.e.  12 octets).  The QOS parameter “Routing Class” maps to the field labelled “Security Tag”.

Security Parameter encoding							Value			Length

Parameter Code 										1101 0101			1

Parameter Length										0000 1101			1

Parameter Value:

Security Format									1100 0000			1

Security Label:

Security Registration ID Length						0000 0110			1

Security Registration ID:

OID, Length, [1.3.27.0.0]						06, 04,43,27,00,00		6

Security Information Length						0000 0100			1

Security Information:

Tag Set Name Length						0000 0011			1

Tag Set Name:

Traffic Type & Associated Routing Policies	0000 1111			1

Tag Set Length								0000 0001			1

Security Tag:

Traffic Type & category

(from Sub-volume 5 Table 6-1)				0000 0011			1

Total:	15 Octets

### No Transport Service quality of service parameters other than those specified in the preceding subsections shall  ([146])  be specified when establishing a transport connection.