CEC TEN-T ATM Task FR/98/228

SPACE

Study and Planning of AMHS Communications in Europe

| WP222: Analyse, select and document address conversion strategy |  |
| --- | --- |
| AMHS address conversion strategy in SPACE participating States/Organisations |  |
| Document Reference: | SPACE/STNA/222/WPR/158 |
| Author: | STNA |
| Revision Number: | Version 1.0 |
| Date: | jj/03/2002 |
| Filename: | input.doc |

Document Control Log

| Edition | Date | Comments | section/pages affected |
| --- | --- | --- | --- |
| 0.1 | 10 September 2001 | Creation of the document |  |
| 0.2 | 28 November 2001 | Expansion of detailed sections | mainly 3, 4 and 5 |
| 0.3 | 16 January 2002 | Insertion of abstract model, refinement of technical description, expansion of assessment, inclusion of transition considerations, insertion of conclusions | mainly 2.7, 3.2 and 4.6, 5.3, 6, 7 |
| 0.4 | 8 February 2002 | Clarification of model, editorial enhancements, inclusion of refinement resulting from NATS comments as in document N067v0_2 | 2.7, all sections |
| 1.0 | 5 March 2002 | Editorial enhancements | all |

Foreword

The work described herein has been undertaken by the author(s) as part of the European Community SPACE (Study and Planning of AMHS Communications in Europe) project, within the framework of the TEN-T programme, with a financial contribution by the European Commission. The project is carried out by a Consortium composed of the following companies and organisations: Service Technique de la Navigation Aérienne (STNA), Aeropuertos Españoles y Navegación Aérea (Aena), DFS Deutsche Flugsicherung GmbH (DFS), Eurocontrol Agency, and National Air Traffic Services Ltd (NATS).

The present document is the result of one specific task of the project, but it must be noted that SPACE is still in progress and this document should therefore not be regarded as the final deliverable by the SPACE Consortium.

Table of contents

1.	Introduction	6

1.1	Scope	6

1.2	Goal of the work package	6

1.3	Goal of the document	6

1.4	Structure of the document	6

2.	The context of AMHS address conversion	7

2.1	What is "AMHS address conversion"?	7

2.2	Outcome of SPACE WP221	8

2.3	AMHS addressing and address conversion in ICAO Document 9705	10

2.3.1	AMHS addressing	10

2.3.2	AMHS address conversion	12

2.3.3	ATN Directory and AMHS	12

2.4	Dimension of look-up tables	13

2.5	AMHS addressing strategy in Europe	14

2.6	Addressing scenarios	14

2.7	Abstract model for Gateway address conversion	15

3.	Detailed description of conversion based on mapping tables	18

3.1	Description of tables	18

3.2	Use and management of the tables	24

4.	Detailed description of Directory-based conversion	25

4.1	What is "Directory-based conversion"?	25

4.1.1	General approach	25

4.1.2	"SARPs-conformant" option	26

4.1.3	"SARPs evolution" approach	27

4.2	A technical view of the Directory in support of address conversion	28

4.2.1	Directory operations used for address conversion	28

4.2.2	Distributed operation of the Directory	28

4.3	Influence of the DIB	30

4.3.1	General aspects	30

4.3.2	Influence of the ATN DIB organisation in support of AMHS address conversion	32

4.4	Influence of the DIT	38

4.4.1	General Principles	38

4.4.2	Influence of the ATN DIT for AMHS address conversion	39

4.5	Possible Directory-based address conversion solutions	39

4.5.1	Option 1: Free AMHS address conversion Directory scheme	40

4.5.2	Option 2: Simple AMHS address conversion Directory scheme	46

4.5.3	Option 3: MD-based AMHS address conversion Directory scheme	49

4.5.4	Option 4: O/R name based AMHS address conversion Directory scheme	52

4.6	Topics for further study	56

4.6.1	Directory communication protocols	56

4.6.2	The impact of LDAP	56

4.6.3	Miscellaneous	57

5.	Assessment of the identified options	58

5.1	Criteria	58

5.2	Assessment	59

5.2.1	Ease of definition and acceptability	60

5.2.2	Flexibility	61

5.2.3	Ease of implementation	62

5.2.4	Time efficiency	63

5.2.5	Ease of use	64

5.2.6	Ease of management	65

5.3	Summary	66

6.	Combination of alternative techniques through transition	67

6.1	General	67

6.2	Co-existence within an ATSO of the APS and of Gateway's AMT	67

6.3	Handling of address conversion information external to the ATSO	68

7.	Conclusions and recommendations	69

7.1	Recommended strategy	69

7.2	Additional conclusions	70

7.2.1	Transitional provisions	70

7.2.2	Additional recommendations	71

8.	Appendix A: ATN-specific Directory object-classes and attribute-types in support of AMHS	72

8.1	Object-classes description	72

8.2	Attribute-type descriptions	74

9.	Appendix B: Fully populated MD look-up table	76

References

[1]	AMHS address conversion options in SPACE participating States/Organisations (Ref. SPACE/Aena/221/WPR/048 version 1.0)

[2]	ICAO Document 9705, Sub-Volume III, Chapter 3.1 - Edition 3

[3]	ICAO Document 9705, Sub-Volume VII - Edition 3

Table of figures

Figure 1- General model for gateway address conversion	17

Figure 2- scenario S1 for DIB Organisation	34

Figure 3- scenario S2 for DIB Organisation	35

Figure 4- scenario S3 for DIB Organisation	36

Figure 5- scenario S4 for DIB Organisation	38

Figure 6- DIT structure for free address conversion directory scheme	43

Figure 7- Example of Directory contents for free address conversion directory scheme	43

Figure 8- Example of aliases in addition to Directory contents based on a Free AMHS address conversion Directory scheme	45

Figure 9- DIT structure for simple AMHS address conversion Directory scheme	48

Figure 10- Example of Directory contents for simple AMHS address conversion Directory scheme	48

Figure 11- DIT structure for MD-based AMHS address conversion Directory scheme	51

Figure 12- Example of Directory contents for MD-based AMHS address conversion Directory scheme	51

Figure 13- DIT structure for O/R name-based AMHS address conversion Directory scheme	54

Figure 14- Example of Directory contents for O/R name-based AMHS address conversion Directory scheme	55

# Introduction

## Scope

This document is the deliverable report for Work Package 222 "Analyse, select and document address conversion strategy" of the "Study and Planning of AMHS Communications in Europe" (SPACE) project that is being run under the European Commission's programme for financial aid in the field of Trans European Networks for Transport (TEN-T), ATM Task FR/98/228.

## Goal of the work package

The initial goal of the work package 222 was to choose an address conversion strategy, from those identified in WP221, and to describe it in detailed. The selection of a strategy would have been based upon evaluation criteria also defined in WP221.

However, the work included in WP221 and the discussions which surrounded it went further than expected, and it highlighted that a particular solution could be recommended. This was included in the WP221 report as Recommendation 3 for an "intermediate (middle path) solution between the use of algorithmic tables and X.500 directory for address conversion".

Conversely, although evaluation criteria were defined in WP221, there was no real formal assessment of the envisaged address conversion options against these criteria, and the recommendation above was formulated based on mutual understandings of project partners instead of being based on a formal analysis.

Finally, WP221 has also generated a discussion on AMHS addressing which then moved to the ICAO ATN Panel and its Working Groups. The discussions held at ATNP and their conclusions have considerably changed (in the direction of simplification) the overall context of AMHS addressing and address conversion.

This situation leads to the need to slightly reformat the goal and contents of WP222.

It would therefore be of little value to reconstruct a formal analysis of the possible options, now that a recommendation has been provided. Nevertheless, this recommendation is sufficiently open for a range of solutions to be conformant. Also, a formal evaluation of the recommended solution is still needed, even though it comes as a late justification, to give an overall view of the characteristics and of the pros and cons of the solution (or range of solutions) which is ultimately recommended.

## Goal of the document

So the goal of this report is to provide a detailed description of these address conversion strategies, to describe their pros and cons against the criteria defined in WP221 and to provide views about solutions able to meet the recommendations made in the course of WP221.

## Structure of the document

This document comprises the following sections:

- the present introduction,

- a reminder describing the AMHS address conversion context, including the outcome of WP221 which was a preliminary approach of AMHS address conversion,

- a detailed description of conversion based on mapping tables, which is one of the alternatives envisaged in this document,

- a detailed description of directory-based conversion, which is the other alternative envisaged here, with several implementation options,

- an assessment of the identified options,

- a description of the combination of techniques which may happen during the transition period,

- a summary of conclusions and recommendations.

# The context of AMHS address conversion

## What is "AMHS address conversion"?

Before entering into the discussion, it is useful for the sake of completeness, and for the benefit of readers not aware of the contents of WP221, to summarise what is meant by "AMHS address conversion".

AMHS address conversion is the function that allows an AMHS user, or an AMHS system:

- to determine the MF-address of a direct or indirect AMHS user from its address in the address space of another ATS messaging technology - typically AFTN or CIDIN - and

- vice-versa, i.e. to determine the AFTN addressee indicator or the CIDIN Ae/Ax of a user from its MF-address.

The considered MF-address (to be determined, or used as a starting point) generally belongs to a MF-Addressing Scheme as defined in ICAO Document 9705, Edition 3.

AMHS address conversion, as considered in this document, contributes to and is a subset of conversions performed in AFTN/AMHS (or CIDIN/AMHS) gateways between AFTN (or CIDIN) information objects and AMHS information objects (messages and reports).

WP221 has also extended the definition of "AMHS address conversion" to include the notion of determining from the MF-address (say "MF1") of a user in an AMHS addressing scheme, another MF-address (say "MF2") for the same user in another AMHS addressing scheme. This would be done for the purpose of replacing MF1 with MF2 in an AMHS message envelope. An example of such a "conversion" is the change from an XF-address to an MF-address compliant with the Common AMHS Addressing Scheme. However, in such an MF-address replacement the remainder of the message is unaltered and stays in MHS/X.400 format. Furthermore such a replacement is not a standard MHS/X.400 function, except in some cases where redirection achieves the same goal but for recipient addresses only. For these various reasons, such "conversions" are not in the scope of WP222 nor of this document.

In summary, the required address conversion is primarily from AFTN to AMHS, and vice-versa; this can be expanded to conversion from CIDIN to AMHS (and vice-versa), although the need for such conversion should be less frequent.

AMHS addresses to be considered include at least those complying with:

- the Common AMHS Addressing Scheme (CAAS) included in ICAO Document 9705, Edition 3, section 3.1.2.1.5.1.4.3, and which had been initially developed as a SPACE deliverable in the course of WP113;

- the XF-Addressing Scheme included in ICAO Document 9705 since its first Edition, section 3.1.2.1.5.1.4.2.

Although ICAO Document 9705 strongly recommends that a State either use the Common AMHS addressing scheme (CAAS) or the XF scheme, this is not mandated. States choosing not to adopt the recommendation and to implement a locally defined AMHS addressing scheme should be warned that they create a risk of message rejection due to erroneous address conversions for the messages which they send (erroneous originator address conversion) or which are sent to them (erroneous recipient address conversion).

In doing so, they also impose a requirement on other AMHS MDs to convert AMHS addresses belonging to the locally defined AMHS addressing scheme. Therefore the ability to convert non-CAAS and non-XF addresses is considered as a useful (although not essential) additional feature to minimise message rejection.

## Outcome of SPACE WP221

The SPACE Work Package 221, named "AMHS address conversion options in SPSOs", included a first analysis of address conversion and was concluded with a set of requirements and recommendations. These are listed below:

WP221/Requirement 1: the selected address conversion solution shall be able to support any X.400 addressing plan and  make use of any address form.

WP221/Requirement 2: the AFTN address of an AFTN or AMHS user is unambiguous, internationally recognised and shall not be replaced by another value.

WP221/Requirement 3: the result of the address conversion performed in an AFTN/AMHS gateway shall depend only on the pre-defined pair of unambiguously associated AFTN and AMHS addresses, and not on the gateway itself, according to the form published by ICAO and defined by the delivering MD.

WP221/Requirement 4: The choice of conversion options shall take into account that the management and use should be as simple as possible.

WP221/Requirement 5: The conversion process shall be performed as efficiently as possible.

WP221/Requirement 6: Originating MDs shall generate addresses according to the form published by the ICAO publication and defined by the delivering MD;

WP221/Requirement 7: Delivering MDs shall be authorised to reject messages received with recipient addresses which do not comply with the address form published in the ICAO publication and defined by the delivering MD.

WP221/Requirement 8: Transit domains should not attempt to perform AMHS to AMHS address mapping unless a specific bilateral agreement has been established with the delivering MD (for the recipient addresses) or the originating MD (for originator’s addresses). Transit MDs should only use the attributes C, A, P (which are invariant and predetermined for all AMHS address forms in the ATS) in selecting a message route.

WP221/Recommendation 1: It is advisable that States and Organisations should use the common international AMHS Addressing Scheme (CAAS) defined by ICAO through ATN Panels.

It should also be noted that Recommendation 1 above was the starting point of the AMHS addressing discussion which took place in the ICAO ATNP Working Groups and which ultimately led to the insertion in ICAO Document 9705 of the SARPs clauses described in sections 2.3.1 and 2.3.2 below. With this respect, this recommendation can already be considered as being complied with.

WP221/Recommendation 2: It is recommended that each gateway performing address conversion should have access to the complete mappings between AFTN addresses and AMHS addresses and vice-versa. The complete mappings between AFTN addresses and their AMHS equivalents should be published (in electronic form) and made available to all gateways that support address translations. This implies a requirement for an ICAO wide publication that subsumes ICAO 7910, ICAO 8585 and includes the equivalent XF and CAAS addresses and any other published MF address forms. This publication should preferably be in electronic as well as paper form.

WP221/Recommendation 3: It is advisable that a middle path solution between the use of algorithmic tables and X.500 directory for the address conversion.

Since it was formulated, Recommendation 3 above has raised a few comments related to its terminology. The term "algorithmic tables" has been questioned, and should indeed be replaced with "algorithmic conversion supported by mapping tables" or "mapping tables" in short. This has been adopted in the remainder of the present document.

The notion of "middle path" should also be made more explicit. This notion was based on the recognition of the following fact. Despite the intrinsic benefits of X.500 directory for the purpose of address conversion, it cannot be envisaged that all ATSOs world-wide will implement ATN Directory services in the first instance, at the same time as AMHS. In particular, ATSOs which would first implement the Basic ATS Message Service are unlikely to deploy Directory Services before considering the move to the Extended ATS Message Service. This may take several years to happen.

In summary, the notion of "middle path" should be understood as a combination of alternative techniques, which may be adopted by different ATSOs for their respective gateways, but still operating in an overall consistent framework oriented towards eventual ATN Directory implementation.

WP221/Recommendation 4: Delivering MDs should have the capability of redirecting potential internal XF addresses to the corresponding MF(S) form addresses for use within their delivering MD, for a transition period of at least 6 months after publication of the appropriate ICAO documentation.

## AMHS addressing and address conversion in ICAO Document 9705

### AMHS addressing

AMHS addressing is specified in both a general and detailed manner in ICAO Document 9705 Sub-Volume III, Chapter 3.1. General and detailed clauses are intimately mixed in the same sections of Document 9705 for document structure reasons. However, to provide a simplified view of these clauses for explanatory purposes, they can be classified in "general" and "detailed" without any major difficulty.

The "general" considerations:

- distinguish two levels of addressing, a "high" level of Management Domains, to allow the unique identification of AMHS MDs, and a "low" level for intra-domain addressing, to allow the unique identification of an AMHS user in an AMHS MD;

- introduce the notion of "ICAO Registry of Management Domains", which is intended to be the document in which Management Domain identifiers are registered and published;

- define a high level procedure for the registration and management and Management Domain identifier by ICAO;

- introduce the notion of "MF-Addressing Schemes", which are internationally recognised AMHS addressing schemes, the description of which is included in Document 9705, with two MF-Addressing Schemes defined in Edition 3 of Document 9705. These are the Common AMHS Addressing Scheme (CAAS), and the XF-Addressing Scheme.

- strongly recommend the adoption of CAAS, although a note is made to still allow (but discourage) the implementation of "locally defined AMHS addressing schemes".

- mandate the use of XF-addresses as a default for ICAO Member States which have not declared the addressing scheme which they are using. This allows AMHS addresses to be allocated to indirect users (e.g. AFN users) even in States that have not yet started to plan and/or implement the use of AMHS.

The detailed specification:

- defines (and registers) default values for the C, A, P attributes identifying AMHS Management Domains. This allows an AMHS MD identifier to be allocated to each ICAO member State, even if it has not yet implemented AMHS;

- includes the detailed definition of the (internationally recognised) MF-Addressing Schemes, which are referred to in 2.1 above as being in the scope of this paper. These are the Common AMHS Addressing Scheme, and the XF-Addressing Scheme.

The CAAS and the XF Addressing Scheme are described in a tabular form below.

Table 1: MHS Address attributes for the Common AMHS Addressing Scheme

| Attribute acronym | Attribute Name | Assigned by | Value |
| --- | --- | --- | --- |
| C | Country | ITU-T | XX |
| A | ADMD | ITU-T | ICAO |
| P | PRMD | ICAO | Aena, CFMU, EG, France, Germany |
| O | Organisation name | ATSO | Region concept. To be defined by each ATSO |
| OU1 | Organisation Unit name | ATSO | 4-character ICAO location indicator (Doc. 7910) |
| CN | Common name | ATSO | Either: - 8-character AFTN address, - CIDIN Ax address |

Table 2: MHS Address attributes for the XF Addressing Scheme

| Attribute acronym | Attribute Name | Assigned by | Value |
| --- | --- | --- | --- |
| C | Country | ITU-T | XX |
| A | ADMD | ITU-T | ICAO |
| P | PRMD | ICAO | The default value, in case of no explicit registration, comprises the two ICAO Nationality Letters (cf. Doc 7910) identifying the ICAO member State. |
| O | Organisation name | SARPs | AFTN |
| OU1 | Organisation Unit name | ATSO | 8-character ICAO AFTN addressee indicator |

### AMHS address conversion

AMHS address conversion is specified in a functional way in ICAO Document 9705. The specification includes:

- a description of the look-up tables which are needed in an AMHS gateway to enable address conversion. Two look-up tables are defined:

- a MD look-up table defined in SARPs clause of 3.1.2.3.3.2.1, which allows derivation of an AMHS MD identifier from character combinations extracted from the considered AFTN/CIDIN address. Most frequently the MD identifier (C, A, P attributes) will be derived from the first two characters of the AFTN/CIDIN addresses which are the ICAO Nationality Letters. However specific cases may require more characters to  determine the corresponding MD. The order of precedence between different matches is provided in Doc 9705. Examples of such cases are given in Edition 2 of ICAO Document 9739;

- a user address look-up table defined in SARPs clause of 3.1.2.3.3.2.2, which allows derivation of a complete MF-address from the considered AFTN/CIDIN address (clauses 3.1.2.3.3.2.2.1/3), and, for the specific use of the CAAS, which allows derivation of the C, A, P, O and OU1 attributes (i.e. a partial MF-address) from the ICAO Location Indicator included in the AFTN/CIDIN address, i.e. its first four characters (clauses 3.1.2.3.3.2.2.4/5).

- a description of how these tables must be used for address conversion, in each direction - from AFTN to AMHS or conversely - depending on the AMHS addressing scheme adopted for the considered user: e.g. CAAS, XF or some other, locally defined scheme.

The specification is functional in the sense that it only defines the mapping to be performed, and the order of priorities to be adopted when considering the result of such derivations, etc. The detailed format of the tables is not specified, nor is the detailed conversion algorithm, although both may be easily inferred from the specification. A note also clarifies that the implementation of an algorithm is not constrained by the text, provided that the outcome is identical between the "theoretical algorithm" of Doc 9705 and an actually implemented algorithm.

For example, precedence is given to a "full user address match" over a "location indicator match" in the user address look-up table. However, if it is known a priori that there are no full user addresses configured for a given user in a given MD, because that MD has adopted the CAAS, there is no need to try and perform a full user address match which would obviously fail.

### ATN Directory and AMHS

The DIT, object-classes and attribute types for the ATN in general are specified in the SARPs (Sub-Volume 7 of Doc 9705, Edition 3).

This includes the specification of Directory information in support of AMHS, using two different methods:

- a reference to standard MHS object-classes and attribute types specified in X.402,

- the definition of specific ATN (AMHS)-related object-classes and attribute types directly included in Sub-Volume 7.

A naming hierarchy (DIT structure) is also defined for AMHS-related entries in Sub-Volume 7.

Among the object-classes included in Sub-Volume 7 for AMHS, there are in particular three object-classes that have been designed for the inclusion in the ATN Directory of entries related to AMHS address conversion:

- the specific atn-AmhsMD object-class aims at the inclusion of attributes in entries for describing AMHS Management Domains, for the very purpose of address conversion;

- the specific atn-AmhsUser object-class aims at the inclusion of attributes in entries for describing (direct or indirect) AMHS users, including the information needed for address conversion in the form of specific attribute types;

- the specific atn-AmhsDistributionList object-class aims at the inclusion of attributes in entries describing AMHS distribution lists, including information needed for address conversion in the form of specific attribute types.

Item a) above can be used as an equivalent to the MD look-up table in the ATN Directory, whilst items b) and c) include the information needed for the full user-address look-up table.

To simplify the discussion, AMHS DLs are not specifically considered in this document, except when DIT structures are represented. In practice, an AMHS DL would be handled exactly in the same way as an AMHS user, as far as address conversion is concerned.

The ASN.1 description for these object-classes is provided in Appendix A.

Nevertheless it should be noted that there is currently no object-class specific to AMHS for the description of locations, which would include the values of the ICAO Location Indicator and of the C, A, P, O and OU1 address attributes. This might be needed if  the ATN Directory is to support the CAAS. In the DIT structures studied in this document, assumptions are made with this respect, making use of atn-specific object-classes, in particular atn-organization and atn-organizational-unit. However this might be optimised to avoid confusion of terms and/or intentions.

Recommendation 1: The ATNP should examine the possibility of including in Document 9705, Sub-Volume 7, one or several atn-specific directory object-classes and attributes-types for the representation of ICAO locations and of AMHS regions, in support of the Common AMHS Addressing Scheme (CAAS). Alternatively an existing object-class such as atn-organization could be specified for this purpose.

## Dimension of look-up tables

It may be useful to recall the expected size of tables at a global level:

- the MD look-up table is expected to include a few hundred entries, this number corresponds approximately to the number of nationality letters combinations defined in ICAO Doc 7910,

- the part of the user address look-up table dedicated to the CAAS may include up to ten thousand entries, this number corresponds approximately to the number of location indicators defined in Doc 7910,

- the user address look-up table, if populated with all the full AMHS addresses potentially used for AFTN and AMHS users (which would be necessary for non-XF and non CAAS schemes), might include more than a hundred thousand entries. This does not mean that there would be so many actual AMHS users, but due to the potential construction of AFTN addresses by combination of Doc 7910 four character indicators and Doc 8585 three character identifiers, there might be a need for inclusion of that number of entries.

Such tables need to be included in each AFTN/AMHS gateway, or at least this conversion information needs to be reachable from each gateway.

The benefit of the CAAS clearly appears here, because it avoids populating the full user address look-up table, which may even not be needed at all if all ICAO Member States and Organisations adopt either the CAAS or the default XF-Addressing Scheme.

## AMHS addressing strategy in Europe

During the ICAO AFSG/4 meeting held in the EUR Regional Office in Paris in April 2001, the EUR Region AMHS addressing Plan was reviewed. This addressing plan had been developed by SPACE in the light of both Regional and Global developments, and it was aligned with the Common AMHS Addressing Scheme, although at that time the term "Common AMHS Addressing Scheme" did yet not exist. The meeting agreed that States should be urged to implement the CAAS, thereby making it the target addressing plan for AMHS in Europe.

## Addressing scenarios

Four scenarios for AMHS addressing on a world-wide basis have been described in the WP221 report:

- existence of different X.400 MF-addressing schemes,

- use of a single X.400 MF-addressing scheme,

- use of the 'XF' addressing scheme for international traffic,

- solutions based on 'tunneling' or 'double enveloping'.

Among these scenarios described in section 4.4 of the WP221 report, the associated reflections of WP221 exclude the last two scenarios.

Scenario 2 is obviously the most desirable, because it simplifies the address conversion by reducing the heterogeneity of the implemented addressing schemes considerably. The definition of the CAAS makes Scenario 2 possible, but unfortunately it cannot be ascertained that every ICAO Member State or Organisation will adhere to the CAAS or to the (default) XF-Addressing Scheme. Usage of the Common AMHS Addressing Scheme (CAAS) is a very strong recommendation in Edition 3 of ICAO Document 9705, but it cannot be made mandatory.

In conclusion, it may realistically be assumed that the addressing scenario in which AMHS address conversion will operate should be composed of:

- States/organisations remaining with XF-addressing, either because they have not yet implemented AMHS or because they consider XF-addressing to be simpler (this may be true when there is a small number of users in the considered State),

- among the States/organisations which declare use of an addressing scheme different from XF, a vast majority of States implementing the CAAS in compliance with the recommendations of Document 9705,

- among the States/organisations which declare use of an addressing scheme different from XF, a small number (if any) of States implementing locally defined addressing schemes.

## Abstract model for Gateway address conversion

A model of gateway address conversion is depicted in Figure 1 below. This figure is derived from the WP221 report.

The figure represents information exchanges between ICAO and three ATSOs implementing AMHS Gateways, concerning address conversion.

The dotted blue arrows represent exchanges that are performed in a non-electronic way, e.g. through "paper" procedural exchanges. The full line green arrows represent exchanges that are performed electronically using appropriate communication protocols.

Among the three ATSOs, two of them (ATSO1 and ATSO2) implement a distributed address publishing service (APS), typically by means of ATN Directory Services. This allows electronic distribution and/or retrieval of address conversion information. ATSO3 provides this information to ICAO for manual collation and distribution (e.g. on paper), and does not support a directory.

The numbered arrows correspond to the following exchanges:

- Provision of ATSO address conversion information to ICAO for registration purposes. This is a low frequency (e.g. twice a year) exchange which covers the initial provision and periodic updates;

- Internal ICAO process for gathering of address information to be published;

- Distribution of consolidated address conversion information by ICAO using manual/documentary procedures. This information is used either to feed an electronic APS or as a direct input to the gateway depending on whether the ATSO has implemented a distributed APS or not. This ICAO distribution process is periodic and may be performed e.g. once or twice per year. As for other ICAO documents, the date of applicability needs to be specified at the time of publication;

- Retrieval or update of address conversion information by electronic means, which will be more up-to-date than the information periodically published by ICAO;

- Synchronisation and/or retrieval of address conversion information within the distributed address publishing service;

- Retrieval of information by (personal) users at end systems (e.g. at an ATS message user agent).

The model identifies a number of components that are necessary to address conversion:

- Collection and distribution of the basic addressing information that establishes equivalences between the different addresses identifying each AMHS/AFTN/CIDIN user; the content of this information must be standardised since it must be distributed and made available to all AMHS/AFTN/CIDIN Gateways. However, there is a need to publish this information in a number of different standardised formats because some gateways may accept electronic (e.g. X.500 Directory) forms, while others will not (and must use e.g. paper based publications or some simpler electronic form – e.g. CSV - comma separated variables). This document (WP 222) specifies two possible forms – an X.500 directory form, and an alternative ‘Mapping Table’ form of publication;

- Access to, and/or import of the basic addressing information into AFTN/AMHS gateways. Each gateway will have to import the basic addressing information distributed by (1) above into internal storage suitable for use with that gateway’s particular address conversion procedures; since this is very dependent on the particular gateway implementation it cannot be standardised. This document (WP 222) makes suggestions only on how the information may be extracted from the X.500 Directory (APS);

- Re-structuring the basic addressing information into a format suitable for use by each gateway’s internal address conversion procedures (AMI). This is very gateway implementation specific – and is therefore not dealt with further in this document (WP222);

- The internal procedures and data structures of the gateway (AMP and AMT) that make use of the re-structured addressing information. This is very gateway implementation specific – and is therefore not specified in this document (WP222) apart from indicating a potential Directory based solution and describing an example of how a particular gateway's AMT might be structured.

Figure 1- General model for gateway address conversion

In summary, AMP, AMT and AMI are internal to an ATSO's gateway, and as such, the selection of a given technique for these processes is an implementation matter to be decided locally by each ATSO and/or product developer. Conversely, the distributed APS is an externally exposed service, for international use, which is therefore subject to ICAO standardisation and/or recommendation processes.

Possible scenarios for APS implementation are studied in further sections of this document.

Moreover, it is important to ascertain that technical solutions exist for the implementation of all these elements (AMP, AMT, AMI and distributed APS), even if the technical choice is then left to each  individual implementor. To demonstrate such feasibility is one of the goals of the following sections 3 and 4 which provide a detailed description of the envisaged techniques.

Address Mapping Information Content

The address mapping information content held in AMT and distributed through APS is identical. An example of this information as potentially used by AMT is provided in section 3. However its format may be different in the two environments to reflect the different requirements for management and access. Thus the following possible structures are valid:

AMT will often be structured into tables that support easy access by AMP, and be optimised for speed of operation of AMP. Generally, the gateway supplier will determine this structure according to the required speed optimisation, the way in which AMP operates and the particular platform on which AMT and AMP reside. AMT is not, therefore, an easy subject for standardisation. However, a particular (standard) way of structuring AMT is to hold it in a local ‘private’ Directory Information Tree either using the same form as the DIT specified by APS, or by adopting a more efficient structure such as using the DIT outlined in Section 4.5.4;

The structure of APS must be compatible with many different systems (e.g. different ATSO’s Gateways), and must therefore be standardised. There are a number of possibilities for structuring APS:

- As an X.500 Directory Information Tree;

- By some other electronic means (e.g. CSV files);

- On paper.

The DIT solutions are discussed and evaluated in section 4 of this paper. Other electronic forms and paper are not evaluated.

# Detailed description of conversion based on mapping tables

Two distinct but potentially overlapping things are discussed in this section:

- the ‘Abstract’ content of the Address conversion information,

- an example of how a particular gateway’s AMT might be structured.

## Description of tables

Conversion based on mapping tables stems directly from the clauses of ICAO Document 9705.

As described above, Document 9705 foresees two tables, called the MD look-up table and the user look-up table. However in terms of AMT implementation, many other structures are possible, from one single table to a combination of multiple tables (e.g. one table per MD).

Examples of such tables are given in Tables 3, 4 and 5. The algorithm using these tables is given in Table 6:

- an MD look-up table strictly conformant to Document 9705, already populated with known values for SPSOs and examples of default values for ICAO Member States. The table also includes a few additional fictitious examples, to illustrate how other cases (e.g. non XX/ICAO) could be handled;

- a user look-up table strictly conformant to Document 9705, already populated with known example values for SPSOs. The table also includes a few additional fictitious examples, to illustrate how other cases (e.g. non CAAS) could be handled;

- a simplified user look-up table, showing how a reduced amount of information can achieve the goal of Document 9705, through the removal of redundant information (once it is known to be redundant).

These examples also highlight a set of minor issues or questions that would need to be sorted out prior to actual implementation on an international basis.

Note:	in the first two tables, columns in italics (apart from the "comments" column) are not strictly mandated by SARPs but considered extremely useful.

Notes regarding the MD look-up table

Note 1

The MD table shows that the same nationality letters (i.e. the same pair of initial letters in an ICAO location indicator) are sometimes used by different States or Territories. "FM" is an example that is used to identify three different States (Comoros, Reunion (France) and Madagascar). Document 9705 enables differentiation between these three States if four characters (i.e. the location indicator) is used instead of the two characters for the so-called nationality letters. However this is not optimal and it may lead to unnecessary entries in the MD table. A solution to reduce this would be to allow the use of three characters as a discriminator for an AMHS MD and for the "nationality letters". In the above example, "FMC" would be used for Comoros, "FME" for Reunion and "FMM" for Madagascar.

Note 2

A conventional representation of unused characters (wild card) appears to be needed for the "nationality letters", in cases of five characters combinations. This would also be another way to resolve the issue above, using for example "FMC?", "FME?" and "FMM?".

Note 3

The example of the "AIRLINE1" MD is introduced because such cases had been initially envisaged when the AMHS SARPs were drafted. The 5-character combination that isallowed for AMHS MD discrimination directly stems from this requirement. However, the registration of companies such as Airlines in an ICAO Registry of AMHS MDs might be seen as an institutional issue. At this stage, the example is maintained but this point needs to be addressed in the SPACE Work Package 233 ("Financial and Institutional Issues") to analyse whether or not this situation should be considered.

Table 3: example of MD look-up table

Table 4: example of user look-up table conformant to Document 9705

Table 5: example of user look-up table optimised to reduce volume of information

Table 6: Conversion algorithm associated with the optimised user look-up table

| AMHS  AFTN | AFTN  AMHS |
| --- | --- |
| - Extract the Country, ADMD and PRMD attributes from AMHS address. - Search in the MD look-up table for an entry of which the country-name, ADMD name, PRMD-name provide an exact match with the C, A, P address attributes extracted in step 1 and save the corresponding addressing scheme value. - Depending on the addressing scheme value: - if value=xf, extract the OU1 address attribute value from the AMHS address and use as the AF-Address; - if value=caas, extract the CN address attribute value from the AMHS address and use as the AF-Address; - if value=other, search in the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value (result of step 2) for an atn-AmhsUser entry with a mhs-or-address attribute value matching the AMHS address. Extract the atn-AF-Address attribute value of that entry. - extract the "low level" address attributes (other than C, A, P) from the AMHS address; - search for an entry in the user look-up table, for which the value in the " O/R address components " column corresponds exactly to these address attributes, and for which the MD value corresponds to the MD identified in step a), - in the row corresponding to that entry, read the " location indicator or AFTN address " column of the user look-up table and extract the value of the 8-character AFTN address. | - Extract the character combinations potentially identifying an AMHS MD from the AFTN address (2, 4, 5 or 7 letters). - Search in the MD look-up table for an entry of which the "nationality letters" provide the best exact match with one of these character combinations, and save the corresponding country-name, ADMD name, PRMD-name and addressing scheme values. - Depending on the addressing scheme value: - if value=xf, construct an XF address using the C, A and P address attribute values and the AFTN address; - if value=caas, - search for an entry of the user look-up table, for which the value in the "location indicator or AFTN address" column corresponds exactly to the first four characters extracted from the AFTN address, and for which the MD value corresponds to the MD identified in step a), - in the row corresponding to that entry, read the "organisation name" column of the user look-up table the value of the corresponding organisation-name address attribute, - construct an AMHS CAAS address using the C,A,P address attributes found in step a), the O address attribute found in step (ii), the location indicator (4 characters) extracted from the AFTN address (OU address attribute) and the AFTN address (CN address attribute). - if value=other, - search for an entry of the user look-up table, for which the 8 character-value in the "location indicator or AFTN address" column corresponds exactly to the AFTN address, and for which the MD value corresponds to the MD identified in step a), - in the row corresponding to that entry, read the "O/R address components" column of the user look-up table for the value of the "low level" O/R address attributes of the AMHS address, - construct an AMHS address using the C,A,P address attributes found in step a), and the address attributes found in step (ii). |

## Use and management of the tables

The tables above are mainly specified for use in AMHS Gateways.

They could be used and managed as follows, with reference to the general model presented in Figure 1.

The tables represent a potential format for the ICAO Published Address Information (AI), and for distribution of AI ( in the general model). They also give an example of a gateway's address mapping tables (AMT), with the above algorithm being part of address mapping procedures (AMP). This is particularly applicable to ATSOs that do not have access to the distributed address publishing service (APS), such as ATSO3 in the model.

However, their use is more important for the initial population and periodic updates of the Distributed Address Publishing Service (APS). This applies to ATSOs having implemented the Distributed APS, or having access to this service, by means of operations  and  like ATSO1 and ATSO2 in the model.

AI should be collected and maintained by means of an ICAO procedure (or a set of) along the following principles:

- an initial MD table should be created by ICAO based on default values defined in SARPs,

- each row of the MD table should be confirmed or updated by the relevant ATSO (operation ),

- for the support of CAAS, the set of rows of the user look-up table providing location indicators and associated organisation-names for regions within a given MD should be supplied and updated when appropriate by the relevant ATSO (operation ).

It should be noted that their existence does not resolve the requirement that a direct (personal) user at an AMHS UA may have to construct an AMHS address for a recipient whose AFTN 8-character address only is known ( in the model). For this purpose, an ad-hoc mechanism needs to be defined in order to (for example) populate the local address book of the UA with full AMHS addresses, referred to by short names.

Another essential point to be recalled is the following. Each AMHS gateway without an access to the Distributed APS needs to include the full table with entries related to all AMHS MDs world wide, and to all ICAO location indicators in States having declared use of CAAS.

This is where the requirement for well-defined administrative procedures comes from, i.e. - to allow any MD to distribute its local information to any other MD. This clearly cannot work properly in an "any-to-any" mode, and it is the justification of the requirement for ICAO AMHS address information collection, publication and distribution procedures as depicted in operations ,  and .

Requirement 1:

ICAO procedures need to be defined, and appropriate means need to be allocated, to allow:

- first to gather address conversion information from all MDs into a central point (e.g. ICAO Secretariat);

- then to format, collate, publish and distribute this information to all MDs.

This document makes the assumption that such procedures are defined prior to generalised deployment of table-based address conversion.

# Detailed description of Directory-based conversion

## What is "Directory-based conversion"?

### General approach

Based on the description of section 2.1, the notion of Directory-based conversion can be introduced as follows:

Directory-based AMHS address conversion is the function which allows a user of the ATN Directory (typically a user at an AMHS User Agent including an ATN DUA, or the DUA Component of an AFTN/AMHS Gateway):

- to find information corresponding to the MF-address of a direct or indirect AMHS user, by providing its AFTN or CIDIN address as an input to the ATN Directory query - and

- vice-versa, i.e. to find information corresponding to the AFTN addressee indicator or to the CIDIN Ae/Ax of a user, providing its MF-address as an input to the Directory query.

To be able to do so, the ATN Directory needs to include both addresses of the user, in one way or another, and to provide the functions that allow mapping from one to the other.

The refinement of the definition above requires some assumptions about the way in which the information is stored in the ATN Directory. Two alternatives are possible at this stage.

The first option consists in using the DIT, object-classes and attribute types currently specified in the SARPs (Sub-Volume 7 of Doc. 9705, Edition 3) to the largest possible extent. Divergence from the SARPs in this case would be minimal and envisaged only where they do not allow fulfilment of the objectives of conversion. In short, AMHS address information would be stored in the ATN Directory using attributes of the atn-AmhsUser object class. This option is described in section 4.1.2 below.

The second option consists of complementing the SARPs with clauses based on ISO/IEC 10021-10 (MHS Routing), which allow the direct representation of MHS OR-addresses in the form of Directory Names that locate a user’s entry using the OR-Address. In this case the DN of such an entry is very similar to the OR-address. Dedicated object-classes to support the DIT structure are defined for this purpose in ISO/IEC 10021-10, but support of these object-classes is not currently mandated (nor recommended) by the SARPs. These object-classes and the associated attribute-types would also need to be complemented to support AFTN or CIDIN address information. This option is described in section 4.1.3 below.

### "SARPs-conformant" option

This option essentially relies on the use of the object-classes currently specified in the SARPs (Sub-Volume 7 of Doc 9705, Edition 3). These object-classes are:

- atn-AmhsMD, which provides a directory representation of an entry belonging to the MD look-up table defined in Sub-Volume 3;

- atn-AmhsUser, which is used for the representation of an AMHS user, and which allows for the storage of its address in both MF-Address form and AF-Address form (or Ax/Ae in case of address conversion to/from CIDIN). This object-class provides, among other elements, a directory representation of an entry belonging to the user address look-up table defined in Sub-Volume 3, and describing the full MF-address of a user.

The address conversion process can be summarised as follows:

Knowing the (unique) attribute-value of an attribute ‘A’ belonging to an entry of the atn-AmhsUser object-class, determine the (unique) attribute-value of another attribute ‘B’ of the same entry. The attribute types of ‘A’ and ‘B’ are atn-AF-address and mhs-or-addresses, or vice-versa, or atn-Ax-or-primary-Ax-address and mhs-or-addresses (in case of address conversion to/from CIDIN).

It may be seen that in this case it is not mandatory to use the DN allocated to the considered atn-AmhsUser entry, since it is not directly used in the conversion process. However using the DN may ease or improve the process, depending upon the details of the adopted solution.

The DIT specified in the SARPs (Sub-Volume 7 of Doc 9705, Edition 3) is then applicable. It provides an overall framework, or a "superset" of all the naming hierarchies that can be adopted for an AMHS user. This means that a degree of freedom is offered to ATSOs for the definition of the Directory Name (DN) of such an AMHS user. A minor caveat to the general applicability of the SARPs DIT is that it may still require minor additions/amendments to structure rules, and possibly by addition of a few object-classes.

### "SARPs evolution" approach

This option relies upon the use of the object-classes and attribute-types defined for the initial purpose of MHS Routing in ISO/IEC 10021-10. This standard defines an entire OR-Address-subtree, where each X.400 address attribute is mapped onto a directory entry of a corresponding OR-address attribute object-class.

The standard obviously does not include any provision for AFTN or CIDIN, because it has been defined in the context of X.400/MHS (not AMHS). In order to be useful in the context of AMHS address conversion, from a technical viewpoint, it needs to be complemented with object-classes and attribute-types, as appropriate, in which AFTN or CIDIN addressing information is to be stored. This may be done by definition of one or several specific attribute-types, which could bear names similar to those currently used in SV7. The question then arising would be to define in which object-class these attributes would be inserted.

Subject to these additions being formally endorsed by the ATN Community, for example by means of an Amendment to Doc 9705 Sub-Volume 7 (as part of Edition 4 for example), the address conversion process could be summarised as follows:

1) in the direction AMHS to AFTN/CIDIN: Knowing the (unique) DN belonging to an entry in the OR-address subtree that corresponds to the MF-address of an AMHS user, determine the value of the attribute atn-AF-address or atn-Ax-or-primary-Ax-address included in the last RDN of the DN.

2) in the direction AFTN/CIDIN to AMHS: Knowing the (unique) attribute-value of an attribute atn-AF-address or atn-Ax-or-primary-Ax-address belonging to an OR-address element entry part of the OR-address subtree, determine the full DN of the entry which corresponds to the MF-address of an AMHS user.

In this case, the DIT used for the inclusion of AMHS address information in the ATN Directory is constrained by the subtree structure specified in ISO/IEC 10021-10, Annex D, and by the AMHS addressing scheme adopted the ATSO in charge of the considered AMHS users.

However, this does not preclude in principle the simultaneous use of the current Directory SARPs specification (Sub-Volume 7 of Doc 9705, Edition 3) for purposes other than address conversion, e.g. for the representation of the capabilities of an AMHS user. Minor amendments would be required in that part of the SARPs to modify the status of some attributes, so as to avoid potential redundancies and inconsistencies.

## A technical view of the Directory in support of address conversion

### Directory operations used for address conversion

Retrieval of information from the Directory is performed by means of abstract-operations that are executed by the Directory upon request by a DUA through an "access point". This access point may be seen as the DSA that is first queried by the DUA.

Once the ATN Directory has been populated, the main X.500 abstract-operations available for the purpose of address conversion (as described above in section 4.1) are the following:

- read: this operation provides results consisting of entry information held in the Directory, i.e. an entry's DN, attribute types and values. The argument to be submitted for this application includes the DN of the selected entry. In summary, read is useful to retrieve information from a single, explicitly identified entry, for which the DN is already known;

- search: this operation also provides results consisting of entry information, i.e. an entry's DN, attribute types and values. The argument to be submitted for this application does not generally include the DN of the considered entry, but only the base object under which the search operation is to be performed and selection criteria by means of a filter. The argument includes the "depth" of the search operation under the base object, for example, the depth may be limited to a single level of subordinate entries, or it may allow a search of the whole subtree. In summary, the search operation is useful to locate and retrieve attributes from one or several entries that match a filter in an identified subtree where the DN of the required entry is not known.

### Distributed operation of the Directory

#### Base standard mechanisms

The Directory as specified in X.500 has been designed to allow distributed operation. This means that the DIB (Directory Information Base) may be distributed over several DSAs, which co-operate to provide a reply to a query submitted by a DUA. This distributed operation is feasible provided that all of the considered information belongs to a single DIT structure, with a common root for the DIT. Each DSA then holds a part of the DIB that corresponds to one or several subtrees of the entire DIT.

If the "initial" DSA to which the query (read or search) has been submitted contains the whole DIB, then it may by itself provide the expected results. If the DIB is distributed over several DSAs, the "initial" DSA may not contain the desired entry (either "target" entry for read or base object for search), and the operation will be fragmented over several DSAs. This will happen by means of chaining between DSAs (or it may cause referrals).

Chaining is described in ITU-T X.518 / ISO/IEC 9594-4. Chaining is performed on the basis of knowledge information held by each DSA. This knowledge information provides a given DSA with a "view" of the implemented DIB’s organisation. This allows among other things, one DSA to locate another DSA that holds a required entry, the name of which is known. This process is known as "name resolution". The knowledge held by a DSA is defined in terms of a set of one or more knowledge references where each reference associates directly or indirectly entries of the DIB (represented by whole or part of their DN) with the DSA that holds those entries. The DSA identified in the reference is represented by its Access Point, including a PSAP+AET address allowing establishment of a DAP or DSP association with the DSA.

A DSA may hold the following types of knowledge reference:

- a superior reference (within the same DIT branch as the considered DSA);

- immediate superior references (within the same DIT branch as the considered DSA);

- subordinate references (within the same DIT branch as the considered DSA);

- non-specific subordinate references (within the same DIT branch as the considered DSA), and

- cross references (either within the same DIT branch or within other branches).

The knowledge possessed by the initial DSA allows "navigation" through the topology of DSAs that are inter-related by means of the knowledge references. These DSAs together hold the entirety of the ATN Directory, broken down into the DIT subtrees being held each by a DSA. In this way the query submitted to an initial DSA is "forwarded" from DSA to DSA until it reaches a DSA holding the required entry, where the query can be resolved. This forwarding is the chaining operation referred above.

#### Distributed operations with copy entries

The Directory standards allow for copy (also called a shadow) of entries from one DSA (the shadow supplier) to another DSA (the shadow consumer) by means of the Directory information shadow service.

The Directory information shadow service provides a standardized mechanism to provide and support shadowed (i.e. copied) information. In outline, the shadow supplier maintains, for each shadowing agreement, information that is to be copied (the shadowed information). This information is replicated by protocol exchange between the shadow supplier and the shadow consumer. The information to be shadowed is all or a subset of the information held by the shadow supplier’s DSA Information Tree. The shadow consumer's shadowed information becomes part of its DSA Information Tree.

To use the directory information shadow service, the Administrative Authorities of two DSAs must first reach an agreement on the terms under which shadowing will take place.

Shadowing (or replication) operations can be performed either under a push (supplier-initiated) or pull (consumer-initiated) model. In all cases, such operations are time and bandwidth consuming, and administrative agreements and rules are generally defined to avoid an excessive number of shadow operations. The replication frequency is generally defined in relation with the dynamics of changes in the considered directory: the more frequent the changes, the more frequent the replications. Typical frequencies for mail/phone directory replications in a company are weekly or monthly.

A reply to a query submitted to the Directory may be supplied from a copy. However, the submitting user may specify in its query that only original entries are to be used to provide the reply ("don’t use copy" parameter).

In general, the use of shadowing greatly enhances the overall efficiency of a distributed Directory. However, for both technical (bandwidth) (access rights management) and administrative (access rights management) reasons, they are more useful and practical in a hierarchical directory model (e.g. one master and several copies) than in a "flat" directory model.

#### Other mechanisms

The functions and mechanisms described in the two sections above aim at allowing the Directory as a whole to provide a reply to a single user query or operation, rather than relying only on the contents of the DSA to which the DUA is connected. In this way, the potential complexity of the Directory is hidden from the user, at the cost of a potentially increased response times when chaining is used.

As soon as the user accepts the use of several subsequent operations for the retrieval of a single piece of information, many more ways of working are possible. As an example, intermediate directory location references (e.g. referrals, or other ad-hoc iterative processes) may be accessed in an initial operation, prior to searching a given DSA for the desired result itself. Such combinations can clearly be envisaged for AMHS address conversion, where the main users are gateway systems making use of programmatic interfaces.

## Influence of the DIB

It is necessary to take into account the organisation of the DIB (Directory Information Base) concerning the location of Directory Servers and the information stored in each Directory Server, to ensure that the data needed for conversion is available, and to ascertain the efficiency of the process. This is clearly a difficulty because the DIB organisation is not the subject of this work package, and it has not been defined yet by ATNP, SPACE, nor the Regional Planning Groups. In the context of this SPACE work package, it is therefore necessary to make realistic assumptions about the organisational options that could be implemented.

### General aspects

#### Impact on query resolution time

The time required to obtain the result of an X.500 query (read or search) performed on a distributed directory depends on the number of executed chaining operations. Indeed, a chaining operation can be broken down into the following main operations:

- find the target DSA (i.e. the DSA which will be able to solve the received query),

- establish the connection with it,

- send the query,

- await a response  or time-out.

These operations will be performed on each DSA questioned until the query is either resolved or the time allowed for this operation on the first DSA be expired.

If the DIB is highly distributed, i.e. spread over a high number of different DSAs, the operation of finding the target DSA may itself consist in a number of chaining operations.

The interrogation of a given DSA requires the establishment of an association with the DSA (Bind operation), the transmission of the data associated with the (initial or chaining) Directory operation, the reception of the operation results, and the release of the association (Unbind). The time needed to perform these tasks obviously depends upon numerous conditions (such as the bandwidth of underlying networks, efficiency of directory software and communication layers implementation, processing power the DSA executing the operation, size of the database part hosted in the DSA, type of requested operation, etc.). However, typical orders of magnitude are illustrated in the table below, for both the directory operation itself in the DSA (local database interrogation) and the time to remotely start, perform and terminate the interrogation. These orders of magnitude are provided for a single DSA, and for an operation querying an overall DIB, in two different configurations (highly centralised and highly distributed).

Table 6: Relative orders of magnitude for the time to perform directory operations

|  | time for execution of the local operation in the DSA | communication time for a remote interrogation | overall time |
| --- | --- | --- | --- |
| query to a single DSA | 1 | 10 | 1+10 |
| query to an overall distributed DIT (highly centralised configuration) | 1 x n (database size and CPU power factor) | 10 | 1 x n + 10 |
| query to an overall distributed DIT (highly distributed configuration) | 1 | 10 x m (number of chained operations factor) | 1 + 10 x m |

The goal of Table 6 is to show that the effect of inter-DSA communication times(which are multiplied by the number of chaining operations in a distributed environment), increases the time required to perform a given operation when the overall DIT is more widely distributed over different DSAs.

In summary, directory operations are generally more time-efficient when the overall DIT is concentrated over a smaller number of (high capacity) DSAs.

#### Impact on management of Directory information

The primary management task required  to set up a Directory is the population of the database, i.e. the initial input of information. ??? is this a correct translation??

Then, in parallel with its use by means of queries, the information stored in the Directory needs to be updated and complemented as appropriate to reflect changes in the objects described by the directory entries (e.g. changes in network or O/R addresses, etc.).

Such management functions are performed more easily when the Directory operator, and the DSA itself, are closer geographically and/or organisationally to the objects represented in the Directory, i.e. when entries are held in a DSA which is "local" with respect to their organisation or organisational unit. Such notions obviously tend to increase the number of DSAs within a given overall Directory.

Whilst such considerations may be not critical in an environment consisting of a single organisation (as a company for example), they become essential in a community composed of several organisations such as ATSOs grouped within ICAO. To illustrate this, it appears that technically, a single central DSA operated by ICAO for world-wide coverage would probably be preferable. However, it would be very difficult to manage requiring very frequent changes related to each ICAO member State.

#### Design of a DIB organisation

In addition to the elements above, there are clearly other factors that impact the design of a DIB organisation, among which are the replication capabilities and requirements, and the availability (or not) of efficient lower layers communications, etc. Cost constraints also have an obvious impact, either on the server capacity, or on communications costs, etc.

The task of taking into account of all of these factors and constraints, and to derive from this a DIB architecture in an environment as large  as the ATSO community (and composed of multiple organisations), is a project in itself. As stated at the beginning of 4.3, this is not in the scope of the SPACE project. So, at this stage, it is preferable to limit the approach to general principles, with a summary of the above which shows that the design of a DIB organisation is generally a compromise between the need for management efficiency and  efficiency of use.

### Influence of the ATN DIB organisation in support of AMHS address conversion

The requirement for fast address conversion generates a practical requirement to limit the number of subsequent queries to various DSAs for a single address conversion operation, ideally to a number of one or two DSAs being "explored".

This can be achieved only under the following  conditions:

- for any AMHS user or gateway querying the ATN Directory for the purpose of address conversion, the whole address conversion information for the entire AMHS world wide is stored in the same DSA, or

- for any AMHS user or gateway querying the ATN Directory for the purpose of address conversion, the queried DSA includes either the requested conversion information or sufficient data to identify directly the DSA that holds the requested information.

In the first case, the query result is obtained directly from a single DSA. In the second case, one or two DSAs are queried, depending upon the requested address conversion information.

#### DIB organisation with each DSA containing the whole conversion information

This organisation corresponds to case 1 above. It means that any ATN DSA used in support of AMHS address conversion stores a considerable amount of information, related to the entire AMHS world-wide. This can be envisaged only if there is a "single master DSA", which holds the complete information base. If there are "local DSAs" in operation for performance purposes, then these include a replication of the entire information base.

Three main scenarios may consequently be envisaged:

S1	a single central DSA is implemented for the whole AMHS world-wide; entry inputs and modifications are made remotely, queries are submitted remotely to this central DSA from all over the world;

S2	a single central master DSA is implemented for the whole AMHS world-wide; entry inputs and modifications are made remotely from all over the world, replications from the master DSA are performed towards several "local" DSAs (typically one per Region or per State). Queries from users are submitted to the local DSAs and replied locally;

S3	a single central master DSA is implemented for the whole AMHS world-wide, by consolidating the information held in a set of "local" DSAs (typically one per Region or per State) distributed all over the world; entry inputs and modifications are made locally to the local DSA for the concerned area. Replications from the consolidated master DSA are performed towards the local DSAs. Queries from users are submitted to the local DSAs.

Figure 2- scenario S1 for DIB Organisation

Architectures such as S2 and S3 are frequently implemented for company-wide directories. However, they raise a number of issues in the ICAO environment:

- which body operates the central DSA ? The only appropriate body holding a sufficiently central position in the aeronautical community is the ICAO Secretariat in Montreal. However, it is not the role of ICAO to implement and maintain operational systems, even for the benefit of the aeronautical community;

- on a world-wide basis, synchronisation (upload from the local DSA to the central DSA) and replication operations may require a considerable bandwidth, and generate very high communication costs, to ensure that the information held in the directory is up-to-date;

- there may be huge differences in the volume of address conversion information local to each ICAO Member State. The organisation envisaged in S2 and S3 mandates every State to operate DSAs of the same capacity. The excess cost generated by this architecture would most severely impact the States with the most limited number of AMHS users, this is clearly not logical.

Figure 3- scenario S2 for DIB Organisation

Figure 4- scenario S3 for DIB Organisation

The difficulty of implementing S1 is even more obvious: it places a highly critical requirement on the single central DSA and on the underlying network architecture (ATN or other) allowing access to it, and represents a single point of failure. The role of ICAO, which was already an issue for S2 and S3, is further emphasised: in fact, the ICAO Secretariat becomes the only implementor and operator of DSAs for the purpose of AMHS address conversion. This cannot be envisaged in the ICAO environment.

In summary, none of the scenarios envisaged to comply with case 1 under 4.3.2 appears to be feasible in the ICAO environment.

#### DIB organisation allowing address conversion in two queries

The number of queries or chaining (just one chaining = only two DSAs are questioned) for a single address conversion operation can be limited if the first DSA (i.e. the DSA which receives the query from the DUA) knows the exact location of the DSA that holds the result of the query. For this, the DIT structure must allow location of the target DSA quickly.

At this stage, an appropriate compromise (in relation with the design principles presented in section 4.3.1) needs to be defined to combine the need for a "local" management and for usage efficiency.

The implementation of one DSA in each State (or Organisation) to cover the address conversion requirements of the users in the AMHS Management Domain implemented by the State or Organisation appears to offer an adequate level of proximity of DSAs for both use and management of the Directory. In most cases, a homogeneous network infrastructure will be available to users in the considered State/Organisation to access the "local" DSA.

There is a major difference between the solution proposed here and scenarios S2 and S3 presented in section 4.3.2.1 (which also suggested one DSA per State), which is that the local DSA defined here does not include the whole world wide address conversion information.

The local DSA implemented in a State or Organisation logically contains the Directory Information for the corresponding AMHS Management Domain. Without entering the details of how this information is structured according to the DIT, this means that the DIT subtree including all entries belonging to the AMHS Management Domain is stored in this "local" DSA.

The local DSA must also be accessible from other States and Organisations: to be able to perform the conversion of an AMHS address belonging to the local AMHS Management Domain, a remote Directory user will need to query the local DSA to obtain the desired address conversion information.

On the other hand, the local DSA will be the entry point to the global ATN Directory for AMHS users in the local AMHS Management Domain. This means that the local DSA will be queried by all users in the Management Domain for any AMHS address conversion. The information needed to convert addresses belonging to the local Management Domain will be found in the local DSA, but for addresses belonging to other (remote) MDs, the appropriate DSA holding the relevant address conversion information will need to be identified.

This means that the local DSA must contain the entirety of world wide atn-AmhsMD entries (or equivalent information) so as to enable to locate the appropriate DSA for each AMHS Management Domain.

This referencing information is common to all local DSAs world wide, and could be loaded into DSAs based on information published and managed by ICAO, in any format (paper "directory of directories", CD-Rom, on-line file, or a DIT subtree of an ICAO DSA). This information would be much easier to manage by the ICAO Secretariat than the central master DSA envisaged in section 4.3.2.1, for the following reasons:

- it is essentially an extension of the ICAO Registry of AMHS Management Domains, which will be managed and published by the ICAO Secretariat;

- the amount of information to be managed is much more limited, since it represents a maximum of a few entries (maybe even only one) for each AMHS Management Domain;

- the information contained in such a directory subtree is relatively static, because it changes only when a local DSA is implemented in an ICAO member State or Organisation (or when it is reconfigured). As such, it has no "real-time" aspect.

This scenario is illustrated in scenario S4.

Figure 5- scenario S4 for DIB Organisation

In summary, the suggested DIB organisation consists in implementing a local DSA in each State or Organisation that has implemented an AMHS Management Domain, this DSA contains:

- the entirety of world wide atn-AmhsMD entries (ideally replicated from a master DSA managed by ICAO) allowing location of the appropriate DSA for each AMHS Management Domain. This subtree is similar in contents to the MD look-up table defined in the SARPs; and

- the whole AMHS Directory Information for its own AMHS Management Domain (e.g. AMHS user locations for the sake of the CAAS, including O=Region and OU1=Location Indicator values, and AMHS users) This subtree is similar in contents to the user look-up table defined in the SARPs.

Recommendation 2: when implementing ATN Directory Services in support of AMHS address conversion, ATSOs should store in a single DSA, as a minimum, the entirety of world wide atn-AmhsMD entries and the complete address conversion information for their own AMHS MD so as to optimise the response time of directory operations.

This principle DIB organisation may be subject to variations, when implemented, without compromising the views expressed here. Such variations may include for example:

- the implementation, in large countries, of several DSAs. In that case what is called above "local DSA" must be a master DSA of the various DSAs in the country, holding a consolidation of the information held in these DSAs. This allows to search only the master DSA, when querying an address from a remote country;

- the implementation of a Regional (in the ICAO sense) DSA, instead of a national DSA for the so-called "local DSA". This may happen for example when smaller countries group together to implement a single common DSA.

## Influence of the DIT

### General Principles

The Directory Information Tree (DIT) has generally a great importance in X.500 Directory Services, because it provides the tree representation of entries included in the Directory. The DIT directly defines how entries are related to each other, and how each entry is named within the Directory, by means of a directory-name (DN).

It may be noted at this stage that the name of an entry is not necessarily unique: an alias entry may be defined with another DN, but referring to the same entry.

The entry DN directly reflects the DIT hierarchy of entries above the entry. However it does not imply that all entries of the same nature (of the same object-class) obey to the same hierarchy.

Queries to the Directory are essentially based on DNs, and thus on the DIT, for read and list operations. For a search operation, the main influence of the DIT is to limit the subtree within which the operation is performed.

In general, a DIT structure may be spread over several DSAs, depending on the DIB organisation, even "vertically" (i.e. the upper part of a DIT subtree may be held by one DSA, and lower parts held in another DSA). The DIT structure hides the potential complexity of the DIB, which is only a database storage infrastructure. This may be beneficial in some organisations and for certain purposes. However, for the purpose of AMHS address conversion, it is not optimal since such a DIB organisation generates additional response times without being visible based on the DIT structure.

In view of the AMHS address conversion requirements, it is much better to envisage DIT/DIB combinations where an entire DIT subtree (from just under the root) of the ICAO ATN Directory is stored in a single DSA.

When designing a Directory Information Tree, the main elements to be taken into account are generally:

- the user-friendliness and the predictability of the DN, to ease the creation of directory queries for the user;

- the alignment of the DIT (and of DNs) along logical identification structures, such as:

- geographical structures (e.g. country> location> person),

- organisational structures (e.g. country> organisation> organisation unit (level 1)> organisation unit (level 2)> system) or

- addressing structures;

- gathering of objects which are logically grouped together into a common subtree, for ease of replication of subtrees of the DIT.

### Influence of the ATN DIT for AMHS address conversion

Concerning AMHS and ATN in general, the DIT is in principle constrained by the ICAO SARPs and Document 9705, Sub-Volume 7, as presented in 2.3.3. The ATN DIT provides the set of all possible DIT structures that could be implemented by States within their AMHS Management Domain for their AMHS users. Nevertheless section 4.1.3 points out that alternative approaches are also possible, not using the ATN DIT but using extensions to this.

Irrespective of the above constraint, the DIT retained for AMHS will have the following consequences on AMHS address conversion:

- the DIT eases (or it may make more difficult) the implementation of the DIB structure recommended in section 4.3.2.2;

- the DIT may prevent or enable the use of read operations for AMHS address conversion: if the DN of the queried entry is not easily predictable, a read operation cannot be performed because the DN cannot be provided as an input to the query;

- the DIT limits the extent of the DIB which must be queried when submitting a search operation. Thus, it has an impact on time-efficiency when a smaller or higher number of entries are searched for a given attribute value. However, if the DIT subtree to be searched is stored on a single DSA, this time impact is likely to be less significant than the time impact of chaining (or of subsequent queries) on multiple DSAs.

## Possible Directory-based address conversion solutions

It is clearly not possible to define and study all potential AMHS DIT structures that could be used for AMHS address conversion.

Based on the elements above, a set of four distinct possible Directory-based address conversion solutions are described below. Each of these options need to be adopted at ICAO level by the community of ATSOs, to provide a globally workable directory-based address conversion strategy.

They all include a DIT subtree dedicated to the representation of AMHS MDs, based on the use of atn-AmhsMD object-classes. The master DIT subtree should be considered as managed by ICAO, either really or virtually, and copies of this subtree could be held in each DSA to meet the requirement explained in section 4.4.

In the following list, the first three solutions may be seen as "SARPs-compliant" solutions, although they imply minor additions and/or amendments to the SARPs. The fourth solution is typically a "SARPs-evolution" approach, as presented in section 4.1.3.

- free AMHS address conversion Directory scheme (two variants 1a and 1b: with or without mandatory aliasing);

- simple AMHS address conversion Directory scheme;

- MD-based AMHS address conversion Directory scheme;

- O/R name-based AMHS address conversion Directory scheme.

Each of these solutions is detailed below, together with a description of the algorithms which could be used for address conversion based on such a Directory structure. The algorithms handle conversion between AFTN and AMHS addresses. They may be expanded to also handle CIDIN Ax/Ae addresses.

In each case, the DIT structure is illustrated, and examples of object entries (and of alias entries when appropriate) are provided.

Distribution lists are included in the DIT structures, for the sake of completeness. They are not represented in the examples of object entries, nor discussed in the algorithm descriptions, to simplify the discussion. In practice, an AMHS DL would be handled exactly in the same way as an AMHS user, as far as address conversion is concerned.

In accordance with section 4.3, these diagrams are based on a two-level management of the AMHS Directory:

- a central management of the ATN/AMHS Directory base, e.g. by the ICAO Secretariat;

- a "local" management by each ATSO of its "local" DSA (DSA1, DSA2 etc. in the figures) containing the sub-tree related to the ATSO ("O=NATS", "C=FR", etc. in the figures).

To do so, an ICAO Directory server should in theory be implemented, which could either include the root entry (if the ICAO ATN/AMHS Directory operates in isolation from the Global Directory), or hang somewhere in a more global DIT. However, for administrative and technical reasons, it might not be practical for ICAO to implement such a server. In such a case, ad-hoc mechanisms should be implemented in each ATSO DSA to simulate the virtual ICAO "root" entry and "central DSA".

All these solutions make the assumption that Directory is implemented at the same time as AMHS in each AMHS MD. Transition considerations will be discussed in further sections of this document.

These solutions may serve different purposes, with reference to the general model provided in Figure 1:

- they form the basis for the Distributed Address Publishing Service (APS),

- they may also be used as a replacement for gateway's Address Mapping Tables (AMT),

- depending on the "location" in the model of the Directory systems, as APS and/or AMT, the algorithms described here may serve as Address Mapping Data Import Process (AMI) only, in conjunction with operation  of the model, or also as Address Mapping Procedures (AMP).

However, one solution may be more appropriate than an other in each of these roles. This will be identified in section 5 (assessment of options).

### Option 1: Free AMHS address conversion Directory scheme

The principle of this scheme is to allow an ATSO to choose any directory scheme compliant with the general ATN DIT defined in Doc 9705, Sub-Volume 7. Each ATSO would then be able to choose the organisation of its own directory subtree. The selection of a geographical or functional organisation and the number of levels in the tree would be free, subject to maintaining compliance with the superset defined by the SARPS DIT structure.

User address information would be held in the relevant attributes of the atn-AmhsUser object-class. In support of the CAAS, an appropriate object-class should be defined to represent the ICAO location, and an appropriate attribute to include the value of the "region" attribute (X.400 organisation-name address attribute) should also be specified. Possible choices for this include atn-Facility, atn-OrganisationUnit and atn-Organisation. In the example below, the atn-OrganisationUnit is retained for this purpose.

The Directory example includes four DSAs, one foreach AMHS MD. Each DSA includes the subtree for its own organisation and the MD-registry subtree. Furthermore each DSA must include a cross-reference to each of the naming contexts identified in the MD-registry subtree.

#### Option 1 Variant 1a: without aliases

Figure 6- DIT structure for free address conversion directory scheme

Note: dotted lines correspond to additions to the ATN DIT included in Doc 9705, Sub-Volume 7

Figure 7- Example of Directory contents for free address conversion directory scheme

Note: dotted lines delimit the contents of each DSA

Associated conversion algorithm (variant 1a without aliases)

| AMHS  AFTN | AFTN  AMHS |
| --- | --- |
| - Extract the Country, ADMD and PRMD attributes from AMHS address. - Search the O=ICAO-MD-Registry subtree of the local DSA for an atn-AmhsMD entry with an atn-Global-domain-identifier attribute value matching these C, A and P attributes, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, extract the OU1 address attribute value from the AMHS address and use as AF-Address; - if value=caas, extract the CN address attribute value from the AMHS address and use as AF-Address; - if value=other, search the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value (result of step 2) for an atn-AmhsUser entry with a mhs-or-address attribute value matching the AMHS address. Extract the atn-AF-Address attribute value of that entry. | - Extract the character combinations potentially identifying an AMHS MD from the AFTN address (2, 4, 5 or 7 letters). - Search the O=ICAO-MD-Registry subtree of the local DSA for the atn-AmhsMD entry with an atn-Icao-country-code attribute value providing the best exact match with one of these character combinations, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, construct an XF address using the atn-Global-domain-identifier attribute value and the AFTN address; - if value=caas or other, search the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value (result of step 2) for an atn-AmhsUser entry with an atn-AF-address attribute value matching the AFTN address. Extract the associated mhs-or-Address of that entry. |

#### Option 1 Variant 1b: with mandatory aliases

The goal of this variant is to eliminate search operations that are needed in variant 1a for conversion of AFTN addresses into CAAS-compliant AMHS addresses.

This variant of the Free AMHS address conversion Directory scheme includes, in addition to the Directory scheme described above, a mandatory requirement to include aliases. An alias must be defined directly under the "most superior" entry of the ATSO-related subtree (i.e. the "Country" or "Organisation" entry which represents the ATSO):

- for each atn-Organisational-Unit entry representing an ICAO location, and

- for each atn-AmhsUser entry.

Such aliases are to be included in the DSAs (and DIT subtrees) corresponding to each AMHS MD.

The value of the alias is OU=location indicator (four characters) for an atn-Organisational-Unit entry and CN=AFTN address (eight characters) for an atn-AmhsUser entry.

Alias examples corresponding to the Directory contents of Figure 7 are illustrated in the figure below. The full Directory contents is then the superposition of Figures 7 and 8 (the "O=ICAO-MD-Registry" subtree is not repeated in Figure 8 because it does not need aliases).

Figure 8- Example of aliases in addition to Directory contents based on
a Free AMHS address conversion Directory scheme

Associated conversion algorithm (variant 1b with mandatory aliases)

| AMHS  AFTN | AFTN  AMHS |
| --- | --- |
| - Extract C, A and P attributes from AMHS address. - Search the O=ICAO-MD-Registry subtree of the local DSA for an atn-AmhsMD entry with an atn-Global-domain-identifier attribute value matching these C, A and P attributes, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, extract the OU1 address attribute value from the AMHS address and use as AF-Address; - if value=caas, extract the CN address attribute value from the AMHS address and use as AF-Address; - if value=other, search the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value (result of step 2) for an atn-AmhsUser entry with a mhs-or-address attribute value matching the AMHS address. Extract the atn-AF-Address attribute value of that entry. | - Extract the character combinations potentially identifying an AMHS MD from the AFTN address (2, 4, 5 or 7 letters). - Search the O=ICAO-MD-Registry subtree of the local DSA for the atn-AmhsMD entry with an atn-Icao-country-code attribute value providing the best exact match with one of these character combinations, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, construct an XF address using the atn-Global-domain-identifier attribute value and the AFTN address; - if value=caas, - construct an alias DN using the atn-AmhsMD-namingcontext value and the location indicator (4 characters) extracted from the AFTN address, - in the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value, read the atn-facility attribute value of the atn-organisational-unit entry referenced by the alias, - construct an AMHS CAAS address using the atn-global-domain-identifier (C,A,P address attributes), the atn-facility attribute value (O address attribute), the location indicator extracted from the AFTN address (OU address attribute) and the AFTN address (CN address attribute). - if value=other, - construct an alias DN using the atn-AmhsMD-namingcontext value and the AFTN address, - in the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value, read the mhs-or-address attribute value of the atn-amhsUser entry referenced by the alias. |

### Option 2: Simple AMHS address conversion Directory scheme

The principle of this scheme is to specify a simple, common directory scheme aimed at AMHS address conversion. Naming structures are aligned with the AFTN and AMHS addressing conventions. In the Directory subtree related to an AMHS MD:

- the "most superior" entry of the ATSO-related subtree (i.e. the entry which represents the ATSO) may be freely selected as a country or organisation object-class;

- an atn-Organisation entry must be included for each ICAO location, its RDN being its ICAO location indicator (4 characters as found in Doc 7910) and including in the atn-Facility attribute the value of the (region) Organisation-name AMHS address attribute. This is needed in support of the CAAS. It is sufficient if only XF and CAAS addressing structures are in use;

- an atn-AmhsUser entry must be included for each AMHS user, if the addressing scheme is neither XF nor CAAS. It may be included if a CAAS addressing structure is in use. Its RDN must be its AFTN address (8 characters).

The Directory example includes four DSAs for an equal number of AMHS MDs. Each DSA includes the subtree for its own organisation and the MD-registry subtree.

Furthermore each DSA must include a cross-reference to each of the naming contexts identified in the MD-registry subtree.

This Directory scheme allows easy construction of DNs for locations and for AMHS users, and thus it enables easy retrieval of information in the Directory by means of read operations.

Figure 9- DIT structure for simple AMHS address conversion Directory scheme

Note: dotted lines correspond to additions to the ATN DIT included in Doc 9705, Sub-Volume 7

Figure 10- Example of Directory contents for simple AMHS address conversion Directory scheme

Associated conversion algorithm

| AMHS  AFTN | AFTN  AMHS |
| --- | --- |
| - Extract C, A and P attributes from AMHS address. - Search the O=ICAO-MD-Registry subtree of the local DSA for an atn-AmhsMD entry with an atn-Global-domain-identifier attribute value matching these C, A and P attributes, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, extract the OU1 address attribute value from the AMHS address and use as AF-Address; - if value=caas, extract the CN address attribute value from the AMHS address and use as AF-Address; - if value=other, search the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value (result of step 2) for an atn-AmhsUser entry with a mhs-or-address attribute value matching the AMHS address. Extract the atn-AF-Address attribute value of that entry. | - Extract the character combinations potentially identifying an AMHS MD from the AFTN address (2, 4, 5 or 7 letters). - Search the O=ICAO-MD-Registry subtree of the local DSA for the atn-AmhsMD entry with an atn-Icao-country-code attribute value providing the best exact match with one of these character combinations, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, construct an XF address using the atn-Global-domain-identifier attribute value and the AFTN address; - if value=caas, - construct an atn-organisation entry DN using the atn-AmhsMD-namingcontext value and the location indicator (4 characters) extracted from the AFTN address, - in the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value, read the atn-facility attribute value of the atn-organisation entry identified by the DN constructed in (i), - construct an AMHS CAAS address using the atn-global-domain-identifier (C,A,P address attributes), the atn-facility attribute value (O address attribute), the location indicator extracted from the AFTN address (OU address attribute) and the AFTN address (CN address attribute). - if value=other, - construct an atn-amhsUser DN using the atn-AmhsMD-namingcontext value, the location indicator (4 characters) of the user (for O=) and the AFTN address (8 characters) of the user (for CN=), - in the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value, read the mhs-or-address attribute value of the atn-amhsUser entry identified by the DN constructed in (ii). |

### Option 3: MD-based AMHS address conversion Directory scheme

This directory scheme is similar to the simple AMHS address conversion Directory scheme defined in section 4.5.2, but with the difference that it makes use of an even more limited number of directory object-classes.

Naming structures are aligned on AFTN and AMHS addressing conventions. In the Directory subtree related to an AMHS MD:

- the "most superior" entry of the ATSO-related subtree (i.e. the entry which represents the ATSO) is an Atn-AmhsMD entry, with the same RDN as the equivalent Atn-AmhsMD entry in the ICAO-MD-Registry subtree;

- an atn-Organisation entry must be included for each ICAO location, its RDN being its ICAO location indicator (4 characters as found in Doc 7910) and including in the atn-Facility attribute the value of the (region) Organisation-name AMHS address attribute. This is needed in support of the CAAS. It is sufficient if only XF and CAAS addressing structures are in use;

- an atn-AmhsUser entry must be included for each AMHS user, if the addressing scheme is neither XF nor CAAS. It may be included if a CAAS addressing structure is in use. Its RDN must be its AFTN address (8 characters).

The Directory example includes four DSAs for an equal number of AMHS MDs. Each DSA includes the subtree for its own MD and the MD-registry subtree.

Furthermore each DSA must include a cross-reference to each of the MD subtrees identified in the MD-registry subtree.

This Directory scheme allows easy construction of DNs for locations and for AMHS users, and thus it enables easy retrieval of information in the Directory by means of read operations.

Figure 11- DIT structure for MD-based AMHS address conversion Directory scheme

Figure 12- Example of Directory contents for MD-based AMHS address conversion Directory scheme

Associated conversion algorithm

| AMHS  AFTN | AFTN  AMHS |
| --- | --- |
| - Extract C, A and P attributes from AMHS address. - Search the O=ICAO-MD-Registry subtree of the local DSA for an atn-AmhsMD entry with an atn-Global-domain-identifier attribute value matching these C, A and P attributes, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, extract the OU1 address attribute value from the AMHS address and use as AF-Address; - if value=caas, extract the CN address attribute value from the AMHS address and use as AF-Address; - if value=other, search the remote (or local) DSA containing the DIT subtree identified by the atn-Global-domain-identifier value (result of step 2) for an atn-AmhsUser entry with a mhs-or-address attribute value matching the AMHS address. Extract the atn-AF-Address attribute value of that entry. | - Extract the character combinations potentially identifying an AMHS MD from the AFTN address (2, 4, 5 or 7 letters). - Search the O=ICAO-MD-Registry subtree of the local DSA for the atn-AmhsMD entry with an atn-Icao-country-code attribute value providing the best exact match with one of these character combinations, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, construct an XF address using the atn-Global-domain-identifier attribute value and the AFTN address; - if value=caas, - construct an atn-organisation entry DN using the atn-global-domain-identifier value and the location indicator (4 characters) extracted from the AFTN address, - in the remote (or local) DSA containing the DIT subtree identified by the atn-global-domain-identifier value, read the atn-facility attribute value of the atn-organisation entry identified by the DN constructed in (i), - construct an AMHS CAAS address using the atn-global-domain-identifier (C,A,P address attributes), the atn-facility attribute value (O address attribute), the location indicator extracted from the AFTN address (OU address attribute) and the AFTN address (CN address attribute). - if value=other, - construct an atn-amhsUser DN using the atn-AmhsMD-namingcontext value, the location indicator (4 characters) of the user (for O=) and the AFTN address (8 characters) of the user (for CN=), - in the remote (or local) DSA containing the DIT subtree identified by the atn-AmhsMD-namingcontext value, read the mhs-or-address attribute value of the atn-amhsUser entry identified by the DN constructed in (ii). |

### Option 4: O/R name based AMHS address conversion Directory scheme

The principle of this scheme is to use Directory elements (object-classes, attribute types, etc.) belonging to the MHS-routing OR-address-subtree defined in ISO/IEC 10021-10 (Annex C), for the representation of AMHS user addresses. This principle applies for AMHS MDs making use of CAAS or of any other addressing scheme (distinct from XF).

In the Directory subtree of an AMHS Management Domain, the full DN of an entry corresponds to an AMHS O/R address. To include AFTN addresses in this representation, many solutions are possible. The proposal here is to include, as a leaf to the O/R address DN subtree, a new object-class called atn-AFS-Address and including always the same RDN, for example CN=AFTN, and including the AF-address as an attribute type and value.

Furthermore to avoid search operations, the use of aliases is mandated for each "O/R DN", each alias being located under the mHSPRMD object-class, and allocating the value of the AFTN address to the alias RDN.

Each DSA must include a reference to each of the MD subtrees identified in the MD-registry subtree, a MD subtree being identified by a DN identical to the atn-global-domain-identifier attribute found in the MD-registry subtree.

The Directory example includes four DSAs for an equal number of AMHS MDs. Each DSA includes the subtree for its own MD and the MD-registry subtree.

Figure 13- DIT structure for O/R name-based AMHS address conversion Directory scheme

Note 1: dotted lines and ellipses identify object-classes and structural rules that are not essential in the AMHS environment.

Note 2: apart from Organisation and atn-AmhsMD, none of the object-classes above is currently included in Document 9705 Sub-Volume 7. Suitable Structural Object-classes to create the DIT form with a name prefixed by "MHS" are defined in ISO/IEC 10021-10 (Annex C) and may be simply adopted for AMHS purposes. However, further object classes need to be created (e.g. the Atn-AFS-Address above), to hold the AFTN/CIDIN etc. addressing information together with further structural object classes to represent AFTN and CIDIN addresses.

Figure 14- Example of Directory contents for O/R name-based AMHS address conversion Directory scheme

Associated conversion algorithm

| AMHS  AFTN | AFTN  AMHS |
| --- | --- |
| - Extract C, A and P attributes from AMHS address. - Search the O=ICAO-MD-Registry subtree of the local DSA for an atn-AmhsMD entry with an atn-Global-domain-identifier attribute value matching these C, A and P attributes, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, extract the OU1 address attribute value from the AMHS address and use as AF-Address; - if value=caas or other, read in the remote (or local) DSA containing the DIT subtree identified by the 3 first MHS address attribute values, the AF-address attribute value of the atn-AFS-Address entry identified by the DN composed of the O/R address attributes (of the AMHS address to be converted) concatenated with the entry RDN (CN=AFTN). The AF-address attribute value corresponds to the desired AFTN address. | - Extract the character combinations potentially identifying an AMHS MD from the AFTN address (2, 4, 5 or 7 letters). - Search the O=ICAO-MD-Registry subtree of the local DSA for the atn-AmhsMD entry with an atn-Icao-country-code attribute value providing the best exact match with one of these character combinations, and save the entry's attribute values. - Depending on the atn-amhs-addr-scheme attribute value: - if value=xf, construct an XF address using the atn-Global-domain-identifier attribute value and the AFTN address; - if value=caas or other, - construct an alias DN using the atn-global-domain-identifier value and the AFTN address to be converted, - in the remote (or local) DSA containing the DIT subtree identified by the atn-global-domain-identifier value, read the aliasedEntryName attribute value of the alias, - construct an AMHS address using the components of the aliasedEntryName attribute as O/R address attributes. |

## Topics for further study

### Directory communication protocols

The discussion above makes the assumption that all DSAs are equally reachable in terms of protocols, both at application layer and at lower communication layers, and of connectivity. This will obviously not be true immediately during transition to AMHS and ATN Directory.

The selection of appropriate protocols and application profiles for Directory communications exceeds the scope of this work package and even the scope of SPACE.

The question of an optimised Directory topology will also arise, in conjunction with these protocol considerations. Again, this is beyond the scope of SPACE.

However, these questions do lead to the issue of X.500 vs. LDAP, which is detailed below.

### The impact of LDAP

In the beginning there was X.500 DAP - the Directory Access Protocol – defined in ASN.1, as determined by ISO and complemented with DSP and DISP for DSA to DSA communications.

Because of the apparent complexity of these protocols, mostly caused by the usual reluctance of the TCP/IP related protocols and products to adopt ASN.1 encoding, LDAP was developed by implementors and product vendors with the initial goal of simplifying the communication protocols (i.e. by using character encoding scheme). However, there are areas such as X.509 certificates that cannot be reliably character encoded, so these have been progressively integrated into LDAP products in ASN.1 encoding.

As a result of this, whilst there are many LDAP products on the market, X.500 DAP is not so common. Also, DSA products invariably support LDAP (but not all DSAs support X.500 DAP).

The ICAO SARPs, however, in line with the general ATN policy to make use of ISO OSI protocols, are based on conformance to ISO and X.500 standards.

The value of the Directory Service Protocol (DSP) has also been questioned, and although it is probably available in products, it is little used and there is not much experience with it.

The reality of today is that there now exist very many non-global directories that are accessed by LDAP.

In recent years, this has begun to change, as organisations begin to realise that for some activities (e.g. Open Security and Public Key Infrastructures, and for some truly ‘global’ users and corporations) a truly distributed/replicated and partitioned DIT is required. However, they began talking not in terms of DSP, but of ‘meta – directories’ and ‘directories of directories’, based on LDAP and proprietary protocols rather than DAP/DSP/DISP.

More precisely, it is doubtful that the LDAP market products will support true X.500 Distributed Operations (Chaining), and X.500 shadowing.

One good thing is that the Directory Schema seems to be quite stable and well adopted by current LDAP and Meta Directory systems.

The following technical questions arise:

- Can the ICAO Directory concept be adapted to LDAP and Meta-Directories?

- Are DAP and DSP necessary?

- What effect will LDAP have on the operational concept of the ICAO directory?

- How does LDAP affect distribution of and access to the address conversion data?

- Is it possible to have a common underlying protocol infrastructure for LDAP (based on TCP/IP) and AMHS?

Based on the solutions described in section 4.5, a first analysis tends to show that chaining and shadowing are not mandatory for AMHS address conversion – Referrals would suffice. Thus, the solutions proposed here could be supported using LDAP rather than truly X.500 protocols.

In summary, for directory-based address conversion envisaged in this document, the use of an LDAP Directory offers a possibly valid alternative to the currently specified ATN/AMHS Directory based on ISO OSI protocols. However, the selection of this alternative requires further study by States/Organisations interested in this option, from both an institutional perspective (diverge from ICAO ATN SARPs) and a technical perspective.

### Miscellaneous

Other aspects that would need to be studied in the framework of a specific AMHS Directory project include:

- security considerations in relation with directory access;

- cycles of updates and replications (if used);

- detailed analysis of co-existence between directory entries and attributes in support of AMHS address conversion and in support of other directory-related functions;

- relationship to distribution lists.

# Assessment of the identified options

## Criteria

A structured comparison using assessment criteria is needed to identify the relative benefits and drawbacks of the various address conversion options described in this document. However, the result should be taken with care: criteria at this stage are obviously qualitative only, and thus necessarily subjective.

The following criteria are proposed for the assessment of the address conversion options described above:

Ease of definition and acceptability

This criterion is related to the volume of changes required in ICAO Documentation, and in particular in Document 9705, for the inclusion of provisions supporting the considered option. This notion is associated with the acceptability of the option: very likely, more changes in the SARPs will make the option more difficult to accept by the ATN and AFS communities. The apparent complexity of an option also has an obvious impact on its acceptability.

Flexibility

The SARPs and ICAO ATN Documentation are not meant to be an implementation specification. Furthermore, X.500 ATN Directory Services may be used for many purposes, and not exclusively for AMHS address conversion. To that extent, flexibility may be needed to provide an optimum compromise between the address conversion requirement and other intended uses of the Directory. Since this document only deals with address conversion, flexibility in this area can be beneficial for an overall Directory design and implementation.

Implementers may also wish to have some flexibility to better optimise the design of their systems.

Ease of implementation

Although in principle the validity of such a criterion is easy to understand, it may be quite difficult to assess a given option against this criterion. The elements taken into account in this document include the number of new directory object-classes involved, the complexity of the algorithm, the risk of potential confusion generated by the design of the solution, etc.

Time efficiency

This criterion has been identified since WP221 (see requirement 5 in section 2.2) as an important element for the evaluation of address conversion solutions. Considering that the DIB organisation is identical for all directory-based solutions described in this paper, the type and number of directory operations involved in an address conversion are the main factor impacting the time efficiency of a given solution.

Ease of use

This criterion has been identified since WP221 (see requirement 4 in section 2.2) sees this as an important element for the evaluation of address conversion solutions.

It is more significant when the address conversion option is used by AMHS direct users than when it is used by systems such as AFTN/AMHS Gateways.

However, the address conversion function is usually implemented at gateways. In this case, use of the directory (or of look-up tables) is performed programmatically and ease of use then becomes a matter of ease of implementation.

Ease of management

This criterion has been identified since WP221 (see requirement 4 in section 2.2) as an important element for the evaluation of address conversion solutions.

Considering that the selected option will need to be managed on a world-wide basis by a number of separate organisations, ease of management is essential to proper working of the address conversion.

## Assessment

The criteria listed above are used in this section for assessing each of the technical options described in this document.

It should also be noted that the assessment may provide different results for a given option, depending on the considered AMHS addressing scheme. In this context, precedence is obviously given to CAAS in the assessment.

### Ease of definition and acceptability

|  | Option | Comments | Rating |
| --- | --- | --- | --- |
|  | mapping tables | This solution is certainly the most acceptable among the considered options: the CAAS has been designed specifically to enable conversion by means of mapping tables, and the solution is already included in Document 9705. There is consequently no further definition to be performed, except possibly some guidance (depending on the feedback from implementers) | ++ |
| 1a | free directory scheme (without aliasing) | The free directory scheme is very similar to the current Directory specification included in the SARPs. It consequently implies only few changes to the current Document 9705. Guidance material is likely to be needed to ensure an optimum use of such an open directory scheme. | + |
| 1b | free directory scheme (with aliasing) | Although very similar to option 1 (without aliases), this option creates a few more constraints in its definition. Use of AFTN addresses as mandatory aliases may be seen as inhibiting the evolution towards an "AFTN-free" AFS messaging environment. | 0 |
| 2 | simple directory scheme | Although quite restrictive, this option has the major benefit of being simple. Discussions held during the specification of the CAAS have demonstrated that ATSOs are generally in favour of solutions not subject to variations or interpretations. Furthermore the restrictions referred to above apply in the general context of the existing Document 9705 through refinement rather than modification. As such they do not compromise the already achieved results. | + |
| 3 | MD-based directory scheme | This solution is not far from option 2 above, although even more restrictive. Having the MD object-class located right under the directory base might also be prejudicial to the integration of the AMHS Directory into a wider ATN Directory for each ATSO. | 0 |
| 4 | O/R name-based directory scheme | This solution is entirely new in comparison to existing Document 9705. Although it is based on existing ISO/IEC standards, it would require significant work for the inclusion into ICAO ATN documentation, whilst the current Document 9705 is currently considered to include appropriate material for address conversion (even if some refinement is needed): it might be not straightforward to obtain a consensus about the need for such major additions to the Directory SARPs. | - |

### Flexibility

This criterion offers very distinct results for the various studied options, because of their definition. The notion of flexibility relates here mostly to the DIT and to the DN forms.

|  | Option | Comments | Rating |
| --- | --- | --- | --- |
|  | mapping tables | restricted to enable algorithmic conversion by AMP | - |
| 1a | free directory scheme (without aliasing) | obviously the most flexible option as far as ATSO are concerned | ++ |
| 1b | free directory scheme (with aliasing) | flexibility is slightly lowered in comparison to option 1a due to the mandatory use of aliases | + |
| 2 | simple directory scheme | the only level of flexibility resides in the object-class used right under the directory base | - |
| 3 | MD-based directory scheme | this scheme is similar to option 2 but it does not even provide the minimum flexibility offered for the object-class at the top of the subtree | -- |
| 4 | O/R name-based directory scheme | this scheme offers flexibility for AMHS addresses, which are not restricted, but directory names are constrained to be identical to O/R addresses. Directory names consequently do not provide naming flexibility, they only benefit from the O/R address form flexibility, when the latter is left open. However this is not the case for AMHS where fixed schemes (CAAS and XF) are preferred. | - |

### Ease of implementation

This criterion is probably the most subjective and the most difficult to assess among the set of criteria used in this section.

|  | Option | Comments | Rating |
| --- | --- | --- | --- |
|  | mapping tables | As an entirely local mechanism, the use of mapping tables appears to be the easiest to implement | + |
| 1a | free directory scheme (without aliasing) | Because of its flexibility, and despite the relative simplicity of the algorithm, this solution is probably more complex to implement than option 2 among the directory-based options | - |
| 1b | free directory scheme (with aliasing) | This solution is similar to option 1a, although the additional use of aliases makes it somewhat more complex | -- |
| 2 | simple directory scheme | This option appears to be comparatively the most easy to implement of the directory-based options: although its description is long, the algorithm is particularly simple for CAAS and other AMHS addresses | 0 |
| 3 | MD-based directory scheme | This solution is similar to option 2; nevertheless the further restriction in the DIT and the use of identical OCs (and RDNs) in two distinct subtrees implies special care when implementing the solution. | - |
| 4 | O/R name-based directory scheme | This option is probably the most complex to implement: from a strict directory viewpoint, it makes use of the highest number of specific (i.e. non X.521) object-classes, not taking into account the addition of atn-specific OCs which create a deviation from the base principle of ISO/IEC 10021-10. The use of aliases also contributes to its complexity. | -- |

### Time efficiency

This criterion highlights that the use of mapping tables is far more time-efficient, because the whole address conversion information is stored locally in the considered system. However, the comparison between a distributed solution such as any directory-based address conversion and a local solution is obviously biased.

The table below should consequently be considered more as a comparison between directory-based options, or only in combination with other criteria (see the "summary" section below) to provide an overall comparative view between "local" and "distributed" solutions.

|  | Option | Comments | Rating |
| --- | --- | --- | --- |
|  | mapping tables | This option is without doubt the most efficient from a performance time viewpoint, because the whole address conversion information is stored locally in the considered system. | ++ |
| 1a | free directory scheme (without aliasing) | Among the directory-based options, this is certainly the least time-efficient one, because of the use of search operations. | -- |
| 1b | free directory scheme (with aliasing) | The use of aliases allows read operations rather than search operation as in option 1a. Variations with options 2 and 3 (due to alias de-referencing) are too minimal to be significant here. | 0 |
| 2 | simple directory scheme | The use of simple DNs allows reading into the directory, rather than searching as in option 1a. | 0 |
| 3 | MD-based directory scheme | The use of simple DNs allows reading into the directory, rather than searching as in option 1a. | 0 |
| 4 | O/R name-based directory scheme | The use of DNs aligned on O/R addresses, combined with the use of aliases, allows an optimum use (from a time perspective) of the tree-shaped directory information. This option is also the only one which allows conversion of a non-CAAS AMHS address into an AFTN address without a search operation. | + |

### Ease of use

|  | Option | Comments | Rating |
| --- | --- | --- | --- |
|  | mapping tables | This solution is clearly not designed for access by direct users, but only for retrieval of information programmatically by a system such as a gateway. As such, it is the least convenient for use and it may imply the definition and implementation of other mechanisms for direct users. | -- |
| 1a | free directory scheme (without aliasing) | The need to search the directory is less convenient for use than solutions based on read operations. | - |
| 1b | free directory scheme (with aliasing) | The implementation of aliases enhances the ease of use of the directory in comparison to option 1a. Nevertheless, at the same time such aliases hide the actual DIT structure and this may be a drawback in some cases (in particular when comparing to solutions 2, 3 and 4). | 0 |
| 2 | simple directory scheme | This solution is among the most easy to use, thanks to the simple DIT and DN structure which is adopted. | + |
| 3 | MD-based directory scheme | This solution is among the most easy to use, thanks to the simple DIT and DN structure which is adopted. | + |
| 4 | O/R name-based directory scheme | Although somewhat more complex than options 2 and 3, this solution is among the most easy to use, thanks to aliases and alignment on O/R address structures. | + |

### Ease of management

|  | Option | Comments | Rating |
| --- | --- | --- | --- |
|  | mapping tables | This solution is the most complex to manage because in does not include any inherent mechanism for distribution of information. | -- |
| 1a | free directory scheme (without aliasing) | This option is a "standard" solution in terms of directory management: neither is it specially easy (few DIT restrictions) nor specially difficult (no alias nor specific mechanism). | 0 |
| 1b | free directory scheme (with aliasing) | The use of aliases always generates complexity in directory management, to ensure that aliases are always defined, and to ensure consistency of references between aliases and actual entries. | - |
| 2 | simple directory scheme | This solution is the easiest directory-based option in terms of management: simple DIT, no aliases. | ++ |
| 3 | MD-based directory scheme | Like option 2, this solution is also easy to manage, for the same reasons: simple DIT, no aliases. A drawback in comparison to option 2 is the duplicate use of atn-AmhsMD entries that may potentially cause some confusion. | + |
| 4 | O/R name-based directory scheme | The use of aliases always generates complexity in directory management, to ensure that aliases are always defined, and to ensure consistency of references between aliases and actual entries. | - |

## Summary

The following table provides a summary of the assessment above.

|  | Option | Ease of defini-tion | Flexibility | Ease of implem-entation | Time effi-ciency | Ease of use | Ease of manag-ement |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | mapping tables | ++ | - | + | ++ | -- | -- |
| 1a | free directory scheme (without aliasing) | + | ++ | - | -- | - | 0 |
| 1b | free directory scheme (with aliasing) | 0 | + | -- | 0 | 0 | - |
| 2 | simple directory scheme | + | - | 0 | 0 | + | ++ |
| 3 | MD-based directory scheme | 0 | -- | - | 0 | + | + |
| 4 | O/R name-based directory scheme | - | - | -- | + | + | - |

From the assessment above, it results in relation with the general model that:

- mapping tables and the O/R name-based directory scheme appear to be more appropriate for use as gateway's address mapping tables (AMT), because of their time efficiency,

- any Directory-based address conversion will considerably ease the overall address conversion process by enabling the implementation of the Distributed address publishing service (APS),

- the simple directory scheme and the MD-based directory scheme are more appropriate for the Distributed APS, based on ease of specification, use and management. Among these preference should be given to the simple directory scheme (DIT option 2),

- mapping tables are obviously the only valid solution for ATSOs with no implementation of, nor access to, the Distributed address publishing service (APS).

# Combination of alternative techniques through transition

## General

For the purpose of study, the analysis and assessment above were made on the assumption that all ATSOs implement the same technique and proceed at the same speed in the implementation of AMHS address conversion.

However, the reality of implementation is obviously different, and it is important to study the combination of the alternative envisaged techniques to ensure that the overall model presented in Figure 1 operates in a consistent manner. This is also true from a transitional perspective, where an ATSO may evolve e.g. from Basic Service to Extended Service and change the technique used for address conversion.

The combination of alternative techniques is illustrated in Figure 1, where ATSO3 does not make use of the Distributed APS (Directory) while ATSO1 and ATSO2 use it.

ATSO3 in such a case makes use of the information published by ICAO e.g. twice a year, to populate and update mapping tables in accordance with the techniques described in Section 3. The ICAO information includes the necessary elements for the support of XF and CAAS addressing schemes by any ATSO in the world.

The questions arising for ATSO1 (and ATSO2), and relevant to combination of techniques, belong to two distinct areas:

- what are the options for the ATSO1-internal co-existence of a DSA for the Distributed APS and of Gateway's AMT ?

- how is the address conversion information external to ATSO1 (and ATSO2) handled in the ATSO1 DSA and Gateway ?

## Co-existence within an ATSO of the APS and of Gateway's AMT

Internally to an ATSO implementing the Distributed APS (e.g. ATSO1 in the model), three main options are available for the co-existence of the DSA for the Distributed APS and of the Gateway's AMT:

- use separate mechanisms in the gateway and in the DSA, such as (locally defined) mapping tables for AMT and directory services for APS based on the simple AMHS address conversion directory scheme;

- make use of a single directory system for both purposes, thereby "collapsing" the directory system into the gateway, using one directory scheme for AMT (e.g. the O/R name-based directory scheme) and another for the distributed APS for access by other ATSOs;

- make use of a single (integrated) directory system with a single directory scheme (DIT?) for both purposes (the simple AMHS address conversion directory scheme which is recommended for external use), again "collapsing" the directory system into the gateway.

Using a single directory system for both purposes has the major benefit of avoiding use of multiple distinct table/database technologies within a system that can be integrated. However the ultimate decision is an implementation matter for which each ATSO is free to make its own choices. Implementors considering this approach should also be aware of the potential impact of query traffic generated by read and search operations that may be initiated by other ATSOs, and the potential security risks that such a configuration would introduce.

## Handling of address conversion information external to the ATSO

In ATSO1, having implemented a DSA for the purpose of the Distributed APS, there is a need to take into account of ATSOs such as ATSO3 which have not yet proceeded towards the Distributed APS. More specifically the address conversion information related to ATSO3 needs to be handled in the ATSO1's Gateway and DSA.

The minimum is that the gateway's AMT needs to include ATSO3’s information.

The main options are:

- input ATSO3's information directly into AMT, based on ICAO documentation, and use the Distributed APS only for more dynamic information related exclusively to ATSOs such as ATSO1 and ATSO2. This is logical only when option 1 in section 6.2 has been retained;

- input ATSO3's information into ATSO1's DSA, by means of a specific sub-tree dedicated to ATSO3. This sub-tree may then be used by ATSO1 (and other ATSOs such as ATSO2) for address conversion purposes. It is more logical when option 2 in section 6.2 has been retained, and it becomes mandatory when option 3 has been selected.

There is nothing to prevent an ATSO from entering information related to other ATSOs in its own DSA. However, it is highly preferable in such a case that a common pre-determined (and inflexible) directory scheme, such as the simple AMHS address conversion directory scheme, be used and adopted on a global basis.

If the scheme is pre-determined and leads to predictable directory names, then ATSO1 may enter in its own DSA information regarding ATSO3 that remains valid when ATSO3 also implements a DSA. The ATSO1 DSA may then be seen as a replication of ATSO3's DSA, which would be created before the original! Conversely, if the scheme implemented in ATSO1's DSA for ATSO3 offers flexibility, then the whole sub-tree represented ATSO3 in ATSO1's DSA will need to be replaced when ATSO3 implements a DSA with other directory names.

Obviously, this whole process will be much more efficient if it is co-ordinated between ATSOs implementing AMHS. The minimum set of participants to such co-ordination is the set of ATSOs belonging to a common AMHS island.

The main conclusion to be made here is that there is nothing to prevent co-existence of the alternative envisaged AMHS address conversion techniques (mapping tables + directory-based conversion in accordance with one of the options studied in this paper). This ensures that the overall model presented in Figure 1 can operate in a consistent manner through transition periods, where each ATSO proceeds to AMHS and Directory Services implementation at its own speed.

# Conclusions and recommendations

## Recommended strategy

The recommended AMHS address conversion strategy aims at ensuring a coherent and efficient method for conversion between AFTN/CIDIN addresses and AMHS addresses that comply with:

- the Common AMHS Addressing Scheme (CAAS) included in ICAO Document 9705, Sub-Volume 3, Edition 3, and already recommended and adopted by SPACE in the course of WP113,

- the XF-Addressing Scheme included in ICAO Document 9705, Sub-Volume 3, since its 1st Edition.

Additionally, the conversion strategy also enables conversion AFTN/CIDIN addresses and AMHS addresses belonging to an AMHS MD having implemented a locally defined AMHS addressing scheme.

The recommended AMHS address conversion strategy is the means by which the general model represented in Figure 1, section 2.7, could be realised by States and the EUR Region.

In summary, the recommended strategy is made of the following elements:

- the establishment, at ICAO Secretariat level, of an ICAO Registration and Publication process as a set of procedures for collecting and publishing AMHS address conversion information on a periodic basis (e.g. twice yearly). This will include:

- the MD information included in the ICAO Registry of AMHS Management Domains, specifying the type of implemented addressing scheme,

- for those MDs having implemented the Common AMHS Addressing Scheme (CAAS), the mapping information providing the organisation-name address attribute for each ICAO location indicator,

- for those MDs having implemented a locally defined AMHS addressing scheme, the mapping information providing the full O/R address for each AFTN addressee indicator;

- a Distributed Address Publishing Service (APS), based on ATN Directory Services, that allows publication of real-time AMHS address conversion information. This is to be implemented at the earliest opportunity upon ATSOs' initiative, with the following principles:

- use of the directory scheme described in section 4.5.2 of this document (called "simple AMHS address conversion directory scheme");

- initial population of the Directory Information Base with the information distributed through the ICAO Registration and Publication process;

- implementation of a single DSA per ATSO to hold the MD Registry sub-tree, the local AMHS MD address conversion information sub-tree, the world-wide CAAS information distributed through the ICAO Registration and Publication process, and the local AMHS MD address conversion information sub-tree comprising:

- the local MD CAAS information, if the ATSO has implemented the CAAS; or

- the full mapping information providing the full O/R address for each AFTN addressee indicator, if the ATSO has not followed the Doc 9705 recommendations and has implemented a locally defined addressing scheme;

and

- in co-existence with the use of Address Mapping Tables (AMT) directly derived from the information published through the ICAO Registration and Publication process, for ATSOs that choose to defer the implementation of ATN Directory Services.

As a local implementation matter, ATSOs that envisage implementation of Directory Services for the purpose of the Distributed address publication service (APS) at the same time as they implement AMHS, should also consider the use of directory solutions as a technical option for the gateway's address mapping tables (AMT).

Item 1a above has been requested to ICAO through the ATN Panel and it is an outstanding activity of the ICAO Secretariat. The recommendation to complement the action in progress with items 1b and 1c will be submitted to the ATN Panel on behalf of SPACE.

## Additional conclusions

The strategy above can be complemented by the following transitional provisions and additional elements that also arise from the study of the AMHS address conversion strategy which has been carried out in this document.

### Transitional provisions

For the  case that item 1 in the above strategy is not implemented by ICAO in a timeframe compatible with early AMHS implementations, an equivalent process may be set up on an ad-hoc basis between all ATSOs forming an AMHS island. This is particularly applicable to SPSOs and other potential European ATSOs being early AMHS implementers. For SPSOs the necessary information is already available in the SPACE WP113 database.

For the case of ATSOs following item 2 in the above strategy that prefer initially to group together for the implementation of a single ICAO Regional DSA, the latter should hold:

- the MD Registry sub-tree,

- a local AMHS MD CAAS information sub-tree for each of the ATSOs in the group,

- and the world-wide CAAS information distributed through the ICAO Registration and Publication process,

The Regional DSA thereby becomes an aggregation of the local DSAs envisaged in the principle strategy.

In the EUR Region, WP234 recommended the creation of an Offline Management Centre to consolidate co-ordinate and distribute AMHS user address changes across the region. This Offline Management Centre should implement such a Regional DSA in support of its address management activities.

### Additional recommendations

The following recommendations are extracted from the body of this document or derived from the considerations that it includes.

Recommendation 1: The ATNP should examine the possibility of including in Document 9705, Sub-Volume 7, one or several atn-specific directory object-classes and attribute-types for the representation of ICAO locations and of AMHS regions, in support of the Common AMHS Addressing Scheme (CAAS). Alternatively an existing object-class such as atn-organization could be specified for this purpose.

It should be noted that the simple AMHS address conversion directory scheme, included in item 2a) of the recommended strategy above, gives preference to the use of the atn-organization object-class.

Recommendation 2: When implementing ATN Directory Services in support of AMHS address conversion, ATSOs should store in a single DSA, as a minimum, the entirety of world wide atn-AmhsMD entries and the complete address conversion information for their own AMHS MD so as to optimise the response time of directory operations.

Recommendation 3: The ATNP should examine the possibility of including in ICAO Documents 9705 and/or 9739, a single ICAO directory naming scheme for the purpose of AMHS address conversion, in accordance with directory scheme described in section 4.5.2 of this document (called "simple AMHS address conversion directory scheme").

Recommendation 4: The ATNP should request the ICAO Secretariat to expand the scope of the procedure foreseen for the Management of the ICAO Registry of AMHS MDs, to cover address conversion information (at least CAAS, and local if needed).

# Appendix A: ATN-specific Directory object-classes and attribute-types in support of AMHS

Text in italics is additional or modified in comparison with Document 9705 Sub-Volume 7.

## Object-classes description

- atn-AmhsMD		OBJECT-CLASS	::=
	SUBCLASS OF	{ top }
	MUST CONTAIN	{ CommonName
			   atn-global-domain-identifier,
			   atn-icao-country-code*, 
			   atn-Amhs-addressing-scheme }
	MAY CONTAIN	{ atn-AmhsMD-namingcontext }
	ID		{ id-atn-oc-amhsMD }

- atn-AmhsUser		OBJECT-CLASS	::=
	SUBCLASS OF	{ mhs-User }
	MUST CONTAIN	{ atn-amhs-extended-service-support,
			   atn-amhs-direct-access}
	MAY CONTAIN	{ atn-PerCertificate,
			   atn-DerCertificate,
			   atn-AF-address,
			   atn-Cidin-mcf,
			   atn-Ax-or-primary-Ax-address,
			   atn-secondary-Ax-address }
	ID		{ id-atn-oc-AmhsUser }

- atn-AmhsDistributionList		OBJECT-CLASS	::=
	SUBCLASS OF	{ mhs-distributionList }
	MUST CONTAIN	{ atn-amhs-extended-service-support }
	MAY CONTAIN	{ atn-PerCertificate,
			   atn-DerCertificate,
			   atn-AF-address,
			   atn-Cidin-mcf,
			   atn-Ax-or-primary-Ax-address }
	ID		{ id-atn-oc-AmhsDistributionList }

- atn-Organization		OBJECT-CLASS	::=
	SUBCLASS OF	{ organization }
	MUST CONTAIN	{ atn-facilityName }
	MAY CONTAIN	{ atn-PerCertificate, 
			   atn-DerCertificate, 
			   atn-version }
	ID		{ id-atn-oc-Organization }

- atn-OrganizationalUnit 	OBJECT-CLASS	::=
	SUBCLASS OF	{ organizationalUnit }
	MUST CONTAIN	{ }
	MAY CONTAIN	{ atn-PerCertificate, 
			   atn-DerCertificate, 
			   atn-facilityName,
			   atn-version }
	ID		{ id-atn-oc-OrganizationalUnit }

## Attribute-type descriptions

- atn-global-domain-identifier	ATTRIBUTE	::=	{
		WITH SYNTAX	GlobalDomainIdentifier
		ID			id-atn-at-amhs-global-						domain-identifier}

- GlobalDomainIdentifier	::=	SEQUENCE {
	country-name			CountryName,
	administration-domain-name	AdministrationDomainName
	private-domain-name		PrivateDomainName }

- CountryName		::=	CHOICE {
	x121-dcc-code		NumericString(SIZE(ub-country-name-				numeric-length)),
	iso-3166-alpha2-code	PrintableString(SIZE(ub-country-name-alpha-			length))}

- AdministrationDomainName	::=	CHOICE {
	numeric		NumericString(SIZE(0..ub-domain-name-			length)),
	printable		PrintableString(SIZE(1..ub-domain-name-			length))}

- PrivateDomainName	::=	CHOICE {
	numeric 		NumericString(SIZE(1..ub-domain-name-			length)),
	printable	::=	PrintableString(SIZE(1..ub-domain-name-			length))}

- atn-icao-country-code	ATTRIBUTE	::=	{
		WITH SYNTAX		PrintableString(SIZE(1..7))
		ID			id-atn-at-icao-country-code}

- Atn-Amhs-addressing-scheme 	ATTRIBUTE	::=	
		WITH SYNTAX		INTEGER {
						xf (1),
						caas (2),
						other (3) }
		SINGLE VALUE	TRUE
		ID			id-atn-at-Amhs-addressing-scheme}

- Atn-AmhsMD-namingcontext	ATTRIBUTE	::=	{
		WITH SYNTAX	PrintableString(SIZE(1..64))
		ID			id-atn-at-AmhsMD-						namingcontext}

- Atn-amhs-extended-service-support	ATTRIBUTE	::=	{
		WITH SYNTAX	BOOLEAN
		ID			id-atn-at-amhs-extended-					service-support}

- atn-amhs-direct-access	ATTRIBUTE	::=	{
		WITH SYNTAX	BOOLEAN
		ID			id-atn-at-amhs-direct-access}

- atn-PerCertificate		ATTRIBUTE	::=	{
		WITH SYNTAX	{atnCertificate
		ID			id-atn-at-PerCertificate}

- atn-DerCertificate		ATTRIBUTE	::=	{
		WITH SYNTAX	{atnCertificate
		ID			id-atn-at-DerCertificate}

- atn-AF-address		ATTRIBUTE	::=	{
		WITH SYNTAX	PrintableString(SIZE(8))
		SINGLE VALUE	TRUE
		ID			id-atn-at-AF-address}

- atn-Cidin-mcf		ATTRIBUTE	::=	{
		WITH SYNTAX	INTEGER {
						opmsg (1),
						aftn (2),
						opmet (3) }
		SINGLE VALUE	TRUE
		ID			id-atn-at-Cidin-mcf}

- atn-Ax-or-primary-Ax-address	ATTRIBUTE	::=	{
		WITH SYNTAX	PrintableString(SIZE(5..8))
		SINGLE VALUE	TRUE
		ID			id-atn-at-Ax-or-primary-Ax-					address}

- atn-secondary-Ax-address	ATTRIBUTE	::=	{
		WITH SYNTAX	PrintableString(SIZE(5..8))
		SINGLE VALUE	TRUE
		ID			id-atn-at-secondary-Ax-					address}

- atn-facilityName		ATTRIBUTE	::=	{
		WITH SYNTAX	PrintableString(SIZE(3..8))
		ID			id-atn-at-facilityName}

- atn-Version		ATTRIBUTE	::=	{
		WITH SYNTAX	INTEGER
		ID			id-atn-at-version}

# Appendix B: Fully populated MD look-up table

[to be inserted]

In this document, the terms "AMHS address" and "MF-address" are used without distinction.

CIDIN to AMHS address conversion is needed only for the OPMET and OPMSG CIDIN applications, which have a smaller number of users and generate less traffic than the AFTN application. For the AFTN application of the CIDIN, the AFTN address (Ad) - and not the entry/exit addresses (Ae/Ax) - is converted to/from an AMHS address.

The CAAS is published in Edition 3 of ICAO Document 9705. However, it should be noted that it has been inserted in ICAO Document 9705 by means of a PDR (Potential Defect Report) which also applies to Edition 2. Thus, implementations conforming to Edition 2 should also take into account the specification of the CAAS.

This requirement has no longer the same strength following discussions within the ATNP on CAAS (see the discussion at the end of para 2.1 and Recommendation 1 below).

currently existing as a Draft Edition 2 available at usual ATNP archiving locations.

In case of update resulting from a change, WP221/Recommendation 4 states that old forms should be supported during 6 months from starting from applicability of the new form. The change implicitly means that support of the old form cannot be assumed 6 months hence.

The example below is an excerpt of the fully populated table provided as Appendix B in this document.

"unique" means here that there should be one single atn-AmhsUser entry in the overall ATN Directory including such an attribute value.

"unique" means here that there should be one single entry in the overall OR-address subtree of the ATN Directory corresponding to this MF-address.

"unique" means here that there should be one single entry in the overall OR-address subtree of the ATN Directory including such an attribute value.

"Local" consequently means here either "national" or "local to an organisation".

This is related only to the minimal number  which would not follow the ICAO Recommendations on MF-addressing schemes included in Doc 9705. Clause 3.1.2.1.5.1.4.1.2 of Doc 9705, Ed3, recommends that "An AMHS Management Domain should avoid deviating from the Common AMHS Addressing Scheme and refrain from implementing a locally defined AMHS addressing scheme unless specific (e.g. regulatory) unavoidable constraints apply to the AMHS Management Domain."

*	 The name may need to be replaced e.g. with icao-entity-identifier to also cover organisations.

	 The definition of this attribute in  Doc 9705 Sub-Volume VII, specifies PrintableString(SIZE(1..19))