ATNP/WG A/SG A3

WP/6-5

16/07/02

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL(ATNP)

Working Group A - Applications and implementation

SUBGROUP A3 – GROUND-GROUND Applications

Toulouse, 2nd October – 3rd October 2002 (sixth meeting)

Agenda Item 3.2 : ATS Message Handling Services / 
Guidance Material on AMHS addressing and address conversion

Pro Formas for AMHS Address management

Presented by Jean-Marc Vacher

Summary

The specification of the CAAS and the notion of ICAO Register of AMHS Management Domains have both been introduced in Edition 3 of Document 9705.

The ICAO AMHS MD and addressing information registration and publication process now needs to be implemented.

The goal of this WP is to provide a set of procedures that the ICAO Secretariat could use to gather AMHS MD and addressing information from its member States, and to publish this information to all States for the purpose of AMHS deployment. Such procedures were specifically requested by the ATN Panel Secretary in the 4th SG A3 meeting. The procedures proposed in this paper were discussed on the basis of a first draft presented during the 5th SGA3 meeting (Brussels, July 2002).

This WP incorporates the agreed changes and additions discussed in Brussels.

TABLES OF CONTENTS

1.	Introduction	3

2.	Requirements	3

3.	Development of the required pro formas	4

3.1	Draft of the State letter to be sent by ICAO to its Contracting States	4

3.2	Draft form to be sent by Contracting States to the ICAO Secretariat	4

3.3	Draft Register of AMHS MDs and addressing information	4

4.	Recommendations to the meeting	4

Appendix A - Draft State Letter from the ICAO Secretariat	5

Introduction	5

Allocation of an AMHS MD identifier to your State	5

Declaration of AMHS addressing schemes	6

Appendix B: Draft Reply Form [blank]	7

Appendix C - Draft ICAO Register of AMHS Management Domains and Addressing Schemes	9

## Introduction

To ease the implementation of AMHS and the transition from the current AFS (AFTN/CIDIN messaging) environment, a common unique worldwide addressing scheme has been agreed by the subgroup in its 2nd and 3rd meetings. This addressing scheme is called "Common AMHS Addressing Scheme" or CAAS. At the same time, the notion of an ICAO Register of AMHS Management Domains has also been recognized as a mandatory requirement for efficient AMHS operation on a world wide basis.

The specification of the CAAS and the notion of ICAO Register of AMHS Management Domains have both been introduced in Edition 3 of Document 9705.

The ICAO AMHS MD and addressing information registration and publication process now needs to be implemented.

The goal of this WP is to provide a set of procedures that the ICAO Secretariat could use to gather AMHS MD and addressing information from its member States, and to publish this information to all States for the purpose of AMHS deployment. Such procedures were specifically requested by the ATN Panel Secretary in the 4th SG A3 meeting.

The procedures proposed in this paper were discussed on the basis of a first draft presented during the 5th SGA3 meeting (Brussels, July 2002). This WP incorporates the changes and additions agreed in Brussels.

## Requirements

Three different pro formas are required by the ICAO Secretariat :

- the draft of a State letter, to be sent by ICAO to its Contracting States, to inform them that AMHS address registration and publication is being established, to inform them about the values (eg. PRMD-name) reserved for them and to invite them to publish officially their intended address information. This can be expanded as follows :

- to inform them that the Register of AMHS Management Domains is being established,

- to inform them that a PRMD name made of their Nationality Letters has been reserved to their State ("default" value as specified in 3.1.2.1.5.1.3.4),

- to invite them, if they wish to do so, to request modification of the PRMD-name value into another proposed value, subject to validation of the proposed value by ICAO,

- to recommend that they use the CAAS (Common AMHS Addressing Scheme),

- to invite them to provide CAAS (Common AMHS Addressing Scheme) address information,

- to inform them that if no CAAS address information is provided, XF-addressing will be assumed for their State;

- the draft of a standard form that States could use in reply to the State letter above, to provide the requested address information for publication. This can be refined in the following goals :

- to declare a PRMD-name value chosen by themselves (not the "default" value),

- to provide CAAS address information;

- a draft structure for tables forming the Register of AMHS MDs and the AMHS Address publication service, that could be published by the Secretariat on the ICAO web site.

## Development of the required pro formas

### Draft of the State letter to be sent by ICAO to its Contracting States

A proposal for this draft State letter is provided in Appendix A.

### Draft form to be sent by Contracting States to the ICAO Secretariat

A blank form should be attached to the State letter. It should in fact be partly pre-filled by the ICAO Secretariat, and partly left blank for provision of information by the State.

A proposal for this draft form is provided in Appendix B.

### Draft Register of AMHS MDs and addressing information

Version 0.1 of the proposed Register is attached as an Appendix to this paper.

The following principles agreed in Brussels have been applied:

- The asterisk * is used as a single character wild card;

- For States with multiple pairs of Nationality letters, the rule to define the default PRMD-name is to take the first pair of Nationality letters in alphabetical order, when the first letter is identical. When the first letter of the different pairs are different (this corresponds to locations in different Regions), distinct default values are maintained. Concerned States may of course take the initiative of registering another proposed value than the default value;

- The Nationality letters (or the equivalent designator) include always a minimum of two characters. For States such as Australia, Canada or the U.S. which are unambiguously identified using one letter, the “one letter + wild card” combination is used as the Nationality letters or designator value in the Register;

- For States sharing the same Nationality letters (e.g. “FM”, “TN” or “UT”), a combination of four characters (three letters + wild card) is used as the Nationality letters or designator value in the Register, to comply with the SARPs specification of 2, 4, 5 or 7 characters;

- Concerning the detailed CAAS information, the published tables reflect the hierarchy of AMHS addresses, i.e. they show only one table with the Organisation-name column to the left of the organization-unit-name (location indicators) column. The reverse column is needed and could be confusing.

## Recommendations to the meeting

The meeting is invited to comment as appropriate the proposed pro formas for AMHS address management included in this paper, and endorse them for provision to ICAO as the base procedures for AMHS MD and addressing information Registration and Publication.

It is expected that ICAO will edit these procedures as appropriate to make them fit the internal ICAO rules and practices for such documents.

## Appendix A - Draft State Letter from the ICAO Secretariat

[Dear Sirs,]

### Introduction

The Aeronautical Telecommunication Network (ATN), and its ground ATN application named “ATS Message Handling Services” (ATSMHS), have been standardized by ICAO as it may be found in Annex 10, Volume III, Part 1, Chapter 3 and Document 9705. The ATSMHS application has been defined as the communication standard aimed at replacing the AFTN and CIDIN systems and procedures. To enable the world wide implementation of the ATSMHS, and its interoperation with the existing AFTN/CIDIN systems, the ICAO Secretariat is now in the process of establishing the Register of ATS Message Handling Systems (AMHS) Management Domains (MDs), as per Recommendation 5/3 of the ATNP/3 meeting.

The goal of this Register is to facilitate the establishment of an addressing scheme as uniform as possible across ICAO and its Contracting States, and to enable Contracting States and Air Traffic Service Organisations to operate under the “ICAO” Administrative Management Domain (ADMD) name that has been registered at International Telecommunications Union – Telecom Standards (ITU-T) for ICAO as an international organisation.

This Register will maintain a list of all Management Domains contributing to the ATS Message Handling System (AMHS), and of the MD identifiers allocated to each ICAO Contracting State, or Air Traffic Service Organisation (ATSO) operating under the aegis of ICAO, for the implementation of an AMHS Management Domain. The Register will include an entry for each State or ATSO.

The ICAO Register of AMHS MDs will be published on the ICAO web site with an applicability date of [July, 1st, 2003, date to be confirmed by the Secretariat]. The Registration procedure is attached to this letter.

### Allocation of an AMHS MD identifier to your State

The allocation of a MD identifier is a preliminary registration that does not mandate the simultaneous implementation of an AMHS MD, which may take place at the date that better suits the plans of “your State” and of the Planning and Implementation Regional Group in your Region. This MD identifier takes the form of a Private Management Domain (PRMD) name. It needs to be unique in the ICAO ADMD.

In application of Document 9705, a PRMD name has been reserved for your State. This PRMD name is [state nationality letters].

The PRMD name can be modified upon your request, if you wish to propose another value for registration, using the attached reply form. Such a proposed value must comply with the specifications of Document 9705, Edition 3, [section 3.1.2.1.5.1.3]. Guidance to select a proposed value can be found in Document 9739, Edition 2[, Part III, section 6.2.1.3]. The ICAO Secretariat reserves the right to reject the modification request, or to further co-ordinate the registration with your State, in case of non-conformance or non-unicity of the proposed value.

The full AMHS MD identifier for the AMHS MD operated in your State, when implemented, will therefore be either:

C=XX / A=ICAO / P=[nationality letters]  
or
C=XX / A=ICAO / P=[your proposed value, if valid]

This MD identifier will be associated with the nationality letters currently allocated to your State, as per Document 7910.

For the publication of the Register applicable on [July, 1st, 2003], modification requests should be received at the ICAO Secretariat on [April, 1st, 2003; 3 months before the publication date] at the latest. In case of no reply being received by the Secretariat at this date, reserved values will be deemed accepted by your State and will be published accordingly. [rule to apply in case of “no reply” to be aligned on standard ICAO practices]

### Declaration of AMHS addressing schemes

The ICAO Register of AMHS MDs must also include the declaration of the addressing scheme under which AMHS user addresses will be allocated in each AMHS MD.

You are therefore invited to declare the addressing scheme which will be applicable to the AMHS MD “in your State”. The second part of the attached reply form should be used for this purpose.

The addressing scheme called the “Common AMHS Addressing Scheme” (CAAS) and specified in Document 9705, Edition 3, is recommended as soon as an AMHS implementation plan is established in your State.

This addressing scheme allows to group the (existing or future) AMHS user locations of your State, identified by their ICAO 4-character location indicator, as specified in Document 7910, into geographical units or more generally into groups of locations using the same (existing or future) ATS Message Server. Guidance and examples on how to define such groups of locations may be found in Document 9739, Edition 2.

The detailed addressing information associating a group of location indicators to an organisation-name identifying this group must be declared to the ICAO Secretariat for publication in conjunction with the ICAO Register of AMHS Management Domains. The second part of the attached reply form should be used for this purpose.

If this addressing scheme has not been defined, or if there is no near-term AMHS implementation plan in your State, the addressing scheme called the “XF Addressing Scheme” and specified in Document 9705 can apply as the default addressing scheme. This will allow interoperation between the AFTN (or CIDIN) COM Centre in your State and AMHS systems in other Regions or States.

For the publication of the Register applicable on [July, 1st, 2003], declarations of addressing schemes should be received at the ICAO Secretariat on [April, 1st, 2003] at the latest. In case of no reply being received by the Secretariat at this date,or of incomplete CAAS addressing declaration, the use of the XF Addressing Scheme will be assumed.

[ending formulas etc.]

## Appendix B: Draft Reply Form [blank]

Part 1: PRMD-name Registration

State: …………..…………	[pre-filled in by the Secretariat]

Nationality letters: ……….	[pre-filled in by the Secretariat]

The reserved PRMD-name value for the AMHS MD in the State identified above is:

P = [Nationality letters, pre-filled in by the Secretariat]

Name of ATSO managing the AMHS MD (if applicable):……………………………

Name of the contact point within the replying State: ………………………………………………………………	
Postal/electronic mail address and telephone/fax number: …………………………………………………………

Name and title of the requestor (if different from above, e.g. in the ATSO): ……………………………

Select one of the following options:

Choice A:	the PRMD-name shall take its reserved value

Choice B:	the PRMD-name shall be modified to take the value proposed below, after validation by the Secretariat of the proposed value

Please specify your choice (A or B):

If choice is B, please specify the proposed PRMD-name value:

P =

Additional comments:

Part 2: Declaration of addressing scheme

State: …………..…………	[pre-filled in by the Secretariat]

Nationality letters:……….	[pre-filled in by the Secretariat]

Name of ATSO managing the AMHS MD (if applicable):……………………………

Name of the contact point within the replying State: ………………………………………………………………	
Postal/electronic mail address and telephone/fax number: …………………………………………………………

Name and title of the declaring person (if different from above, e.g. in the ATSO): ……………………………

Select one of the following options:

Choice A:	AMHS user addresses shall be allocated by application of the (recommended) CAAS addressing scheme in the AMHS MD operated in the above State.

Choice B:	AMHS user addresses shall be allocated by application of the (default) XF addressing scheme in the AMHS MD operated in the above State.

Please specify your choice (A or B):

If choice is A (CAAS), please fill in the following table for all location indicators found in Document 7910 under the above nationality letters.

| Organisation-name for the group of locations | Location indicators |
| --- | --- |

(table to be expanded as appropriate)

## Appendix C - Draft ICAO Register of AMHS Management Domains and Addressing Schemes