Report for the DG7

European Commission

Deliverable 2

Contract C3-B97-B59500-01-SIN 7340-ETU

The Introduction of ATN in Europe:

A Regulatory Framework

Annexes

Document Control Log

| Revision Number | Date | Description of Change |
| --- | --- | --- |
| 0.0 | 15/02/1998 | Document initialisation |
| 1.0 | 23/06/1998 | First Issue (annex of first deliverable) |
| 1.2 | 26/02/99 | Final Issue (annex of second deliverable) |

Contributors:

Claire Bordet, Sirius

Giovanna De Maio, ISTEV

Laurent Gille, Sirius

Christelle Lemesle, L'Informatique Communicante

Philippe Lemoisson, Sirius

Eric Nizard, L'Informatique Communicante

Luca Remotti, ISTEV

BIPE Sirius

119 avenue Jacques Cartier

34000 - Montpellier - France

Tel: 33 (0) 4 67 15 95 67

Fax: 33 (0) 4 67 15 95 50

e-mail: gille@sirius.fr

L’Informatique Communicante

215 rue Jean Jacques Rousseau

92136 Issy les Moulineaux - France

Tel: 33 (0) 1 46 62 9100

Fax: 33 (0) 1 46 62 63 26

e-mail: nizard_lic@compuserve.com

ISTEV

Via Roberto Scott, 62

00147 Roma – Italia

Tel: 39 06 51 41 393

Fax: 39 06 51 37 868

e-mail: LucaRemotti@compuserve.com

Table of Contents

Annex 1 -	References	5

Annex 2 -	Glossary of Acronyms and Definitions	10

Annex 3 -	The network evolution	51

Annex 4 -	ATN Design	63

Functional Components	63

Subnetworks	63

Routing	64

Protocols	65

Naming and addressing model	67

Management domains	67

System management, framework and protocol	68

Quality of Service framework	68

Security model	69

Priority model	70

System certification	70

Annex 5 -	The example of Nav Canada	72

Efficiency Improvement	73

User Charges Evolution	74

Safety Management	75

Annex 6 -	Geographical Data	79

Annex 7 -	Frequencies used in the aeronautical world	83

Annex 8 -	Interviews	87

Annex 9 -	Warsaw Convention	89

Annex 10 -	Council Regulation on air carrier liability in the event of accidents	95

# References

Institutional References

| Publisher |  |  |  | Title |
| --- | --- | --- | --- | --- |
|  |  |  |  | CNS/ATM-1 Package SARPs and Guidance Material, Part V - Internet Communications Service, v3.0, 7 August 1995 |
|  |  |  |  | European Network Operating Concept (ENOC), draft 3.0, 3 June 1994 |
|  |  |  |  | Introduction to Advanced CNS/ATM Concepts and the CNS/ATM-1 Package, draft 1, 30 January 1996 |
| ACCESS | [1] |  |  | Current Communications Infrastructure, ACCESS TEN-T ATM Task UK/96/94, March 1998 |
| ACCESS | [2] |  |  | ATN Datalink in the ACCESS Area, ACCESS TEN-T ATM Task UK/96/94, March 1998 |
| ACCESS | [3] |  |  | Definition of the European ATN Routing Architecture, ACCESS TEN-T ATM Task UK/96/94, March 1998 |
| ACCESS | [4] |  |  | Review of activities related to ATN implementation in Europe, ACCESS TEN-T ATM Task UK/96/94, March 1998 |
| ATNI-TF | [1] |  |  | ATN Scenario Definition, Eurocontrol, June 1998 |
| ATNP | [1] |  |  | Analysis of CIDIN Protocols and their Relationship to the Use of X.25 as a Subnetwork of the ATN, ATNP wg1, 11 August 1995 |
| ATNP | [2] |  |  | Analysis of the Mobile IP Proposal and Comparison with ATN Mobile Routing, ATNP wg 2, 15 august 1995, Paper presented by Henk Hof, Prepared by Tony Whyman. |
| ATNP | [3] |  |  | Approach to Validation of CNS/ATM-1 Package SARPs (revised), ATNP wg3, 10 October 1995 |
| ATNP | [4] |  |  | ATN Implementation Scenarios, Annexe 3 |
| ATNP | [5] |  |  | ATN Naming, Addressing and Registration Concept, v1.1 |
| ATNP | [6] |  |  | ATN Upper Layers - Status of Relevant Work in ISO/IEC, Aeronautical Telecommunications network Panel wg3, 6 October 1994 |
| ATNP | [7] |  |  | IDRP Route Aggregation and ATN Requirements for Route Aggregation, Aeronautical Telecommunications network Panel (ATNP) working group one (wg1), 15 December 1995 |
| ATNP | [8] |  |  | Progress Report and Initial Results of IDRP Large Scale Simulations, ATNP wg2, 26 January 1996 |
| ATNP | [9] |  |  | Report of the Sixth Meeting, ATNP wg2, issue 1.0, pp. 33-37 |
| ATNP | [10] |  |  | Report on Agenda Item 2 - Communications aspects of interfacing applications to the ATN (e.g. transitional use of ARINC Specification 622), ATNP |
| ATNP | [11] |  |  | Requirements Made By CNS/ATM Package 1 ATN Applications on the Internet, & Consequent Transition Strategy, ATNP wg1, 6 October 1994 |
| ATNP | [12] |  |  | Requirements placed on the ATN Communication Service by Air/Ground Applications, ATNP wg1 |
| ATNP | [13] |  |  | Upper Layer Requirements as fulfilled by A2CSE, Aeronautical Telecommunications network Panel working group three, 26 October 1994 |
| ATNP | [14] |  |  | World Wide Planning Document for the Evolutionary Implementation of the ATN, ATNP wg1, draft, 29 September 1995 |
| Boeing | [1] |  |  | Air Traffic Management, Concept Baseline Definition, Boeing Commercial Airplane Group, October 31, 1997 (Nextor Report # RR-97-3) |
| CAA | [1] |  |  | ADS Europe Flights Commence, CAA UK, August 1996 |
| EEC | [1] |  |  | APAS Project: PRESTATN - Area: Telematics for Air Transport, Part 1: ATN Requirements Assessment, v1.1, 16 February 1996 Part 2: Survey of ATN Related Activities, v1.1, 16 February 1996 Part 3: ATN: Recommendations for Future Studies, v1.0, 28 February 1996 |
| EEC | [2] |  |  | COPICAT, DGXIII Executive Summary Part 1: Users Requirements and Expectation, v1.1, 29/11/1996 Part 2: Technical Aspects, v1.1, 27/11/1996 Part 3: Economic Assessment & Proposed ATN Organisation, v1.1, 29/11/1996 |
| EEC | [3] |  |  | Air Traffic Management, Freeing Europe's Airspace, White Paper, Com (96) 57 Final, 6.03.1996 |
| EURATN | [1] |  |  | European Aeronautical Network EURATN, sub-task 3.1 Network Management Architecture, chap. 3.4: Management Information Base Structure, chap. 3.5: Interaction with local management Systems, v1.0, 30 April 1992 chap. 4: Functional requirements for network management applications, v1.1, 1 June 1992 EURATN, sub-task 3.1 EURATN network Management Demonstration Requirements, v2.0, 19 March 1992 |
| Eurocontrol | [1] |  |  | Accord multilatéral relatif aux redevances de route, 11 p. |
| Eurocontrol | [2] |  |  | ATM Strategy for 2000+, 1st ed., October 1997 |
| Eurocontrol | [3] |  |  | ATN Project, ATN Implementation Report, Interim Report, Dec 1997 |
| Eurocontrol | [4] |  |  | EAMTS Operational Concept Document (OCD) ed. 1.0 |
| Eurocontrol | [5] |  |  | Guidelines for the economic appraisal of EATCHIP projects — the effective use of cost-benefit studies, ed. 1.2, June 1996 |
| Eurocontrol | [6] |  |  | Harmonisation Studies - European ATN validation Plan, EURATN - EUROCONTROL, draft, 27 December 1995 |
| Eurocontrol | [7] |  |  | Overall Security Concept, Aeronautical Telecommunications network Panel working group one, EUROCONTROL, 4 October 1995 |
| Eurocontrol | [8] |  |  | Principles for establishing the Cost-Base for Route Facility Charges and the Calculation of the Unit Rates, January 1998, 35 p. |
| Eurocontrol | [9] |  |  | Satellite Communications and ground Earth Station Institutional Issues Study, Final report, vol.2, Dec. 1995, 168 p. |
| Eurocontrol | [10] |  |  | Operational Requirements for Air Traffic Management Air/Ground Data Communications Services, EATCHIP, January 1998 |
| Eurocontrol | [11] |  |  | Eurocontrol Revised Convention, September 1997 |
| Eurocontrol |  | [12] | Compendium of the legal instrument relating to the early implementation of the revised convention during the transitional period, January 1998 edition |  |
| Eurocontrol |  | [13] | ATN Project, ATN IMPLEMENTATION TASK FORCE, ATN Scenario Definitions, Issue 2.1, 4 June 1998 |  |
| FAA | [1] |  |  | Capital Investment Plan (CIP) of the Aviation System, 1996 |
| FAA | [2] |  |  | Flight 2000, Initial Program Plan, 16 July 1997, FAA |
| FAA | [3] |  |  | Economic Justifications for FAA’s Flight 2000 Plan, September 1997, FAA |
| FAA | [4] |  |  | Response to the request for comments on the FAA National Airspace System (NAS) Architecture, v. 2.0, FAA |
| FCC | [1] |  |  | North American Numbering Council, Local Number Portability Administration (LNPA), Selection Working Group, Report, April 25 1997 |
| IATA | [1] |  |  | FANS Manual - Overview and Benefits, IATA, first edition v1.1, May 1995 |
| ICAO | [1] |  |  | A planning Guide for the Evolutionary Development of the Data Interchange Portion of the Aeronautical Fixed Service, 1996, Circ. 261 |
| ICAO | [2] |  |  | Aeronautical Telecommunications, International SARPs, Annex 10 to the Convention of International Civil Aviation, Vol I, Radio Navigation Aids, 5th ed., July 1996 Vol II, Communications procedures, 5th ed., July 1995 Vol III, Part 1- Digital Data Communications Systems, 1st ed., July 1995 Vol III, Part 2- Voice Communication Systems, 1st ed., July 1995 Vol IV, Surveillance Radar and Collision Avoidance Systems, 1st ed., July 1995 Vol V, Aeronautical Radio Frequency Spectrum Organisation, 1st ed., July 1996 |
| ICAO | [3] |  |  | Air Traffic Services, International SARPs, Annex 11 to the Convention of International Civil Aviation, 11th ed., July 1997 |
| ICAO | [4] |  |  | Airport Traffic 1995, Digest n° 442 |
| ICAO | [5] |  |  | ATN Manual, second edition, 19 November 1993 |
| ICAO | [6] |  |  | Civil Aviation Statistics of the World 1995, ICAO Statistical Yearbook, Doc 9180 |
| ICAO | [7] |  |  | Economics of Satellite-Based Air Navigation Services, 1995, Circ. 257 |
| ICAO | [8] |  |  | Global Air Navigation Plan for CNS/ATM Systems, may 1998 Volume I: Operational Concepts and General Planning Principles Volume II: Implementation |
| ICAO | [9] |  |  | Guidelines for the Introduction and Operational Use of the GNSS, 1996, Circ. 267 |
| ICAO | [10] |  |  | Institutional factors and aspects from the technical point of view, Special Committee for the Monitoring and Co-ordination of Development and Transmission Planning for the FANS (Phase II), 4th Meeting, sept. 1993 |
| ICAO | [11] |  |  | Manual of the Aeronautical Telecommunication Network, 1st ed., 1991 |
| ICAO | [12] |  |  | Manual of Mode S Specific Services, 1st ed., 1997, Doc 9688 |
| ICAO | [13] |  |  | Manual on Air Navigation Services Economics, 3rd ed., 1997, Doc 9161 |
| ICAO | [14] |  |  | Manual on the Planning and Engineering of the Aeronautical Fixed Telecommunication Network, 5th ed., 1991, Doc 8259 |
| ICAO | [15] |  |  | Progress report on the revision of the global co-ordinated plan for transition to the ICAO CNS/ATM systems, Air Navigation Commission, Working Papers, November 1997 |
| ICAO | [16] |  |  | Outlook for Air Transport to the Year 2005, 1997, Circ. 270 |
| ICAO | [17] |  |  | Proposed ATN Systems RFP Protocol Implementation Conformance Statements (PICS), ICAO ATN Panel, 5-9 February 1996 |
| ICAO | [18] |  |  | Statements by the Council to Contracting States on Charges for Airports and Navigation Services, ICAO, 5th Edition, 1997 (Doc 9082/5) |
| ICAO | [19] |  |  | Air Navigation Plan, European Region, 33rd Edition, 1985, update 09/94, (Doc 7754) |
| IFATCA | [1] |  |  | Towards the 21st century, a vision document, 1997 |
| NTIA | [1] |  |  | US National Spectrum Requirements, Projection and Trends, US Department of Commerce, National Telecommunications and Information Administration, 1995 |

## Legislation

## International Conventions

Chicago Convention, 7 December 1944.

Warsaw Convention, 12 October 1929

## European Directives

Council Directive of 28 june 1990 on the establishment of the internal market for telecommunications services through the implementation of open network provision – 90/387/EEC – amended by Directive 97/51/EC (6/10/1997)

Council Directive 92/44/EEC of 5 June 1992 on the application of open network provision to leased lines.

Council Directive of 13 December 1995 on the application of ONP to voice telephony – 95/62/EC –replaced by Directive agreed by Council and European Parliament on 10/12/1997

Directive of the European parliament and the Council of 10 April 1997 on a common framework for general authorisations and individual licences in the field of telecommunications services – 97/13/EC

Directive of the European parliament and the Council of 30 June 1997 on interconnection in telecommunications with regards to ensuring universal service and interoperability through application of the principles of ONP – 97/33/EC

## Jurisprudence

Decision n.1336/97 /EC of the European Parliament and of the Council on 17 June 1997 on a series of guidelines for trans-European telecommunications networks.

Academic References

| Name |  | Title and Publisher |
| --- | --- | --- |
| ABI | La Banca e il Sistema Informativo: contratti e garanzia di Qualità, June 1996. |  |
| Arcadu-Baccelli | Prodotto difettoso e responsabilità per danni derivanti dall’esercizio del software (Defective product and liability for damages caused in the use of software), Rivista di diritto commerciale, 1992 |  |
| Bieber, Roland, ed. |  | Droit Constitutionnel de l'Union Européenne, Recueil de textes, Nomos Verlagsgesellschaft, Baden Baden, 1996 |
| Cizault, Gisèle |  | IPv6, Théorie et pratique, O'Reilly, Paris 1998 |
| Cornes, Richard & Sandler, Todd |  | The theory of Externalities, Public Goods and Club Goods, Cambridge University Press, 1986 |
| Gazzoni, F. | Manuale di diritto privato, 1997, |  |
| Godet, Michel |  | Manual de Prospective Stratégique, Dunod, 1997 |
| Grard, Loïc |  | Le droit aérien, PUF, Paris 1995 |
| Grynfogel, Catherine |  | Droit Communautaire de la Consurrence, LGDJ, Paris, 1997 |
| Huang J. | “ICAO panel of experts examining the many legal issues pertaining to GNSS”, International Civil Aviation Organisation (ICAO) Journal, October 1997. |  |
| International law association | Project d’instrument international concernant la responsabilite des services de la circulation aerienne, par Guido Rinaldi Baccelli, Committee on the legal aspects of air traffic control Cairo Conference (1992) |  |
| Jennison M. | “A Legal Framework for CNS/ATM”, International Civil Aviation Organisation (ICAO), World Wide CNS/ATM Systems Implementation Conference Rio de Janeiro, 11 - 15 May 1998 |  |
| Korath, Valentine |  | EC Competition Law and Practice, 6th Ed., Hart Publishing, Oxford, 1997 |
| Lenoir, Nathalie |  | Une approche économique de la congestion aérienne, Thèse de Doctorat, Université des Sciences Sociales de Toulouse, Mars 1995 |
| Lenoir, Nathalie |  | Tarification et priorités dans le contrôle aérien, Nouvelle Revue d'Aéronautique et d'Astronautique, n°1, 1998 |
| Maignan, Georges |  | Le contrôle de la circulation aérienne, PUF, Paris 1991 |
| Nordeng T. | “International Legal Impact on National Implementation of global Navigation Satellite Systems (GNSS)”, International Civil Aviation Organisation (ICAO) World Wide CNS/ATM Systems Implementation Conference Rio de Janeiro, 11 - 15 May 1998 |  |
| Pujolle, Guy |  | Les réseaux, 2ème éd., Eyrolles, 1998 |
| Rattray K.O. | “Legal and Institutional Challenges for GNSS - The need for fundamental obligatory norms”, International Civil Aviation Organisation (ICAO) World Wide CNS/ATM Systems Implementation Conference, Rio de Janeiro, 11 - 15 May 1998 |  |
| Rose, Marshall T. |  | The Open Book : a practical perspective on OSI, Prentice Hall, NJ, 1990 |
| Smith & Keenan’s | English Law, 12th edition, FT Financial Times Pitman Publishing, 1998 |  |
| Tanenbaum, Andrew S. |  | Computer Networks, 3rd Ed., Prentice Hall, NJ, 1996 |
| Van Bael, Ivo & Belli, Jean François |  | Competition Law of the European Community, 3rd Ed., CCH Europe, Oxford, 1994 |

# Glossary of Acronyms and Definitions

Compilation from different sources (ICAO, Eurocontrol, industry and international sources)

| Acronym | Definition and Concept |
| --- | --- |
| (OR) | Off-Route in AM(OR)S. Off-route services are intended for communications, including those related to flight co-ordination, primarily outside national or international civil air routes. Off-route services concern mainly military flights. |
| (R) | (Route) in AM(R)S and AMS(R)S. Route services are for communications relating to safety and regularity of flight, primarily along national or international civil air routes. |
| 2D-3D-4D | Two-Three-Four Dimensional |
| A-SMGCS | Advanced Surface Management Guidance and Control Systems System introduced at major airports and integrated with the ATC infrastructure within the EAMTS project. |
| A/G | air–to–ground communication Two-way communication between aircraft and stations or locations on the surface of the earth |
| AAC | Airline Administrative Communications or Aeronautical Administrative Communications (ICAO) Communications provided to enable messages to be conveyed between aircraft and the aircraft operating agency regarding on-board provisions and inventory, passenger travel arrangements, co-ordination with airline ground services for passengers, and passenger data for on board service. |
| AAGDI/AGADE | Automated A/G Data Interchange It is the means by which airborne avionics and processing systems communicate with ground-based systems |
| ABAS | Aircraft-based Augmentation System Augmentation system for GPS and Glonass signals. One type of ABAS is called receiver autonomous integrity monitoring (RAIM) which can be used if there are five or more satellites with suitable geometry in view. Other aircraft-based augmentations can also be implemented and are usually termed aircraft autonomous integrity monitoring (AAIM). Augmentation techniques, particularly useful for improving availability of the navigation function, also include inertial and altimetry-aiding, more accurate time sources or some combination of sensors inputs. ABAS is intended to support non precision approach without and with vertical guidance (RNP 0.3 and RNP 0.3/125 respectively). Some restrictions may apply as a result of RAIM outages. |
| ABI | Advance Boundary Information Message between ACC announcing a flight prior to the activation of a flight plan (through OLDI/SYSCO) |
| ACARS | Aircraft Communication Addressing and Reporting System System using VHF radio data link (VDL) to transmit AOC and AAC data between aircraft and their operating agencies through special ground stations and interconnecting networks. Introduced in 1977 in the USA by ARINC, ACARS has not been subject to any ICAO standardisation process. ACARS was first used to just send a short burst of data telling the operation centre of an aircraft it had taken off or landed. Gradually, more information was included such as engine data, weather details or anything of operational use to the airlines. ACARS is now also used to exchange non critical ATC messages by some ATSOs, like pre-departure clearances. |
| ACAS | Airborne Collision Avoidance System ACAS equipment in the aircraft interrogates SSR transponders on other aircraft in its vicinity and listens for the transponder replies. By computer analysis of these replies, the ACAS equipment determines which aircraft represents potential collision threats and provides appropriate indications (advisories) to the flight crew to avoid collisions. ACAS equipment provides two classes of advisories: traffic advisories (TAs) indicate the approximate positions of intruding aircraft that may later cause resolution advisories; resolution advisories (RAs) propose vertical manoeuvres that are predicted to increase or maintain separation from the threatening aircraft. |
| ACC | Area Control Centre A unit established to provide air traffic control services to controlled flights in control areas under its jurisdiction. Each ACC is responsible for supervising the use of the airspace within a territorial area (FIR or UIR). The airspace within each FIR is in turn divided into sectors in ways that best suit the process of controlling aircraft within it. A sector is notionally the volume of airspace that can be controlled by a single controller; but in practice some sectors are amalgamated with others where this makes sense in terms of traffic loads. |
| ACCESS | ATN Compliant Communications European Strategy EEC project that aims at defining a technical planning scenario for ATN introduction |
| ACI | ATN Communications Infrastructure |
| ACI | ATNSI Consortium Inc. |
| ACI | Airports Council International |
| ACL | ATC Clearance Service see CIC |
| ACM | ATC Communications Management When a flight is about to be transferred from one sector/ATSU to another sector/ATSU, the aircrew is instructed to change to the voice channel of the next sector/ATSU to take control of the flight. The ACM Service provides automated assistance to the aircrew and current and next controllers for conducting this transfer of ATC communications. The ACM service encompasses the transfer of all controller/aircrew communications, both the voice channel and the new data communications channel used to accomplish the ACM service. |
| ADAP | Airport Development Aid Program (USA) |
| ADAP | Automated Downlink of Airborne Parameters A means of providing aircraft status and aircrew preferences information to controllers and ground systems using air/ground data communications. ADAP comprises CAP service. |
| ADAS | AWOS Data Acquisition System (USA) |
| ADF | Automatic Direction Finder |
| ADS | Automatic Dependent Surveillance A technique for aircraft surveillance that is based upon the reporting of aircraft-derived position via a data link communication system, data derived from on-board navigation and position fixing systems, including aircraft identification, four-dimensional position and additional data as appropriate. When a satellite is used for the datalink, the ATC centre can know the aircraft's exact position even if it is over an ocean and outside radar cover. |
| ADS-B | ADS Broadcast ADS-B is a logical extension of ADS. The principle behind ADS-B is that an aircraft should broadcast, regularly and frequently, an estimate of its position based on data from its navigation system. Since this information is broadcast, it could be received by other aircraft and ground stations and potentially used in a number of different ways. One technical issue upon ADS-B, which does not affect the concept, is the type of datalink used to broadcast the data: the two possibilities are VDL mode 4 (S-TDMA technique) and Mode S. |
| ADSP | Automatic Dependent Surveillance Panel |
| AEEC | ARINC Electrical Engineering Committee The AECC defines the physical equipment specification including the "form, fit and function". |
| AENA | Aeropuertos Español y Navegacíon Aérea (E) Spanish Authority in charge of airports and air navigation and control. |
| AES | Aircraft Earth Station Earth station on board used for AMSS and AMS(R)S. The AES comprises the avionics and antenna systems on board the aircraft. |
| AEV | Annual Equivalent Value |
| AFS | Aeronautical Fixed Service A telecommunication service between specified fixed points provided primarily for the safety of air navigation and for the regular, efficient and economical operation of air services. |
| AFTN | Aeronautical Fixed Telecommunication Network A worldwide system of aeronautical fixed circuits provided, as part of the aeronautical fixed service, for the exchange of messages and/or digital data between aeronautical fixed stations having the same or compatible communications characteristics. |
| AIDC | ATS Inter-facility Data Communications This ATN application exchanges information between ATS Units (ATSUs) for support of critical Air Traffic Control (ATC) functions: flight notification, flight co-ordination, transfer of control, transfer of communications, transfer of surveillance data and general information exchange. |
| AIG | ACAS Implementation Group |
| AIP | Aeronautical Information Publication A publication issued by or with the authority of a State and containing aeronautical information of a lasting character essential to air navigation |
| AIP | Airport Improvement Program (USA) |
| Aircraft address | A unique combination of 24 bits available for assignment to an aircraft for the purpose of air-ground communications, navigation and surveillance. |
| AIRMET | Information issued by a meteorological watch office concerning the occurrence or expected occurrence of specified en-route weather phenomena which may affect the safety of low level aircraft operations and which was not already included in the forecast issued for low-level flights in the flight information region concerned or sub-area thereof. |
| Airspace Capacity | Traditionally, capacity increases have been achieved by dividing airspace into smaller and smaller airspace sectors to offset the workload that they bring. This method, however, follows a law of diminishing returns because of the additional co-ordination work it generates. There is also evidence that future gains made this way would progressively reduce, and be insufficient to meet the forecast growth in demand. |
| Airspace Regime | A defined method of governing a specified volume of airspace (the term encompasses both the types of airspace and the method of separation applied within them). Three main regimes: MAS, FFAS, UMAS. |
| AIS | Aeronautical Information Services AIS have the objective of ensuring the flow of information necessary for the safety, regularity and efficiency of air navigation. AIS messages shall comprise messages concerning NOTAMs and messages concerning SNOWTAMs, but also Aeronautical Information Publications (AIPs) and Aeronautical Information Circulars (AICs). |
| AISAP | AIS Automation Panel |
| ALOHA | Slotted ALOHA A random access strategy whereby multiple users access the same communications channel independently, but each communication must be confined to a fixed time slot. The same time slot structure is known to all users, but there is no other co-ordination between users. |
| AM | Amplitude Modulation |
| AM(R)S | Aeronautical Mobile (R) Service An aeronautical mobile service reserved for communication relating to safety and regularity of flight, primarily along national or international civil air routes. |
| AMAN | Arrivals Manager |
| AMC | Airspace Management Cell Function introduced in FUA (Flexible Use of Airspace) in the 1996-98 time frame within EATCHIP development |
| AMHS | ATN Message Handling Service This ATN application is provided for the exchange of operational messages between users and/or ground systems. It matches the service currently provided by the AFTN uses -in a transparent manner. The AMHS environment is based on two types of systems both operating the MHS protocols: - the ATS Message Server - the AFTN/ATN Gateway |
| AMS | Aeronautical Mobile Service A mobile service between aeronautical stations and aircraft stations, or between aircraft stations, in which survival craft stations may participate; emergency position-indicating radiobeacon stations may also participate in this service on designated distress and emergency frequencies. |
| AMS(R)S | Aeronautical Mobile Satellite (R) Service An aeronautical mobile-satellite service reserved for circuit-mode communication relating to safety and regularity of flight, primarily along national or international civil air routes. The AMS(R)S circuit-mode service provides a means to establish and maintain a non-shared switched circuit between aircraft and ground users on demand. The AMS(R)S procedures are specially designed to achieve enhanced and consistent call set-up times for air originated voice calls at the safety priorities (i.e. distress/urgency, flight safety and regularity/meteorological) with minimal performance impact from other AMSS traffic. A switched circuit is held for the duration of the call unless automatically pre-empted in order to reassign resources for a higher priority call attempt. |
| AMSS | Aeronautical Mobile Satellite Service A mobile satellite service in which mobile earth stations are located on board aircraft; survival craft stations and emergency position-indicating radiobeacon may also participate in this service. The major elements of the AMSS are the space segment (the satellites and their controlling earth stations), the ground earth station (GES) and the aircraft earth station (AES). The packet data interface allows AMSS to function as a satellite subnetwork of the ATN. The satellite network transfers data packets from air to ground and from ground to air. Packet data transfers are provided in the form of connection-mode service, using ISO 8208 as a subnetwork access protocol. |
| ANP | (Regional) Air Navigation Plan A regional navigation plan is a detailed statement of facilities and services to be provided to meet the needs of international civil aviation within a specified region. The plan is an authoritative, internationally agreed and ICAO Council approved reference document of benefit to both provider States and airspace users. In its technical scope, a regional plan will include the facilities and services in each of the various sub-domains of air navigation, e.g. communications, aeronautical information services, meteorology, aerodromes and ground aids etc. Geographically, the plan will cover one or more of the nine air navigation regions into which ICAO has divided the world. |
| ANS | Air Navigation Services This term includes air traffic services (ATS), aeronautical telecommunications service (COM), meteorological services for air navigation (MET), search and rescue (SAR) and aeronautical information services (AIS). These services are provided to air traffic during all phases of operations (approach, aerodrome control and en route phases). |
| AO | Aircraft Operator or Aircraft Operating Agency The person, organisation or enterprise engaged in, or offering to engage in, an aircraft operation. |
| AOC | Aeronautical Operational Communications Communications required for the exercise of authority over the initiation, continuation, diversion or termination of a flight in the interest of the safety of the aircraft and the regularity and efficiency of a flight. These communications are normally required for the exchange of messages between aircraft and aircraft operating agencies. |
| AOC | Aircraft Operations Control or Centre (ICAO) Aircraft operations control plans and conducts individual flights safely and efficiently in accordance with the mission. |
| APATSI | Airport-ATS Interface As a complement to EATCHIP, ECAC established the APATSI programme to carry out aerodrome operations planning, particularly airport/ATC capacity aspects. |
| APC | Aeronautical Passenger Communications Communications relating to non-safety voice and data services to passengers and crew members for personal communications to the ground. |
| APO | Airport Operator An airport operator makes the most efficient use of available airport facilities and resources in the airport under its control. |
| APP | Approach Control (Office/Service) Approach Control Office: a unit established to provide air traffic services to controlled flights arriving at, or departing from, one or more aerodromes Approach Control Service: Air Traffic control service for arriving or departing controlled flights. |
| APR | Aircraft Parameter Reporting APR Service addresses the automatic delivery of surveillance data from the aircraft to the controllers. |
| Apron | A defined area, on a land aerodrome, intended to accommodate aircraft for purpose of loading or unloading passengers, mail or cargo, fuelling, parking or maintenance |
| APS | Airway Planning Standard (ARINC Aeronautical Radio, Incorporated) |
| APW | Area Proximity Warning APW notifies controllers of aircraft under their control, which are at risk of flying into protected airspace such as a military area. |
| ARINC | Aeronautical Radio, Incorporated ARINC was established by the US airlines in 1929 to be the single licensee for aeronautical communications frequencies on behalf of the US government and to provide aviation safety communications for the aviation industry. ARINC is a 300 m$ communications and engineering company, operating worldwide. |
| ARINC 622 | ARINC 622 standard defines the BOP (Bit Oriented Protocol)/COP (Character Oriented Protocol) conversion functions as well as the cyclic redundancy check which assures message integrity. |
| ARN | ATS Route Network |
| ARSR | Air Route Surveillance Radar |
| ARTAS | ATC Radar Tracker and Server Project managed by Eurocontrol aiming at providing processed radar data to multiple users on a network basis and brings the benefits of radar traffic information within the reach of every ATM unit, large or small. |
| ARTCC | Air Route Traffic Control Centre |
| ARTS | Automated Radar Terminal System |
| ASAS | Airborne Separation Assurance Systems ASAS is a generic term for a number of applications centred around a cockpit display of traffic information. They are intended to give the pilot a greater role in maintaining separation using new or existing procedures. It is generally assumed that these applications will be implemented using some form of ADS-B. Many of the benefits of ASAS in oceanic airspace will come about anyway from the reduced vertical separation minimum (RVSM). |
| ASDE | Airport Surface Detection Equipment |
| ASE | ATM System Engineering |
| ASECNA | Agence pour la sécurité de la navigation aérienne en Afrique et à Madagascar (Air navigation safety agency for Africa and Madagascar) |
| ASM | AirSpace Management A planning function with the primary objective of maximising the utilisation of available airspace by dynamic time-sharing and, at times, segregation of airspace among various categories of users based on short-term needs. In future systems, airspace management will also have a strategic function associated with infrastructure planning. |
| ASMAC | Automated System for the Management of Aeronautical Information and Charting |
| ASR | Airport Surveillance Radar |
| ASTERIX | Radar Data Format which is recognised by most European countries as the European Standard for Radar Data Format. |
| AT | Air Traffic |
| ATC | Air Traffic Control ATC Service is a service provided for the purpose of preventing collisions between aircraft and on the manoeuvring area between aircraft and obstructions, and of expediting and maintaining an orderly flow of air traffic. |
| ATCC | Air Traffic Control Centre see ATCU |
| ATCO | Air Traffic Controller A person holding a licence and/or a rating entitling him to act as an air traffic controller. |
| ATCU | Air Traffic Control Unit A generic term meaning variously area control centre, approach control office or aerodrome control tower. ATC Unit manages air traffic in the areas under its responsibility. ATCU transfers the responsibility for control as a flight progresses from the area of one control unit to another. An orderly transfer process requires that an aircraft be under the control of only one ATCU at any time. ATCUs are sometimes faced with a traffic demand beyond the capacity of a particular location or area, as occurs in busy aerodromes during peak periods. The Annex 11 of ICAO Convention allows ATC units to specify restrictions to the traffic flow, when required, for the purpose of avoiding excessive delays to aircraft in flights. |
| ATFM | Air Traffic Flow Management (ICAO) A service of tactical and strategic planning activities with the objective of ensuring an optimum flow of air traffic to or through areas during times when demand exceeds or is expected to exceed the available capacity of the air traffic control system. See F&CM |
| ATIF | ATN Trial Infrastructure European project aiming at testing full ATN application. The philosophy of the project has been to develop ICAO compliant ATN systems as quickly as possible and to distribute them as widely as possible at low cost to member states for ATN Trials, whilst providing support and training. ATIF users are given TAR, TTS and TES which are the first 7 layer ICAO CNS/ATM-1 Package compliant systems offering ATN communications services anywhere. |
| ATIS | Automatic Terminal Information Service The provision of current or routine information to arriving and departing aircraft by means of continuous and repetitive broadcasts throughout the day or a specified portion of the day. ATIS broadcasts are provided at aerodromes where there is a requirement to reduce the communication load on the ATS VHF air-ground communication channels. A discrete VHF frequency shall be used for ATIS broadcasts. If a discrete frequency is not available, the transmission may be made on the voice channel of the most appropriate terminal navigation aid, preferably a VOR. |
| ATM | Air Traffic Management ATM is a systems approach with the objective of enabling aircraft operators to meet their planned times of departure and arrival and adhere to their preferred flight profiles with minimum constraints, without compromising agreed levels of safety. It comprises ground elements and airborne elements which, when functionally integrated, form a total ATM system. ATM comprises all the functions needed by a safe and regular management of air traffic. ICAO defines ATM as Airspace Management (ASM), Air Traffic Flow Management (AFTM) and Air Traffic Control (ATC). |
| ATMC | Air Traffic Management Centre |
| ATN | Aeronautical Telecommunications Network New network introduced to support the CNS/ATM developments. It will become the backbone for ground/ground and air/ground communications in CNS/ATM or EATMS. Eurocontrol and ICAO are currently working to standardise ATN. The ATN is an internetwork architecture that allows ground, air-ground and avionics data subnetworks to interoperate by adopting common interface services and protocols based on the ISO OSI reference model. |
| ATN Subnetwork | In an ATN environment, subnetworks are connected to other subnetworks through ATN routers which select the best route for transmission of each data message. The different air-ground data links such as AMSS, VDL, SSR mode S, or HF data link, are ATN compatible and can therefore constitute ATN subnetworks. |
| ATNI | ATN Implementation |
| ATNI-TF | ATNI Task Force (Eurocontrol) The ATNI-TF was established under the auspices of the EATCHIP Programme in order to identify, document and where appropriate resolve the issues related to the deployment of the ATN in Europe. |
| ATNP | Aeronautical Telecommunications Network Panel The ATN concept was first developed by the special committee on the FANS of ICAO. Later, the task of developing the necessary provisions was given to an ICAO panel of experts from 20 states and 7 international organisations which is called the ATNP. |
| ATNSI | ATN Systems Inc. (USA) |
| ATS | Air Traffic Services ATS is a generic term meaning variously flight information service (FIS), alerting service, air traffic advisory service and air traffic control service (area control service, approach control service or aerodrome control service). ATS are provided by ground facilities, usually operated by national aviation authorities and, in some cases, by international air traffic organisations composed of members of several nations (e.g. Eurocontrol). The prime objective of ATS is to prevent collisions between aircraft, whether taxiing on the manoeuvring area, taking off, landing, en route or in the holding pattern at the destination aerodrome. ATS deals also with ways of expediting and maintaining an orderly flow of air traffic and of providing advice and information for the safe and efficient conduct of flights and alerting services for aircraft in distress. ATS consist of clearances and information issued by ATC units to achieve longitudinal, vertical or lateral separation between aircraft, in accordance with Annex 11 of the ICAO Convention. |
| ATSC | Air Traffic Services Communications Communications related to air traffic services including air traffic control, aeronautical and meteorological information, position reporting, and services related to safety and regularity of flight. |
| ATSO | Air Traffic Service Organisation |
| ATSU | Air Traffic Service Unit A generic term meaning variously air traffic control unit (ATCU), flight information centre or air traffic services reporting office. |
| ATSU | Aircraft Telecommunication System Unit Airbus name for CMU |
| AWOS | Automated Weather Observing System (USA) |
| B-RNAV | Basic Area Navigation B-RNAV defines European RNAV operations which satisfy a required track keeping accuracy of ± 5 NM for at least 95% of the flight time. This level of navigation accuracy is comparable with that which can be achieved by conventional navigation techniques on ATS routes defined by VOR/DME, when VORs are less than 100 NM apart. The ability to achieve the required level of navigation performance in a given airspace depends not only on the accuracy and functionality of the aircraft navigation equipment, but also upon adequate coverage of navigation aids and position coordinates accuracy provided by the navigation infrastructure of the region. For the determination of aircraft position suitable input data can be derived from the following navigation sources: DME/DME, VOR/DME, INS, Loran C, GNSS. |
| B/C | Benefit/Cost |
| BAAA | Belgian Airport and Airways Agency Belgian CAA |
| BFE | Buyer Furnished Equipment |
| C-Channel | AMSS Channel. The C-Channel is a circuit mode channel used for digital voice or data communications. A C-Channel can be requested by an AES over the R Channel, and assigned by the GES over the P-Channel. Circuit mode single channel per carrier (SCPC), used in both to-aircraft and from-aircraft directions. This channel is time division multiplexed to provide a primary channel for voice or data traffic and a sub-band channel for signalling, supervision and data messages. The use of the channel is controlled by assignment and releases signalling at the start and end of each transaction. |
| CA | Conflict Alert |
| CA/MSAW | Conflict Alert/Minimum Safe Altitude Warning |
| CAA | Civil Aviation Authority (UK) |
| CADF | Centralised Airspace Data Function Function introduced in FUA (Flexible Use of Airspace) in the 1996-98 time frame within EATCHIP development. |
| C/AFT | CNS/ATM Focused Team The C/AFT is a group of airlines, airframe manufacturers and service providers, aiming at making credible investment analyses of alternative systems transitions. |
| CAP | Controller Access Parameters This datalink service aims at enhancing the ATC surveillance and the availability of aircraft parameters to the controller by extracting and downlinking data from the airborne system. The service is foreseen to be primarily used in continental airspace, both in en-route and terminal areas. |
| CAERAF | Common American European Reference ATN Facility The objective of the CAERAF is to provide a test facility to test operational ATN Systems prior to development in the operational environment. Testing will be required in the context of recognised safety assessment, certification and operational approval processes. The CAERAF project is a joint project between the FAA and ATN Systems INC., on one side and Eurocontrol on the other side. The CAERAF is planned to be available end 1999. |
| CAS | Collision Avoidance Systems |
| CASA | Computer Assisted Slot Allocation |
| CASITAF | CNS/ATM Systems Implementation Task Force |
| CBA | Cost-Benefit Analysis |
| CBT | Computer-Based Training |
| CDM | Collaborative Decision Making Both the collective requirements of all airspace users and the individual aircraft operator's preferences will be taken into account in determining solutions to events. The open systems environment and better information management will allow a permanent dialogue between the various parties (ATM, aircraft operations centres, pilots and airport operations) before departure, and as the flight processes through the ATM system. This exchange of information will enable the various organisations to continuously update each other on relevant events in real-time and provide the basis for more efficient decision making. Aircraft operators will have up-to-date and accurate information on which to base decisions about their flights, and will be able to apply factors which are not known to ATM, such as fleet management priorities, fuel consumption figures and other aircraft operating parameters, when determining solutions. |
| CDTI | Cockpit Display of Traffic Information |
| CEAC | Committee for European Airspace Coordination |
| CEATS | Central European Air Traffic Services The CEATS agreement has been signed by Austria, Croatia, the Czech Republic, Hungary, Italy (for Padua FIR only), Slovakia and Slovenia. Bosnia-Herzegovina is expected to sign the agreement. The agreement provides for a new Upper Area Control Centre to be established, to provide ATC in the upper airspace of the participating States. The centre will be operated by Eurocontrol and will be managed in a similar fashion to the Maastrich UACC. |
| CEC | Commission of the European Communities |
| CENA | Centre d'études de la navigation aérienne - France (Air Navigation Study Center) |
| CEU | Central Executive Unit |
| CFMU | Central Flow Management Unit The Eurocontrol CFMU provides an ATFM service to airspace users throughout the ECAC States. Located in Brussels (Harren), the Eurocontrol CFMU became fully operational in the spring of 1996. |
| CFPS | Collaborative Flight Plan Services |
| Character | Character-oriented protocols restricts messages that can be transmitted to those which can be conveyed by upper-case alphabetic characters, numbers, and a very limited set of punctuation marks. |
| Charges | Charges are levies to defray the costs of providing facilities and services for civil aviation whereas taxes are levies to raise general national and local government revenues that are applied for non-aviation purposes. |
| CHI | computer/human interface |
| CIC | Clearance and Information Communications This service describes aircraft/controlling ATSU A/G data communication messages exchanges and procedures for operations within the European region for the following: - aircrew's reports and clearances requests - controller's delivery of clearances, instructions and notifications to aircraft - support and system messages Equiv. To ACL (ATC Clearance Service) |
| CIDIN | Common ICAO Data Interchange Network CIDIN is one part of the aeronautical fixed service which uses bit-oriented procedures and packet switching techniques. CIDIN is a protocol designed for data transmission over AFTN, up to layer 4 (transport) and using network layer X 25 packet protocol associated with a CIDIN network layer. |
| CIP | Capital Investment Plan |
| CIP | Convergence and Implementation Programme CIP will target EATCHIP Phase IV (EATMS) as an "Objectives and Time Scales Road Map". It will be the mechanism for coordinating the acquisition and implementation of EATMS components within the ECAC area. |
| CJEC | Court of Justice of the European Communities The CJEC is based in Luxembourg. Under Art. 164 of the Rome Treaty, its function is to "ensure that in the interpretation of this Treaty the law is observed". Since the Court of First Instance, established pursuant to art. 168 A of the Treaty as inserted by the Single European Act (SEA), first convened from November 1989, the Court of Justice has become a court of appeal in so far as competition decisions of the Commission are concerned. However, the court still enjoys exclusive jurisdiction as regards preliminary rulings originating from member states' courts. |
| CL | Connectionless-mode Service The communication service technique which transfers data between layers without prior coordination. All protocol data units (PDU) are transferred with no implicit association between them. |
| Clearance | (Air Traffic Control) Clearance Authorisation for an aircraft to proceed under conditions specified by an air traffic control unit (ATCU). The abbreviated term "clearance" may be prefixed by the words "taxi", "take-off", "departure", "en route", "approach" or "landing" to indicate the particular portion of the flight to which the air traffic control clearance relates. |
| CLNP | ConnectionLess Network Protocol Layer 3 Protocol originating in DECNet and used in ISO Standard (ISO 8473) |
| CMA | Context Management Application In an ATS data link environment, the CMA allows datalink-equipped aircraft and ground systems to exchange and update datalink application information and flight plan association data between each other. In addition, the CMA defines a communication protocol used by ground systems to exchange via ground networks addressing information received from aircraft. Functions supported by the CMA are: - the logon function which constitutes the initial datalink exchange between the aircraft and the ground - the contact function used by a ground system to request the aircraft to initiate datalink with an other ground system - the update function used by a ground system to uplink ground addressing information to the aircraft - the ground forwarding function used by a ground system to forward the addressing information received from an aircraft to another ground system - the registration function performed locally by both systems to make available the received addressing application to local applications |
| CMIC | Civil/Military Interface Committee Standing committee of Eurocontrol examining any question regarding the civil/military interface in Eurocontrol members. |
| CMU | Communications Management Unit |
| CNS | Communications, Navigation and Surveillance |
| CNS/ATM | Communications, Navigation and Surveillance/ Air Traffic Management (ICAO) Communications, Navigation and Surveillance systems, employing digital technologies, including satellite systems together with various levels of automation, applied in support of a seamless global air traffic management system that will enable aircraft operators to meet their planned times of departure and arrival and adhere to their preferred flight profiles with minimum constraints and without compromising agreed levels of safety. |
| CO | Connection-oriented mode Service The communication service technique which transfers data between peer layers using a prior connection to logically associate the sequence of protocol data units (PDU). |
| CONUS | continental, contiguous, or conterminous United States |
| COPS | Common Operational Performance Specifications |
| COTS | Commercial–Off–The–Shelf Defines widely commercially available products or services. |
| CPDLC | Controller Pilot Data Link Communications CPDLC refers to the transfer of data messages between pilot and controller using a datalink system. This process replaces some of the voice communications that are commonly used today for controller-pilot communications. Messages sent to the pilot are presented on a cockpit display where he can enter a response that is transmitted back down to the controller. CPDLC enables a lot of control related services including delivery of ATC clearances (CIC, DCL and DSC), ATC Communications Management (ACM), Flight Plan Consistency Checking (FLIPCY), Dynamic Route Availability Notification (DYNAV), etc. |
| CRAM | Conditional Route Availability Message |
| CRCO | Central Route Charge Office In 1969, the Member States of Eurocontrol adopted the basic principles for a harmonised regional en-route facility charging system involving a single charge per flight, which came into operation on November 1971. The CRCO was set up to operate this system on behalf of the States. The CRCO operates a harmonised regional system in which a single area charge is billed, irrespective of the use of the number of States overflown. The bill is settled by a single payment, in one currency (the ECU), to one body, Eurocontrol. |
| CRS | Computer Reservation System |
| CSD | Context and Scope Document (EATMS) |
| CWP | Controller Working Position |
| CWS | Collision Warning System |
| CY | calendar year |
| D/L | Data Link |
| D-FIS | Data Link Flight Information Services A means of providing flight information (e.g. ATIS and meteorological information) to aircrew using air/ground data communications. DFIS comprises D-OTIS and D-RVR services. |
| D-OTIS | Datalink Operational Terminal Information Service The D-OTIS Service provides automated assistance in requesting and delivering compiled meteorological and operational flight information derived from ATIS, METAR and NOTAMs/SNOWTAMs, specifically relevant to the departure, approach and landing flight phases. |
| D-RVR | Datalink Runway Visual Range The D-RVR Service provides automated assistance in requesting and delivering the instantaneous Runway Visual Range (RVR) to the aircrew |
| DARP | Dynamic Aircraft (or Airborne) Rerouting Procedure Procedures developed in the Pacific Rim mainly to allow aircraft to avoid bas weather conditions. |
| DCE | Data Communication Equipment An interface between data terminal equipment (DTE) and the transmission mechanism. |
| DCF | Discounted Cash Flow |
| DCL | Departure Clearance A flight due to depart from an aerodrome must first obtain departure information and clearance from the controlling ATSU. The departure clearance service provides automated assistance for requesting and delivering departure information and clearance, with the objective of reducing aircrew and controller workload and diminishing clearance delays. |
| DFL | Division Flight Level |
| DFS | Deutsche Flugsicherung GmbH (Germany) |
| DLIC | Datalink Initiation Capability Service providing access to all data link services (logon) which will enable the other services. In addition to the exchange of naming and addressing information related to air- and ground-based applications, the DLIC service allows for the transfer of the information required for the flight plan correlation (identifying unambiguously an aircraft and associating it with a flight plan). |
| DLR | Deutsche Forschungsanstalt für Luft -und Raumfahrt (Germany) |
| DLS | Direct Link Service A data communication service which makes no attempt to automatically correct errors, detected or undetected, at the link layer of the communications path (errors control may be carried-out by end-user systems). |
| DLTF | Data Link Task Force (IATA) |
| DMAN | Departure Manager |
| DME | Distance Measuring Equipment DME is expected to become the primary source of position fixing in the ECAC area by 2000. DME operates in the 960-1215 MHz band. |
| DO 178 | DO 178 is an RTCA document dealing with Software Considerations in Airborne Systems and Equipment Certification. Its purpose is to provide guidelines for the production of software for airborne systems and equipment that performs their intended function with a level of confidence in safety that complies with airworthiness requirements. These guidelines are in the form of: - objectives for software life cycle processes, which cover the software life cycle, the software planning process and the software development processes. They are based inter alia on the categorisation of failure conditions, system architectural considerations, system requirements considerations for software verification, and software consideration in system verification. - descriptions of activities and design considerations for achieving those objectives. - descriptions of the evidence that indicate that the objectives have been satisfied. For instance, a software compliant with DO178 level C is a software for which an abnormal behaviour, as indicated by the process of system safety declaration, would cause or contribute to a failure of a function resulting from a minor error condition in the aircraft. There are constrains in the production of documents and in the verification of the realisation processes. The software development and the verification tools have themselves to comply to DO178 requirements (tool qualification). The effort that has to be achieved in order to develop a system compliant to DO178 is between 2.5 and 4 times higher than for a system non compliant to DO178. |
| DOC | Direct Operating Cost |
| Domain | A collection of components (for example end systems and subnetworks) organised in a hierarchical manner and capable of interoperability. |
| DOT | Dynamic Ocean Track DOT System is defined to update trans-pacific routes every 24 hours and sometimes more frequently. It takes into account among other factors the wind-aloft effect on aircraft speed and economy. Weather tracking and route modification is designed to allow each aircraft direction over the Pacific to have the most favourable winds aloft. It makes allowance for adverse weather other than head winds and for other conditions as missiles launches or military aircraft manoeuvres as well as temporary restricted areas. The same weather tracking/route modifying procedure is also used on Atlantic routes. |
| DPS | ATM Data Processing Systems |
| DRA | Defence Research Agency (UK) |
| DSB | double side band |
| DSC | Downstream Clearance Service Aircrew in specific instances, need to obtain clearances or information from ATSUs which may be responsible for control of the aircraft in the future, but are not yet in control of it. Such "downstream" clearances and information are often provided through ground/ground coordination, but are also obtained via direct contact with the "downstream" ATSU (D-ATSU) in certain circumstances (e.g. when G/G communications are unavailable or inefficient, due to the size of the airspace, due to the complexity of the route structure, or due to meteorological conditions). DSC Service provides assistance for requesting and obtaining D-ATSU clearances or information, using A/G datalink. |
| DSP | Departure Sequencing Program |
| DTE | Data Terminal Equipment A digital data transmitter/receiver device that includes terminals and computers. A DTE is an end point of a subnetwork connection. |
| DYNAV | Dynamic Route Availability Service DYNAV provides automated assistance for the proposal of alternative routes to aircrew as they become available (e.g. when military areas become free to civil use). This service could take place automatically after FLIPCY. |
| EACE | European ATN Coordinating Entity ATNITF proposed a European entity that will be in charge of ATN implementation and management for multinational/domain issues. |
| EAD | European AIS Database Common component of the EAMTS architecture. |
| EAMG | European ATFM Management Group |
| EANPG | European Air Navigation Planning Group (ICAO) EANPG is the group establishing the European ANP. It is the final authority for the adoption of the European ATN implementation plan |
| EARS | En route analysis and reporting system |
| EARTS | En route automated radar system |
| EATCHIP | European Air Traffic Control Harmonisation and Implementation Programme EATCHIP is a cooperative programme of the 36 ECAC States coordinated and managed by Eurocontrol, in partnership with the national ATM providers of ECAC and supported by the European Union, with the objective of harmonising European ATM services by 1998 and integrating them by the beginning of the 21st century. EATCHIP is designed to meet the overall objectives of the ECAC strategy for the 1990s, i.e. "to provide increasing airspace and control capacity urgently in order to handle traffic expeditiously while maintaining a high level of safety". EATCHIP is a four-phase activity which will extend well beyond the year 2000. Some of the phases overlap and the process is reinitiated for each State which becomes a member of the ECAC. Phase 1 -appraisal and evaluation- was completed in 1991. Phase 2 -programme development- was completed in 1993. Phase 3 -acquisition and implementation of systems to meet the CIP (Convergence and Implementation Programme set in phase 2) objectives- is on schedule. Phase 4 is the transition to EATMS. |
| EATMS | European Air Traffic Management System Phase IV of the EATCHIP decided in march 1992 by the ECAC Transport Minister Conference; EAMTS aims at allowing all airspace users the maximum freedom of movement subject to the needs for safety, cost-effectiveness, environmental aspects and national security requirements. EAMTS concept development is focused on the integration of airborne and ground-based decision making responsibilities, as the key to more efficient decision making by pilots and controllers. EAMTS should become operational in stages throughout the ECAC area from 2000 onwards. EAMTS will be an integral part of the global ICAO CNS/ATM system, taking into account the specific traffic densities and compositions of the ECAC airspace as well as specific technical constraints. |
| EC | European Commission |
| ECAC | European Civil Aviation Conference The ECAC was established in 1955 by the Council of Europe and the ICAO, gathering all European civil aviation administrations to discuss and coordinate their policies. Within the ICAO framework, ECAC is regarded as a regional aviation organisation in the European Region, with the same status as the civil aviation organisations of other ICAO Regions (such as the African Air Transport Commission (AFCAC) and the Latin American Civil Aviation Commission (LACAC)). To alleviate serious capacity problems that emerged during the 80s, ECAC, together with the EC, adopted stringent measures to harmonise the air traffic services of their member states and jointly planned air traffic management within their respective areas of responsibility. Subsequently, the Ministers of Transport of the ECAC States endorsed the EATCHIP developed by ECAC and gave Eurocontrol a mandate to manage its implementation. |
| ECARDA | European Coherent Approach for Research and Development in Air Traffic Management |
| ECBAG | EATCHIP Cost-Benefit Advisory Group |
| ECU | European Currency Unit |
| EEC | EUROCONTROL Experimental Centre |
| EGNOS | European Geostationary Navigation Overlay Service Developed by a three party group (EU, ESA, Eurocontrol) to complement facilities and enhance the performance capabilities of GPS and GLONASS systems over Europe. Initial operational capability should be available in 1999 for supplemental means use and full operational capability in 2002. EGNOS will provide a regional augmentation service for GPS and Glonass signals over Europe, the Atlantic and Indian Oceans, South America, Africa, the Middle-East and Central Asia. This augmentation is called an overlay. Inmarsat will provide the two transponder capacities on the satellites Inmarsat 3 operating on the Atlantic and Indian Ocean. EGNOS transponders will broadcast GPS-like signals, becoming two pore sources and guaranteeing that the minimum number of six satellites required for safety critical applications is in view at all times and all locations in the area zone. |
| ENAC | Ecole nationale de l'aviation civile (France -Civil Aviation National School) |
| ENNA | Etablissement national de la navigation aérienne (France) |
| ENOC | European Network Operating System |
| EOBT | Estimated Off-Block Time |
| EOLIA | European pre-Operational data Link Applications EOLIA is a EC sponsored project which has the objective to develop and evaluate a set of user-oriented ATN compliant, pre-operational ATC data link services in the European environment to enable the improvement of ATM taking into account the interests of the users and the European industry |
| EONS | EUROCONTROL Open and geNeric graphic System |
| EPAC | EATCHIP Policy and Allocation Committee |
| ES | End System A system which initiates network protocol data units (NPDU) for delivery to other systems and receives NPDUs from other systems but does not relay NPDUs. |
| ESA | European Space Agency |
| ESCAPE | EUROCONTROL Simulation Capability And Platform for Experimentation |
| ETG | European Tripartite Group EEC, Eurocontrol and ESA have formed a tripartite group to determine the best WAAS solution for Europe. These organisations have defined a program called EGNOS. The tripartite group submitted a proposal to Inmarsat for the use of the Inmarsat 3 navigation transponders, to provide the signal for the EGNOS. This resulted in the allocation of two transponders to Europe through an agreement signed in 1996. |
| EU | European Union |
| EUR | European Region (ICAO) EUR Region comprises 49 states, of which the 36 ECAC States and the 13 CIS States. |
| EUROCAE | European Organisation for Civil Aviation Electronics EUROCAE is the European organisation of equipment manufacturers and civil aviation entities responsible for standardising aircraft equipment. EUROCAE comprises 77 members from 15 countries, amongst them 42 European aviation manufacturers and 10 CAAs, together with JAA and Eurocontrol. EUROCAE works closely with its North American counterpart RTCA. EUROCAE issues MOPS for on board equipment submitted to its working groups, MOPS that could eventually become mandatory JTSO. |
| EUROCONTROL | European Organisation for the Safety of Air Navigation Founded in 1960 for overseeing air traffic control in the upper airspace of member states, Eurocontrol today has, as its most important goal, the development of a coherent and coordinated air traffic control system in Europe. Its primary objectives are: - to manage EATCHIP on behalf of States belonging to the ECAC - to operate a single European ATFMU to make optimum use of European airspace and to prevent traffic congestion - to implement short- and medium-term action to improve the co-ordination of air traffic control throughout Europe - to carry out research and development work aimed at increasing air traffic control capacity in Europe Eurocontrol comprises 25 member States. |
| EWP | EATCHIP Work Programme |
| F&CM | Flow and Capacity Management Eurocontrol acronym for the ICAO AFTM F&CM is the primary means of ensuring flight punctuality and efficiency. The main principles include co-ordinated timetables, optimised time of arrival and minimum in-flight delay. |
| FAA | Federal Aviation Administration (USA) |
| FANS | Future Air Navigation Systems (ICAO Special Committee) |
| FANS-1 | Package proposed by Boeing on its commercial long-haul jets, as a first step towards CNS/ATM systems. FANS-1 comprised an ACARS-based ADS system for air traffic management, with satellite datalink, initially for applications in the South Pacific. The package is based on a number of industry standards, effectively frozen in 1992, and is not fully standardised, since different types of aircraft fitted with the system have different capabilities. Approximately 15 airlines have purchased at the end of 1997 350 ship-sets of FANS-1 upgrades for the 747-400. The 777 includes FANS-1 as a basic feature and Boeing is certifying its 757, 767 and MD-90 with FANS-1 during 1998. FANS-1 will be an option on the 717. FANS-1 route includes principally south-pacific oceanic routes. Turkey to Singapore route is currently upgraded to accommodate FANS-1 operation. |
| FAP | Future ATM Profile |
| FAR | Federal Aviation/Acquisition Regulations |
| FCC | Federal Communications Commission US Telecommunications National Regulatory Authority |
| FDP | Flight Data Processing Part of the MADAP system used by UAC Centre of Eurocontrol. FDP is concerned with the determination of the route of flights, calculation of times and levels at significant waypoints, and the detection of potential separation infringements by extrapolation of flight trajectories and conflict analysis. This process uses basic flight plan information received from the Initial Flight Plan Processing System of the CFMU at Brussels, as well as aircraft performance data; wind and temperature data from the Meteorological Centre at Brussels. Through correlation with radar data, highly precise flight trajectories are calculated, actual positions of aircraft are determined and deviation from the intended route are monitored and corrected. The system is capable of automatically exchanging flight plan and flight coordination data with neighbouring ATS Units using Eurocontrol OLDI standard. |
| FDPS | Flight Data Processing System (radar & flight plan) |
| FFAS | Free Flight Airspace Airspace regime with: * known traffic environment * free routing and autonomous operations * responsibility for separation with the air |
| FFM | Free Flight Mode Operations comprising both free routing and autonomous separation |
| FIB | Forwarding Information Base A list of possible next-hop addresses used by the relay function of the internetwork sublayer for forwarding network protocol data units (NPDU). |
| FIR | Flight Information Region An airspace of defined dimensions within which flight information service and alerting service are provided. The world's airspace is divided into a series of contiguous FIRs within which ATS are provided. In some cases, the FIRs cover large oceanic areas with relatively low air traffic density, within which only flight information service and alerting service are provided. In other FIRs, large portions of the airspace are controlled airspace within which air traffic control services is provided in addition to flight information and alerting services. As well as being separated horizontally, airspace is divided vertically, generally being divided into Upper and Lower airspace at a specific altitude level (FL 195 or FL 245 in Europe): below this level is the FIR, where flights are controlled in the climb and descend phases (see also UIR). |
| FIS | Flight Information Service The FIS objective of providing advice and information for the safe and efficient conduct of flights will be complemented by a trajectory monitoring service (which will be progressively automated) to maintain situation awareness in the event that the aircraft requires assistance from the ground. The information includes significant meteorological (SIGMET) information, changes in the serviceability of navigation aids and in the condition of aerodromes and associated facilities and any other information likely to affect safety. IFR flights receive, in addition, information on weather conditions at departure, destination and alternate aerodromes, collision hazards to aircraft operating outside of control areas and control zones, and for flight over water, available information on surface vessels. |
| FITAMS | Flight Trials of ATN and Mode S ATIF sub-project. |
| FL | Flight Level A surface of constant atmospheric pressure which is related to a specific pressure datum (1013.2 hPa), and is separated from other such surfaces by specific pressure intervals. Measured in '00 feet. FL 330 means 33 000 ft. The terms "height" and "altitude" indicate altimetric rather than geometric heights and altitudes. |
| Flight plan | Specified information provided to air traffic services units, relative to an intended flight or portion of a flight of an aircraft. A flight plan must be filed with ATS units for all flights that will cross international borders and for most other flights that are engaged in commercial operations. The flight plan provides information on the aircraft's identity and equipment, the point and time of departure, the route and altitude to be flown, the destination and estimated time of arrival, and the alternate airport to be used should landing at destination be impossible. |
| FLIPCY | Flight Plan Consistency Service FLIPCY permits the ground system to check that flight data in the FDPS correspond to actual flight plan data from the aircraft. This service should take place automatically at logon. |
| FM | Frequency Modulation |
| FMG | Frequency Management Group |
| FMP | Flow Management Position Part of the Eurocontrol CFMU concept, and concurrent with the enhanced voice and data communications systems, FMPs have been installed in ACCs and major terminal control units throughout Europe. The FMPs provide a vital flow of information from operational ATC units to the CFMU while the units are able to display the up-to-date European airspace situation at any time. An important part of this information flow is the declared capacity of ATC units and airspace sectors as a function of time. The capacity situation can be seen dynamically at the CFMU and ATC unit FMP positions and thus allows the important tactical decisions to be made which balance capacity and demand on a continuous basis, avoiding delays and congestion. |
| FMS | Flight-Management System FMS is the on board centerpiece that gives the data processing power and offers the applications allowing automatic navigation and flying. FMS includes huge databases, and runs many applications using the information provided by the many sensors placed on board or sent through any communication channel available on board. There is presently almost a unique provider of FMS worldwide, Honeywell. Sextant Avionique and Smiths Industries are developing an alternative FMS. |
| FMU | Flow Management Unit |
| FPMLTS | Future Public Land Mobile Telecommunications System (ITU-R) see UMTS |
| FR | Frame Relay |
| FREER | Free Route Experimental Encounter Resolution |
| FUA | Flexible Use of Airspace (EATCHIP) The FUA concept addresses dynamic airspace sharing and co-ordination procedures between civil and military authorities. Plans exist to extend the FUA principles to encompass all civil airspace authorities. The FUA concept, adopted by the majority of European States in 1996, provides one of the foundations for more efficient management of airspace resources and improvements in route structures by releasing airspace in areas and at times when it was not previously available. This, in turn, will allow the introduction of both additional routes and more direct RNAV routes. Integrated civil and military airspace planning should be in place in most states by 2005, and will progressively lead to the whole airspace in Europe being regarded as a continuum under a collaborative joint civil-military airspace planning system by around 2010. |
| FY | fiscal year |
| FYROM | Former Yugoslav Republic of Macedonia |
| G/G | Ground/Ground |
| GAAS | Generic Approach for ATM Systems |
| GAT | General Air Traffic Civil and military traffic operating in accordance with ICAO rules and regulations, as opposed to OAT. |
| gate to gate | Characteristic of the future CNS/ATM service, starting from the first interaction of a flight with ATM to switch-off of the aircraft engines, including charging processes. A flight will be considered as a continuum from the departure gate to final destination gate. The primary objective will be for flights to operate as much close to their preferences, from first interaction with the ATM system and throughout the flight. |
| Gateway | A system used to interconnect dissimilar networks. A gateway may contain all seven layers of the OSI reference model. |
| GBAS | Ground-based Augmentation System see LAAS |
| GDLP | Ground Data-Link Processor |
| GDP | Gross Domestic Product |
| GEO | Geostationnary Circular orbit in the equatorian plan, where satellites are stable viewed from the earth surface. |
| GES | Ground Earth Station An earth station in the fixed satellite service, or, in some cases, in the aeronautical mobile-satellite service, located at a specified fixed point on land to provide a feeder link for the aeronautical mobile-satellite service |
| GETALIS | Sistema de Gestão do Tráfego Aéreo de Lisboa (Portugal) |
| GLONASS | Global Orbiting Navigation Satellite System Russian Federation satellite system for positioning services The GLONASS constellation comprises 24 satellites in at intermediate circular orbit at an altitude of 19 100 km. Three orbital planes are used inclined at 65.8°. |
| GMU | Global positioning system Monitoring Unit |
| GNSS | Global Navigation Satellite System (ICAO) A first phase, GNSS-1, will be based on existing systems, GPS, GLONASS and the overlay system currently developed in Europe, EGNOS, as well as other augmentation systems developed in North America and Japan. It will be followed by GNSS-2, which is to be fully under civilian control, at 2005 horizon. |
| GNSS 2 | Global Navigation Satellite System Phase 2 GNSS2 is widely studied, but no architecture has been yet fixed. Its design should provide an optimum trade-off between performance and cost, in a particular environment where the GPS and GLONASS systems are available free of charge, for periods of respectively 10 and 16 years from 1996. There are currently different proposals for GNSS 2 systems, for example from Europe (Alcatel) and USA (Boeing GPS IIF system). |
| GPRS | General Packet Radio Service GPRS is a packet-switched mobile data communication service that is widely accepted to be the next major step forward in the evolution of GSM technology. GPRS, further enhancing GSM networks to carry data communication services, is also an important component in the GSM evolution entitled GSM+. GPRS enables high-speed, user-friendly mobile data communication usage with efficient use of existing bandwidth. It is optimal for "bursty" data applications such as mobile Internet services, e-mail and push technologies. A minimum data rate of 115 kbps is foreseen for the service. |
| GPS | Global Positioning System US satellite system for positioning services. GPS is a system of 24 satellites owned and managed by the Department of Defence of the USA. GPS signals, with the best navigation and positioning accuracy of up to 16 meters, are continuously encrypted, allowing their use only by receivers delivered to US military forces, those of allied nations and to other approved users. Non encrypted GPS signals are made available to the public, but they are deliberately degraded to an accuracy of 100 meters. The 24 satellites are placed in an intermediate circular orbit at an altitude of 20 000 km. Satellites are arranged on six planes, each containing 4 satellites. These planes are inclined at 55° relative to the equatorial plane. |
| GPWS | Ground Proximity Warning System GPWS alerts pilots when they may be too close to the ground |
| GRADE | Graphical Analysis and Design Environment |
| GRAS | Ground Regional Augmentation Systems Equivalent to LAAS |
| GTTR | Guaranteed Total Time to Repair Defined as the time needed to restore a service, but not necessarily the time to repair a failing element, when the latter has an automatic back-up, e.g. a local access loop which is automatically switched to the local PSTN or ISDN in case of failure; generally expressed in hours. |
| HSCSD | High Speed Circuit Switched Data HSCSD is a GSM service that will combine two or more time-slots in the GSM radio channel to deliver a circuit switched data service of up to 64 kbps or more. |
| HF | High Frequency Radio communications using HF band for over the horizon contacts have reliability limitations due mainly to the variability of propagation characteristics. It is envisaged that with the increased use of AMSS in oceanic/remote areas, congestion on HF channels will be relieved and eventually, the use of HF for routine communications will diminish. |
| HIPS | Highly Interactive Problem Solver Application developed by Eurocontrol that help controllers, in densely-packed European airspace, match issued clearance to new request from the aircraft as closely as is safely possible. |
| HMI | Human Machine Interface |
| HMU | Height Monitoring Unit |
| HR | Human Resources |
| Hub and Spoke | A hub is a central (and usually) major international airport, while spokes are the routes between the hub and other (mostly) regional airports. |
| IAA | Irish Aviation Authority |
| IATA | International Air Transport Association |
| ICAO | International Civil Aviation Organisation |
| IDA | European R&D program. The mission of IDA Programme is to implement telematics networks and services between public administrations across Europe, while establishing various projects in many sectors of administration to ensure maximum interoperability and cost effectiveness. |
| IFALPA | International Federation of Air Line Pilots Association |
| IFATCA | International Federation of Air Traffic Controllers' Association |
| IFPS | [Integrated] Initial Flight Plan Processing System IFPS is an essential application of the Eurocontrol CFMU. The IFPS is a pivotal element in the provision of an efficient flow management service, providing for the first time a dynamic picture of the complete European air traffic situation. Before the IFPS was established, operators have to send each flight plan individually to every ATC unit whose airspace the flight would pass through. Now, for flights within the IFPS airspace, operators send the flight plan only to the IFPS, which acknowledged receipt, processes the data, stores it in the CFMU database and sends the information to the ATC units which will be concerned with the flight. Flight plans containing errors are referred for manual processing to an IFPS operator at either Harren or Bretigny, depending on the aerodrome of departure of the flight. |
| IFPU | IFPS Unit There are two IFPS units, one located with the CFMU at Harren, and the other at Brétigny-sur-Orge, south of Paris. These two units provide a vital contingency back-up to each other and are responsible for complementary geographic areas for day-to-day operation. |
| IFR | Instrument Flight Rules An aircraft must be flown with either the VFR or the IFR. Most airliners fly under IFR at all times. To fly under IFR, an aircraft must be equipped with suitable instruments and navigation equipment appropriate to the route to be flown. When operating under ATC, the aircraft must maintain precisely the route and altitude that have been assigned to it and keep ATC informed about its position. Flights operating under IFR are either kept separated by ATC units or provided with collision hazard information. Under IFR, the aircraft fly from one radio aid to the next or by reference to self-contained airborne navigation equipment from which the pilot can determine the aircraft's position at all times. All aircraft flying wholly or partly under IFR are normally liable to pay en route charges. |
| IIMES | Initial Implementation of Mode S Enhanced Surveillance |
| ILS | Instrument Landing System ILS transmits two radio beams telling pilots when aircraft is correctly positioned for landing: "localiser", aligned with runway centreline and "glide path" giving angle of descend. Airborne and ground systems can be linked for automatic landings so operations can continue even in bad weather. |
| IMC | Instrument Meteorological Conditions Meteorological conditions expressed in terms of visibility, distance from cloud, and ceiling, less than the minima specified for visual meteorological conditions. |
| IMO | International Maritime Organisation |
| IMT-2000 | International Mobile Telecommunications 2000 (ITU) see UMTS |
| INESC | INstituto de Engenharia de Sistemas e Computadores |
| Inmarsat | International Maritime Satellite Organisation |
| Internetwork | A set of interconnected, logically independent heterogeneous subnetworks. The consistent subnetworks are usually administered separately and may employ different transmission media. |
| IP | Internet Protocol Network Layer Protocol used in the Internet Network |
| IPAS | Integrated Preparation and Analysis System |
| IRMA | Intermediate Redesign of MCC Architecture |
| IRR | Internal Rate of Return |
| IS | Intermediate System |
| ISO | International Standards Organisation |
| ITU | International Telecommunication Union |
| JAA | Joint Aviation Authorities The JAA are an associated body of ECAC representing the civil aviation regulatory authorities of a number of European States which have agreed to co-operate in developing and implementing common safety regulatory standards and procedures. 19 ECAC members states are full members of the JAA. |
| J-TSO | JAA - Technical Standard Order Technical Standard set-up by the European JAA. Similar to the US TSO. |
| kHz | kilohertz |
| LAAS | Local Area Augmentation Systems equiv. GBAS (ICAO) The GNSS as designed does not satisfy all civil aviation requirements for navigation and landing: augmentations will be required to improve the system's integrity, provide sufficient accuracy for precision approaches and assure availability necessary to meet required navigation specifications. LAAS are systems that enable the GNSS to meet air navigation requirements for terminal, non precision approach, precision approach (Category II and III), autolanding and ground taxi phases of flight. The LAAS architecture includes a ground station equipped with multiple GNSS receivers. The location of the ground station compares the position calculated using its GPS receivers to an actual surveyed position of the station. The difference between these positions is placed in a correction message which is broadcast to all users within a 20-30 mile radius. Properly equipped aircraft can compare these LAAS corrections to their GPS-derived positions, and make adjustments accordingly. Using this corrected position, the pilot can fly a precision approach to the surface of the runway. Curved approach paths, impossible using the current instrument landing system, will be possible for Cat II & III aircraft under LAAS. Approaches could be designed to avoid obstacles, noise sensitive areas, or congested airspace. Unlike current landing systems, LAAS can provide precision approach capability to multiple runways. Additionally, LAAS will provide a new service - surface navigation in low visibility. LAAS alone or in combination with other augmentation systems is intended to support all precision approach and landing operations down to Cat II (RNP 0.01/15) and III (RNP 0.003), and surface operation (RNP 0.003) as well. |
| LAN | Local Area Network A network connecting various data communications devices in a localised geographical area such as a single office building or a group of building. |
| Layer | Most networks are organised as a series of layers, each built upon the one below. Layer n on one machine communicates with layer n on another machine. Rules and conventions used in this communication are collectively known as the layer n protocol. |
| LEO | Low Earth Orbit Orbit in the 800-1500 km range from the surface of the earth. The industry is setting a few telecommunications satellite constellations on such orbits (Iridium, Globalstar…). |
| LINCS | Leased Interfacility NAS Communications System |
| Link Handoff | The process by which peer LMEs, already in communication with each other, create a link between an aircraft and a new ground station before disconnecting the old link between the aircraft and the current station. Following link establishment during a handoff, the link that was previously the "current" link becomes the "old" link. |
| LME | Link Management Entity A protocol state machine capable of acquiring, establishing, and maintaining a connection to a single peer system. An LME establishes data link and subnetwork connections, "hand-off" those connections, and manages the media access control sub-layer and physical layer. An aircraft LME tracks how well it can communicate with the ground station of a single ground system. An aircraft VDL Management Entity (VME) instantiates an LME for each ground station that it monitors. Similarly, the ground VME instantiates an LME for each aircraft that it monitors. |
| Loran | long–range navigation Loran C operates on 100 kHz and is a maritime and aeronautical radio navigation system. It is a long range radionavigation system that possesses an inherent high degree of accuracy at ranges up to 1900 km. Loran-C was originally developed for military use, but was selected for civil maritime because of its high accuracy and relatively inexpensive receiving equipment. |
| LOS | Line of Sight |
| LRR | long–range radar |
| LTEP | Legal and Technical Experts Panel ICAO Panel established in December 1995 to study the legal framework required by the introduction of GNSS. |
| LVB | Luchtverkeers-beveiliging (NL) |
| MAC | Mid-Air Collision |
| MADAP | Maastrich Automatic Data Processing and Display System The UAC centre in Maastrich operated by Eurocontrol is based on an advanced data processing complex that collects, processes and stores the information needed to establish and maintain a real-time synthetic picture of the air traffic situation for all the airspace controlled by Maastrich UAC. The system is called MADAP. |
| MAS | Managed Airspace Airspace regime with: * known traffic environment * 3D routes and routings * responsibility for separation with the ground |
| MASPS | Minimum Aircraft Systems Performance Specifications/Standard |
| MATSE | Meeting [of ECAC Transport Ministers] on the Air Traffic System in Europe |
| MCC | Main Computer Complex |
| MCDRS | Meteorological Data Collection and Reporting System Aircraft equipped with MCDRS provide upper-air meteorological data, especially wind and temperature measurements and possibly humidity measures. One drawback to current aircraft-based data is that most of the observations are being collected along a limited number of fixed routes, so that horizontal and vertical gradients in atmospheric conditions are not well resolved. Under free flight rules, allowing operators to fly preferred routes with aircraft equipped with MCDRS capability will help improve this situation. |
| MCI | Mode C intruder |
| MECU | Millions of ECU |
| MET | Meteorological Services for Air Navigation |
| METAR | Meteorological Aerodrome Report |
| MFUR | Most Frequently Used Route |
| MLS | Microwave Landing System MLS was selected in 1978 by ICAO as the international standard precision approach system, with implementation targeted for 1998. The MLS was expected to gradually replace ILS in national and international civil aviation. The MLS has a number of advantages over the ILS because the MLS signals can be used to support curved approaches. The MLS operates in the 5000-5150 MHz band. |
| Mode C | altitude reporting mode of secondary radar |
| Mode S | Selective Co-operative Secondary Surveillance System (Mode Select) SSR Selective Interrogation Mode SSR mode S data link provides surveillance capability. SSR mode S air/ground data link refers to Mode S Subnetwork. |
| Mode S Subnetwork | A means of performing an interchange of digital data trough the use of secondary surveillance radar (SSR) mode S interrogators and transponders in accordance with defined protocols. An air ground data link is specifically suitable for limited data messaging in high-density areas. Mode S operates on 1030 and 1090 MHz. |
| modem | modulator–demodulator |
| MOPS | Minimum Operation Performance Standard RTCA and EUROCAE defined level of standard |
| MOSD | Mission, Objectives and Strategy Document (EATMS) |
| MSAW | Minimum Safe Altitude Warning MSAW notifies controllers of aircraft which are at risk of flying too close to the ground. |
| MSS | Mobile Satellite Service MSS is an ITU-defined service in which satellites are used to deliver communications services (voice or data usually, one -or two-way) to mobile users such as cars, trucks, ships and planes. It is a generic term that encompasses several types of mobile services delivered by satellite, including Maritime MSS (MMSS), Aeronautical MSS (AMSS) and Land MSS (LMSS). |
| MSSR | Monopulse Secondary Surveillance Radar |
| MTCD MTCA | Medium-Term Conflict Detection Medium-Term Conflict Alert MTCD will provide the means to look ahead for periods of around 20 minutes to identify and resolve potential conflicts. This will reduce the ATC workload associated with routine monitoring and conflict detection, and the number of times that ATC has to intervene and change flight profiles to resolve potential conflicts. The performance of the tools will depend to a large extend on the degrees of interoperability of FDP systems, and the quality and accuracy of the flight data used for trajectory prediction. |
| MTOW | Maximum Take-off Weight |
| MTTR | Mean Time To Repair The average elapsed time from the occurrence of an incident to resolution of the incident. |
| MUAC | Maastrich Upper Airspace Control Centre Eurocontrol UACC covering Benelux countries and part of Germany located in Maastrich. |
| Multicast | A transmission intended to be received by multiple stations |
| Multilateration | Technique that allows the identification and location of an aircraft on ground. The precise control of aircraft movements at an airport is essential if the airport is to operate safely and to maximum capacity. Controllers at airports use primary radar to track aircraft movements, but this means does not provide identification of individual aircraft. Multilateration is a technique, using Mode S transmissions broadcast from aircraft, which not only provides position information, but also solves the identification problem. By timing the arrival of Mode S signals at a number of airport's sites, it is possible to work out an aircraft's position by triangulation and to know its identity, since Mode S address is contained in the broadcast message. |
| MXEU | Millions of ECU (Financial Context) |
| NADIN | National Airspace Data Interchange Network (USA) |
| NAILS | National Airspace Integrated Logistics Support (USA) |
| NAS | National Airspace System (USA) |
| NAS/ATMOs | National Administrations/Air Traffic Management Organisation |
| NASMAP | NAS management automation program (USA) |
| NASPAC | National Airspace System Performance Analysis Capability |
| NAT SPG | North Atlantic Systems Planning Group |
| NATO | North Atlantic Treaty Organisation |
| NATS | National Air Traffic Services Navigation and ATC Service provider in the UK. Public Corporation owned by the Government. |
| NEI | Noise Exposure Index |
| NLR | Nationaal Lucht-en-Ruimtevaartlaboratorium (NL) |
| NM | Nautical Miles |
| NOTAM | Notice To Airmen A notice containing information concerning the establishment, condition or change in any aeronautical facility, service, procedure or hazard, the timely knowledge of which is essential to personnel concerned with flight operations. Class 1 distribution means distribution by means of telecommunications. Class 2 distribution means distribution by means other than telecommunications. |
| NPV | Net Present Value |
| NSCC | Non-Share Capital Corporation Specific type of corporation that are allowed under the Canada Corporations Act or various provincial equivalents. Nav Canada, the entity in charge of Air Navigation Services in Canada, has been incorporated as an NSCC in 1996. In the establishment of an NSCC, the constating documents define a number of key items that are referred to as the fundamental characteristics. Theses include the method of determining who will be members, the method of determining who will be directors, and the specification of the objects of the company. Members essentially fulfil the role of shareholders, except that they will never receive dividends or any other distributions from the corporation. Ideally, an NSCC will not be controlled by any one constituency. The objects of the corporation will essentially set out its reason for existence and define the activities that it can undertake; they are particularly important in the absence of the typical goal of maximising shareholder value. The originating government will play a major role in defining the Fundamental Characteristics of an NSCC. Once created, the corporation will be independent of the government and able to act like any other corporation. Such a corporation is similar to other non profit organisation legislation in other countries except that it has nearly a full corporation capability. |
| OACC | Oceanic Area Control Centre |
| OAT | Operational Air Traffic Flights that cannot comply with the rules for GAT, essentially military flights. |
| OCD | Operational Concept Document (EATMS) |
| Oceanic Navigation | Outside of the coverage of VHF communications, pilots and controllers communicate through HF operators. Navigation is based mostly on inertial navigation systems and flight management systems. While surveillance radar coverage may be provided on scarce islands (like Iceland on North Atlantic routes), beyond radar coverage, ATC relies on position reports by pilots for surveillance air traffic. |
| ODIAC | Operational Development of Initial Air/Ground data Communications Services |
| ODID | Operational Display and Input Development |
| ODS | Operator Display System |
| OFIS | Operational Flight Information Service HF or VHF OFIS broadcast are provided according to regional air navigation plan. The broadcast message should not exceed 5 minutes, be continuous and repetitive, and contains meteorological information and operation information concerning navigation aids and aerodromes. |
| OLDI/SYSCO | On-Line Data Interchange/System Coordination |
| ONP | Open Network Provision Principle of the European Commission Telecommunication Policy. The concept of ONP aims at ensuring that access to networks is provided to users, service providers and other network operators on a transparent, non-discriminatory basis and at a fair and reasonable cost. ONP measures have been adopted in two successive phases: the first addresses the situation where a telecommunications organisation retains monopoly powers over network and most service provision and the second addresses the post 1998 situation when the telecommunication network is fully liberalised. During the first phase, in 1990, the Community adopted an ONP framework Directive (90/387/EEC) which established general principles related to the provision use of networks and services, tariff principles and technical interfaces of network connections. It then adopted a series of implementing ONP measures designed to flesh-out the details for applying these principles to specific sectors (ONP Leased Line Directive -92/44/EEC; ONP Packet Switched Data Services Recommendation -92/382/EEC; ONP ISDN Recommendation -92/383/EEC; ONP Voice Telephony Directive -95/62/EEC; ONP Principles applied to Interconnection -97/33/EC). The ONP measures have established an array of rights and obligations which in general seek to benefit end-users and service providers, for example, by ensuring: (i) a minimum set of service offerings; (ii) open access to end use of networks and services; (iii) non-discrimination on terms and tariffs vis-à-vis the incumbent and others; (iv) tariffs are fair, cost-oriented and unbundled and assured through the National Regulatory Authority (NRA) approved cost accounting procedures; (v) service quality through a quality target reporting scheme; and (vi) the functional and legal independence of NRA. |
| OOOI | Out-Off-On-In Sequence of flight events (out of gate, take-off, landing and parking). Each of these events generated the first ACARS reports, send by aircraft to their operation centres. |
| OPMET | Operational Meteorological Data |
| ORCAM | Originating Region Code Allocation Method |
| Outsourcing | A strategy whereby an organisation contracts the provision of its requirements for services such as telecommunications, data networking or information technology to another independent organisation. |
| P-Channel | AMSS Channel. Packet mode time division multiplex (TDM) channel transmitted continuously from the aeronautical ground earth station (GES) in the to-aircraft direction to carry signalling and user data. At least one P-Channel is transmitted continuously by each GES that forms part of the AMSS service. Each AES must continuously monitor the P-Channel transmitted by the GES to which it is logged on. |
| PANS | Procedures for Air Navigation Services |
| PAR | Precision Approach Radar |
| PATIO | Platform for ATM Tool Integration up to pre-Operation |
| PAX | Passenger |
| PD | PHARE Demonstration |
| PDU | Protocol Data Unit A unit of data transferred between peer entities within a protocol layer consisting of protocol control information and higher layer user data (i.e. service data units). |
| PEAX | Pan-European ATS X25 Pan-European Network formed by the interconnection of European ATSOs X25 networks. |
| PETAL | Preliminary EUROCONTROL Test of Air/Ground data Link |
| PHARE | Poland-Hungary: Aid for the Reconstruction of the Economy (EC) |
| PHARE | Programme for Harmonisation of ATM Research in EUROCONTROL PHARE is a research programme investigating the concept of an integrated air/ground ATM system for 2005+. PHARE partners are CAA, DRA, CENA, STNA, DFS, DLR, LVB, NLR, EEC with 50% funding from Eurocontrol. It has developed "tools" for both air and ground components in support this concept, namely: experimental 4D FMS and HMI, trajectory predictor, conflict probe, flight path monitor, negotiation manager etc. |
| PIAC | Peak Instantaneous Aircraft Count |
| PIRG | Planning and Implementation Regional Group |
| POEMS | Pre-Operational European Mode S |
| PPD | Pilot Preferences Downlink PPD permits the downlink of pilot preferences to the ground for display to controllers. These preferences relate to flight parameters having operational implications for ATC and not requiring controller response. |
| PRAISE | Preparation of an R&D Programme in Support of EAMTS |
| PRC | Performance Review Commission Commission established under Article 7.3 of the Eurocontrol revised convention "to ensure the effective management of the European air traffic management system through a strong, transparent and independent performance review and target setting system". The PRC is supported by the Performance Review Unit (PRU) operating administratively under the Eurocontrol agency, but independent from it. |
| PROATN | Prototype Aeronautical Telecommunications Network PROATN is part of CEC DG13's ATC related 4th framework Programme in which Eurocontrol has a specific interest. PROATN main objective is to produce a deployed ATN infrastructure which allows the validation and demonstration of the ATN related issues of the ICAO CNS/ATM concept before it is implemented for operational use in the Member States; it will be developed as a pre-operational and pre-industrial prototype ATN of which the key elements are certifiable on an end-to-end basis. |
| Protocol | A set of rules and formats (semantic and syntactic) which determines the communication behaviour between peer entities in the performance of functions at that layer. A set of layers and protocol is called network architecture. A list of protocols used by a certain system, one protocol per layer, is called a protocol stack. |
| PSDN | Public Switched Data Network Data network managed by a public operator |
| PSR | Primary Surveillance Radar |
| PSTN | Public Switched Telephone Network Telephone network managed by a public operator |
| Pushback | Pushback Clearance Request/Delivery service is used by the aircrew to request to and obtain from the ground controller a pushback clearance. |
| PV | Present Value |
| QoS | Quality of Service |
| R&D | Research and Development |
| R-Channel | AMSS Channel. Random access (slotted Aloha) channel used in the from-aircraft direction to carry signalling and user data. All AESs will log on to AMSS using a channel that has been designated for that purpose in the particular service area of the AES. For the duration of its log-on period, an AES will be assigned one or more R-Channels (at the appropriate channel rates). Over these R- Channels, the AES can transmit signalling, short data packets, and requests for additional capacity on the T-Channel. |
| Radnet | Radar Network Four states (Belgium, Germany, Luxembourg and Netherlands) and Eurocontrol radar distribution network, used for the Maastrich UAC Centre. The development of Radnet enables all ATC Centres in the 4 states area to have access to radar data from over 30 radar stations scattered throughout the area. |
| RAF | Reference ATN Facility |
| RAFC | Regional Area Forecast Centre (Meteorology) |
| RAMS | Reorganised ATC Mathematical Simulator |
| RAP | Recognised Air Picture |
| RASA | Requirements Analysis and System Architecture (EATCHIP Subgroup) |
| RASCAL | Radar Sharing Calculation |
| RASS | Radar Analysis Support System |
| RASS-C | RASS - Centre |
| RASS-S | RASS - Site |
| RCC | Rescue Coordination Centre |
| RCL | Radio Communications Link |
| RCP | Required Communication Performance RCP is a statement of the operational communication performance delay, integrity and availability necessary for flight within a defined airspace, or for an aircraft to perform a specified operation or procedure. RCP concept is in development and not yet approved by ICAO. |
| RCR | Routing and Circuit Restoral |
| RDP | Radar Data Processing RDP System (RDPS) is part of the MADAP System of the Eurocontrol UAC. RDP starts at the radar site itself. The analogue signals from both primary and secondary radar sub-systems are processed by video extractors to obtain target position, altitude and identity codes. This information is assembled into a target report message which is transmitted to Maastrich UAC either via dedicated lines or via the Radnet network. |
| RF | radio frequency |
| RFI | Radio Frequency Interference |
| RFP | Request For Proposal |
| RFS | Radar Fallback System |
| RHSM | Reduced Horizontal Separation Minima Reduced Horizontal Separation from 60 NM to 30 NM, with minimum time between aircraft cut by half to about five minutes. |
| RLS | Reliable Link Service A data communications service provided by a subnetwork which automatically provides for error control over its link through error detection and requested retransmission of signal units found to be in error |
| RMCDE | Radar Message Conversion and Distribution Equipment |
| RML | Radar Microwave Link |
| RMP | Required Monitoring Performance RMP refers to capabilities of an airspace user to monitor other users and be monitored by other users to a level sufficient for participation of the user in specified strategic and tactical operations requiring surveillance, conflict assessment, separation assurance, conformance and/or collision avoidance functions. RMP is intended to characterise aircraft trajectory prediction capability and received accuracy, integrity, continuity of service, and availability of a monitoring system for a given volume of airspace and/or phase of operation. |
| RNAV | Area Navigation A method of navigation which enables aircraft operation on any desired flight path within the coverage of station-referenced navigation aids or within the limits of the capability of self-contained aids, or a combination of these. Airborne RNAV equipment automatically determines aircraft position by processing data from one or more sensors and guides the aircraft in accordance with appropriate routing instructions. Additional navigation parameters such as distance and bearing to a preselected waypoint can also be computed from the aircraft position and the location of the waypoint, depending upon the capability of the RNAV equipment. Position can be displayed to the pilot in various ways, most practically in terms of the aircraft position relative to precomputed desired track. RNAV was identified by ICAO FEATS as the proposed navigation system in the EUR Region. In 1990, the ECAC Transport Ministers agreed a strategy which includes requirements for the mandatory carriage of on-board RNAV equipment from 1998. Aircraft not equipped and approved for RNAV operations will not be permitted in RNAV airspace. |
| RNP | Required Navigation Performance Level A statement of the navigation performance accuracy necessary for operation within a defined airspace. RNP "types" have been defined to provide existing and future levels of navigation accuracy. They are identified by a single value expressed in nautical miles (NM). For example, RNP 10 specifies the accuracy of navigation, whereby it is assumed that all flights will be within 10 NM of the intended position for 95% of the total flying time. The RNP types identified for en-route operations are RNP 1, 4, 10, 12.6 and 20. RNP 5, a derivative of RNP 4, was provided for an interim period in order to permit the continued use of present navigation equipment without modification of existing route structures. RNP 20 is the present minimum requirements of international air navigation. The following table shows the main aspects of RNP types: RNP Type Spacing Separation minima Implementation (accuracy) (lateral separation) (longitudinal distance-based separation) 20 100 NM 80 NM Worldwide 12.6 60 NM 50 NM North Atlantic 10 50 NM North Pacific 5/4 16.5 unidirectional Europe 18 bi-directional 1 Future To day, as far the accuracy of the system is concerned, the B-RNAV can be considered to equate RNP 5; thus, simplistically, B-RNAV=RNP5 RNAV. However, the expected introduction of the RNP n RNAV concept, with n<5 (only from post 2005) will introduce functional and operational requirements for RNAV equipment. An RNP type approval does not mean that an aircraft may be operated wherever that RNP type applies. An RNP type approval is specific to a particular type of navigation equipment and application (FMS), in relation to ground infrastructure. For example, an aircraft could be approved for RNP 5 in the B-RNAV airspace of Europe using RNAV equipment requiring input from ground-based navigation aids such as VOR/DME. This same aircraft could not operate in RNP 10 airspace where no ground-based navaids are available. |
| ROA | Return on Assets |
| ROCE | Return on Capital Employed |
| Router | An intermediate system used to interconnect subnetworks conforming to the OSI protocol architecture. It is a special form of gateway in which only the lower three layers need to be implemented to achieve interconnection. |
| RPF | Rapid Prototyping Facility |
| RPL | Repetitive Flight Plans |
| RSO | Route per State Overflown |
| RSP | Required System Performances RSP is intended to characterise airspace and/or aircraft operating in airspace, and the level of separation applicable. The concept of RSP necessarily contains elements of navigation, communication and surveillance performance and is related to RCP, RNP and RMP. These RSP components establish the basis for an environment in which operational access is explicitly performance-based, in place of current practice in which the basis of approval is indirect and implicitly related to capability, based on equipage sets including navigation sensors used. |
| RTCA | Requirements and Technical Concepts for Aviation Inc. RTCA is a private, not-for-profit organisation that addresses requirements and technical concepts for aviation. RTCA products are recommended standards and guidance documents that focus on the application of electronics technology to implement new or modified concepts and to satisfy related requirements. RTCA was organised as the Radio Technical Commission for Aeronautics in 1935. It changed its name in 1991. RTCA has served the aviation community ever since by continuously providing a forum where government and industry representatives gather to address aviation issues and to develop consensus based recommendations. RTCA is primarily funded by dues from Members, Academic Associates and International Associates. Current membership includes approximately 145 United States government and business entities. Consensus is central to the process by which RTCA recommendations are developed. As aviation communication, navigation and surveillance requirements and related technical concepts begin to evolve, RTCA is frequently asked to form a special committee that will consider the topic and recommend minimum operational performance standards (MOPS) or appropriate technical guidance documents. RTCA Technical Management Committee discusses the topic and, based on consensus, initiates special committee action. This activity includes selecting a special committee chairman and providing terms of reference for the activity. During the special committee meetings, volunteers from government and industry explore the operational and technical ramifications of the selected topic and develop consensus recommendations. These recommendations are then presented to the RTCA Technical Management Committee, which either approves the special committee report or directs additional special committee work. Recommendations often provide the basis for government policy decisions. RTCA works closely with its European counterpart, EUROCAE. |
| RVSM | Reduced Vertical Separation Minima Reducing the vertical separation between aircraft over specific routes (like North Atlantic) from 2,000 feet to 1,000 feet increases capacity by effectively doubling the number of routes. With effect from 27 March 1997, the first phase of RVSM was implemented in the North Atlantic minimum navigation performance specification (MNPS) airspace. This enables RVSM-approved aircraft, initially between flight levels 330 and 370 inclusive, to operate with 1000 ft vertical separation instead of 2000 ft, as was previously required. Ultimately, it is intended to extend the application of RVSM to all levels between 290 and 410 inclusive. Aircraft which want to take advantage of the reduced separation have to get airworthiness approval. To ensure they can cope with 1,000 feet separation, they have to comply with stringent altimetry system standard. Height-keeping performance is measured by a specific station in West Wales (UK) measuring the ability of each aircraft to keep correct height, on behalf of the North Atlantic Systems Planning group (NAT SPG). Each individual aircraft able to keep its height is recorded in a database. Some airlines have rotated their aircraft to make sure that each one in their fleet has flown over the Wales station to have its performance recorded and then be able to fly in RVSM. Introduction of RVSM of 1000ft between aircraft operating in the upper European airspace is planned in 2001. This will increase the physical capacity of the airspace and help reduce traffic complexity by providing more opportunities to separate opposite-direction routes and crossing traffic vertically. It will also allow more aircraft to fly close to their optimum flight level. |
| SADIS | WAF (Weather Aeronautical Forecast) System installed in London |
| SAP | Service Access Point The layer n SAPs are the places where layer n+1 can access the service offered. Each SAP has an address that uniquely identifies it. |
| SAP | System Access Parameters SAP Service addresses the automatic delivery of position data from aircraft to automated system assisting the controllers, e.g. in detection and the resolution of conflicts or route deviation. |
| SAPPHIRE | Satellite and Aircraft Database Programme for System Integrity Research Sapphire will allow investigation of various integrity algorithms to be carried out and will contribute to the certification of satellite navigation. Satellite navigation data collected by Lufthansa and Crossair on their commercial flights will be processed. This will be supplemented later by data from British Airways. |
| SAR | Search and Rescue |
| SARPs | Standard and Recommended Practices (ICAO) Art. 37 - Chicago Convention: "Each contracting State undertakes to collaborate in securing the highest, practicable degree of uniformity in regulations, standards, procedures, and organisation in relation to aircraft, personnel, airways, and auxiliary services in all matters in which such uniformity will facilitate and improve air navigation. To this end, the ICAO shall adopt and amend from time to time, as may be necessary, international standards and recommended practices (SARPs) and procedures". The domains concerned with SARPs are described in the annexes of the Chicago Convention. |
| SATCOM | Satellite Communications |
| SBAS | Satellite-based Augmentation System see WAAS |
| Separation | Separation assurance ensures that aircraft in the air and on the ground are safely separated from hazards and other aircraft. In the present ATM concepts, separation assurance in controlled airspace is the responsibility of the ground ATC team, and in particular the tactical radar controller. It is the workload associated with this process which is the biggest limitation on capacity increase. |
| SFE | Selectable Furnished Equipment |
| SICTA | Sistemi innovati per il controllo del traffico aereo (Italy) |
| SID | Standard Instrument Departure |
| SIGMET | Information issued by a meteorological watch office concerning the occurrence or expected occurrence of specified en-route weather phenomena which may affect the safety of aircraft operations |
| SIM5+ | EEC Real-time simulator (Enhanced Fifth Generation) |
| SIMMOD | Simulation Model |
| SITA | Société internationale de télécommunications aéronautiques |
| SLA | Service Level Agreement A written agreement or "contract" between the customers and the service provider which documents the agreed service levels for a service. For a network service, it typically covers service hours, service availability, customer support levels, throughputs and terminal response times, restrictions, functionality and the service to be provided in a contingency. It may also include security and accounting policy and can include penalties. |
| SMGCS | Surface Movement Guidance and Control System The ground control of aircraft is conducted through radar or visual means. Automation to support surface movement guidance and control systems of aircraft and vehicles is today lacking, and many major airports operate in gridlock conditions during periods of peak demand. SMGCS may optimised the use of runway, taxiway, stand and apron resources in a gate-to-gate strategy. |
| SNOWTAM | A special series NOTAM notifying the presence or removal of hazardous conditions due to snow, ice, slush or standing water associated with snow, slush and ice on the movement area by means of a specific pro-forma. |
| SPS | Standard Positioning Service |
| SRC | Safety Regulation Commission A Commission established by the Eurocontrol Council to ensure, through cooperation between States on safety regulation, consistent high levels of safety in air traffic management within the ECAC area. |
| SSR | Secondary Surveillance Radar A surveillance radar that is used to determine the position of an aircraft in range and azimuth. A SSR is a surveillance radar system that uses transmitters/receivers (interrogators) and transponders on board. |
| STAR | Standard Terminal Instrument Arrival Route |
| STCA | Short Term Conflict Alert STCA is an automated system which alerts controllers to potential conflicts in the short term (the next two minutes) between aircraft via the radar display. STCA recognises an aircraft by reference to its Mode A code. Multi-radar track data passes through a "coarse filter" to determine those pairs of aircraft which are or could possibly come into conflict within the next two minutes. |
| STDMA | Self-organising TDMA |
| Subnetwork | An actual implementation of a data network which employs a homogeneous protocol and addressing plan, and is under control of a single authority. |
| SYSCO | System Supported Co-ordination |
| T-ADLP | Trials Aircraft Data Link Processor |
| T-Channel | AMSS Channel. Reservation TDMA channel used in the from-aircraft direction only. The receiving GES reserves time slots for transmissions requested by aircraft earth stations (AESs) according to message length. The sending AES transmits the message in the reserved time slots according to priority. For the duration of its log-on period, an AES will be assigned one or more T Channel frequencies (at the appropriate channels rates). These channels are shared with a number of other AESs on a demand-assigned basis. That is, these channels are assigned on a conflict-free basis to individual AESs by the GES for short period of time. The requests for T-Channel capacity can be made over the R-Channel, or over the T-Channel, if capacity has already been assigned. |
| T-GDLP | Trials Ground Data Link Processor |
| TACAN | Tactical Air Navigation Military version of the DME |
| TACT | Tactical Computer System (CFMU) Tactical ATFM is the work carried out on the current operational day. Flights taking place on that day receive the benefit of ATFM, which includes the allocation of individual aircraft departure times, re-routings to avoid bottlenecks and alternative flight profiles to maximise efficiency. |
| TACT | Tactical Flow Management Working Position |
| TAF | Terminal Aerodrome Forecast |
| TAR-TTS | Trial ATN Router and Transport Server Systems used in ATIF project. The TAR-TTS product comprises the ATN Internet functionnality and as such is the most complex ATN component providing configurable mobile and ground ATN routers. Portation to a PC-Unix environment has been completed: TAR-TTS supports an interface to ARINC 429, the standard aircraft bus. |
| Taxi | Taxi Request/Delivery Service is used by the aircrew to request to and obtain from the ground controller taxiing guidelines before take-off. |
| TCAS | Traffic Alert and Collision Avoidance System TCAS tells pilots of aircraft in their immediate vicinity and provides avoidance instructions when it detects a potential conflict. TCAS generates two levels of alert, the first of which, a Traffic Advisory, is a simple warning of a potential threat. The second level alert, a Resolution Advisory, warns that there is a risk of imminent collision and advises the pilot of the appropriate avoiding action. In 1993, TCAS was made mandatory for all larger passenger airliners in US airspace. TCAS will be mandated in Europe for large aircraft from 1st January 2000. TCAS system is based on on-board radar and altitude transmission capability from other aircrafts (whatever the means). |
| TCP | Transmission Control Protocol Connection oriented transport layer protocol used in the Internet Network. Part of the TCP/IP stack related to the transport layer. |
| TDM | Time Division Multiplex A channel sharing strategy in which packets of information from the same source but with different destinations are sequenced in time on the same channel |
| TDMA | Time Division Multiple Access A channel sharing strategy in which packets of information from different sources and with the same or different destinations are sequenced in time on the same channel |
| TEN | Trans-European Network |
| TES | Trial End System System used in ATIF project. The TES product comprises the ATN upper layers and ADS, CPDLC and CM applications. These upper layers are integrated with TAR-TTS on the same platform using the transport service interface. |
| TESTA | Trans-European Services for Telematics between Administrations Testa is a global network and service solution offered by Global One under an IDA call for tender. |
| TFTS | Terrestrial Flight Telephone System |
| TIBA | Traffic Information Broadcast by Aircraft TIBA are intended to permit reports and relevant supplementary information of an advisory nature to be transmitted by pilots on a designated VHF radio telephone frequency for the information of pilots of other aircraft in the vicinity. |
| TMA | Terminal Control Area A control area normally established at the confluence of ATS routes in the vicinity of one or more major aerodromes. |
| TMA | Terminal Manoeuvring Area Transition area between en-route area and final approach/initial departure areas. The TMA arrival/departure phase is generally operated through published SID and STAR procedures. |
| TS | Task Scheduler/Reminder |
| TSA | Temporary Segregated Airspace |
| TSO | Technical Standard Order Technical Standard set-up by the US FAA as mandatory |
| TSC | Technical Surveillance Complex |
| TWDL | Two Way Datalink |
| TWIP | Terminal Weather Information for Pilots |
| UACC | Upper Area Control Centre Eurocontrol's Maastricht Upper Area Control Centre is Europe's first truly international ATC Centre, founded by Eurocontrol on the request of Belgium, Luxembourg, the Netherlands and part of Germany: it provides multinational ATC Services in the upper airspace of these four States, since 1972. |
| UAS | Upper Airspace |
| UDP | User Datagram Protocol Connection-less transport layer protocol used in the Internet Network. Part of the UDP/IP stack related to the transport layer. Used for communications that do not need TCP's sequencing or flow control and wish to provide their own. Widely used for client-server type request-reply queries and applications in which prompt delivery is more important than accurate delivery, such as voice or video services. |
| UHF | Ultra High Frequency |
| UIR | Upper Flight Information Region As well as being separated horizontally, airspace is divided vertically, generally being divided into Upper and Lower airspace at a specific altitude level (FL 195 or FL 245 in Europe): above this level is the UIR, where flights are controlled at their cruising altitude. |
| UMAS | Unmanaged Airspace Airspace regime with: * unknown traffic environment * rules of the air |
| UMTS | Universal Mobile Telecommunication System (ETSI) UMTS is the third-generation mobile system which will provide an evolutionary path from today's disparate cellular network -including GSM and the Japanese and American cellular standards- to a future global, seamless, broadband system. High speed data and mobile multimedia are at the heart of the UMTS vision, which also encompasses the integration of satellite communications. Frequencies in the 2.5 GHz spectrum has been allocated globally to enable seamless worldwide cellular communications. The Universal Mobile Telecommunication System (UMTS) will offer flexible service capabilities, such as voice, fax, video, multi-media, data and other information service capabilities via seamless wireless access to anyone equipped with a single easy to use terminal. UMTS is intended to enter into service at the beginning of the 21st century (1.1.2002). UMTS will use Wideband Code Division Multiple Access (W-CDMA) for wide area public services networks, and a hybrid that combines time division techniques with code division (TD-CDMA) for private applications including business and residential cordless data. ITU name for UMTS is IMT-2000. ITU-R name for frequencies allocated to UMTS is FPLMTS. |
| Undertaking | Term used in the Title V of the Treaty of Rome related to competition regulation, but not defined in the Treaty. According to the Commission, "the term undertaking must be viewed in the broadest sense covering any entity engaged in economic or commercial activities such as production, distribution or the supply of services and ranging from small shops run by one individual to large industrial companies". According to the Court of Justice, the concept of undertaking embraces any entity carrying on an economic activity regardless of its legal status or the way it is financed. |
| URD | User Requirements Document (EATMS) |
| UTRA | UMTS Terrestrial Radio Access Name of the common air interface defined for UMTS, using both W-CDMA and TD-CDMA. |
| UVA | Utility Value Analysis |
| VAR | Voice Communications, Ancilliary Equipment and Remote Control |
| VCS | Voice Communication System |
| VDL | VHF Digital Link The use of analogue VHF radio for data interchange was first initiated in the late 70s. Existing airborne VHF radios have been used to transmit AOC and AAC data between aircraft and their operating agencies through special ground stations and interconnecting networks. This system is called ACARS. VDL mode 1 has a data rate of 2400 baud. VDL mode 2 has already been standardised by ICAO and uses digital radio techniques. The nominal data rate of 31.5 kbps is compatible with the 25 kHz channel spacing. The modulation scheme used is capable of supporting different protocol suites for different operational applications. VDL mode 3 uses a TDMA technique and is also standardised by ICAO. VDL mode 3 can integrate both voice and data communications systems. The improved utilisation of the VHF spectrum is achieved through the provision of four separate channels over one carrier (25 kHz channel spacing). VDL mode 4 uses a self-organising TDMA (STDMA) technique which in addition to providing the data communication functions is also intended to make available navigation and surveillance data link capabilities. The VDL is a constituent mobile subnetwork of the ATN, operating in the VHF frequency band. In addition, the VDL may provide non-ATN functions, such as, for instance, digitised voice. |
| VFR | Visual Flight Rules An aircraft must be flown with either the VFR or the IFR. Flight in accordance with VFR is permitted if a flight crew is able to remain clear of clouds by a distance of at least 1500 NM horizontally and at least 300 NM vertically and to maintain a forward visibility of at least 8 km. For flights in some portions of the airspace and at low altitudes, and for helicopters, the requirements are less stringent. An aircraft cannot be flown under VFR at night or above 20 000 ft except with special permission. Under VR, the pilots are responsible for avoiding collision, in accordance with the principle of see-and-avoid. |
| VHF | Very High Frequency |
| VHF A/G Data Link | Two-way data communication in the 118-137 MHz VHF band between aircraft and aeronautical stations. |
| VMC | Visual Meteorological Conditions Meteorological conditions expressed in terms of visibility, distance from cloud and ceiling, equal to or better than specified minima. |
| VOIP | Voice Over IP |
| VoL | Value of Life |
| VOR | VHF Omni-directional Radio Range (ICAO) VOR provides azimuth readings to aircraft. A collocated DME (Distance Measuring Equipment) system (VOR/DME) provides the distance from the aircraft to the DME transmitter. The VOR system operates in the 108-118 MHz band. The maintenance of VOR infrastructure in the ECAC area beyond 2005 may not be guaranteed. |
| VORTAC | VOR collocated with TACAN |
| Vortex | Every aircraft creates turbulence as it flies, a major element of which takes the form of two rotating conical masses of air trailing behind the wing tips. If a second aircraft flies into one of these vortices, the rotating air can cause the plane to pitch and roll. The effects of wake vortices are most important during take-off and landing, particularly the latter. The vortices created by heavy aircraft, such as wide-bodied jets, are severe and in 1970, soon after this type of aircraft came into service, separation standards were introduced which laid down the minimum distance between pairs of arriving and departing aircraft. |
| WAAS | Wide Area Augmentation System equiv. SBAS (ICAO) WAAS is the US name for a system that augments the capability of GNSS over a large area. The system needs a network of ground stations: signals form the GNSS satellites are received by the user as well as the WAAS ground stations. These ground stations are precisely surveyed, allowing each to determine any error in the GNSS signals being received in a geographical location (due for example to ionospheric conditions). Each station in the network relays this data to a WAAS master control facility where correction information are processed and integrated. A navigation message is uplinked to a GEO communications satellite via an earth ground station. The message is then broadcast on the same frequency as GNSS by the communication satellite to receivers on board aircraft flying within the broadcast coverage area of the WAAS. The WAAS navigation message improves GNSS signal accuracy from 100 meters to approximately 7 meters. The information in the navigation message, when processed by a WAAS receiver, allows the communications satellites to be used as additional GPS-like satellites. This increases the availability of the satellite constellation. These navigation messages also contain very important integrity information about the system, to ensure the satellites being used are providing in-tolerance navigation data. The accuracy, availability and integrity enhancements provided by WAAS will enable GNSS to serve as a primary means of navigation for all phases of flight through Cat I precision approach. WAAS (in combination with ABAS) is intended to support near Cat 1 precision approach and landing operations (RNP 0.03/50 and RNP 0.02/40). WAAS service are to be provided in Europe and other regions by the EGNOS system |
| WAFC | World Area Forecast Centre (Meteorology) |
| WAN | Wide Area Network A data network providing communication services over distance greater than those supported by LAN. |
| Waypoint | A specified geographical location used to define an area for navigation route or the flight path of an aircraft employing area navigation. |
| WGS | World Geodesic System The successful global implementation of satellite navigation is predicated on the existence of a coordinate and procedures database of a very high quality. In support of evolving satellite-based technology, ICAO adopted WGS-84 as the common geodetic reference datum for civil aviation. Implementation of WGS-84 involves, among other things, WGS-84 surveying or transformation of existing coordinates and reference datums to WGS-84. |

# The network evolution

The historical evolution of information flows appears to be a good means of understanding the dynamics of the evolution of the networks.

Up until the ‘90s, air transportation based its navigation on the close verbal exchange between pilots and air controllers. After submitting a flight plan, from take-off to landing, the pilot passed from one control sector to another, with the flight plan transmitted on the ground from ACC to ACC* (Area Control Center), as the aircraft progressed. In parallel to that strong capacity to interact shared by the control centres and the aircraft, ground relationships became necessary to allow the circulation of these plans and the tracking of the aircraft from controller to controller. This was the main objective of the OLDI/SYSCO (On-Line Data Interchange/System Coordination) procedure.

Within each country, networks have been set up to provide the required communications. ACCs and APPs (Approach control services) have thus been connected together to provide OLDI links, with the ground stations providing the necessary air-ground links for voice communications. As for the radar systems, they are connected to the processing centres (RMCDE), which are able to transmit elaborate images to control centres. Each country has thus established a multitude of networks dedicated to specific aims and functioning according to very specific protocols.

International communications were provided at that time by a network, standardised by the ICAO, called AFTN*. This network notably enabled access from any point to the AIS* and OPMET databases of the different countries and supported a certain number of message exchanges among administrations. In parallel to this meshed network which operates message switching, direct links among ACCs were established, often via leased lines, in order to enable OLDI exchanges. The radar image formats and the message formats, mainly character-oriented according to the ICAO’s recommendations, have been progressively improved as needs have arisen. In the 1980’s, a new message transmission protocol, the CIDIN, was progressively substituted, on more highly sollicited routes, for the previous protocol (AFTN). The CIDIN protocol improved the efficiency of communications and extended the range of messages that could be exchanged.

As the technologies improved, the ATSOs set up integrated networks capable of carrying all types of communications necessary for their activity. It was generally X25 networks which were developed with an X121 standard addressing scheme. These networks now support the national OLDI links and access to the AIS* and OPMET databases. They interconnect with the national AFTN nodes which enable access to foreign databases.

Next came the revolution brought to European airspace by the creation of a CFMU* (Central Flow Management Unit) which ensures the centralised management of flight plans on the European level and which gave rise to the circulation of a large quantity of information. Two types of flows are generated by the CFMU:

- European navigation now relies for its management on the IFPS* (Initial Flight Plan Processing System). This system manages a consolidated database of all flight plans, on the European level and serves as the basis of the CFMU’s activity. This application receives and transmits after processing, the flight plans for all of Europe, in place of the processing of flight plans on the national level. Introduced in 1996, the IFPS application is shared by two locations, Brétigny in France and Haren in Brussels. For security reasons, the flight plans and associated messages are addressed by the aircraft operation centre directly or via the national centres to the two IFPS centres. Depending on the airport of departure, one of the centres processes the plan and informs the requesting party of the result of his request (clearance).

- A second type of flow, called TACT* (Tactical Computer System), regroups all interactive communications, between the controllers and the flight plan processing centres (the CFMU). These communications concern the provision by the FMPs* (Flow Management Position) to the CFMU of information on real time traffic and the capacity of airspace sectors, and reciprocally, the provision by the CFMU to FMPs of an overview of overall airspace occupation.

- The establishment of the CFMU and its applications has led to a considerable restructuring of information flows, by increasing the need for international communications. To enable the management of communications, the CFMU has decentralised its accesses by contracting with two network operators:

- BT, through its pan-European X25 and FR (Frame Relay) network, links the two CFMU sites to a certain number of AFTN nodes in order to deconcentrate traffic.

- In parallel, by connecting with the SITA’s network in Paris and in Brussels, the CFMU enables access to its applications via the SITA’s data network.

All exchanges with the CFMU are managed through the national ATC services’ integrated networks, when they exist. When they do not, the exchanges are done via AFTN nodes, using the available protocol (AFTN or CIDIN).

In addition to this interconnection with the CFMUs, the interconnection of national integrated networks is progressively being implemented, allowing, little by little, their substitution for the direct links established, for example, to manage OLDI communications.

We witness today the following situation: a certain number of countries have developed national integrated networks or regional data transmission networks. The main example is the RAPNET network, set up by the Benelux countries and Germany, based on Nortel’s DPN 100 equipment. This network, which comprises 22 nodes, ensures most of the data links for the ATSOs of that area and is linked to Maastricht’s common centre. Other networks are France’s RENAR and Spain’s REDAN networks, which are also based on DPN 100 equipment. The NATS, of Great Britain, has developed a similar network, called CAPSIN, as has AustroControl, with its VAS network.

ATSO Integrated Networks

Portugal, Italy and Ireland have undertaken to do likewise. Italy even intends to integrate on its future network - AVNET - not only data but voice networks.

A more detailed mapping of these networks shows that they provide an increasingly extensive linking throughout Europe. As they are extended, not only are the ACCs connected, but also the APPs and the centres which house the AIS and OPMET databases. These networks obviously offer an interconnection with the AFTN nodes, in general via the CIDIN protocol, but in addition offer interconnections among themselves, more and more frequently, with a standard X75-type protocol. The Rapnet is connected with RENAR, CAPSIN and VAS, and links are planned between Renar and Redan. A double tendancy is thus appearing, of intensification of these networks, of increased traffic and of interconnection with neighboring networks, especially to allow the transfer of OLDI dedicated links towards these integrated networks.

Detailed Configuration of ATSOs' Integrated Networks

The international mapping of these OLDI links, whatever their implementation, shows the intensity of relations among ACCs. There is thus a progressive shift from direct links to links through interconnections with integrated national networks. The following map, which is not entirely complete, shows the density of these international relations.

Examples of OLDI Links between ACCs

Finally, there are two networks, essential on the international level, which must be superimposed on these maps. The first is the AFTN network structure which has handled the communication of nearly all international messages up until now, notably allowing access from any site to the national AIS and OPMET databases. In each country, a message switching network allowed any point needing to connect to the AFTN to access the national interconnection node. As we have seen, these message networks are increasingly carried by integrated data transmission networks.

AFTN Network

In addition, we now find on the European scene, a specific CFMU access network. This network, coupled with the AFTN, comprises nevertheless some direct connections. This access network is provided thanks to the services provided by BT and SITA.

CFMU Network

All of these networks are thus formed by routes or services leased or purchased from telecommunication operators. In the case of national integrated networks, the direct OLDI links or the AFTN links, the lines are leased from traditional operators: Deutsche Telekom, Belgacom, KPN, France Télécom, Telefonica, etc. In the case of access networks to the CFMU, these services are purchased from BT and from the SITA.

Then we have the air-ground lines of which there are two types:

- Voice lines between pilots and controllers or between pilots and AOs. These lines are provided either by ground station networks held and operated by the ATSOs for air navigation needs, or by the airlines themselves or third parties (ACSPs) on behalf of the companies. A tendancy towards outsourcing these ATSO networks towards the ACSPs has appeared over the last few years.

- Data links, which today concern almost exclusively communications between aircraft and their operators. These links are mainly provided by the ACSPs which have set up networks having numerous ground stations operating in VHF frequencies over continental areas and operating satellite links over the oceans. These communications use either the ACARS standard or the A 622 standard, developed by ARINC.

The ACARS protocol was developed in the 70’s by ARINC and is a character-oriented protocol. Two frequency bands were allotted for the ACARS service in Europe; one for SITA, one for ARINC. ACARS messages are composed of a maximum of 220 characters, but longer messages can be transmitted in a fragmented mode. This network uses a centralised management of messages (located in Singapore for SITA’s network). The central computer updates the aircraft’s position via messages that the aircraft transmits. The ATSOs have never wished to use ACARS networks because of their very limited security.

The emergence of the A 622 standard offered a partial solution to the fears expressed by the ATSOs, by doubling the ACARS network with a bit-oriented communication protocol. From then on, certain ATSOs* set up ATC operational communications on the ACARS/A 622 network, for non-critical messages, such as Pre-Departure Clearances and ATIS (Automatic Terminal Information Services). In certain low traffic areas, automatic Controller-Pilot communications (CPDLC) are being tested (Air New Zealand with Sita in the South Pacific).

SITA A/G VHF Network

ARINC, an American company, has rapidly expanded its position in Europe, as shown on the map below. Having won two contracts with ATSOs (NATS and DFS), ARINC has established itself, contrary to SITA, in the CPDLC communications domain.

ARINC A/G VHF Network

Different options are being considered for the introduction of the ATN. The idea which presides over this reflection is that it is not possible nor, no doubt, desirable that there be a rapid migration of all telecommunications infrastructures to ATN architectures. We must therefore envisage a progressive deployment which takes into account the following constraints:

- The deployment must be consistant, that is, it must allow the migration to the ATN of communication flows relative to certain applications, and it must be coherent, that is effectively anticipate the deployment of the elements necessary for the functions required.

- The deployment must be financable, that is, entail the improvements in efficiency and the economies that allow the amortization of its set up, without necessitating a too rapid withdrawal of equipment that prove to be cost-effective.

# ATN Design

## Functional Components

The ATN architecture is based on the OSI model, and all the protocols used within the ATN are standards of the ISO OSI model. The ATN internet is defined on the OSI layer 1-4 services. But the Physical and the Data Link layers (layer 1 and 2) together with the subnetwork access function in layer 3 (Network layer) are only dependent on the subnetworks used to link ESs and ISs and are not ATN relevant. Consequently, only the parts of the Network layer independent of real networks and the Transport layer are part of the ATN architecture.

The ATN is composed of two kinds of functional components: End Systems (ES) and Intermediate Systems (IS). The ATM applications nested in the ESs exchange application specific data through the ATN internet. The ISs relay and route NPDUs (Network Protocol Data Unit) to reach the targeted ES.

The routing is performed thanks to the routing information base (RIB) stored in ISs. It contains information such as:

- Next-hop routing tables;

- Lists of known ESs and ISs;

- Quality of Service (QoS) of subnetwork hops;

- Network topology maps.

This enables the routing algorithm to compute the paths interconnecting Routing Domains, taking into account the QoS requested.

In order to optimise the routing information, the RIB is reduced into a FIB (Forward Information Base) which simply lists the forwarding paths available for each QoS and routing policy to reach each known destination.

The ATN is divided into three types of subnetworks: ground subnetworks, avionics subnetworks and air-to-ground subnetworks, the latter being mobile subnetworks. Therefore, two communication modes arise, the air-ground communications and the ground-ground communications. All the CNS/ATM (Communication, Navigation & Surveillance / Air Traffic Management) applications using the ATN services follow one of these data links.

The avionics communications link the airborne ESs with the airborne IS. But this aspect is not focused on in the ATN reference model because the aircraft is seen as a global ES by all the ATN mobile applications.

## Subnetworks

The ATN is a collection of interconnected subnetworks which form a global internet including AOC, AAC, ATS, APC. These subnetworks interconnect ISs and ESs for the purpose of transferring data between applications located in ESs.

Three categories of subnetworks can be distinguished:

- general topology subnetworks, like WANs, which are used to connect different routing domains, passing through several ISs;

- broadcast subnetworks, like LANs, which are used to connect ESs to ISs within a single routing domain;

- mobile subnetworks, which use radio (VDL), radar (Mode S) or satellite (AMSS) services to connect the avionics subnetworks to the ground subnetworks.

All these subnetworks are required to use at least the connectionless-mode service. The end-to-end service is therefore provided by CLNP (ISO 8473 ConnectionLess Network Protocol) by means of datagrams. But they can also take advantage of the connection-oriented services in some particular cases, using appropriate convergence functions. It can be especially adapted to air-ground data links to obtain an efficient use of the scarce media bandwidth.

## Routing

a) Routing framework

The ATN is designed to become a very large internet. Therefore, it is necessary to adopt a hierarchical structure to separate the network into several domains and to enable an efficient routing strategy. The OSI routing framework satisfies this requirement thanks to two functional entities: the administrative domain and the routing domain.

b) Administrative domains

Since the network is owned and operated by several organisations, it is necessary to create the possibility for each of them to introduce their own routing policies within their domain and at their boundaries. The administrative domain is therefore a set of ESs, ISs and subnetworks belonging to a single organisation and communicating with a high level of trust among them.

The various levels of trust existing between different administrative domains drive them to fix restricted routing rights when transferring data across domain boundaries.

An administrative domain can be administered by governmental as well as by public or private aeronautical services or organisations, like ATS or AIS organisations.

c) Routing domains (RD)

Since even within a single administrative domain, that is to say within an organisation, there may be a need for distinct routing strategies, the routing domain is a subdivision of the administrative domain which is based on a strong homogeneity. Indeed, it is made up of a set of ESs and ISs bound by a common routing procedure. This means that they use:

- the same set of routing metrics;

- compatible metric measurement techniques;

- the same information distribution protocol;

- the same path computation algorithm.

This enables a greater routing flexibility since all the above characteristics can be different between several routing domains. And it provides enhanced robustness and control of each routing domain by the introduction of new boundaries inside an administrative domain. When an IS is at the boundary of an RD, it is called a Boundary Intermediate System (BIS).

An administrative domain contains at least one routing domain. The architecture is further subdivided when the organisation administrating the domain grows significantly. And a routing domain is always wholly contained in a single administrative domain. It can comprise ground systems or mobile systems. Airborne subnetworks make up one or more routing domain(s) and belong to the administrative domain of the aircraft operator.

d) Routing areas

The routing area is a further subdivision of a routing domain and generally comprises one or more ISs and possibly one or more ESs. The introduction of routing areas is based on data links bandwidth consideration: inter-routing area data links (WANs) have lower bandwidth than inter-routing area ones (LANs). Consequently, even if the routing area is a logical notion, it is often based on geographical constraints.

e) Routing domain confederations

This is the final element introduced to achieve a completely flexible ATN internet architecture. In order to administer a growing number of administrative domains, there is the possibility for a group of them to join recursively into a routing domain confederation. It is realised on the basis of private arrangements among members, without any global co-ordination.

Routing domain confederations may overlap. Routing domains can belong to zero, one or several routing domain confederations.

It does not require common routing rules, but it is from this co-ordination that the efficiency arises: when linked to the addressing plan, it reduces the routing information flow and simplifies IDRP (Inter Domain Routing Protocol) procedures.

## Protocols

The ATN protocols follow the reference model specified by ISO in ISO 7498-1. The protocols specifically relevant to the ATN deal with the OSI layers 3 and 4. The Physical and Data Link layers, together with the subnetwork access function of the Network layer are only dependent on the subnetwork the system is connected to, X.25 or Ethernet for example. For the upper layers, the Application, Presentation and Session layers are dependent on the application being serviced by the network.

The Network layer uses a connectionless oriented protocol, CLNP (ConnectionLess Network Protocol), and provides the interface to accommodate the differing protocol requirements of local "real networks and systems". Therefore, it is necessary to provide a gradual transition between the connectionless network service and the Data Link and Physical layers. Consequently, the Network layer is subdivided into two or three functions.

The lower function provides the adaptation to the subnetwork used. It is called Subnetwork Access Protocol (SNAcP). As it is stated above, this function is not specifically relevant to the ATN. It provides the protocol access mechanism for data transfer between adjacent communication nodes over a particular data link.

The higher function consists of an inter-network protocol (CLNP, ISO 8473), Routing Protocols (RP), management protocols and a relay function. It is the subnetwork independent convergence function and it is in charge of the relaying and routing throughout the ATN internet.

When these sublayers cannot inter-operate directly, a Subnetwork Dependent Convergence Protocol (SNDCP) is added in-between to help the inter-network function to utilise the SNACP.

The routing protocols are used by the routing function of the inter-network sublayer. Since the routing domain notion has been introduced to structure and simplify the routing procedures, three types of protocols can be distinguished depending on the routing information flows.

The inter-domain routing information exchanges are handled by IDRP (ISO 10747). For the intra-domain routing information exchanges, the routing between two ISs is performed by ISO 10589 IS-IS whereas the routing between an ES and an IS is done by ISO 9542 ES-IS.

The Transport layer is in charge of providing a transparent transfer of data between Transport service users. Its protocol is defined on an End-to-End basis, where the "Ends" are the two transport entities located in the communicating ATN ESs.

Whereas the ATN Network layer is definitely connectionless oriented, the Transport layer can be either connection oriented (COTS) or connectionless oriented (CLTS) depending on the service required. The former case provides a datagrams service, with no guarantee of delivery but without the need to establish a connection prior to the communication. The latter requires the peer transport service users to negotiate a communication channel, specifying the features of the communication, but guarantees the delivery of data.

Graph

The transport service access point (TSAP) represents the specific interface between the application using the ATN service (session and presentation layers) and the transport layer.

The subnetwork point of attachment (SNPA) links the subnetwork used to the ATN stack in order to provide a global communication service.

## Naming and addressing model

One of the necessary tasks of network functioning is to identify and locate the applications to which the information sent is addressed. These applications can be seen as objects nested in an ES.

Therefore, two notions have been developed by OSI, names and addresses. Names provide an unambiguous identification of these objects, whereas addresses enable to locate them, which is useful when these objects move in the internet.

The ATN objects which need a name are:

- ATM Application Entities (AEs) which form the communicating part of the applications and which communicate with one another;

- Network Layer Entities, whose addresses are used by the routing protocols;

- System Management Application Entities (SMAEs);

- Routing Domains and Routing Domain Confederations.

The Routing Domains and Routing Domain Confederations can be located simply by their name.

The addresses are necessary to locate AEs, upper-layer and Network layer entities.

The NET (Network Entity Title) of the Network layer entities is sufficient to both name and address them.

But the AEs necessitate a presentation address, which contains:

- a Presentation Service Selector (PSAP);

- a Session Service Selector (SSAP);

- a Transport Service Selector (TSAP);

- an NSAP address.

These selectors enable the AEs to make use of several services in the ATN stack layers. But in most cases, an ES can be identified with only its NSAP address. The combination of the NSAP and the TSAP addresses form the Transport Address.

These addresses are different from subnetwork addresses such as X.25 DTE addresses, Ethernet MAC addresses, etc. which are named SNPA (subnetwork point of access) addresses.

## Management domains

In order to manage the global ATN internet, each administrative domain is responsible for managing its own portion of the ATN. Any subdivision in the domain is decided by the administrative domain only.

A management domain needs at least:

- to have a name which identifies it unambiguously;

- to group objects members of the domain;

- to define its relationships with adjacent domains.

A special kind of management domain describes the case where the managed objects are all under the responsibility of a single administrative domain. It is a Management Administrative Domain. It requires mechanisms to establish and maintain the authorities of each management domain, to organise the way some of them overlap and to manage the transfer of control resources between adjacent management domains.

## System management, framework and protocol

The ATN uses the OSI management protocol, CMIP (Common Management Information Protocol), to provide the necessary mechanisms for fault reporting and diagnosis, accounting and security services.

The model used to build the management system is based on managed objects, which are abstractions of real-world resources distributed throughout the network. Management applications are called MIS-users and are assigned one of two roles, agent or manager role.

In order for the manager to manage its agents, a Management Information Base (MIB) stores the management knowledge. It includes:

- protocol knowledge like the application context;

- function knowledge, the list of the functions and the functional units;

- managed object knowledge, giving the classes, the instances and the identification of the managed objects and of their attributes;

- constraints on functions supported and relationships between these functions and managed objects.

This management knowledge must be available at all times, and especially prior to management procedures establishment.

This management protocol (ISO 9595) functions at the application layer level. Therefore, all systems in the ATN network must support the representation of their managed resources as managed objects.

## Quality of Service framework

The Quality of Service (QoS) requirements made by the application users on the ATN internet are of prime importance since some aeronautical communications can be very critical in terms of transit delay for example while others can focus more on the price of the communication. It is therefore necessary to provide a service enabling users to specify what they expect in terms of transit delay, throughput, cost or reliability (low residual error rate) and means of replying to these requests. Therefore, the QoS is a way to ensure the Required Communications Performance (RCP).

The QoS parameters are composed of :

- data integrity, maintained by end-to-end checksums, at ESs ;

- transit delay, ensured by the network design and by congestion management and data prioritisation services, at ISs ;

- availability, thanks to multiple alternative routes and to re-routing mechanisms in case of failures, provided based on the routing information exchanges between ISs ;

- mis-delivery prevention, handled by the transport protocol either at ESs or at ISs, but only reporting the probability that mis-delivery has occurred ;

- report non-delivery, in case a transport connection has failed, is provided to the ESs ;

- ensure receipt, also provided to the ESs.

Since the network layer is connectionless oriented, these requirements will be specified in each PDU (Protocol Data Unit) transiting on the ATN network. In fact, these requirements are transmitted from one OSI layer to the next throughout the ATN stack. They are used to determine a route during the routing process.

The design of the network will take into account the desired QoS. But once in use, the Performance Management Function will have to measure the real performances of the network and the results may push to revise the network topology in order to match requested with actual QoS.

## Security model

A security policy has been introduced into the ATN model in order to control the access to resources reserved for air-safety related applications, and to guarantee the separation of traffic serving operational and non-operational users. It also enables paying services of commercial service providers to be accessed only by paying customers.

The mechanism used is based on a Security Label, which tags data or routes through the ATN. The router rejects traffic types from resources not available to that type or re-routes it to corresponding resources. The different types of traffic available are Systems Management Traffic, Operational Traffic, Administrative Traffic and General Communications (default value). These categories are used to ensure that traffic only passes through service providers that will accept the data.

The security policies established by ICAO, which is the security authority, are presented in the SARPs. The baseline policy includes the following rules :

- it shall not be possible for anyone to modify a message once it has been produced in such a way as to alter the information content of the message or its original sender;

- it shall always be possible to reliably tell from a message who the original sender was ;

- for data link application messages, it shall be possible to recognise the correct sequence of messages, so that if a message is received out of sequence, this fact is recognisable ;

- the service that supports messages to and from aircraft (including when these messages are on the ground links) shall be protected against denial of service attacks to a chosen level of probability of compromise (to be ascertained) ;

- all ATN hosts and routers shall be protected against attack from the unauthorised person, such that only authorised persons shall be able to send ATN messages or access ATN data.

These rules can be achieved, among other things, thanks to authentication and digital signature mechanisms.

## Priority model

As for all connectionless networks, the ATN has to be protected against the risk of congestion. The best way is to ensure that the load sent to the network never exceeds the network capacity. However, to manage bursts of traffic, a mechanism needs to be introduced to prioritise the data contained within the packets. One use of the priority flag is for the ISs to speed high priority data on its way. But the main use is to discard low priority data first in case of congestion in a router. It is also possible to allocate resources specifically for certain types of data traffic to ensure the transport of high priority data in case of outages.

When applied specifically to the ATN, the prioritisation of traffic has driven to define fourteen categories of application traffic. But the levels of priority are used only to select communication resources needed for data travelling between different pairs of systems and not between the same pair of ESs.

## System certification

As for all systems used in connection with airborne equipment, it is necessary to certify that ATM applications relevant to flight safety satisfy every safety requirement. This is done via the certification process, which studies the evidence that proves:

- that all potential hazards have been identified and that appropriate controls are provided to detect them;

- that the safety management procedures developed for the system are adequate to ensure that the design and operation of the system meet the safety requirements;

- that the evidence available  to show the compliance with safety requirements is sufficient to allow the system to proceed to the next life cycle phase or continue in operation, as appropriate.

Of course, the certification process starts together with the design process. And from the beginning, the search for potential hazards and their control must drive the building of the system.

But the certification process is also closely linked to the QoS concept. Indeed, since it is not cost-effective to re-certify the ATN for each new application using it, the goal is to decouple the approval of the ATN from the individual applications. Therefore, the target is to certify an application based on the assumption that the communication service exists and is provided with the assumed QoS. Similarly, it should be possible to specify a QoS for the ATN and to demonstrate to the satisfaction of the approval authorities that such a QoS is provided. The decoupling achieved this way will ensure that the ATN can be implemented, deployed and approved for use independently from its using applications and enable the installation of a new application on the existing ATN without having to re-approve the ATN.

# The example of Nav Canada

Canada is one of the countries that has pushed the furthest the separation of the State and air navigation control services. It was in May 1995 that Nav Canada was incorporated as a non-share capital corporation (NSCC)*. That possibility was offered by Part II of the Canada Corporations Act. That category concerns entities such as universities. In April 1996, the Federal Government and Nav Canada signed an agreement to transfer ownership and operation of the Air Navigation Service (ANS) to Nav Canada for C$1.5 billion. That transfer was consequent to the promulgation, on 20 June 1996 of the Air Navigation Services Commercialization Act (the "ANS Act"). The fundamental principles governing the mandate conferred on Nav Canada by the ANS Act include the exclusive right to provide certain services and the ability to set and collect user charges for air navigation services. The ANS Act provides that Nav Canada may establish new charges and amend existing charges for its services, and revise such charges as required to generate revenue sufficient to meet its current and future financial requirements, including operating costs, debt service costs, depreciation, financial requirements necessary to maintain an appropriate credit rating and reasonable reserves for future expenditures and contingencies. Nav Canada is exempt from income taxes and it carries on its operations without monetary gain to its members.

After 60 years of ownership and control of air navigation activities, the Canadian government thus sold and transferred operating responsibility for air navigation to a private entity, charging it with the responsibility of improving management while increasing the quality of services offered, especially in terms of safety.

Nav Canada is governed by four founding members, and a stakeholder board of Directors. The four members are the Government, the airlines, general aviation and the unions. The members each appoint a slate of directors, for a total of 10. Within this plan, the Government conserves regulatory functions and control of the functioning and activity of Nav Canada.

In November 1996, Nav Canada began fully operating as a private sector company and over 6,300 employees were transferred from the Government to the new company. Since the company did not own capital, it had to borrow the totality of funds necessary to constitute its operating budget and to acquire (for C$1.5 billion) navigation and air traffic control installations.

In December 1996, Nav Canada made its first public issue of C$750 million of long term revenue bonds, rated "AA". In March 1997, Nav Canada placed a second public issue of C$750 million of long term revenue bonds. Nav Canada obtained the highest bond rating of any private sector organisation in Canada. This success is due to the rules applied to its economy, as stated very clearly the representative of RBC Dominion, which managed that line of credit:

Nav Canada’s "credit strength is derived from four fundamental credit factors:

- It offers an essential service. Without air navigation and air traffic control services, air traffic would be substantially unable to fly in or through Canada.

- It has a legislated monopoly to provide the core services involved in civil air navigation in Canada

Subject to certain broad charging principles, it has the legislated right to set user charge rates without regulatory approval, and to levy these charges against all civil aircraft for the availability or provision of services.

It has the legislated right to apply ex parte to seize aircraft in order to collect delinquent user charges. In addition, owners and operators of aircraft are jointly and severally liable for any user charges incurred".

This financial presentation of Nav Canada illustrate the very low risk represented by the financing - under the conditions of autonomy and regulation indicated - of air control activities. The inelasticity of traffic to the charges requested, coupled with the monopoly situation, offers guarantees that few activities can offer, especially if they are accompanied by the quasi-certainty of recovery of the sums due.

Three lessons can be learned from the first year of operation of Nav Canada:

- Nav Canada expects to improve its economy through the planned reduction of its ACC

- The establishment of Nav Canada deeply changes the charging system

- Security control becomes a joint responsibility shared by the government and Nav Canada

## Efficiency Improvement

The Canadian Air Navigation System, when it was transferred to Nav Canada, consisted of seven area control centres (ACC), 44 air traffic control towers, 83 flight service stations (FSS) and a network of 1,400 electronic navigational aids. The system supports 7 million aircraft movements a year, but with a continuous increase in the number of controllers and a regular decrease in the ratio of aircraft movement to controller:

Canada's Air Navigation System Workload Statistics

|  | Aircraft Movements1 at Canadian Airports | Air Traffic Controllers | Aircraft Movements per Controller |
| --- | --- | --- | --- |
| 1991 | 7 284 079 | 1 619 | 4 511 |
| 1992 | 7 037 735 | 1 744 | 4 035 |
| 1993 | 6 778 955 | 1 854 | 3 656 |
| 1994 | 6 858 337 | 1 908 | 3 595 |
| 1995 | 6 832 144 | 1 957 | 3 491 |
| 1996 | 6 739 967 | 1 947 | 3 462 |
| 1997 | 7 004 252 | 2 209 | 3 171 |

1 Aircraft movements = arrivals and departures of aircraft

This conclusion led Nav Canada, in November 1997, to announce a plan to reduce the corporation's 6,300 employee positions by 1,000 over three years. It expects to achieve an 18% reduction in operating costs from this restructuring and downsizing, which are matters for Nav Canada to determine in accordance with the requirements of the Civil Air Navigation Act and the Aeronautics Act. This objective will probably only be attainable through a reduction of the number of ACCs and a considerable automatising of flight processing.

While maintaining the priority of safety and quality, Nav Canada has begun a reflection on its Level of Services Policy, notably because of its programme regarding the closing or relocation of operational facilities. Nav Canada is under a statutory requirement to file a Level of Services Policy. The policy sets out the criteria for initiating, increasing, decreasing or eliminating the provision of service in any particular location. Any reflection on the improvement of the economy of services will lead to the discrimination of level of services, first by geographical area, and then, probably, by type of traffic.

## User Charges Evolution

Until 1998, users of navigation services (airline companies, among others) were obliged to pay an Air Transportation Tax, passed on to the passengers. During the 20 years of its existence, that tax increased by 1100% versus a 200% increase in the Consumer Price Index during the same period. That increase and the problems encountered in the management of services, which led to a freezing of employee salaries, set up the conditions for a consensus on the creation of Nav Canada. Nav Canada thus had the daunting task, from its inception, of proposing a new fee system to be progressively substituted for the public tax collected up until that time.

Nav Canada’s financial prudence is governed by a certain number of conventions with its financiers:

- First, a rate covenant establishes that Nav Canada must charge sufficient rates to generate enough revenue to pay all costs, including debt service costs, with a coverage ratio of 1.25x.

- Secondly, in order to ensure that operations continue smoothly and debt service payments are made on time, reserve covenants establish that Nav Canada must maintain both a debt service reserve (one year's debt service costs) and an operating and maintenance reserve (three months operating and maintenance costs).

- Finally, an additional indebtedness covenant provides that Nav Canada cannot increase its borrowing without following a process to ensure that it will be able to service the increased debt burden through rates.

We can see that under these conditions, advantageous financial packages have easily been offered to Nav Canada. Within this context, productivity gains are becoming indispensable for Nav Canada, in order to keep down the fees that users are charged. The economy realised by that structure as compared to a share-based corporation (with a financial structure including approximately 40% equity and 60% debt), is close to 10% of operating costs.

In 1997, Nav Canada participated in a series of consultations with customers and stakeholders to co-operatively shape a fee structure.  That structure was approved in mid 1997 by the Company's Board of Directors and subsequently confirmed by Canada's Minister of Transport. Effective March 1, 1998, Nav Canada introduces domestic and international enroute and terminal service user fees at approximately 50% of the final rate which will be in place at November 1, 1998 and will increase overflight enroute charge by approximately 50%. This introduction of user fees coincides with a 50% reduction in transfer payments from the federal government and a roughly equivalent reduction in the Air Transportation Tax charged to the travelling public. On 1st November 1998, the new system will replace both the Air Transportation Tax and the government’s assumption of intermediate payments, set independently of traffic levels. The result will be a negotiation taking into account the interests of the General Aviation Community and users operating in Canada's North, that is the need to not penalise the low-traffic, but broad surface, areas. The need for universal service is met, in this case, on a contractual basis.

While attempting to retain the same overall level of revenues, the creation of Nav Canada has resulted in a profound restructuring of fees and an increased autonomy in fixing them.

## Safety Management

With the divestiture of airports and air navigation system assets, Transport Canada's role has changed from that of an owner/operator/regulator to that of regulator alone. But that function of regulator must find a balance between its positive and negative effects. As is mentioned in the official documents: "it is the Federal government's policy to ensure that use of regulatory powers results in the greatest net benefit to Canadian society. However, this does not mean regulation at any cost; regulatory intervention (inspection, audit, increased regulation, standards and enforcement) must be balanced against a backdrop of social, economic and fiscal realities".

The regulatory principles proposed in Canada are the following:

"a. where a problem or risk exists, it is sufficient to justify federal government regulatory intervention;

b. Canadians are consulted and (...) they are afforded the opportunity to participate in developing regulations and regulatory programs;

c. the benefits of regulation outweigh the costs to Canadians, their governments and businesses;

d. the adverse impacts on the capacity of the economy to generate wealth and employment are minimized and (...) no unnecessary regulatory burden is imposed;

e. intergovernmental agreements are respected and full advantage is taken of opportunities for coordination with other governments and agencies;

f. the regulations are clear and concise;

g. the regulations meet the needs of those to be regulated and (...) they are equitable to all;

h. where possible, the regulations are harmonized with those of other countries; and

i. systems are in place to ensure the effective management of the program.

The challenges of globalization, continued fiscal restraint and the rapid pace of technological change will continue to have a dramatic impact on the safe management of the national civil air navigation system. Responsiveness and flexibility will be key as the safety oversight regime evolves. This means:

- making greater use of standards;

- making greater use of new forms of regulations;

- rewriting regulations to focus on essential objectives and encouraging industry to participate in identifying the best ways of meeting them;

- working in partnership with the regulatee wherever possible; and,

- making use of performance standards rather than detailed technical standards where possible, and

- incorporating them by reference in regulations"

The preferred approach is risk-based and includes three phases: risk identification, risk evaluation, risk control. The approach can only be implemented through the collaboration and increased communication among the concerned parties, in particular Transport Canada and Nav Canada.

Eléments clés de Nav Canada (C$ Millions)

| Consolidated Statement of Operations | 19971 |  |
| --- | --- | --- |
| Revenue | 776.3 |  |
| Operating expenses | 584.5 |  |
| Excess of revenue before undernoted items | 191.8 |  |
| Interest | 74.1 |  |
| Depreciation and amortization | 47.4 |  |
| Restructuring | 57.0 |  |
| Excess of revenue (transferred to retained earnings) | 13.4 |  |
| Balance Sheet | 1997 (08-31) | 1996 (08-31) |
| Current assets | 180.1 | 0.5 |
| Reserve funds | 400.0 | - |
| Land, buildings, system and equipment | 350.5 | 1.2 |
| Air Navigation Rights | 1344.8 | 18.1 |
| Deferred financing costing | 19.3 | - |
| Total assets | 2294.7 | 19.8 |
| Current Liabilities | 234.6 | 19.8 |
| Long term debt: Bank Loans | 546.7 | - |
| Long term debt: Bonds payable | 1500.0 | - |
| Retained earnings | 13.4 | - |
| Total Liabilities | 2294.7 | 19.8 |

1 Ten month period ended August 1997

Source: Nav Canada, 1997 annual report

The tariffing structure which was is to be finalised for November 1998 should show a drop of about 20% in comparison with Canada's existing federal air transportation tax. That decrease is partially attributable to Nav Canada’s performance improvement, but more so to the redistribution of charges which will be higher for foreign airlines using Canadian airspace and cargo shippers.

# Geographical Data

| State | ECAC Member | Eurocontrol Member | CRCO Member | APANPIRG Member | NATSPG Member | EANPG Member(1) |
| --- | --- | --- | --- | --- | --- | --- |
| Albania |  |  |  |  |  | X |
| Armenia | X |  |  |  |  | X |
| Austria | X | X | X |  |  | X |
| Belgium | X | X | X |  |  | X |
| Belarus |  |  | X |  |  | X |
| Bosnia & Herzegovina |  |  |  |  |  | X |
| Bulgaria | X | X |  |  |  | X |
| Croatia | X | X |  |  |  | X |
| Czech Republic | X | X | X |  |  | X |
| Cyprus |  | X | X |  |  | X |
| Denmark | X | X | X |  |  | X |
| Estonia | X |  |  |  |  | X |
| Finland | X |  |  |  |  | X |
| France | X | X | X | X | X | X |
| Germany | X | X | X |  |  | X |
| Greece | X | X | X |  |  | X |
| Hungary | X | X | X |  |  | X |
| Iceland | X |  |  |  | X | X |
| Ireland | X | X | X |  | X | X |
| Italy | X | X | X |  |  | X |
| Latvia | X |  | X |  |  | X |
| Lithuania | X |  | X |  |  | X |
| Luxembourg | X | X | X |  |  | X |
| Malta | X | X | X |  |  | X |
| Moldova | X |  | X |  |  | X |
| Monaco | X | X |  |  |  | X |
| Netherlands | X | X | X |  | X | X |
| Norway | X | X | X |  |  | X |
| Poland | X |  |  |  |  | X |
| Portugal | X | X | X |  | X | X |
| Romania | X | X | X |  |  | X |
| Russian Fed. |  |  |  |  |  | X |
| Slovakia | X | X | X |  |  | X |
| Slovenia | X | X | X |  |  | X |
| Spain | X | X | X |  |  | X |
| Sweden | X | X | X |  |  | X |
| Switzerland | X | X | X |  |  | X |
| FYROM | X |  |  |  |  | X |
| Turkey | X | X | X |  |  | X |
| UK | X | X | X | X | X | X |
| Ukraine |  |  | X |  |  | X |
| TOTAL | 36 | 27 | 24+4 |  |  | 49 |

(1) EANPG (European Air Navigation Planning Group), established in 1972, is the ICAO EUR Region regional planning body. European ANP (Air Navigation Plan) is done within Regional Air Navigation (RAN) meetings by EANPG.

ICAO has divided the world into 9 regions:

- Africa - Indian Ocean (AFI Region)

- Asia (ASIA Region)

- Caribbean (CAR Region)

- European (EUR Region)

- Middle East (MID Region)

- North American (NAM Region)

- North Atlantic (NAT Region)

- Pacific (PAC Region)

- South American (SAM Region)

ICAO EUR Region comprises a continental area extending from the NAT Region in the West to the Pacific ocean and the Asia/Pacific Region in the East. EUR Region comprises all the states of the former Soviet Union.

| State | Surface FIR | Separation FIR/UIR (FL) | Number of ACC | Number of UCC | Number of APP | Number of sectors en route | Number of Radars |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Austria | 84000 | 245 | 1 |  | 6 | 14 |  |
| Belgium | 30500 | 195 | 1 |  | 2 | 4 |  |
| Bulgaria |  |  |  |  |  |  |  |
| Croatia |  |  |  |  |  |  |  |
| Czech Republic |  |  |  |  |  |  |  |
| Cyprus |  |  |  |  |  |  |  |
| Denmark |  | 245 | 1 |  | 3 | 9 |  |
| Finland |  | 245 | 2 |  |  | 4 |  |
| France | 768600 | 195 | 5 |  | 6 | 74 |  |
| Germany | 297600 | 245 | 6 | 1 | 12 | 61 |  |
| Greece | 277200 | 245 | 3 |  | 2 | 9 |  |
| Hungary |  |  |  |  |  |  |  |
| Ireland | 168000 | 245 | 2 |  | 3 | 9 |  |
| Italy |  | 245 | 4 |  | 9 | 28 |  |
| Luxembourg |  | 245 |  |  |  |  |  |
| Malta |  |  |  |  |  |  |  |
| Monaco |  |  |  |  |  |  |  |
| Netherlands | 34000 | 195 | 1 |  | 3 | 6 |  |
| Norway |  |  |  |  |  |  |  |
| Poland |  |  |  |  |  |  |  |
| Portugal | 276000 | 245 | 1 |  | 2 | 5 |  |
| Romania |  |  |  |  |  |  |  |
| Slovakia |  |  |  |  |  |  |  |
| Slovenia |  |  |  |  |  |  |  |
| Spain | 621875 | 245 | 5 |  | 16 | 32 |  |
| Sweden |  | 245 | 3 |  | 8 | 19 |  |
| Switzerland |  |  |  |  |  |  |  |
| Turkey |  |  |  |  |  |  |  |
| UK | 575000 | 245 | 3 |  | 16 | 41 |  |
| Maastricht UAC |  |  |  | 1 |  | 7 |  |
| TOTAL |  |  |  |  |  |  |  |

# Frequencies used in the aeronautical world

| Frequency |  | Unit | Bandwith | Service | Usage | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 70 | 72 | kHz | 2 | Radionavigation | Decca Navigator |  |
| 84 | 86 | kHz | 2 | Radionavigation | Decca Navigator |  |
| 90 | 110 | kHz | 20 | Radionavigation | Loran C |  |
| 112 | 115 | kHz | 3 | Radionavigation | Decca Navigator |  |
| 115 | 117.6 | kHz | 2.6 | Radionavigation | Decca Navigator |  |
| 126 | 129 | kHz | 3 | Radionavigation | Decca Navigator |  |
| 255 | 283.5 | kHz | 28.5 | Aeronautical radionavigation - permitted | Shared with broadcasting - primary |  |
| 283.5 | 315 | kHz | 31.5 | Aeronautical radionavigation - permitted | Shared with maritime radionavigation - primary |  |
| 315 | 325 | kHz | 10 | Aeronautical radionavigation - primary | Shared with maritime radionavigation - secondary |  |
| 325 | 405 | kHz | 80 | Aeronautical radionavigation - primary |  |  |
| 405 | 415 | kHz | 10 | Radionavigation |  |  |
| 415 | 435 | kHz | 20 | Aeronautical radionavigation - primary | Shared with maritime mobile - permitted |  |
| 435 | 495 | kHz | 60 | Aeronautical radionavigation - secondary | Shared with maritime mobile - primary |  |
| 505 | 526.5 | kHz | 21.5 | Aeronautical radionavigation - permitted | Shared with maritime mobile - primary |  |
| 2.850 | 3.025 | MHz | 0.175 | Aeronautical Mobile (R) | HF Communications | 7 |
| 3.4 | 3.5 | MHz | 0.1 | Aeronautical Mobile (R) | HF Communications | 7 |
| 4.65 | 4.7 | MHz | 0.05 | Aeronautical Mobile (R) | HF Communications | 7 |
| 5.480 | 5.68 | MHz | 0.2 | Aeronautical Mobile (R) | HF Communications | 7 |
| 6.525 | 6.685 | MHz | 0.16 | Aeronautical Mobile (R) | HF Communications | 7 |
| 8.815 | 8.965 | MHz | 0.15 | Aeronautical Mobile (R) | HF Communications | 7 |
| 170.005 | 10.1 | MHz | 0.095 | Aeronautical Mobile (R) | HF Communications | 7 |
| 11.275 | 11.4 | MHz | 0.125 | Aeronautical Mobile (R) | HF Communications | 7 |
| 13.26 | 13.36 | MHz | 0.1 | Aeronautical Mobile (R) | HF Communications | 7 |
| 17.9 | 17.97 | MHz | 0.07 | Aeronautical Mobile (R) | HF Communications | 7 |
| 21.87 | 21.924 | MHz | 0.054 | Aeronautical Fixed | WARC 95 changed to fixed services | 7 |
| 21.924 | 22 | MHz | 0.076 | Aeronautical Mobile (R) | HF Communications | 7 |
| 74.8 | 75.2 | MHz | 0.4 | Aeronautical radionavigation | Marker beacons |  |
| 108 | 117.575 | MHz | 9.575 | Aeronautical radionavigation | ILS (Localiser 108-112 Mhz) VOR |  |
| 117.975 | 121.4 | MHz | 19 | Aeronautical Mobile (R) | VHF Communications - International and national |  |
| 121.5 |  | MHz |  | Aeronautical Mobile (R) | Emergency |  |
| 121.6 | 122 | MHz |  | Aeronautical Mobile (R) | International and national, reserved for ground movements, pre-flight checking, ATS clearances… |  |
| 122 | 123 | MHz |  | Aeronautical Mobile (R) | National |  |
| 123.1 |  | MHz |  | Aeronautical Mobile (R) | Auxiliary frequency Search and Rescue |  |
| 123.15 | 123.7 | MHz |  | Aeronautical Mobile (R) | National |  |
| 123.7 | 129.7 | MHz |  | Aeronautical Mobile (R) | International and national |  |
| 129.7 | 130.9 | MHz |  | Aeronautical Mobile (R) | National |  |
| 130.9 | 131.375 | MHz |  | Aeronautical Mobile (R) | National (AOC) |  |
| 131.4 | 131.975 | MHz |  | Aeronautical Mobile (R) | National |  |
| 132.0 | 136.775 | MHz |  | Aeronautical Mobile (R) | International and national |  |
| 136.8 | 136.875 |  |  | Aeronautical Mobile (R) | International and national (AOC) |  |
| 136.9 | 137 | MHz |  | Aeronautical Mobile (R) | International and national (AOC) |  |
| 328.6 | 335.4 | MHz | 6.8 | Aeronautical radionavigation | ILS (Glide path) |  |
| 406 | 406.1 | MHz | 0.1 | Mobile satellite | Earth space |  |
| 416.4 |  | MHz |  | Data Link | A/G/A Data |  |
| 416.675 |  | MHz |  | Data Link | A/G/A Data |  |
| 455.475 | 455.850 | MHz | 0.375 | Land mobile | Airports… |  |
| 460.775 | 461.225 | MHz | 0.450 | Land mobile | Airports… |  |
| 590 | 598 | MHz | 8 | Aeronautical radionavigation Radiolocation - sec | Ground Radar |  |
| 960 | 1215 | MHz | 255 | Aeronautical radionavigation | Tacan DME (ILS, VOR) |  |
| 1030 |  | MHz |  | Aeronautical radionavigation | SSR/IFF (G/A) - Mode S |  |
| 1090 |  | MHz |  | Aeronautical radionavigation | SSR/IFF (A/G) - Mode S |  |
| 1215 | 1350 | MHz | 135 | Radiolocation | Ground Radar - 1227 GPS | 5 |
| 1544 | 1545 | MHz | 1 | Mobile satellite | Satellite communications (Downlink) |  |
| 1545 | 1555 | MHz | 10 | Aeronautical Mobile satellite | Satellite communications (Downlink) | 1 |
| 1559 | 1610 | MHz | 51 | Aeronautical radionavigation satellite | 1574-1576 GPS, 1558.5 Glonass | 4 |
| 1610 | 1626.5 | MHz | 16.5 | Aeronautical radionavigation |  |  |
| 1645.5 | 1646.5 | MHz | 1 | Mobile satellite | Satellite communications (Uplink) |  |
| 1646.6 | 1656.5 | MHz | 10 | Aeronautical Mobile satellite | Satellite communications (Uplink) | 1 |
| 1670 | 1675 | MHz | 5 | TFTS | APC |  |
| 1800 | 1805 | MHz | 5 | TFTS | APC |  |
| 2700 | 3100 | MHz | 400 | Aeronautical radionavigation | Radar | 6 |
| 2900 | 3100 | MHz | 200 | Radiolocation - sec | Military and meteorological radars |  |
| 4200 | 4400 | MHz | 200 | Aeronautical radionavigation | Radio altimeters |  |
| 5000 | 5250 | MHz | 250 | Aeronautical radionavigation | Landing aids, Airborne aids MLS, | 2 |
| 5350 | 5460 | MHz | 110 | Aeronautical radionavigation | Airborne weather radars |  |
| 5460 | 5470 | MHz | 10 | Aeronautical radionavigation Maritime radionavigation Radiolocation - sec | Airborne weather radars Marine radars Military radars |  |
| 8750 | 8850 | MHz | 100 | Radiolocation Aeronautical radionavigation | Military radars Airborne doppler radar |  |
| 9000 | 9200 | MHz | 200 | Aeronautical radionavigation Radiolocation - sec | Harbour radar NIB to aeronautical military radar |  |
| 9300 | 9500 | MHz | 200 | Aeronautical radionavigation Maritime radionavigation Radiolocation - sec | Airfield radars, airborne weather radars shipborne radars, racans, harbour radars airfield radars |  |
| 13.25 | 13.4 | GHz | 0.150 | Aeronautical radionavigation | Doppler navigation aids |  |
| 15.4 | 15.7 | GHz | 0.300 | Aeronautical radionavigation | ASMI, Rafio altimeters | 3 |
| 15.7 | 17.3 | GHz | 1.6 | Radiolocation | ASMI |  |

1. These bands were allocated exclusively to the AMS(R)S until the WARC 97. They are now allocated to a generic Mobile Satellite Service (MSS). As a consequence, aviation will have to share the band with maritime and land-mobile users.

2. In the 5090-5150 MHz band allocated to the MLS, the introduction of allocations to the fixed-satellite service has led to restrictions to access by MLS.

3. These bands allocated to airborne and ground based radar systems now have to share use with the fixed-satellite service: protection of airborne radars against interference is no longer guaranteed on a global basis. Continued availability of the bands needs to be ensured against attempts by MSS operators to reduce the constraint imposed on their systems by the need to protect the band for aeronautical use.

4. At WARC 97, a proposal to introduce the Mobile Satellite Service (space-to-earth) in the 1559-1567 MHz band, i.e. within the 1559-1610 MHz band which is currently used by the Radionavigation Satellite Service for GNSS (GPS and Glonass) was only rejected after a long and intense debate. The review of the feasibility of an allocation of the 1559-1567 MHz band to MSS will take place in the WARC 99. In ICAO’s view, sharing is not feasible in practice: for civil aviation, this issue is seen as one of the most important in the coming WARC. The use of the 1559-1610 MHz band for fixed service (in more than 80 countries) may interfere with the reception of GNSS signals. In case harmful interference from the fixed service is proven likely, removal of the fixed service from this band will be necessary in order to allow the use of GNSS in those countries.

5. New allocation in the 1215-1300 MHz range to provide for frequency diversity in GPS and Glonass will be requested at WARC 99. Frequency diversity will improve the performance of satellite positioning systems.

6. Proposals are to be expected at WARC 99 to introduce non aeronautical services in the 2700-3100 MHz band, heavily used for ground based radar systems for aeronautical and maritime usage.

7. Review of the use of the HF band between 2.8 - 22 MHz allocated to AM(R)S will take place at WARC 99 and may affect the available HF spectrum for aviation, at a time when the need for spectrum is necessary for the introduction of HF datalink.

# Interviews

At the end of June 1998, some 70 interviews had been carried out by ATNI2

ATNI2 representatives were present during the ICAO CNS/ATM Conference in Rio, C/AFT meeting in Paris (May 1998) and ATN Implementation Task Force (April 1998).

| Organisation | # |
| --- | --- |
| - ACSPs: |  |
| ARINC Europe | 1 |
| ARINC USA | 1 |
| FCR (France Telecom) | 2 |
| Global One | 1 |
| Inmarsat | 1 |
| SITA | 3 |
| Swift | 1 |
| - Airlines |  |
| Air France | 2 |
| Alitalia | 1 |
| British Airways | 1 |
| FedEx | 2 |
| Virgin Atlantic | 3 |
| - ATSOs |  |
| AENA | 1 |
| CENA | 1 |
| DFS | 1 |
| DGAC | 2 |
| ENAV | 6 |
| Eurocontrol | 6 |
| Luftfartsverket | 1 |
| LVB | 1 |
| NATS Infrastructure Services | 3 |
| STNA | 1 |
| - Avionics - Telecommunications Systems - Aircraft Manufacturers |  |
| Aérospatiale | 3 |
| Alcatel | 2 |
| Alenia Difesa | 1 |
| Boeing | 1 |
| Cisco | 1 |
| Eurocae | 1 |
| Oki | 1 |
| Raytheon | 1 |
| Sextant Avionique | 1 |
| SICTA (ENAV-Alenia) | 1 |
| Sofreavia | 3 |
| Thomson Airsys | 1 |
| Vitrociset | 1 |
| - Other |  |
| ART | 2 |
| EC-DG13 | 2 |
| Météo France | 1 |
| SIA - Societa Interbancaria per l'Automazione | 1 |
| WMO | 1 |
| - Airports |  |
| ACI - Europe | 1 |
| Aéroport de Paris | 2 |
| Aeroporti di Roma | 1 |

# Warsaw Convention

Convention for the Unification of Certain Rules
Relating to International Carriage by Air
(Warsaw Convention)

Signed at Warsaw on 12 October 1929, As Amended By The Protocol Done At The Hague on 28 September 1955,  By The Protocol Done at Guatemala City on 8 March 1971 and By the Additional Protocols , Signed at Montreal, on 25 September 1975

(…)

Chapter III - Liability of the Carrier

Article 17 
1. The carrier is liable for damage sustained in case of death or personal injury of a passenger upon condition only that the event which caused the death or injury took place on board the aircraft or in the course of any of the operations of embarking or disembarking. However, the carrier is not liable if the death or injury resulted solely from the state of health of the passenger.

2. The carrier is liable for damage sustained in case of destruction or loss of, or of damage to, baggage upon condition only that the event which caused the destruction, loss or damage took place on board the aircraft or in the course of any of the operations of embarking or disembarking or during any period within which the baggage was in charge of the carrier. However, the carrier is not liable if the damage resulted solely from the inherent defect, quality or vice of the baggage.

3. Unless otherwise specified, in this Convention the term "baggage" means both checked baggage and objects carried by the passenger.

Article 18 
1. The carrier is liable for damage sustained in the event of the destruction or loss of, or damage to, any registered baggage, if the occurrence which caused the damage so sustained took place during the carriage by air.

2. The carrier is liable for damage sustained in the event of the destruction or loss of, or damage to, cargo upon condition only that the occurrence which caused the damage so sustained took place during the carriage by air.

3. However, the carrier is not liable if he proves that the destruction, loss of, or damage to, the cargo resulted solely from one or more of the following:

(a) inherent defect, quality or vice of that cargo;

(b) defective packing of that cargo performed by a person other than the carrier or his servants or agents;

(c) an act of war or an armed conflict;

(d) an act of public authority carried out in connexion with the entry, exit or transit of the cargo.

4. The carriage by air within the meaning of the preceding paragraphs of this Article comprises the period during which the baggage or cargo is in the charge of the carrier, whether in an airport or on board an aircraft, or, in the case of a landing outside an airport, in any place whatsoever.

5. The period of the carriage by air does not extend to any carriage by land, by sea or by river performed outside an airport. If, however, such carriage takes place in the performance of a contract for carriage by air, for the purpose of loading, delivery or transhipment, any damage is presumed, subject to proof to the contrary, to have been the result of an event which took place during the carriage by air.

Article 19 
The carrier is liable for damage occasioned by delay in the carriage by air of passengers, luggage or goods.

Article 20 
In the carriage of passengers and baggage, and in the case of damage occasioned by delay in the carriage of cargo, the carrier shall not be liable if he proves that he and his servants and agents have taken all necessary measures to avoid the damage or that it was impossible for them to take such measures.

Article 21 
1. In the carriage of passengers and baggage, if the carrier proves that the damage was caused by or contributed to by the negligence of the person suffering the damage the Court may, in accordance with the provisions of its own law, exonerate the carrier wholly or partly from his liability.

2. In the carriage of cargo, if the carrier proves that the damage was caused by or contributed to by the negligence or other wrongful act or omission of the person claiming compensation, or the person from whom he derives his rights, the carrier shall be wholly or partly exonerated from his liability to the claimant to the extent that such negligence or wrongful act or omission caused or contributed to the damage.

Article 22 
1. In the carriage of persons the liability of the carrier for each passenger is limited to the sum of 16 600 Special Drawing Rights. Where, in accordance with the law of the court seised of the case, damages may be awarded in the form of periodic payments, the equivalent capital value of the said payments shall not exceed this limit. Nevertheless, by special contract, the carrier and the passenger may agree to a higher limit of liability.

2. (a) In the carriage of registered baggage, the liability of the carrier is limited to a sum of 17 Special Drawing Rights per kilogramme, unless the passenger or consignor has made, at the time when the package was handed over to the carrier, a special declaration of interest in delivery at destination and has paid a supplementary sum if the case so requires. In that case the carrier will be liable to pay a sum not exceeding the declared sum, unless he proves that that sum is greater than the passenger's or consignor's actual interest in delivery at destination.

(b) In the carriage of cargo, the liability of the carrier is limited to a sum of 17 Special Drawing Rights per kilogramme, unless the consignor has made, at the time when the package was handed over to the carrier, a special declaration of interest in delivery at destination and has paid a supplementary sum if the case so requires. In that case the carrier will be liable to pay a sum not exceeding the declared sum, unless he proves that the sum is greater than the consignor's actual interest in delivery at destination.

(c) In the case of loss, damage or delay of part of registered baggage or cargo, or of any object contained therein, the weight to be taken into consideration in determining the amount to which the carrier's liability is limited shall be only the total weight of the package or packages concerned. Nevertheless, when the loss, damage or delay of a part of the registered baggage or cargo, or of an object contained therein, affects the value of other packages covered by the same baggage check or the same air waybill, the total weight of such package or packages shall also be taken into consideration in determining the limit of liability.

3. As regards objects of which the passenger takes charge himself the liability of the carrier is limited to 332 Special Drawing Rights per passenger.

4. The limits prescribed in this Article shall not prevent the court from awarding, in accordance with its own law, in addition, the whole or part of the court costs and of the other expenses of the litigation incurred by the plaintiff. The foregoing provision shall not apply if the amount of the damages awarded, excluding court costs and other expenses of the litigation, does not exceed the sum which the carrier has offered in writing to the plaintiff within a period of six months from the date of the occurrence causing the damage, or before the commencement of the action, if that is later.

5. The sums mentioned in terms of the Special Drawing Right in this Article shall be deemed to refer to the Special Drawing Right as defined by the International Monetary Fund. Conversion of the sums into national currencies shall, in case of judicial proceedings, be made according to the value of such currencies in terms of the Special Drawing Right at the date of the judgment. The value of a national currency, in terms of the Special Drawing Right, of a High Contracting Party which is a Member of the International Monetary Fund, shall be calculated in accordance with the method of valuation applied by the International Monetary Fund, in effect at the date of the judgment, for its operations and transactions. The value of a national currency, in terms of the Special Drawing Right, of a High Contracting Party which is not a Member of the International Monetary Fund, shall be calculated in a manner determined by that High Contracting Party.

Nevertheless, those States which are not Members of the International Monetary Fund and whose law does not permit the application of the provisions of paragraphs 1, 2(a) and 3 of Article 22 may at the time of ratification or accession or at any time thereafter, declare that the limit of liability of the carrier in judicial proceedings in their territories is fixed at a sum of 250 000 monetary units per passenger with respect to paragraph 1 of Article 22; 250 monetary units per kilogramme with respect to paragraph 2(a) of Article 22; and 5 000 monetary units per passenger with respect to paragraph 3 of Article 22. This monetary unit corresponds to sixty-five and a half milligrammes of gold of millesimal fineness nine hundred. These sums may be converted into the national currency concerned in round figures. The conversion of these sums into national currency shall be made according to the law of the State concerned.

6. The sums mentioned in terms of the Special Drawing Right in this Article shall be deemed to refer to the Special Drawing Right as defined by the International Monetary Fund. Conversion of the sums into national currencies shall, in case of judicial proceedings, be made according to the value of such currencies in terms of the Special Drawing Right at the date of the judgment. The value of a national currency, in terms of the Special Drawing Right, of a High Contracting Party which is a Member of the International Monetary Fund, shall be calculated in accordance with the method of valuation applied by the International Monetary Fund, in effect at the date of the judgment, for its operations and transactions. The value of a national currency, in terms of the Special Drawing Right, of a High Contracting Party which is not a Member of the International Monetary Fund, shall be calculated in a manner determined by that High Contracting Party. Nevertheless, those States which are not Members of the International Monetary Fund and whose law does not permit the application of the provisions of paragraph 2 (b) of Article 22 may, at the time of ratification or accession or at any time thereafter, declare that the limit of liability of the carrier in judicial proceedings in their territories is fixed at a sum of two hundred and fifty monetary units per kilogramme. This monetary unit corresponds to sixty-five and a half milligrammes of gold of millesimal fineness nine hundred. This sum may be converted into the national currency concerned in round figures. The conversion of this sum into the national currency shall be made according to the law of the State concerned.

Article 23 
1. Any provision tending to relieve the carrier of liability or to fix a lower limit than that which is laid down in this Convention shall be null and void, but the nullity of any such provision does not involve the nullity of the whole contract, which shall remain subject to the provisions of this Convention.

2. Paragraph 1 of this Article shall not apply to provisions governing loss or damage resulting from the inherent defect, quality or vice of the cargo carried.

Article 24 
1. In the carriage of passengers and baggage, any action for damages, however founded, can only be brought subject to the conditions and limits set out in this Convention, without prejudice to the question as to who are the persons who have the right to bring suit and what are their respective rights.

2. In the carriage of cargo, any action for damages, however founded, whether under this Convention or in contract or in tort or otherwise, can only be brought subject to the conditions and limits of liability set out in this Convention without prejudice to the question as to who are the persons who have the right to bring suit and what are their respective rights. Such limits of liability constitute maximum limits and may not be exceeded whatever the circumstances which gave rise to the liability.

Article 25 
In the carriage of passengers and baggage, the limits of liability specified in Article 22 shall not apply if it is proved that the damage resulted from an act or omission of the carrier, his servants or agents, done with intent to cause damage or recklessly and with knowledge that damage would probably result; provided that, in the case of such act or omission of a servant or agent, it is also proved that he was acting within the scope of his employment.

Article 25 A

1. If an action is brought against a servant or agent of the carrier arising out of damage to which the Convention relates, such servant or agent, if he proves that he acted within the scope of his employment, shall be entitled to avail himself of the limits of liability which that carrier himself is entitled to invoke under this Convention.

2. The aggregate of the amounts recoverable from the carrier, his servants and agents, in that case, shall not exceed the said limits.

3. In the carriage of passengers and baggage, the provisions of paragraphs 1 and 2 of this Article shall not apply if it is proved that the damage resulted from an act or omission of the servant or agent done with intent to cause damage or recklessly and with knowledge that damage would probably result.

Article 26 
1 . Receipt by the person entitled to delivery of luggage or goods without complaint is prima facie evidence that the same have been delivered in good condition and in accordance with the document of carriage.

2. In the case of damage, the person entitled to delivery must complain to the carrier forthwith after the discovery of the damage, and, at the latest, within seven days from the date of receipt in the case of baggage and fourteen days from the date of receipt in the case of cargo. In the case of delay the complaint must be made at the latest within twenty-one days from the date on which the baggage or cargo have been placed at his disposal.

3 . Every complaint must be made in writing upon the document of carriage or by separate notice in writing despatched within the times aforesaid.

4 . Failing complaint within the times aforesaid, no action shall lie against the carrier, save in the case of fraud on his part.

Article 27 
In the case of the death of the person liable, an action for damages lies in accordance with the terms of this Convention against those legally representing his estate.

Article 28 
1 . An action for damages must be brought, at the option of the plaintiff, in the territory of one of the High Contracting Parties, either before the Court having jurisdiction where the carrier is ordinarily resident, or has his principal place of business, or has an establishment by which the contract has been made or before the Court having jurisdiction at the place of destination.

2. In respect of damage resulting from the death, injury or delay of a passenger or the destruction, loss, damage or delay of baggage, the action may be brought before one of the Courts mentioned in paragraph 1 of this Article, or in the territory of one of the High Contracting Parties, before the Court within the jurisdiction of which the carrier has an establishment if the passenger has his domicile or permanent residence in the territory of the same High Contracting Party.

3 . Questions of procedure shall be governed by the law of the Court seised of the case.

Article 29 
1 . The right to damages shall be extinguished if an action is not brought within two years, reckoned from the date of arrival at the destination, or from the date on which the aircraft ought to have arrived, or from the date on which the carriage stopped.

2 . The method of calculating the period of limitation shall be determined by the law of the Court seised of the case.

Article 30 
1 . In the case of carriage to be performed by various successive carriers and falling within the definition set out in the third paragraph of Article 1, each carrier who accepts passengers, luggage or goods is subjected to the rules set out in this Convention, and is deemed to be one of the contracting parties to the contract of carriage in so far as the contract deals with that part of the carriage which is performed under his supervision.

2 . In the case of carriage of this nature, the passenger or his representative can take action only against the carrier who performed the carriage during which the accident or the delay occurred, save in the case where, by express agreement, the first carrier has assumed liability for the whole journey.

3 . As regards luggage or goods, the passenger or consignor will have a right of action against the first carrier, and the passenger or consignee who is entitled to delivery will have a right of action against the last carrier, and further, each may take action against the carrier who performed the carriage during which the destruction, loss, damage or delay took place. These carriers will be jointly and severally liable to the passenger or to the consignor or consignee.

Article 30 A

Nothing in this Convention shall prejudice the question whether a person liable for damage in accordance with its provisions has a right of recourse against any other person.

Chapter IV - Provisions Relating to Combined Carriage

Article 31 
1 . In the case of combined carriage performed partly by air and partly by any other mode of carriage, the provisions of this Convention apply only to the carriage by air, provided that the carriage by air falls within the terms of Article 1.

2 . Nothing in this Convention shall prevent the parties in the case of combined carriage from inserting in the document of air carriage conditions relating to other modes of carriage, provided that the provisions of this Convention are observed as regards the carriage by air.

Chapter V - General and Final Provisions

Article 32 
Any clause contained in the contract and all special agreements entered into before the damage occurred by which the parties purport to infringe the rules laid down by this Convention, whether by deciding the law to be applied, or by altering the rules as to jurisdiction, shall be null and void. Nevertheless for the carriage of goods arbitration clauses are allowed, subject to this Convention, if the arbitration is to take place within one of the jurisdictions referred to in the first paragraph of Article 28.

Article 33 
Except as provided in paragraph 3 of Article 5, nothing in this Convention shall prevent the carrier either from refusing to enter into any contract of carriage or from making regulations which do not conflict with the provisions of this Convention.

Article 34 
The provisions of Articles 3 to 8 inclusive relating to documents of carriage shall not apply in the case of carriage performed in extraordinary circumstances outside the normal scope of an air carrier's business.

Article 35 
The expression "days" when used in this Convention means current days not working days.

Article 35 A

No provision contained in this Convention shall prevent a State from establishing and operating within its territory a system to supplement the compensation payable to claimants under the Convention in respect of death, or personal injury, of passengers. Such a system shall fulfil the following conditions:

(a) it shall not in any circumstances impose upon the carrier, his servants or agents, any liability in addition to that provided under this Convention;

(b) it shall not impose upon the carrier any financial or administrative burden other than collecting in that State contributions from passengers if required so to do;

(c) it shall not give rise to any discrimination between carriers with regard to the passengers concerned and the benefits available to the said passengers under the system shall be extended to them regardless of the carrier whose services they have used;

(d) if a passenger has contributed to the system, any person suffering damage as a consequence of death or personal injury of such passenger shall be entitled to the benefits of the system.

# Council Regulation on air carrier liability in the event of accidents

Council Regulation (EC) No 2027/97 of 9 October 1997 on air carrier liability in the event of accidents
Official journal NO. L 285 , 17/10/1997 P. 0001 - 0003

THE COUNCIL OF THE EUROPEAN UNION,

Having regard to the Treaty establishing the European Community, and in particular Article 84 (2) thereof,
Having regard to the proposal from the Commission (1),
Having regard to the opinion of the Economic and Social Committee (2),
Acting in accordance with the procedure laid down in Article 189 (c) of the Treaty (3),
(1) Whereas, in the framework of the common transport policy, it is necessary to improve the level of protection of passangers involved in air accidents;
(2) Whereas the rules on liability in the event of accidents are governed by the Convention for the Unification of Certain Rules Relating to International Carriage by Air, signed at Warsaw on 12 October 1929, or that Convention as amended at The Hague on 28 September 1955 and the Convention done at Guadalajara on 18 September 1961, whichever may be applicable each being hereinafter referred to, as applicable, as the 'Warsaw Convention`; whereas the Warsaw Convention is applied worldwide for the benefit of both passengers and air carriers;
(3) Whereas the limit set on liability by the Warsaw Convention is too low by today's economic and social standards and often leads to lengthy legal actions which damage the image of air transport; whereas as a result Member States have variously increased the liability limit, thereby leading to different terms and conditions of carriage in the internal aviation market;
(4) Whereas in addition the Warsaw Convention applies only to international transport; whereas, in the internal aviation market, the distinction between national and international transport has been eliminated; whereas it is therefore appropriate to have the same level and nature of liability in both national and international transport;
(5) Whereas a full review and revision of the Warsaw Convention is long overdue and would represent, in the long term, a more uniform and applicable response, at an international level, to the issue of air carrier liability in the event of accidents; whereas efforts to increase the limits of liability imposed in the Warsaw Convention should continue through negotiation at multilateral level;
(6) Whereas, in compliance with the principle of subsidiarity, action at Community level is desirable in order to achieve harmonization in the field of air carrier liability and could serve as a guideline for improved passenger protection on a global scale;
(7) Whereas it is appropriate to remove all monetary limits of liability within the meaning of Article 22 (1) of the Warsaw Convention or any other legal or contractual limits, in accordance with present trends at international level;
(8) Whereas, in order to avoid situations where victims of accidents are not compensated, Community air carriers should not, with respect of any claim arising out of the death, wounding or other bodily injury of a passenger under Article 17 of the Warsaw Convention, avail themselves of any defence under Article 20 (1) of the Warsaw Convention up to a certain limit;
(9) Whereas Community air carriers may be exonerated from their liability in cases of contributory negligence of the passenger concerned;
(10) Whereas it is necessary to clarify the obligations of this Regulation in the light of Article 7 of Council Regulation (EEC) No 2407/92 of 23 July 1992 on licensing of air carriers (4); whereas, in this regard, Community air carriers should be insured up to a certain limit laid down in this Regulation;
(11) Whereas Community air carriers should always be entitled to claim against third parties;
(12) Whereas prompt advance payments can considerably assist the injured passengers or natural persons entitled to compensation in meeting the immediate costs following an air accident;
(13) Whereas the rules on the nature and limitation of liability in the event of death, wounding or any other bodily injury suffered by a passenger form part of the terms and conditions of carriage in the air transport contract between carrier and passenger; whereas, in order to reduce the risk of distorting competition, third-country carriers should adequately inform passengers of their conditions of carriage;
(14) Whereas it is appropriate and necessary that the monetary limits expressed in this Regulation be reviewed in order to take into account economic developments and developments in international fora;
(15) Whereas the International Civil Aviation Organization (ICAO) is at present engaged in a review of the Warsaw Convention; whereas, pending the outcome of such review, actions on an interim basis by the Community will enhance the protection of passengers; whereas the Council should review this Regulation as soon as possible after the review by ICAO,

HAS ADOPTED THIS REGULATION:


Article 1 
This Regulation lays down the obligations of Community air carriers in relation to liability in the event of accidents to passengers for damage sustained in the event of death or wounding of a passenger or any other bodily injury suffered by a passenger, if the accident which caused the damage so sustained took place on board an aircraft or in the course of any of the operations of embarking or disembarking.
This Regulation also clarifies some insurance requirements for Community air carriers.
In addition, this Regulation sets down some requirements on information to be provided by air carriers established outside the Community which operate to, from or within the Community.

Article 2 
1. For the purpose of this Regulation:
(a) 'air carrier` shall mean an air transport undertaking with a valid operating licence;
(b) 'Community air carrier` shall mean an air carrier with a valid operating licence granted by a Member State in accordance with the provisions of Regulation (EEC) No 2407/92;
(c) 'person entitled to compensation` shall mean a passenger or any person entitled to claim in respect of that passenger, in accordance with applicable law;
(d) 'ecu` shall mean the unit of account in drawing up the general budget of the European Communities in accordance with Articles 207 and 209 of the Treaty;
(e) 'SDR` shall mean a Special Drawing Right as defined by the International Monetary Fund;
(f) 'Warsaw Convention` shall mean the Convention for the Unification of Certain Rules Relating to International Carriage by Air, signed at Warsaw on 12 October 1929, or the Warsaw Convention as amended at The Hague on 28 September 1955 and the Convention supplementary to the Warsaw Convention done at Guadalajara on 18 September 1961 - whichever is applicable to the passenger contract of carriage, together with all international instruments which supplement, and are associated with, it and are in force.
2. Concepts contained in this Regulation which are not defined in paragraph 1 shall be equivalent to those used in the Warsaw Convention.

Article 3 
1. (a) The liability of a Community air carrier for damages sustained in the event of death, wounding or any other bodily injury by a passenger in the event of an accident shall not be subject to any financial limit, be it defined by law, convention or contract.
(b) The obligation of insurance set out in Article 7 of Regulation (EEC) No 2407/92 shall be understood as requiring that a Community air carrier shall be insured up to the limit of the liability required under paragraph 2 and thereafter up to a reasonable level.
2. For any damages up to the sum of the equivalent in ecus of 100 000 SDR, the Community air carrier shall not exclude or limit his liability by proving that he and his agents have taken all necessary measures to avoid the damage or that it was impossible for him or them to take such measures.
3. Notwithstanding the provisions of paragraph 2, if the Community air carrier proves that the damage was caused by, or contributed to by, the negligence of the injured or deceased passenger, the carrier may be exonerated wholly or partly from its liability in accordance with applicable law.

Article 4 
In the event of death, wounding or any other bodily injury suffered by a passenger in the event of an accident, nothing in this Regulation shall
(a) imply that a Community air carrier is the sole party liable to pay damages; or
(b) restrict any rights of a Community air carrier to seek contribution or indemnity from any other party in accordance with applicable law.

Article 5 
1. The Community air carrier shall without delay, and in any event not later than fifteen days after the identity of the natural person entitled to compensation has been established, make such advance payments as may be required to meet immediate economic needs on a basis proportional to the hardship suffered.
2. Without prejudice to paragraph 1, an advance payment shall not be less than the equivalent in ecus of 15 000 SDR per passanger in the event of death.
3. An advance payment shall not constitute recognition of liability and may be offset against any subsequent sums paid on the basis of Community air carrier liability, but is not returnable, except in the cases prescribed in Article 3 (3) or in circumstances where it is subsequently proved that the person who received the advance payment caused, or contributed to, the damage by negligence or was not the person entitled to compensation.

Article 6 
1. The provisions contained in Articles 3 and 5 shall be included in the Community air carrier's conditions of carriage.
2. Adequate information on the provisions contained in Articles 3 and 5 shall, on request, be available to passengers at the Community air carrier's agencies, travel agencies and check-in counters and at points of sale. The ticket document or an equivalent shall contain a summary of the requirements in plain and intelligible language.
3. Air carriers established outside the Community operating to, from or within the Community and not applying the provisions referred to in Articles 3 and 5 shall expressly and clearly inform the passengers thereof, at the time of purchase of the ticket at the carrier's agencies, travel agencies or check-in counters located in the territory of a Member State. Air carriers shall provide the passengers with a form setting out their conditions. The fact that only a liability limit is indicated on the ticket document or an equivalent shall not constitute sufficient information.

Article 7 
No later than two years after the entry into force of this Regulation, the Commission shall draw up a report on the application of the Regulation which, inter alia, takes into account economic developments and developments in international fora. Such report may be accompanied by proposals for a revision of this Regulation.

Article 8 
This Regulation shall enter into force one year after the date of its publication in the Official Journal of the European Communities.

This Regulation shall be binding in its entirety and directly applicable in all Member States.
Done at Luxembourg, 9 October 1997.
For the Council
The President
M. DELVAUX-STEHRES

(1) OJ C 104, 10. 4. 1996, p. 18 and OJ No C 29, 30. 1. 1997, p. 10.
(2) OJ C 212, 22. 7. 1996, p. 38.
(3) Opinion of the European Parliament of 17 September 1996 (OJ No C 320, 28. 10. 1996, p. 30), Council Common Position of 24 February 1997 (OJ No C 123, 21. 4. 1997, p. 89) and Decision of the European Parliament of 29 May 1997 (OJ C 182, 16. 6. 1997).
(4) OJ L 240, 24. 8. 1992, p. 1.

Today these services are offered on a world-wide basis by four ACSPs: SITA (Aircom service) world-wide except for the USA; ARINC, mainly in the USA but since recently also available in some other areas; Air Canada in Canada; and AVICOM in Japan.

From "Nav Canada, a Blueprint for Air Traffic Control Commercialization", Duncan McCallum, Vice President & Director, Investment Banking RBC Dominion Securities, Communication at ICAO Conference, Rio de Janeiro, May 1998.

"It is widely recognized that there are substantial opportunities to improve the management of our operations across Canada. In particular, there are opportunities to reduce duplication within regions by consolidating operations into single facilities, thus benefiting from lower costs at the same or better levels of safety", statement made in the Nav Canada Strategic Plan (Shaping our Future, 1997-2000, Sept. 1997).

Overflight enroute charges are levied against aircraft which fly over Canadian sovereign airspace but do  not take-off or land in Canada.

Previously, carriers acted only as agents in collecting the Air Transportation Tax from passengers.

Safety Oversight of the Civil Air Navigation System - A framework, Office of ANS & Airspace Safety Oversight, Transport Canada, Web Site.

Idem