WGA 2nd Meeting (Honolulu, Hawaii, U.S.A)

5th March 2001

WGA/02 – Flimsy_02REVA

ATNP WGA Responses to the METLINKSG comments

on Doc 9705 draft Edition 3

# Introduction

The METLINK Study Group provided ATNP with comments on Doc 9705  - draft Edition 3 - Manual of Technical Provisions for the ATN (email dated 23/11/00 and message dated 02/01/01), as reproduced below:

Reference is made to the draft Sub-Volume II which was sent to the Secretaries of the relevant  panels and study groups for comment.

It is noted with pleasure that the most recent versions (i.e. "Extended MET info" in the case of the ADS message and "Version 2" in the case of FIS messages) have been fully aligned with the Annex 3 - Meteorological Service for International Air Navigation provisions.  Therefore, there are few suggestions which are related to the "last-minute" changes introduced as a result of the final review by the Air Navigation Commission (ANC) (Note that the earlier versions were not reviewed and that the comments hereunder refer to the updated versions as it is understood that it would not be possible to amend the earlier versions at this stage):

- ADS

- Page II-113, item v).  It should read: "meteorological information, indicating wind direction, wind speed, temperature and turbulence (intensity), and in version 2, wind direction, wind speed, wind quality flag, temperature, turbulence (index and time of occurrence) and humidity." (reason: the time of occurrence of turbulence cannot be separated from the turbulence index since, as a stand-alone value, it would be of no use);

- CPDLC

- Page II-326.  The range and resolution of the altimeter(metric) should read "range (850...1100)" and "resolution = 1";

- Page II-332.  According to the Procedures for Air Navigation Services - Rules of the Air and Air Traffic Services (PANS-RAC, Doc 4444), there are only "light", " moderate" and "severe" icing; therefore, the category (0) should be deleted (or labelled "nil");

- Page II-343.  The resolution for "RVR Meters" should be amended to read: 25 m for RVR (0...400); 50 m for RVR (400...800) and 100 m for RVR (800..1500);

- Page II-348.  The ranges and resolutions for "Wind Speed English" and "Wind Speed Metric" should read (0...250), resolution = 1 and (0...500), resolution = 2, respectively;

- FIS (only Version 2 commented on)

- Page II-513.  The ranges for the wind speed should be amended to read (0...399) if expressed in km/h, (0...199) if expressed in kt and (0...99) if expressed in m/s (reason: the ranges were amended as a result of the final review by the ANC);

- Pages II-523 and II-524.  Under "Extended Runway Designator", provision should be made for the use of code figure "88" (i.e. all runways), which is used in the EUR region.  Furthermore, it may be noted that the term "SNOCLO" is used in lieu of  the "Runway state" group when the aerodrome is closed due to extreme deposit of snow (note: the latter change was omitted due to an oversight in the initial amendment proposal to Annex 3 but it has since been introduced in Amendment 72 to Annex 3 which will be considered by the ICAO Council in March 2001)

.

# ATNP responses to METLINKSG

These comments were reviewed by WGA – Sub-Group A2 (Air/Ground ATN Applications) and the ATNP Configuration Control Board during the ATNP WG meetings in March 2001. Two Proposed Defect Reports (PDR Numbers M0110002 and M1010002) were created to document the discussion. The following resolutions were agreed:

- ADS: The proposed text will be fully integrated in Doc 9705 Edition 3.

- DFIS: The ASN.1 definition for Wind Speed, Runway State and Extended Runway Designator will be changed as proposed in the METLINKSG comments.

- CPDLC: The ASN.1 definition for Icing will be modified to fully align the PANS-RAC definition. The proposed changes on the range and resolution of Altimeter, RVR and Wind Speed will not be implemented, given that the current ASN.1 definition already supports the proposed range and resolution. In addition, implementing these changes would jeopardise the interoperability capability between existing CPDLC version 1 systems and version 2 systems.

# Recommendation

The METLINKSG is invited to note that all comments made on Doc 9705 so far have been considered by ATNP and have resulted in amendments of the current draft, when applicable, and as described in this flimsy.