# Aeronautical Telecommunication Network Panel (ATNP)

Working Group B

- Information Paper

- A Comparison of the Connection-Less Network Protocol (CLNP) to the Internet Protocol (IP) versions 4 and 6 (IPv4/IPv6)

- Prepared by: Leon Sayadian

- Presented by: Leon Sayadian (USA)

March 2002

This paper provides a comparison between Aeronutical Telecommunican Network (ATN)/International Standards for Organization (ISO) CLNP, Internet Protocol version 4, and 6 features and services (including IPv6 list of vendors).

### 1.0 Background

Both Connection-Less Network Protocol (CLNP) and Internet Protocol (IP) support connection-less services. They are functionally similar; the major differences between the two include addressing, Quality of Service (QoS) parameters and packet formats.  IPv4 is currently facing limitations on addressing, routing, security, optional services, and Quality of Service (QoS) capabilities.  To mitigate many of these issues, the Internet Engineering Task Force (IETF) reviewed the IPv4 network layer and the Internet Control Message Protocol version 4 (ICMPv4) capabilities.  As a result, enhancements were provided in a series of Request For Comments (RFCs) documents describing the next generation Internet Protocol (IPng), including IPv6, and ICMPv6. This paper will discuss these issues and provide a tabular comparison between the subject protocols.

### 2.0 Discussion

To assist the Aeronautical Telecommunication Network (ATN) Working Group B in implementing connectivity among Ground-Ground Air Traffic Management (ATM) systems, this section discusses the various features of applicable subnetwork protocol functions and services of CLNP, IPv4, and IPv6.

## 2.1 Header Comparison

- Table 1 provides a comparison between ATN/ISO CLNP, IPv4 and IPv6 header formats and services.

Figurs 1, 2, 3, 4 and 5 depict the various formats of CLNP, IPv4, basic IPv6, IPv6 with extension header packets and ICMP error messages.

Table 1		Comparison of ATN/CLNP to IPv4/6

| Services | ATN/CLNP | IPv4 | IPv6 | Comments |
| --- | --- | --- | --- | --- |
| Addressing Size | 20 bytes | 4 bytes | 16 bytes | RFC 1888 describes an experimental method to map IPv6 addressing space to CLNP |
| Addressing Class Structure | None | 4 classes or Classless Inter-Domain Routing (CIDR) | None | CLNP and IPv6 offer more efficient routing due to no class structure |
| Special Addressing | Multicast/Broadcast | Multicast/Broadcast | Multicast/Anycast | IPv6 offers anycast addressing, to send a packet to any one of a group of nodes |
| Embedded Hierarchical Addresses | Supported through NSAP address structure | Not available due to IP addressing size | Not available due to IP addressing size | NSAP addressing structure supports embedded hierarchical addressing for end-to-end services. |
| Security | Network layer security as per ISO DIS 11577 Security option in ISO/IEC 8348 | Optional support for conveyance of security level, restriction codes, and user group parameters (RFC 1108 and 1825) | Supports IPSec with RFC 2401 | IPv6 supports authentication, data integrity and confidentiality, encryption, PPP, VPN under IPSec. |

Table 1 (con’t)		Comparison of ATN/CLNP to IPv4/6

| Services | ATN/CLNP | IPv4 | IPv6 | Comments |
| --- | --- | --- | --- | --- |
| Quality of Services (QoS) | QoS supported (see Fig. 1) | Limited to ToS (see RFC 791) | QoS supported with Traffic Class and Flow Label (see RFC 2460) | QoS in CLNP and IPv6 provides more robust quality-ensuring services than IPv4 |
| Support for Mobility | ISO NSAP addressing will allow future mobility support | Public domain software is available that supports this functionality using RFCs 2002, 2290, and 2794. | In accordance with RFC 2026 (may be modified by Internet Draft “Mobility Support in IPv6”, July 2, 2001) | RFC not yet approved for Mobile IPv6 |
| Header Length | 1 Octet, in units of octets | 4 bits, in units of 32-bit words | Removed | IPv6 does not include an Internet Header Length field |
| Version Identification | 1 Octet | 4 bits | 4 bits | -- |
| Segment/fragment offset | 16 bits, in units of octets (Segment) | 13 bits, in units of octets (Fragment) | Not present; See comment | No fragmentation allowed in basic IPv6; see Figure 4 for IPv6 extension that allows fragmentation for packets larger than Maximum Transmission Unit (MTU) |
| Protocol Identifier | 1 Octet | 1 Octet | 1 Octet | IPv6 Next Header field is equivalent to IPv4 Protocol field |
| Total length | 16 bits, in units of octets | 16 bits, in units of octet | Similar to IPv4 Total Length, with exception – see Comment | IPv4 Total Length measures header and data; IPv6 Payload Length only measures data |
| Packet Longevity | 1 Octet, in 500 ms units (Lifetime) | 1 Octet, in units of seconds (Time to Live) | 1 Octet (Hop Limit) | CLNP and IPv4 measure packet longevity in time; IPv6 limits number of hops for each packet |
| Packet Header size | Variable | 20-40 octet | 40 Octet | IPv6 header is 40 octets. As option Extension Header can be add (see RFC 2406) |
| Header Checksum | 2 Octet | 2 Octet | Removed | In IPv6 error detection for packet is performed by the link layer |
| Flags | Segmentation permitted; More segments; error reports | Don’t fragment; More fragments | Removed | Fragmentation for IPv6 contained on extension header |

Table 1 (con’t)		Comparison of ATN/CLNP to IPv4/6

| Services | ATN/CLNP | IPv4 | IPv6 | Comments |
| --- | --- | --- | --- | --- |
| Options | Priority Complete source routing Partial source routing Record route Padding Not present Reason for discard (ER PDU only) | Precedence bits in ToS Strict source route Loose source route Record route Padding Timestamp N/A | Removed | IPv4 options are replaced by IPv6 extension header (see RFCs 2460, 2402, and 2406) |

# 2.2 Error Reporting

- Table 2 compares the features and services of ICMPv4, ICMPv6, and CLNP Error Reporting.

- Table 2 Comparison of ATN CLNP Error Reports and ICMPv4/6 Messages

| Category | CLNP Error Report | IPv4 ICMP Message | IPv6 ICMP Message | Comments |
| --- | --- | --- | --- | --- |
| General | Reason not specified | Parameter problem -Type 12, code 0 (see RFC 792) | Parameter problem | See note below |
|  | Protocol procedure error | Parameter problem (same as above) | Parameter problem | See note below |
|  | Incorrect checksum | Parameter problem (same as above) | Parameter problem | See note below |
|  | PDU discarded due to congestion | Source quench - Type 4, code 0 (see RFC 792) | N/A | See note below |
|  | Header syntax error (cannot be parsed) | Parameter problem –Type 12 , code 0 (see RFC 792) | Parameter problem | See note below |
|  | Incomplete PDU received | Parameter problem (same as above) | Parameter problem | See note below |
|  | Duplicate option | Parameter problem (same as above) | Parameter problem | See note below |
|  | Segmentation needed, not permitted | Destination Unreachable - Fragmentation needed, but Don’t Fragment flag is set -Type 3, code 4 (see RFC 792) | Packet Too Big -Type 2, code 0 (see RFC 2463) | -- |
| Addressing-related | Destination address unreachable | Destination unreachable- host unknown - Type 3, code 1 (see RFC 792) | Destination unreachable – Address unreachable -Type 1, code 3 (see RFC 2463) | Type/Code values are different between IPv4 and IPv6 |
|  | Destination address unknown | Destination unreachable- Network unreachable - Type 3, code 1 (see RFCs 792 & 1122 ) | Destination unreachable- No routing to destination -Type 1, code 0 (see RFC 2463) | Type/Code values are different between IPv4 and IPv6 |
| Source routing | Unspecified source-routing error | Destination unreachable – Type 3, Code 5 (see RFCs 792) | Not supported | -- |

Table 2 Comparison of ATN CLNP Error Reports and ICMPv4/6 Messages (con’t)

| Category | CLNP Error Report | IPv4 ICMP Message | IPv6 ICMP Message | Comments |
| --- | --- | --- | --- | --- |
|  | Syntax error in source routing field | Parameter problem – Type 12, Code 0 (see RFC 792) | Not Supported | Type/Code values are different between IPv4 and IPv6 |
|  | Unknown address in source-routing field | Destination unreachable – Type 3, Code 5 (see RFCs 792 & 1122) | Not Supported | -- |
|  | Path not acceptable | Destination unreachable – Type 3, Code 5 (see RFCs 792 & 1122) | Not Supported | -- |
| Lifetime | Lifetime expired while data unit was in transit | Time Exceeded - Time to live exceeded in transit -Type 11, code 0 (see RFC 792) | Time Exceeded – hop limit exceeded in transit -Type 3, code 0 (see RFC 2463) | Type/Code values are different between IPv4 and IPv6 |
|  | Lifetime expired during reassembly | Time Exceeded - fragment reassembly time exceeded -Type 11, code 1 (see RFC 792) | Time exceeded - fragment reassembly time exceeded -Type 3, code 1 (see RFC 2463) | Type/Code values are different between IPv4 and IPv6 |
| Reassembly | Reassembly interference | Time Exceeded -fragment reassembly time exceeded -Type 11, code 1 (see RFC 792) | Time exceeded - fragment reassembly time exceeded -Type 3, code 1 (see RFC 2463) | Type/Code values are different between IPv4 and IPv6 |
| PDU discarded | Unsupported option not specified | Parameter problem -Type 12, code 0 (see RFC 792) | Parameter problem – Type 4, Code 2 (see RFC 2463) | See note below |
|  | Unsupported protocol version | Parameter problem (same as above) | Parameter problem (same as above) | See note below |
|  | Unsupported security option | Parameter problem (same as above) | Parameter problem (same as above) | See note below |
|  | Unsupported source-routing option | Parameter problem (same as above) | Parameter problem (same as above) | See note below |
|  | Unsupported recording of route option | Parameter problem (same as above)-Type | Parameter problem (same as above) | See note below |

Note:

An ICMPv6 Parameters Problem message is either sent by a router or by the destination. This occurs when an error is detected in either the IPv6 header or in an extension header - see RFC 2463.

### 3.0 Vendor implementation of IPv6

The following is a list of selected manufacturers, product categories, and site addresses for IPv6 – related Internetworking Products:

- Cisco System Inc.                           Router  ( www.cisco.com)

- Compaq Computer Corp.               End-systems (http://www.compaq.com/ipv6/Tru64UNIX.html)

- IBM                                                Router, End-systems (http://www.networking.ibm.com/220/220over.html)

- Silicon Graphics, Inc.		     End-systems (http://www.sgi.com/peripherals/networking/future_tech.html)

- 3 Com Corporation                        Router (http://www.3com.com/corpinfo/en_US/pressbox/press_release.jsp?INFO_ID=2001772)

- Mentat Inc.,                                   End-systems (http://www.mentat.com/tcp/tcpfaq.html - #functionality)

- Nokia, Inc.			    VoIP (http://www.nokia.com/ipv6/backgrounder_1.html)

- Sun Microsoft systems Inc.,          End-systems (http://www.sun.com/solaris/ipv6/)

- Ericsson Telebit 		    Router (http://www.ericsson.com/infocenter/news/ipv6_trial.html)

- Hewlett-Packard                           End-systems (http://www.software.hp.com/cgi-bin/swdepot_parser.cgi/cgi/displayProductInfo.pl?productNumber=T1305AA)

- Hitachi, Ltd.			    Gateway/Router (http://www.hitachi.co.jp/Prod/comp/network/nr60e.htm)

- Microsoft, Inc.		    End-systems (http://www.research.microsoft.com/msripv6/)

### 4.0 References

# 4.1 Request For Comments (RFCs)

RFC 791	Internet Protocol, Version 4 (IPv4) Specification, Sept. 1981

RFC 792	Internet Control Message Protocol, Sept. 1981

RFC 950	Internet Standard Subnetting Procedure, Aug.1985

RFC 1108	U.S. DoD Security Options for the IPv4, Nov. 1991

RFC 1122	Requirements for Internet Hosts -- Communication Layers, Oct. 1989

RFC 1191	Path MTU Discovery, Nov. 1990

RFC 1561	Use of ISO CLNP in TUBA Environments, Dec. 1993

RFC 1825	Security Architecture for the IPv4/IPv6, Aug. 1995

RFC 2002	IP Mobility Support, Oct. 1996

RFC 2026	The Internet Standard Process – Revision 3, Oct. 1996

RFC 2290	Mobile – IPv4 Configuration Option for PPP IPCP, Feb. 1998

RFC 2401	Security Architecture for IPSec, Nov. 1998

RFC 2402	IP Authentication Header, Nov. 1998

RFC 2406	IP Encapsulation Security Payload (ESP), Nov. 1998

RFC 2460	Internet Protocol, Version 6 (IPv6) Specification, Dec. 1998

RFC 2463	Internet Control Message Protocol (ICMPv6) for the IPv6 Specification, Dec. 1998

RFC 2794	Mobile IP Network Access Identifier Extension for IPv4, March 2000

# 4.2 ISO Standards

ISO/IEC 8473-1 	Information technology – Protocol for providing the connectionless-mode network services, 1994(E)

# 4.3 Academic References

W. Richard Stevens and Gary R. Wright: TCP/IP Illustrated Volume 1 and 2 the Protocols, 1996

David M. Piscitello and A. Lyman Chapin: Open Systems Networking TCP/IP and OSI, 1994

### 5.0 Abbreviations

ATN	Aeronautical Telecommunication Network

CIDR	Classless Inter-Domain Routing

CLNP 	Connection-Less Network Protocol

DoD 	(US) Department of Defense

ER          Error Report

FAA	Federal Aviation Administration

ICMP	Internet Control Message Protocol

IETF	Internet Engineering Task Force

IPng	Internet Protocol next generation

IPSec	Internet Protocol security

IPCP 	Internet Protocol Control Protocol

IPv4/6	Internet Protocol version 4 or 6

MTU 	Maximum Transmission Unit

N/A	Not Available or Applicable

PDU	Protocol Data Unit

PPP 	Point-to-Point Protocol

QoS	Quality of Services

RFC	Request For Comments

ToS	Type of Service

TUBA 	TCP/UDP with Bigger Addresses

VPN	Virtual Private Network