|  | ATNP/WGB/WP 02- 26th February 2001 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP B (ATN COMMUNICATION SERVICES)

Honolulu, Hawaii, 26st February– 3rd March 2001

B2 - Infrastructure Communication Services

Secured Dialogue Service – Contribution to Guidance material

Prepared by: G. Mittaux-Biron

Presented by: A. Dedryvère

SUMMARY

This working paper is proposed as a contribution to the guidance material of SV 4 including the Security mechanisms.

## S-ASO Service

### Introduction

The SA-Service is a service that allows two users to establish a shared security context (when the Dialogue supports key management), to protect their exchanges (in term of integrity and peer entities authentication), and, in case of security exception, to cancel a previously established security context.

### Description and rationale

On analysis of the requirements for provision of security to ATN applications, it was found that, with respect to security, two kinds of SA-Service Users could be identified:

- a pair of SA-Service users that establish a set of shared security information, the security context,

- a pair of SA-Service users that take advantage of the previously established security context to protect their exchanges by appending security tags to the transferred data.

As a consequence, the services that needed to be provided by the S-ASO were found to be the following:

- A service that provides support for key management between peer users. This service allows for establishing a set of security information, the security context, that is shared by the two communicating entities,

- A service that provides support for data integrity and peer entity authentication during data transfer phases,

Each ATN application includes an element, the S-ASO, which provides the services for protecting the communications with a peer application. The S-ASO uses the services provided by two different types of functions:

- The System Security Function, performs the security-related processing, i.e. computing and checking signatures, computing and appending security appendixes to user information, checking the validity of security appendixes, and removal of security contexts. The System Security Function is visible at its external boundaries which are, in the case of the Upper Layers, described in Sub-Volume VIII Chapter 6 “ATN System Security Object”. The S-ASO takes advantage of the following set of functionality:

  - SSO-GetCertificatePath, which is activated in order to retrieve a certificate,

  - SSO-Sign, which is activated in order to compute a security appendix (signature or mac tag),

  - SSO-ProtectSign, which is activated in order to protect user information by appending security information,

  - SSO-SignCheck, which is activated in order to check a security appendix (signature or mac tag),

  - SSO-ProtectSignCheck, which is activated in order to check the security information appended to protected information,

  - SSO-Stop, which is activated in order to remove security contextual information between two peers.

- The Security Communication Function supports the transfer of security related information between the communicating peers. In the case of the ATN, this is an OSI standard ASE, the Secured Exchanges Service Element (SESE), which involves a set of security exchange functions. The transfer of application-protocol-control-information that supports the communication of security information is modelled by the Security Exchanges. For the purpose of the ATN, the following security exchanges have been defined:

  - The atnEstablishSE security exchange: this exchange involves the transfer of two pieces of information under the form of atnEstablish security exchange items, and supports a 2-way session-key negociation, together with entity authentication and insurance of data integrity. Each exchanged item may contain entity certificates, encryption algorithm identification, validity information (time field, random number, exchange counter), and an appendix (signature or mac tag) used both to authenticate the communicating entities and to validate the integrity of the transferred data.

  - The atnProtectSignSE security exchange: this exchange involves the transfer of one piece of information under the form of atnProtectSign security exchange items, and supports insurance of data integrity and data origin authentication. Each exchanged item may contain, together with unprotected user data, encryption algorithm identification, validity information (time field, random number, exchange counter), and an appendix (signature or mac tag) used both to authenticate the communicating entities and to validate the integrity of the transferred data.

### Scope

The SA-Service allows two systems to establish or renew a set of common security information (security context) that will be shared by two communicating systems. Once established, the SA-Service allows the use of this context in order to transfer protected information on Dialogues established by the ASEs.

Note that any security related error will be managed exactly the same way as a protocol error, and will cause a provider abort of the related Dialogue, with loss of user information. Returned cause will report a communication problem: no security specific diagnostic or information is provided to any user, and no correlation is made with other Dialogues. As a consequence, a security related error leads to the incapacity of some ASEs or all the ASEs to establish secured Dialogue with their remote peers until a new security context has been established, i.e. a new Dialogue supporting key management is established.

### Mapping of S-ASO service primitives

The following chapters illustrate the sequence of services that are invoked following a D-START request. Three cases arise, one related to air initiated Dialogue establishment, with support for key management, the second, related to ground initiated Dialogue establishment, with support for key management, and the last, related to secured Dialogues.

Each case description is organized as follows:

- A description of the security requirements together with a description of how they have been taken into account and then mapped into Application service primitives,

- The sequence of service primitive activation, on initiator and receiver sides. This description of the service activation is associated with an explanation of the parameter, when relevant.

- A summary of the scenario, under the full sequence of service primitive activation.

#### Secured Dialogue supporting key management

##### Air initiated Air-Ground Communications

###### Dialogue establishment: Request-Indication phase

Mapping the expression of the need to upper layers functionality

Initiator side

The expressed need : «The aircraft CMA entity forms a CM-logon request message Data1 consisting of some login data including an indication that it is requesting a secure connection, its identity U, the ground CMA entity’s identity V, a time field TimeU, and a field Applications containing a list of which air-ground applications it may wish to invoke. It signs Data1 using ECDSA under its private signing key dsig,U : sU = Sign (dsig,U; Data1). The aircraft CMA entity sends the CM-logon request message along with sU to the ground CMA. The aircraft CMA entity retains its signature sU for use later. »

The translation of the need : « When a request for establishing a Dialogue specifies that this Dialogue should support key management, there need to be a security item sent from the initiating peer to the receiving peer, which permits the former to provide security related information to the latter.»

- The implementation of the need : « The D-START Request following the CM-LOGON Request, because it contains a request for establishing a Dialogue supporting Key Management, triggers the computation of an appendix (sU) on the D-START Request User data, the airborne (U) and ground (V) Dialogue users and a locally computed time field (TimeU). The Dialogue control function then transmits to its peer : the D-START Request User Data, the airborne Dialogue user (U), the time field (TimeU) and the appendix (sU). To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing a validity element (the time field (TimeU) and the appendix (sU)), and an AARQ APDU containing the identity of the Dialogue User (U) in calling-AP-title and calling-AE-qualifier, the D-START Request user data in user-information, the sender-acse-requirements set to « Authentication », and the SESE APDU in the calling-authentication-value. » This AARQ APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.1.1.1.2.

Receiver side :

- The expressed need : « The ground CMA entity receives the CM-logon request message along with sU and sees that the message requests a secure connection. It recovers from the message the aircraft CMA entity’s identity U, its identity V, the time field TimeU, and the application list. The ground CMA entity then retrieves certificate paths from the public key of its State CA to the aircraft CMA entity U’s public keys along with the associated CRLs, and certificate paths from the public key of its State CA to the appropriate ground application entities’ key agreement public keys along with the associated CRLs using its Certificate Delivery Service. It checks the certificate paths and CRLs using its copy of its State CA’s public key. It retrieves the aircraft CMA entity’s public signing key Qsig,U from its signature key certificate. It verifies the received signature sU is a valid ECDSA signature on Data1 using Qsig,U, and it verifies that V is correct and TimeU corresponds with the current time.»

- The translation of the need : « When the receiving peer receives the security item containing security information, it first checks the validity of the associated information and then computes the security related data which it will use for subsequent exchanges with its peer.»

- The implementation of the need : « The incoming AARQ APDU, because it specifies need for authentication, triggers the check of the appendix (sU), using the identity of the peer Dialogue User (U), the identity of the local Dialogue User (V), the user information, and the time field (TimeU) contained in the AARQ APDU. The need for authentication is specified, in the incoming AARQ APDU, by the fact that the sender-acse-requirements is set to « Authentication » ; this triggers the check of the appendix (sU), and the time field (TimeU) contained in the SESE APDU, itself contained in the calling-authentication-value field, together with the identity of the peer Dialogue User (U) contained in the calling-AP-title and calling-AE-qualifier fields, the identity of the local Dialogue User (V) and the user-information field of the AARQ APDU. »

Associated service activations

Initiator side

The CM-LOGON Request leads, according to «2.1.3.2.6.1 Upon receipt of a CM-logon service request », to the activation of the D-START Request service with the following parameters:

- D-START Request

| Called Peer ID | CM LOGON facility Designation |
| --- | --- |
| Called Sys-ID |  |
| Called Presentation Address |  |
| Calling Peer ID | CM LOGON Aircraft Address |
| Calling Sys-ID |  |
| Calling Presentation Address |  |
| DS-User Version Number | CM-air-ASE version number = 2 |
| Security Requirements | Abstract value “Secured Dialogue Supporting Key Management”. |
| Quality-of-Service | Routing Class: CM LOGON Class of Communication Priority: “flight regularity communications” RER: “Low” |
| User Data | CMAircraftMessage APDU (cmLogonRequest) |

- SA-START Request

| Calling Entity ID | Airborne CM |
| --- | --- |
| Called Entity ID | Ground CM |
| User Data | CMAircraftMessage APDU (cmLogonRequest) |

- SSO-Sign

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| Appendix type | Signature-appendix |
| User data | CMAircraftMessage APDU (cmLogonRequest) |

The Destination peer addresses the ground CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.2.2.2 a) ) from the ICAO Facility Designator present in the SA-START Request Calling Entity ID parameter. The Source peer addresses the airborne CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.2.2.2 a) ) from the ICAO Aircraft Identifier present in the SA-START Request Calling Entity ID parameter.

- SE-Transfer Request

| Security exchange identifier | atnEstablishSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnEstablish { atnCertificates: absent atnSignature { algorithmId : absent validity : timeField(ATNSecurityDateTime) value : ecdsa-Signature(ECDSA-Sig-Value) } } |
| Item identifier | atn-establish = {1 3 27 5 1 2} |
| Start flag | True |
| End flag | Not used |

- A-Associate Request

| Mode | Not Used (default value) |
| --- | --- |
| Application Context Name | CM ASE = {1 3 27 3 2} |
| Application Context Name List | Not Used |
| Calling AP Title | Retrieved from Calling Peer ID (CM LOGON Aircraft Address), of from local knowledge. |
| Calling AE Qualifier | Not Used (No Calling Sys-ID) |
| Calling AP Invocation-identifier | Not Used |
| Calling AE Invocation-identifier | Not Used |
| Called AP Title | Not Used |
| Called AE Qualifier | Not Used |
| Called AP Invocation-identifier | Not Used |
| Called AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not Used |
| Authentication-value | SESE PDU |
| User Information | D-Start User data (CMAircraftMessage APDU (cmLogonRequest)) |
| Calling Presentation Address | Local Calling Presentation Address |
| Called Presentation Address | Derived from Called Peer ID (CM LOGON Facility Designation) |
| Presentation Context Definition List | Not Used |
| Default Presentation Context Name | Not Used |
| Quality of Service | D-Start QOS { Routing Class: - CM LOGON Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Presentation Requirements | Not Used (Default value) |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not Used |
| Initial Assignment of Tokens | Not Used |
| Session-connection Identifier | Not Used |

Receiver side

- A-Associate Indication

| Mode | Absent |
| --- | --- |
| Application Context Name | CM ASE = {1 3 27 3 2} |
| Application Context Name List | Absent |
| Calling AP Title | CM Aircraft AP Title |
| Calling AE Qualifier | Absent |
| Calling AP Invocation-identifier | Absent |
| Calling AE Invocation-identifier | Absent |
| Called AP Title | Absent |
| Called AE Qualifier | Absent |
| Called AP Invocation-identifier | Absent |
| Called AE Invocation-identifier | Absent |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Absent |
| Authentication-value | SESE PDU |
| User Information | CMAircraftMessage APDU |
| Calling Presentation Address | Local Calling Presentation Address |
| Called Presentation Address | CM LOGON Facility Designation |
| Presentation Context Definition List | Absent |
| Default Presentation Context Name | Absent |
| Quality of Service | D-Start QOS { Routing Class: - CM LOGON Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Presentation Requirements | Absent |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Absent |
| Initial Assignment of Tokens | Absent |
| Session-connection Identifier | Absent |

- SE-Transfer Indication

| Security exchange identifier | atnEstablishSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnEstablish { atnCertificates: absent atnSignature { algorithmId : absent validity : timeField(ATNSecurityDateTime) value : ecdsa-Signature(ECDSA-Sig-Value) } } |
| Item identifier | atn-establish = {1 3 27 5 1 2} |
| Start flag | True |
| End flag | Not used |

- SSO-SignCheck

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| User Data | CM LOGON Message |
| Certificate path | Not used |
| Security item | atnSignature { algorithmId : absent validity : timeField(ATNSecurityDateTime) value : ecdsa-Signature(ECDSA-Sig-Value) } |

The Destination peer addresses the ground CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.2.2.2 a) ) from the ICAO Facility Designator present in the SA-START Request Calling Entity ID parameter. The Source peer addresses the airborne CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.2.2.2 a) ) from the ICAO Aircraft Identifier present in the SA-START Request Calling Entity ID parameter.

- SA-Start Indication

| Calling Entity ID | Airborne CM |
| --- | --- |
| Called Entity ID | Ground CM |

- D-Start Indication

| Calling Peer ID | CM Aircraft Address, from Calling AP Title. |
| --- | --- |
| Calling Sys-ID | Not Used (No Calling Sys-ID) |
| Calling Presentation Address | Not Used |
| DS-User Version Number | CM-ground-ASE version number = 2 |
| Security Requirements | Abstract value “Secured Dialogue Supporting Key Management” |
| Quality-of-Service | D-Start QOS { Routing Class: - CM LOGON Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| User Data | A-ASSOCIATE User Information parameter |

###### Dialogue establishment : Response-Confirmation phase

Mapping the expression of the need to an implementation

Receiver side

The expressed need : « The ground CMA entity forms a CM-logon response message Data2 including an indication that it has accepted the aircraft CMA entity’s request for a secure connection, the aircraft CMA entity’s identity U (i.e., U’s CM AE-title), the ground CMA entity’s identity V (i.e., V’s CM AE-title), the ground CMA entity’s random challenge RandV, and ground application entity identities and public keys for the applications that the aircraft entity supports along with indications of whether the public keys for these applications are shared by all application locations within the CM domain. It calculates the tag on its identity V, the aircraft CMA entity’s identity U, a sixteen bit unsigned counter value CountV,U which is initially 000116, Data2 and sU using HMAC under the CMA session key MacKeyU,V as: MAC (MacKeyU,V; V || U || CountV,U || Data2 || sU). The ground CMA entity sends the CM-logon response Data2 along with a certificate path from the aircraft’s State CA’s public key to its key agreement public key and the tag to the aircraft entity. »

- The translation of the need : « When receiving peer accepts the establishment of a Dialogue which supports key management, a security item needs to be sent back to the initiating peer, which permits the former to provide security related information to the latter.»

- The implementation of the need : «The D-START Response following the CM-LOGON Response, because it contains a acceptance for establishing a Dialogue supporting Key Management, triggers the computation of a new appendix (tag) on the session key (MacKeyU,V ), the airborne (U) and ground (V) Dialogue users, the sixteen bit unsigned counter value (CountV,U) of the exchanged information from the ground peer to the airborne peer, the D-START Response User Data, the certificate path from the aircraft’s State CA’s public key to its key agreement public key, and the initial appendix (sU). To do this, the S-ASO builds a SESE APDU containing a validity element (the random challenge (RandV), the appendix (tag), and the certificate path from the aircraft’s State CA’s public key to its key agreement public key, and an AARE APDU containing the D-START Response user data in user-information, the responder-acse-requirements set to « Authentication », and the SESE APDU in the responding-authentication-value. ». This AARE APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.2.2.

Initiator side

- The expressed need : « The aircraft CMA entity reconstructs V || U || CountV,U || Data2 || sU and checks the tag it received is valid using HMAC with SHA-1 under the session key MacKeyU,V. If this check is successful, the aircraft CMA entity accepts the ground CMA entity’s login response and concludes that the CM-logon security check has been successful.»

- The translation of the need : « When the initiating peer receives the security item in response, it first checks the validity of the associated information and, then, computes the security related data which it will use for subsequent exchanges with its peer.»

- The implementation of the need : « The incoming AARE APDU, because it specifies acceptance for authentication, triggers the check of the appendix (tag) using the session key (MacKeyU,V ), the airborne (U) and ground (V) Dialogue users, the counter of exchanges between the ground peer and the airborne peer (CountV,U), and the user information contained in the AARE APDU. The acceptance for authentication is specified, in the incoming AARE APDU, by the fact that the responder-acse-requirements is set to « Authentication » ; this triggers the check of the appendix (tag), and the random challenge (RandV), contained in the SESE APDU, itself contained in the responding-authentication-value field, together with the identity of the airborne (U) and ground (V) Dialogue Users, and the user-information field of the AARE APDU. »

Associated service activations

Receiver side

- D-Start Response

| Security Requirements | Abstract value “Secured Dialogue Supporting Key Management”. |
| --- | --- |
| Quality-of-Service | Routing Class: CM LOGON Class of Communication Priority: “flight regularity communications” RER: “Low” |
| Result | “accepted” |
| User Data | CMGroundMessage APDU (cmSecureLogonResponse) |

- SA-START Response

| Called Entity ID | Ground CM |
| --- | --- |
| Calling Entity ID | Airborne CM |
| User Data | CMGroundMessage APDU (cmSecureLogonResponse) |

- SSO-GetCertificatePath

| Entity ID | Ground CM |
| --- | --- |
| Key usage | “keyAgreement” |

The Entity ID addresses the ground CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.3.2.2 a) ) from the ICAO Facility Designator present in the SA-START Response Called Entity ID parameter.

- SSO-Sign

| Source peer | Ground CM |
| --- | --- |
| Destination Peer | Airborne CM |
| Appendix type | MAC-appendix |
| User Data | CMGroundMessage APDU (cmSecureLogonResponse) |

The Source peer addresses the ground CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.3.2.2 a) ) from the ICAO Facility Designator present in the SA-START Response Called Entity ID parameter. The Destination peer addresses the airborne CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.3.2.2 a) ) from the ICAO Aircraft Identifier present in the SA-START Response Calling Entity ID parameter.

- SE-Transfer Request

| Security exchange identifier | atnEstablishSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnEstablish { atnCertificates: ground CM Key Agreement Certificate atnSignature { algorithmId : absent validity : random value : hmac-Tag } } |
| Item identifier | AtnEstablish security exchange item |
| Start flag | False |
| End flag | absent |

- A-Associate Response

| Application Context Name | CM ASE = {1 3 27 3 2} |
| --- | --- |
| Responding AP Title | Not Used |
| Responding AE Qualifier | Not Used |
| Responding AP Invocation-identifier | Not Used |
| Responding AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not used |
| Authentication-value | SESE PDU |
| User Information | D-Start User data (CMGroundMessage APDU (cmSecureLogonResponse)) |
| Result | “accepted” |
| Diagnostic | Not used |
| Responding Presentation Address | Local Presentation address |
| Presentation Context Definition Result List | Not used |
| Default Presentation Context Result | Not used |
| Quality of Service | D-Start QOS { Routing Class: - CM LOGON Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Presentation Requirements | Not used |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not used |
| Initial Assignment of Tokens | Not used |
| Session-connection Identifier | Not used |

Initiator side

- A-Associate Confirmation

| Application Context Name | CM ASE = {1 3 27 3 2} |
| --- | --- |
| Responding AP Title | Not Used |
| Responding AE Qualifier | Not Used |
| Responding AP Invocation-identifier | Not Used |
| Responding AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not Used |
| Authentication-value | SESE PDU |
| User Information | D-Start User data (CMGroundMessage APDU (cmSecureLogonResponse)) |
| Result | “accepted” |
| Result Source |  |
| Diagnostic | Not Used |
| Responding Presentation Address | Ground CM Presentation address |
| Presentation Context Definition Result List | Not Used |
| Default Presentation Context Result | Not Used |
| Quality of Service | D-Start QOS { Routing Class: - CM LOGON Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Presentation Requirements | Not Used |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not Used |
| Initial Assignment of Tokens | Not Used |
| Session-connection Identifier | Not Used |

- SE-Transfer Indication

| Security exchange identifier | atnEstablishSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnEstablish { atnCertificates: ground CM Key Agreement Certificate atnSignature { algorithmId : absent validity : random value : hmac-Tag } } |
| Item identifier | AtnEstablish security exchange item |
| Start flag | False |
| End flag | absent |

- SSO-SignCheck

| Local peer | Ground CM |
| --- | --- |
| Remote Peer | Airborne CM |
| User Data | CMGroundMessage APDU (cmSecureLogonResponse) |
| Certificate path | Ground CM key agreement key certificate path |
| Security item | atnEstablish { atnCertificates: ground CM Key Agreement Certificate atnSignature { algorithmId : absent validity : random value : hmac-Tag } } |

- SA-Start Confirmation

| Called Entity ID | Ground CM |
| --- | --- |
| Calling Entity ID | Airborne CM |

- D-Start Confirmation

| DS-User Version Number | CM-air-ASE version number = 2 |
| --- | --- |
| Security Requirements | Abstract value “Secured Dialogue Supporting Key Management” |
| Quality-of-Service | D-Start QOS { Routing Class: - CM LOGON Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Result | “accepted” |
| Reject Source | Not used |
| User Data | A-ASSOCIATE User Information parameter |

###### Ground initiated data transfer phase

Mapping the expression of the need to an implementation

Sender side

The expressed need : « Subsequent to CM-logon when the ground CMA entity and the aircraft CMA entity wish to exchange a data item Data3, such as a request for additional application entity public keys, the sender, say V, authenticates the data item by MACing it using the CMA session key MacKeyU,V established during CM-logon as follows: MAC ( MacKeyU,V; V || U || CountV,U || Data3 ); V is the sender’s identity and U is the recipient’s identity. Inclusion of U and V enables detection of attacks in which an adversary bounces a data item sent by the aircraft or ground back to its sender. CountV,U is a 16-bit counter indicating the number of data items sent over the RF channel by its sender secured under MacKeyU,V during this CMA session. Inclusion of CountV,U enables detection of attacks in which an adversary replays a previously sent data item. The data item Data3 is then sent over the RF channel along with the MAC value calculated on it. This ensures the authenticity of Data3. »

The translation of the need : « When a request for sending Data over a secured Dialogue is made, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

- The implementation of the need : « The D-DATA Request over a secured Dialogue, triggers the computation of an appendix on the D-DATA Request User data, the senders (V) and the receiving (U) Dialogue users, on a counter (CountV,U) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyU,V) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-DATA Request user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-DATA Request user data, and the appendix.» This SESE APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.3.2.1.

Receiver side

- The implementation of the need : « The incoming SESE APDU contained on the Presentation user data parameter, because it arrives on a secured Dialogue, triggers the check of the appendix contained in the SESE APDU, using the identity of the peer Dialogue User (V), the identity of the local Dialogue User (U), the counter (CountV,U) of the User data sent from the sending to the receiving peers, and the unprotected part of the SESE APDU (i.e. original D-DATA request user data). » This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.3.2.2.

Associated service activations

Sender side

- D-Data Request

| User Data | CMGroundMessage APDU (cmContactRequest) |
| --- | --- |

- SA-Send Request

| Local Entity ID | Ground CM |
| --- | --- |
| Remote Entity ID | Airborne CM |
| User Data | CMGroundMessage APDU (cmContactRequest) |

- SSO-ProtectSign

| Source peer | Ground CM |
| --- | --- |
| Destination Peer | Airborne CM |
| User Data | CMGroundMessage APDU (cmContactRequest) |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : CMGroundMessage APDU (cmContactRequest) appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

Receiver side

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : CMGroundMessage APDU (cmContactRequest) appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Ground CM |
| --- | --- |
| Destination Peer | Airborne CM |
| Security item | atnProtectSign { unprotected : CMGroundMessage APDU (cmContactRequest) appendix { algorithmId : absent validity : absent value : hmac-Tag } } |

- SA-Send Indication

| Local Entity ID | Airborne CM |
| --- | --- |
| Remote Entity ID | Ground CM |
| User Data | CMGroundMessage APDU (cmContactRequest) |

- D-Data Indication

| User Data | CMGroundMessage APDU (cmContactRequest) |
| --- | --- |

###### Air initiated data transfer phase

Mapping the expression of the need to an implementation

Sender side

The expressed need : « Subsequent to CM-logon when the ground CMA entity and the aircraft CMA entity wish to exchange a data item Data3, such as a request for additional application entity public keys, the sender, say V, authenticates the data item by MACing it using the CMA session key MacKeyU,V established during CM-logon as follows: MAC ( MacKeyU,V; V || U || CountV,U || Data3 ); V is the sender’s identity and U is the recipient’s identity. Inclusion of U and V enables detection of attacks in which an adversary bounces a data item sent by the aircraft or ground back to its sender. CountV,U is a 16-bit counter indicating the number of data items sent over the RF channel by its sender secured under MacKeyU,V during this CMA session. Inclusion of CountV,U enables detection of attacks in which an adversary replays a previously sent data item. The data item Data3 is then sent over the RF channel along with the MAC value calculated on it. This ensures the authenticity of Data3. »

- The translation of the need : « When a request for sending Data over a secured Dialogue is made, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

- The implementation of the need : « The D-DATA Request over a secured Dialogue, triggers the computation of an appendix on the D-DATA Request User data, the senders (V) and the receiving (U) Dialogue users, on a counter (CountV,U) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyU,V) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-DATA Request user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-DATA Request user data, and the appendix.» This SESE APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.4.2.1.

Receiver side

The implementation of the need : « The incoming SESE APDU contained on the Presentation user data parameter, because it arrives on a secured Dialogue, triggers the check of the appendix contained in the SESE APDU, using the identity of the peer Dialogue User (V), the identity of the local Dialogue User (U), the counter (CountV,U) of the User data sent from the sending to the receiving peers, and the unprotected part of the SESE APDU (i.e. original D-DATA request user data). » This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.4.2.2.

Associated service activations

Sender side

- D-Data Request

| User Data | CMAircraftMessage APDU (cmContactResponse) |
| --- | --- |

- SA-Send Request

| Local Entity ID | Airborne CM |
| --- | --- |
| Remote Entity ID | Ground CM |
| User Data | CMAircraftMessage APDU (cmContactResponse) |

- SSO-ProtectSign

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| User Data | CMAircraftMessage APDU (cmContactResponse) |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : CMAircraftMessage APDU (cmContactResponse) appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

Receiver side

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : CMAircraftMessage APDU (cmContactResponse) appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| Security item | atnProtectSign { unprotected : CMAircraftMessage APDU (cmContactResponse) appendix { algorithmId : absent validity : absent value : hmac-Tag } } |

- SA-Send Indication

| Local Entity ID | Ground CM |
| --- | --- |
| Remote Entity ID | Airborne CM |
| User Data | CMAircraftMessage APDU (cmContactResponse) |

- D-Data Indication

| User Data | CMAircraftMessage APDU (cmContactResponse) |
| --- | --- |

###### Dialogue end : Request-Indication phase

Mapping the expression of the need to an implementation

Initiating side

No explicit need was expressed for Dialogue ending phase. The choice was to consider this phase as equivalent to data transfer phase, with respect to security consideration, and to transfer security related information together with user data using ACSE Release service.

- « When a request for ending a secured Dialogue is made, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

The implementation of the need : « The D-END Request over a secured Dialogue, triggers the computation of an appendix on the D-END Request User data, the senders (V) and the receiving (U) Dialogue users, on a counter (CountV,U) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyU,V) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-END Request user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-END Request user data, and the appendix.» This SESE APDU is sent using the A-RELEASE Request service. This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.5.2.1.

Receiver side

- The implementation of the need : « The incoming SESE APDU contained on the A-RELEASE Indication user information parameter, because it arrives on a secured Dialogue, triggers the check of the appendix contained in the SESE APDU, using the identity of the peer Dialogue User (V), the identity of the local Dialogue User (U), the counter (CountV,U) of the User data sent from the sending to the receiving peers, and the unprotected part of the SESE APDU (i.e. original D-END request user data). » This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.1.1.5.2.2.

Associated service activations

Initiating side

- D-End Request

| User Data | Not used |
| --- | --- |

- SA-Send Request

| Local Entity ID | Ground CM |
| --- | --- |
| Remote Entity ID | Airborne CM |
| User Data | Not Used |

- SSO-ProtectSign

| Local peer | Airborne CM |
| --- | --- |
| Remote Peer | Ground CM |
| User Data | Not Used |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- A-Release Request

| Reason | “normal” |
| --- | --- |
| User Information | SESE PDU |

Receiver side

- A-Release Indication

| Reason | “normal” |
| --- | --- |
| User Data | SESE PDU |

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Ground CM |
| --- | --- |
| Destination Peer | Airborne CM |
| Security item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |

- SA-Send Indication

| Local Entity ID | Airborne CM |
| --- | --- |
| Remote Entity ID | Ground CM |
| User Data | Not Used |

- D-End Indication

| User Data | Not Used |
| --- | --- |

###### Dialogue end : Response-Confirmation phase

Mapping the expression of the need to an implementation

Receiver side

No explicit need was expressed for Dialogue ending phase. The choice was to consider this phase as equivalent to data transfer phase, with respect to security consideration, and to transfer security related information together with user data using ACSE Release service.

- « When a response for ending a secured Dialogue is made, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

The implementation of the need : « The D-END Response over a secured Dialogue, triggers the computation of an appendix on the D-END Response User data, the senders (V) and the receiving (U) Dialogue users, on a counter (CountV,U) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyU,V) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-END Response user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-END Response user data, and the appendix.» This SESE APDU is sent using the A-RELEASE Response service. This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.1.1.6.2.1.

Initiating side

- The implementation of the need : « The incoming SESE APDU contained on the A-RELEASE Confirmation user information parameter, because it arrives on a secured Dialogue, triggers the check of the appendix contained in the SESE APDU, using the identity of the peer Dialogue User (V), the identity of the local Dialogue User (U), the counter (CountV,U) of the User data sent from the sending to the receiving peer, and the unprotected part of the SESE APDU (i.e. original D-END Response user data). » This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.6.2.2.

Associated service activations

Receiving side

- D-End Response

| Result | “affirmative” |
| --- | --- |
| User Data | Not Used |

- SA-Send Request

| Local Entity ID | Airborne CM |
| --- | --- |
| Remote Entity ID | Ground CM |
| User Data | Not used |

- SSO-ProtectSign

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| User Data | Not used |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- A-Release Response

| Reason | “normal” |
| --- | --- |
| User Data | SESE PDU |
| Result | « affirmative » |

Initiating side

- A-Release Confirmation

| Reason | “normal” |
| --- | --- |
| User Data | SESE PDU |
| Result | « affirmative » |

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| Security item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| CheckResult |  |

- SA-Send Indication

| Local Entity ID | Ground CM |
| --- | --- |
| Remote Entity ID | Airborne CM |
| User Data | Not Used |

- D-End Confirmation

| Result | “affirmative” |
| --- | --- |
| User Data | Not Used |

The full scenario described above is summarized in the following picture:

| Dialogue | SA-Service | SESE | ACSE | Présentation | Session | Transport | Session | Présentation | ACSE | SESE | SA-Service | Dialogue |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| D-Start req | SA-Start req | SE-Transfer req | A-Associate req | P-Connect req | S-Connect req | T-Connect req | T-Connect ind |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T-Connect cnf | T-Connect rsp |  |  |  |  |  |  |
|  |  |  |  |  |  | T-Data req | T-Data ind | S-Connect ind | P-Connect ind | A-Associate ind | SE-Transfer ind | SA-Start ind | D-Start ind |
| D-Start cnf | SA-Start cnf | SE-Transfer ind | A-Associate cnf | P-Connect cnf | S-Connect cnf | T-Data ind | T-Data req | S-Connect rsp | P-Connect rsp | A-Associate rsp | SE-Transfer req | SA-Start rsp | D-Start rsp |
| D-Data ind | SA-Send ind | SE-Transfer ind |  | P-Data ind | S-Data ind | T-Data ind | T-Data req | S-Data req | P-Data req |  | SE-Transfer req | SA-Send req | D-Data req |
| D-Data req | SA-Send req | SE-Transfer req |  | P-Data req | S-Data req | T-Data req | T-Data ind | S-Data ind | P-Data ind |  | SE-Transfer ind | SA-Send ind | D-Data ind |
| D-End ind | SA-Send ind | SE-Transfer ind | A-Release ind | P-Data ind | S-Data ind | T-Data ind | T-Data req | S-Data req | P-Data req | A-Release req | SE-Transfer req | SA-Send req | D-End req |
| D-End rsp | SA-Send req | SE-Transfer req |  | P-Data req | S-Data req | T-Data req | T-Data ind | S-Data ind | P-Data ind |  | SE-Transfer ind | SA-Send ind | D-End cnf |
|  |  |  |  | P-Pabort ind | S-Pabort ind | T-Disconnect ind | T-Disconnect req | S-Uabort req | P-Uabort req |  |  |  |  |

##### Ground initiated Air-Ground Communications

###### Dialogue establishment: Request-Indication phase

Mapping the expression of the need to upper layers functionality

Initiator side

The expressed need : « The ground CMA entity forms a CM-update request message Data1 consisting of some update data including an indication that it is requesting a secure connection, its identity V, the aircraft CMA entity’s identity U, a time field TimeV, and ground application entities’ identities and public keys for the applications that the aircraft supports along with indications for each application type of whether the public key for the application type is shared by all locations of that application type within the CM domain (e.g., all ADS applications use the same public key). It signs Data1 using ECDSA under its private signing key dsig,V: sV = Sign (dsig,V; Data1). The ground CMA entity sends the CM-update request message along with sV and certificate paths from the aircraft’s State CA’s public key to the ground CMA entity’s signing and key agreement public keys to the aircraft CMA. The ground CMA entity retains its signature sV for use later.»

- The translation of the need : « When a request for establishing a Dialogue specifies that this Dialogue should support key management, there need to be a security item sent from the initiating peer to the receiving peer, which permits the former to provide security related information to the latter.»

- The implementation of the need : « The D-START Request following the CM-UPDATE Request, because it contains a request for establishing a Dialogue supporting Key Management, triggers the computation of an appendix (sV) on the D-START Request User data, the airborne (U) and ground (V) Dialogue users and a locally computed time field (TimeV). The Dialogue control function then transmits to its peer : the D-START Request User Data, the ground Dialogue user (V), the time field (TimeV) and the appendix (sV). To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing a validity element (the time field (TimeV), the certificate paths from the aircraft’s State CA’s public key to the ground CMA entity’s signing and key agreement public keys, and the appendix (sV)), and an AARQ APDU containing the identity of the Dialogue User (V) in calling-AP-title and calling-AE-qualifier, the D-START Request user data in user-information, the sender-acse-requirements set to « Authentication », and the SESE APDU in the calling-authentication-value. » This AARQ APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.1.2.1.2.1.

Receiver side

- The expressed need : « The aircraft CMA entity receives the CM-update request message along with sV and sees that the message requests a secure connection. It recovers from the message the ground CMA entity’s identity V, its identity U, the time field TimeV, the application entities’ identities and public keys, and the certificate paths. It checks the certificate paths using its copy of its State CA’s public key. It retrieves the ground CMA entity’s public signing key Qsig,V from its signature key certificate. It verifies the received signature sV is a valid ECDSA signature on Data1 using Qsig,V, and it verifies that U is correct and TimeV corresponds with the current time. (Note that the aircraft CMA entity may optionally cache sV and use it to check that initial CM-update requests are not replayed even during a single time period.) If all these checks are successful, the aircraft CMA entity accepts the ground CMA entity’s update request.»

- The translation of the need : « When the receiving peer receives the security item containing security information, it first checks the validity of the associated information and then computes the security related data which it will use for subsequent exchanges with its peer.»

- The implementation of the need : « The incoming AARQ APDU, because it specifies need for authentication, triggers the check of the appendix (sV), using the identity of the peer Dialogue User (V), the identity of the local Dialogue User (U), the user information, the certificate paths from the aircraft’s State CA’s public key to the ground CMA entity’s signing and key agreement public keys, and the time field (TimeV) contained in the AARQ APDU. The need for authentication is specified, in the incoming AARQ APDU, by the fact that the sender-acse-requirements is set to « Authentication » ; this triggers the check of the appendix (sV), and the time field (TimeV) contained in the SESE APDU, itself contained in the calling-authentication-value field, together with the identity of the peer Dialogue User (V) contained in the calling-AP-title and calling-AE-qualifier fields, the identity of the local Dialogue User (U) and the user-information field of the AARQ APDU. »

Associated service activations

Initiator side

Initiator side

The CM-UPDATE Request leads, according to «2.1.5.3.3.7 CM-update and CM-server-facility-update service request», to the activation of the D-START Request service with the following parameters:

- D-START Request

| Called Peer ID | CM Update Aircraft Address |
| --- | --- |
| Called Sys-ID |  |
| Called Presentation Address |  |
| Calling Peer ID | CM Update Facility Designation |
| Calling Sys-ID |  |
| Calling Presentation Address |  |
| DS-User Version Number | CM-air-ASE version number = 2 |
| Security Requirements | Abstract value “Secured Dialogue Supporting Key Management”. |
| Quality-of-Service | Routing Class: CM UPDATE Class of Communication Priority: “flight regularity communications” RER: “Low” |
| User Data | CMGroundMessage APDU (cmSecureUpdate) |

- SA-START Request

| Calling Entity ID | Ground CM |
| --- | --- |
| Called Entity ID | Airborne CM |
| User Data | CMGroundMessage APDU (cmSecureUpdate) |

- SSO-Sign

| Source peer | Ground CM |
| --- | --- |
| Destination Peer | Airborne CM |
| Appendix type | Signature-appendix |
| User data | CMGroundMessage APDU (cmSecureUpdate) |

The Destination peer addresses the ground CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.2.2.2 a) ) from the ICAO Facility Designator present in the SA-START Request Calling Entity ID parameter. The Source peer addresses the airborne CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.2.2.2 a) ) from the ICAO Aircraft Identifier present in the SA-START Request Calling Entity ID parameter.

- SE-Transfer Request

| Security exchange identifier | atnEstablishSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnEstablish { atnCertificates { ground CM signature key certificate path key agreement key certificate path } atnSignature { algorithmId : absent validity : timeField(ATNSecurityDateTime) value : ecdsa-Signature(ECDSA-Sig-Value) } } |
| Item identifier | atn-establish = {1 3 27 5 1 2} |
| Start flag | True |
| End flag | Not used |

- A-Associate Request

| Mode | Not Used (default value) |
| --- | --- |
| Application Context Name | CM ASE = {1 3 27 3 2} |
| Application Context Name List | Not Used |
| Calling AP Title | Retrieved from Calling Peer ID (CM Update Facility Designation), of from local knowledge. |
| Calling AE Qualifier | Not Used (No Calling Sys-ID) |
| Calling AP Invocation-identifier | Not Used |
| Calling AE Invocation-identifier | Not Used |
| Called AP Title | Not Used |
| Called AE Qualifier | Not Used |
| Called AP Invocation-identifier | Not Used |
| Called AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not Used |
| Authentication-value | SESE PDU |
| User Information | D-Start User data (CMGroundMessage APDU (cmSecureUpdate)) |
| Calling Presentation Address | Local Calling Presentation Address |
| Called Presentation Address | Derived from Called Peer ID (CM Update Aircraft Address) |
| Presentation Context Definition List | Not Used |
| Default Presentation Context Name | Not Used |
| Quality of Service | D-Start QOS { Routing Class: - CM UPDATE Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Presentation Requirements | Not Used (Default value) |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not Used |
| Initial Assignment of Tokens | Not Used |
| Session-connection Identifier | Not Used |

Receiver side

###### Dialogue establishment : Response-Confirmation phase

Mapping the expression of the need to an implementation

Receiver side

- The expressed need : « The aircraft CMA entity forms an initial CM-update response message Data2 (i.e., ACSE AARE APDU) including an indication that it has accepted the ground CMA entity’s request for a secure connection, the ground CMA entity’s identity V (i.e., V’s CM AE-title), the aircraft CMA entity’s identity U (i.e., U’s CM AE-title), and the aircraft CMA entity’s random challenge RandU. It calculates the tag on its identity U, the ground CMA entity’s identity V, a sixteen bit unsigned counter value CountU,V which is initially 000116, Data2 and sV using HMAC under the CMA session key MacKeyU,V as: MAC (MacKeyU,V; U || V || CountU,V || Data2 || sV). The aircraft CMA entity sends the initial CM-update response Data2 along with the tag to the ground CMA entity.»

- The translation of the need : « When receiving peer accepts the establishment of a Dialogue which supports key management, a security item needs to be sent back to the initiating peer, which permits the former to provide security related information to the latter.»

The implementation of the need : «The D-START Response negative following the CM-UPDATE Indication, because it contains a refusal for establishing a Dialogue supporting Key Management, triggers the computation of a new appendix (tag) on the session key (MacKeyU,V ), the airborne (U) and ground (V) Dialogue users, a sixteen bit unsigned counter value (CountU,V), and the initial appendix (sV). To do this, the S-ASO builds a SESE APDU containing a validity element (the random challenge (RandU), the appendix (tag), and an AARE APDU containing the responder-acse-requirements set to « Authentication », and the SESE APDU in the responding-authentication-value. ». This AARE APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.1.2.2.2.1.

Initiator side

- The expressed need : « The ground CMA entity reconstructs U || V || CountU,V || Data2 || sV and checks the tag it received is valid using HMAC with SHA-1 under the session key MacKeyU,V. If this check is successful, the ground CMA entity accepts the aircraft CMA entity’s update response and concludes that the initial CM-update security check has been successful.»

- The translation of the need : « When the initiating peer receives the security item in response, it first checks the validity of the associated information and, then, computes the security related data which it will use for subsequent exchanges with its peer.»

- The implementation of the need : « The incoming AARE APDU, because it specifies acceptance for authentication, triggers the check of the appendix (tag) using the session key (MacKeyU,V ), the airborne (U) and ground (V) Dialogue users, the counter of exchanges between the airborne peer and the ground peer (CountU,V), and the user information contained in the AARE APDU. The acceptance for authentication is specified, in the incoming AARE APDU, by the fact that the responder-acse-requirements is set to « Authentication » ; this triggers the check of the appendix (tag), and the random challenge (RandU), contained in the SESE APDU, itself contained in the responding-authentication-value field, together with the identity of the airborne (U) and ground (V) Dialogue Users, and the user-information field of the AARE APDU. »

Associated service activations

Receiver side

- D-Start Response

| Security Requirements | Abstract value “Secured Dialogue Supporting Key Management”. |
| --- | --- |
| Quality-of-Service | Routing Class: CM UPDATE Class of Communication Priority: “flight regularity communications” RER: “Low” |
| Result | “rejected(permanent)” |
| User Data | Not used |

- SA-START Response

| Called Entity ID | Airborne CM |
| --- | --- |
| Calling Entity ID | Ground CM |
| User Data | Not used |

- SSO-GetCertificatePath

| Entity ID | Ground CM |
| --- | --- |
| Key usage | “keyAgreement” |

The Entity ID addresses the ground CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.3.2.2 a) ) from the ICAO Facility Designator present in the SA-START Response Called Entity ID parameter.

- SSO-Sign

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| Appendix type | MAC-appendix |
| User Data | Not Used |

The Source peer addresses the ground CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.3.2.2 a) ) from the ICAO Facility Designator present in the SA-START Response Called Entity ID parameter. The Destination peer addresses the airborne CM and is under the form of an AP-Title or PSAP derived (4.3.3.3.3.2.2 a) ) from the ICAO Aircraft Identifier present in the SA-START Response Calling Entity ID parameter.

- SE-Transfer Request

| Security exchange identifier | atnEstablishSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnEstablish { atnCertificates: absent atnSignature { algorithmId : absent validity : random value : hmac-Tag } } |
| Item identifier | AtnEstablish security exchange item |
| Start flag | False |
| End flag | absent |

- A-Associate Response

| Application Context Name | CM ASE = {1 3 27 3 2} |
| --- | --- |
| Responding AP Title | Not Used |
| Responding AE Qualifier | Not Used |
| Responding AP Invocation-identifier | Not Used |
| Responding AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not used |
| Authentication-value | SESE PDU |
| User Information | Not Used |
| Result | “rejected(permanent)” |
| Diagnostic | Not used |
| Responding Presentation Address | Local Presentation address |
| Presentation Context Definition Result List | Not used |
| Default Presentation Context Result | Not used |
| Quality of Service | D-Start QOS { Routing Class: - CM UPDATE Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Presentation Requirements | Not used |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not used |
| Initial Assignment of Tokens | Not used |
| Session-connection Identifier | Not used |

- SA-END Request

| Local Entity ID | Airborne CM |
| --- | --- |
| Remote Entity ID | Ground CM |
| Severity | “low” |

Initiator side

- A-Associate Confirmation

| Application Context Name | CM ASE = {1 3 27 3 2} |
| --- | --- |
| Responding AP Title | Not Used |
| Responding AE Qualifier | Not Used |
| Responding AP Invocation-identifier | Not Used |
| Responding AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not Used |
| Authentication-value | SESE PDU |
| User Information | Not Used |
| Result | “rejected(permanent)” |
| Result Source | ACSE Service User |
| Diagnostic | Not Used |
| Responding Presentation Address | Ground CM Presentation address |
| Presentation Context Definition Result List | Not Used |
| Default Presentation Context Result | Not Used |
| Quality of Service | D-Start QOS { Routing Class: - CM UPDATE Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Presentation Requirements | Not Used |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not Used |
| Initial Assignment of Tokens | Not Used |
| Session-connection Identifier | Not Used |

- SE-Transfer Indication

| Security exchange identifier | atnEstablishSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnEstablish { atnCertificates: absent atnSignature { algorithmId : absent validity : random value : hmac-Tag } } |
| Item identifier | AtnEstablish security exchange item |
| Start flag | False |
| End flag | absent |

- SSO-SignCheck

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| User Data | Not used |
| Certificate path | Not used |
| Security item | atnEstablish { atnCertificates: absent atnSignature { algorithmId : absent validity : random value : hmac-Tag } } |

- SA-Start Confirmation

| Called Entity ID | Airborne CM |
| --- | --- |
| Calling Entity ID | Ground CM |

- SA-END Request

| Local Entity ID | Ground CM |
| --- | --- |
| Remote Entity ID | Airborne CM |
| Severity | “low” |

- D-Start Confirmation

| DS-User Version Number | CM-air-ASE version number = 2 |
| --- | --- |
| Security Requirements | Abstract value “Secured Dialogue Supporting Key Management” |
| Quality-of-Service | D-Start QOS { Routing Class: - CM UPDATE Class Of Communication Service, or - “ATSC: No Traffic Type Policy Preference”, Priority: “flight regularity communications”, RER: “low” } |
| Result | “rejected(permanent)” |
| Reject Source | DS User |
| User Data | Not used |

#### Secured Dialogue

##### Dialogue establishment: Request-Indication phase

###### Mapping the expression of the need to upper layers functionality

Initiator side

The expressed need : « When the ground application entity and the aircraft application entity wish to exchange a data item Data4, such as a D-Start or D-Data item, the sender, say W, authenticates the data item by MACing it using the application session key MacKeyY,W as follows: MAC (MacKeyY,W; W || Y || CountW,Y || Data4 ) W is the sender’s identity and Y is the intended recipient’s identity. The data item Data4 is then sent over the RF channel along with the MAC value calculated on it. This ensures the authenticity of Data4. »

The translation of the need : « When a request for establishing a Dialogue specifies that this Dialogue should be secured without support for key management, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

- The implementation of the need : « The D-START Request over a secured Dialogue, triggers the computation of an appendix on the D-STARTRequest User data, the senders (W) and the receiving (Y) Dialogue users, on a counter (CountW,Y) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyY,W) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-START Request user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-START Request user data, and the appendix, and an AARQ APDU containing the identity of the Dialogue User (W) in calling-AP-title and calling-AE-qualifier, the SESE APDU in the calling-authentication-value, the sender-acse-requirements set to « Authentication ». » This AARQ APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.2.1.2.1.

Receiver side

The implementation of the need : « The incoming The incoming AARQ APDU, because it specifies need for authentication, triggers the check of the appendix contained in the SESE APDU, using the identity of the peer Dialogue User (W), the identity of the local Dialogue User (Y), the counter (CountW,Y) of the User data sent from the sending to the receiving peers, and the unprotected part of the SESE APDU (i.e. original D-START request user data). » This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.2.1.2.2.

###### Associated service activations

Initiator side

- D-START Request

| Called Peer ID | Aircraft address parameter value as specified in contract request |
| --- | --- |
| Called Sys-ID |  |
| Called Presentation Address |  |
| Calling Peer ID | ICAO facility designation parameter value as specified in contract request |
| Calling Sys-ID |  |
| Calling Presentation Address |  |
| DS-User Version Number | Not used |
| Security Requirements | Abstract value “Secured Dialogue”. |
| Quality-of-Service | Routing Class: Air Traffic Service Communication ATSC Priority: “high prirority flight safety message” RER: “Low” |
| User Data | aDS-periodic-contract-PDU |

- SA-Send Request

| Local Entity ID | Ground ADS |
| --- | --- |
| Remote Entity ID | Airborne ADS |
| User Data | aDS-periodic-contract-PDU |

- SSO-ProtectSign

| Source peer | Ground ADS |
| --- | --- |
| Destination Peer | Airborne ADS |
| User Data | aDS-periodic-contract-PDU |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : aDS-periodic-contract-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- A-Associate Request

| Mode | Not Used (default value) |
| --- | --- |
| Application Context Name | {1 3 27 3 0} |
| Application Context Name List | Not Used |
| Calling AP Title | Retrieved from Calling Peer ID (ICAO facility designation parameter value as specified in contract request), of from local knowledge. |
| Calling AE Qualifier | Not Used (No Calling Sys-ID) |
| Calling AP Invocation-identifier | Not Used |
| Calling AE Invocation-identifier | Not Used |
| Called AP Title | Not Used |
| Called AE Qualifier | Not Used |
| Called AP Invocation-identifier | Not Used |
| Called AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not Used |
| Authentication-value | SESE PDU |
| User Information | D-Start User data (aDS-periodic-contract-PDU) |
| Calling Presentation Address | Local Calling Presentation Address |
| Called Presentation Address | Derived from Called Peer ID (Aircraft address parameter value as specified in contract request) |
| Presentation Context Definition List | Not Used |
| Default Presentation Context Name | Not Used |
| Quality of Service | D-Start QOS { Routing Class: Air Traffic Service Communication ATSC, Priority: “high prirority flight safety message”, RER: “low” } |
| Presentation Requirements | Not Used (Default value) |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not Used |
| Initial Assignment of Tokens | Not Used |
| Session-connection Identifier | Not Used |

Receiver side

- A-Associate Indication

| Mode | Absent |
| --- | --- |
| Application Context Name | {1 3 27 3 0} |
| Application Context Name List | Absent |
| Calling AP Title | ADS Aircraft AP Title |
| Calling AE Qualifier | Absent |
| Calling AP Invocation-identifier | Absent |
| Calling AE Invocation-identifier | Absent |
| Called AP Title | Absent |
| Called AE Qualifier | Absent |
| Called AP Invocation-identifier | Absent |
| Called AE Invocation-identifier | Absent |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Absent |
| Authentication-value | SESE PDU |
| User Information | aDS-periodic-contract-PDU |
| Calling Presentation Address | Airborne Calling Presentation Address |
| Called Presentation Address | CM LOGON Facility Designation |
| Presentation Context Definition List | Absent |
| Default Presentation Context Name | Absent |
| Quality of Service | D-Start QOS { Routing Class: Air Traffic Service Communication ATSC, Priority: “high prirority flight safety message”, RER: “low” } |
| Presentation Requirements | Absent |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Absent |
| Initial Assignment of Tokens | Absent |
| Session-connection Identifier | Absent |

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : aDS-periodic-contract-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Ground CM |
| --- | --- |
| Destination Peer | Airborne CM |
| Security item | atnProtectSign { unprotected : aDS-periodic-contract-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |

- SA-Send Indication

| Local Entity ID | Airborne CM |
| --- | --- |
| Remote Entity ID | Ground CM |
| User Data | aDS-periodic-contract-PDU |

- D-Start Indication

| Calling Peer ID | ADS Aircraft Address, from Calling AP Title. |
| --- | --- |
| Calling Sys-ID | Not Used (No Calling Sys-ID) |
| Calling Presentation Address | Not Used |
| DS-User Version Number | Not Used |
| Security Requirements | Abstract value “Secured Dialogue” |
| Quality-of-Service | Routing Class: Air Traffic Service Communication ATSC Priority: “high prirority flight safety message” RER: “Low” |
| User Data | aDS-periodic-contract-PDU |

##### Dialogue establishment : Response-Confirmation phase

###### Mapping the expression of the need to an implementation

Receiver side

The expressed need : « When the ground application entity and the aircraft application entity wish to exchange a data item Data4, such as a D-Start or D-Data item, the sender, say W, authenticates the data item by MACing it using the application session key MacKeyY,W as follows: MAC (MacKeyY,W; W || Y || CountW,Y || Data4 ) W is the sender’s identity and Y is the intended recipient’s identity. The data item Data4 is then sent over the RF channel along with the MAC value calculated on it. This ensures the authenticity of Data4. »

The translation of the need : « When a the receiving peer accepts the establishment of a secured Dialogue, a security item needs to be appended to the Dialogue User data, which permits the initiating peer to check the integrity of the received data and the origin of the peer. »

- The implementation of the need : «The D-START Response following the ADS-periodic-report, because it contains an acceptance for establishing a secured Dialogue, triggers the computation of a new appendix (tag) on the D-START Response User data, the sender (Y) and the receiving (W) Dialogue users, on a counter (CountY,W) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyY,W) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-START Response user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-START Response user data, and the appendix, and an AARE APDU containing the SESE APDU in the calling-authentication-value, the sender-acse-requirements set to « Authentication ». » This AARE APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.2.2.2.1.

Initiator side

###### Associated service activations

Receiver side

- D-Start Response

| Security Requirements | Abstract value “Secured Dialogue”. |
| --- | --- |
| Quality-of-Service | Not Used |
| Result | “accepted” |
| User Data | aDS-periodic-report-PDU |

- SA-Send Request

| Local Entity ID | Airborne ADS |
| --- | --- |
| Remote Entity ID | Ground ADS |
| User Data | aDS-periodic-report-PDU |

- SSO-ProtectSign

| Source peer | Airborne ADS |
| --- | --- |
| Destination Peer | Ground ADS |
| User Data | aDS-periodic-report-PDU |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : aDS-periodic-report-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- A-Associate Response

| Application Context Name | {1 3 27 3 0} |
| --- | --- |
| Responding AP Title | Not Used |
| Responding AE Qualifier | Not Used |
| Responding AP Invocation-identifier | Not Used |
| Responding AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not used |
| Authentication-value | SESE PDU |
| User Information | Not used |
| Result | “accepted” |
| Diagnostic | Not used |
| Responding Presentation Address | Local Presentation address |
| Presentation Context Definition Result List | Not used |
| Default Presentation Context Result | Not used |
| Quality of Service | D-Start QOS { Routing Class: Air Traffic Service Communication ATSC, Priority: “high prirority flight safety message”, RER: “low” } |
| Presentation Requirements | Not used |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not used |
| Initial Assignment of Tokens | Not used |
| Session-connection Identifier | Not used |

Initiator side

- A-Associate Confirmation

| Application Context Name | {1 3 27 3 0} |
| --- | --- |
| Responding AP Title | Not Used |
| Responding AE Qualifier | Not Used |
| Responding AP Invocation-identifier | Not Used |
| Responding AE Invocation-identifier | Not Used |
| ACSE Requirements | “Authentication” |
| Authentication-mechanism Name | Not Used |
| Authentication-value | SESE PDU |
| User Information | Not Used |
| Result | “accepted” |
| Result Source |  |
| Diagnostic | Not Used |
| Responding Presentation Address | Ground ADS Presentation address |
| Presentation Context Definition Result List | Not Used |
| Default Presentation Context Result | Not Used |
| Quality of Service | D-Start QOS { Routing Class: Air Traffic Service Communication ATSC, Priority: “high prirority flight safety message”, RER: “low” } |
| Presentation Requirements | Not Used |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not Used |
| Initial Assignment of Tokens | Not Used |
| Session-connection Identifier | Not Used |

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : aDS-periodic-report-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| Security item | atnProtectSign { unprotected : aDS-periodic-report-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |

- SA-Send Indication

| Local Entity ID | Ground CM |
| --- | --- |
| Remote Entity ID | Airborne CM |
| User Data | aDS-periodic-report-PDU |

- D-Start Confirmation

| DS-User Version Number | Absent |
| --- | --- |
| Security Requirements | Abstract value “Secured Dialogue Supporting Key Management” |
| Quality-of-Service | D-Start QOS { Routing Class: Air Traffic Service Communication ATSC, Priority: “high prirority flight safety message”, RER: “low” } |
| Result | “accepted” |
| Reject Source | Not used |
| User Data | aDS-periodic-report-PDU |

##### Air initiated data transfer phase

###### Mapping the expression of the need to an implementation

Sender side

- The expressed need : « When the ground application entity and the aircraft application entity wish to exchange a data item Data4, such as a D-Start or D-Data item, the sender, say W, authenticates the data item by MACing it using the application session key MacKeyY,W as follows: MAC (MacKeyY,W; W || Y || CountW,Y || Data4 ) W is the sender’s identity and Y is the intended recipient’s identity. The data item Data4 is then sent over the RF channel along with the MAC value calculated on it. This ensures the authenticity of Data4. »

The translation of the need : « When a request for sending Data over a secured Dialogue is made, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

- The implementation of the need : « The D-DATA Request over a secured Dialogue, triggers the computation of an appendix on the D-DATA Request User data, the senders (W) and the receiving (Y) Dialogue users, on a counter (CountW,Y) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyY,W) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-DATA Request user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-DATA Request user data, and the appendix.» This SESE APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.2.3.2.1.

Receiver side

The implementation of the need : « The incoming SESE APDU contained on the Presentation user data parameter, because it arrives on a secured Dialogue, triggers the check of the appendix contained in the SESE APDU, using the identity of the peer Dialogue User (W), the identity of the local Dialogue User (Y), the counter (CountW,Y) of the User data sent from the sending to the receiving peers, and the unprotected part of the SESE APDU (i.e. original D-DATA request user data). » This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.1.4.2.2.

###### Associated service activations

Sender side

- D-Data Request

| User Data | aDS-periodic-report-PDU |
| --- | --- |

- SA-Send Request

| Local Entity ID | Airborne ADS |
| --- | --- |
| Remote Entity ID | Ground ADS |
| User Data | aDS-periodic-report-PDU |

- SSO-ProtectSign

| Source peer | Airborne ADS |
| --- | --- |
| Destination Peer | Ground ADS |
| User Data | aDS-periodic-report-PDU |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : aDS-periodic-report-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

Receiver side

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : aDS-periodic-report-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| Security item | atnProtectSign { unprotected : aDS-periodic-report-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |

- SA-Send Indication

| Local Entity ID | Ground ADS |
| --- | --- |
| Remote Entity ID | Airborne ADS |
| User Data | aDS-periodic-report-PDU |

- D-Data Indication

| User Data | aDS-periodic-report-PDU |
| --- | --- |

##### Ground initiated data transfer phase

###### Mapping the expression of the need to an implementation

Sender side

The expressed need : « When the ground application entity and the aircraft application entity wish to exchange a data item Data4, such as a D-Start or D-Data item, the sender, say W, authenticates the data item by MACing it using the application session key MacKeyY,W as follows: MAC (MacKeyY,W; W || Y || CountW,Y || Data4 ) W is the sender’s identity and Y is the intended recipient’s identity. The data item Data4 is then sent over the RF channel along with the MAC value calculated on it. This ensures the authenticity of Data4. »

- The translation of the need : « When a request for sending Data over a secured Dialogue is made, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

The implementation of the need : « The D-DATA Request over a secured Dialogue, triggers the computation of an appendix on the D-DATA Request User data, the senders (W) and the receiving (Y) Dialogue users, on a counter (CountW,Y) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyY,W) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-DATA Request user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-DATA Request user data, and the appendix.» This SESE APDU is sent using the Presentation services. This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.2.3.2.1.

Receiver side

The implementation of the need : « The incoming SESE APDU contained on the Presentation user data parameter, because it arrives on a secured Dialogue, triggers the check of the appendix contained in the SESE APDU, using the identity of the peer Dialogue User (W), the identity of the local Dialogue User (Y), the counter (CountW,Y) of the User data sent from the sending to the receiving peers, and the unprotected part of the SESE APDU (i.e. original D-DATA request user data). » This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.1.4.2.2.

###### Associated service activations

Sender side

- D-Data Request

| User Data | aDS-cancel-contract-PDU |
| --- | --- |

- SA-Send Request

| Local Entity ID | Ground ADS |
| --- | --- |
| Remote Entity ID | Airborne ADS |
| User Data | aDS-cancel-contract-PDU |

- SSO-ProtectSign

| Source peer | Ground ADS |
| --- | --- |
| Destination Peer | Airborne ADS |
| User Data | aDS-cancel-contract-PDU |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : aDS-cancel-contract-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

Receiver side

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : aDS-cancel-contract-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Ground CM |
| --- | --- |
| Destination Peer | Airborne CM |
| Security item | atnProtectSign { unprotected : aDS-cancel-contract-PDU appendix { algorithmId : absent validity : absent value : hmac-Tag } } |

- SA-Send Indication

| Local Entity ID | Airborne ADS |
| --- | --- |
| Remote Entity ID | Ground ADS |
| User Data | aDS-cancel-contract-PDU |

- D-Data Indication

| User Data | aDS-cancel-contract-PDU |
| --- | --- |

##### Dialogue end : Request-Indication phase

###### Mapping the expression of the need to an implementation

Initiating side

No explicit need was expressed for Dialogue ending phase. The choice was to consider this phase as equivalent to data transfer phase, with respect to security consideration, and to transfer security related information together with user data using ACSE Release service.

- « When a request for ending a secured Dialogue is made, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

The implementation of the need : « The D-END Request over a secured Dialogue, triggers the computation of an appendix on the D-END Request User data, the senders (V) and the receiving (U) Dialogue users, on a counter (CountV,U) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyU,V) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-END Request user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-END Request user data, and the appendix.» This SESE APDU is sent using the A-RELEASE Request service. This can be implemented following the sequence of service primitives described on the right part of the drawing and in 1.1.4.1.1.1.5.2.1.

Receiver side

- The implementation of the need : « The incoming SESE APDU contained on the A-RELEASE Indication user information parameter, because it arrives on a secured Dialogue, triggers the check of the appendix contained in the SESE APDU, using the identity of the peer Dialogue User (V), the identity of the local Dialogue User (U), the counter (CountV,U) of the User data sent from the sending to the receiving peers, and the unprotected part of the SESE APDU (i.e. original D-END request user data). » This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.1.1.1.5.2.2.

###### Associated service activations

Initiating side

- D-End Request

| User Data | Not used |
| --- | --- |

- SA-Send Request

| Local Entity ID | Ground CM |
| --- | --- |
| Remote Entity ID | Airborne CM |
| User Data | Not Used |

- SSO-ProtectSign

| Local peer | Airborne CM |
| --- | --- |
| Remote Peer | Ground CM |
| User Data | Not Used |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- A-Release Request

| Reason | “normal” |
| --- | --- |
| User Information | SESE PDU |

Receiver side

- A-Release Indication

| Reason | “normal” |
| --- | --- |
| User Data | SESE PDU |

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Ground CM |
| --- | --- |
| Destination Peer | Airborne CM |
| Security item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |

- SA-Send Indication

| Local Entity ID | Airborne CM |
| --- | --- |
| Remote Entity ID | Ground CM |
| User Data | Not Used |

- D-End Indication

| User Data | Not Used |
| --- | --- |

##### Dialogue end : Response-Confirmation phase

###### Mapping the expression of the need to an implementation

Receiver side

No explicit need was expressed for Dialogue ending phase. The choice was to consider this phase as equivalent to data transfer phase, with respect to security consideration, and to transfer security related information together with user data using ACSE Release service.

- « When a response for ending a secured Dialogue is made, a security item needs to be appended to the Dialogue User data, which permits the receiving peer to check the integrity of the received data and the origin of the sending peer. »

The implementation of the need : « The D-END Response over a secured Dialogue, triggers the computation of an appendix on the D-END Response User data, the senders (V) and the receiving (U) Dialogue users, on a counter (CountV,U) of the User data sent from the sending to the receiving entity, and on the session key (MacKeyU,V) established between the sending and receiving peers. The Dialogue control function then transmits to its peer the D-END Response user data together with the appendix. To do this, the S-ASO builds an exchange item, under the form of a SESE APDU containing the D-END Response user data, and the appendix.» This SESE APDU is sent using the A-RELEASE Response service. This can be implemented following the sequence of service primitives described on the left part of the drawing and in 1.1.4.1.1.1.6.2.1.

###### Associated service activations

Receiving side

- D-End Response

| Result |  |
| --- | --- |
| User Data | Not Used |

- SA-Send Request

| Local Entity ID | Airborne CM |
| --- | --- |
| Remote Entity ID | Ground CM |
| User Data | Not used |

- SSO-ProtectSign

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| User Data | Not used |
| Algorithm Identifier | MAC-appendix |

- SE-Transfer Request

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- A-Release Response

| Reason | “normal” |
| --- | --- |
| User Data | SESE PDU |
| Result | « affirmative » |

Initiating side

- A-Release Confirmation

| Reason | “normal” |
| --- | --- |
| User Data | SESE PDU |
| Result | « affirmative » |

- SE-Transfer Indication

| Security exchange identifier | atnProtectSignSE |
| --- | --- |
| Invocation Identifier | Unambiguous Identifier |
| Security exchange item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| Item identifier | atnProtectSign security exchange item |
| Start flag | True |
| End flag | absent |

- SSO-ProtectSignCheck

| Source peer | Airborne CM |
| --- | --- |
| Destination Peer | Ground CM |
| Security item | atnProtectSign { unprotected : absent appendix { algorithmId : absent validity : absent value : hmac-Tag } } |
| CheckResult |  |

- SA-Send Indication

| Local Entity ID | Ground CM |
| --- | --- |
| Remote Entity ID | Airborne CM |
| User Data | Not Used |

- D-End Confirmation

| Result | “affirmative” |
| --- | --- |
| User Data | Not Used |

The full scenario described above is summarized in the following picture:

| Dialogue | SA-Service | SESE | ACSE | Présentation | Session | Transport | Session | Présentation | ACSE | SESE | SA-Service | Dialogue |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| D-Start req | SA-Send req | SE-Transfer req | A-Associate req | P-Connect req | S-Connect req | T-Connect req | T-Connect ind |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T-Connect cnf | T-Connect rsp |  |  |  |  |  |  |
|  |  |  |  |  |  | T-Data req | T-Data ind | S-Connect ind | P-Connect ind | A-Associate ind | SE-Transfer ind | SA-Send ind | D-Start ind |
| D-Start cnf | SA-Send ind | SE-Transfer ind | A-Associate cnf | P-Connect cnf | S-Connect cnf | T-Data ind | T-Data req | S-Connect rsp | P-Connect rsp | A-Associate rsp | SE-Transfer req | SA-Send req | D-Start rsp |
| D-Data req | SA-Send req | SE-Transfer req |  | P-Data req | S-Data req | T-Data req | T-Data ind | S-Data ind | P-Data ind |  | SE-Transfer ind | SA-Send ind | D-Data ind |
| D-Data ind | SA-Send ind | SE-Transfer ind |  | P-Data ind | S-Data ind | T-Data ind | T-Data req | S-Data req | P-Data req |  | SE-Transfer req | SA-Send req | D-Data req |
| D-End ind | SA-Send ind | SE-Transfer ind | A-Release ind | P-Data ind | S-Data ind | T-Data ind | T-Data req | S-Data req | P-Data req | A-Release req | SE-Transfer req | SA-Send req | D-End req |
| D-End rsp | SA-Send req | SE-Transfer req | A-Release rsp | P-Data req | S-Data req | T-Data req | T-Data ind | S-Data ind | P-Data ind | A-Release cnf | SE-Transfer ind | SA-Send ind | D-End cnf |
|  |  |  |  | P-Pabort ind | S-PAbort ind | T-Disconnect ind | T-Disconnect req | S-UAbort req | P-UAbort req |  |  |  |  |