ATNP/WG3/SG1

20/02/97

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL(ATNP)

Working Group 3 - Applications and Upper Layers

SUBGROUP 1 - GROUND APPLICATIONS SUBGROUP

Draft ATN SARPs

3.1.	ATS Message Handling Services (ATSMHS)

Proposed Version 3.0

Date : 20/02/97

Prepared by ATSMHS SARPs Editor

Summary

This document is the proposed version 3.0 of the Draft SARPs for ATS Message Handling Services (ATSMHS), which is section 3.1 of the ATN SARPs (Appendix A).

It is the outcome of the 10th SG1 meeting, proposed for endorsement to WG3 and to the ATNP Working Group of the Whole meeting in Thailand (March 1997), as final CNS/ATM-1 package SARPs for the ATSMHS application.

This version, which uses a word processing format internal to WG3 and SG1 for the sake of working group convenience, is based on the ICAO Version 1.0 which is a constitutive part of the ATNP/2 meeting report, all changes being identified by means of revision marks.

Document Control Log

Contact :	J.M. Vacher, Service Technique de la Navigation Aérienne, France
E-Mail :		VACHER_Jean-Marc@stna.dgac.fr

| Section(s) | Date | New Version | Reason for change |
| --- | --- | --- | --- |
| All | 02/03/95 | 0.0 (provisional) | identification by the Editor of areas where additions/ amendments/ deletions are needed |
| All | 27/04/95 | 0.0 (approved) | integration into the document of the additional areas for changes suggested prior to approval of version 0.0 |
| All | 31/05/95 | 0.1 (provisional) | inclusion of changes in preparation for discussion at the Brussels SG1 meeting. |
| All | 27/06/95 | 0.1 (approved) | approval by the Brussels SG1 meeting of revisions proposed in Version 0.1 (provisional). |
| All | 06/10/95 | 0.2 (provisional) | restructuration for inclusion in overall CNS/ATM-1 Package SARPs, separation of Types A and B environments, in preparation for discussion at the Banff SG1 meeting. |
| All | 23/11/95 | 0.3 (provisional) | 1st MHS DG meeting (Paris 8-10 November 1995) and Editor's work to take into account the meeting conclusions, in preparation for discussion at the London SG1 meeting |
| All | 26/01/96 | 1.0 (provisional) | incorporation of the London SG1 meeting conclusions, of the 2nd MHS DG meeting (Paris 10-16 January 1996) conclusions, full development of section 2.3 in preparation for the Brisbane SG1 and WG3 meetings |
| All | 04/02/96 | 1.0 | incorporation of the Brisbane MHS DG sessions and SG1 meeting conclusions |
| All | 25/03/96 | 1.0a (1st amended proposal) | incorporation of the Brisbane WG3 meeting conclusions : important editorial changes, limited technical changes (3.1.2.2.1.2.1, 3.1.2.2.2.2.1, 3.1.2.2.2.2.4, and 3.1.2.3.5.3.2.1) |
| All | 15/04/96 | 1.0b (2nd amended proposal) | incorporation of the Brussels SG1 meeting conclusions |
| All | 23/04/96 | 1.0z (baseline version) | incorporation of the sixth WG3 meeting (Brussels) conclusions on MHS SARPs |
| see change history | 21/06/96 | 1.1 (proposed 2.0) | incorporation of the sixth WG3 meeting (Brussels) conclusions on SARPs editorial rules, of SG1 ad-hoc discussions on logging provisions and of changes resulting from comments on version 1.0z |
| see change history | 27/06/96 | 1.2 (proposed 2.0) | incorporation of the seventh WG3 meeting (Munich) initial discussions on SARPs |
| see change history | 04/07/96 | 1.3 (approved 2.0 with change-bars) | incorporation of the seventh WG3 meeting (Munich) final discussions on ATSMHS SARPs, approved for distribution as version 2.0 subject to incorporation of agreed changes |
| see 1.3 | 04/07/96 | 2.0 | adoption of all revisions in version 1.3 |
| see change history | 10/02/97 | 2.1 | ATNP/2 output, incorporating the change pages agreed in the eigth WG3 meeting (Alexandria) and presented as WP-31/Flimsy 7 at ATNP/2. Alignment on ICAO Version 1.0 as included in ATNP/2 meeting report. |
| see change history | 12/02/97 | proposed 3.0 | incorporation of the 10th SG1 meeting (Toulouse, Jan. 97) conclusions |

Change history starting from the baseline version

| Version |  | Description | Affected Parts |  | Date |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0z |  | Output Brussels Draft SARPS (baseline version) |  |  | 24/04/96 |  |  |  |  |
| 1.1 |  | Input to ATNP/WG3 at Munich (proposed 2.0) |  |  | 21/06/96 |  |  |  |  |
|  |  | Chapter 1 |  |  |  |  |  |  |  |
|  |  | - Conversion of introductory material into Notes | §3.1.1 |  |  |  |  |  |  |
|  |  | - Deletion of references for inclusion in Sub-Volume 1 | §3.1.1 |  |  |  |  |  |  |
|  |  | - Deletion of definitions/glossary for inclusion in Sub-Vol. 1 | §3.1.1 |  |  |  |  |  |  |
|  |  | - Spelling out of acronyms at first use | all chapters |  |  |  |  |  |  |
|  |  | Chapter 2 |  |  |  |  |  |  |  |
|  |  | - Replacement of former 3.1.1.1.4.3 with 3.1.1 Notes 8 and 9 | legend of all tables in §3.1.2 |  |  |  |  |  |  |
|  |  | - Use of capital letters to express ISP classifications | Tables 3.1.2-1, 3.1.2-5, 3.1.2-6, 3.1.2-8, 3.1.2-9, 3.1.2-11, 3.1.2-12, 3.1.2-14, 3.1.2-15, 3.1.2-17, 3.1.2-18 |  |  |  |  |  |  |
|  |  | - Use of the terms ISO/IEC 10021 or Message Handling System to replace the term MHS where reference to standards is made | §3.1.1 Note 10, §3.1.2.1.2.1.1, §3.1.2.1.2.2.1, §3.1.2.2.1.2.1, §3.1.2.3.2.2.4, §3.1.2.3.2.3.4, §3.1.2.3.5.1.5 |  |  |  |  |  |  |
|  |  | - Clarification of the AMHS PRMD / ADMD relationship | §3.1.2.1.3 |  |  |  | 26 |  |  |
|  |  | - Clarification of the situation of Aeronautical Industry Addressing Schemes | §3.1.2.1.5.1.4.1 |  |  |  | 117 |  |  |
|  |  | - Simplification of the logging requirements at an ATS Message Server (deletion of probe-related and MTA-bind/unbind-related event logging), one single clause for message-related event logging, one single clause for report-related event logging | §3.1.2.2.2.3 |  |  |  | 115 |  |  |
|  |  | - Clarification of the processing of AFN service messages requesting to the originator repetition of a message which was detected as mutilated upon reception | §3.1.2.3.2.1.12 |  |  |  | 129 |  |  |
|  |  | - Simplification of the MTCU logging requirements (deletion of probe-related logging), use of event date/time, clarification of the Origin of an AFTN message | §3.1.2.3.3.1 |  |  |  | 116 |  |  |
|  |  | - Deletion of a requirement for reporting to the control position where reporting was for information only | §3.1.2.3.4.4.1.5 |  |  |  | 137 |  |  |
|  |  | - Specification of non-delivery report components for the conversion of an AFTN unknown addressee service message | §3.1.2.3.4.4.1.6 |  |  |  | 133 |  |  |
|  |  | - Clarification of the generation of a NDR resulting from the conversion of an AFTN unknown addressee service message | §3.1.2.3.4.4.1.6 |  |  |  | 136 |  |  |
|  |  | - Correction of cross-references | §3.1.2.3.4.4.1.5, §3.1.2.3.5.2.1.8 |  |  |  | 130, 132 |  |  |
|  |  | - Terminology enhancement for character repertoires | §3.1.2.3.5.2.2.9 |  |  |  | 126 |  |  |
|  |  | - Clarification of the use of clause 3.1.2.3.5.6 for the generation of NDRs either as the result of message rejection at an AFTN/AMHS Gateway or as the result of the conversion of an AFTN unknown addressee service message | §3.1.2.3.5.6.1.1 |  |  |  | 135 |  |  |
|  |  | - Use of abstract-values for the specification of report generation conditions | §3.1.2.3.5.6.1.3, §3.1.2.3.5.6.2.12, §3.1.2.3.5.6.2.16 |  |  |  | 138 |  |  |
|  |  | - Clarification of conditions for the generation of certain report elements | Table 3.1.2-18, Part 1, item 2.1.2, §3.1.2.3.5.6.2.23, §3.1.2.3.5.6.2.24, §3.1.2.3.5.6.2.25 |  |  |  |  |  |  |
|  |  | Chapter 3 |  |  |  |  |  |  |  |
|  |  | - Editorial upgrade of address definitions | §3.1.3.1.8 |  |  |  | 127 |  |  |
|  |  | - Deletion of duplicate clause | §3.1.3.3.3.5.9 |  |  |  | 128 |  |  |
| 1.2 |  | Interim Result of ATNP/WG3 at Munich (proposed 2.0) |  |  | 27/06/96 |  |  |  |  |
|  |  | all chapters |  |  |  |  |  |  |  |
|  |  | - Deletion of the words SARPs, Sub-Volume, Part, Chapter, Section |  |  |  |  |  |  |  |
|  |  | Chapter 1 |  |  |  |  |  |  |  |
|  |  | - Deletion of Title 3.1.1.1 | §3.1.1 |  |  |  |  |  |  |
|  |  | - Conversion of ex-Note 1 into plain text, subsequent renumbering of Notes | §3.1.1 |  |  |  |  |  |  |
|  |  | - Refinement of some terms, few deletions in Note 6 | §3.1.1 Note 6 |  |  |  |  |  |  |
|  |  | - Clarification of the need for a refined PRL classification | §3.1.1 Note 7 |  |  |  |  |  |  |
|  |  | - Clarification of the note on SARPs applicability | §3.1.1 Note 10 |  |  |  |  |  |  |
|  |  | Chapter 2 |  |  |  |  |  |  |  |
|  |  | - Replacement of direct/indirect user with direct/indirect AMHS user | 15 clauses |  |  |  |  |  |  |
|  |  | - Replacement of acknowledgement message with AFTN acknowledgement message | 5 clauses |  |  |  |  |  |  |
|  |  | - Deletion of paragraph titles with only notes underneath, consequential renumbering | §3.1.2.1.2.3, §3.1.2.1.5.2.3, §3.1.2.2.1.1, §3.1.2.2.1.2, §3.1.2.2.2.1, §3.1.2.2.2.2 |  |  |  |  |  |  |
|  |  | - Addition of a Note clarifying the requirement for long-term logging in origination | §3.1.2.1.7 |  |  |  | 118 |  |  |
|  |  | - Addition of part numbers | Table 3.1.2-1 |  |  |  |  |  |  |
|  |  | - Correction of a cross-reference | Table 3.1.2-1, Part 2, 1.1 |  |  |  |  |  |  |
|  |  | - Addition of a clause and of two notes regarding the generation of AMHS information objects at the AFTN/AMHS Gateway control position | §3.1.2.3.2.6.3 |  |  |  | 134 |  |  |
|  |  | - Harmonization of the handling of error situations upon receipt of AFTN service messages in the MTCU | §3.1.2.3.4.3.1.2 |  |  |  | 134 |  |  |
|  |  | Chapter 3 |  |  |  |  |  |  |  |
|  |  | - Replacement of authority with ATS Organisation | 5 clauses |  |  |  | 140 |  |  |
|  |  | - Clarification of the AFTN/ATN Type A Gateway users | §3.1.3.1.4 |  |  |  | 141 |  |  |
|  |  | - Renaming of title for clarification of flow control | §3.1.3.1.10 |  |  |  | 144, 145 |  |  |
|  |  | - Harmonization of the AFTN/ATN Type A Gateway security and management provisions with the AFTN/AMHS Gateway specification | §3.1.3.1.5.2, §3.1.3.1.6, §3.1.3.1.7 |  |  |  | 142, 150 |  |  |
|  |  | - Clarification of the note concerning the impact of configurations on the AFTN procedure handling by an AFTN/ATN Type A Gateway | §3.1.3.1.6.2, §3.1.3.2.3, §3.1.3.2.4 |  |  |  |  |  |  |
|  |  | - Clarification of the ATN address form to refer to an ICAO facility designator | §3.1.3.1.8, §3.1.3.3.3.5.4, §3.1.3.3.3.5.5, §3.1.3.3.3.5.9 |  |  |  | 193, 195, 202 |  |  |
|  |  | - Clarification of routing in an environment including AFTN/ATN Type A Gateways | §3.1.3.1.9 |  |  |  |  |  |  |
|  |  | - Definition of the GA-Data service parameters | §3.1.3.3.2.2 |  |  |  | 151, 152 |  |  |
|  |  | - Insertion of a note to indicate that the CF specification does not constrain implementations | §3.1.3.3.2.4 |  |  |  | 156 |  |  |
|  |  | - Clarification of the use of the Dialogue Service | Tables 3.1.3-2, 3.1.3.-3, 3.1.3-5, 3.1.3-6 |  |  |  | 157, 158, 165, 176 |  |  |
|  |  | - Addition of detailed clauses on Dialogue Service aborts | §3.1.3.3.2.4.18, §3.1.3.3.2.4.19, §3.1.3.3.2.4.20 |  |  |  | 159, 178, 179, 180 |  |  |
|  |  | - Refinement of the AFTN priority mapping | Table 3.1.3-7 |  |  |  | 182 |  |  |
|  |  | - Clarification of the role of the address mapping performed by an AFTN/ATN Type A Gateway | §3.1.3.3.3.2 |  |  |  | 185 |  |  |
|  |  | - Clarification of the use of the GA-Data Service by the MTCU | §3.1.3.3.3.5.2, §3.1.3.3.3.5.7, §3.1.3.3.3.5.8 |  |  |  | 191, 192, 197, 198 |  |  |
| 1.3 |  | Result of ATNP/WG3 at Munich (approved 2.0, with change-bars) |  |  | 04/07/96 |  |  |  |  |
|  |  | Chapter 3 |  |  |  |  |  |  |  |
|  |  | - Clarification of the ATN Pass-Through Service environment | §3.1.3.1.1 |  |  |  | 207 |  |  |
|  |  | - Further clarification of the AFTN/ATN Type A Gateway users | §3.1.3.1.4 |  |  |  | 208 |  |  |
|  |  | - Change of the Priority (transmission) parameter name | §3.1.3.3.2.2.1, Table 3.1.3-2, §3.1.3.3.3.5.6, §3.1.3.3.3.5.10 |  |  |  | 209, 216 |  |  |
|  |  | - Definition of the AFTN Priority parameter | §3.1.3.3.2.2.5 |  |  |  | 213 |  |  |
|  |  | - Clarification of the definition of the User Data in the GA-Data Service | §3.1.3.3.2.2.2 |  |  |  | 211 |  |  |
|  |  | - Clarification of the definition of the address parameters in the GA-Data Service | §3.1.3.3.2.2.3, §3.1.3.3.2.2.4 |  |  |  | 212 |  |  |
|  |  | - Specification of the ATN Component Control Function as also incorporating the UL Communication Service CF | §3.1.3.3.2.3, §3.1.3.3.2.4 |  |  |  | 163, 215 |  |  |
|  |  | - Use of consistent parameter names and abstract-values in the CF specification and in the Dialogue Service priority specification | Tables 3.1.3-2, 3.1.3-3, 3.1.3-4, 3.1.3-5, 3.1.3-7, §3.1.3.3.2.4.7, §3.1.3.3.2.4.9, |  |  |  | 217, 220, 222, 223, 224, 225, 226, 227, 229, 230, 231 |  |  |
|  |  | - Aggregation of clauses with the goal of having self-contained specifications, and subsequent renumbering | §3.1.3.3.2.4.6, §3.1.3.3.2.4.7, §3.1.3.3.2.4.8/17 |  |  |  |  |  |  |
|  |  | - Use of the Calling Address by the CF for invocation of a GA-Data indication | §3.1.3.3.2.4.9 (Note 2) |  |  |  | 210 |  |  |
|  |  | - Clarification of clauses on Dialogue Service aborts | §3.1.3.3.2.4.15, §3.1.3.3.2.4.16 |  |  |  | 228 |  |  |
|  |  | - Clarification of the use of priorities within the AFTN/ATN type A Gateway | §3.1.3.3.2.5.2, §3.1.3.3.3.6.1, §3.1.3.3.3.6.2 |  |  |  | 204, 232 |  |  |
|  |  | - Deletion of an uncertain informative statement | §3.1.3.3.3.3.2 (Note) |  |  |  | 233 |  |  |
|  |  | - Clarification of the use of the GA-Data Service for the interface between the MTCU and the ATN Component | §3.1.3.3.3.4.1, §3.1.3.3.3.5 |  |  |  | 234, 235/238, 240 |  |  |
| 2.0 |  | Result of ATNP/WG3 at Munich (approved 2.0) |  |  | 04/07/96 |  |  |  |  |
|  |  | - Adoption of all changes included in 1.3 |  |  |  |  |  |  |  |
| 2.1 |  | Result of ATNP/2 Meeting (alignment on ICAO Version 1.0) |  |  | 10/02/97 |  |  |  |  |
|  | - Addition of new clauses to specify the processing to be performed upon receipt of a D-START-confirmation; renumbering of subsequent clauses. |  |  | §3.1.3.3.2.4.4 §3.1.3.3.2.4.8/11 |  |  |  | 218 |  |
|  | - Addition of a note to indicate that the definition of an "acceptable" value is a local matter. |  |  | §3.1.3.3.2.4.6 |  |  |  | 219 |  |
|  | - Addition of a note to indicate that the definition of an "unacceptable" value is a local matter. |  |  | §3.1.3.3.2.4.7 |  |  |  | 221 |  |
|  | - Status of Calling Address changed to "M" in Table 3.1.3-1. Indication in clause 3.1.3.3.3.5.9 that parameter is taken from there. Deletion of Note 2 under 3.1.3.3.2.4.7. |  |  | Table 3.1.3-1, §3.1.3.3.2.4.7 §3.1.3.3.3.5.9 |  |  |  | 239 |  |
|  | - Note 2 is changed into a "shall" statement requiring that ITA-2/IA-5 conversion be performed in the AFTN Component. |  |  | §3.1.3.1.5 |  |  |  | 241 |  |
|  | - Inclusion of reference to Sub-Volume 4. |  |  | §3.1.3.1.8.1b) |  |  |  | 243 |  |
|  | - Correction of normative reference |  |  | §3.1.2.2.2.1 a) |  |  |  | 244 |  |
|  | - Correction of normative reference to refer to ISP 12062-3. |  |  | §3.1.2.3.2.3.4 |  |  |  | 245 |  |
|  | - Addition of a note to clarify that PDAIs are converted into MF-addresses. |  |  | §3.1.2.3.4.2.3.2 |  |  |  | 246 |  |
|  | - Inclusion of mention that only MF-Addresses marked with the responsibility flag have to be translated. |  |  | §3.1.2.3.5.2.2.6.2 |  |  |  | 247 |  |
|  | - deletion of reference |  |  | Table 3.1.2-17/2.1.1 |  |  |  | 249 |  |
|  | - Replacement of "information objects" with "messages, probes and reports" to reflect processing of all information objects. |  |  | §3.1.2.3.2.3.4 |  |  |  | 250 |  |
|  | - full spelling of term PRL |  |  | §3.1.1 Note 8 |  |  |  | 251 |  |
|  | - typo - missing hyphen in IA5 added |  |  | §3.1.2.2.3.2.4 |  |  |  | 252 |  |
|  | - typo - word "an" in excess deleted |  |  | §3.1.2.3.2.1.15 |  |  |  | 253 |  |
|  | - typo - missing space added |  |  | §3.1.2.3.4.2.1.2 |  |  |  | 254 |  |
|  | - title of table changed, rewording of clause |  |  | §3.1.3.3.2.4.5, Table 3.1.3.-2 |  |  |  | 256 |  |
|  | - sequence of terms changed |  |  | §3.1.2.3.2.4.2 |  |  |  | 257 |  |

|  |  | - Change to specify that the object is stored for appropriate action at the control position, addition of the conversion of AFTN SVC messages (ack and ADS UNKNOWN) into IPMs when conversion into RNs and NDRs cannot be performed by addition of two clauses. |  | §3.1.2.3.4.1.3, §3.1.2.3.4.2.1.4.1, §3.1.2.3.4.3.1.1, §3.1.2.3.4.3.1.5 (new), §3.1.2.3.4.4.1.1/4, §3.1.2.3.4.4.1.6, §3.1.2.3.4.4.1.7 (new), §3.1.2.3.5.1.2, §3.1.2.3.5.1.5, §3.1.2.3.5.2.3.3, §3.1.2.3.5.3.1.1/2, §3.1.2.3.5.4.1.1/3, §3.1.2.3.5.4.2.1, §3.1.2.3.5.4.2.8/9 |  |  |  | 258 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | - Addition of a clause 3.1.2.3.5.4.4 specifying that NDRs are converted into AFTN service messages requesting message repetition. |  | §3.1.2.3.5.4.1.3 §3.1.2.3.5.4.4 |  |  |  | 259 |  |  |
|  |  | - status of Text item changed to "G", deletion of references in column "converted from AMHS parameter" |  | Table 3.1.2-13 |  |  |  | 260 |  |  |
|  |  | - deletion of "CNS/ATM-1 Package" terms |  | §3.1.1 |  |  |  | 261 |  |  |
|  |  | - reference in table legend corrected (3.1.1.1 changed to 3.1.1) |  | All Tables |  |  |  | 262 |  |  |
|  |  | - cross-references corrected |  | §3.1.2.2.2 (Note); §3.1.2.3.4.2.1.3 (Note); §3.1.2.3.4.2.3.8 d); Table 3.1.2-7, Text item; Table3.1.2-11/ Part 4/11; §3.1.2.3.5.2.3.2, Notes 2 and 3; Table 3.1.2-12/ Part 2/5.1 to 5.4; Table 3.1.2-14/ Part 1/1; §3.1.2.3.4.3.1.2; Table 3.1.2.-17/Part 1/2.2.5 |  |  |  | 263 |  |  |
|  |  | - replacement of "content-identifier" with "returned-content" |  | §3.1.2.3.5.6.2.16 |  |  |  | 264 |  |  |
|  |  | - addition of term "component" in section title |  | §3.1.3.3.2.4 |  |  |  | 265 |  |  |
|  |  | - typo - word "been" deleted |  | §3.1.2.3.4.4.1.6 |  |  |  | 268 |  |  |
|  |  | - insertion of "and of the non-delivery-report" where appropriate |  | §3.1.2.3.5.4.2.9 |  |  |  | 271 |  |  |
|  |  | - typo - word "AFTN" converted to bold |  | §3.1.1. Note 6 a) |  |  |  | 275 |  |  |
|  |  | - typo - parenthesis added |  | §3.1.1. Note 6 b) |  |  |  | 276 |  |  |
|  |  | - typo - dash added after "Note." |  | §3.1.2.1.2.1.1. Note |  |  |  | 277 |  |  |
|  |  | - typo - text converted to italics |  | §3.1.2.1.5.1.2. Note |  |  |  | 278 |  |  |
|  |  | - merging of columns "Element" and "Repertoire identifier(s)" into a single column "element", common column heading for all parts |  | Table 3.1.2-1Part4 |  |  |  | 279 |  |  |
|  |  | - typo - note number added (6) |  | §3.1.2.2.2.1.1. last Note |  |  |  | 280 |  |  |
|  |  | - typo - text converted to italics |  | §3.1.2.3.5.1.1.Note1 |  |  |  | 281 |  |  |
|  |  | - insertion of "of the non-delivery-report and" where appropriate |  | §3.1.2.3.5.4.1.2, sub-items b)2) and c)2) |  |  |  | 282 |  |  |
|  |  | - "CCITT" changed into "ITU-T" |  | §3.1.2.3.5.6.2.29 |  |  |  | 283 |  |  |
|  |  | - "ICAO facility designator" replaced with "facility designation" |  | §3.1.3.1.8.1 b), §3.1.3.1.8.2, §3.1.3.3.2.2.3, §3.1.3.3.2.2.4, §3.1.3.3.3.5.4, §3.1.3.3.3.5.5 |  |  |  | 284 |  |  |
|  |  | - "confirm" replaced with "confirmation" in title |  | Table 3.1.3-3. |  |  |  | 285 |  |  |
|  |  | - deletion of of the words "is optional", "and", ",if provided". |  | §3.1.3.3.3.5.5 |  |  |  | 286 |  |  |
|  | 3.0p |  | Presentation to ATNP WGOW (Thailand March 1997) |  |  |  | 20/02/97 |  |  |  |
|  |  | - Inclusion of the requirement to pass NDRs to the control position. Actions which can then be performed depend on rejection cause, will be detailed in GM and may be a) request message repetition, b) correct error condition in AMHS, c) send AFTN "text message" indicating non-delivery d) any other as appropriate. Amendment of clauses on control position in accordance with this principle. |  | §3.1.2.3.5.4.1.1-3, §3.1.2.3.2.6.1, §3.1.2.3.2.6.2, §3.1.2.3.2.6.3 |  |  |  | 270 |  |  |
|  |  | - Clause deleted. See comment 270 for entire description of the solution. |  | §3.1.2.3.5.4.4 |  |  |  | 272 |  |  |
|  |  | - Note added |  | §3.1.3.3.2.4.21 |  |  |  | 273 |  |  |
|  |  | - Use of the extension parameters which are "critical for delivery" is excluded when the parameter absence means default value. Deletion of clauses 3.1.2.3.4.2.3.9/11, replacement with a note. Renumbering of all subsequent clauses. |  | Table 3.1.2-6 (Part 1/1.1.11.1 and 1.1.11.2), §3.1.2.3.4.2.3.8 to §3.1.2.3.4.2.3.11 |  |  |  | 295 |  |  |
|  |  | - Replacement of the "X" in the "Action" column with "G." b. Replacement of the "-" in the "Mapping / Notes" column with "See 3.1.2.3.4.2.3.20" c. Addition of clause 3.1.2.3.4.2.3.20 which indicates that content-return-request should take its default value. Renumbering of all subsequent clauses. |  | Table 3.1.2-6(Part 2/4.4), §3.1.2.3.4.2.3.20 (new) |  |  |  | 274 |  |  |
|  |  | - typo - change made to correct "internationl" |  | §3.1.1 Note 2 |  |  |  | 287 |  |  |
|  |  | - typo - "IA5"changed to "IA-5" |  | §3.1.2.2.3.2 |  |  |  | 288 |  |  |
|  |  | - typo - "by" deleted |  | §3.1.2.3.3.2.1.1 |  |  |  | 289 |  |  |
|  |  | - reinsert element 1 in table above element 1.1 |  | Table 3.1.2-5 |  |  |  | 290 |  |  |
|  |  | - "CCITT" changed into "ITU-T" |  | §3.1.2.3.4.2.3.17 |  |  |  | 291 |  |  |
|  |  | - typo - "take" replaced with "takes" |  | §3.1.2.3.4.3.1.5 |  |  |  | 292 |  |  |
|  |  | - text converted to italics |  | §3.1.2.3.5.2.4.5Note |  |  |  | 293 |  |  |
|  |  | - text changed to read "has any abstract-value other than" |  | §3.1.2.3.5.4.1.3 |  |  |  | 294 |  |  |
|  |  | - Correction of cross-references to Annex 10 Volume II changed as the result of Amendment 71 |  | §3.1.2.3.2.1.4 §3.1.2.3.2.1.5 §3.1.2.3.2.1.7 §3.1.2.3.2.1.12 §3.1.2.3.2.5.2 §3.1.2.3.4.1.1 Table 3.1.2-10 §3.1.2.3.5.2.2.6.2 §3.1.2.3.5.3.2.5 §3.1.2.3.5.3.2.6 Table 3.1.2-16 §3.1.3.1.8.1 |  |  |  | 297 |  |  |

Table of Contents

3.1.1.  INTRODUCTION

3.1.2.  ATS MESSAGE SERVICE

3.1.2.1. System level provisions

3.1.2.1.1. ATS Message Service Users

3.1.2.1.2. AMHS Model

3.1.2.1.2.1. AMHS functional model

3.1.2.1.2.1.1. Model components

3.1.2.1.2.1.2. ATS Message Server

3.1.2.1.2.1.3. ATS Message User Agent

3.1.2.1.2.1.4. AFTN/AMHS Gateway

3.1.2.1.2.2. AMHS information model

3.1.2.1.2.2.1. Messages

3.1.2.1.2.2.2. Probes

3.1.2.1.2.2.3. Reports

3.1.2.1.2.3. Security and management models

3.1.2.1.3. Organization of the AMHS

3.1.2.1.4. AMHS Management Domain configurations

3.1.2.1.4.1. Minimal set of systems

3.1.2.1.4.2. Interconnection between two AMHS Management Domains

3.1.2.1.5. Naming and addressing principles

3.1.2.1.5.1. AMHS Naming and Addressing

3.1.2.1.5.1.1. AMHS O/R Names

3.1.2.1.5.1.2. Structure of a MF-Address

3.1.2.1.5.1.3. AMHS Management Domain identifier

3.1.2.1.5.1.4. AMHS Addressing Schemes

3.1.2.1.5.2. Upper Layer Naming and Addressing

3.1.2.1.5.2.1. Application Process Titles

3.1.2.1.5.2.2. Application Entity Qualifiers

3.1.2.1.5.2.3. Transport, Session and Presentation Addresses

3.1.2.1.6. AMHS Routing and rerouting

3.1.2.1.7. AMHS Traffic logging upon origination

3.1.2.2. ATS Message Service Specification

3.1.2.2.1. ATS Message User Agent Specification

3.1.2.2.1.1. Additional provisions on parameters

3.1.2.2.1.1.1. Message Content Profile Specification

3.1.2.2.1.1.2. Additional requirements upon MT-Elements of Service at an ATS Message User Agent

3.1.2.2.1.2. Traffic logging requirements at an ATS Message User Agent

3.1.2.2.2. ATS Message Server Specification

3.1.2.2.2.1. Profile Specification

3.1.2.2.2.1.1. Upper Layer Requirements

3.1.2.2.2.1.2 Use of the Transport Service

3.1.2.2.2.2. Traffic logging requirements at an ATS Message Server

3.1.2.2.3. Parameters

3.1.2.2.3.1. AMHS Addresses

3.1.2.2.3.2. Text

3.1.2.2.3.2.1. ATS Message Priority

3.1.2.2.3.2.2. ATS Message Filing Time

3.1.2.2.3.2.3. ATS Message Optional Heading Info

3.1.2.2.3.2.4. ATS Message Text

3.1.2.2.3.3. Notification requests

3.1.2.3. AFTN/AMHS Gateway Specification

3.1.2.3.1. General

3.1.2.3.2. AFTN/AMHS Gateway components

3.1.2.3.2.1. AFTN component

3.1.2.3.2.2. ATN Component

3.1.2.3.2.3. Message Transfer and Control Unit

3.1.2.3.2.4. Interface between the ATN Component and the Message Transfer and Control Unit

3.1.2.3.2.5. Interface between the AFTN Component and the Message Transfer and Control Unit

3.1.2.3.2.6. AFTN/AMHS Gateway Control Position

3.1.2.3.3. General functions

3.1.2.3.3.1. Traffic logging

3.1.2.3.3.2. Address look-up tables

3.1.2.3.3.2.1. MD look-up Tables

3.1.2.3.3.2.2. User address look-up Tables

3.1.2.3.4. AFTN to AMHS Conversion

3.1.2.3.4.1. Control function

3.1.2.3.4.2. Conversion of AFTN Messages

3.1.2.3.4.2.1. Use of AFTN Message components

3.1.2.3.4.2.2. Generation of IPM

3.1.2.3.4.2.3. Generation of Message Transfer Envelope

3.1.2.3.4.3. Conversion of AFTN Acknowledgement Messages

3.1.2.3.4.3.1. Initial processing of AFTN Acknowledgement Message

3.1.2.3.4.3.2. Use of AFTN Service Message components

3.1.2.3.4.3.3. Generation of RN

3.1.2.3.4.3.4. Differences in the generation of Message Transfer Envelope

3.1.2.3.4.4. Conversion of AFTN Service Messages related to unknown addressee indicators

3.1.2.3.4.4.1. Initial Processing of the AFTN Service Message

3.1.2.3.5. AMHS to AFTN Conversion

3.1.2.3.5.1. Control Function

3.1.2.3.5.2. AMHS IPM Conversion

3.1.2.3.5.2.1. Initial processing of AMHS Messages

3.1.2.3.5.2.2. Generation of AFTN Message

3.1.2.3.5.2.3. Use of IPM elements

3.1.2.3.5.2.4. Use of Message Transfer Envelope parameters

3.1.2.3.5.3. AMHS RN Conversion

3.1.2.3.5.3.1. Initial processing of AMHS Receipt Notifications

3.1.2.3.5.3.2. Generation of the AFTN acknowledgement message

3.1.2.3.5.3.3. Use of RN fields

3.1.2.3.5.3.4. Use of Message Transfer Envelope parameters conveyed with a RN

3.1.2.3.5.4. AMHS Non-delivery Report Conversion

3.1.2.3.5.4.1. Initial processing of AMHS Non-Delivery Reports

3.1.2.3.5.4.2. Generation of unknown address AFTN service message

3.1.2.3.5.4.3. Use of Report Transfer Envelope and Content parameters

3.1.2.3.5.5. Action upon reception of AMHS Probe

3.1.2.3.5.6. Generation of AMHS Reports

3.1.2.3.5.6.1. General

3.1.2.3.5.6.2. Generation of Report Transfer Envelope and Content

3.1.3. ATN PASS-THROUGH SERVICE

3.1.3.1. System level provisions

3.1.3.1.4. AFTN/ATN Type A Gateway users

3.1.3.1.5 AFTN/ATN Type A Gateway model

3.1.3.1.5.1. AFTN/ATN Type A Gateway information model

3.1.3.1.5.2. Security and managements models

3.1.3.1.6. AFTN/ATN Type A Gateway System configurations

3.1.3.1.7. AFTN/ATN Type A Gateway System naming principles

3.1.3.1.8. AFTN/ATN Type A Gateway System addressing principles

3.1.3.1.9. Routing principles

3.1.3.1.10. Processing of communication failure

3.1.3.2. ATN Pass-Through Service Specification

3.1.3.3. AFTN/ATN Type A Gateway Specification

3.1.3.3.1. AFTN component

3.1.3.3.2. ATN component

3.1.3.3.2.2. ATN Component service

3.1.3.3.2.4. ATN Component Control Function

3.1.3.3.2.5. Dialogue Service QoS (Priority)

3.1.3.3.3. Message Transfer and Control Unit Component

3.1.3.3.3.1. General functions

3.1.3.3.3.2. Determination of gateway address

3.1.3.3.3.3. AFTN to ATN mapping

3.1.3.3.3.4. ATN to AFTN mapping

3.1.3.3.3.5. Interface between the ATN Component and the Message Control Unit Component

3.1.3.3.3.6. Interface between the AFTN Component and the Message Control Unit Component

# 3.1.  ATS MESSAGE HANDLING SERVICES (ATSMHS)

# 3.1.1.  Introduction

The ATS (Air Traffic Services) Message Handling Services (ATSMHS) applications allow ATS Messages to be exchanged between service users. The ATS Message Handling Services are the ATS Message Service and the ATN Pass-Through Service.

Note 1.—  These ATS Message Handling Services aim at providing generic message services over the Aeronautical Telecommunication Network (ATN) Internet. They may in turn be used as a communication system by user-applications communicating over the ATN. This may be achieved e.g. by means of application program interfaces to either the ATS Message Service or to the ATN Pass-Through Service.

Note 2.-  ATS Message Service

a)	The ATS Message Service is provided by the implementation over the ATN Internet Communication Services of the Message Handling Systems specified in ISO/IEC (International Organization for Standardization/ International Electrotechnical Commission) 10021 and CCITT (Consultative Committee of International Telegraph and Telephone) or ITU-T (International Telecommunication Union - Telecommunications Standards) X.400, and complemented with the additional requirements specified in 3.1. The two sets of documents, the ISO/IEC MOTIS (Message-Oriented Text Interchange System) International Standards and the CCITT X.400 Series of Recommendations (1988 or later) are in principle aligned to each other. However there are a small number of differences. In 3.1 reference is made to the relevant ISO International Standards, and International Standardized Profiles (ISP) where applicable. Where necessary, e.g. for reasons of interworking or to point out differences, reference is also made to the relevant X.400 Recommendations.

b)	Two levels of service are intended to be defined within the ATS Message Service:

i)	the Basic ATS Message Service.

ii)	the Extended ATS Message Service.

c)	This specification of the ATS Message Service supports only the Basic ATS Message Service. The Extended ATS Message Service could be incorporated in future packages.

Note 3.-  The ATN Pass-Through Service is the ATS Message Handling Service offered over the ATN Internet Communication Services by the use of the Dialogue Service and of the associated ATN upper layer architecture to exchange AFTN (Aeronautical Fixed Telecommunication Network) Messages formatted in IA-5 (International Alphabet No 5) in compliance with the provisions of Annex 10, Volume II.

Note 4.-  End systems performing ATS Message Handling Services

a)	Four types of ATN End Systems are defined in 3.1:

1)	an ATS Message Server,

2)	an ATS Message User Agent,

3)	an AFTN/AMHS Gateway (Aeronautical Fixed Telecommunication Network / ATS Message Handling System), and

4)	an AFTN/ATN Type A Gateway.

b)	Connections may be established over the Internet Communications Service between any pair constituted of these ATN End Systems and listed in Table 3.1.1-1. Although included in table 3.1.1-1, the communication between an ATS Message Server and an ATS Message User Agent is not specified in 3.1.

Table 3.1.1-1.   Communications between ATN End Systems implementing ATS Message Handling Services

| ATN End System 1 | ATN End System 2 |
| --- | --- |
| ATS Message Server | ATS Message Server |
| ATS Message Server | AFTN/AMHS Gateway |
| ATS Message Server | ATS Message User Agent |
| AFTN/AMHS Gateway | AFTN/AMHS Gateway |
| AFTN/ATN Type A Gateway | AFTN/ATN Type A Gateway |

Note 5.-  Structure of 3.1

a)	3.1.1: INTRODUCTION contains the purpose and structure, and a summary of the functionalities offered by the ATS Message Handling Services.

b)	3.1.2: ATS MESSAGE SERVICE contains three sections as follows:

1)	3.1.2.1: System Level Provisions, provides a high level specification of the application and of the environment in which it operates;

2)	3.1.2.2: ATS Message Service Specification, provides the detailed specification of the service and protocol requirements for each type of ATN End System (ATS Message User Agent and ATS Message Server) implementing the ATS Message Service;

3)	3.1.2.3: AFTN/AMHS Gateway Specification, provides the detailed specification of an AFTN/AMHS Gateway and of the related functional requirements such as conversion.

c)	3.1.3: ATN PASS-THROUGH SERVICE contains three sections as follows:

1)	3.1.3.1: System Level Provisions, provides a high level specification of the application and of the environment in which it operates;

2)	3.1.3.2: ATN Pass-Through Service Specification, provides the detailed specification of the protocol requirements between two AFTN/ATN Type A Gateways implementing the ATN Pass-Through Service;

3)	3.1.3.3: AFTN/ATN Type A Gateway Specification, provides the detailed specification of an AFTN/ATN Type A Gateway and of the related functional requirements.

Note 6.-  The following terminology applies in 3.1:

a)	AFTN acknowledgement message: an AFTN service message acknowledging the receipt of an AFTN message which priority indicator has the value "SS".

b)	direct AMHS user: an ATS Message Service user who engages in the ATS Message Service at an ATS Message User Agent. A direct AMHS user may belong to two subgroups as follows:

1)	human users who interact with the ATS Message Service by means of an ATS Message User Agent connected to an ATS Message Server; and

2)	host users which are computer applications running on ATN end systems and interacting with the ATS Message Service by means of application programme interfaces.

c)	indirect AMHS user: an ATS Message Service user at an AFTN station using an AFTN/AMHS Gateway to communicate with other ATS Message Service users.

d)	subject AFTN message: an AFTN message which causes an AFTN service message or an AMHS report to be generated.

e)	subject AMHS message: an AMHS message which causes an AFTN service message or an AMHS report to be generated.

f)	subject IPM: the IPM which is the content of an AMHS message and which causes an AMHS Receipt Notification to be generated.

g)	unknown address AFTN service message: an AFTN service message requesting correction by the originator of a message received with an unknown addressee indicator.

Note 7.-  The classifications defined in the ISPs apply for expressing conformance requirements - i.e. static capability - in 3.1. The ISP classifications refine the ISO/IEC 9646-7 classification to include different levels of mandatory support, depending on the level of functionality to be supported by the considered Message Handling System. These classifications include the following elements, of which the complete definition may be found in each referenced ISP:

a)	mandatory full support (M).

b)	mandatory minimal support (M-).

c)	mandatory O/R name minimal support (M1) (see ISO/IEC ISP 12062-2).

d)	optional support (O).

e)	conditional support (C).

f)	out of scope (I).

g)	not applicable (-).

Note 8.-  The following classification applies for expressing dynamic behaviour requirements - i.e. the action performed by the ATN end system - related to parameters or elements in the Profile Requirement Lists (PRLs) included in 3.1.2.3, for the specification of the AFTN/AMHS Gateway:

a)	generated (G): used to describe the generation of an AMHS or AFTN information object. It means that the element is generated by the AFTN/AMHS Gateway, and that its value does not depend on the value of an element of the information object received by the AFTN/AMHS Gateway which caused the current generation of an information object, but that the value of the element is based on parameters related to the AFTN/AMHS Gateway itself or takes a pre-determined value. If an element comprises several components, then the element is classified as generated if at least one of its components is generated, and the others are either generated or excluded;

b)	optionally generated (G1): used with the same meaning as "generated", with the exception that the generation of the element is optional, the decision being a matter of policy local to the Management Domain operating the AFTN/AMHS Gateway;

c)	conditionally generated (G2): used only to describe the generation of an AMHS report or RN (Receipt Notification) element. It means, for a report generation, that the element is generated in the report or RN based on some condition related to the subject AMHS message being true. If the element is generated, it takes a value derived from elements present in the received AMHS information object which caused the generation of the report or RN;

d)	translated (T): used to describe either the generation of an AMHS or AFTN information object or the use of a received information object. It means that the element is translated by the AFTN/AMHS Gateway, using a dependence relationship between the value of an element of the received information object and the value of the translated element in the generated information object. If an element comprises several components, then the element is classified as translated if at least one of its components is translated, and the others are either generated or excluded in generation, discarded or out of scope in reception;

e)	conditionally translated (T1): used with the same meaning as "translated", with the exception that the translation of the element is subject to some condition being true, e.g. the presence of an optional element in the received information object;

f)	discarded (D): used to describe the use of a received AMHS or AFTN information object. It means that the value of the element is not used by the Message Transfer and Control Unit when generating the elements of the information object converted from the received information object, and that the semantic information conveyed in the element is discarded during the process of conversion in the AFTN/AMHS Gateway. However the presence or value of the element may be used by the Message Transfer and Control Unit for purposes other than conversion, such as report generation and logging;

g)	excluded (X): used to describe either the generation of an AMHS or AFTN information object or the use of a received information object. Upon generation of an information object, it means that the element is not used nor present in the generated information object. Upon reception of an AMHS information object, it means that the presence of the element causes rejection of the information object, and generation of an AMHS non-delivery report as appropriate;

h)	out of scope or not-applicable (-): used to describe the use of a received information object, when the element is either a format element which cannot be processed in any way or an element which is not in the scope of the section, but which presence is included in the ISPICS (ISP Implementation Conformance Statement) serving as a basis for the mapping specification.

Note 9.-  Application Functionalities

a)	The Basic ATS Message Service meets the basic requirements of the Message Handling Systems Profiles published by ISO/IEC as ISPs (International Standardized Profiles), and it incorporates additional features to support the service offered by the AFTN. The Basic ATS Message Service is further specified in 3.1.2.2. This includes the specification of which ISPs apply in this context.

b)	The ATN Pass-Through Service encapsulates and decapsulates AFTN messages at an AFTN/ATN type A Gateway, using the Dialogue Service and the associated upper layer protocol architecture. The ATN Pass-Through Service is further specified in 3.1.3.2.

Note 10.-  Applicability

a)	The implementation of the ATS Message Service is mandatory for conformance with 3.1. However, as a matter of organisations’ policy, interim conformance may be achieved with the implementation of the ATN Pass-Through Service. The choice to implement the ATN Pass-Through Service as an interim solution does not replace the requirement to implement the ATS Message Service at the earliest possible date.

b)	The interoperability between the ATS Message service and the ATN Pass-Through Service is a local implementation matter, provided that such an implementation has an external behaviour identical to that of an AFTN/AMHS Gateway and of an AFTN/ATN Type A Gateway, as appropriate. The choice to implement the ATN Pass-Through Service implies the requirement to provide the interoperability facilities to the ATS Message Service implementations.

# 3.1.2.  ATS Message Service

## 3.1.2.1.	System level provisions

The ATS Message Service shall be implemented for conformance with 3.1.

### 3.1.2.1.1.	ATS Message Service Users

3.1.2.1.1.1.  Direct AMHS users shall use the Basic ATS Message Service at an ATS Message User Agent.

3.1.2.1.1.2.  Indirect AMHS users shall use only that part of the Basic ATS Message Service which corresponds to AFTN functionalities, by using the interworking capability provided by an AFTN/AMHS Gateway as specified in 3.1.2.3.

### 3.1.2.1.2.	AMHS Model

#### 3.1.2.1.2.1.	AMHS functional model

##### 3.1.2.1.2.1.1.	Model components

The systems comprising the AMHS shall themselves be comprised of the following functional objects, the general role of which is described in ISO/IEC 10021-2:

a)	message transfer agent(s) (MTA),

b)	user agent(s) (UA),

c)	message store(s) (MS), and

d)	access unit(s) (AU).

Note.-  The ISO/IEC 10021 Elements of Service and Protocols used by these functional objects are specified in 3.1.2.2 and 3.1.2.3.

##### 3.1.2.1.2.1.2.	ATS Message Server

An ATS Message Server shall include a MTA and optionally one or several MSs, as specified in 3.1.2.2.2.

##### 3.1.2.1.2.1.3.	ATS Message User Agent

An ATS Message User Agent shall include a UA as specified in 3.1.2.2.1.

##### 3.1.2.1.2.1.4.	AFTN/AMHS Gateway

An AFTN/AMHS Gateway shall include a MTA, which is part of the ATN Component of the AFTN/AMHS Gateway, and an AU, as specified in 3.1.2.3.

Note.-  The AU is the Message Transfer and Control Unit of the AFTN/AMHS Gateway.

#### 3.1.2.1.2.2.	AMHS information model

The following three categories of AMHS information objects shall be used:

a)	messages;

b)	probes; and

c)	reports.

##### 3.1.2.1.2.2.1.	Messages

Note.-  The provisions in 3.1.2 concerning ISO/IEC 10021 envelopes apply to Transfer Envelopes only .

In the Basic ATS Message Service, each AMHS message shall correspond unequivocally to an ATS Message.

##### 3.1.2.1.2.2.2.	Probes

Only direct AMHS users shall be able to submit AMHS probes.

##### 3.1.2.1.2.2.3.	Reports

AMHS reports shall be delivered only to direct AMHS users.

#### 3.1.2.1.2.3.	Security and management models

Recommendation.-  In the Basic ATS Message Service, security should be obtained by procedural means rather than by technical features inherent to the AMHS.

Note 1.-  In the Basic ATS Message Service, the security at each ATS Message Server or AFTN/AMHS Gateway is deemed a local issue to be addressed by the authority in charge of the system.

Note 2.-  In the Basic ATS Message Service, management is limited to the logging provisions which are defined for the ATS Message User Agent, for the ATS Message Server and for the AFTN/AMHS Gateway. No provision is made for retrieval or exchange of this information, which is deemed a local issue to be addressed by the authority in charge of the system.

### 3.1.2.1.3.	Organization of the AMHS

The AMHS shall be organisationally composed of AMHS Management Domains.

Note 1.-  An AMHS Management Domain may elect to operate as either an ADMD (Administration Management Domain) or a PRMD (Private Management Domain), depending on the national telecommunications regulation in force in the country(ies) where it operates and on its relationships with other Management Domains.

Note 2.-  A PRMD which is subordinate to one or several AMHS ADMDs may qualify as AMHS Management Domain if it satisfies the provisions of 3.1.2.

### 3.1.2.1.4.	AMHS Management Domain configurations

#### 3.1.2.1.4.1.	Minimal set of systems

The minimal set of systems implemented and operated by an AMHS Management Domain shall be one of the following:

a)	an ATS Message Server and one or several ATS Message User Agents;

b)	an AFTN/AMHS Gateway; or

c)	any combination of a) and b).

#### 3.1.2.1.4.2.	Interconnection between two AMHS Management Domains

An interconnection between two AMHS Management Domains shall be implemented as one of the following:

a)	a connection between two ATS Message Servers;

b)	a connection between an ATS Message Server and an AFTN/AMHS Gateway; or

c)	a connection between two AFTN/AMHS Gateways.

### 3.1.2.1.5.	Naming and addressing principles

#### 3.1.2.1.5.1.	AMHS Naming and Addressing

##### 3.1.2.1.5.1.1.	AMHS O/R Names

For the support of the Basic ATS Message Service, the O/R (originator/recipient) name of an AMHS user shall comprise:

a)	the O/R address of the AMHS user, called an MF-address; and

b)	optionally the directory name of the AMHS user, if the policy of the AMHS Management Domain, to which the AMHS user belongs, includes the local support of directory-names.

Note.-  As a matter of policy local to an AMHS Management Domain, the directory name component of an O/R name may be used by the implementation of the Optional DIR (Use of Directory) FG (Functional Group).

##### 3.1.2.1.5.1.2.	Structure of a MF-Address

The MF-Address (MHS-form address) of an AMHS user shall comprise:

a)	a set of attributes as specified in 3.1.2.1.5.1.3, identifying the AMHS Management Domain of which the AMHS user, either direct or indirect, is a service-user; and

b)	a set of attributes as specified in 3.1.2.1.5.1.4, identifying uniquely the AMHS user within the AMHS Management Domain, in compliance with the AMHS addressing scheme implemented by the AMHS Management Domain.

Note.-  The attributes present in the identifier defined in item b) may include any standard or domain-defined attribute as specified in section 18 of ISO/IEC 10021-2, other than country-name, administration-domain-name and private-domain-name.

##### 3.1.2.1.5.1.3.	AMHS Management Domain identifier

The attributes identifying an AMHS Management Domain shall include the following standard attributes as specified in ISO/IEC 10021-2, section 18.3, depending on the status under which the AMHS Management Domain has elected to operate:

a)	country-name,

b)	administration-domain-name,

c)	private-domain-name, if the AMHS Management Domain has elected to operate as a PRMD.

##### 3.1.2.1.5.1.4.	AMHS Addressing Schemes

###### 3.1.2.1.5.1.4.1.	General provisions

Note 1.-  It is a matter of policy local to each AMHS Management Domain to implement either a locally defined AMHS Addressing Scheme, or a Common AMHS Addressing Scheme, or a combination of these. The single Common ICAO AMHS Addressing Scheme defined in the present version of this document  is the XF-Addressing Scheme. Aeronautical Industry X.400 Addressing Schemes are defined in appropriate Aeronautical Industry Standards.

Note 2.-  Each AMHS Addressing Scheme includes the set of attributes identifying the AMHS Management Domain as specified in 3.1.2.1.5.1.3.

###### 3.1.2.1.5.1.4.2.	XF-Addressing Scheme

The XF-Address (translated address) of a direct or indirect AMHS user shall be composed exclusively of the following:

a)	an AMHS Management Domain identifier as specified in 3.1.2.1.5.1.3;

b)	an organization-name attribute:

1)	as specified in ISO/IEC 10021-2, Section 18.5,

2)	taking the 4-character value “AFTN”, and

3)	encoded as a Printable String; and

c)	an organizational-unit-names attribute:

1)	as specified in ISO/IEC 10021-2, Section 18.5,

2)	comprising a sequence of one single element, which takes the 8-character alphabetical value of the AF-Address (AFTN-form address) of the user, and

3)	encoded as a Printable String.

Note 1.-  An XF-Address is a particular MF-Address of which the attributes identifying the user within an AMHS Management Domain (i.e. those attributes other than country-name, administration-domain-name and private-domain-name) may be converted by an algorithmic method to and from an AF-Address. The algorithmic method requires the additional use of look-up tables which are limited, i.e. which include only a list of AMHS Management Domains rather than a list of individual users, to determine the full MF-address of the user.

Note 2.-  No distinction is made between upper case and lower case.

#### 3.1.2.1.5.2.	Upper Layer Naming and Addressing

##### 3.1.2.1.5.2.1.	Application Process Titles

3.1.2.1.5.2.1.1.  Recommendation.-  The Application Process Title of an ATS Message Server should be as specified in 4.3.3.2.

3.1.2.1.5.2.1.2.  Recommendation.-  The Application Process Title of an AFTN/AMHS Gateway should be as specified in 4.3.3.2.

3.1.2.1.5.2.1.3.  Recommendation.-  The Application Process Title of an ATS Message User Agent should be as specified in 4.3.3.2.

##### 3.1.2.1.5.2.2.	Application Entity Qualifiers

3.1.2.1.5.2.2.1.  Recommendation.-  The Application Entity Qualifier of an ATS Message Server should be AMS (7).

3.1.2.1.5.2.2.2.  Recommendation.-  The Application Entity Qualifier of an AFTN/AMHS Gateway should be GWB (8).

3.1.2.1.5.2.2.3.  Recommendation.-  The Application Entity Qualifier of an ATS Message User Agent should be AUA (9).

##### 3.1.2.1.5.2.3.	Transport, Session and Presentation Addresses

The TSAP (Transport Service Access Point) of an ATS Message Server or of an ATS Message User Agent shall comply with the provisions of 5.5.4.

Note 1.-  The assignment of a transport selector value is a matter local to an AMHS Management Domain.

Note 2.-  The format and encoding of a session selector in the AMHS is specified in ISO/IEC ISP 11188-1, section 9.3.

Note 3.-  The assignment and administration of session selectors is a matter local to an AMHS Management Domain.

Note 4.-  The format and encoding of a presentation selector in the AMHS is specified in ISO/IEC ISP 11188-1, section 7.2.

Note 5.-  The assignment and administration of presentation selectors is a matter local to an AMHS Management Domain.

### 3.1.2.1.6.	AMHS Routing and rerouting

3.1.2.1.6.1.  The definition of AMHS routing shall be subject to multilateral agreements.

3.1.2.1.6.2.  The MTAs implemented by an AMHS Management Domain shall be collectively able to route on country-name, ADMD-name, PRMD-name, organization-name and organizational-units-name attributes.

### 3.1.2.1.7.	AMHS Traffic logging upon origination

An AMHS Management Domain shall be responsible for long-term logging of all messages in their entirety which are originated by its direct AMHS users, for a period of at least thirty days.

Note.-  This requirement implies the logging of the entire BER-encoded ASN.1 messages.

## 3.1.2.2.	ATS Message Service Specification

### 3.1.2.2.1.	ATS Message User Agent Specification

Note.-  For the support of the Basic ATS Message Service, an ATS Message User Agent complies with :

a)	profile AMH21 as specified in ISO/IEC ISP 12062-2;

b)	the requirements of Repertoire Group A, for messages including a body part whose type is an Extended Body Part Type of general-text-body-part type;

c)	the additional provisions relating to parameters generated at an ATS Message User Agent, as specified in 3.1.2.2.1.1; and

d)	the provisions related to traffic logging as specified in 3.1.2.2.1.2.

#### 3.1.2.2.1.1.	Additional provisions on parameters

##### 3.1.2.2.1.1.1.	Message Content Profile Specification

In an ATS Message User Agent, the content of the Inter-Personal Messages conveyed in support of the Basic ATS Message Service shall conform to the basic requirements of AMH21 as specified in Clause A.1 of ISO/IEC ISP 12062-2, Annex A and to the additional requirements described in Table 3.1.2-1 which are specific to the Basic ATS Message Service.

Note 1.-  Table 3.1.2-1 specifies the additional requirements in the form of a PRL (Profile Requirement List) expressing restrictions to a set of rows of the AMH21 profile, which are referred to using their reference in ISO/IEC ISP 12062-2.

Note 2.-  There is no profile specification for the ATS Message User Agent at the level of the access protocol, i.e. at the level of the communication with the associated ATS Message Server, as this is considered to be a matter local to each AMHS Management Domain. If it is desired to use standard ISO/IEC 10021 protocols for this communication, then profile AMH23 (for P3) or profile AMH24 (for P7) as specified in ISO/IEC ISP 12062-4 or ISO/IEC ISP 12062-5, respectively, may be implemented.

Note 3.-  The use of the ia5-text body part as specified in Table 3.1.2-1/AMH21/A1.3/1 ensures operability with both 1984 and 1988 IPM (Inter-Personal Message) UAs for the exchange of unstructured character data.

Table 3.1.2-1.   Requirements specific to the Basic ATS Message Service in addition to profile AMH21

| Ref | Element | Origination |  |  | Reception |  |  |  | Basic ATS Message Service Support |  |  | ATN reference |  |  | ISP 12062-2 Notes/References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP |  | Base | ISP |  |  |  |  |  |  |  |  |  |
| Part 1 : AMH21/A.1.3 IPM body |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | ia5-text | O | O |  | O | M |  | O/M |  |  |  |  |  |  |  |
| 1.2 | data | M | M |  | M | M |  | M/M |  |  | 3.1.2.2.3.2 |  |  |  |  |
| Part 2 : AMH21/A.1.3.1 Extended body part support |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | ia5-text-body-part | O | O |  | O | M |  |  | O/M |  |  |  |  |  | see AMH21/ A.1.3/1 |
| 11 | general-text-body-part | O | M |  | O | M |  |  | M/M |  |  | 3.1.2.2.3.2 and Table 3.1.2-1 Part 4 |  |  |  |
| Part 3 : AMH21/A.1.5 Common data types |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ref | Element | Origination |  |  | Reception |  |  |  | Basic ATS Message Service Support |  |  | ATN reference |  |  | ISP 12062-2 Notes/References |
|  |  | Base | ISP |  | Base | ISP |  |  |  |  |  |  |  |  |  |
| 1 | RecipientSpecifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.2 | notification-requests | O | O |  | M | M |  |  | M/M |  |  | 3.1.2.2.3.3 |  |  |  |
| 1.2.1 | rn | O | O |  | O | O |  |  | M/M |  |  | 3.1.2.2.3.3 |  |  |  |
| 1.2.2 | nrn | O | O |  | M | M |  |  | M/M |  |  |  |  |  |  |
| 2 | ORDescriptor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | formal-name | M | M1 |  | M | M1 |  |  | M1/M1 |  |  | 3.1.2.2.3.1 |  |  |  |

| Ref | Element | Origination |  | Reception |  |  | Basic ATS Message Service Support |  | ATN reference |  | ISP 12062-2 Notes/References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Base | ISP |  |  |  |  |  |  |
| Part 4 : AMH21/A.1.3.2 General text repertoire support |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Basic (ISO 646) (repertoire identifiers {1, 6}) | M | M | M | M |  | M/M |  |  |  | Repertoire Group A |
| 2 | Basic-1 (ISO 8859-1) (repertoire identifiers {1, 6, 100}) | O | M | O | M |  | O/O |  |  |  | Repertoire Group B |

Legend : see 3.1.1
M =	mandatory support
M1 =	mandatory O/R name minimal support
O =	optional support

##### 3.1.2.2.1.1.2.	Additional requirements upon MT-Elements of Service at an ATS Message User Agent

For the support of the Basic ATS Message Service, the priority element of an AMHS Message generated at an ATS Message User Agent shall take the value "urgent" if, and only if, the value of the priority-indicator in the ATS-Message-Priority as specified in 3.1.2.2.3.2.1 is "SS".

### 3.1.2.2.1.2.	Traffic logging requirements at an ATS Message User Agent

Note.-  The requirement expressed in 3.1.2.1.7 may be implemented in the ATS Message User Agent.

### 3.1.2.2.2.	ATS Message Server Specification

Note.-  For the support of the Basic ATS Message Service, an ATS Message Server complies with:

a)	the profile specification expressed in 3.1.2.2.2.1; and

b)	the provisions related to traffic logging as specified in 3.1.2.2.2.2.

#### 3.1.2.2.2.1.	Profile Specification

##### 3.1.2.2.2.1.1.	Upper Layer Requirements

In an ATS Message Server, the Message Transfer (P1) implementation of the IPM Service in support of the Basic ATS Message Service shall conform to:

a)	the basic requirements of AMH22 as specified in Clause B.1 of ISO/IEC ISP 12062-3, Annex B; and

b)	the additional requirements described in Clause B.2.2. for the support of the IPM Distribution List Functional Group.

Note 1.-  This in turn places no requirements concerning the P1 implementation other than:

a)	the basic requirements of AMH11 specified for Common Messaging in annex A.1 of ISO/IEC ISP 10611-3, implying the mandatory support of the AMH111 Profile implementing the mts-transfer application context; and

b)	the additional requirements specified for the Common Messaging DL (Distribution List) Functional Group in annex A.2.2 of ISO/IEC ISP 10611-3.

Note 2.-  As a consequence of Note 2 in 3.1.2.2.1.1.1, the optional implementation of Message Stores (MS) in an ATS Message Server, being related to the access protocol from an ATS Message User Agent to an ATS Message Server, is a matter local to each AMHS Management Domain.

Note 3.-  The additional support by an ATS Message Server of the AMH112 Profile as specified in ISO/IEC ISP 10611-3, for conformance to CCITT X.400 in order to interconnect with public ADMDs is a matter of policy local to each AMHS Management Domain.

Note 4.-  For the use of the Association Control Service Element (ACSE) by an AMHS application, the application-context name which is used as a parameter in an A-ASSOCIATE is defined in the base standards (see ISO/IEC 10021-6).

Note 5.-  The specification in 3.1.2.2.2.1.1 places no requirements for the Reliable Transfer Service Element (RTSE) and for ACSE other than conformance with ISO/IEC ISP 10611-2 in accordance with the P1 application-context(s) for which conformance is claimed.

Note 6.-  The specification in 3.1.2.2.2.1.1 places no requirements for the Presentation and Session Layers other than conformance with ISO/IEC ISP 10611-2 in accordance with the P1 application-context(s) for which conformance is claimed.

##### 3.1.2.2.2.1.2	Use of the Transport Service

3.1.2.2.2.1.2.1.  The Basic ATS Message Service shall make use of the Connection Mode Transport Service as specified in 5.5.5.

Note.-  For the support of the Basic ATS Message Service, the use of the expedited data option at the establishment of the transport connection is a local matter which may depend on the implemented application-context.

3.1.2.2.2.1.2.2.  For the support of the Basic ATS Message Service, transport connections shall be established over the ATN Transport Service between systems belonging to the AMHS using the Residual Error Rate (RER) abstract-value "high".

3.1.2.2.2.1.2.3.  For the support of the Basic ATS Message Service, transport connections shall be established over the ATN Transport Service between systems belonging to the AMHS using the Transport Connection Priority  abstract-value "6", which corresponds to the message category "flight regularity communications".

3.1.2.2.2.1.2.4.  For the support of the Basic ATS Message Service, transport connections shall be established over the ATN Transport Service between systems belonging to the AMHS using the value of the ATN Security Label as specified in 5.5.6, which corresponds to:

a)	the ATN Traffic Type "ATN Operational Communications";

b)	the Sub-Type "Air Traffic Services Communications" (ATSC); and

c)	"No Traffic Type Policy Preference".

### 3.1.2.2.2.2.	Traffic logging requirements at an ATS Message Server

3.1.2.2.2.2.1.  The ATS Message Server shall perform a long-term logging, for a period of at least thirty days, of the actions taken with respect to every message received at the ATS Message Server, whether from an ATS Message User Agent or from another ATS Message Server, and to every report received or generated at the ATS Message Server.

3.1.2.2.2.2.2.  For the long-term logging of information related to a message submitted to or received by an ATS Message Server, the following parameters related to the message shall be logged:

a)	message-identifier;

b)	priority;

c)	content-type;

d)	originator-name;

e)	recipient-name elements on responsibility list;

f)	message-content-size;

g)	last element of the trace-information (if any);

h)	arrival-time or submission-time;

i)	transfer destination (if any);

j)	transfer time (if any);

k)	this-recipient-name (if message delivery is performed by the ATS Message Server);

l)	delivery-time (if any);

m)	delivery and/or non-delivery reports generated (if any); and

n)	event date/time.

Note.-  The responsibility list identifies recipients whose perRecipientIndicator responsibility bit has the abstract-value "responsible".

3.1.2.2.2.2.3.  For the long-term logging of information related to a report generated or received by an ATS Message Server, the following parameters related to the report shall be logged:

a)	report-identifier;

b)	subject-identifier;

c)	actual-recipient-name elements;

d)	report-type elements;

e)	report-destination-name;

f)	last element of the trace-information (if any);

g)	arrival-time in the ATS Message Server or generation time;

h)	transfer destination (if any);

i)	transfer time (if any);

j)	OR-name of the report recipient (if report delivery is performed by the ATS Message Server);

k)	delivery-time (if any); and

l)	event date/time.

### 3.1.2.2.3.	Parameters

#### 3.1.2.2.3.1.	AMHS Addresses

In the AMHS, the O/R address of a direct AMHS user belonging to an AMHS Management Domain shall be a MF-Address.

#### 3.1.2.2.3.2.	Text

The body of an Inter-Personal Message (IPM) shall comprise a single body part carrying IA-5 characters and structured as depicted in Table 3.1.2-2.

Note 1.-  This body part structure and its components which are described in the subsequent clauses are specific to the Basic ATS Message Service.

Note 2.-  This clause places no constraint on its implementation, which may take place at the level of the user-interface.

Table 3.1.2-2.   Structure of an IPM in the Basic ATS Message Service

| Ref | Element | Basic ATS Message Service Support |  | Value | IA-5 Encoding |
| --- | --- | --- | --- | --- | --- |
|  |  | Orig | Rec |  |  |
| 1 | ATS-Message-Header | M | M |  |  |
| 1.1 | start-of-heading | M | M | (SOH) | (0/1) |
| 1.2 | ATS-Message-Priority | M | M |  |  |
| 1.2.1 | priority-prompt | M | M | PRI:(single space) | (5/0)(5/2)(4/9)(3/10)(2/0) |
| 1.2.2 | priority-indicator | M | M | see 3.1.2.2.3.2.1 | see 3.1.2.2.3.2.1 |
| 1.2.3 | priority-separator | M | M | (CR)(LF) | (0/13)(0/10) |
| 1.3 | ATS-Message-Filing-Time | M | M |  |  |
| 1.3.1 | filing-time-prompt | M | M | FT:(single space) | (4/6)(5/4)(3/10)(2/0) |
| 1.3.2 | filing-time | M | M | see 3.1.2.2.3.2.2 | see 3.1.2.2.3.2.2 |
| 1.3.3 | filing-time-separator | M | M | (CR)(LF) | (0/13)(0/10) |
| 1.4 | ATS-Message-Optional-Heading-Info | O | M |  |  |
| 1.4.1 | OHI-prompt | M | M | OHI:(single space) | (4/15)(4/8)(4/9)(3/10)(2/0) |
| 1.4.2 | optional-heading-information | M | M | see 3.1.2.2.3.2.3 | see 3.1.2.2.3.2.3 |
| 1.4.3 | OHI-separator | M | M | (CR)(LF) | (0/13)(0/10) |
| 1.5 | end-of-heading-blank-line | M | M | (LF) | (0/10) |
| 1.6 | start-of-text | M | M | (STX) | (0/2) |
| 2 | ATS-Message-Text | M | M | see 3.1.2.2.3.2.4 | see 3.1.2.2.3.2.4 |

Legend (see 3.1.1) :
M =	mandatory support
O =	optional support

##### 3.1.2.2.3.2.1.	ATS Message Priority

Each message shall be assigned to one of five priority groups which are designated, and have the value of, the priority indicators SS, DD, FF, GG and KK.

##### 3.1.2.2.3.2.2.	ATS Message Filing Time

Each message shall include a filing-time element, designated as a date-time group consisting of six numerical characters, the first two digits representing the date of the month and the last four digits the hours and minutes in UTC.

##### 3.1.2.2.3.2.3.	ATS Message Optional Heading Info

3.1.2.2.3.2.3.1.  It shall be possible to associate an optional heading information with each message.

3.1.2.2.3.2.3.2.  The value of the optional-heading-information element shall comprise a character string with a maximum length of 54 characters.

##### 3.1.2.2.3.2.4.	ATS Message Text

The ATS-Message-Text element shall be composed of IA-5 characters with no further restriction.

#### 3.1.2.2.3.3.	Notification requests

The notification-requests element in a RecipientSpecifier in an IPM Heading shall take the abstract-value "rn" if, and only if, the value of the priority-indicator is "SS".

Note.-  This clause places no constraint on its implementation, which takes place at the level of the user-interface.

## 3.1.2.3.	AFTN/AMHS Gateway Specification

### 3.1.2.3.1.	General

3.1.2.3.1.1.  An AFTN/AMHS Gateway shall provide for an interworking between the AFTN and the ATN such that communication with other AFTN/AMHS Gateways and with ATS Message Servers is possible.

3.1.2.3.1.2.  An AFTN/AMHS Gateway shall consist of the four following logical components:

a)	AFTN Component;

b)	ATN Component;

c)	Message Transfer and Control Unit; and

d)	Control Position.

Note.– This division into logical components is a convenient way of specifying functions of a gateway. There is no requirement for an AFTN/AMHS Gateway to be implemented according to this structure.

3.1.2.3.1.3.  An AFTN/AMHS Gateway shall be able to perform actions upon receipt of any category of AMHS information object by its ATN Component.

3.1.2.3.1.4.  An AFTN/AMHS Gateway shall be able to perform actions upon receipt of any type of AFTN message by its AFTN Component.

### 3.1.2.3.2.	AFTN/AMHS Gateway components

#### 3.1.2.3.2.1.	AFTN component

3.1.2.3.2.1.1.  The AFTN component shall handle the interface to the AFTN and provide an interface to the Message Transfer and Control Unit, implementing:

a)	all the applicable requirements of Annex 10, Volume II in a manner so as to be indistinguishable from an operational AFTN station by the AFTN centre to which the gateway is connected; and

b)	additional requirements which are necessary due to the AFTN Component pertaining to an AFTN/AMHS Gateway.

3.1.2.3.2.1.2.  If an AFTN/AMHS Gateway is connected to an AFTN centre which is capable of using only ITA-2 (International Telegraph Alphabet No 2) format, the AFTN component shall convert messages to/from the IA-5 format.

Note.-  This allows the Message Transfer and Control Unit to use IA-5 characters internally, as specified in 3.1.2.3.2.3.2.

3.1.2.3.2.1.3.  The AFTN Component shall incorporate an AFTN procedure handler providing for all AFTN functions prescribed for the interface to the AFTN.

3.1.2.3.2.1.4.  When received by the AFTN Component, AFTN service messages as generally specified in Annex 10, Volume II, 4.4.1.1.9 and subclauses, shall be handled by the AFTN Component of the Gateway in one of four mutually exclusive manners, depending on the category of the service message:

a)	transfer to the Message Transfer and Control Unit to be processed as specified in 3.1.2.3.4 if the service message is an AFTN acknowledgement message, as specified in Annex 10, Volume II, 4.4.10.1.6.1 and 4.4.15.6;

b)	transfer to the Message Transfer and Control Unit to be processed as specified in 3.1.2.3.4 if the service message is an AFTN service message requesting correction of a message received with an unknown addressee indicator as specified in Annex 10, Volume II, 4.4.11.13.3;

c)	processing as specified in 3.1.2.3.2.1.12 if the service message is an AFTN service message requesting from the originator repetition of an incorrectly received message when it is detected that a message has been mutilated, as specified in Annex 10, Volume II, 4.4.11.1 and 4.4.16.2.2; or

d)	processing in compliance with the provisions of Annex 10, Volume II, without being passed to the Message Transfer and Control Unit, if the service message belongs to any other category of AFTN service message.

3.1.2.3.2.1.5.  When received by an AFTN/AMHS Gateway, AFTN channel-check transmissions as specified in Annex 10, Volume II, 4.4.9.3 and 4.4.15.5 shall:

a)	be handled by the AFTN Component in compliance with the provisions of Annex 10, Volume II; and

b)	be prevented from being passed to the Message Transfer and Control Unit.

3.1.2.3.2.1.6.  The AFTN Component shall pass all messages, other than those referred to in 3.1.2.3.2.1.4 c) and d), and in 3.1.2.3.2.1.5, received from the AFTN to the Message Transfer and Control Unit for processing as specified in 3.1.2.3.4, and provided that the conditions of 3.1.2.3.2.1.7 are met.

3.1.2.3.2.1.7.  The processing by the AFTN Component shall ensure that all messages and service messages received from the AFTN and passed to the Message Transfer and Control Unit for further processing by the AFTN/AMHS Gateway are constructed in strict accordance with the provisions of Annex 10, Volume II, paragraphs 4.4.15.1 through 4.4.15.3.12. and 4.4.15.6.

3.1.2.3.2.1.8.  The AFTN Component shall perform short-term retention of all messages transmitted towards the AFTN in a manner equivalent to that specified for an AFTN communication centre in Annex 10, Volume II, 4.4.1.7.

3.1.2.3.2.1.9.  The AFTN Component shall perform long-term retention of the heading, address and origin parts of all messages received from the AFTN, with the message receipt-time and the action taken thereon, for a period of at least thirty days.

3.1.2.3.2.1.10.  The AFTN Component shall perform long-term retention of all AFTN messages, in their entirety, that it generates, for a period of at least thirty days.

3.1.2.3.2.1.11.  The AFTN Component shall perform long-term retention of the heading, address and origin parts of all messages received from the Message Transfer and Control Unit and the action taken thereon, for a period of at least thirty days.

3.1.2.3.2.1.12.  Upon reception by an AFTN/AMHS Gateway of an AFTN service message requesting repetition by the originator of an incorrectly received message as specified in Annex 10, Volume II, 4.4.11.1 or 4.4.16.2.2, the AFTN Component shall perform one of the following actions:

a)	terminate the procedure and report an error situation to a control position if the referenced subject AFTN message did not pass through the gateway or if the AFTN Component is not in possession of an unmutilated copy of the subject AFTN message; or

b)	reassume responsibility for the mutilated message and repeat the message in compliance with the provisions of Annex 10, Volume II, 4.4.11.3, if the mutilated message is detected as having passed through the gateway and if the AFTN Component is in possession of an unmutilated copy of the message.

Note.-  The determination whether the AFTN Component is in possession of an unmutilated copy of the message, as mentioned in items a) and b) above, may require the assistance of a control position.

3.1.2.3.2.1.13.  If, for any reason, the Message Transfer and Control Unit is unable to accept AFTN messages passed by the AFTN Component, then the AFTN Component shall handle this situation in compliance with the provisions of Annex 10, Volume II, 4.4.1.5.2.3.

Note.-  Such a condition may be caused by the inability of the Message Transfer and Control Unit to pass AMHS messages to the ATN Component.

3.1.2.3.2.1.14.  The AFTN Component shall ensure that all information objects constructed by the Message Transfer and Control Unit for transmission over the AFTN are handled in accordance with the AFTN procedure, in application of 3.1.2.3.2.1.3 above.

3.1.2.3.2.1.15.  If the AFTN Component is unable to handle an AFTN service message or an AFTN channel-check transmission in compliance with the provisions of Annex 10, Volume II, as specified in 3.1.2.3.2.1.4 d) or 3.1.2.3.2.1.5, then the error condition shall be logged and reported to a control position.

3.1.2.3.2.1.16.  An AFTN address shall be allocated to the AFTN Component.

#### 3.1.2.3.2.2.	ATN Component

3.1.2.3.2.2.1.  The ATN Component shall allow the AFTN/AMHS Gateway to function as an end system on the ATN.

3.1.2.3.2.2.2.  The ATN Component shall handle the interface to the AMHS, and provide an interface to the Message Transfer and Control Unit as specified in 3.1.2.3.2.4, implementing a MTA complying with the profile specification included in 3.1.2.2.2.1 so as to be externally indistinguishable from an ATS Message Server by the ATS Message Server(s) or other AFTN/AMHS Gateway(s) to which it is connected.

3.1.2.3.2.2.3.  If, for any reason, the Message Transfer and Control Unit is unable to accept messages or probes passed by the ATN Component, then the ATN Component shall behave as follows:

a)	attempt to reroute the message or probe as specified in ISO/IEC 10021-4, 14.3.4.4;

b)	if no alternate route is available in the MTA-routing tables or all such routes cannot be succesfully used, reject the message for all the message recipients, whose responsibility element in the per-recipient-indicators has the abstract-value "responsible" in the received message, with the non-delivery-reason-code and non-delivery-diagnostic-code elements of the non-delivery report taking the abstract-values specified in the base standards (ISO/IEC 10021-4, 14.3.4.4., item 1)).

Note.-  Such a condition may be caused by the inability of the Message Transfer and Control Unit to pass AFTN messages to the AFTN Component.

3.1.2.3.2.2.4.  If the AMHS Management Domain operating an AFTN/AMHS Gateway desires to implement Message Handling System optional functional groups in addition to the specification of 3.1.2.3.2.2.2 above, this shall be performed in the ATN Component.

Note.-  This applies in particular to the Redirection Functional Group. If implemented, redirection may be performed by the ATN Component, caused by a failure situation as envisaged in 3.1.2.3.2.2.3 above for example.

3.1.2.3.2.2.5.  The ATN Component shall ensure that all information objects constructed by the Message Transfer and Control Unit for transfer in the AMHS are handled in accordance with the procedures specified in the base standards for a relaying MTA implementing the profile specified in 3.1.2.2.2.1, in application of 3.1.2.3.2.2.2 above.

3.1.2.3.2.2.6.  The ATN Component shall implement a traffic logging function identical to that of the MTA included in an ATS Message Server as specified in 3.1.2.2.2.2.

3.1.2.3.2.2.7.  The ATN Component shall ensure that all AMHS information objects passed to the Message Transfer and Control Unit comply with the base standards.

#### 3.1.2.3.2.3.	Message Transfer and Control Unit

3.1.2.3.2.3.1.  The Message Transfer and Control Unit in an AFTN/AMHS Gateway shall provide a bi-directional conversion facility between the AFTN component and the ATN component, consisting of:

a)	a set of general functions as specified in 3.1.2.3.3; and

b)	AFTN/AMHS conversion functions as respectively specified in 3.1.2.3.4 for the AFTN to AMHS conversion and in 3.1.2.3.5 for the AMHS to AFTN conversion.

3.1.2.3.2.3.2.  The Message Transfer and Control Unit shall use IA-5 characters internally.

3.1.2.3.2.3.3.  The Message Transfer and Control Unit in an AFTN/AMHS Gateway shall pass all the AMHS information objects which it constructs in application of 3.1.2.3.4 and 3.1.2.3.5.6 to the ATN Component of the gateway, for further conveyance in the AMHS.

3.1.2.3.2.3.4.  For the generation of AMHS messages and reports, and for the processing of received AMHS messages, probes and reports, the Message Transfer and Control Unit shall have the capability to interpret the semantics and to perform actions related to the ISO/IEC 10021 Elements of Service which are part of the basic requirements of the MT service as specified in ISO/IEC ISP 12062-3.

3.1.2.3.2.3.5.  The Message Transfer and Control Unit in an AFTN/AMHS Gateway shall pass all the AFTN messages which it constructs in application of 3.1.2.3.5 and 3.1.2.3.4.2.1.4.2 to the AFTN Component of the AFTN/AMHS Gateway, for further conveyance in the AFTN.

3.1.2.3.2.3.6.  The Message Transfer and Control Unit shall ensure that all the AMHS information objects which it constructs comply with section 7 (for IPMs) and section 8 (for RNs) of ISO/IEC 10021-7, complemented with the additional requirements included in Table 3.1.2-1, and with the section 12.2.1.1 of ISO/IEC 10021-4 (for messages) and section 12.2.1.3 of ISO/IEC 10021-4 (for reports).

3.1.2.3.2.3.7.  The Message Transfer and Control Unit shall ensure that all the AFTN information objects which it constructs comply with Annex 10, Volume II, 4.4.15.

#### 3.1.2.3.2.4.	Interface between the ATN Component and the Message Transfer and Control Unit

3.1.2.3.2.4.1.  The ATN Component shall exchange information objects with the Message Transfer and Control Unit via its MTA transfer-port as specified in ISO/IEC 10021-4, section 12.2.

3.1.2.3.2.4.2.  The ATN Component shall invoke the Message-transfer, Report-transfer and Probe-transfer abstract operations, respectively, to pass AMHS messages, reports and probes and to the Message Transfer and Control Unit.

3.1.2.3.2.4.3.  The Message Transfer and Control Unit shall invoke the Message-transfer and Report-transfer abstract operations, respectively, to pass AMHS messages and reports to the ATN Component.

#### 3.1.2.3.2.5.	Interface between the AFTN Component and the Message Transfer and Control Unit

3.1.2.3.2.5.1.  An AFTN message or service message passed by the AFTN Component to the Message Transfer and Control Unit in application of 3.1.2.3.2.1.4 items a) and b), 3.1.2.3.2.1.6 and 3.1.2.3.2.1.7 shall be:

a)	transferred according to the table of priorities as specified in Annex 10, Volume II, 4.4.1.2.1; and

b)	passed as received by the AFTN Component from the adjacent AFTN centre, with the possible exception of an ITA-2 to IA-5 conversion performed in application of 3.1.2.3.2.1.2, and including the unaltered AFTN heading if present in the received message.

3.1.2.3.2.5.2.  An AFTN message or service message passed by the Message Transfer and Control Unit to the AFTN Component in application of 3.1.2.3.2.3.5 shall be:

a)	transferred according to the table of priorities as specified in Annex 10, Volume II, 4.4.1.2.1; and

b)	passed as constructed by the Message Transfer and Control Unit, and thus without message heading as specified in Annex 10, Volume II, 4.4.15.1.1.

3.1.2.3.2.5.3.  The AFTN Component shall return to the Message Transfer and Control Unit, as the result of the transfer operation described in 3.1.2.3.2.5.2, the Transmission Identification, if any, constructed by the AFTN Component for the transmission of the message or service message over the AFTN.

#### 3.1.2.3.2.6.	AFTN/AMHS Gateway Control Position

3.1.2.3.2.6.1.  The AFTN/AMHS Gateway Control Position shall be used as the place where errors which occurred in the AFTN/AMHS Gateway and certain non-deliveries which occurred in the AMHS are reported for appropriate action.

3.1.2.3.2.6.2.  The appropriate action to be undertaken on reporting of an error or of a non-delivery to an AFTN/AMHS Gateway control position shall be either:

a)	a matter of policy which is local to the AMHS Management Domain operating the AFTN/AMHS Gateway; or

b)	subject to multilateral agreements.

Note.-  For some categories of error situations, 3.1.2 specify the actions to be taken, e.g. message rejection and generation of an appropriate service message (to the AFTN) or non-delivery report (to the AMHS). The specified actions aim at minimizing the assistance of the control position. However it may be a matter of policy local to the AMHS Management Domain operating an AFTN/AMHS Gateway to try to reduce the occurrence of message rejection with the assistance of the control position.

3.1.2.3.2.6.3.  When the action chosen to handle an error situation includes the generation of an AMHS information object, the category of information object used for this purpose shall be an IPM conveying appropriate service information.

Note 1.-  The service information to be conveyed may be derived, for example, from an AFTN service message.

Note 2.-  The presentation of the service information is a matter of local policy.

### 3.1.2.3.3.	General functions

#### 3.1.2.3.3.1.	Traffic logging

3.1.2.3.3.1.1.  The Message Transfer and Control Unit shall perform long-term logging, as specified in 3.1.2.3.3.1.2 to 3.1.2.3.3.1.6, for a period of at least thirty days, of information related to the following exchanges of information objects with the ATN Component and with the AFTN Component:

a)	AMHS message transfer out (to the ATN Component);

b)	AMHS report transfer out (to the ATN Component);

c)	AMHS message transfer in (from the ATN Component);

d)	AMHS report transfer in (from the ATN Component);

e)	AFTN message conveyance out (to the AFTN Component);

f)	AFTN message conveyance in (from the AFTN Component);

g)	AFTN service message indicating an unknown addressee indicator conveyance in (from the AFTN Component); and

h)	AFTN service message indicating an unknown addressee indicator conveyance out (to the AFTN Component).

3.1.2.3.3.1.2.  For the long-term logging of information related to an AMHS Message Transfer In and AFTN message conveyance out, the following parameters, relating to the messages, shall be logged by the Message Transfer and Control Unit:

a)	input message-identifier;

b)	IPM-identifier, if any;

c)	common-fields and either receipt-fields or non-receipt-fields of IPN (Inter-Personal Notification), if any;

d)	action taken thereon (reject with non-delivery-reason-code and non-delivery-diagnostic-code, convert as AFTN message, convert as AFTN acknowledgement message, splitting due to number of recipients or message length, delivery report generation);

e)	event date/time;

f)	Origin line of converted AFTN message or service message, if any; and

g)	transmission identification of AFTN message(s) or service message(s), if returned by the AFTN Component.

3.1.2.3.3.1.3.  For the long-term logging of information related to AFTN message conveyance in and AMHS Message Transfer Out, the following parameters, relating to the messages, shall be logged by the Message Transfer and Control Unit:

a)	Origin line of AFTN message (or AFTN acknowledgement message);

b)	transmission identification of AFTN message or service message, if any;

c)	action taken thereon (reject with rejection cause, convert as IPM, convert as RN, AFTN service message indicating an unknown addressee indicator generation);

d)	event date/time;

e)	MTS-identifier, if any; and

f)	IPM-identifier, if any.

3.1.2.3.3.1.4.  For the long-term logging of information related to an AMHS Message Report In and/or AFTN Service Message indicating an unknown addressee indicator conveyance out, the following parameters, relating to the report and/or service message, shall be logged by the Message Transfer and Control Unit:

a)	report-identifier (if report in);

b)	subject-identifier (if report in);

c)	action taken thereon if report in (discard, convert into AFTN service message);

d)	event date/time;

e)	Origin line of converted AFTN service message (if service message out);

f)	Origin line of subject AFTN message (if service message out and no report in); and

g)	transmission identification of AFTN message or service message, if any.

3.1.2.3.3.1.5.  For the long-term logging of information related to an AFTN Service Message indicating an unknown addressee indicator conveyance in and/or to an AMHS Message Report Out, the following parameters, relating to the service message and/or report, shall be logged by the Message Transfer and Control Unit:

a)	Origin line of converted AFTN service message (if service message in);

b)	Origin line of subject AFTN message (if service message in);

c)	transmission identification of AFTN message or service message, if any;

d)	action taken thereon if AFTN service message in (discard, convert into AMHS report);

e)	report-identifier (if report out);

f)	subject-identifier (if report out); and

g)	event date/time

#### 3.1.2.3.3.2.	Address look-up tables

The Message Transfer and Control Unit shall include look-up tables used for address conversion, covering two aspects:

a)	a MD look-up table as specified in 3.1.2.3.3.2.1, for the algorithmic conversion of an AF-Address to an XF-Address; and

b)	a user address look-up table of individual users as specified in 3.1.2.3.3.2.2, for the conversion of an AF-Address to and from an MF-Address of any AMHS Addressing Scheme.

Note.-  The way in which these tables are populated and maintained up-to-date is an organisational matter.

##### 3.1.2.3.3.2.1.	MD look-up Tables

3.1.2.3.3.2.1.1.  The MD (Management Domain) look-up table maintained in the Message Transfer and Control Unit shall include a list of entries identifying an organizational entity, which either is an AMHS Management Domain, or collectively uses the services of a given AMHS Management Domain, each entry comprising:

a)	a string of characters identifying one of the following:

1)	a country (two-letter designator as specified in ICAO Document 7910);

2)	a location (four-letter designator as specified in ICAO Document 7910);

3)	an organization within a country (combination of a two-letter designator as specified in ICAO Document 7910 with a three-letter designator as specified in ICAO Document 8585); or

4)	an organization at a location (combination of a four-letter designator as specified in ICAO Document 7910 with a three-letter designator as specified in ICAO Document 8585); and

b)	the set of attributes identifying either the AMHS Management Domain implemented by the organizational entity defined in a), if existing, or the AMHS Management Domain whose AFTN/AMHS Gateway may be used to communicate with indirect AMHS users within the aforementioned organisational entity, this set of attributes being composed of:

1)	country-name;

2)	ADMD-name; and

3)	PRMD-name (if any).

3.1.2.3.3.2.1.2.  It shall be possible to derive unambiguously a single item b) from item a) by a search operation in the MD look-up table.

##### 3.1.2.3.3.2.2.	User address look-up Tables

3.1.2.3.3.2.2.1.  The user address look-up table maintained by the Message Transfer and Control Unit shall include a list of entries, each of them comprising:

a)	the AF-Address of either an indirect AMHS user who also has a MF-Address, or of a direct AMHS user who has an AF-Address for communication with indirect AMHS users; and

b)	the MF-Address of that AMHS user, either direct or indirect, including all its address attributes.

3.1.2.3.3.2.2.2.  It shall be possible to derive unambiguously item b) from item a), and vice-versa, by a searching operation in the user address look-up table.

3.1.2.3.3.2.2.3.  In order not to restrict the potential form of an MF-Address, a user address look-up table shall support in the attributes included under item b) all the general attribute types authorized in ISO/IEC 10021-2, section 18.5, Table 10.

### 3.1.2.3.4.	AFTN to AMHS Conversion

Note.-  This clause specifies the actions to be performed by an AFTN/AMHS Gateway upon reception of messages from the AFTN for conveyance in the AMHS, after the accomplishment of the AFTN-related procedures by the AFTN Component as specified in 3.1.2.3.2.1.

#### 3.1.2.3.4.1.	Control function

3.1.2.3.4.1.1.  Upon reception by the Message Transfer and Control Unit of a message passed from the AFTN Component, as the result of the provisions of 3.1.2.3.2.1.4 items a) and b), and of 3.1.2.3.2.1.6, the received message shall be processed in one of three mutually exclusive manners depending on the message category:

a)	processing as specified in 3.1.2.3.4.3, if the received message is an AFTN acknowledgement message as specified in Annex 10, Volume II, 4.4.15.6;

b)	processing as specified in 3.1.2.3.4.4, if the received message is an AFTN service message requesting correction by the originator of a message received with an unknown addressee indicator as specified in Annex 10, Volume II, 4.4.11.13.3; or

c)	processing as specified in 3.1.2.3.4.2, if the received message is other than those referred to in a) and b) above.

3.1.2.3.4.1.2.  Upon completion of the processing specified in 3.1.2.3.4.1.1, the following transfers shall take place:

a)	transfer of the resulting AMHS information objects, if any, to the ATN Component for conveyance in the AMHS; and

b)	transfer of the resulting AFTN service messages, if any, to the AFTN Component for conveyance over the AFTN.

3.1.2.3.4.1.3.  If, for any reason, the processing specified in clauses 3.1.2.3.4.1.1 and 3.1.2.3.4.1.2 cannot be properly achieved, the procedure shall unsuccessfully terminate, resulting in:

a)	logging of the error situation and reporting to a control position; and

b)	storage of the AFTN message for appropriate action at the control position.

#### 3.1.2.3.4.2.	Conversion of AFTN Messages

Upon reception by the Message Transfer and Control Unit of an AFTN message passed from the AFTN Component to be conveyed over the AMHS, this AFTN message shall be converted into an IPM conveyed with a Message Transfer Envelope to be transferred and delivered in the AMHS in compliance with the following:

a)	the specification of how the components of the AFTN Message are used for mapping onto the AMHS message parameters, as included in 3.1.2.3.4.2.1;

b)	the specification of how the IPM is generated, as included in 3.1.2.3.4.2.2; and

c)	the specification of how the Message Transfer Envelope elements are generated, as included in 3.1.2.3.4.2.3.

##### 3.1.2.3.4.2.1.	Use of AFTN Message components

3.1.2.3.4.2.1.1.	Each component of an AFTN Message shall be processed as specified in the column "action" of Table 3.1.2-3.

3.1.2.3.4.2.1.2.	These components which are classified as "T" or "T1" in the column "action" of Table 3.1.2-3 shall be translated into the AMHS parameter specified in the column "AMHS parameter" of Table 3.1.2-3 and according to the specification in the clause referred to in the column "mapping".

Table 3.1.2-3.   Use of AFTN Message Components

| AFTN Message Part | Component | Action | AMHS parameter | Mapping |
| --- | --- | --- | --- | --- |
| Heading | Start-of-Heading Character | - | - | – |
|  | Transmission Identification | D | - | - |
| Address | Alignment Function | - | - | - |
|  | Priority Indicator | T | ATS-Message-Priority (see Table 3.1.2-5/Part 5/1.2) priority (see Table 3.1.2-6/Part 1/1.1.6) | see 3.1.2.3.4.2.1.3 |
|  | Addressee Indicator(s) | T | primary-recipients (see Table 3.1.2-5/Part 2/4) recipient-name (see Table 3.1.2-6/Part 1/1.2.1) | see 3.1.2.3.4.2.1.4.2 |
|  | Alignment Function | - | - | – |
| Origin | Filing Time | T | ATS-Message-Filing-Time (see Table 3.1.2-5/Part 5/1.3) | see 3.1.2.3.4.2.1.5 |
|  | Originator Indicator | T | originator (see Table 3.1.2-5/Part 2/2) this-IPM (see Table 3.1.2-5/Part 2/1) originator-name (see Table 3.1.2-6/Part 1/1.1.2) | see 3.1.2.3.4.2.1.4.1 |
|  | Priority Alarm | D | - | - |
|  | Optional Heading Information | T1 | ATS-Message-Optional-Heading-Info (see Table 3.1.2-5/Part 5/1.4) | see 3.1.2.3.4.2.1.6 |
|  | Alignment Function | - | - | - |
|  | Start-of-Text Character | - |  | - |
| Text |  | T | ATS-Message-Text (see Table 3.1.2-5/Part 5/2) | see 3.1.2.3.4.2.1.7 |
| Ending | Alignment Function | - | - | - |
|  | Page-feed sequence | - | - | - |
|  | End-of-Text Character | - | - | - |
| Legend: | (see 3.1.1) T1 = conditionally translated D = discarded T = translated - = not applicable |  |  |  |

3.1.2.3.4.2.1.3.  The value of the priority indicator of an AFTN message shall be:

a)	mapped into the abstract-value of the priority element of the message transfer envelope of the converted AMHS message as specified in the second column of Table 3.1.2-4; and

b)	conveyed as the value of the priority-indicator in the ATS-Message-Priority element of the IPM text of the converted AMHS message as specified in the third column of Table 3.1.2-4.

Note.-  The transport priority used for the conveyance of AMHS messages is specified in 3.1.2.2.2.1.2.3.

Table 3.1.2-4.   Mapping of AFTN Priority Indicator

| AFTN Priority Indicator | AMHS Message Transfer Envelope priority | AMHS ATS-Message-Priority priority-indicator |
| --- | --- | --- |
| SS | urgent | SS |
| DD | normal | DD |
| FF | normal | FF |
| GG | non-urgent | GG |
| KK | non-urgent | KK |

3.1.2.3.4.2.1.4.  The value of an AFTN address included in an AFTN message shall be converted into an MF-Address as respectively specified in 3.1.2.3.4.2.1.4.1 and 3.1.2.3.4.2.1.4.2 depending whether it is an originator indicator or an addressee indicator.

3.1.2.3.4.2.1.4.1.  The following actions shall be performed in order to translate the originator indicator of an AFTN Message into the MF-Address included in the originator-name of the converted AMHS message:

a)	translation into the single MF-Address matching exactly the AF-Address of the originator, if such an MF-Address can be determined from the User address look-up table maintained in the Message Transfer and Control Unit; or

b)	if a) cannot be achieved, translation into the XF-address constructed using the single Management Domain identified by the set of country-name, administration-domain-name and (if any) private-domain-name attributes, determined among the entries in the MD look-up table, if any, matching exactly the following character substrings of the AFTN address and selected among these entries, if several are found, on the basis of a decreasing order of precedence from 1) to 4):

1)	characters 1 to 7,

2)	characters 1, 2, 5, 6 and 7,

3)	characters 1, 2, 3 and 4,

4)	characters 1 and 2; or

c)	if no adequate entry can be found in the MD look-up table, or if the procedure defined in b) does not result in a single resulting MD, unsuccessful termination of the procedure resulting in:

1)	logging of the error situation and reporting to a control position, and

2)	storage of the AFTN message for appropriate action at the control position.

Note.-  The specification above does not constrain the search algorithm provided that the expected result is achieved.

3.1.2.3.4.2.1.4.2.  Each addressee indicator of an AFTN Message shall be translated into the MF-Address included in a recipient-name of the converted AMHS message in the same way as an originator indicator, with the exception that the unsuccessful termination for one or several addressee indicators additionally results in the generation, in compliance with the provisions of Annex 10, Volume II, 4.4.11.13.3, of an AFTN service message requesting correction by the originator of a message received with an unknown addressee indicator, the unknown addressee indicator(s) included in item 8) of the text message taking the value of these addressee indicators for which the translation process failed.

Note.-  A PDAI included in the addressee indicator(s) of an AFTN Message is translated into an MF-Address in the same way as any addressee indicator.

3.1.2.3.4.2.1.5.  The value of the Filing Time of an AFTN message shall be conveyed as the value of the filing-time element in the ATS-Message-Filing-Time element of the IPM text of the converted AMHS message.

3.1.2.3.4.2.1.6.  The ATS-Message-Optional-Heading-Info element of the IPM text in the converted AMHS message shall either:

a)	convey the value of the Optional Heading Information of the AFTN message as the value of its optional-heading-information element, if the Optional Heading Information element is present in the AFTN message; or

b)	be omitted in the converted AMHS message, if the Optional Heading Information element is not present in the AFTN message.

3.1.2.3.4.2.1.7.  The content of the Text of an AFTN message, shall be conveyed in its entirety as the value of the ATS-Message-Text element in the IPM text of the converted AMHS message.

##### 3.1.2.3.4.2.2.	Generation of IPM

3.1.2.3.4.2.2.1.  Each of the elements composing the IPM resulting from the conversion of an AFTN message in the Message Transfer and Control Unit shall be processed as specified in the column "action" of Table 3.1.2-5.

3.1.2.3.4.2.2.2.  These elements which are classified as "G" or "T" in the column "action" of Table 3.1.2-5 shall be either generated or translated according to the specification in the clause referred to in the column "mapping" of Table 3.1.2-5.

Note 1.-  Table 3.1.2-5 is structured as a PRL derived from the profile specification included in 2.2 and consequently from the ISPICS Proforma included in ISO/IEC ISP 12062-2 (AMH21) as well as from Table 3.1.2-2 in 3.1.2.2.3.2. The columns "Base" and "ISP" under "Origination" are extracted from ISO/IEC ISP 12062-2 and the column "Basic ATS Message Service" specifies the static capability of an IPM AU supporting the Basic ATS Message Service, i.e. the ability to generate the element as part of an IPM carrying an ATS Message. The references to the ISP Profile are indicated in the part titles as AMH21/ref where appropriate. The references in column Ref are those of the ISP.

Table 3.1.2-5.   IPM Generation

| Ref | Element | Origination |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| Part 1 : AMH21/A.1.1 Supported information objects |  |  |  |  |  |  |
| 1 | Interpersonal Message (IPM) | M | M | M | T | see Part 1/1.1 and 1.2 |
| 1.1 | heading | M | M | M | T | see Part 2 |
| 1.2 | body | M | M | M | T | see Part 3 |
| 2 | Interpersonal Notification (IPN) | M | M | M | - | out of the scope of this clause |
| Part 2 : AMH21/A.1.2 IPM heading fields |  |  |  |  |  |  |
| 1 | this-IPM | M | M | M | T | see Part 4/3 |
| 2 | originator | M | M | M | T | see 3.1.2.3.4.2.2.3 and Part 4/2 |
| 3 | authorizing-users | O | O | O | X | - |
| 4 | primary-recipients | M | M | M | T | see 3.1.2.3.4.2.2.4 and Part 4/1 |
| 5 | copy-recipients | M | M | M | X | - |
| 6 | blind-copy-recipients | O | O | O | X | - |
| 7 | replied-to-IPM | M | M | M | X | - |
| 8 | obsoleted-IPMs | O | O | O | X | - |
| 9 | related-IPMs | O | O | O | X | - |
| 10 | subject | M | M | M | X | - |

| Ref | Element | Origination |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| 11 | expiry-time | O | O | O | X | - |
| 12 | reply-time | O | O | O | X | - |
| 13 | reply-recipients | O | O | O | X | - |
| 14 | importance | O | O | O | X | - |
| 15 | sensitivity | O | O | O | X | - |
| 16 | auto-forwarded | O | O | O | X | - |
| 17 | extensions | O | O | O | X | - |
| 17.1 | incomplete-copy | O | O | O | X | - |
| 17.2 | languages | O | O | O | X | - |
| 17.3 | auto-submitted | O | I | I | X | - |
| Part 3 : AMH21/A.1.3 IPM body |  |  |  |  |  |  |
| 1 | ia5-text | O | O | M | T | see Part 3/1.1 and 1.2 |
| 1.1 | parameters | M | M | M | G | see Part 3/1.1.1 |
| 1.1.1 | repertoire | O | O | O | G | see 3.1.2.3.4.2.2.5 |
| 1.2 | data | M | M | M | T | see Part 5 |
| 2 | voice | I | I | I | X | - |
| 3 | g3-facsimile | O | O | O | X | - |
| 4 | g4-class-1 | O | O | O | X | - |
| 5 | teletex | O | O | O | X | - |
| 6 | videotex | O | O | O | X | - |

| Ref | Element | Origination |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| 7 | encrypted | I | I | I | X | - |
| 8 | message | O | O | O | X | - |
| 9 | mixed-mode | O | O | O | X | - |
| 10 | bilaterally-defined | O | O | O | X | - |
| 11 | nationally-defined | O | O | O | X | - |
| 12 | externally-defined | O | M | M | X | - |
| Part 4 : AMH21/A.1.5 Common data types |  |  |  |  |  |  |
| 1 | RecipientSpecifier |  |  |  |  |  |
| 1.1 | recipient | M | M | M | T | see 3.1.2.3.4.2.2.6 and Part 4/2 |
| 1.2 | notification-requests | O | O | M | T | see Part 4/1.2.1-1.2.3 |
| 1.2.1 | rn | O | O | M | T | see 3.1.2.3.4.2.2.7 |
| 1.2.2 | nrn | O | O | M | T | see 3.1.2.3.4.2.2.7 |
| 1.2.3 | ipm-return | O | O | O | X | - |
| 1.3 | reply-requested | O | O | O | X | - |
| 1.4 | recipient-extensions | O | I | I | X | - |
| 2 | ORDescriptor |  |  |  |  |  |
| 2.1 | formal-name | M | M1 | M | T | see 3.1.2.3.4.2.2.8 |
| 2.2 | free-form-name | O | O | O | X | - |

| Ref | Element | Origination |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| 2.3 | telephone-number | O | O | O | X | - |
| 3 | IPMIdentifier |  |  |  |  |  |
| 3.1 | user | M | M | M | T | see 3.1.2.3.4.2.2.9 |
| 3.2 | user-relative-identifier | M | M | M | G | - |
| Part 5 : IPM Support of the Basic ATS Message Service |  |  |  |  |  |  |
| 1 | ATS-Message-Header | - | - | M | T | see Part 5/1.1-1.6 |
| 1.1 | start-of-heading | - | - | M | G | see 3.1.2.2.3.2 |
| 1.2 | ATS-Message-Priority | - | - | M | T | see Part 5/1.2.1-1.2.3 |
| 1.2.1 | priority-prompt | - | - | M | G | see 3.1.2.2.3.2 |
| 1.2.2 | priority-indicator | - | - | M | T | see 3.1.2.3.4.2.1.3 |
| 1.2.3 | priority-separator | - | - | M | G | see 3.1.2.2.3.2 |
| 1.3 | ATS-Message-Filing-Time | - | - | M | T | see Part 5/1.3.1-1.3.3 |
| 1.3.1 | filing-time-prompt | - | - | M | G | see 3.1.2.2.3.2 |
| 1.3.2 | filing-time | - | - | M | T | see 3.1.2.3.4.2.1.5 |
| 1.3.3 | filing-time-separator | - | - | M | G | see 3.1.2.2.3.2 |
| 1.4 | ATS-Message-Optional-Heading-Info | - | - | O | T1 | see Part 5/1.4.1-1.4.3 |
| 1.4.1 | OHI-prompt | - | - | M | G | see 3.1.2.2.3.2 |
| 1.4.2 | optional-heading-information | - | - | M | T | see 3.1.2.3.4.2.1.6 |
| 1.4.3 | OHI-separator | - | - | M | G | see 3.1.2.2.3.2 |

| Ref | Element | Origination |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| 1.5 | end-of-heading-blank-line | - | - | M | G | see 3.1.2.2.3.2 |
| 1.6 | start-of-text | - | - | M | G | see 3.1.2.2.3.2 |
| 2 | ATS-Message-Text | - | - | M | T | see 3.1.2.3.4.2.1.7 |
| Legend (see 3.1.1) : M = mandatory support M1 = minimal O/R name mandatory support O = optional support I = out of scope - = not applicable G = generated T = translated X = excluded (not used) |  |  |  |  |  |  |

3.1.2.3.4.2.2.3.  The originator heading field shall:

a)	identify the indirect AMHS user who originated the AFTN message; and

b)	be structured as specified in Table 3.1.2-5/ Part 4/2.

3.1.2.3.4.2.2.4.  The primary-recipients heading field shall:

a)	include the identification of the recipient(s) of the AFTN message; and

b)	be structured as specified in Table 3.1.2-5/ Part 4/1.

3.1.2.3.4.2.2.5.  The element repertoire shall take the abstract value "ia5".

3.1.2.3.4.2.2.6.  The element(s) recipient in the primary-recipients heading field shall:

a)	identify the recipient(s) of the AFTN message; and

b)	be structured as specified in Table 3.1.2-5/ Part 4/2.

3.1.2.3.4.2.2.7.  The values "rn" and "nrn" shall be taken simultaneously by the element notification-requests if, and only if the element priority-indicator included in the message, as specified Table 3.1.2-5 / Part 5/1.2.2, has the value "SS".

3.1.2.3.4.2.2.8.  The element formal-name shall:

a)	take the form of an MF-Address; and

b)	be converted as specified in 3.1.2.3.4.2.1.4.

3.1.2.3.4.2.2.9.  The element user in the this-IPM heading field shall:

a)	be the MF-Address of the indirect AMHS user who originated the AFTN message; and

b)	be converted as specified in 3.1.2.3.4.2.1.4.1.

##### 3.1.2.3.4.2.3.	Generation of Message Transfer Envelope

3.1.2.3.4.2.3.1.  Each of the elements composing the Message Transfer Envelope conveyed with an IPM resulting from the conversion of an AFTN message shall be processed as specified in the column "action" of Table 3.1.2-6.

3.1.2.3.4.2.3.2.  These elements which are classified as "G", "G1" and "T" in the column "action" of Table 3.1.2-6 shall be handled according to the specification in the clause referred to in the column "mapping" of Table 3.1.2-6.

Note 1.-  Table 3.1.2-6 is structured as a PRL derived from the ISPICS Proforma included in ISO/IEC ISP 10611-3. The columns "Base" and "ISP" are extracted from ISO/IEC ISP 10611-3, and the column "Basic ATS Message Service" specifies the static capability of an AU, for the MT-Elements of Service, i.e. the ability to convey, handle and act in relation with the element. The references to the ISP Profile are indicated in the part titles as AMH11/ref where appropriate.

Table 3.1.2-6.   MessageTransfer for conveyance of an IPM

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Part 1 : AMH11/A.1.4.2 MessageTransfer |  |  |  |  |  |  |
| 1 | MessageTransferEnvelope | M | M | M | T | see Part 1/1.1 and 1.2 |
| 1.1 | (per message fields) |  |  |  |  |  |
| 1.1.1 | message-identifier | M | M | M | G | see Part 2/1 |
| 1.1.2 | originator-name | M | M | M | T | see 3.1.2.3.4.2.3.3 |
| 1.1.3 | original-encoded-information-types | M | M- | M- | G | see 3.1.2.3.4.2.3.4 and Part 2/3 |
| 1.1.4 | content-type | M | M- | M- | G | see 3.1.2.3.4.2.3.5 and Part 2/8 |
| 1.1.5 | content-identifier | M | M | M | G1 | see 3.1.2.3.4.2.3.6 |
| 1.1.6 | priority | M | M | M | T | see 3.1.2.3.4.2.1.3 |
| 1.1.7 | per-message-indicators | M | M | M | G | see Part 2/4 |
| 1.1.8 | deferred-delivery-time | O | M- | M- | X | - |
| 1.1.9 | per-domain-bilateral-information | O | M- | M- | G1 | see 3.1.2.3.4.2.3.7 and Part 2/5 |
| 1.1.10 | trace-information | M | M | M | G | see Part 2/6 |
| 1.1.11 | extensions | M | M | M | G/X | see 3.1.2.3.4.2.3.8 and Part 3/1 |
| 1.1.11.1 | recipient-reassignment-prohibited | O | M | M | X | - |
| 1.1.11.2 | dl-expansion-prohibited | O | M | M | X | - |
| 1.1.11.3 | conversion-with-loss-prohibited | O | M | M | X | - |

| Ref | Element |  | Origination |  |  |  |  |  | Action |  | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Base |  | ISP |  | Basic ATS Mess. Service |  |  |  |  |
| 1.1.11.4 | latest-delivery-time | O |  | M- |  | M- |  | X |  | - |  |
| 1.1.11.5 | originator-return-address | O |  | M- |  | M- |  | X |  | - |  |
| 1.1.11.6 | originator-certificate | O |  | M- |  | M- |  | X |  | - |  |
| 1.1.11.7 | content-confidentiality-algorithm-identifier | O |  | M- |  | M- |  | X |  | - |  |
| 1.1.11.8 | message-origin-authentication-check | O |  | M- |  | M- |  | X |  | - |  |
| 1.1.11.9 | message-security-label | O |  | M- |  | M- |  | X |  | - |  |
| 1.1.11.10 | content-correlator | M |  | M |  | M |  | G1 |  | see 3.1.2.3.4.2.3.6 |  |
| 1.1.11.11 | dl-expansion-history | M |  | M- |  | M- |  | X |  | see Note 2 |  |
| 1.1.11.12 | internal-trace-information | M |  | M |  | M |  | G |  | see Part 3/5 |  |
| 1.2 | per-recipient-fields | M |  | M |  | M |  | T |  | see Part 1/1.2.1-1.2.5 |  |
| 1.2.1 | recipient-name | M |  | M |  | M |  | T |  | see 3.1.2.3.4.2.3.9 |  |
| 1.2.2 | originally-specified-recipient-number | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.10 |  |
| 1.2.3 | per-recipient-indicators | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.11 |  |
| 1.2.4 | explicit-conversion | O |  | M- |  | M- |  | X |  | - |  |
| 1.2.5 | extensions | M |  | M |  | M |  | X |  | - |  |
| 2 | content | M |  | M |  | M |  | T |  | see 3.1.2.3.4.2.2 |  |
| Part 2 : AMH11/A.1.5 Common data types |  |  |  |  |  |  |  |  |  |  |  |
| 1 | MTSIdentifier |  |  |  |  |  |  |  |  |  |  |
| 1.1 | global-domain-identifier | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.12 and Part 2/2 |  |

| Ref | Element |  | Origination |  |  |  |  |  | Action |  | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Base |  | ISP |  | Basic ATS Mess. Service |  |  |  |  |
| 1.2 | local-identifier | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.13 |  |
| 2 | GlobalDomainIdentifier |  |  |  |  |  |  |  |  |  |  |
| 2.1 | country-name | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.14 |  |
| 2.2 | administration-domain-name | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.15 |  |
| 2.3 | private-domain-identifier | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.16 |  |
| 3 | EncodedInformationTypes |  |  |  |  |  |  |  |  |  |  |
| 3.1 | built-in-encoded-information-types | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.4 |  |
| 3.2 | (non-basic parameters) | O |  | M- |  | M- |  | X |  | - |  |
| 3.3 | extended-encoded-information-types | M |  | M |  | M |  | X |  | - |  |
| 4 | PerMessageIndicators |  |  |  |  |  |  |  |  |  |  |
| 4.1 | disclosure-of-other-recipients | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.17 |  |
| 4.2 | implicit-conversion-prohibited | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.18 |  |
| 4.3 | alternate-recipient-allowed | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.19 |  |
| 4.4 | content-return-request | O |  | M- |  | M- |  | G |  | see 3.1.2.3.4.2.3.20 |  |
| 4.5 | reserved | O |  | M- |  | M- |  | X |  | - |  |
| 4.6 | bit-5 | O |  | M- |  | M- |  | X |  | - |  |
| 4.7 | bit-6 | O |  | M- |  | M- |  | X |  | - |  |
| 4.8 | service-message | O |  | M- |  | M- |  | X |  | - |  |

| Ref | Element |  | Origination |  |  |  |  |  | Action |  | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Base |  | ISP |  | Basic ATS Mess. Service |  |  |  |  |
| 5 | PerDomainBilateralInformation |  |  |  |  |  |  |  |  |  |  |
| 5.1 | country-name | M |  | M- |  | M- |  | G1 |  | see 3.1.2.3.4.2.3.21 |  |
| 5.2 | administration-domain-name | M |  | M- |  | M- |  | G1 |  | see 3.1.2.3.4.2.3.21 |  |
| 5.3 | private-domain-identifier | O |  | M- |  | M- |  | G1 |  | see 3.1.2.3.4.2.3.21 |  |
| 5.4 | bilateral-information | M |  | M- |  | M- |  | G1 |  | see 3.1.2.3.4.2.3.22 |  |
| 6 | TraceInformation |  |  |  |  |  |  |  |  |  |  |
| 6.1 | TraceInformationElement | M |  | M |  | M |  | G |  | see Part 2/6.1.1 and 6.1.2 |  |
| 6.1.1 | global-domain-identifier | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.23 and Part 2/2 |  |
| 6.1.2 | domain-supplied-information | M |  | M |  | M |  | G |  | see Part 2/6.1.2.1-6.1.2.4 |  |
| 6.1.2.1 | arrival-time | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.24 |  |
| 6.1.2.2 | routing-action | M |  | M |  | M |  | G |  | see Part 2/6.1.2.2.1 and 6.1.2.2.2 |  |
| 6.1.2.2.1 | relayed | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.25 |  |
| 6.1.2.2.2 | rerouted | O |  | C1 |  | C1 |  | X |  | see Note 3 |  |
| 6.1.2.3 | attempted-domain | O |  | C1 |  | C1 |  | X |  | see Note 3 |  |
| 6.1.2.4 | (additional actions) |  |  |  |  |  |  |  |  |  |  |
| 6.1.2.4.1 | deferred-time | M |  | C2 |  | C2 |  | X |  | - |  |
| 6.1.2.4.2 | converted-encoded-information-types | O |  | M- |  | M- |  | X |  | - |  |

| Ref | Element |  | Origination |  |  |  |  |  | Action |  | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Base |  | ISP |  | Basic ATS Mess. Service |  |  |  |  |
| 6.1.2.4.3 | other-actions | O |  | M- |  | M- |  | X |  | - |  |
| 6.1.2.4.3.1 | redirected | O |  | M- |  | M- |  | X |  | see Note 4 |  |
| 6.1.2.4.3.2 | dl-operation | O |  | M- |  | M- |  | X |  | see Note 2 |  |
| 8 | ContentType |  |  |  |  |  |  |  |  |  |  |
| 8.1 | built-in | M |  | M- |  | M- |  | G |  | see 3.1.2.3.4.2.3.5 |  |
| 8.2 | extended | O |  | M- |  | M- |  | X |  | - |  |
| Part 3 : AMH11/A.1.6 Extension data types |  |  |  |  |  |  |  |  |  |  |  |
| 1 | ExtensionField |  |  |  |  |  |  |  |  |  |  |
| 1.1 | type | M |  | M |  | M |  | G |  | see Part 3/1.1.1 and 1.1.2 |  |
| 1.1.1 | standard-extension | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.8 |  |
| 1.1.2 | private-extension | O |  | M- |  | M- |  | X |  | - |  |
| 1.2 | criticality | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.8 |  |
| 1.3 | value | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.8 |  |
| 5 | InternalTraceInformation |  |  |  |  |  |  |  |  |  |  |
| 5.1 | global-domain-identifier | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.23 |  |
| 5.2 | mta-name | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.26 |  |
| 5.3 | mta-supplied-information | M |  | M |  | M |  | G |  | see Part 3/5.3.1-5.3.4 |  |
| 5.3.1 | arrival-time | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.24 |  |

| Ref | Element |  | Origination |  |  |  |  |  | Action |  | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Base |  | ISP |  | Basic ATS Mess. Service |  |  |  |  |
| 5.3.2 | routing-action | M |  | M |  | M |  | G |  | see Part 3/5.3.2.1-5.3.2.2 |  |
| 5.3.2.1 | relayed | M |  | M |  | M |  | G |  | see 3.1.2.3.4.2.3.25 |  |
| 5.3.2.2 | rerouted | O |  | C1 |  | C1 |  | X |  | see Note 3 |  |
| 5.3.3 | attempted | O |  | C1 |  | C1 |  | X |  | see Note 3 |  |
| 5.3.4 | (additional actions) |  |  |  |  |  |  |  |  |  |  |
| 5.3.4.1 | deferred-time | M |  | C2 |  | C2 |  | X |  | - |  |
| 5.3.4.2 | converted-encoded-information-types | O |  | M- |  | M- |  | X |  | - |  |
| 5.3.4.3 | other-actions | O |  | M- |  | M- |  | X |  | - |  |
| 5.3.4.3.1 | redirected | O |  | M- |  | M- |  | X |  | see Note 4 |  |
| 5.3.4.3.2 | dl-operation | O |  | M- |  | M- |  | X |  | see Note 2 |  |
| Legend (see 3.1.1) : M = mandatory support M- = minimal mandatory support O = optional support I = out of scope - = not applicable C1 = if rerouting is supported then M else M- C2 = if deferred delivery is supported then M else M- G = generated G1 = optionally generated T = translated X = excluded |  |  |  |  |  |  |  |  |  |  |  |

Note 2.-  The DL-expansion capability of an AFTN/AMHS Gateway is implemented in the ATN Component rather than in the Message Transfer and Control Unit.

Note 3.-  The rerouting capability of an AFTN/AMHS Gateway, if any, is implemented in the ATN Component rather than in the Message Transfer and Control Unit.

Note 4.-  The redirection capability of an AFTN/AMHS Gateway, if any, is implemented in the ATN Component rather than in the Message Transfer and Control Unit.

3.1.2.3.4.2.3.3.  The value of the element originator-name shall:

a)	be the address of the indirect AMHS user who originated the AFTN message;

b)	take the form of an MF-Address; and

c)	be converted as specified in 3.1.2.3.4.2.1.4.1.

3.1.2.3.4.2.3.4.  The element original-encoded-information-types shall:

a)	take the abstract-value "ia5-text", which is a value of type BuiltInEncodedInformationTypes; and

b)	be formed as specified in Table 3.1.2-6/ Part 2/ 3.

3.1.2.3.4.2.3.5.  The element content-type shall:

a)	take the abstract-value "interpersonal-messaging-1984", which is a value of type BuiltInContentType; and

b)	be formed as specified in Table 3.1.2-6/ Part 2/ 8.

3.1.2.3.4.2.3.6.  The generation of this element shall be optional, as a matter of policy local to the AMHS Management Domain operating the AFTN/AMHS Gateway.

3.1.2.3.4.2.3.7.  The element per-domain-bilateral-information shall be:

a)	optionally generated, as a matter of policy local to the AMHS Management Domain operating the AFTN/AMHS Gateway; and

b)	if present, structured as specified in Table 3.1.2-6/ Part 2/ 5.

3.1.2.3.4.2.3.8.  The only extensions used shall:

a)	belong to the type "standard-extension";

b)	contain the following elements:

1)	content-correlator, if used ; and

2)	internal-trace-information;

c)	take a criticality value as specified in ISO/IEC 10021-4, Figure 2; and

d)	take values as specified in 3.1.2.3.4.2.3.6 and Table 3.1.2-6/Part 3/5, respectively.

Note.-  The non-use of the elements recipient-reassignment-prohibited, dl-expansion-prohibited and conversion-with-loss-prohibited implies, in compliance with ISO/IEC 10021-4, that they are assumed to take their default abstract-values, which are "recipient-reassignment-allowed", "DL-expansion-allowed" and "conversion-with-loss-allowed", respectively.

3.1.2.3.4.2.3.9.  The value of the element recipient-name in each of the per-recipient-fields elements shall:

a)	be the address of each addressee indicated in the AFTN message, respectively;

b)	take the form of a MF-Address; and

c)	be converted as specified in 3.1.2.3.4.2.1.4.2.

3.1.2.3.4.2.3.10.  The value of the element originally-specified-recipient-number in each of the per-recipient-fields elements shall be generated by the Message Transfer and Control Unit as specified in ISO/IEC 10021-4, 12.2.1.1.1.5.

3.1.2.3.4.2.3.11.  The components of the element per-recipient-indicators in each of the per-recipient-fields elements shall be generated taking the following abstract-values:

a)	"responsible" for the responsibility element;

b)	"non-delivery-report" for the originating-MTA-report-request element; and

c)	"non-delivery-report" for the originator-report-request element.

3.1.2.3.4.2.3.12.  The element global-domain-identifier in the MTS-identifier shall:

a)	identify the AMHS Management Domain operating the AFTN/AMHS Gateway; and

b)	be composed as specified in Table 3.1.2-6 / Part 2/2.

3.1.2.3.4.2.3.13.  The element local-identifier in the MTS-identifier shall be generated locally so as to ensure that it distinguishes the message from all other messages, probes or reports generated in the AMHS Management Domain operating the AFTN/AMHS Gateway.

3.1.2.3.4.2.3.14.  The element country-name in the global-domain-identifier element of the MTS-identifier and of the first trace-information-element shall:

a)	be part of the identification of the AMHS Management Domain operating the AFTN/AMHS Gateway by taking one of the following values:

1)	the two-character alphabetical country-indicator as specified in ISO 3166 for the country, or for one of the countries, where the AMHS Management Domain has been registered, if the AMHS Management Domain has been subject to national or multi-national registration; or

2)	a two-character alphabetical indicator dedicated to an international organization, if the AMHS Management Domain has been subject to international registration as specified in ITU-T Recommendation X.666; and

b)	be encoded as a Printable String.

3.1.2.3.4.2.3.15.  The element administration-domain-name in the global-domain-identifier element of the MTS-identifier and of the first trace-information-element shall:

a)	be part of the identification of the AMHS Management Domain operating the AFTN/AMHS Gateway by taking one of the following values, depending on its status:

1)	the name of the ADMD under which the AMHS Management Domain has been registered, either nationally or internationally, if the AMHS Management Domain operates as an ADMD;

2)	the name of the ADMD to which the AMHS Management Domain is connected, if the AMHS Management Domain operates as a PRMD; or

3)	the value single-space if the AMHS Management Domain operates as a PRMD and is unique with regard to the country-name identifying the area where it is registered, either nationally or internationally; and

b)	be encoded as a Printable String.

3.1.2.3.4.2.3.16.  The element private-domain-identifier in the global-domain-identifier element of the MTS-identifier and of the first trace-information-element shall be handled in one of the following manners, depending on the status under which the AMHS Management Domain operates:

a)	generation of the element, with the value of the name of the PRMD, encoded as a Printable String, if the AMHS Management Domain operates as an PRMD; or

b)	omission in the global-domain-identifier if the AMHS Management Domain operates as an ADMD.

3.1.2.3.4.2.3.17.  The element disclosure-of-other-recipients shall take its default abstract-value, which is "disclosure-of-other-recipients-prohibited".

3.1.2.3.4.2.3.18.  The element implicit-conversion-prohibited shall take its default abstract-value, which is "implicit-conversion-allowed".

3.1.2.3.4.2.3.19.  The element alternate-recipient-allowed shall take the abstract-value "alternate-recipient-allowed".

3.1.2.3.4.2.3.20.  The element content-return-request shall take its default abstract-value, which is "content-return-not-requested".

3.1.2.3.4.2.3.21.  The elements country-name, administration-domain-name and private-domain-identifier shall together identify the AMHS Management Domain for which the bilateral-information is intended if, and only if, the element bilateral-information as specified in 3.1.2.3.4.2.3.22 is present.

3.1.2.3.4.2.3.22.  The generation of this element shall be optional, as a matter of bilateral agreement between the AMHS Management Domain operating the AFTN/AMHS Gateway and an other AMHS Management Domain.

3.1.2.3.4.2.3.23.  The element global-domain-identifier in the trace-information or in the internal-trace-information shall:

a)	identify the AMHS Management Domain operating the AFTN/AMHS Gateway; and

b)	be composed as specified in Table 3.1.2-6 / Part 2/2.

3.1.2.3.4.2.3.24.  The element arrival-time in the first element of trace-information or of internal-trace-information shall take the semantic value of the time when the message was received by the Message Transfer and Control Unit for conveyance in the AMHS.

3.1.2.3.4.2.3.25.  The element routing-action in the first element of trace-information or of internal-trace-information shall take the abstract-value "relayed".

3.1.2.3.4.2.3.26.  The element mta-name in the first element of internal-trace-information shall be the mta-name assigned to the Message Transfer and Control Unit included in the AFTN/AMHS Gateway.

Note.-  The structure of the mta-name of the Message Transfer and Control Unit included in an AFTN/AMHS Gateway within an AMHS Management Domain is a matter of policy internal to the AMHS Management Domain.

#### 3.1.2.3.4.3.	Conversion of AFTN Acknowledgement Messages

##### 3.1.2.3.4.3.1.	Initial processing of AFTN Acknowledgement Message

3.1.2.3.4.3.1.1.  Upon reception by the Message Transfer and Control Unit of an AFTN acknowledgement message, passed from the AFTN Component to be conveyed in the AMHS, the received message shall be processed in one of the following manners depending on whether or not the subject AFTN message previously passed through the Message Transfer and Control Unit:

a)	processing as specified in 3.1.2.3.4.3.1.2, if the subject AFTN message, as identified in the text of AFTN acknowledgement message, previously passed through the Message Transfer and Control Unit; or

b)	processing as follows, if the subject AFTN message did not previously pass through the Message Transfer and Control Unit:

1)	logging of the error situation and reporting to a control position; and

2)	conversion of the AFTN acknowledgement message into an IPM conveyed with a Message Transfer Envelope as specified in 3.1.2.3.4.3.1.5.

3.1.2.3.4.3.1.2.  If the subject AFTN message previously passed through the Message Transfer and Control Unit, the AFTN acknowledgement message shall then be processed in one of the following manners depending on whether the subject IPM was received from the AMHS without or with receipt-notification-request:

a)	processing as follows, if the subject IPM was received from the AMHS without receipt-notification-request:

1)	conversion into an IPM conveyed with a Message Transfer Envelope as specified in 3.1.2.3.4.3.1.5; and

2)	logging of the error situation and reporting to a control position; or

b)	processing as specified in 3.1.2.3.4.3.1.3, if the subject IPM was received from the AMHS with receipt-notification-request.

3.1.2.3.4.3.1.3.  If the subject IPM had been received from the AMHS with receipt-notification-request, the AFTN acknowledgement message shall be converted by the AFTN/AMHS Gateway into an Interpersonal Notification (IPN) taking the form of a Receipt Notification (RN), conveyed with a Message Transfer Envelope generated in compliance with the provisions of 3.1.2.3.4.3.1.4.

3.1.2.3.4.3.1.4.  When the provisions of 3.1.2.3.4.3.1.3 apply, the generation of the RN and of the Message Transfer Envelope shall be performed in compliance with the following:

a)	the specification of how the components of the AFTN Service Message are used, as included in 3.1.2.3.4.3.2;

b)	the specification of how the RN is generated, as included in 3.1.2.3.4.3.3; and

c)	the provisions of 3.1.2.3.4.2.3 concerning the generation of the Message Transfer Envelope, with the exception of the differences specified in 3.1.2.3.4.3.4.

3.1.2.3.4.3.1.5.  When an AFTN acknowledgement message is converted into an IPM as the result of 3.1.2.3.4.3.1.1 or 3.1.2.3.4.3.1.2, the specification of 3.1.2.3.4.2 shall apply with the exception of the subject element in the IPM heading fields, initially specified in Table 3.1.2-5/Part 2/10, which is then generated and takes the value "AFTN service information".

##### 3.1.2.3.4.3.2.	Use of AFTN Service Message components

3.1.2.3.4.3.2.1.	Each component of an AFTN acknowledgement message shall be processed for the generation of a RN as specified in the column "action" of Table 3.1.2-7.

3.1.2.3.4.3.2.2.	These components which are classified as "T" or "T1" in the column "action" of Table 3.1.2-7 shall be translated into the AMHS parameter specified in the column "AMHS parameter" of Table 3.1.2-7 and according to the specification in the clause referred to in the column "mapping".

Table 3.1.2-7.   Use of AFTN Service Message Components

| AFTN Message Part | Component | Action | AMHS parameter | Mapping |
| --- | --- | --- | --- | --- |
| Heading | Start-of-Heading Character | - | - | – |
|  | Transmission Identification | D | - | - |
| Address | Alignment Function | - | - | - |
|  | Priority Indicator | T | priority (see Table 3.1.2-6/Part 1/1.1.6) | see 3.1.2.3.4.2.1.3 |
|  | Addressee Indicator | T | recipient-name (see Table 3.1.2-6/Part 1/1.2.1) | see 3.1.2.3.4.2.1.4.2 |
|  | Alignment Function | - | - | – |
| Origin | Filing Time | T | receipt-time (see Table 3.1.2-8/Part 2/7.1) | see 3.1.2.3.4.3.2.4 |
|  | Originator Indicator | T | ipn-originator (see Table 3.1.2-8/Part 2/2) originator-name (see Table 3.1.2-6/Part 1/1.1.2) | see 3.1.2.3.4.3.2.3 see 3.1.2.3.4.2.1.4.1 |
|  | Priority Alarm | D | - | - |
|  | Optional Heading Information | D | - | - |
|  | Alignment Function | - | - | - |
|  | Start-of-Text Character | - |  | - |
| Text |  | D | - | - |
| Ending | Alignment Function | - | - | - |
|  | Page-feed sequence | - | - | - |
|  | End-of-Text Character | - | - | - |
| Legend: | (see 3.1.1.) D = discarded T = translated - = not applicable |  |  |  |

3.1.2.3.4.3.2.3.  Upon generation of a RN as the result of the receipt of an AFTN acknowledgement message by the Message Transfer and Control Unit, the originator indicator element of the AFTN acknowledgement message shall be translated into the ipn-originator element of the RN.

3.1.2.3.4.3.2.4.  Upon generation of a RN as the result of the receipt of an AFTN acknowledgement message by the Message Transfer and Control Unit, the filing time of the AFTN acknowledgement message shall be converted into the receipt-time element, which is of ASN.1 (Abstract syntax notation one) type UTCTime, as the result of the following:

a)	generation by the Message Transfer and Control Unit of the YY figures identifying the year (characters 1 and 2 of the string) in the receipt-time element;

b)	generation by the Message Transfer and Control Unit of the MM figures identifying the month (characters 3 and 4 of the string) in the receipt-time element;

c)	mapping of the value of the first two figures of the date-time group into the value of the DD figures identifying the day (characters 5 and 6 of the string) in the receipt-time element;

d)	mapping of the value of the four last figures of the date-time group, which together represent the hours and minutes, into the value of the hhmm figures (characters 7 to 10 of the string) in the receipt-time element; and

e)	addition by the Message Transfer and Control Unit of an eleventh and last character in the string composing the receipt-time element taking the value "Z".

##### 3.1.2.3.4.3.3.	Generation of RN

3.1.2.3.4.3.3.1.  Each of the elements composing the RN resulting from the receipt of an AFTN acknowledgement message in the Message Transfer and Control Unit shall be processed as specified in the column "action" of Table 3.1.2-8.

3.1.2.3.4.3.3.2.  These elements are classified as "G" or "T" in the column "action" of Table 3.1.2-8 shall be either generated or translated according to the specification in the clause referred to in the column "mapping" of Table 3.1.2-8.

Note.-  Table 3.1.2-8 is structured as a PRL derived from the profile specification included in 2.2 and consequently from the ISPICS Proforma included in ISO/IEC ISP 12062-2 (AMH21). The columns "Base" and "ISP" under "Origination" are extracted from ISO/IEC ISP 12062-2, and the column "Basic ATS Message Service" specifies the static capability of an IPM AU supporting the Basic ATS Message Service, i.e. the ability to generate the element as part of an IPN in the AMHS. The references to the ISP Profile are indicated in the part titles as AMH21/ref where appropriate. The references in column Ref are those of the ISP.

Table 3.1.2-8.   RN Generation

| Ref | Element | Origination |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| Part 1 : AMH21/A.1.1 Supported information objects |  |  |  |  |  |  |
| 1 | Interpersonal Message (IPM) | M | M | M | - | out of the scope of this clause |
| 2 | Interpersonal Notification (IPN) | M | M | M |  | see Part 2 |
| Part 2 : AMH21/A.1.4 IPN fields |  |  |  |  |  |  |
| 1 | subject-ipm | M | M | M | G | see 3.1.2.3.4.3.3.3 |
| 2 | ipn-originator | O | M | M | T | see 3.1.2.3.4.3.2.3 and Part 3/2 |
| 3 | ipm-preferred-recipient | M | M | M | G2 | see 3.1.2.3.4.3.3.4 |
| 4 | conversion-eits | O | O | O | G2 | see 3.1.2.3.4.3.3.5 |
| 5 | notification-extensions | O | I | I | X | - |
| 6 | non-receipt-fields | M | M | M | X | - |
| 7 | receipt-fields | O | O | O | T | see Part 2/7.1-7.4 |
| 7.1 | receipt-time | M | M | M | T | see 3.1.2.3.4.3.2.4 |
| 7.2 | acknowledgment-mode | O | O | O | G | see 3.1.2.3.4.3.3.6 |
| 7.3 | suppl-receipt-info | O | O | O | X | - |
| 7.4 | rn-extensions | O | I | I | X | - |
| 8 | other-notification-type-fields | O | I | I | X | - |

| Ref | Element | Origination |  |  |  |  | Action | Mapping / Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP |  | Basic ATS Mess. Service |  |  |  |  |
| Part 3 : AMH21/A.1.5 Common data types |  |  |  |  |  |  |  |  |  |
| 2 | ORDescriptor |  |  |  |  |  |  |  |  |
| 2.1 | formal-name | M | M1 |  | M |  | T | see 3.1.2.3.4.3.3.7 |  |
| 2.2 | free-form-name | O | O |  | O |  | X |  |  |
| 2.3 | telephone-number | O | O |  | O |  | X |  |  |
| Legend (see 3.1.1) : M = mandatory support M1 = minimal O/R name mandatory support O = optional support I = out of scope G = generated G2 = conditionally generated T = translated X = excluded (not used) |  |  |  |  |  |  |  |  |  |

3.1.2.3.4.3.3.3.  The element subject-ipm shall take the value of the this-IPM heading field of the subject IPM.

3.1.2.3.4.3.3.4.  The element ipm-preferred-recipient shall:

a)	be present if, and only if:

1)	it would be different from the ipn-originator specified in 3.1.2.3.4.3.2.3; and

2)	it would not be the result of a DL-expansion;

b)	if present, identify the recipient of the subject IPM which caused the receipt of the AFTN acknowledgement message by the Message Transfer and Control Unit (as a result of the receipt by its addressee of the subject AFTN message); and

c)	if present, be the O/R descriptor of the recipient of the subject IPM.

3.1.2.3.4.3.3.5.  The element conversion-eits shall:

a)	be present if, and only if, this encoded-information-types is different of the originally-encoded-information-types included in the subject IPM; and

b)	if present, take the value of the encoded-information-types of the subject IPM received by the Message Transfer and Control Unit.

3.1.2.3.4.3.3.6.  The element acknowledgement-mode shall take the abstract-value "manual", which is its default value.

3.1.2.3.4.3.3.7.  The element formal-name in an ORDescriptor shall take the form of an O/R address and be converted from the originator indicator of the AFTN acknowledgement message as specified in 3.1.2.3.4.2.1.4.1.

##### 3.1.2.3.4.3.4.	Differences in the generation of Message Transfer Envelope

3.1.2.3.4.3.4.1.  The elements composing the Message Transfer Envelope which is conveyed with a RN resulting from the receipt of an AFTN acknowledgement message by the Message Transfer and Control Unit, which are different from the specification of 3.1.2.3.4.2.3 shall be processed according to the specification in the clause referred to in the column "mapping" of Table 3.1.2-9.

3.1.2.3.4.3.4.2.  An element subject to the provisions of 3.1.2.3.4.3.4.1 shall be processed as specified in the column "action" of Table 3.1.2-9, and in accordance with the specification referred to in the column "mapping" of Table 3.1.2-9.

Note.-  Table 3.1.2-9 is structured as an extract of Table 3.1.2-6. The references used in the part titles and in the column "Ref" are those of Table 3.1.2-6.

Table 3.1.2-9.   MessageTransfer Envelope generation for conveyance with a RN (Differences with Table 3.1.2-6)

| Ref | Element | Base | ISP | Basic ATS Mess. Service |  | Action |  | Mapping / Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part 1 : AMH11/A.1.4.2 MessageTransfer |  |  |  |  |  |  |  |  |  |
| 1 | MessageTransferEnvelope | M | M | M |  | T |  | see Part 1/1.1 and 1.2 |  |
| 1.1 | (per message fields) |  |  |  |  |  |  |  |  |
| 1.1.3 | original-encoded-information-types | M | M- | M- |  | X |  | - |  |
| 1.1.6 | priority | M | M | M |  | T |  | see 3.1.2.3.4.3.4.3 |  |
| 1.1.7 | per-message-indicators | M | M | M |  | G |  | see Part 2/4 |  |
| 1.2 | per-recipient-fields | M | M | M |  | T |  | see Part 1/1.2.1 and 1.2.3 |  |
| 1.2.1 | recipient-name | M | M | M |  | T |  | see 3.1.2.3.4.3.4.4 |  |
| 1.2.3 | per-recipient-indicators | M | M | M |  | G |  | see 3.1.2.3.4.3.4.5 |  |
| 2 | content | M | M | M |  | T |  | see 3.1.2.3.4.3.3 |  |
| Part 2 : AMH11/A.1.5 Common data types |  |  |  |  |  |  |  |  |  |
| 4 | PerMessageIndicators |  |  |  |  |  |  |  |  |
| 4.2 | implicit-conversion-prohibited | M | M | M |  | G |  | see 3.1.2.3.4.3.4.6 |  |
| Legend (see 3.1.1) : M = mandatory support M- = minimal mandatory support G = generated T = translated X = excluded (not used) |  |  |  |  |  |  |  |  |  |

3.1.2.3.4.3.4.3.  The element priority shall take the same value as that of the subject IPM.

3.1.2.3.4.3.4.4.  The element recipient-name shall:

a)	identify the originator of the subject IPM; and

b)	take the form of an MF-Address.

3.1.2.3.4.3.4.5.  The components of the element per-recipient-indicators shall be generated taking the following abstract-values:

a)	"responsible" for the responsibility element;

b)	"non-delivery-report" for the originating-MTA-report-request element; and

c)	"no-report" for the originator-report-request element.

3.1.2.3.4.3.4.6.  The element implicit-conversion-prohibited shall take the abstract-value "implicit-conversion-prohibited".

#### 3.1.2.3.4.4.	Conversion of AFTN Service Messages related to unknown addressee indicators

##### 3.1.2.3.4.4.1.	Initial Processing of the AFTN Service Message

3.1.2.3.4.4.1.1.  Upon reception by the Message Transfer and Control Unit of an unknown address AFTN service message, passed from the AFTN Component to be conveyed in the AMHS, the received message shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.4.4.1.2, if the subject AFTN message, as identified in the unknown address AFTN service message text, previously passed through the Message Transfer and Control Unit; or

b)	if the subject AFTN message did not previously pass through the Message Transfer and Control Unit, conversion of the unknown address AFTN service message into an IPM conveyed with a Message Transfer Envelope as specified in 3.1.2.3.4.4.1.7.

3.1.2.3.4.4.1.2.  If the subject AMHS message previously passed through the Message Transfer and Control Unit, the received message shall be processed in either of the following manners depending on whether or not the unknown addressee indicator(s) which caused the generation of the unknown address AFTN service message can be determined:

a)	processing as specified in 3.1.2.3.4.4.1.3, if at least one valid addressee indicator which caused the generation of the unknown address AFTN service message can be found; or

b)	if no such valid addressee indicator can be found, conversion of the unknown address AFTN service message into an IPM conveyed with a Message Transfer Envelope as specified in 3.1.2.3.4.4.1.7.

3.1.2.3.4.4.1.3.  For the addressee indicators determined as causing the generation of the unknown address AFTN service message, as the result of 3.1.2.3.4.4.1.2, the received message shall be processed as follows, depending on whether or not the conversion of each unknown addressee indicator into a recipient MF-Address in the same way as specified for an originator indicator in 3.1.2.3.4.2.1.4.1 can be succesfully performed by the Message Transfer and Control Unit:

a)	processing as specified in 3.1.2.3.4.4.1.4, for the set of unknown addressee indicators which can be succesfully translated into an MF-Address, if any; and

b)	for the set of unknown addressee indicator which cannot be succesfully translated, if any, processing as follows:

1)	deletion in the text of the unknown address AFTN service message of all unknown addressee indicators processed as specified in a) above; and

2)	conversion of the resulting unknown address AFTN service message into an IPM conveyed with a Message Transfer Envelope as specified in 3.1.2.3.4.4.1.7.

3.1.2.3.4.4.1.4.  For the unknown recipient MF-Addresses determined as the result of 3.1.2.3.4.4.1.3 a), the received message shall be processed as follows, depending on the abstract-values of the originator-report-request and of the originating-MTA-report-request elements in the per-recipient-indicators in the corresponding per-recipient-fields of the subject AMHS message:

a)	processing as specified in 3.1.2.3.4.4.1.5, for the set of recipients which meet the following condition, if any:

1)	the abstract-value of the originator-report-request differs from "report"; and

2)	the abstract-value of the originating-MTA-report-request differs from "report" and from "audited-report"; or

b)	processing as follows, for all other recipients, if any:

1)	replacement, in the text of the unknown address AFTN service message, of the entire list of unknown addressee indicators with a list restricted to the addressee indicators of these recipients; and

2)	conversion of the resulting unknown address AFTN service message into an IPM conveyed with a Message Transfer Envelope as specified in 3.1.2.3.4.4.1.7.

Note.-  This clause aims at avoiding the generation of a non-delivery-report after the generation of a delivery-report by the MTCU for the same subject AMHS message.

3.1.2.3.4.4.1.5.  For each unknown recipient MF-Address which has not been subject to the generation of a delivery-report, the received message shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.4.4.1.6, if, for a given recipient, no non-delivery report has been generated yet in relation with the same subject AMHS message and with the same message recipient; or

b)	discarding of the unknown address AFTN service message for the considered unknown recipient MF-Address and termination of the procedure for the given recipient if a non-delivery report has already been generated in relation with the same subject AMHS message and with the same message recipient.

Note.-  This clause aims at avoiding the generation of a multiple non-delivery-reports in relation with a single subject AMHS message which would have been split in several AFTN messages when converted from the AMHS to the AFTN, as the result of 3.1.2.3.5.2.1.7.

3.1.2.3.4.4.1.6.  A non-delivery report related to the unknown recipient MF-Addresses, which have not caused the conversion of the unknown address AFTN service message into an IPM as the result of 3.1.2.3.4.4.1.4 and 3.1.2.3.4.4.1.5, shall be generated in compliance with:

a)	the specification of 3.1.2.3.5.6 using the elements of the subject AMHS message; and

b)	the following specification of abstract-values:

1)	"unable-to-transfer" for the non-delivery-reason-code; and

2)	"unrecognised-OR-name" for the non-delivery-diagnostic-code; and

c)	the exception with respect to 3.1.2.3.5.6, that the actual-recipient-name element(s) in each per-recipient-fields element of the report take the value of the unknown recipient MF-Address(es) as determined in 3.1.2.3.4.4.1.5.

Note.-  The potential future reception of an unknown address AFTN service message to be converted into a non-delivery-report requires the retention by the AFTN/AMHS Gateway of certain elements of the subject AMHS message for later report generation, if required.

3.1.2.3.4.4.1.7.  When an unknown address AFTN service message is converted into an IPM as the result of 3.1.2.3.4.4.1.1 to 3.1.2.3.4.4.1.4, the specification of 3.1.2.3.4.2 shall apply with the exception of the subject element in the IPM heading fields, initially specified in Table 3.1.2-5/Part 2/10, which is then generated and takes the value "AFTN service information".

### 3.1.2.3.5.	AMHS to AFTN Conversion

Note.-  This clause specifies the actions to be performed by an AFTN/AMHS Gateway upon reception of information objects from the AMHS for conveyance over the AFTN, after the accomplishment of the AMHS-related procedures by the ATN Component as specified in 3.1.2.3.2.2.

#### 3.1.2.3.5.1.	Control Function

3.1.2.3.5.1.1.  Upon reception by the Message Transfer and Control Unit of an AMHS message passed by the ATN Component, the received message shall be processed in one of the following manners, depending on the abstract-value of the content-type element in the Message Transfer Envelope:

a)	processing as specified in 3.1.2.3.5.1.2 if the abstract-value of the element is either "interpersonal-messaging-1984", or "interpersonal-messaging-1988"; or

b)	if the abstract-value of the element is neither "interpersonal-messaging-1984", nor "interpersonal-messaging-1988":

1)	rejection of the message for all the message recipients for which the responsibility element of the per-recipient-indicators had the abstract-value "responsible"; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values:

i)	"unable-to-transfer" for the non-delivery-reason-code; and

ii)	"content-type-not-supported" for the non-delivery-diagnostic-code.

Note 1.-  The message recipients towards which the Message Transfer and Control Unit conveys the message are those identified by a recipient-name element in the per-recipient-fields element of the Message Transfer Envelope, and for which the responsibility element in the per-recipient-indicators element has the abstract-value "responsible". In 3.1.2.3.5 the term "message recipient" refers to such a recipient.

Note 2.-  Support of other content-types, e.g. edi-messaging, may be added in future packages.

3.1.2.3.5.1.2.  Upon reception by the Message Transfer and Control Unit of an AMHS message whose content-type is either "interpersonal-messaging-1984" or "interpersonal-messaging-1988" passed from the ATN Component, the message shall be processed for conversion into an AFTN message in one of three mutually exclusive manners, depending on the nature of the content:

a)	processing for conversion into an AFTN message as specified in 3.1.2.3.5.2, if the content is an IPM;

b)	processing for conversion into an AFTN service message as specified in 3.1.2.3.5.3, if the content is an IPN which is a Receipt Notification (RN); or

c)	unsuccessful termination of the procedure, if the content is an IPN but not a RN, resulting in:

1)	logging of the error situation and reporting to a control position; and

2)	storage of the message for appropriate processing at the control position.

3.1.2.3.5.1.3.  Upon reception by the Message Transfer and Control Unit of an AMHS non-delivery report passed from the ATN Component, the report shall be processed as specified in 3.1.2.3.5.4.

3.1.2.3.5.1.4.  Upon reception by the Message Transfer and Control Unit of an AMHS probe passed by the ATN Component, the received probe shall be processed in one of the following manners, depending on the abstract-value of the content-type element in the Probe Transfer Envelope:

a)	processing for conveyance test as specified in 3.1.2.3.5.5 if the abstract-value of the element is either "interpersonal-messaging-1984", or "interpersonal-messaging-1988"; or

b)	if the abstract-value of the element is neither "interpersonal-messaging-1984", nor "interpersonal-messaging-1988":

1)	rejection of the probe for all the probe recipients for which the responsibility element of the per-recipient-indicators had the abstract-value "responsible"; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values:

i)	"unable-to-transfer" for the non-delivery-reason-code; and

ii)	"content-type-not-supported" for the non-delivery-diagnostic-code.

3.1.2.3.5.1.5.  Upon reception by the Message Transfer and Control Unit of an ISO/IEC 10021 information object other than those referred to in clauses 3.1.2.3.5.1.1 to 3.1.2.3.5.1.4 above, the processing by the Message Transfer and Control Unit shall unsuccesfully terminate, resulting in:

a)	logging of the error situation and reporting to a control position; and

b)	storage of the information object for appropriate processing at the control position.

Note.-  The Message Transfer and Control Unit requests non-delivery-reports, but never delivery-reports when generating AMHS messages.

3.1.2.3.5.1.6.  Upon completion by the Message Transfer and Control Unit of the processing specified in clauses 3.1.2.3.5.1.1 to 3.1.2.3.5.1.4 above, the resulting AFTN message(s) or AFTN service message(s), if any, shall be passed to the AFTN component, for conveyance over the AFTN.

3.1.2.3.5.1.7.  If the generation of a report is required in relation with the result of the processing specified in clauses 3.1.2.3.5.1.1 or 3.1.2.3.5.1.4 above, either due to message rejection or probe test failure by the Message Transfer and Control Unit, or due to a delivery-report request in the subject AMHS message or probe, an appropriate AMHS report shall be generated as specified in 3.1.2.3.5.6.

#### 3.1.2.3.5.2.	AMHS IPM Conversion

Upon reception by the Message Transfer and Control Unit of an IPM conveyed with a Message Transfer Envelope passed from the ATN Component to be conveyed over the AFTN, this message shall be converted into an AFTN message in compliance with the following:

a)	the specification of the initial processing to be performed by the Message Transfer and Control Unit to determine the ability to convert the message and to split it into individually convertible messages, as included in 3.1.2.3.5.2.1;

b)	the specification of how the AFTN message is generated and how the AFTN message components are mapped from AMHS parameters, as included in 3.1.2.3.5.2.2;

c)	the specification of how the elements of the received IPM are handled, as included in 3.1.2.3.5.2.3; and

d)	the specification of how the Message Transfer Envelope elements are handled, as included in 3.1.2.3.5.2.4.

##### 3.1.2.3.5.2.1.	Initial processing of AMHS Messages

3.1.2.3.5.2.1.1.  Upon reception by the Message Transfer and Control Unit of an IPM conveyed with a Message Transfer Envelope, the received message shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.5.2.1.2 if the abstract-value of the implicit-conversion-prohibited in the per-message-indicators element in the Message Transfer Envelope differs from "prohibited"; or

b)	if the abstract-value of the element is "prohibited":

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"conversion-not-performed" for the non-delivery-reason-code;

ii)	"implicit-conversion-prohibited" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN" for the supplementary-information.

3.1.2.3.5.2.1.2.  A message which was not rejected as the result of 3.1.2.3.5.2.1.1 shall be processed in one of the following manners, depending on the abstract-value of the current encoded-information-types, determined as either the abstract-value of the latest converted-encoded-information-types, if existing, in the trace-information element, or as the abstract-value of the original-encoded-information-types element if the previous does not exist:

a)	processing as specified in 3.1.2.3.5.2.1.3 if the abstract-value of the current encoded-information-types is "ia5-text" or extended "ia5-text"; or

b)	if the abstract-value differs from built-in "ia5-text" and from extended "ia5-text":

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code; and

ii)	"encoded-information-types-unsupported" for the non-delivery-diagnostic-code.

3.1.2.3.5.2.1.3.  A message which was not rejected as the result of 3.1.2.3.5.2.1.2 shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.5.2.1.4 if there is one single body part in the IPM body; or

b)	if there are multiple body parts in the IPM body:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"content-syntax-error" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN due to multiple body parts" for the supplementary-information.

3.1.2.3.5.2.1.4.  A message which was not rejected as the result of 3.1.2.3.5.2.1.3 shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.5.2.1.5 if the body part type is one of the following:

1)	a basic body part type "ia5-text";

2)	a standard extended body part type "ia5-text-body-part";

3)	a standard extended body part type "general-text-body-part" of which the repertoire set description is Basic (ISO 646);

4)	a standard extended body part type "general-text-body-part" of which the repertoire set description is Basic-1 (ISO 8859-1), if and only if the local policy of the AMHS Management Domain is to support the conversion of this repertoire set into IA5IRV characters according to locally defined conversion rules;

5)	a basic body part type "message" with the body part types of the innermost IPM being one of the body part types 1) to 4) above; or

6)	a standard extended body part type "message-body-part" with the body part types of the innermost IPM being one of the body part types 1) to 4) above; or

b)	if the body part type is different from the body part types 1) to 6) under a) above, or if the local policy of the AMHS Management Domain is not to support the conversion of the ISO 8859-1 repertoire set:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"content-syntax-error" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN due to unsupported body part type" for the supplementary-information.

Note.-  The locally defined conversion rules mentioned in bullet 4), item a) may be for example CCITT Recommendation X.408.

3.1.2.3.5.2.1.5.  A message not rejected as the result of 3.1.2.3.5.2.1.4 shall then be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.5.2.1.6 if the text structure in the body part in the body part complies with the requirements of 3.1.2.2.3.2; or

b)	if the text structure does not comply with the requirements of 3.1.2.2.3.2:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"content-syntax-error" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN due to ATS-Message-Header syntax error" for the supplementary-information.

Note.-  The compliance requested to meet the condition of item b) includes the requirement that the element is present and has a value which is syntactically valid for the priority indicator, i.e. a value among SS, DD, FF, GG and KK, and for the filing time, i.e. a value in which the first six figures in the sequence build a valid date-time group.

3.1.2.3.5.2.1.6.  A message which was not rejected as the result of 3.1.2.3.5.2.1.5 shall be processed in one of five mutually exclusive manners:

a)	processing as specified in 3.1.2.3.5.2.1.7 if the abstract-value of the conversion-with-loss-prohibited element in the extensions of the per message fields is"allowed";

b)	if the abstract-value of the element conversion-with-loss-prohibited is "prohibited" and at least one line in the message exceeds 69 characters:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"conversion-not-performed" for the non-delivery-reason-code; and

ii)	"line-too-long" for the non-delivery-diagnostic-code;

c)	if the abstract-value of the element conversion-with-loss-prohibited is "prohibited" and at least one punctuation symbol in the text is not authorized in Annex 10, Volume II, 4.1.2:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"conversion-not-performed" for the non-delivery-reason-code; and

ii)	"punctuation-symbol-loss" for the non-delivery-diagnostic-code;

d)	if the abstract-value of the element conversion-with-loss-prohibited is "prohibited" and at least one alphabetical character in the text is not authorized in Annex 10, Volume II, 4.1.2:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"conversion-not-performed" for the non-delivery-reason-code; and

ii)	"alphabetical-character-loss" for the non-delivery-diagnostic-code; or

e)	if several of the conditions under b) to d) above are simultaneously met:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"conversion-not-performed" for the non-delivery-reason-code; and

ii)	"multiple-information-loss" for the non-delivery-diagnostic-code.

3.1.2.3.5.2.1.7.  A message which was not rejected as the result of 3.1.2.3.5.2.1.6 shall be processed in one of three mutually exclusive manners:

a)	if the length of the ATS-Message-Text element exceeds 1800 characters, and if, due to system resource limitation, the procedure proposed in Annex 10, Volume II, Attachment D cannot be properly achieved by the AFTN/AMHS Gateway:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"content-too-long" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN due to message text length" for the supplementary-information.

b)	if the length of the ATS-Message-Text element exceeds 1800 characters, and if the procedure proposed in Annex 10, Volume II, Attachment D is applied in the AFTN/AMHS Gateway:

1)	splitting of the message, internally to the Message Transfer and Control Unit, into several messages in accordance with the aforementioned Annex 10 procedure:

i)	each of the resulting messages having for conversion purposes the same Message Transfer Envelope, the same IPM Heading and the ATS-Message-Header as the message subject to the splitting; and

ii)	only the ATS-Message-Text element varying between the different resulting messages; and

2)	processing of each of these messages as specified in 3.1.2.3.5.2.1.8; or

c)	processing as specified in 3.1.2.3.5.2.1.8 if the length of the ATS-Message-Text element does not exceed 1800 characters.

3.1.2.3.5.2.1.8.  A message resulting from the situations in items b) and c) of 3.1.2.3.5.2.1.7 above shall be processed in one of three manners, depending on the number of message recipients towards which the Message Transfer and Control Unit is responsible for conveyance of the message, and on the AFTN/AMHS Gateway resources:

a)	if this number exceeds 21 message recipients:

1)	attempt to split the message, internally to the Message Transfer and Control Unit, into several messages, each of them with no more than 21 message recipients:

i)	each of the resulting messages having for conversion purposes the same per-message-fields in the Message Transfer Envelope, and the same content as the message subject to the splitting; and

ii)	only the per-recipient-fields elements in the Message Transfer Envelope varying between the different resulting messages; and

2)	processing of each of these messages as specified in 3.1.2.3.5.2.2 to 3.1.2.3.5.2.4;

b)	if this number exceeds 21 message recipients, and if, due to system resource limitation, the splitting attempt made by the gateway as specified in item a) above cannot be properly achieved:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"too-many-recipients" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN due to number of recipients" for the supplementary-information; or

c)	processing as specified in 3.1.2.3.5.2.2 to 3.1.2.3.5.2.4, if this number does not exceed 21 message recipients.

Note 1.-  In the processing defined in item a), the per-recipient-fields related to a particular recipient remain unchanged by the splitting. This applies in particular to the originally-specified-recipient-number, which is not altered by the processing specified in this clause.

Note 2.-  The combination of 3.1.2.3.5.2.1.7 and 3.1.2.3.5.2.1.8 above may result in a very high number of AFTN messages being generated from one single AMHS message. Items 3.1.2.3.5.2.1.7 a) and 3.1.2.3.5.2.1.8 b) may, as a local matter, be used under such circumstances.

##### 3.1.2.3.5.2.2.	Generation of AFTN Message

3.1.2.3.5.2.2.1.	Each message resulting from the processing specified in 3.1.2.3.5.2.1 above shall be converted by the Message Transfer and Control Unit into an AFTN Message composed of elements as specified in Table 3.1.2-10.

3.1.2.3.5.2.2.2.	Those components which are classified as "G" in the column "action" of Table 3.1.2-10 shall be generated in compliance with the provisions of Annex 10, Volume II referred to in the column "mapping".

3.1.2.3.5.2.2.3.	Those components which are classified as "T" or "T1" in the column "action" of Table 3.1.2-10 shall be converted from the AMHS parameter specified in the column "converted from AMHS parameter" of Table 3.1.2-10 and according to the specification in the clause referred to in the column "mapping".

Table 3.1.2-10.   AFTN Message Generation

| AFTN Message Part | Component | Action | Converted from AMHS parameter | Mapping |
| --- | --- | --- | --- | --- |
| Heading | Start-of-Heading Character | X | - | - |
|  | Transmission Identification | X | - | see 3.1.2.3.5.2.2.4 |
| Address | Alignment Function | G | - | see Annex 10, Vol. II, 4.4.15.2.1 |
|  | Priority Indicator | T | ATS-Message-Priority (see Table 3.1.2-11/ Part 6/1.2) | see 3.1.2.3.5.2.2.5 |
|  | Addressee Indicator(s) | T | recipient-name (see Table 3.1.2-12/Part 1/1.2.1) | see 3.1.2.3.5.2.2.6.2 |
|  | Alignment Function | G | - | see Annex 10, Vol. II, 4.4.15.2.1 |
| Origin | Filing Time | T | ATS-Message-Filing-Time (see Table 3.1.2-11/Part 6/1.3) | see 3.1.2.3.5.2.2.7 |
|  | Originator Indicator | T | originator-name (see Table 3.1.2-12/Part 1/1.1.2) | see 3.1.2.3.5.2.2.6.1 |
|  | Priority Alarm | G | - | see Annex 10, Vol. II, 4.4.15.2.2 |
|  | Optional Heading Information | T1 | ATS-Message-Optional-Heading-Info (see Table 3.1.2-11/Part 6/1.4) | see 3.1.2.3.5.2.2.8 |
|  | Alignment Function | G | - | see Annex 10, Vol. II, 4.4.15.2.2 |
|  | Start-of-Text Character | G | - | see Annex 10, Vol. II, 4.4.15.2.2 |
| Text |  | T | ATS-Message-Text (see Table 3.1.2-11/Part 6/2) | see 3.1.2.3.5.2.2.9 |
| Ending | Alignment Function | G | - | see Annex 10, Vol. II, 4.4.15.3.12 |
|  | Page-feed sequence | G | - | see Annex 10, Vol. II, 4.4.15.3.12 |
|  | End-of-Text Character | G | - | see Annex 10, Vol. II, 4.4.15.3.12 |
| Legend: | (see 3.1.1.) X = excluded (not used) T1 = conditionally translated G = generated T = translated |  |  |  |

3.1.2.3.5.2.2.4.  As specified in 3.1.2.3.2.5.3, the element transmission identification shall be:

a)	generated by the AFTN Component rather than by the Message Transfer and Control Unit; and

b)	returned to the Message Transfer and Control Unit as the result of the operation transferring the generated AFTN Message from the Message Transfer and Control Unit to the AFTN Component.

3.1.2.3.5.2.2.5.  The value of the priority indicator of the converted AFTN message shall be the value of the priority-indicator in the ATS-message-priority element of the AMHS message.

3.1.2.3.5.2.2.6.  The value of an AF-Address included in the converted AFTN message shall be converted from an MF-Address as respectively specified in 3.1.2.3.5.2.2.6.1 and 3.1.2.3.5.2.2.6.2 depending whether it is an originator MF-Address or a recipient MF-Address.

3.1.2.3.5.2.2.6.1.  The originator MF-Address included in an AMHS message shall be processed for translation into the originator indicator of the converted AFTN Message in one of three mutually exclusive manners, depending on the value of the organization-name attribute and on the contents of the User address look-up table, after preliminary conversion of the value of all AMHS address attributes from lower case IA5IRV characters, if any, to upper case IA5IRV characters:

a)	allocation of the value of the first element of the organizational-unit-names attribute to the originator indicator of the converted AFTN Message, if this value is a syntactically valid AF-Address and if the organization-name attribute has the value "AFTN";

b)	determination of an AF-Address matching exactly the MF-Address of the originator in the User address look-up table maintained in the Message Transfer and Control Unit, if the value of the organization-name attribute differs from "AFTN" and if such an exact match can be found; or

c)	if none of the conditions in a) and b) can be met, then:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"invalid-arguments" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN due to unrecognized originator O/R address" for the supplementary-information.

3.1.2.3.5.2.2.6.2.  To build the address part of the converted AFTN Message as specified in Annex 10, Volume II, 4.4.15.2.1, each of the recipient MF-Addresses included in an AMHS message, whose responsibility element in the per-recipient-indicators has the abstract-value "responsible", shall be processed for translation into an addressee indicator in one of three mutually exclusive manners:

a)	allocation of the value of the first element of the organizational-unit-names attribute, converted from lower case IA5IRV characters, if any, to upper case IA5IRV characters, to an addressee indicator in the converted AFTN Message, if this value is a syntactically valid AF-Address and if the organization-name attribute has the value "AFTN";

b)	determination of an AF-Address matching exactly the MF-Address of the recipient in the User address look-up table maintained in the Message Transfer and Control Unit, if the value of the organization-name attribute differs from "AFTN" and if such an exact match can be found; or

c)	if none of the conditions in a) and b) can be met, then:

1)	rejection of the message for the considered message recipient; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code; and

ii)	"unrecognised-OR-name" for the non-delivery-diagnostic-code.

Note 1.-  Although the potential generation of a non-delivery report is mentioned for each recipient-name which cannot be properly translated into an AF-Address, a single report with different per-recipient-fields may be generated for all recipient-names which cannot be translated.

3.1.2.3.5.2.2.7.  The value of the filing time of a converted AFTN message shall be the value of the filing-time component in the ATS-Message-Filing-Time element of the AMHS message.

3.1.2.3.5.2.2.8.  The Optional Heading Information of a converted AFTN message shall either:

a)	take the value of the optional-heading-information in the ATS-Message-Optional-Heading-Info element, if this element is present; or

b)	be omitted in the converted AFTN message, if the ATS-Message-Optional-Heading-Info element is absent from the AMHS message.

3.1.2.3.5.2.2.9.  The content of the Text part of a converted AFTN message shall be derived from the value of the ATS-Message-Text element of the IPM text of the AMHS message, in compliance with the following procedure:

a)	conversion of each character which is not in the IA5IRV character repertoire, into an IA5IRV character according to the locally defined conversion rules;

b)	conversion of each IA5IRV character, if it is in lower case, into the equivalent upper case character;

c)	replacement by question-marks ("?") of all characters or character sequences in the text, if any, of which the use is not authorized in Annex 10, Volume II, 4.1.2;

d)	folding of any line longer than 69 characters; and

e)	allocation of the result of items a) to d) above to the Text part of the converted AFTN message.

Note 1.-  The locally defined conversion rules mentioned in item a) may be for example CCITT Recommendation X.408, if support of the ISO 8859-1 character set is a local policy of the AMHS Management Domain.

Note 2.-  A lower case IA5IRV character is one whose position is between 6/1 and 6/15 or 7/0 and 7/10. The corresponding upper case IA5IRV characters have positions extending from 4/1 to 4/15 and 5/0 to 5/10.

##### 3.1.2.3.5.2.3.	Use of IPM elements

3.1.2.3.5.2.3.1.  Each of the elements composing the IPM in an AMHS message to be converted into an AFTN message in the Message Transfer and Control Unit shall be processed as specified in the column "action" of Table 3.1.2-11.

3.1.2.3.5.2.3.2.  The elements composing the IPM shall be used according to the specification in the clause referred to in the column "mapping" of Table 3.1.2-11.

Note 1.-  Table 3.1.2-11 is structured as a PRL derived from the profile specification included in 2.2 and consequently from the ISPICS Proforma included in ISO/IEC ISP 12062-2 as well as from Table 3.1.2-2 in 3.1.2.2.3.2. The columns "Base" and "ISP" under "Reception" are extracted from ISO/IEC ISP 12062-2 and the column "Basic ATS Message Service" specifies the static capability of an IPM AU supporting the Basic ATS Message Service, i.e. the ability to handle in reception the element as part of an IPM carrying an ATS Message. The references to the ISP Profile are indicated in the part titles as AMH21/ref where appropriate. The references in column Ref are those of the ISP.

Table 3.1.2-11.   Use of IPM Elements

| Ref | Element | Reception |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| Part 1 : AMH21/A.1.1 Supported information objects |  |  |  |  |  |  |
| 1 | Interpersonal Message (IPM) | M | M | M | T | see Part 1/1.1 and 1.2 |
| 1.1 | heading | M | M | M | T | see Part 2 |
| 1.2 | body | M | M | M | T | see Part 3 |
| 2 | Interpersonal Notification (IPN) | O | M | M | - | out of the scope of this clause |
| Part 2 : AMH21/A.1.2 IPM heading fields |  |  |  |  |  |  |
| 1 | this-IPM | M | M | M | D | - |
| 2 | originator | M | M | M | D | - |
| 3 | authorizing-users | M | M | M | D | - |
| 4 | primary-recipients | M | M | M | D | see 3.1.2.3.5.2.3.3 and Part 5/1 |
| 5 | copy-recipients | M | M | M | D | see 3.1.2.3.5.2.3.3 and Part 5/1 |
| 6 | blind-copy-recipients | M | M | M | D | see 3.1.2.3.5.2.3.3 and Part 5/1 |
| 7 | replied-to-IPM | M | M | M | D | - |
| 8 | obsoleted-IPMs | M | M | M | D | - |
| 9 | related-IPMs | M | M | M | D | - |
| 10 | subject | M | M | M | D | - |

| Ref | Element | Reception |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| 11 | expiry-time | M | M | M | D | - |
| 12 | reply-time | M | M | M | D | - |
| 13 | reply-recipients | M | M | M | D | - |
| 14 | importance | M | M | M | D | - |
| 15 | sensitivity | M | M | M | D | - |
| 16 | auto-forwarded | M | M | M | D | - |
| 17 | extensions | M | M | M | D | - |
| 17.1 | incomplete-copy | O | M | M | D | - |
| 17.2 | languages | M | M | M | D | - |
| 17.3 | auto-submitted | O | I | I | D | - |
| Part 3 : AMH21/A.1.3 IPM body |  |  |  |  |  |  |
| 1 | ia5-text | O | M | M | T | see Part 3/1.1 and 1.2 |
| 1.1 | parameters | M | M | M | D | - |
| 1.1.1 | repertoire | M | M | M | D | - |
| 1.2 | data | M | M | M | T | see Part 6 |
| 2 | voice | I | I | I | X | see Note 2 |
| 3 | g3-facsimile | O | O | O | X | see Note 2 |
| 4 | g4-class-1 | O | O | O | X | see Note 2 |
| 5 | teletex | O | O | O | X | see Note 2 |
| 6 | videotex | O | O | O | X | see Note 2 |

| Ref | Element | Reception |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| 7 | encrypted | I | I | I | X | see Note 2 |
| 8 | message | O | M | M | T | see Part 3/8.1 and 8.2 |
| 8.1 | parameters | M | M | M | D | - |
| 8.1.1 | delivery-time | O | M | M | D | - |
| 8.1.2 | delivery-envelope | O | M | M | D | - |
| 8.2 | data | M | M | M | T | see 3.1.2.3.5.2.3.4 and Part 6 |
| 9 | mixed-mode | O | O | O | X | see Note 2 |
| 10 | bilaterally-defined | O | O | O | X | see Note 2 |
| 11 | nationally-defined | O | O | O | X | see Note 2 |
| 12 | externally-defined | O | M | M | X/T | see Note 3 and Part 4 |
| Part 4 : AMH21/A.1.3.1 Extended body part support |  |  |  |  |  |  |
| 1 | ia5-text-body-part | O | M | M | T | see Part 3/1 |
| 2 | g3-facsimile-body-part | O | O | O | X | see Note 2 |
| 3 | g4-class1-body-part | O | O | O | X | see Note 2 |
| 4 | teletex-body-part | O | O | O | X | see Note 2 |
| 5 | videotex-body-part | O | O | O | X | see Note 2 |
| 6 | encrypted-body-part | I | I | I | X | see Note 2 |
| 7 | message-body-part | O | M | M | T | see Part 3/8 |
| 8 | mixed-mode-body-part | O | O | O | X | see Note 2 |

| Ref | Element | Reception |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| 9 | bilaterally-defined-body-part | O | O | O | X | see Note 2 |
| 10 | nationally-defined-body-part | O | O | O | X | see Note 2 |
| 11 | general-text-body-part | O | M | M | T/X | see 3.1.2.3.5.2.1.4, 3.1.2.3.5.2.3.5 and Part 6 |
| 12 | file-transfer-body-part | O | I | I | X | see Note 2 |
| 13 | voice-body-part | O | I | I | X | see Note 2 |
| 14 | oda-body-part | O | O | O | X | see Note 2 |
| Part 5 : AMH21/A.1.5 Common data types |  |  |  |  |  |  |
| 1 | RecipientSpecifier |  |  |  |  |  |
| 1.1 | recipient | M | M | M | D | - |
| 1.2 | notification-requests | M | M | M | D | see Part 5/1.2.1-1.2.3 |
| 1.2.1 | rn | O | O | O | D | see 3.1.2.3.5.2.3.3 |
| 1.2.2 | nrn | M | M | M | D | - |
| 1.2.3 | ipm-return | O | O | O | D | - |
| 1.3 | reply-requested | M | M | M | D | - |
| 1.4 | recipient-extensions | O | I | I | D | - |

| Ref | Element | Reception |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| Part 6 : IPM Support of the Basic ATS Message Service |  |  |  |  |  |  |
| 1 | ATS-Message-Header | - | - | M | T | see Part 6/1.1-1.6 |
| 1.1 | start-of-heading | - | - | M | - | - |
| 1.2 | ATS-Message-Priority | - | - | M | T | see Part 6/1.2.1-1.2.3 |
| 1.2.1 | priority-prompt | - | - | M | - | - |
| 1.2.2 | priority-indicator | - | - | M | T | see 3.1.2.3.5.2.2.5 and 3.1.2.3.5.2.3.3 |
| 1.2.3 | priority-separator | - | - | M | - | - |
| 1.3 | ATS-Message-Filing-Time | - | - | M | T | see Part 6/1.3.1-1.3.3 |
| 1.3.1 | filing-time-prompt | - | - | M | - | - |
| 1.3.2 | filing-time | - | - | M | T | see 3.1.2.3.5.2.2.7 |
| 1.3.3 | filing-time-separator | - | - | M | - | - |
| 1.4 | ATS-Message-Optional-Heading-Info | - | - | M | T1 | see Part 6/1.4.1-1.4.3 |
| 1.4.1 | OHI-prompt | - | - | M | - | - |
| 1.4.2 | optional-heading-information | - | - | M | T | see 3.1.2.3.5.2.2.8 |
| 1.4.3 | OHI-separator | - | - | M | - | - |
| 1.5 | end-of-heading-blank-line | - | - | M | - | - |
| 1.6 | start-of-text | - | - | M | - | - |

| Ref | Element | Reception |  |  | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP | Basic ATS Mess. Service |  |  |
| 2 | ATS-Message-Text | - | - | M | T | see 3.1.2.3.5.2.2.9 |
| Legend (see 3.1.1) : M = mandatory support O = optional support I = out of scope - = not applicable T1 = conditionally translated D = discarded T = translated X = excluded |  |  |  |  |  |  |

Note 2.-  This body part type is excluded as the result of 3.1.2.3.5.2.1.4.

Note 3.-  This body part type may be either excluded or translated, depending on whether or not it is a standard extended body part type, and if yes,depending on the type of extended body part type, as specified in Part 4 and as the result of 3.1.2.3.5.2.1.4.

3.1.2.3.5.2.3.3.  If the priority-indicator of a received AMHS message has the value "SS" and if the notification-requests element of either a primary-recipient, or a copy-recipient, or a blind-copy-recipient element has an abstract-value different from "rn" and if the responsibility element of the corresponding per-recipient-fields of the Message Transfer Envelope has the value "responsible", then an error situation shall be logged and reported to a control position for appropriate action.

Note 1.-  The Message Transfer and Control Unit generates RNs only for SS priority messages, since they are the only messages for which an end-to-end acknowledgement is possible in the AFTN. A receipt-notification-request included in a message with another priority is ignored, considering that the Message Transfer and Control Unit cannot ensure the actual reception of the message by the end-user.

Note 2.-  The above specified error situation, if any, does not cause message rejection.

3.1.2.3.5.2.3.4.  If the body-part type of the IPM included in an AMHS message is "message", then the AMHS message shall be converted as if the body of the innermost IPM included in the data component of the "message" body part were the body of the IPM being converted.

Note.-  This specification places the same conditions on the body of the innermost IPM as on the body of an IPM converted into an AFTN message. In particular this implies that the body part type of the body of the innermost IPM is supported in the conversion process, i.e. that it is either" ia5-text" or "general-text".

3.1.2.3.5.2.3.5.  The components of a general-text body part shall be used as follows for the conversion of the IPM body into the text of the AFTN Message:

a)	the parameters component identify the character set used for the message, as specified in ISO/IEC 10021-7, B.2; and

b)	the data component of a general-text body part are used for the generation of the converted AFTN message as specified in Part 6 of Table 3.1.2-11.

##### 3.1.2.3.5.2.4.	Use of Message Transfer Envelope parameters

3.1.2.3.5.2.4.1.  Each of the elements composing the Message Transfer Envelope of an AMHS message to be converted into an AFTN message in a Message Transfer and Control Unit shall be processed as specified in the column "action" of Table 3.1.2-12.

3.1.2.3.5.2.4.2.  The elements composing the Message Transfer Envelope shall be handled according to the specification in the clause referred to in the column "mapping" of Table 3.1.2-12.

Note 1.-  Table 3.1.2-12 is structured as a PRL derived from the ISPICS Proforma included in ISO/IEC ISP 10611-3. The columns "Base" and "ISP" are extracted from ISO/IEC ISP 10611-3 and the column "Basic ATS Message Service" specifies the static capability of an AU in relation with the MT-EoS (Message Transfer Elements of Service), i.e. the ability to convey, handle and act in relation with the element. The references to the ISP Profile are indicated in the part titles as AMH11/ref where appropriate.

Note 2.-  Although not used for mapping, some elements may generate specific actions for the gateway in the handling of the considered message.

Note 3.-  Some elements may have two classifications, e.g. D/X where certain values of the element may cause message rejection, while other values are simply discarded when the AMHS message is converted into an AFTN message.

Table 3.1.2-12.   Use of the MessageTransfer Envelope

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Part 1 : AMH11/A.1.4.2 MessageTransfer |  |  |  |  |  |  |
| 1 | MessageTransferEnvelope | M | M | M | T | see Part 1/1.1 and 1.2 |
| 1.1 | (per message fields) |  |  |  |  |  |
| 1.1.1 | message-identifier | M | M | M | D | - |
| 1.1.2 | originator-name | M | M | M | T | see 3.1.2.3.5.2.2.6.1 |
| 1.1.3 | original-encoded-information-types | M | M- | M- | D/X | see 3.1.2.3.5.2.1.2 |
| 1.1.4 | content-type | M | M- | M- | D/X | see 3.1.2.3.5.1.1 |
| 1.1.5 | content-identifier | M | M | M | D | - |
| 1.1.6 | priority | M | M | M | D | - |
| 1.1.7 | per-message-indicators | M | M | M | D | see Part 2/4 |
| 1.1.8 | deferred-delivery-time | O | M- | M- | D | see 3.1.2.3.5.2.4.4 |
| 1.1.9 | per-domain-bilateral-information | O | M- | M- | D | see 3.1.2.3.5.2.4.5 and Part 2/5 |
| 1.1.10 | trace-information | M | M | M | D | see Part 2/6 |
| 1.1.11 | extensions | M | M | M | D/X | see 3.1.2.3.5.2.4.6 and Part 3/1 |
| 1.1.11.1 | recipient-reassignment-prohibited | O | M | M | D | see 3.1.2.3.5.2.4.3 |
| 1.1.11.2 | dl-expansion-prohibited | O | M | M | D | see 3.1.2.3.5.2.4.7 |
| 1.1.11.3 | conversion-with-loss-prohibited | O | M | M | D/X | see 3.1.2.3.5.2.1.6 |
| 1.1.11.4 | latest-delivery-time | O | M- | M- | D/X | see 3.1.2.3.5.2.4.8 |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1.11.5 | originator-return-address | O | M- | M- | D | - |
| 1.1.11.6 | originator-certificate | O | M- | M- | X | see 3.1.2.3.5.2.4.9 |
| 1.1.11.7 | content-confidentiality-algorithm-identifier | O | M- | M- | X | see 3.1.2.3.5.2.4.9 |
| 1.1.11.8 | message-origin-authentication-check | O | M- | M- | X | see 3.1.2.3.5.2.4.9 |
| 1.1.11.9 | message-security-label | O | M- | M- | X | see 3.1.2.3.5.2.4.9 |
| 1.1.11.10 | content-correlator | M | M | M | D | - |
| 1.1.11.11 | dl-expansion-history | M | M- | M- | D | - |
| 1.1.11.12 | internal-trace-information | M | M | M | D | - |
| 1.2 | per-recipient-fields | M | M | M | T | see Part 1/1.2.1-1.2.5 |
| 1.2.1 | recipient-name | M | M | M | T | see 3.1.2.3.5.2.2.6.2 |
| 1.2.2 | originally-specified-recipient-number | M | M | M | D | - |
| 1.2.3 | per-recipient-indicators | M | M | M | D | - |
| 1.2.4 | explicit-conversion | O | M- | M- | D | - |
| 1.2.5 | extensions | M | M | M | D/X | see 3.1.2.3.5.2.4.6 and Part 3/1 |
| 1.2.5.1 | originator-requested-alternate-recipient | O | M- | M- | D | see 3.1.2.3.5.2.4.3 |
| 1.2.5.2 | requested-delivery-method | O | M- | M- | D | see 3.1.2.3.5.2.4.10 |
| 1.2.5.3 | physical-forwarding-prohibited | O | M- | M- | X | see 3.1.2.3.5.2.4.11 |
| 1.2.5.4 | physical-forwarding-address-request | O | M- | M- | X | see 3.1.2.3.5.2.4.11 |
| 1.2.5.5 | physical-delivery-modes | O | M- | M- | X | see 3.1.2.3.5.2.4.11 |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1.2.5.6 | registered-mail-type | O | M- | M- | X | see 3.1.2.3.5.2.4.11 |
| 1.2.5.7 | recipient-number-for-advice | O | M- | M- | X | see 3.1.2.3.5.2.4.11 |
| 1.2.5.8 | physical-rendition-attributes | O | M- | M- | X | see 3.1.2.3.5.2.4.11 |
| 1.2.5.9 | physical-delivery-report-request | O | M- | M- | X | see 3.1.2.3.5.2.4.11 |
| 1.2.5.10 | message-token | O | M- | M- | X | see 3.1.2.3.5.2.4.9 |
| 1.2.5.11 | content-integrity-check | O | M- | M- | X | see 3.1.2.3.5.2.4.9 |
| 1.2.5.12 | proof-of-delivery-request | O | M- | M- | X | see 3.1.2.3.5.2.4.9 |
| 1.2.5.13 | redirection-history | M | M- | M- | D | - |
| 2 | content | M | M | M | T | see 3.1.2.3.5.2.3 |
| Part 2 : AMH11/A.1.5 Common data types |  |  |  |  |  |  |
| 4 | PerMessageIndicators |  |  |  |  |  |
| 4.1 | disclosure-of-other-recipients | M | M | M | D | - |
| 4.2 | implicit-conversion-prohibited | M | M | M | D/X | see 3.1.2.3.5.2.1.1 |
| 4.3 | alternate-recipient-allowed | M | M | M | D | see 3.1.2.3.5.2.4.3 |
| 4.4 | content-return-request | O | M- | M- | D | - |
| 4.5 | reserved | O | M- | M- | D | - |
| 4.6 | bit-5 | O | M- | M- | D | - |
| 4.7 | bit-6 | O | M- | M- | D | - |
| 4.8 | service-message | O | M- | M- | D | - |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | PerDomainBilateralInformation |  |  |  |  |  |
| 5.1 | country-name | M | M- | M- | D | see 3.1.2.3.4.2.4.5 |
| 5.2 | administration-domain-name | M | M- | M- | D | see 3.1.2.3.4.2.4.5 |
| 5.3 | private-domain-identifier | O | M- | M- | D | see 3.1.2.3.4.2.4.5 |
| 5.4 | bilateral-information | M | M- | M- | D | see 3.1.2.3.4.2.4.5 |
| 6 | TraceInformation |  |  |  |  |  |
| 6.1 | TraceInformationElement | M | M | M | D | - |
| 6.1.1 | global-domain-identifier | M | M | M | D | - |
| 6.1.2 | domain-supplied-information | M | M | M | D | - |
| 6.1.2.1 | arrival-time | M | M | M | D | - |
| 6.1.2.2 | routing-action | M | M | M | D | - |
| 6.1.2.2.1 | relayed | M | M | M | D | - |
| 6.1.2.2.2 | rerouted | O | C1 | C1 | D | - |
| 6.1.2.3 | attempted-domain | O | C1 | C1 | D | - |
| 6.1.2.4 | (additional actions) |  |  |  | D | - |
| 6.1.2.4.1 | deferred-time | M | C2 | C2 | D | - |
| 6.1.2.4.2 | converted-encoded-information-types | O | M- | M- | D | see 3.1.2.3.5.2.1.2 |
| 6.1.2.4.3 | other-actions | O | M- | M- | D | - |
| 6.1.2.4.3.1 | redirected | O | M- | M- | D | - |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2.4.3.2 | dl-operation | O | M- | M- | D | - |
| Part 3 : AMH11/A.1.6 Extension data types |  |  |  |  |  |  |
| 1 | ExtensionField |  |  |  |  |  |
| 1.1 | type | M | M | M | D/X | see Part 3/1.1.1 and 1.1.2 |
| 1.1.1 | standard-extension | M | M | M | D/X | see 3.1.2.3.5.2.4.6 |
| 1.1.2 | private-extension | O | M- | M- | D/X | see 3.1.2.3.5.2.4.6 |
| 1.2 | criticality | M | M | M | D/X | see 3.1.2.3.5.2.4.6 |
| 1.3 | value | M | M | M | D | - |
| Legend (see 3.1.1) : M = mandatory support M- = minimal mandatory support O = optional support C1 = if rerouting is supported then M else M- C2 = if deferred delivery is supported then M else M- D = discarded T = translated X = excluded |  |  |  |  |  |  |

3.1.2.3.5.2.4.3.  The elements alternate-recipient-allowed and originator-requested-alternate-recipient shall be discarded by the Message Transfer and Control Unit, since the optional Redirection Functional Group, if implemented in an AFTN/AMHS Gateway, is supported by the ATN Component and not by the Message Transfer and Control Unit.

3.1.2.3.5.2.4.4.  The element deferred-delivery-time shall be discarded by the Message Transfer and Control Unit, since this functionality, if implemented in an AFTN/AMHS Gateway, is supported by the ATN Component and not by the Message Transfer and Control Unit.

3.1.2.3.5.2.4.5.  For mapping purposes the whole per-domain-bilateral-information element shall be discarded.

Note.-  If the elements country-name, administration-domain-name and private-domain-identifier in an element of the per-domain-bilateral-information together identify the AMHS Management Domain operating the AFTN/AMHS Gateway, the use made of the bilateral-information element is a local matter.

3.1.2.3.5.2.4.6.  If any extension-field is present in the extensions of the Message Transfer Envelope and not semantically understood by the Message Transfer and Control Unit, then the element shall either:

a)	cause the following actions to be performed if its criticality is set to "CRITICAL FOR TRANSFER" or to "CRITICAL FOR DELIVERY":

1)	message rejection of the message for either:

i)	all the message recipients if the extension is part of the per-message-fields; or

ii)	the considered message recipient if the extension is part of the per-recipient-fields; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in the appropriate per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code; and

ii)	"unsupported-critical-function" for the non-delivery-diagnostic-code; or

b)	be simply discarded if there is no criticality given.

3.1.2.3.5.2.4.7.  The element dl-expansion-prohibited shall be discarded by the Message Transfer and Control Unit, since the DL-expansion capability of an AFTN/AMHS Gateway is supported by the ATN Component and not by the Message Transfer and Control Unit.

3.1.2.3.5.2.4.8.  If the latest-delivery-time element is present, and if, when the AMHS message is handled by the Message Transfer and Control Unit, the current time exceeds the value of the latest-delivery-time, then the following actions shall be performed:

a)	message rejection for all the message recipients; and

b)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in the appropriate per-recipient-fields of the report:

1)	"transfer-failure" for the non-delivery-reason-code; and

2)	"maximum-time-expired" for the non-delivery-diagnostic-code.

3.1.2.3.5.2.4.9.  The Message Transfer and Control Unit does not implement Security Elements of Service. Thus, if any security-related extension-field set to "CRITICAL FOR DELIVERY" is present in the extensions of the Message Transfer Envelope, the following actions shall be performed:

a)	message rejection of the message for either:

1)	all the message recipients if the extension is part of the per-message-fields; or

2)	the considered message recipient if the extension is part of the per-recipient-fields; and

b)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in the appropriate per-recipient-fields of the report:

1)	"unable-to-transfer" for the non-delivery-reason-code; and

2)	"unsupported-critical-function" for the non-delivery-diagnostic-code.

3.1.2.3.5.2.4.10.  The element requested-delivery-method shall be discarded by the Message Transfer and Control Unit.

Note.-  The Message Transfer and Control Unit handles the message irrespective of the value of this attribute, since it indicates only a preferred delivery method (see Technical Corrigendum 5 to ISO/IEC 10021-4).

3.1.2.3.5.2.4.11.  The Message Transfer and Control Unit does not implement Physical Delivery Elements of Service. Thus, if any physical delivery-related extension-field set to "CRITICAL FOR DELIVERY" is present in the extensions of the Message Transfer Envelope, the following actions shall be performed:

a)	message rejection of the message for either:

1)	all the message recipients if the extension is part of the per-message-fields; or

2)	the considered message recipient if the extension is part of the per-recipient-fields; and

b)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in the appropriate per-recipient-fields of the report:

1)	"physical-rendition-not-performed" for the non-delivery-reason-code; and

2)	"unsupported-critical-function" for the non-delivery-diagnostic-code.

#### 3.1.2.3.5.3.	AMHS RN Conversion

Upon reception by the Message Transfer and Control Unit of a RN conveyed with a Message Transfer Envelope passed from the ATN Component, for the acknowledgement of a SS message, this message shall be converted into an AFTN acknowledgement message in compliance with the following:

a)	the specification of the initial processing performed to determine the Message Transfer and Control Unit ability to convert the RN, as included in 3.1.2.3.5.3.1;

b)	the specification of how the AFTN service message is generated and how the AFTN service message components are mapped from AMHS parameters, as included in 3.1.2.3.5.3.2;

c)	the specification of how the elements of the received RN are handled, as included in 3.1.2.3.5.3.3; and

d)	the specification of how the Message Transfer Envelope elements are handled, as included in 3.1.2.3.5.3.4.

##### 3.1.2.3.5.3.1.	Initial processing of AMHS Receipt Notifications

3.1.2.3.5.3.1.1.  Upon reception by the Message Transfer and Control Unit of a RN, passed from the ATN Component to be potentially converted into an AFTN acknowledgement message, the received RN shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.5.3.1.2, if the subject IPM has been previously generated by the Message Transfer and Control Unit; or

b)	unsuccessful termination of the procedure, if the subject IPM has not been previously generated by  the Message Transfer and Control Unit, resulting in:

1)	logging of the error situation and reporting to a control position;

2)	storage of the RN for appropriate action at the control position; and

3)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"invalid-arguments" for the non-delivery-diagnostic-code; and

iii)	"unable to convert RN to AFTN Ack service message due to misrouted RN" for the supplementary-information.

3.1.2.3.5.3.1.2.  For an AMHS RN passed from the ATN Component to the Message Transfer and Control Unit and not rejected as the result of 3.1.2.3.5.3.1.1, the received RN shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.5.3.1.3, if the value of the priority indicator of the subject AFTN message was "SS"; or

b)	unsuccessful termination of the procedure, if the value of the priority indicator was different from "SS", resulting in:

1)	logging of the error situation and reporting to a control position; and

2)	storage of the RN for appropriate action at the control position.

3.1.2.3.5.3.1.3.  An AMHS RN passed from the ATN Component to the Message Transfer and Control Unit and not rejected as the result of 3.1.2.3.5.3.1.2 shall be processed as specified in 3.1.2.3.5.3.2.

##### 3.1.2.3.5.3.2.	Generation of the AFTN acknowledgement message

3.1.2.3.5.3.2.1.  An AMHS RN received by the Message Transfer and Control Unit and not rejected as the result of 3.1.2.3.5.3.1 shall be converted into an AFTN acknowledgement message in compliance with:

a)	the specification of 3.1.2.3.5.2.2 with the exception of the components listed in Table 3.1.2-13; and

b)	the classification of the components included in Table 3.1.2-13, as specified in the column "action" of Table 3.1.2-13.

3.1.2.3.5.3.2.2.  These components which are classified as "G" shall be generated in compliance with the clause referred to in the column "mapping" of Table 3.1.2-13.

3.1.2.3.5.3.2.3.  These components which are classified as "T" shall be converted from the AMHS parameter specified in the column "converted from AMHS parameter" of Table 3.1.2-13 and according to the specification in the clause referred to in the column "mapping".

Table 3.1.2-13.   Generation of AFTN Acknowledgement message

| AFTN Message Part | Component | Action | converted from AMHS parameter | Mapping |
| --- | --- | --- | --- | --- |
| Address | Priority Indicator | G | - | see 3.1.2.3.5.3.2.4 |
| Origin | Filing Time | T | receipt-time (see Table 3.1.2-14/Part 1/7.1) | see 3.1.2.3.5.3.2.5 |
|  | Optional Heading Information | X | - | - |
| Text |  | G | - | see 3.1.2.3.5.3.2.6 |
| Legend: | (see 3.1.1.) G = generated T = translated X = excluded (not used) |  |  |  |

3.1.2.3.5.3.2.4.  In an AFTN acknowledgement message, generated as the result of the conversion of an AMHS RN message, the priority indicator component shall take the value SS.

3.1.2.3.5.3.2.5.  In an AFTN acknowledgement message, generated as the result of the conversion of an AMHS RN message, the filing time component shall:

a)	be a date-time group as specified in Annex 10, Volume II, 4.4.15.2.2.1; and

b)	take the value of the six characters between the fifth and tenth position from the receipt-time element of the RN.

3.1.2.3.5.3.2.6.  In an AFTN acknowledgement message, generated as the result of the conversion of an AMHS RN message, the value of the Text component shall be generated as specified in Annex 10, Volume II, 4.4.15.6 using the origin of the subject AFTN message.

##### 3.1.2.3.5.3.3.	Use of RN fields

3.1.2.3.5.3.3.1.  Each of the elements composing the RN to be converted into an AFTN acknowledgement message in an AFTN/AMHS Gateway shall be processed as specified in the column "action" of Table 3.1.2-14.

3.1.2.3.5.3.3.2.  The elements composing the RN shall be handled according to the specification in the clause referred to in the column "mapping" of Table 3.1.2-14.

Note.-  Table 3.1.2-14 is structured as a PRL derived from the profile specification included in 2.2 and consequently from the ISPICS Proforma included in ISO/IEC ISP 12062-2 (AMH21). The columns "Base" and "ISP" under "Reception" are extracted from ISO/IEC ISP 12062-2, and the column "Basic ATS Message Service" specifies the static capability of an IPM AU supporting the Basic ATS Message Service, i.e. the ability to handle in reception the element as part of a RN. The references to the ISP Profile are indicated in the part titles as AMH21/ref where appropriate. The references in column Ref are those of the ISP.

Table 3.1.2-14.   Use of RN fields

| Ref | Element | Reception |  |  |  |  | Action |  | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | ISP |  | Basic ATS Mess. Service |  |  |  |  |
| Part 1: AMH21/A.1.4 IPN fields |  |  |  |  |  |  |  |  |  |
| 1 | subject-ipm | M | M |  | M |  | D |  | - |
| 2 | ipn-originator | M | M |  | M |  | D |  | - |
| 3 | ipm-preferred-recipient | M | M |  | M |  | D |  | - |
| 4 | conversion-eits | M | M |  | M |  | D |  |  |
| 5 | notification-extensions | O | I |  | I |  | - |  |  |
| 6 | non-receipt-fields | O | M |  | M |  | - |  | out of the scope of this clause |
| 7 | receipt-fields | O | M |  | M |  | T |  | see Part 1/7.1-7.4 |
| 7.1 | receipt-time | M | M |  | M |  | T |  | see 3.1.2.3.5.3.2.5 |
| 7.2 | acknowledgment-mode | M | M |  | M |  | D |  | - |
| 7.3 | suppl-receipt-info | O | O |  | O |  | D |  | - |
| 7.4 | rn-extensions | O | I |  | I |  | - |  | - |
| 8 | other-notification-type-fields | O | I |  | I |  | - |  | - |
| Legend (see 3.1.1) : M = mandatory support O = optional support I = out of scope - = not applicable D = discarded T = translated - = out of scope |  |  |  |  |  |  |  |  |  |

##### 3.1.2.3.5.3.4.	Use of Message Transfer Envelope parameters conveyed with a RN

3.1.2.3.5.3.4.1.  The elements composing the Message Transfer Envelope conveyed with a RN to be converted into an AFTN acknowledgement message shall be used in compliance with:

a)	the specification of 3.1.2.3.5.2.4 with the exception of those elements included in Table 3.1.2-15; and

b)	the specification included in the clause referred to in the column "Mapping" of Table 3.1.2-15.

Note.-  Table 3.1.2-15 is structured as an extraction of Table 3.1.2-12.

Table 3.1.2-15.   Use of the MessageTransfer Envelope conveyed with a RN (differences from Table 3.1.2-12)

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Part 1 : AMH11/A.1.4.2 MessageTransfer |  |  |  |  |  |  |
| 1 | MessageTransferEnvelope | M | M | M | T | see Part 1/1.1 and 1.2 |
| 1.1 | (per message fields) |  |  |  |  |  |
| 1.1.3 | original-encoded-information-types | M | M- | M- | D | see 3.1.2.3.5.3.4.2 |
| 1.1.7 | per-message-indicators | M | M | M | D | see Part 2/4 |
| 1.1.10 | trace-information | M | M | M | D | see Part 2/6 |
| 1.2 | per-recipient-fields | M | M | M | D | see Part 1/1.2.1 |
| 1.2.1 | recipient-name | M | M | M | D | see 3.1.2.3.5.3.4.3 |
| 2 | content | M | M | M | T | see 3.1.2.3.5.3.3 |
| Part 2 : AMH11/A.1.5 Common data types |  |  |  |  |  |  |
| 4 | PerMessageIndicators |  |  |  |  |  |
| 4.2 | implicit-conversion-prohibited | M | M | M | D | see 3.1.2.3.5.3.4.2 |
| 6 | TraceInformation |  |  |  |  |  |
| 6.1 | TraceInformationElement | M | M | M | D | - |
| 6.1.2 | domain-supplied-information | M | M | M | D | - |
| 6.1.2.4 | (additional actions) |  |  |  | D | - |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2.4.2 | converted-encoded-information-types | O | M- | M- | D | see 3.1.2.3.5.3.4.2 |
| Legend (see 3.1.1) : M = mandatory support M- = minimal mandatory support O = optional support D = discarded T = translated |  |  |  |  |  |  |

3.1.2.3.5.3.4.2.  The elements related to the encoded-information-types in the Message Transfer Envelope conveyed with a RN shall be discarded when converting the RN into an AFTN acknowledgement message.

3.1.2.3.5.3.4.3.  The recipient-name element in the Message Transfer Envelope conveyed with a RN shall be discarded when converting the RN into an AFTN acknowledgement message.

Note.-  The Message Transfer and Control Unit uses the information contained in the subject AFTN message to construct an AFTN acknowledgement message.

#### 3.1.2.3.5.4.	AMHS Non-delivery Report Conversion

Upon reception by the Message Transfer and Control Unit of an AMHS Non-Delivery Report passed from the ATN Component, this report shall be processed in compliance with the following:

a)	the specification of the initial processing performed to determine the Message Transfer and Control Unit ability to convert the report, as included in 3.1.2.3.5.4.1;

b)	the specification of how the AFTN service message is generated, if any, and how the AFTN service message components are mapped from AMHS parameters, as included in 3.1.2.3.5.4.2; and

c)	the specification of how the Report Transfer Envelope elements are handled, as included in 3.1.2.3.5.4.3.

##### 3.1.2.3.5.4.1.	Initial processing of AMHS Non-Delivery Reports

3.1.2.3.5.4.1.1.  Upon reception by the Message Transfer and Control Unit of a non-delivery report, passed from the ATN Component to be potentially converted into an AFTN service message, the received non-delivery report shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.5.4.1.2, if the subject AMHS message has been previously generated by the Message Transfer and Control Unit; or

b)	unsuccessful termination of the procedure, if the subject AMHS message has not been previously generated by  the Message Transfer and Control Unit, resulting in:

1)	logging of the error situation and reporting to a control position; and

2)	storage of the non-delivery report for appropriate action at the control position.

3.1.2.3.5.4.1.2.  A non-delivery report received by the Message Transfer and Control Unit, and regarding a subject message which had been generated by the Message Transfer and Control Unit, shall be processed by the Message Transfer and Control Unit in one of three mutually exclusive manners:

a)	processing as specified in 3.1.2.3.5.4.1.3 if there is no originally-intended-recipient-name element with a value different of the actual-recipient-name in any of the per-recipient-fields elements of the report;

b)	processing as follows, if at least one originally-intended-recipient-name element in one of the per-recipient-fields elements has a value different from the value of the actual-recipient-name, and if at least one per-recipient-fields element in the report does not meet the same condition:

1)	logging of the error situation and reporting to a control position;

2)	storage of the non-delivery-report and of the corresponding per-recipient-fields elements for appropriate action at the control position; and

3)	processing of the report as specified in 2.3.5.4.1.3 for the per-recipient-fields where there is no originally-intended-recipient-name element with a value different of the actual-recipient-name; or

c)	processing as follows, if all per-recipient-fields elements of the report include an originally-intended-recipient-name element which has a value different from the value of the actual-recipient-name:

1)	logging of the error situation and reporting to a control position; and

2)	storage of the non-delivery-report and of the corresponding per-recipient-fields elements for appropriate action at the control position.

3.1.2.3.5.4.1.3.  If the non-delivery report did not cause any error situation to be reported, or for the per-recipient-fields of the report which did not cause any error to be reported, the report shall be processed by the Message Transfer and Control Unit in one of the following manners:

a)	conversion of the report into an unknown address AFTN service message as specified in 3.1.2.3.5.4.2, if the non-delivery-diagnostic-code has the abstract-value "unrecognised-OR-name"; or

b)	processing as follows, if the non-delivery-diagnostic-code has any abstract-value other than "unrecognised-OR-name":

1)	logging of the non-delivery situation and reporting to a control position; and

2)	storage of the non-delivery-report for appropriate action at the control position.

##### 3.1.2.3.5.4.2.	Generation of unknown address AFTN service message

3.1.2.3.5.4.2.1.  An AMHS Non-Delivery Report received by the Message Transfer and Control Unit, which non-delivery-diagnostic-code has the abstract-value ""unrecognised-OR-name", and not stored for action at the control position as the result of 3.1.2.3.5.4.1 shall be converted into an AFTN service message to the originator of the subject AFTN message, indicating that an unknown addressee indicator was specified in the subject AFTN message (unknown address AFTN service message) in compliance with:

a)	the specification of Annex 10, Volume II, 4.4.11.13.3; and

b)	the classification of the components included in Table 3.1.2-16, as specified in the column "action" of Table 3.1.2-16 in accordance with the definition in 3.1.1.

3.1.2.3.5.4.2.2.  These components which are classified as "G" shall be generated in compliance with the provisions of Annex 10, Volume II or with the clause referred to in the column "mapping" of Table 3.1.2-16.

3.1.2.3.5.4.2.3.  These components which are classified as "T" shall be converted from the AMHS parameter specified in the column "converted from AMHS parameter" of Table 3.1.2-16 and according to the specification in the clause referred to in the column "mapping".

Table 3.1.2-16.   Generation of unknown address AFTN service message

| AFTN Message Part | Component | Action | converted from AMHS parameter | Mapping |
| --- | --- | --- | --- | --- |
| Heading | Start-of-Heading Character | G | - | see Annex 10, Vol. II, 4.4.15.1.1 |
|  | Transmission Identification | G | - | see Annex 10, Vol. II, 4.4.15.1.1 |
| Address | Alignment Function | G | - | see Annex 10, Vol. II, 4.4.15.2.1 |
|  | Priority Indicator | G | - | see 3.1.2.3.5.4.2.4 |
|  | Addressee Indicator(s) | G | - | see 3.1.2.3.5.4.2.5 |
|  | Alignment Function | G | - | see Annex 10, Vol. II, 4.4.15.2.1 |
| Origin | Filing Time | G | - | see 3.1.2.3.5.4.2.6 |
|  | Originator Indicator | G | - | see 3.1.2.3.5.4.2.7 |
|  | Priority Alarm | G | - | see Annex 10, Vol. II, 4.4.15.2.2 |
|  | Optional Heading Information | X | - | - |
|  | Alignment Function | G | - | see Annex 10, Vol. II, 4.4.15.2.2 |
|  | Start-of-Text Character | G | - | see Annex 10, Vol. II, 4.4.15.2.2 |
| Text |  | T | actual-recipient-name (see Table 3.1.2-17/Part 1/2.2.1) | see 3.1.2.3.5.4.2.8 |
| Ending | Alignment Function | G | - | see Annex 10, Vol. II, 4.4.15.3.12 |
|  | Page-feed sequence | G | - | see Annex 10, Vol. II, 4.4.15.3.12 |
|  | End-of-Text Character | G | - | see Annex 10, Vol. II, 4.4.15.3.12 |
| Legend: | (see 3.1.1.) G = generated T = translated X = excluded (not used) |  |  |  |

3.1.2.3.5.4.2.4.  The priority indicator component shall take the value of the priority indicator of the subject AFTN message.

3.1.2.3.5.4.2.5.  The addressee indicator(s) component shall contain a single AF-Address which is the originator indicator of the subject AFTN message.

3.1.2.3.5.4.2.6.  The filing time component, expressed as a date-time group in compliance with Annex 10, Volume II, 4.4.15.2.2.1, shall take the value of the time at which the AFTN service message is generated by the Message Transfer and Control Unit.

3.1.2.3.5.4.2.7.  The originator indicator shall be the AFTN Address of the AFTN Component of the AFTN/AMHS Gateway, as specified in 3.1.2.3.2.1.16.

3.1.2.3.5.4.2.8.  The value of the message text component shall be structured as follows:

a)	a first line composed as specified in Annex 10, Volume II, 4.4.11.13.3, items 1) to 4), using the origin of the subject AFTN message;

b)	a second line composed as specified in Annex 10, Volume II, 4.4.11.13.3, items 5) and 6), using the line-following-the-heading of the subject AFTN message; and

c)	the third and following lines as appropriate composed as specified in Annex 10, Volume II, 4.4.11.13.3, items 7) to 9), using the AF-Address(es) translated as specified in 3.1.2.3.5.4.2.9 from the actual-recipient-name elements of the per-recipient-fields of the Non-Delivery Report which were not stored for action at the control position as the result of 3.1.2.3.5.4.1.3.

3.1.2.3.5.4.2.9.  Each actual-recipient-name element used to generate an unknown address AFTN service message as specified in item c) of 3.1.2.3.5.4.2.8 above shall be processed for translation into an AF-Address in one of three mutually exclusive manners, after preliminary conversion of the value of all AMHS address attributes from lower case IA5IRV characters, if any, to upper case IA5IRV characters:

a)	allocation of the value of the first element of the organizational-unit-names attribute to the AF-Address, if this value is a syntactically valid AF-Address and if the organization-name attribute has the value "AFTN";

b)	determination of an AF-Address matching exactly the MF-Address of the recipient in the User address look-up table maintained in the Message Transfer and Control Unit, if the value of the organization-name attribute differs from "AFTN" and if such an exact match can be found; or

c)	if none of the conditions in a) and b) can be met, then:

1)	logging of the error situation and reporting to a control position; and

2)	storage of the MF-Address for appropriate action at the control position.

##### 3.1.2.3.5.4.3.	Use of Report Transfer Envelope and Content parameters

3.1.2.3.5.4.3.1.  Each of the elements composing the Report Transfer Envelope and Report Transfer Content of an AMHS report to be converted into an AFTN service message in the Message Transfer and Control Unit shall be processed as specified in the column "action" of Table 3.1.2-17.

3.1.2.3.5.4.3.2.  These elements shall be handled according to the specification in the clause referred to in the column "mapping" of Table 3.1.2-17.

Note.-  Table 3.1.2-17 is structured as a PRL derived from the ISPICS Proforma included in ISO/IEC ISP 10611-3.  The columns "Base" and "ISP" are extracted from ISO/IEC ISP 10611-3, and the column "Basic ATS Message Service" specifies the static capability of  an AU for the MT-EoS, i.e. the ability to convey, handle and act in relation with the element. The references to the ISP Profile are indicated in the part titles as AMH11/ref where appropriate.

Table 3.1.2-17.   Use of Report Transfer Envelope and Content parameters

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Part 1 : AMH11/A.1.4.3 ReportTransfer |  |  |  |  |  |  |
| 1 | ReportTransferEnvelope | M | M | M | D | - |
| 2 | ReportTransferContent | M | M | M | T | see Part 1/2.1 and 2.2 |
| 2.1 | (per report fields) |  |  |  |  |  |
| 2.1.1 | subject-identifier | M | M | M | D | - |
| 2.1.2 | subject-intermediate-trace-information | O | M | M | D | - |
| 2.1.3 | original-encoded-information-types | M | M | M | D | - |
| 2.1.4 | content-type | M | M | M | D | - |
| 2.1.5 | content-identifier | M | M | M | D | - |
| 2.1.6 | returned-content | O | M- | M- | D | - |
| 2.1.7 | additional-information | O | M- | M- | D | - |
| 2.1.8 | extensions | M | M | M | D | - |
| 2.2 | per-recipient-fields | M | M | M |  |  |
| 2.2.1 | actual-recipient-name | M | M | M | T | see 3.1.2.3.5.4.2.8 |
| 2.2.2 | originally-specified-recipient-number | M | M | M | D | - |
| 2.2.3 | per-recipient-indicators | M | M | M | D | - |
| 2.2.4 | last-trace-information | M | M | M | D | - |
| 2.2.5 | originally-intended-recipient-name | M | M | M | X | see 3.1.2.3.5.4.1.2 |
| 2.2.6 | supplementary-information | O | M- | M- | D | - |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Mapping / Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2.7 | extensions | M | M | M | D | - |
| Legend (see 3.1.1) : M = mandatory support M- = minimal mandatory support O = optional support D = discarded T = translated X = excluded |  |  |  |  |  |  |

##### 3.1.2.3.5.5.	Action upon reception of AMHS Probe

3.1.2.3.5.5.1.  Upon reception by the Message Transfer and Control Unit of an AMHS probe which content type is either "interpersonal-messaging-1984" or "interpersonal-messaging-1988", the received probe shall be processed in one of the following manners, depending on the abstract-value of the current encoded-information-types, determined as either the abstract-value of the latest converted-encoded-information-types, if existing, in the trace-information element, or as the abstract-value of the original-encoded-information-types element in the Probe Transfer Envelope if the previous does not exist:

a)	processing as specified in 3.1.2.3.5.5.2 if the abstract-value of the current encoded-information-types is "ia5-text" or extended "ia5-text"; or

b)	if the abstract-value differs from built-in "ia5-text" and from extended "ia5-text":

1)	rejection of the probe for all the probe recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code; and

ii)	"encoded-information-types-unsupported" for the non-delivery-diagnostic-code.

3.1.2.3.5.5.2.  A probe which was not rejected as the result of 3.1.2.3.5.5.1 shall be processed in one of the following manners:

a)	processing as specified in 3.1.2.3.5.5.3 if the abstract-value of the  implicit-conversion-prohibited in the per-message-indicators element in the Probe Transfer Envelope differs from "prohibited"; or

b)	if the abstract-value of the element is "prohibited":

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"conversion-not-performed" for the non-delivery-reason-code;

ii)	"implicit-conversion-prohibited" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN" for the supplementary-information.

3.1.2.3.5.5.3.  A probe which was not rejected as the result of 3.1.2.3.5.5.2 shall be processed in one of three mutually exclusive manners:

a)	if, due to system resource limitation, the value of the element content-length in the Probe Transfer Envelope exceeds the conversion capability of the Message Transfer and Control Unit, then:

1)	rejection of the message for all the message recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code; and

ii)	"content-too-long" for the non-delivery-diagnostic-code; or

b)	processing as specified in 3.1.2.3.5.5.4 for further conveyance test if the content-length does not exceed the conversion capability of the Message Transfer and Control Unit.

Note.-  The way to determine the conversion capability of the Message Transfer and Control Unit in terms of message length is a matter local to the AMHS Management Domain operating the AFTN/AMHS Gateway.

3.1.2.3.5.5.4.  A probe which was not rejected as the result of 3.1.2.3.5.5.3 shall be processed in one of three mutually exclusive manners, depending on the number of probe recipients towards which the Message Transfer and Control Unit is responsible for conveyance test, and on the AFTN/AMHS Gateway resources:

a)	if this number exceeds 21 probe recipients:

1)	attempt to split the probe, internally to the Message Transfer and Control Unit, into several probes, each of them with no more than 21 probe recipients:

i)	each of the resulting probes having for conveyance test purposes the same per-probe-fields in the Probe Transfer Envelope; and

ii)	only the per-recipient-fields elements in the Probe Transfer Envelope varying between the different resulting probes; and

2)	processing of each of these probes as specified in 3.1.2.3.5.5.5;

b)	if this number exceeds 21 probe recipients, and if, due to system resource limitation, the splitting attempt made by the gateway as specified in item a) above cannot be properly achieved:

1)	rejection of the probe for all the probe recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"too-many-recipients" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN due to number of recipients" for the supplementary-information; or

c)	processing as specified in 3.1.2.3.5.5.5, if this number does not exceed 21 probe recipients.

3.1.2.3.5.5.5.  A probe which was not rejected as the result of 3.1.2.3.5.5.4 shall be processed in one of the following manners, depending on the ability of the Message Transfer and Control Unit to translate the originator-name element of the Probe Transfer Envelope into an AF-Address,:

a)	processing as specified in 3.1.2.3.5.5.6 if either of the following conditions is met:

1)	if, after conversion from lower case IA5IRV characters, if any, to upper case IA5IRV characters, the organization-name attribute has the value "AFTN" and if the value of the first element of the organizational-unit-names is a syntactically valid AF-Address; or

2)	if, after conversion from lower case IA5IRV characters, if any, to upper case IA5IRV characters,  the value of the organization-name attribute differs from "AFTN" and if an AF-Address matching exactly the MF-Address of the originator can be found in the User address look-up table maintained in the Message Transfer and Control Unit; or

b)	if none of the conditions 1) or 2) in a) above can be met, then:

1)	rejection of the probe for all the probe recipients; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in all the per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code;

ii)	"invalid-arguments" for the non-delivery-diagnostic-code; and

iii)	"unable to convert to AFTN due to unrecognized originator O/R address" for the supplementary-information.

3.1.2.3.5.5.6.  For each probe recipient, a probe which was not rejected as the result of 3.1.2.3.5.5.5 shall be processed in one of the following manners, depending on the ability of the Message Transfer and Control Unit to translate the considered recipient-name element of the Probe Transfer Envelope into an AF-Address:

a)	processing as specified in 3.1.2.3.5.5.7 if either of the following conditions is met:

1)	if, after conversion from lower case IA5IRV characters, if any, to upper case IA5IRV characters, the organization-name attribute has the value "AFTN" and if the value of the first element of the organizational-unit-names is a syntactically valid AF-Address; or

2)	if, after conversion from lower case IA5IRV characters, if any, to upper case IA5IRV characters, the value of the organization-name attribute differs from "AFTN" and if an AF-Address matching exactly the MF-Address of the recipient can be found in the User address look-up table maintained in the Message Transfer and Control Unit; or

b)	if none of the conditions 1) or 2) in a) above can be met, then:

1)	rejection of the probe for the considered recipient; and

2)	generation of a non-delivery report as specified in 3.1.2.3.5.6 with the following elements taking the following abstract-values in the corresponding per-recipient-fields of the report:

i)	"unable-to-transfer" for the non-delivery-reason-code; and

ii)	"unrecognised-OR-name" for the non-delivery-diagnostic-code.

3.1.2.3.5.5.7.  For the probe recipients which were not rejected as the result of 3.1.2.3.5.5.6, a delivery-report shall be generated as specified in 3.1.2.3.5.6,, if requested, to indicate the successful result of the probe conveyance test.

#### 3.1.2.3.5.6.	Generation of AMHS Reports

##### 3.1.2.3.5.6.1.	General

3.1.2.3.5.6.1.1.  A non-delivery report shall be generated by the Message Transfer and Control Unit:

a)	for each message or probe which was rejected as the result of the procedures described in 3.1.2.3.5.1.1, 3.1.2.3.5.1.4, 3.1.2.3.5.2 and 3.1.2.3.5.5, either for all the recipients or for certain recipients; and

b)	as the result of the conversion of an unknown address AFTN service message, as specified in 3.1.2.3.4.4.1.6.

3.1.2.3.5.6.1.2.  Recommendation.-  When the generation of a non-delivery report is required in relation with the rejection of the subject AMHS message for more than one recipient of the subject AMHS message, a single non-delivery report should be generated to report on the rejection for multiple recipients, using several per-recipient-fields elements in the Report Transfer Content.

3.1.2.3.5.6.1.3.  For each AMHS message which was converted by the Message Transfer and Control Unit as the result of the procedures specified in 3.1.2.3.5.2.2 to 3.1.2.3.5.2.4 and then succesfully passed to the AFTN Component as specified in 3.1.2.3.5.1.6, a delivery report shall be generated by the Message Transfer and Control Unit for each message recipient of which:

a)	the originating-MTA-report-request element has the abstract-value "report" or "audited-report"; or

b)	the originator-report-request element has the abstract-value "report"; or

c)	both conditions a) and b) above are met.

3.1.2.3.5.6.1.4.  Recommendation.-  When the generation of a delivery report is required as specified in 3.1.2.3.5.6.1.3 for more than one recipient of the subject AMHS message, a single delivery report should be generated to report on the conveyance towards multiple recipients, using several per-recipient-fields elements in the Report Transfer Content.

3.1.2.3.5.6.1.5.  When the generation of a delivery report is required in relation with the result of a probe conveyance test as specified in 3.1.2.3.5.5, the clauses 3.1.2.3.5.6.1.3 to 3.1.2.3.5.6.1.4 above shall apply with the difference that the event which triggers the generation of the delivery report is the success of the probe conveyance test.

3.1.2.3.5.6.1.6.  A report resulting from the clauses above shall be generated as specified in 3.1.2.3.5.6.2.

##### 3.1.2.3.5.6.2.	Generation of Report Transfer Envelope and Content

3.1.2.3.5.6.2.1.  Each report resulting from the specification of 3.1.2.3.5.6.1 shall be generated by the Message Transfer and Control Unit, in the form of an AMHS Report Transfer Envelope and Report Transfer Content, composed of elements as specified in the column "action" of Table 3.1.2-18.

3.1.2.3.5.6.2.2.  These elements which are classified as "G" or "G2" shall be either generated or conditionally generated according to the specification in the clause referred to in the column "generation action" of Table 3.1.2-18.

Note.-  Table 3.1.2-18 is structured as a PRL derived from the ISPICS Proforma included in ISO/IEC ISP 10611-3. The columns "Base" and "ISP" are extracted from ISO/IEC ISP 10611-3, and the column "Basic ATS Message Service" specifies the static capability of  an AU in relation with the MT-EoS, i.e. the ability to convey, handle and act in relation with the element. The references to the ISP Profile are indicated in the part titles as AMH11/ref where appropriate.

Table 3.1.2-18.   Generation of AMHS Report

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Generation action |
| --- | --- | --- | --- | --- | --- | --- |
| Part 1 : AMH11/A.1.4.3 ReportTransfer |  |  |  |  |  |  |
| 1 | ReportTransferEnvelope | M | M | M | G | see Part 1/1.1-1.4 |
| 1.1 | report-identifier | M | M | M | G | see 3.1.2.3.5.6.2.3 and Part 2/1 |
| 1.2 | report-destination-name | M | M | M | G | see 3.1.2.3.5.6.2.6 |
| 1.3 | trace-information | M | M | M | G | see 3.1.2.3.5.6.2.7 |
| 1.4 | extensions | M | M | M |  | see 3.1.2.3.5.6.2.8 |
| 1.4.1 | message-security-label | O | M- | M- | X | - |
| 1.4.2 | originator-and-DL-expansion-history | M | M | M | G2 | see 3.1.2.3.5.6.2.9 |
| 1.4.3 | reporting-DL-name | O | M- | M- | X | - |
| 1.4.4 | reporting-MTA-certificate | O | M- | M- | X | - |
| 1.4.5 | report-origin-authentication-check | O | M- | M- | X | - |
| 1.4.6 | internal-trace-information | M | M | M | G | see 3.1.2.3.5.6.2.10 |
| 2 | ReportTransferContent | M | M | M | G | see Part1/2.1 and 2.2 |
| 2.1 | (per report fields) |  |  |  |  |  |
| 2.1.1 | subject-identifier | M | M | M | G | see 3.1.2.3.5.6.2.11 |
| 2.1.2 | subject-intermediate-trace-information | O | M | M | G2 | see 3.1.2.3.5.6.2.12 |
| 2.1.3 | original-encoded-information-types | M | M | M | G | see 3.1.2.3.5.6.2.13 |
| 2.1.4 | content-type | M | M | M | G | see 3.1.2.3.5.6.2.14 |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Generation action |
| --- | --- | --- | --- | --- | --- | --- |
| 2.1.5 | content-identifier | M | M | M | G2 | see 3.1.2.3.5.6.2.15 |
| 2.1.6 | returned-content | O | M- | M- | G2 | see 3.1.2.3.5.6.2.16 |
| 2.1.7 | additional-information | O | M- | M- | X | - |
| 2.1.8 | extensions | M | M | M |  | see 3.1.2.3.5.6.2.8 |
| 2.1.8.1 | content-correlator | M | M | M | G2 | see 3.1.2.3.5.6.2.17 |
| 2.2 | per-recipient-fields | M | M | M |  | see Part1/2.2.1-2.2.7 |
| 2.2.1 | actual-recipient-name | M | M | M | G | see 3.1.2.3.5.6.2.18 |
| 2.2.2 | originally-specified-recipient-number | M | M | M | G | see 3.1.2.3.5.6.2.19 |
| 2.2.3 | per-recipient-indicators | M | M | M | G | see 3.1.2.3.5.6.2.20 |
| 2.2.4 | last-trace-information | M | M | M | G | see Part 2/7 |
| 2.2.5 | originally-intended-recipient-name | M | M | M | G2 | see 3.1.2.3.5.6.2.26 |
| 2.2.6 | supplementary-information | O | M- | M- | G2 | see 3.1.2.3.5.6.2.27 |
| 2.2.7 | extensions | M | M | M |  | see 3.1.2.3.5.6.2.8 |
| 2.2.7.1 | redirection-history | M | M | M | G2 | see 3.1.2.3.5.6.2.28 |
| 2.2.7.2 | physical-forwarding-address | O | M- | M- | X | - |
| 2.2.7.3 | recipient-certificate | O | M- | M- | X | - |
| 2.2.7.4 | proof-of-delivery | O | M- | M- | X | - |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Generation action |
| --- | --- | --- | --- | --- | --- | --- |
| Part 2 : AMH11/A.1.5 Common data types |  |  |  |  |  |  |
| 1 | MTSIdentifier |  |  |  |  |  |
| 1.1 | global-domain-identifier | M | M | M | G | see 3.1.2.3.5.6.2.4 and Part 2/2 |
| 1.2 | local-identifier | M | M | M | G | see 3.1.2.3.5.6.2.5 |
| 2 | GlobalDomainIdentifier |  |  |  |  |  |
| 2.1 | country-name | M | M | M |  | see 3.1.2.3.5.6.2.29 |
| 2.2 | administration-domain-name | M | M | M |  | see 3.1.2.3.5.6.2.30 |
| 2.3 | private-domain-identifier | M | M | M |  | see 3.1.2.3.5.6.2.31 |
| 6 | TraceInformation |  |  |  |  |  |
| 6.1 | TraceInformationElement | M | M | M | G | see Part 2/6.1.1 and 6.1.2 |
| 6.1.1 | global-domain-identifier | M | M | M | G | see 3.1.2.3.5.6.2.32 and Part 2/2 |
| 6.1.2 | domain-supplied-information | M | M | M | G | see Part 2/6.1.2.1-6.1.2.4 |
| 6.1.2.1 | arrival-time | M | M | M | G | see 3.1.2.3.5.6.2.33 |
| 6.1.2.2 | routing-action | M | M | M | G | see Part 2/6.1.2.2.1 and 6.1.2.2.2 |
| 6.1.2.2.1 | relayed | M | M | M | G | see 3.1.2.3.5.6.2.34 |
| 6.1.2.2.2 | rerouted | O | C1 | C1 | X | - |
| 6.1.2.3 | attempted-domain | O | C1 | C1 | X | - |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Generation action |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2.4 | (additional actions) |  |  |  |  |  |
| 6.1.2.4.1 | deferred-time | M | C2 | C2 | X | - |
| 6.1.2.4.2 | converted-encoded-information-types | O | M- | M- | X | - |
| 6.1.2.4.3 | other-actions | O | M- | M- | X | - |
| 6.1.2.4.3.1 | redirected | O | M- | M- | X | - |
| 6.1.2.4.3.2 | dl-operation | O | M- | M- | X | - |
| 7 | LastTraceInformation |  |  |  |  |  |
| 7.1 | arrival-time | M | M | M | G | see 3.1.2.3.5.6.2.21 |
| 7.2 | converted-encoded-information-types | M | M | M | G2 | see 3.1.2.3.5.6.2.22 |
| 7.3 | report-type | M | M | M | G | see Part 2/7.3.1and 7.3.2 |
| 7.3.1 | delivery | M | M | M | G2 | see Part 2/7.3.1.1 and 7.3.1.2 |
| 7.3.1.1 | message-delivery-time | M | M | M | G | see 3.1.2.3.5.6.2.23 |
| 7.3.1.2 | type-of-MTS-user | M | M | M | G | see 3.1.2.3.5.6.2.24 |
| 7.3.2 | non-delivery | M | M | M | G2 | see Part 2/7.3.2.1 and 7.3.2.2 |
| 7.3.2.1 | non-delivery-reason-code | M | M | M | G | see 3.1.2.3.5.6.2.25 |
| 7.3.2.2 | non-delivery-diagnostic-code | M | M | M | G | see 3.1.2.3.5.6.2.25 |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Generation action |
| --- | --- | --- | --- | --- | --- | --- |
| Part 3 : AMH11/A.1.6 Extension Data Types |  |  |  |  |  |  |
| 1 | ExtensionField |  |  |  |  |  |
| 1.1 | type | M | M | M | G | see Part 3/1.1.1 and 1.1.2 |
| 1.1.1 | standard-extension | M | M | M | G | see 3.1.2.3.5.6.2.8 |
| 1.1.2 | private-extension | O | M- | M- | X | - |
| 1.2 | criticality | M | M | M | G | see 3.1.2.3.5.6.2.8 |
| 1.3 | value | M | M | M | G | see 3.1.2.3.5.6.2.8 |
| 5 | InternalTraceInformation |  |  |  |  |  |
| 5.1 | global-domain-identifier | M | M | M | G | see 3.1.2.3.5.6.2.32 |
| 5.2 | mta-name | M | M | M | G | see 3.1.2.3.5.6.2.35 |
| 5.3 | mta-supplied-information | M | M | M | G | see Part 3/5.3.1-5.3.4 |
| 5.3.1 | arrival-time | M | M | M | G | see 3.1.2.3.5.6.2.33 |
| 5.3.2 | routing-action | M | M | M | G | see Part 3/5.3.2.1-5.3.2.2 |
| 5.3.2.1 | relayed | M | M | M | G | see 3.1.2.3.5.6.2.34 |
| 5.3.2.2 | rerouted | O | C1 | C1 | X | - |
| 5.3.3 | attempted | O | C1 | C1 | X | - |
| 5.3.4 | (additional actions) |  |  |  |  |  |
| 5.3.4.1 | deferred-time | M | C2 | C2 | X | - |
| 5.3.4.2 | converted-encoded-information-types | O | M- | M- | X | - |

| Ref | Element | Base | ISP | Basic ATS Mess. Service | Action | Generation action |
| --- | --- | --- | --- | --- | --- | --- |
| 5.3.4.3 | other-actions | O | M- | M- | X | - |
| 5.3.4.3.1 | redirected | O | M- | M- | X | - |
| 5.3.4.3.2 | dl-operation | O | M- | M- | X | - |
| Legend (see 3.1.1) : M = mandatory support M- = minimal mandatory support O = optional support I = out of scope - = not applicable C1 = if rerouting is supported then M else M- C2 = if deferred delivery is supported then M else M- G = generated G2 = conditionally generated X = excluded (not used) |  |  |  |  |  |  |

3.1.2.3.5.6.2.3.  The element report-identifier in the Report Transfer Envelope shall:

a)	be generated locally so as to ensure that it distinguishes the report from all other messages, probes or reports generated in the AMHS, as specified in ISO/IEC 10021-4, 12.2.1.3.1.1; and

b)	be composed as specified in Table 3.1.2-18/Part 2/1.

3.1.2.3.5.6.2.4.  The element global-domain-identifier in the report-identifier, or in the trace-information, or in the internal-trace-information shall:

a)	identify the AMHS Management Domain operating the AFTN/AMHS Gateway; and

b)	be composed as specified in Table 3.1.2-18/Part 2/2.

3.1.2.3.5.6.2.5.  The element local-identifier in the report-identifier shall be generated locally so as to ensure that it distinguishes the report from all other messages, probes or reports generated in the AMHS Management Domain operating the AFTN/AMHS Gateway.

3.1.2.3.5.6.2.6.  The report-destination-name element in the Report Transfer Envelope shall be one of the following:

a)	the last OR-name in the DL-expansion-history element, if present, of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.1.11.11; or

b)	the originator-name of the subject AMHS message, as specified in Table 3.1.2-12/Part 1/1.1.2, if there is no DL-expansion-history element in the subject AMHS message.

3.1.2.3.5.6.2.7.  The first trace-information-element in the trace-information of the Report Transfer Envelope shall be generated as specified in Table 3.1.2-18/Part 2/6.

3.1.2.3.5.6.2.8.  Only extensions of type "standard-extension" as defined in the base standards shall be used, as further specified in the classification of Table 3.1.2-18.

3.1.2.3.5.6.2.9.  If a DL-expansion-history element as specified in Table 3.1.2-12/Part 1/1.1.11.11 was present in the subject AMHS message, the originator-and-DL-expansion-history element shall be generated as the sequence of the originator-name of the subject AMHS message, as specified in Table 3.1.2-12/Part 1/1.1.2, and of the aforementioned DL-expansion-history element of the subject AMHS message.

3.1.2.3.5.6.2.10.  The first internal-trace-information-element in the internal-trace-information of the Report Transfer Envelope shall be generated as specified in Table 3.1.2-18/Part 3/5.

3.1.2.3.5.6.2.11.  The subject-identifier element in the Report Transfer Content shall take the value of the message-identifier element of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.1.1.

3.1.2.3.5.6.2.12.  The subject-intermediate-trace-information element in the Report Transfer Content shall take the value which the trace-information element of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.1.10 had when the subject AMHS message entered the AMHS Management Domain operating the Message Transfer and Control Unit, if and only if the originating-MTA-report-request element in the per-recipient-indicators of all the subject AMHS message recipients in the subject Message Transfer Envelope has the abstract-value "audited-report".

3.1.2.3.5.6.2.13.  The original-encoded-information-types element in the Report Transfer Content shall take the value of the original-encoded-information-types element of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.1.3.

3.1.2.3.5.6.2.14.  The content-type element in the Report Transfer Content shall take the value of the content-type element of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.1.4.

3.1.2.3.5.6.2.15.  The content-identifier element in the Report Transfer Content shall either:

a)	take the value of the content-identifier element of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.1.5, if present; or

b)	be omitted in the report if there is no such element in the subject AMHS message.

3.1.2.3.5.6.2.16.  The returned-content element in the Report Transfer Content shall optionally take the value of the content of the subject AMHS message, if and only if the content-return-request element in the per-message-indicators of the subject AMHS message in the subject Message Transfer Envelope has the abstract-value "content-return-requested".

Note.-  The Message Transfer and Control Unit is not mandated to implement the Return Of Content (RoC) Optional Functional Group as defined in ISO/IEC ISP 10611-1.

3.1.2.3.5.6.2.17.  The content-correlator element in the Report Transfer Content shall either:

a)	take the value of the content-correlator element of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.1.11.10, if present; or

b)	be omitted in the report if there is no such element in the subject AMHS message.

3.1.2.3.5.6.2.18.  The actual-recipient-name element in a per-recipient-fields element of the Report Transfer Content shall take the value of the corresponding recipient-name element in the per-recipient-fields of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.2.1.

3.1.2.3.5.6.2.19.  The originally-specified-recipient-number element in a per-recipient-fields element of the Report Transfer Content shall take the value of the corresponding originally-specified-recipient-number element in the per-recipient-fields of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.2.2.

3.1.2.3.5.6.2.20.  The per-recipient-indicators element in a per-recipient-fields element of the Report Transfer Content shall take the value of the corresponding per-recipient-indicators element in the per-recipient-fields of the subject AMHS message as specified in Table 3.1.2-12/Part 1/1.2.3.

3.1.2.3.5.6.2.21.  The arrival-time element in the last-trace-information of a per-recipient-fields element shall take the value of the time at which the subject AMHS message entered the AMHS Management Domain operating the AFTN/AMHS Gateway, as found in the last trace-information-element of the subject AMHS message, as specified in Table 3.1.2-12/Part 2/6.1.2.1.

3.1.2.3.5.6.2.22.  The converted-encoded-information-types element in the last-trace-information of a per-recipient-fields element shall either:

a)	take the last value of the converted-encoded-information-types element in the trace-information of the subject AMHS message, as specified in Table 3.1.2-12/Part 2/6.1.2.4.2, if this element exists; or

b)	be omitted in the report, if no such element is present in the trace-information of the subject AMHS message.

3.1.2.3.5.6.2.23.  If the report is a delivery-report, the message-delivery-time element in the last-trace-information of a per-recipient-fields element shall be the time at which the subject AMHS message has been successfully passed to the AFTN Component by the Message Transfer and Control Unit.

3.1.2.3.5.6.2.24. If the report is a delivery-report,  the type-of-MTS-user element in the last-trace-information of a per-recipient-fields element shall take the abstract-value "other".

3.1.2.3.5.6.2.25.  If the report is a non-delivery-report, the non-delivery-reason-code and non-delivery-diagnostic-code elements in the last-trace-information of a per-recipient-fields element shall take the abstract-values specified in the clause which caused the generation of the report.

3.1.2.3.5.6.2.26.  The originally-intended-recipient-name element in a per-recipient-fields element shall either:

a)	take the value of the first O/R name found in the redirection-history element of the subject AMHS message, if present, as specified in Table 3.1.2-12/Part 1/1.2.5.13; or

b)	be omitted in the report if there is no redirection-history element in the subject AMHS message.

3.1.2.3.5.6.2.27.  The supplementary-information element in a per-recipient-fields element shall take one of the following values:

a)	the value "This report only indicates successful (potential) conversion to AFTN, not delivery to a recipient" if the report is a delivery-report; or

b)	the value, if any, specified in the clause which caused the generation of the report if it is a non-delivery-report.

3.1.2.3.5.6.2.28.  The redirection-history element in a per-recipient-fields element shall either:

a)	take the value of the redirection-history element of the subject AMHS message, if present, as specified in Table 3.1.2-12/Part 1/1.2.5.13; or

b)	be omitted in the report if there is no redirection-history element in the subject AMHS message.

3.1.2.3.5.6.2.29.  The element country-name in the global-domain-identifier element of the MTS-identifier and of the first trace-information-element shall:

a)	be part of the identification of the AMHS Management Domain operating the AFTN/AMHS Gateway by taking one of the following values:

1)	the two-character alphabetical country-indicator as specified in ISO 3166 for the country, or for one of the countries, where the AMHS Management Domain has been registered, if the AMHS Management Domain has been subject to national or multi-national registration; or

2)	a two-character alphabetical indicator dedicated to an international organization, if the AMHS Management Domain has been subject to international registration as defined in ITU-T Recommendation X.666; and

b)	be encoded as a Printable String.

3.1.2.3.5.6.2.30;  The element administration-domain-name in the global-domain-identifier element of the MTS-identifier and of the first trace-information-element shall:

a)	be part of the identification of the AMHS Management Domain operating the AFTN/AMHS Gateway by taking one of the following values, depending on its status:

1)	the name of the ADMD under which the AMHS Management Domain has been registered, either nationally or internationally, if the AMHS Management Domain operates as an ADMD;

2)	the name of the ADMD to which the AMHS Management Domain is connected, if the AMHS Management Domain operates as a PRMD; or

3)	the value single-space if the AMHS Management Domain operates as a PRMD and is unique with regard to the country-name identifying the area where it is registered, either nationally or internationally; and

b)	be encoded as a Printable String.

3.1.2.3.5.6.2.31.  The element private-domain-identifier in the global-domain-identifier element of the MTS-identifier and of the first trace-information-element shall be handled in one of the following manners, depending on the status under which the AMHS Management Domain operates:

a)	generation of the element, with the value of the name of the PRMD, encoded as a Printable String, if the AMHS Management Domain operates as an PRMD; or

b)	omission in the global-domain-identifier if the AMHS Management Domain operates as an ADMD.

3.1.2.3.5.6.2.32.  The element global-domain-identifier in the trace-information or in the internal-trace-information shall:

a)	identify the AMHS Management Domain operating the AFTN/AMHS Gateway; and

b)	be composed as specified in Table 3.1.2-18 / Part 2/2.

3.1.2.3.5.6.2.33.  The element arrival-time in the first element of trace-information or of internal-trace-information shall take the semantic value of the time when the report was generated by the Message Transfer and Control Unit for conveyance in the AMHS.

3.1.2.3.5.6.2.34.  The element routing-action in the first element of trace-information or of internal-trace-information shall take the abstract-value "relayed".

3.1.2.3.5.6.2.35.  The element mta-name in the first element of internal-trace-information shall be the mta-name assigned to the Message Transfer and Control Unit included in the AFTN/AMHS Gateway.

Note.-  The structure of the mta-name of the Message Transfer and Control Unit included in an AFTN/AMHS Gateway within an AMHS Management Domain is a matter of policy internal to the AMHS Management Domain.

# 3.1.3. ATN Pass-Through Service

## 3.1.3.1.	System level provisions

3.1.3.1.1.  The ATN Pass-Through Service shall provide a message environment for the exchange of IA-5 encoded AFTN messages over the ATN Internet Communication Services and with the AFTN via the AFTN/ATN Type A gateway.

Note 1.-  This service does not provide classical store and forward messages services such as found in the AFTN and the ATS Message Service, nor is it visible to users at AFTN stations.

Note 2.-  As a matter of organisations' policy, the implementation of the ATS Message Service may be deferred. In order to take early advantage of the enhanced connectivity provided by the ATN, ATS Organisations with such a policy may implement and operate in the interim the ATN Pass-Through Service. This service provides connectivity for the AFTN traffic as presently defined in Annex 10, Volume II, through the ATN. The interoperability between the ATS Message Service and the ATN Pass-Through Service is a local implementation matter.

3.1.3.1.2.  Recommendation.- ATS Organisations which choose to implement the ATN Pass-Through Service should plan to implement the ATS Message Service at the earliest possible time.

3.1.3.1.3.  Recommendation.- ATS Organisations which choose to implement the ATN Pass-Through Service should provide the interoperability facilities to the ATS Message Service implementations.

### 3.1.3.1.4.	AFTN/ATN Type A Gateway users

The AFTN/ATN Type A Gateway users shall consist of AFTN stations (as defined in Annex 10, Volume II) exchanging AFTN messages.

### 3.1.3.1.5	AFTN/ATN Type A Gateway model

If an AFTN/ATN Type A Gateway is connected to an AFTN Centre which is capable of using only ITA-2 format, the AFTN Component of the gateway shall convert messages to/from the IA-5 format.

Note.-  An ATS organisation may choose to connect an AFTN/ATN Type A Gateway to the AFTN only via its AFTN Centre.  In this case, some requirements placed on the AFTN Component may not have to be fulfilled, provided that the AFTN Centre and AFTN/ATN Type A Gateway together fulfill all requirements.

#### 3.1.3.1.5.1.	AFTN/ATN Type A Gateway information model

The AFTN/ATN Type A Gateway information elements shall consist only of AFTN messages.

#### 3.1.3.1.5.2.	Security and managements models

Recommendation.-  Security should be obtained by procedural means rather than by technical features inherent in the ATN Pass-Through Service.

Note 1.-  The security at each AFTN/ATN Type A Gateway is deemed a local issue to be addressed by the ATS organisation in charge of the system.

Note 2.-  Management is limited to the logging provisions which are defined for the AFTN/ATN Type Gateway. No provision is made for retrieval or exchange of this information, which is deemed a local issue to be addressed by the authority in charge of the system.

### 3.1.3.1.6.	AFTN/ATN Type A Gateway System configurations

3.1.3.1.6.1.  The minimal set of systems implemented and operated by an ATS organisation for the ATN Pass-Through Service shall be one AFTN/ATN Type A Gateway system.

3.1.3.1.6.2.  The minimal set of communications circuits implemented by an ATS organisation operating an AFTN/ATN Type A Gateway shall be:

a)	when integrated with an AFTN centre, access to one ATN subnetwork;

b)	when not integrated with an AFTN centre, one AFTN circuit utilizing a code and byte independent procedures and access to one ATN subnetwork; or

c)	when not integrated with an AFTN centre, one AFTN circuit utilizing any Annex 10, Volume I controlled or Volume II uncontrolled circuit procedure and access to one ATN subnetwork.

Note. - The effect of selecting either 3.1.3.1.6.2 a) or b) is the elimination of the requirement for the AFTN/ATN Type A Gateway to implement the manual teletypewriter procedures, such as service message procedures, channel-check and transmission identification procedures, and code conversion procedures contained in Annex 10, Volume II.

### 3.1.3.1.7.	AFTN/ATN Type A Gateway System naming principles

Naming for each AFTN/ATN Type A Gateway system shall consist of an AP-title set and an AE-qualifier set, as specified in 4.3.2.

### 3.1.3.1.8.	AFTN/ATN Type A Gateway System addressing principles

3.1.3.1.8.1.  There shall be two address forms used in the AFTN/ATN Type A Gateway System:

a)	an AFTN address comprised of an AFTN addressee indicator as specified in Annex 10, Volume II, 4.4.3.1.2 and 4.4.14.2; and

b)	an ATN end-system address comprised of a facility designation, as specified in 4.

3.1.3.1.8.2.  A facility designation shall be assigned to each AFTN/ATN Type A Gateway.

### 3.1.3.1.9.	Routing principles

Routing of messages shall be provided by the AFTN Centres to which the AFTN/ATN Type A Gateway is connected.

### 3.1.3.1.10.	Processing of communication failure

If, for any reason, the Message Transfer and Control Unit is unable to accept AFTN messages passed by the AFTN Component, then the AFTN Component shall handle this situation in compliance with the provisions of Annex 10, Volume II, 4.4.1.5.2.3.

Note.-  Such a condition may be caused by the inability of the Message Transfer and Control Unit to pass messages to the ATN Component.

## 3.1.3.2.	ATN Pass-Through Service Specification

3.1.3.2.1.  An AFTN/ATN Type A Gateway shall consist of the following three logical components:

a)	AFTN component;

b)	ATN component; and

c)	Message transfer and control unit.

3.1.3.2.2.  The three logical components shall interact according to the architecture specified in Volume III Sub-Volume 4.

3.1.3.2.3.  For both the configurations specified in 3.1.3.1.6.2 a) and b), the ATN Pass-Through Service shall be totally transparent for AFTN messages and AFTN service messages to the users of the service, except when applying the procedures for address stripping.

3.1.3.2.4.  For the configuration specified in 3.1.3.1.6.2 c), the AFTN/ATN Type A Gateways shall handle the AFTN procedures as specified in Annex 10, Volume II.

## 3.1.3.3.	AFTN/ATN Type A Gateway Specification

### 3.1.3.3.1.	AFTN component

3.1.3.3.1.1.  The AFTN component shall handle the interface to the AFTN and provide an interface to the Message Transfer and Control Unit.

3.1.3.3.1.2.  The AFTN component shall implement:

a)	all the applicable requirements of Annex 10, Volume II, in a manner so as to be indistinguishable from an operational AFTN station by the AFTN Centre to which the gateway is connected; and

b)	additional requirements which are not placed on AFTN stations by Annex 10, Volume II but which are necessary due to the AFTN Component requirements pertaining to an AFTN/ATN Type A Gateway.

3.1.3.3.1.3.  The AFTN component shall incorporate an AFTN procedure handler that provides all of the AFTN functions prescribed for the interface to the AFTN.

3.1.3.3.1.4.  The AFTN Component shall isolate all AFTN procedures from the Message Transfer and Control Unit Component.

Note. - The AFTN procedure handler includes the procedures for managing the order of AFTN messages based on the transmission priority specified. In using the AFTN procedure handler for managing priority eliminates the need for the Message Transfer and Control Unit to manage message priorities.

3.1.3.3.1.5.  The AFTN Component of an AFTN/ATN Type A Gateway shall perform short term retention of all messages transmitted towards the AFTN in a manner equivalent to that specified for an AFTN communication centre in Annex 10, Volume II, 4.4.1.7 to provide recovery from communication protocol errors.

3.1.3.3.1.6.  The AFTN Component shall perform long-term retention of all AFTN messages, in their entirety, that it generates, for a period of at least thirty days.

3.1.3.3.1.7.  The AFTN Component shall perform long-term retention of the heading, address and origin parts of all messages received from the Message Transfer and Control Unit and the action taken thereon, for a period of at least thirty days.

### 3.1.3.3.2.	ATN component

3.1.3.3.2.1.  The ATN Component shall implement the procedures required of an ATN End System as specified by the ATS Message protocol stack Type A.

#### 3.1.3.3.2.2.	ATN Component service

3.1.3.3.2.2.1.  The ATN Component service shall consist of a single service primitive between it and the Message Transfer and Control Unit, the GA-Data request and indication.

Table 3.1.3-1.  ATN Component Service

| GA-Data Service Primitive | Req | Ind |
| --- | --- | --- |
| User Data | M | M(=) |
| Called Address | M |  |
| Calling Address | M | M(=) |
| AFTN Priority | U | U(=) |

3.1.3.3.2.2.2.  The User Data parameter shall contain the IA5 form of a complete AFTN message, as defined in Annex 10, Volume II.

3.1.3.3.2.2.3.  The Called Address parameter shall contain the ATN-end system id of the destination AFTN/ATN Type A Gateway consisting of the 8-character facility designation as defined in 4.

3.1.3.3.2.2.4.  The Calling Address parameter shall contain the ATN-end system id of the AFTN/ATN Type A Gateway consisting of the 8-character facility designation as defined in 4.

3.1.3.3.2.2.5.  The AFTN Priority parameter, if present, shall contain the AFTN priority indicator of the AFTN message, as defined in Annex 10, Volume II.

3.1.3.3.2.3.  The ATS Message protocol stack Type A shall consist of protocols and procedures specified in 4, and consisting of:

a)	the ATN Component Control Function, which incorporates the Control Function of the Upper Layer Communication Service as specified in 4.3.3 and the additional provisions specified in 3.1.3.3.2.4;

b)	the Dialogue Service as specified in 4.2, consisting of:

1)	the Association Control Service Element,

2)	the Presentation Efficiency enhancements, and

3)	the Session Efficiency enhancements;

c)	the Application Level Naming and Context Definition as specified in 4.3.2; and

d)	the ATN Communication Services requirements as specified in 5.

#### 3.1.3.3.2.4.	ATN Component Control Function

Note.-  The specification of the ATN Component Control Function (CF) does not constrain the implementation of the ATN Component as long as the latter exhibits the external behaviour of the CF as specified.

3.1.3.3.2.4.1.  The ATN Component Control Function (CF) shall map the GA-Data requests and indications to and from the Dialogue Service as specified in 4.

3.1.3.3.2.4.2.  Upon receipt of a GA-Data request, the CF shall determine if a dialogue exists with the destination ATN End-System by examining the Called Address parameter.

3.1.3.3.2.4.3.  If a dialogue does not exist, the CF shall formulate a D-START-request primitive.

3.1.3.3.2.4.4.  The parameters of the D-START-request shall be set according to Table 3.1.3-2.

Table 3.1.3-2.   D-START-request/indication Parameters

| D-START-request Parameter | GA-Data Request Parameter | Value |
| --- | --- | --- |
| Called Peer ID | Called Address | atn-facility designator |
| Calling Peer ID | Calling Address | atn-facility designator |
| DS-User Version Number | ------- | 1 |
| Security Requirements | ------- | <not used> |
| QOS Parameters Routing class Priority Residual Error Rate | ------ AFTN Priority ----- | "ATSC: No Traffic Type Policy Preference" see Table 3.1.3-7 abstract-value "high" |
| User Data | ------- | <not used> |

3.1.3.3.2.4.5.  Upon receipt of a D-START-indication, the CF shall determine if the parameter values are as indicated in to Table 3.1.3-2.

3.1.3.3.2.4.6.  If the parameters received in a D-START-indication are acceptable and sufficient resources available to support the association, the CF shall accept the association by sending a D-START-response, in which the parameters are set according to Table 3.1.3-3 with the Result parameter set to the abstract-value "accepted".

Note.-  The actual definition of "acceptable values" for the parameters of the D-START-indication is a local matter.

3.1.3.3.2.4.7.  If the parameters received in a D-START-indication are unacceptable or there are insufficient resources available to support the association, the CF shall reject the association by sending an D-START-response, in which the parameters are set according to Table 3.1.3-3, with the Result parameter set to the abstract-value "rejected (permanent)" in the case of invalid parameters and set to the abstract-value "rejected (transient)" if there are insufficient resources.

Table 3.1.3-3.   D-START-response/confirmation Parameters

| D-START-response Parameter | GA-Data Indication Parameter | Value |
| --- | --- | --- |
| DS-User Version | ----- | 1 |
| Security Requirements | ------ | <not used> |
| QOS (routing class, priority, and residual error rate) | ----- | <not used> |
| User Data |  | <not used> |
| Result | ------ | "accepted" "rejected (permanent)" "rejected (transient)" |

Note 1.-  The actual definition of "unacceptable values" for the parameters of the D-START-indication is a local matter.

Note 2.-  The use of a security policy (such as only accepting associations from particular remote ATN end systems) to limit acceptance of associations is a local matter.

3.1.3.3.2.4.8.  If the result parameter in the D-START-confirmation is set to the abstract-value "rejected (transient)", the ATN Component CF shall:

a)	re-attempt (a locally defined number of times) the establishment of a dialogue with the same gateway; and

b)	if subsequent to the procedure defined in a) a dialogue still cannot be established, attempt the establishment of a dialogue with a prioritised list of gateways which are defined as being alternates to the one which has been determined as unreachable.

3.1.3.3.2.4.9.  If the result parameter in the D-START-confirmation is set to the abstract-value "rejected (permanent)", the ATN Component CF shall attempt (a locally defined number of times) the establishment of a dialogue with a prioritised list of gateways which are defined as being alternates to the one which has been determined as unreachable.

3.1.3.3.2.4.10.  If any of the QoS parameters in the D-START-confirmation is unacceptable, the ATN Component CF shall perform the following:

a)	abort the dialogue as specified in 3.1.3.3.2.4.19; and

b)	attempt (a locally defined number of times) the establishment of a dialogue with a prioritised list of gateways which are defined as being alternates to the one which has been determined as unreachable with an acceptable QoS.

Note.-  The actual definition of "unacceptable values" for the parameters of the D-START-confirmation is a local matter.

3.1.3.3.2.4.11.  If subsequent to the procedures defined in 3.1.3.3.2.4.8 to 3.1.3.3.2.4.10 a dialogue still cannot be established, the ATN Component shall:

a)	log the error situation;

b)	store the message for further processing; and

c)	ensure that the message is not discarded.

Note.-  The processing to be performed in application of this clause is a local matter.

3.1.3.3.2.4.12.  Upon the completion of the dialogue set-up, or in the case of using an existing dialogue, the CF shall formulate a D-DATA request by taking the data in the User Data parameter in the GA-Data-request and encoding it as the user data field in the D-DATA-request.

3.1.3.3.2.4.13.  The data received in the User Data parameter of the GA-Data-request is the complete AFTN message, which shall be passed transparently to the destination system.

3.1.3.3.2.4.14.  Upon the receipt of a D-DATA-indication, the CF shall extract the user data and place it in the User Data parameter of the GA-Data-indication.

3.1.3.3.2.4.15.  If the CF does not have any data to send over a dialogue for a time period t1, it shall release the dialogue by formulating an D-END-request.

Note. - The time period to wait before releasing a dialogue is a local matter to be determined by cost and expected data traffic.

3.1.3.3.2.4.16.  The parameters of the D-END-request shall be set according to Table 3.1.3-4.

Table 3.1.3-4.   D-END-request Parameters

| D-END-request Parameter | Type | Value |
| --- | --- | --- |
| User Data |  | <not used> |

3.1.3.3.2.4.17.  Upon receiving an D-END-indication, the CF shall release the dialogue as soon as it no longer has any data to send (over that dialogue) by formulating a D-END-response.

3.1.3.3.2.4.18.  The parameters of the D-END-response shall be set according to Table 3.1.3-5.

Table 3.1.3-5.   D-END-response Parameters

| D-END-response Parameter | Type | Value |
| --- | --- | --- |
| Result | INT | abstract-value "accepted" |
| User Data | ---- | <not used> |

3.1.3.3.2.4.19.  For immediate termination of the dialogue, the D-ABORT-request parameters shall be set according to Table 3.1.3-6.

Note.-  The conditions under which this primitive is used is a local matter.

Table 3.1.3-6.   D-ABORT-request Parameters

| D-ABORT-request Parameter | Type | Value |
| --- | --- | --- |
| Originator | ------ | <not used> |
| User Data | ------ | <not used> |

3.1.3.3.2.4.20. Upon receipt of a D-ABORT-indication the dialogue shall be considered to have failed.

Note.-  The recovery mechanisms, if any, to be applied are a matter of local implementation choices.

3.1.3.3.2.4.21. Upon receipt of a D-P-ABORT-indication the dialogue shall be terminated.

Note.-  The recovery mechanisms, if any, to be applied are a matter of local implementation choices.

#### 3.1.3.3.2.5.	Dialogue Service QoS (Priority)

3.1.3.3.2.5.1.  For transmission of messages across the ATN, the AFTN priority indicators, as found in Annex 10, Sub-Volume II, 4.4.1.2, shall map to Dialogue Service QoS (Priorities) in accordance with Table 3.1.3-7.

Table 3.1.3-7.   AFTN/ATN Priority Mapping

| AFTN Priority Indicator | Dialogue Service QoS (Priority) Parameter |
| --- | --- |
| SS | distress communications |
| DD | urgent communications |
| FF | high priority flight safety messages |
| GG | flight regularity communications |
| KK | aeronautical administrative messages |

3.1.3.3.2.5.2.  The ATN component shall process incoming and outgoing messages according to the priority of the message.

### 3.1.3.3.3.	Message Transfer and Control Unit Component

Note.-  The Message Transfer and Control Unit Component provides a bi-directional conversion facility between the AFTN component and the ATN component and consists of:

a)	a set of general functions as specified in 3.1.3.3.3.1;

b)	a set of AFTN to ATN mapping functions as specified in 3.1.3.3.3.3;

c)	a set of ATN to AFTN mapping functions as specified in 3.1.3.3.3.4;

d)	a set of interface requirements between the Message Transfer and Control Unit Component and the ATN Component as specified in 3.1.3.3.3.5; and

e)	a set of interface requirements between the Message Transfer and Control Unit Component and the AFTN Component as specified in 3.1.3.3.3.6.

#### 3.1.3.3.3.1.	General functions

The Message Transfer and Control Unit of an AFTN/ATN Type A Gateway shall log all messages and information related to the following events that have occurred at its interfaces with the ATN Component and with the AFTN Component, and in its internal procedures:

a)	the messages transferred out (to the ATN Component);

b)	the messages transferred in (from the ATN Component);

c)	the AFTN messages conveyed out (to the AFTN Component);

d)	the AFTN messages conveyed in (from the AFTN Component);

e)	the AFTN service messages indicating unknown addressee indicator conveyed out (to the AFTN Component).

Note. - This requirement is not intended to fulfill the 30 day message requirements for an AFTN station.

#### 3.1.3.3.3.2.	Determination of gateway address

3.1.3.3.3.2.1.  The Message Transfer and Control Unit Component shall maintain an address mapping function which maps between an AFTN addressee indicator and the ATN address of the AFTN/ATN Type A Gateway via which the addressee may be reached.

3.1.3.3.3.2.2.  The address mapping function shall, at a minimum, provide the following mappings:

a)	a map from an entire AFTN address to an ATN address,

b)	a map from sets of AFTN addresses based on a portion of the AFTN address to a single ATN address.

Note.-  All AFTN address Indicators are treated as explicit addresses, including predetermined address indicators (PDAIS), thus a single AFTN address can only map to a single ATN address.

3.1.3.3.3.2.3.  Recommendation-  The address mapping function should provide a default mapping of an AFTN Addressee Indicator to an alternate gateway ATN address when the primary gateway ATN address is not in service.

#### 3.1.3.3.3.3.	AFTN to ATN mapping

3.1.3.3.3.3.1.  Upon the reception by the Message Transfer and Control Unit of a message passed from the AFTN Component, it shall examine the AFTN Address Indicators to determine the onward routing requirements of the message over the ATN Internet.

3.1.3.3.3.3.2.  Prior to delivery of the message to the ATN Component, the Message Transfer and Control Unit Component shall apply the address stripping procedures defined in Annex 10, Volume II, 4.4.8 to omit from the address any AFTN Address Indicators not related to the selected ATN address and provide for message replication if more than one ATN address is required.

Note.-  In applying the procedures of 3.1.3.3.3.3.2 the Message Transfer and Control Unit Component provides sufficient copies of the message to reach each ATN address obtained by applying the procedures of 3.1.3.3.3.2.1. In most cases, a set of AFTN addresses will map to a single ATN address (the address of the corresponding ATN Gateway)

3.1.3.3.3.3.3.  The Message Transfer and Control Unit shall send an appropriate service message to the AFTN originator indicator advising of an unknown address indicator according to the following:

a)	the abbreviation SVC,

b)	the procedure signal ADS,

c)	the alignment function,

d)	the indication UNKNOWN,

e)	the unknown address indicator(s),

f)	the end-of-text signal.

#### 3.1.3.3.3.4.	ATN to AFTN mapping

3.1.3.3.3.4.1.  Upon the reception by the Message Transfer and Control Unit of a GA-Data-Indication passed from the ATN Component, the message shall be extracted from the User Data parameter.

3.1.3.3.3.4.2.  The extracted message shall be passed unmodified to the AFTN Component.

#### 3.1.3.3.3.5.	Interface between the ATN Component and the Message Control Unit Component

3.1.3.3.3.5.1.  The interface between the ATN Component and the Message Control Unit Component shall be according to the ATN Component service as specified in 3.1.3.3.2.2.

3.1.3.3.3.5.2.  To send an AFTN message across the ATN, the Message Transfer and Control Unit Component shall invoke a GA-Data-request primitive to the ATN Component.

Note.-  The requirement to invoke the GA-Data-request to the ATN component is not intended to constrain an implementation. The requirement is to pass the required information to the ATN component in a manner consistent with the logical service.

3.1.3.3.3.5.3.  The AFTN message, forwarded by the Message Transfer and Control Unit, shall comprise the User Data parameter.

3.1.3.3.3.5.4.  The called address parameter in the GA-Data-request shall be the facility designation of the destination AFTN/ATN Type A Gateway.

3.1.3.3.3.5.5.  The calling address parameter in the GA-Data-request shall be the local facility designation of the AFTN/ATN Type A Gateway.

3.1.3.3.3.5.6.  The AFTN priority parameter in the GA-Data-request shall be set according to the value of the AFTN priority indicator of the message.

3.1.3.3.3.5.7.  Upon receipt of a D-DATA-indication primitive, the ATN Component shall invoke a GA-Data-indication to the Message Transfer and Control Unit Component.

Note.-  The requirement to invoke an GA-Data-indication primitive to the Message Transfer and Control Unit Component is not intended to constrain an implementation. The requirement is to pass the required information with the Message Transfer and Control Unit Component in a manner consistent with the logical service.

3.1.3.3.3.5.8.  The AFTN message, as found in the User Data parameter of the D-DATA-indication, shall comprise the User Data parameter of the GA-Data-indication.

3.1.3.3.3.5.9.  The calling address parameter in the GA-Data-indication shall be the ICAO facility designator of the AFTN/ATN Type A Gateway which initiated the GA-Data-request, as found in the D-START-indication Calling Address parameter.

3.1.3.3.3.5.10.  The AFTN priority parameter, if present in the GA-Data-indication, shall be derived, using Table 3.1.3-7, from the value of the QoS (priority) parameter of the corresponding D-START-indication.

#### 3.1.3.3.3.6.	Interface between the AFTN Component and the Message Control Unit Component

3.1.3.3.3.6.1.  All AFTN messages or service messages passed by the AFTN Component to the Message Transfer and Control Unit shall be transferred in the order received.

3.1.3.3.3.6.2.  An AFTN message or service message passed by the Message Transfer and Control Unit to the AFTN Component shall be transferred in the order received.