# ATN Concept

## Purpose of the ATN

The ATN is a data communications internetwork that:

- provides a common communications service for all ATSC and AINSC applications that require either ground/ground or air-ground data communications services. [c5 t 0020]

- integrates and uses existing communications networks and infrastructure wherever possible. [c5 t 0030]

- provides a communications service which meets the security and safety requirements of ATSC and AINSC applications, including the reliable and timely delivery of user data to its intended destination. [c5 t 0040]

- accommodates the different grades of service required by each ATSC and AINSC application, and the organisational policies for interconnection and routing specified by each participating organisation. [c5 t 0050]

While these capabilities might, at first sight, appear ambitious, the reality is that for the ATN's users, the internetwork will be straightforward and simple to use.  This is because the ATN’s architecture deliberately places the responsibility for routing and maintaining an internetwork's operational status on the "routers" and therefore enables the End Systems (cf.  Host Computers) to have only a minimal networking capability.

## Technical Benefits

The ATN has been specified to meet the requirements of the Civil Aviation Community and gives the following technical benefits to its users:

| - Use of Existing Infrastructure | The ATN is an internetwork built on top of existing networks through the use of routers as gateways between those networks. Investment in existing LANs, leased lines, CIDIN and X.25 networks is preserved. Furthermore, the ATN can make full use of emerging network technologies such as Frame Relay and Asynchronous Transfer Mode (ATM). |
| --- | --- |
| - High Availability | The ATN has been designed to provide a high availability network by ensuring that there is no single point of failure, and by permitting the availability of multiple alternative routes to the same destination with dynamic switching between alternatives. The same techniques apply to both fixed and mobile communications giving mobile communications an availability level that would have been unrealistic for older technologies based on directory lookups (e.g. ACARS). |
| - Mobile Communications | The ATN fully supports mobile communications over a wide variety of mobile communications networks including AMSS, VDL and Mode S. With the ATN, it is possible for a ground system to communicate with airborne avionics in any part of the world. |
| - Prioritised end-to-end resource management | All ATN user data is given a relative priority on the network in order to ensure that low priority data does not impede the flow of high priority data. Advanced Congestion Management techniques that “throttle back” low priority data when the network becomes near to saturation, ensure that high priority data always gets a low transit delay. In the ATN, traffic load is balanced to the availability of communications resources. |
| - Scaleability | The ATN provides both a large Address Space and an approach to routing that ensures the scaleability of the network well beyond currently foreseen requirements. |
| - Policy based Routing | The ATN’s routing procedures support a wide range of Organisational and National policies, including the enforcing of restrictions on what types of traffic can pass over both ground and air/ground data links, and control over which air/ground data link types are used by which applications. Administrations and Organisations that interconnect the networks are free to enforce routing policies that control which types of data are exchanged and whose data is routed through their networks, and whose data is not. |
| - Future Proofing | The ATN is a way of using networking technologies that can be readily extended to include new ground and air/ground data links technologies, with local rather than global impact of the use of new networking technologies. |

## Construction of the ATN Internet

A general model of the ATN is shown in Figure 2-1. In this model, the ATN consists of a fixed ground network which links satellite, VHF and Mode S ground stations together with ground based Host computers, including both large scale data processing engines and workstations.  ATM avionics on board aircraft are then linked to the rest of the network through satellite, VHF and Mode S datalinks, as appropriate, and may have more than one air/ground datalink in use simultaneously.

The fixed network is not a single entity but itself consists of many different networks all linked together, as illustrated in Figure 2-2.  The ATN ground environment will consist of multiple networks, owned by different administrations and organisations, and implemented using many different technologies.  In some cases, these will be existing networks with spare capacity made available to the ATN.  Others will be new networks implemented specifically to support ATN use.  There will be X.25 Private Packet Switched Data Networks (PPSDNs), Frame Relay Data Networks, Integrated Services Digital Networks (ISDNs), Local Area Networks (LANs) e.g. Ethernet, and others.  These networks are then linked together through routers which provide the connectivity between the different types of data network, and to the air/ground networks. Host computers are directly connected to a nearby data network, typically a LAN.

User data is switched by the routers as discrete packets formatted according the ISO Connectionless Network Protocol (CLNP).  Each packet is viewed as a separate event and routed according to a “route map” of the ATN.  In the ATN, each router has a portion of the full ATN route map and builds and maintains this route map dynamically using routing information passed to it by its neighbouring (adjacent) routers.

Host computers communicate with each other either directly over a common data network, or use the services of a router to provide a communications path to a Host on another data network.  It is the responsibility of the routers working together to find a suitable path through the networks which they interconnect, and data may travel through many different routers and via many different networks on its journey between two Hosts.  In order to build an ATN route map for this purpose, the routers exchange, amongst themselves, information on which hosts are local to them (i.e. reachable via a single data network and with no intermediate router), and on how they relate to other routers. From such information, the routers can plot the course of data through the ATN.

Figure 2-1 General Model of the ATN

The ATN Ground Environment permits each participating organisation to organise its networks and systems as it wishes, and form a separate “Administrative Domain”, However, it is also anticipated that Administrations and ATN regions will, wherever possible, co-ordinate their addressing plans and network topologies, such that the amount of routing information passed between organisations and regions can be kept to an absolute minimum.

The ATN Addressing Plan apportions a separate part of the address space to each ICAO Administration and to each IATA airline and other organisations.  This allows for great flexibility in use, however, participating organisations are, as indicated above, strongly recommended to co-ordinate the allocation of addresses.  For example, in Europe, Administrations should implement a co-ordinated addressing plan with a unique address prefix for Europe and address assignment that reflects the actual topology of the European ATN Internet.  For similar reasons, airlines, and especially small regional airlines should consider service provider relative addresses.

## Users’ View of the ATN

First, it is worth considering who or what is an ATN user.  In principle, the ATN User is an ATM application, or some other application supporting an aeronautical application.  But, this is very much an end system view.  From the networking point of view, there are many interfaces over which "a user" accesses a service.  At each such interface, each user can be considered to be an ATN User.  In the remainder of this section, the term "ATN User" is used in this sense, i.e. as a user of the network service or transport service, depending on context.

ATN User Communications Capabilities

Figure 2-2 The ATN Ground Environment

The ATN provides its users with a robust and reliable communications service, together with the option of a datagram service. Formally, all communications aspects of a user's system are part of the ATN, but from a "user's point of view", the ATN is out there, separate from their own system.  It is this "user's view" of the ATN that is illustrated in Figure 2-3.  This figure shows the ATN as an abstract "cloud" which indeed is all the user need be aware of, with its complexity hidden from view.  At this level, the ATN is a simple network that provides a datagram service to its users.

A ground based ATN user's system, which might be anything from a complete ATC system to an entry level PC, accesses ATN services via some ATN access point.  This access point is a notional socket into which the user "plugs" their system and thereby gains access to the ATN.  However, this socket is not as tangible as an electrical power socket.  A user's access to the ATN is first via an "access subnetwork", such as an Ethernet or an X.25 PSDN, and then an ATN Router.  The user's system is directly connected to the access subnetwork, and this may very well involve a physical connection provided by a wall-socket, and, using the access subnetwork, the user's system communicates with the ATN Router.  It is then through the ATN Router that access is gained to ATN services.

The communications capabilities of the user's system must obviously include the hardware and software necessary to use the "access subnetwork".  Furthermore, the user's system must also support the ISO/IEC 8473 Connectionless Network Protocol  in order to be part of the ATN Internet, and is recommended to support the ISO/IEC 9542 End System to Intermediate System Routing Protocol.

Figure 2-3 Individual End User's View of the ATN

#### ISO/IEC 8473 CLNP Protocol

CLNP is a simple protocol supporting the transfer of "datagrams" i.e. packets of data transferred from sender to receiver without the need for a connection to be established in advance.  Data transferred using CLNP is formatted as a block of data preceded by a protocol header containing the addresses of the sender and destination, the priority of the data, any security label associated with it, and quality of service requirements.  Header and data must together not exceed 64 kilobytes.

An ATN user may, at any time send a CLNP formatted datagram to any valid destination address.  The user does this by passing the datagram over the access subnetwork to the ATN Router.  The ATN Router will inspect the protocol header, and it is then the ATN Router's responsibility to forward the datagram through the ATN to the ATN Router which provides ATN access to the addressed destination.  How it does this is internal to the ATN and hence hidden from the user, although the forwarding process must respect the data priority and the Quality of Service and Security requirements identified in the protocol header.  Once the datagram has arrived at the ATN Router which provides ATN access to the addressed destination, it is then transferred over the destination's access subnetwork to the destination user.  If the destination user is offline (e.g., switched off), the datagram is discarded  and an error report is optionally returned to the sender.

The operation of these processes is essentially how the user perceives the ATN.  The simple CLNP is the protocol ATN users use to communicate, and permits those users to exchange information as discrete blocks of data.

#### ISO/IEC 9542 ES-IS Protocol

The other protocol that users are recommended to support - the ES-IS protocol - is really just for local administration.  The user's system uses the ES-IS protocol to report its own address to the ATN Router, and this information is regularly repeated so that the ATN Router can monitor a user's online status.  It is also used to report the existence and operational status of an ATN Router to its users, and enables an ATN user to have access to multiple ATN Routers, possibly over different access subnetworks, so as to provide a high availability service.

#### CLNP Delivery Probability

The ATN itself does not make any demands on the syntax or semantics of the data carried in a CLNP packet.  However, the simplicity of the service does carry a penalty and this is that delivery of datagrams is not guaranteed.  When a user transfers an ISO/IEC 8473 formatted packet to an ATN Router, that user is only guaranteed a probability of delivery dependent on the data priority.  The probability of delivery is high, and while no targets have yet been set for delivery probability, 97% - 98% is certainly realistic. Considerations that affect this figure include:

- the error rates on subnetworks such as Ethernets which may lose data in transit due to line errors (although this consideration does not apply to X.25 subnetworks and similar examples, which provide a reliable transfer service)

- network overload which results in low priority data being discarded in order to free up congested resources

- component failures. [c5 t 0080]

The actual delivery probability that is provided is a design issue.  Once actual targets have been provided then it is possible to design a network to meet the requirement.  This is achieved by minimising the use of lower reliability subnetworks, increasing overall network capacity, and through component redundancy.

However, the network can never provide a 100% delivery probability.  When an ATN user does require reliable data transfer, then the end to end ISO/IEC 8073 class 4 transport protocol is required, in addition to the CLNP.  This protocol itself uses CLNP packets to convey information between ATN users.  The protocol can detect data loss and recovers from it by retransmission.  It can also provide end to end flow control and multiplexing of different data streams between the same pair of users.  When this protocol is used, the impact of a comparatively low delivery probability is on mean transit delay (the average time it takes to transfer data from source to destination).  This is because recovery from data loss is by retransmission, and hence the lower the delivery probability, the longer the mean transit delay.  There is hence a need to offset the impact of an increased mean transit delay against the cost and design implications of higher delivery probability.

ATN users that do not require a high delivery probability (this class includes time critical applications such as radar related data transfer,) could in principle directly use the transfer service provided by CLNP, but this is not permitted in the ATN.  ATM applications for which the ISO/IEC 8073 COTP Class 4 is not appropriate are instead required to use the ISO/IEC 8602 connectionless transport protocol, which specifies a format for data transferred by the CLNP. The advantage of this protocol is that it decouples the internal structure of a user's system and the applications it hosts, from network routing.  This is because ISO/IEC 8602 enables multiple users to be reached through one network address rather than one per user, which would be less efficient from the network point of view, and the number of such addresses is limited.

### The User as an Organisation

When the ATN user is an organisation it has multiple systems and subnetworks to consider. It is no longer attaching to the ATN as an individual End System, but as an organisation, which may access the ATN and provide transit services to other ATN users i.e. be an ATN Service Provider.

Figure 2-4 illustrates the changed perception.  The "organisational" ATN user has both Host Computers which are individual ATN users as before, but also operates a portion of the ATN cloud, with at least one link to the rest of the ATN.

Figure 2-4 Organisational End User's View of the ATN

#### Organisations and  Routing Domains

The portion of the ATN operated by the organisational user may be no more than a single ATN Router.  The portion of the ATN operated by such a user is terned an Administrative Domain. Alternatively, a larger organisational user may operate a large number of ATN Routers, interconnected by various subnetworks also operated by the organisational user, and providing ATN access to all the systems owned by the organisational user which require ATN access.  Such Routers may also be general purpose and be part of that user's own internal network.  However, regardless of how many ATN Routers there are within an organisation, the ATN Routers and the Host Computers to which they provide ATN access, typically form what is known as a Routing Domain.  The Routing Domain is a structure specified in the ISO standards and imposed on the ATN Internet in order to enable a structured approach to be taken to solving the routing problem. This is illustrated in Figure 2-5.

Within a Routing Domain, ATN users are recommended to use the ISO/IEC 10589 intra-domain routing protocol.  This is a simple and robust routing information exchange protocol that is specified for use between systems that mutually trust each other (i.e., belong to the same user).  This protocol exchanges connectivity information throughout the Routing Domain and enables each ATN Router to build up a compete topology map of the Routing Domain, so that every ATN Router knows which routers within the Routing Domain provide access to which Host Computers, and how the routers themselves are interconnected.  Routes can then be plotted through the Routing Domain, and CLNP packets forwarded to their addressed destinations within the Routing Domain.

Within an ATN Routing Domain, there will be one or more ATN Routers that are permitted to route CLNP packets to external destinations, i.e., addressed destinations that are located in the "rest of the ATN".  These routers are known as Boundary Intermediate Systems (BISs), because they exist at the boundaries of Routing Domains.

Figure 2-5 The ISO Routing Domain

The ATN simply consists of multiple Routing Domains.  Each such Routing Domain is self-consistent and capable of internal routing.  However, key to the ATN being a single internetwork as opposed to a collection of separate Routing Domains, is the capability of inter-domain routing between BISs.

In the ATN, it is mandatory for a BIS to support the ISO/IEC 10747 Inter-Domain Routing Protocol (IDRP).  For inter-domain communications, this protocol requires that BISs communicate directly over a common subnetwork, which may be owned by the owner of either BIS, or by a third party.  Rather than exchanging connectivity information, as is done between routers within a Routing Domain, BISs advertise routes to each other, where a route consists of the set of addresses which identifies the destinations reachable over the router, and information about the route's path including the Quality of Service and Security available over the route.

#### Use of Policy Based Routing by Organisations

It is the BIS's responsibility to determine which routes, if any, it will advertise to another BIS, and the use it will make of routes which it receives.  When the BISs within a Routing Domain receive alternative routes to the same destination, then they must collectively determine which is the best route and hence which of the alternatives will be used.  The set of rules which determines the advertisement and use of routes is known as a Routing Policy, and each organisational user of the ATN must determine and apply their own Routing Policy.

It is the need for policy based routing between different organisations that underlies the need for the existence of Routing Domains.  Policy based routing enables users to control external access to their communications resources, and to protect themselves from problems elsewhere in the internetwork. BISs may also, depending on Routing Policy, advertise to BISs in other Routing Domains routes that have been received from another Routing Domain, and thereby offer transit facilities.  However, Routing Policy may also prevent such routes from being re-advertised and hence deny transit facilities.

Organisational ATN users must therefore ensure that they either have direct connections with the ATN Routing Domains with which communication is necessary, or that those Routing Domains with which direct connections exist also offer suitable transit facilities to the remainder.  In principle, this could be done on a bilateral basis between ATN organisational users on an "as needs" basis, and this is generally what is expected for the support of ground-ground communications.  However, for air-ground applications support, this is unlikely to be an efficient strategy and may actually prevent useful communication by putting too high a cost on establishing a usable path even when connectivity already exists.

Instead, it is intended that ATN interconnections for support of air-ground communications are coordinated on both a regional and worldwide basis, so that an ATN backbone (of Routing Domains offering general transit facilities) is created, with either a clear apportionment of costs, or a known tariff, for use of transit facilities.  This way users can gain access to the full capabilities of the ATN quickly and cheaply.

Policy based routing plays a significant role in the ATN, where it is used to support user requirements for control over the user of data links and for optimising the distribution of routing information for routes to mobile systems.

### Mobile Users

The ATN will incorporate many "mobile" subnetworks.  Examples of such subnetworks include SSR Mode S, AMSS and VDL.  If an aircraft were to attach to one mobile subnetwork only and never to any other, then even though sometimes it may be attached and at other times not attached, this has no consequence for the ATN.  This is because from the point of view of the rest of the ATN, it would be no different from a fixed system that was occasionally off-line.  However, that is not how mobile subnetworks are used. An aircraft will attach to many different mobile subnetworks during the course of its flight.  A long haul aircraft may move between the coverage areas of different satellites; an aircraft flying over a land mass will fly between different Mode S subnetworks as it passes over different countries.  And, at the same time, the applications on board the aircraft will need to maintain contact with applications on the ground.  Mobile platforms thus require special routing considerations.

In the ATN, mobile "platforms" are treated in a similar manner as organisational users.  That is, the systems on board an aircraft are required to form a Routing Domain and hence must include an ATN Router that is also a BIS.  This is partly because the ISO/IEC 10747 routing protocol provides a relatively efficient mechanism for the transfer of routing information over low bandwidth links, but also because aircraft are almost always organisationally separate to the ground systems with which they are in contact and the same requirements for policy based routing apply.

The existence of mobile users has a significant impact on the organisation of the ground based ATN.  While the ground topology will change only slowly, each aircraft's point of contact with the ground ATN will change rapidly with a consequent impact on the volume of routing  information exchanged, and the routing tables in each router.  A strategy is necessary for containing this high rate of information flow, and also to avoid the  problems of routing instability caused by a rapid turnover of routing information.

This is the ATN Mobile Routing Strategy and is based on a two level concept of default route providers. The first level is provided by a default route provider to all aircraft in a given region (known as an ATN Island). This default route provider is kept informed about routes to all aircraft currently in that region and hence can always provide a path to such aircraft. Several such default route providers may exist in the same region and collectively they are said to form the ATN Island’s Backbone.

The second level is provided by an aircraft’s home. The "home" of an aircraft does not necessarily relate to an airline's headquarters, its maintenance facilities, or indeed any geographical concept of "home".  It is simply a particular ATN Routing Domain, and, in principle, any ATN RD  will do.  It may be an RD belonging to an aircraft's airline, but equally it may belong to a Service  Provider or an Administration.  Typically, all aircraft belonging to the same airline, or the General  Aviation (GA) aircraft of a single country share the same home.

The ATN’s default route providers in each ATN Island keep a “home” informed about the location of all of its aircraft that are current in ATN communication. Thus, if a particular default route provider needs to route a packet to an aircraft for which it does not have an explicit route (i.e. it is not in the same region), all it has to do is to route the packet to the aircraft’s known home and from there it can be forwarded to the ATN Island with which it is in contact and thence to that aircraft.

#### Route Initiation

The establishment of a communications path between BISs in any two Routing Domains is known as “Route Initiation”.  These procedures apply to the establishment of both ground/ground and air/ground communications.  However, as opposed to the ground/ground case, Route Initiation for mobile users is dynamic and has to follow ICAO specified procedures for which guidance is given in section 5.10.

### Routing Control

An important user requirement is that its users can specify, on a per application basis, routing control requirements. For AOC Applications, the requirement is for control over the air/ground data link used for air/ground applications. For ATSC Applications, the requirement is to follow only ATSC approved routes, and further, to be able to classify routes in the range class ‘A’ to class ‘H’, and for the user data to follow a route of the most appropriate class. Some administrations may also restrict the type of traffic carried over certain air/ground data links. Such restrictions must also be taken into account.

The ATN meets these requirements by:

- Permitting user’s to identify, in the CLNP Header, the traffic type of the data being conveyed (e.g. ATSC, AOC, General Communications, etc.), and the routing control requirements.

- Carrying information about the air/ground data links a route traverses, and any restrictions placed upon those data links, in each IDRP Route.

- Carrying information about whether a route is approved for ATSC purposes, and the assigned ATSC Class of the route, in each IDRP Route.

When a CLNP PDU is forwarded by an ATN Router, the user requirements are matched against the available routes and the appropriate route followed.

In the case of AOC traffic, the user’s requirements are enforced in a “strong” manner. That is, if a route meeting the user’s requirements is not available, then the data is discarded.

In the case of ATSC traffic, a similar “strong” interpretation is made of the requirement to follow an ATSC approved route. However, the router will then simply choose the route with the best matching ATSC Class. This is a route with the requested class, or a higher class, or if no higher class route available, then the ATSC approved route with the highest specified class out of those available.

## Technical Overview of the ATN

### Protocol Architecture

The ATN Protocol Architecture is illustrated in Figure 2-6. This shows the protocols specified for two types of ATN End System and two types of ATN Router, and how those protocols relate to each other and in which OSI layer they are located, according to the OSI reference model specified by ISO in ISO/IEC 7498-1. The figure also shows a number of important interfaces.

The description of the ATN in 2.4 above has illustrated the fact that there is not one ATN interface, but instead there are many ATN interfaces, each of which serves a different user in a different role.  In order to avoid confusion, a taxonomy of interfaces has been developed.  This taxonomy identifies each significant interface as a reference point, and at each such reference point, there is an interface between two ATN entities, one taking on the role of a user, and the other as the service provider. Figure 2-6 illustrates the location of each ATN Reference Point, and the meaning of each reference point is given below.

|  | Reference Point One, the Transport Reference Point, is the OSI Transport Service interface and follows ISO/IEC 8072. This reference point is wholly contained within an ATN Host Computer, and represents access to the ATN Internet by an ATN Host Computer at the Transport Layer level. The user of the service provided in this example, is the OSI Session Entity; the service provider is the transport layer entity. |
| --- | --- |
|  | Reference Point Two, the Network User Reference Point is the interface between the services of the network layers located in an ATN Host Computer and an ATN Router providing access to the ATN Internet. It comprises the OSI protocols used to access ATN Services. Note. - An ATN Router relays data between ATN Host Computers either directly to another ATN Host Computer or via one or more further ATN routers, which may or may not belong to other ATN Service Providers. When both Host Computer and Router are owned by the same organisation, then the protocols that provide this interface are not mandated; the specification only recommends an appropriate stack. |
|  | Reference Point Three, the Network Provider Reference Point, is at the interface between two Routers belonging to different organisations. It comprises the OSI protocols used to support end-to-end communications via multiple ATN Routers, possibly via multiple subnetworks. Note. When both Routers belong to the same ATN Service Provider then the protocols that provide this interface are not mandated; the specification only recommends an appropriate stack. |
|  | Reference Point Four, the Subnetwork Provider Reference Point, is at the interface between an ATN Host Computer or an ATN Router and a subnetwork. It comprises the OSI or subnetwork specific protocols used to access the service provided by that subnetwork and identifies the services provided by a real subnetwork used to connect two ATN components. This reference point identifies the lower boundary of the scope of the ATN ICS SARPs. |

The provider of the service at reference point 4 is out of the scope of the ATN ICS SARPs.  As indicated above, the ATN places only very limited constraints on the service provided at reference point 4, and this enables almost any subnetwork to be used as an ATN subnetwork.

#### The ATN Transport Layer

The OSI Transport Layer service provides transparent transfer of data between Transport Service users.  All protocols defined in the Transport Layer have an 'End-to-End' significance, where the 'Ends' are defined as co-operating transport entities on two ATN host computers.  The Transport protocol operates only between end systems.  Within the ATN, Transport Layer entities communicate over the ATN using the Network Service provided by the ATN Network Layer Entities.

There are two modes of the transport service, the Connectionless mode Transport Service and the Connection-mode Transport Service.  The connectionless mode service allows two transport users to exchange individual datagrams, without flow control or the need to have previously established a connection, but with no guarantee of delivery.  The connection-mode service allows two transport service users to negotiate a communications channel with a set of common characteristics, including reliable delivery of data units, and guaranteed (very high probability) order of delivery.

The two OSI protocols that provide the two modes of the transport service have separate specifications, and operate independently.  Based on the higher level protocols operating within a given ATN host computer, one or both of the transport protocols may be implemented.  Neither transport protocol is concerned with routing and relaying of data between End Systems, which is the responsibility of the Network Layer.  The protocol in support of the CLTS is specified in ISO/IEC 8602, and the protocol in support of the COTS is specified to be ISO/IEC 8073 Class 4.  The implementation of these protocols within the ATN is further described in Chapter 4.

The Transport Service boundary corresponds with ATN reference point 1.

The ATN Network Layer

Figure 2-6 ATN Protocol Architecture

The OSI Network Layer Service, like the OSI transport service is specified to provide both a connection mode and a connectionless mode service.  However, in the ATN, the Network Layer Service is restricted to the connectionless mode only.  This is because, unlike the transport layer, the same network protocols must be implemented in every system in the internetwork, if interoperability is to be guaranteed.  In the case of the transport layer, the mode of the service required depends on the requirements of the users, and those End Systems that implement the same applications must also implement the same transport layer protocols.  However, the internetwork itself must relay the data of all users, regardless of the mode of the transport service used.  In order to provide universal connectivity, a consistent set of protocols must be implemented across the internetwork.  Even if universal connectivity was ruled out, in practice, most ISs would still have to support all modes implemented by ESs, because of the tendency for data pathways to cross each other, regardless of the network service mode supported by each such data pathway.

It is thus cost effective to support only one mode of the network service.  Implementation costs are reduced, and the complexity of validation is also reduced. Furthermore, mobile routing is not yet believed to be practicable when using the connection mode network service.

The Network Layer Service is independent of the Transport Layer Service and may be used by ISO/IEC 8602 to provide the CLTS, and by ISO/IEC 8073 (class 4 procedures only) to provide the COTS.

The OSI Network Layer comprises three sub-layers or roles:

- Subnetwork Independent Convergence Role, which is responsible for providing a consistent Network Layer Service regardless of the underlying subnetwork

- Subnetwork Dependent Convergence Role, which decouples the functions of the Subnetwork Independent Convergence Role from the characteristics of different subnetworks.

- Subnetwork Access Role, which contains those aspects of the network layer specific to each subnetwork.

##### The Subnetwork Independent Role

In an ES, the Subnetwork Independent Role is responsible for providing the OSI Network Service independent of the real subnetwork(s) to which the ES is attached.  In an IS, the Subnetwork Independent Role is responsible for the routing and relaying of user data along its route between the two communicating users.  The protocols that support the exchange of routing information are also contained within this functional area.

In support of the connectionless mode Network Service, it is a mandatory requirement that all ATN ESs and ISs implement the ISO/IEC 8473 internetworking protocol.  This is a subnetwork independent protocol and supports the relaying of connectionless data PDUs over multiple subnetworks.  By choosing such a protocol as its unifying characteristic, the ATN is cast as a subnetwork independent internetwork.  CLNP supports the ISO global network addressing plan, quality of service specification, congestion control, and segmentation and reassembly of data packets.  Additionally, provisions exist within CLNP for diagnostic actions such as end-to-end route recording and error reporting.

Three Routing Information Exchange Protocols are also specified in support of ISO/IEC 8473 within the ATN.  These are:

- ISO/IEC 9542 - the End-System to Intermediate-System (ES-IS) protocol

- ISO/IEC 10589 - the Intermediate-System to Intermediate-System (IS-IS) intra-domain routing information exchange protocol

- ISO/IEC 10747 - the Inter-Domain Routing Protocol (IDRP)

The use of these protocols is outlined below and described in more detail in Chapter 4.

###### End-System to Intermediate System Routing Protocol

The ISO/IEC 9542 ES-IS protocol provides a mechanism for ESs and ISs to exchange connectivity information within a local subnetwork environment.  It is recommended for implementation in all ATN ESs and all ATN ISs that support ES attachment.  In this role, its use applies to reference point 2.

The protocol enables ESs and ISs to dynamically discover each other when attached to the same subnetwork (only on broadcast subnetworks), and for ISs to inform ESs of optimal routes.  In the absence of ISs (on broadcast subnetworks), ESs may also locate each other on an as needs basis.

The ES-IS protocol also complements the IS-IS routing protocols to support dynamic discovery of other ISs and/or their NETs, and is also used in a similar manner to support the Inter-Domain Routing Protocol over mobile subnetworks.

###### Intra-Domain Routing Information Exchange Protocol

The ISO/IEC 10589 IS-IS intra-domain routing information exchange protocol is used by ISs within the same Routing Domain to exchange connectivity and QOS information.  As the ISs within a single Routing Domain are always operated by the same organisation, this protocol is not used at any of the ATN interfaces identified by reference points.

The protocol works at two levels.  Level 1 operates within the same Routing Area, while level 2 operates between Routing Areas.  From the information exchanged by this protocol, ISs build up a topography map of the local Routing Area at level 1, or Routing Area connectivity, at level 2.  From this map, optimal routes can be plotted, and the relevant information provided to each IS's Forwarding Information Base.

###### The Inter-Domain Routing Protocol (IDRP)

The ATN has adopted the ISO/IEC 10747 Inter-domain Routing Protocol, for the exchange of dynamic routing information at the inter-domain level. IDRP is a “vector distant” routing protocol and is concerned with the distribution of routes where a route comprises a set of address prefixes for all destinations along the route and the route’s path i.e. the list of Routing Domains through which the route passes in order to reach those destinations.  In addition, a route may be further characterised by various service quality metrics (e.g. transit delay).

Under IDRP, specialised Boundary Routers in each Routing Domain advertise to Boundary Routers in adjacent Routing Domains, routes to the systems contained in that Routing Domain. Typically, there is a route for each performance metric and security category supported, and the destination of these routes is the Address Prefix(es) that characterises the Routing Domain. The receiving Routing Domains then store this information and use it when they need to route packets to destinations within the other Routing Domain.  A route so received may also be re-advertised to other Routing Domains adjacent to the Routing Domain that first received it, and onwards throughout the ATN Internet.  Ultimately, every Routing Domain in the ATN Internet can receive a route to every other Routing Domain.

However, without any other functionality, IDRP would not provide a scaleable approach to routing.  In order to provide such a scaleable architecture, IDRP enables the aggregation of routes to Routing Domains with common address prefixes, into a single route.  It is thereby possible for the number of routes known to any one router to be kept within realistic limits without reducing connectivity within the Internetwork.

##### Subnetwork Dependent Role

The OSI Subnetwork Dependent Role is responsible for decoupling the functions of the subnetwork independent role from the characteristics of different subnetworks and provides a consistent service to any protocols implemented by the subnetwork independent role.  In doing so, it may implement a convergence protocol, implemented on a hop-by-hop basis, independently over each subnetwork.  This is a Subnetwork Dependent Convergence Protocol .

ISO/IEC 8473 may be adapted to all known subnetwork types and hence a SNDCP is not specifically required.  However, each subnetwork class does require a different adaptation, and each such adaptation is known as a Subnetwork Dependent Convergence Function.  Chapter 4 discusses the SNDCFs that may be used to interface ISO/IEC 8473 to ATN subnetworks.

However, while ISO/IEC 8473 does not require an SNDCP, there is justifiable concern over the ISO/IEC 8473 protocol overhead in respect of the low bandwidth communications provided by the mobile subnetworks.  For this reason, an SNDCP has been specified to provide compression of the ISO/IEC 8473 protocol header over mobile subnetworks.  This is known as the LREF compression algorithm and is further described in Chapter 4 as part of the description of the Mobile SNDCF which supports LREF and other compression algorithms..

##### Subnetwork Access Role

The Subnetwork Access Role comprises the functions necessary to support access to a specific subnetwork.  This is dependent on the specification of each subnetwork and is hence outside of the scope of this document.  The service provided by the Subnetwork Access Role to the Subnetwork Dependent Role is at ATN reference point 4, which identifies the lower boundary of this manual.

### Congestion Management

Congestion is a phenomenon experienced by a Router in an Internetwork when the queuing delays through that Router exceed the maximum acceptable limit. In such a situation, the end-to-end transit delay is likely to exceed the maximum acceptable for the internetwork’s users. In the extreme case, a congested router, due to lack of buffer space, may not be able to accept incoming NPDUs at the rate that an adjacent router is trying to send them, and is hence forced to discard lower priority NPDUs, or those near the expiry of their lifetime, in order to make way for higher priority NPDUs.

Congestion is not a problem for an internetwork. Congested routers can simply discard NPDUs when they start running out of buffers. However, it is a serious problem for the users of the internetwork. Congestion first results in an acceptably long transit delay. However, if network users assume that the lack of arrival of an end-to-end acknowledgement is due to packet loss, rather than simply an unexpectedly long delay in the network, then they can retransmit such unacknowledged packets, thus adding to the load on the network.

In fact, a catastrophic degradation in transit delay and throughput can be observed in a congested network. First the network becomes congested, then users start retransmitting, making the network even more congested, resulting in more retransmissions, and so on, until the point is reached where only insignificant amounts of data can be transferred. It is therefore vital that Congestion Avoidance mechanisms are put in place in any internetwork, if it is not to be perceived as unstable and unreliable.

The Congestion Avoidance Procedures specified for the ATN represent a significant improvement on those which many will be familiar with from experience with the TCP/IP Internet. TCP Congestion Management only recognises that a network is congested when their is a need for a sender to retransmit a packet, when it is assumed that packet loss has occurred due to an overloaded router discarding packets. The sending system then first reduces the rate at which it transmits packets and then gradually speeds up again until it needs to retransmit, and so on.

The problem with this approach is that it constantly pushes the network into overload and, like a congested motorway, results in poor and turbulent traffic flow and a much greater loss in throughput than should have occurred.

The ATN takes a quite different approach. In the ATN a router that is approaching congestion indicates this by setting a “flag bit” in each packet header (this situation occurs when the outgoing packet queue is longer than one packet), and this flag is recognised by the receiving system. If enough flags are set in a given sampling period, the receiving system reduces the credit it offers to the sender, thus reducing the load on the network.

This approach has the significant advantage that it “kicks in” before network performance starts degrading thus permitting a much more stable traffic flow and enables near optimal throughput to be maintained. The ATN Congestion Management algorithm is discussed in more detail in Chapter 6.

### Addressing

Every system within a network such as the ATN, must have a unique address.  This address may then be used to identify the source and destination of a packet sent through the network. ATN routers use a packet’s destination address to determine how the packet is routed to its destination.

An address is therefore more than a unique identifier for each system, and to be truly useful, it must be possible to use an address to find out how to reach the addressed system i.e. to select the most appropriate route.  That is an address must somehow relate to a network’s topology.

A useful example of this concept is provided by telephone numbers.  The ITU has published a global numbering plan for telephone number allocation and, in principle, each telephone number consists of a “country code”, an “area code”, an “exchange code”, and a “subscriber number”.  Telephone numbers are closely related to the topology of the telephone network. Given a subscriber’s telephone number it is possible to identify the country and area in which their local exchange is located, and by using this close relationship between topology and telephone numbers, the routing of telephone calls can be readily accomplished.  As long as, for example, each inter-area telephone exchange has routing tables that identify the routes to each other area within the same country, telephone calls between different areas within the same country can be readily made.  Only within an area does the connectivity between exchanges inside the area, need to be known.

Routing Domains can be viewed as being like telephone areas, and like all subscriber numbers in a telephone area, the addresses of systems within the same Routing Domain should all have a common prefix.  Then a packet sent to any system in the Routing Domain, can be sent to the Routing Domain without the routers along the way having to have any knowledge of the topology of the networks and routers within that Routing Domain.

Routing Domains are, however, a more flexible concept than telephone areas.  The requirement for a single common address prefix is not absolute, and it is possible to have more than one address prefix that characterises a single Routing Domain.  The geographical country is also not present in either the ISO Routing Framework, or as a fixed quantity in the Address Plan.  Instead, there is the very general concept of the Routing Domain.

There is also no requirement in the ISO Routing Framework for the address prefixes that characterise adjacent (i.e. linked by a common network) Routing Domains, to have any similarity (i.e. for there to be another (shorter) address prefix common to each Routing Domain’s address prefix).

If all inter-Domain interconnections are simply developed on an ad hoc basis with no aim to create a Global ATN Internet, then any lack of similarity between the address prefixes assigned to adjacent Routing Domains is not an issue.  However, if a scaleable routing architecture (i.e. one which permits effectively unlimited growth) is to result then there does need to be some similarity between the address prefixes characterising adjacent Routing Domains.  Then it will be possible to group Routing Domains together and advertise routes to a group of Routing Domains, rather than to each individually.  This is similar to telephone networks grouping of all the areas in one country together and treating them as a whole from other countries.  With such a strategy it is possible to develop a scaleable routing architecture such that the further away a router is from a packet’s destination, the less detailed the routing information needs be to successfully route the packet.

This is a very important feature of a scaleable architecture, because if the amount of routing information required by at least one router is in proportion to the size of the ATN Internet then the maximum size that router can be, places a limit on the size of the network as a whole.

An ATN Addressing Plan has therefore been specified which permits both the delegation of Address Management to Administrations and other Organisations, but which also permits the development of a scaleable internetwork.